Greenhouse Gas Inventories, for England, Scotland, Wales and Northern Ireland: 1990 - 2001

S Baggott, R Milne, T Misslebrook, TP Murrells, G Thistlethwaite, J Watterson

February 2004

Greenhouse Gas Inventories, for England, Scotland, Wales and Northern Ireland: 1990 - 2001

S Baggott, R Milne, T Misslebrook, TP Murrells, G Thistlethwaite, J Watterson

February 2004

| Title | Greenhouse Gas Inventories, for England, Scotland, Wales and Northern Ireland: 1990 - 2001 |
| --- | --- |
| Customer | Department of Environment, Food and Rural Affairs |
| Customer reference | EPG 1/1/120 |
| Confidentiality, copyright and reproduction | Copyright AEA Technology plc 2003 All rights reserved. Enquiries about copyright and reproduction should be addressed to the Commercial Manager, AEA Technology plc. |
| File reference | /tmp/in/input.doc |
| Report number | AEAT/ENV/R/1481 |
| Report status | Issue 2 |
| ISBN number | 0-9547136-1-3 |
|  | National Environmental Technology Centre AEA Technology plc, Culham Science Centre Abingdon, Oxfordshire, OX14 3ED Telephone 0870 190 6594 Facsimile 0870 190 6607 |
|  | AEA Technology is the trading name of AEA Technology plc AEA Technology is certificated to BS EN ISO 9001: 2000, The TickIT Guide Issue 5 and ISO 14001: 1996. |

|  | Name | Signature |  | Date |
| --- | --- | --- | --- | --- |
| Authors | S Baggott, R Milne, T Misslebrrook, TP Murrells, G Thistlethwaite J Watterson |  |  |  |
| Reviewed by | J Watterson / G Thistlethwaite |  |  |  |
| Approved by | J Goodwin |  |  |  |

Appendices

CONTENTS

| Appendix 1 | Estimation Methodology |
| --- | --- |
| Appendix 2 | Regional Greenhouse Gas Inventories |

Appendix 1 Estimation Methodology

CONTENTS

1.1	Introduction	7

1.1.1	Reporting Format	7

1.1.2	General Approach	7

1.2	Energy Industries	11

1.2.1	Electricity Production	11

1.2.2	Petroleum Refining	11

1.2.3	Manufacture of Solid Fuels	14

1.2.4	Other Energy Industries	14

1.3	Manufacturing Industries and Construction	15

1.3.1	Iron and Steel	15

1.3.2	Other Industry	18

1.4	Transport	20

1.4.1	Aviation and Navigation	20

1.4.2	Road Transportation	20

1.4.3	Railways	32

1.5	Other Sectors	33

1.5.1	Commercial & Institutional	33

1.5.2	Residential	36

1.5.3	Agriculture, Forestry & Fisheries	37

1.6	Military	37

1.7	Fugitive Emissions from Fuels	38

1.7.1	Coal Mining	38

1.7.2	Solid Fuels Transformation	38

1.7.3	Oil and Natural Gas	38

1.8	Industrial Processes	42

1.8.1	Minerals Industries	42

1.8.2	Chemical Production	42

1.8.3	Metal Production	47

1.8.4	Use of Halocarbons and SF6	47

1.9	Agriculture	49

1.10	Land Use Change and Forestry	49

1.10.1	Introduction	49

1.10.2.	Land use change in Northern Ireland	50

1.10.3	Emissions of CO2 from Soil due to Liming	50

1.10.4.	Other Sources and Sinks (Table 5E)	52

1.11	Waste	52

1.11.1	Solid Waste Disposal on Land	52

1.11.2	Waste Water Handling	54

1.11.3	Waste Incineration	54

1.12 Uncertainties	55

1.12.1	Introduction	55

1.12.2	Regional Uncertainty Estimation	56

References	58

## 1.1	Introduction

The UK Greenhouse Gas Inventory compiles estimates of greenhouse gas emissions for submission to the UN Framework Convention on Climate Change.  The most recent version of the inventory reports total UK greenhouse gas emissions from 1990 to 2001 (Baggott et al, 2003).  This report presents separate inventories of greenhouse gas emissions for England, Scotland, Wales and Northern Ireland for the years 1990, 1995, 1998, 1999, 2000 and 2001 that are consistent with the 2001 UK Inventory.

The six direct greenhouse gases are considered:

- carbon dioxide (CO2)

- methane (CH4)

- nitrous oxide (N2O)

- hydrofluorocarbons (HFCs)

- perfluorocarbons (PFCs)

- sulphur hexafluoride (SF6).

### 1.1.1	Reporting Format

Emissions are reported according to the Sectoral Tables in the IPCC Common Reporting Format with some modifications.  Where it is not possible to allocate emissions from certain sources to any one constituent country of the United Kingdom, such emissions are calculated and reported as “Unallocated”.  These sources include:

- Domestic Aviation

- Domestic Navigation

- Emissions from the offshore oil and gas industry

- Military Aviation

- Naval

Emissions from onshore oil and gas terminals are allocated to the country in which they occur.

The UK Inventory also reports emissions from marine and aviation bunkers separately, as required by the Intergovernmental Panel on Climate Change (IPCC).  These emissions have not been allocated within the regional inventories.

### 1.1.2	General Approach

The UK Inventory is based on UK statistics for activities producing greenhouse gas emissions.  These include fuel consumption, industrial production, agriculture and land use change and forestry.  In principle, it is necessary to obtain a complete set of equivalent statistics for each region to compile each inventory.

Such a set of statistics is not available for all sources and for all constituent countries and hence it is necessary to disaggregate UK emissions into the four constituent countries by an estimation procedure.  For most sources in the UK Inventory, the emission of a pollutant from a source is calculated from the general equation

E=Ae		[Equation 1]

where

E	=	Emission of pollutant (tonnes)

A	=	Activity (unit activity)

e	=	Emission Factor (tonnes pollutant/unit activity)

The activity unit may be fuel combustion (tonnes), or production of product (tonnes) or numbers of animals.

A modified equation is used for the regional inventories:

Ei=diAej=15dj	[Equation 2]

where

Ei	=	Emission (in tonnes) from either England (1), Scotland (2), 		Wales (3), Northern Ireland (4) or “Unallocated” (5)

di	=	A driver representing the contribution of the region to 			UK emissions

i	=	1, 2, 3, 4, 5

The driver, di can be any one of:

- The value of the activity data for the region.  [For example, consumption of specific fuels or industrial production figures for the region.]

- The fraction of the UK activity in the region.

- The value of a surrogate activity data statistic in the region.  Where the required activity is unavailable on a regional basis, a surrogate value may be used.  [For example, employment statistics or manufacturing output of a specific product, used as a surrogate for consumption data of a given fuel.]

- In cases where the emissions are derived from a complex model, the driver will be the actual emission for the region calculated from the model.

The modified equation [2] ensures that the sum of the emissions from England, Scotland, Wales and Northern Ireland, plus any “unallocated” (e.g. offshore) emissions, equals the total UK emission reported within the national inventory.

Where the driver is fuel consumption, then the sum of the drivers should add up to the UK consumption.  However, in practice this may not be the case if the data is taken from different sources or may be based on the financial rather than the calendar year.  The estimation procedure removes such discrepancies.

Thus the compilation of the regional inventories reduces to the estimation of a set of drivers, each appropriate to emissions from a specific source.  In compiling the 2001 regional inventories, 126 drivers have been calculated.

Subsequent sections discuss the estimation of the drivers for each source category.  Most of the detailed discussion is concentrated on the more complex categories, whilst simpler sources are summarised in Tables A1.1 to A1.10.  The IPCC classification is used throughout (IPCC, 1997a), and the following section provides a description of the abbreviations used throughout the Appendix 1 discussion.

- Summary of Abbreviations used in Tables A1.1 to A1.10

| BCA | British Cement Association |
| --- | --- |
| BGlass | British Glass |
| CA | Coal Authority |
| DTI | Department of Trade and Industry |
| DEFRA | Department for Environment, Food and Rural Affairs |
| DETR | Department of Environment, Transport & the Regions (now DEFRA) |
| DTLR | Department for Local Government, Transport and the Regions |
| E | England |
| EAF | Electric Arc Furnace |
| EM | Enviros March previously MCG |
| GB | Great Britain |
| IPCC | Intergovernmental Panel on Climate Change |
| ISSB | Iron and Steel Statistics Bureau |
| LPG | Liquefied petroleum gas |
| LRC | London Research Centre |
| MAFF | Ministry of Agriculture, Fisheries and Food (now DEFRA) |
| MCG | March Consulting Group (now EM) |
| MSW | Municipal Solid Waste |
| NA | Not Available |
| NAEI | National Atmospheric Emissions Inventory |
| NIO | Northern Ireland Office |
| NO | Not occurring |
| ONS | Office for National Statistics |
| OPG | Other petroleum gas |
| PI | Pollution Inventory |
| S | Scotland |
| SSF | Solid smokeless fuel |
| SPRU | Science Policy Research Unit |
| UKOOA | UK Offshore Operators Association |
| UKPIA | United Kingdom Petroleum Industry Association |
| WO | Welsh Office |
| WS | Welsh Statistics |

## 1.2	Energy Industries

The drivers used for the energy industries are summarised in Table A1.2.  This shows the base sources used in the National Atmospheric Emissions Inventory (NAEI) database, which correspond to the IPCC sources.  The activity data used in the UK Inventory are shown together with the drivers used in the regional inventory for 1990, 1995 and 1998 to 2001.  In some cases the derivation of the drivers differs between years depending on the availability of data.

### 1.2.1	Electricity Production

Emissions are based on fuel consumption data provided by the major power generators in Great Britain and the Northern Ireland Office.  These include Scottish Power (2002), Scottish and Southern Energy (2002), Bell (2002), Innogy (2002), PowerGen (2002), TXU (2002), AES (2002), Edison Mission (2002) and data reported in the Pollution Inventory (Environment Agency, 2002).  Amongst the changes made in the 2001 Inventory, there are 10 new gas-fired power stations accounted for, all located in England, and figures for fuel oil and gas oil consumption in Northern Ireland power stations have been reviewed for 1998 to 2001.

Emissions from plant generating electricity from municipal solid waste combustion are less certain for pre-1999, but all the plant are known to be in England for 1990 to 98 and so the emissions will correspond to the UK emissions.  Since 1999, two plants have been commissioned in Scotland, at Lerwick and Dundee, and emissions estimations are based on their capacities.

A small number of plants generate heat rather than electricity and these are categorised as 1A4a commercial and institutional.  Some generating plant burn poultry litter, or meat and bone meal, and these are all located in England.

The distribution of landfill gas and sewage gas generation was assumed to correspond to the distribution of landfill sites and sewage treatment plant (see Waste, Section 1.11).

### 1.2.2	Petroleum Refining

UKPIA have provided a site by site breakdown of UK refining emissions for 1997 and 1999 - 2001 (UKPIA, 1998; 2002), and have advised that refinery throughput did not vary significantly between 1990 and 1997.  The CO2 emissions data are used as a surrogate for all fuel consumption.  Emissions for 1998 are based on CO2 emissions reported in the Pollution Inventory (Environment Agency, 1999a).  Scottish emissions are based on CO2 emissions data from Grangemouth Refinery (BP, 2002).

- Table A1.2a: Energy Industries (Base Year – 1990)1

| IPCC Category | NAEI Sources | Activity: Fuel Consumption | 1990 |
| --- | --- | --- | --- |
| Electricity Production | Power Stations | Coal, oil, , natural gas | Consumption data from Power Generators |
|  |  | Unrefined natural gas | NO |
|  |  | Sewage gas | Sewage methane recovered |
|  |  | Landfill gas | As landfill methane |
|  |  | Orimulsion, MSW, poultry litter and tyres | All plant in England |
| Petroleum Refining | Refineries | All fuels | UKPIA CO2 emission estimates for pre-1997 |
| Manufacture of Solid Fuels | Coke Production | Colliery Methane | All such plant assumed to be in England. |
|  |  | Coke Oven gas, natural gas | Coal feed to coke ovens, ISSB, WS, DTI |
|  |  | Coke | Coke breeze consumption, ISSB |
|  |  | Blast Furnace gas | Coke consumed in blast furnaces, ISSB |
|  | SSF Production | All fuels | Coal feed to SSF plant, DTI, WS |
| Other Energy Industries | Collieries | All other fuels | Deep mined coal production, data from British Coal Authority |
|  |  | Coke oven gas | All such plant assumed to be in England. |
|  | Gas Production | Colliery methane | Deep mined coal production, data from British Coal Authority |
|  |  | Other fuels | Arrivals of natural gas, DTI |
|  | Offshore Own Gas Use / Gas Separation Plant | Unrefined natural gas, LPG, OPG | Extrapolated from 1995 on oil and gas arrivals, DTI |
|  | Nuclear Fuel Prod. | natural gas | All plant in England. |

1	See Section 1.1.3 for abbreviations

- Table A1.2b: Energy Industries (1995; 1998 to 2001)

| IPCC Category | NAEI Sources | Activity: Fuel Consumption | Data Sources / Comments |
| --- | --- | --- | --- |
| Electricity Production | Power Stations | Coal, oil, natural gas | Consumption data from Power Generators, plus Pollution Inventory data from 2000 onwards |
|  |  | Unrefined natural gas | Some power facilities, all located in Scotland, have used this fuel since 1995 |
|  |  | Sewage gas | Sewage methane recovered |
|  |  | Landfill gas | As landfill methane |
|  |  | Orimulsion, MSW, poultry litter | From 1999, some MSW plant now also in Scotland. |
| Petroleum Refining | Refineries | All fuels | UKPIA CO2 emission estimates for pre-1997. PI CO2 emission estimates for 1998. UKPIA data for 1999, 2000 & 2001. BP Grangemouth data for Scottish emissions data. |
| Manufacture of Solid Fuels | Coke Production | Colliery Methane | All such plant assumed to be in England. |
|  |  | Coke oven gas, natural gas | Coal feed to coke ovens, ISSB, WS, DTI and (since 1999) PI data. |
|  |  | Coke | Coke breeze consumption, ISSB. |
|  |  | Blast Furnace gas | Coke Consumed in Blast Furnaces, ISSB. |
|  | SSF Production | All fuels | Coal feed to SSF plant, DTI, WS. |
| Other Energy Industries | Collieries | All other fuels | Deep mined coal production, data from British Coal Authority |
|  |  | Coke oven gas | (1995 – current) No such plant operating. |
|  | Gas Production | Colliery methane | Deep mined coal production, data from British Coal Authority |
|  |  | Other fuels | Arrivals of natural gas, DTI |
|  | Offshore Own Gas Use / Gas Separation Plant | Unrefined natural gas, LPG, OPG | (1995 – current) UKOOA / SCOPEC CO2 estimates for terminals. |
|  | Nuclear Fuel Prod. | natural gas | (1995 – current) Data not available. |

### 1.2.3	Manufacture of Solid Fuels

This category comprises the production of coke and solid smokeless fuel (SSF).  Regional data on coke ovens in the iron and steel industry are reported in detail by ISSB (2002, 2001, 2000, 1999, 1996 & 1991).  Recent issues of UK Iron and Steel Statistics are less detailed than earlier ones, although the additional detail is available on request.  Two coke ovens in England and Wales are not attached to an integrated iron and steel facility, and the consumption of coal by these ovens is estimated from WO (1998) and UK data (DTI: 1991, 2000, 2001, 2002).  The Welsh statistics are only available to 1993, so this data is used as an estimate of the Welsh non-iron and steel coking coal consumption in 1995.  For 1998 to 2001, the non-iron and steel coking coal consumption data is apportioned between England and Wales using CO2 emissions for the particular sites reported in the Pollution Inventory (Environment Agency: 1999a, 2000, 2001 & 2002).

The generic driver for coke oven fuel consumption is the consumption of coking coal which is in effect the regional capacity of coke ovens.  This driver is also used for natural gas consumption and coke oven gas consumption.  Some coke ovens use blast furnace gas as fuel and the availability depends on blast furnace gas capacity (see Industrial Processes).  Small amounts of colliery methane are consumed in the manufacture of solid fuels and this was judged to occur entirely in England where coking occurs in close proximity to deep mining.  Small amounts of coke breeze are also used, and this has been disaggregated using data on other coke consumption in ISSB (2002, 2001, 2000, 1999, 1996, 1991).

The estimation of emissions from SSF production is rather uncertain, since there are a number of processes operating and the available fuel data are not detailed.  Moreover, many of these are the new briquetting processes rather than coking processes and produce negligible emissions.  For SSF plant operating in England and Wales, it is possible to estimate regional consumption using UK data (DTI: 1991, 2000, 2001 & 2002) and Welsh data (WO: 1998).  Welsh data for 1995 has been estimated, whilst all SSF coking plant still operating since 1998 are known to be in England.  Thus the driver used is coal consumed by SSF plant.

### 1.2.4	Other Energy Industries

This category consists of a number of small emissions from collieries, the gas industry, the nuclear fuel industry and a large emission from offshore natural gas use. In the regional inventory, the emissions from oil and gas terminals are based on data provided by SCOPEC (2002) and UKOOA (1999).  Data are only available for post-1995, and so emissions for 1990 have been extrapolated based on the arrivals of crude oil and natural gas in Scotland and England, (DTI: 1991, 1996).  The category of “gas separation plant” is assumed to be a subset of the gas used in oil and gas terminals and is treated in the same way as “offshore own gas use”, with emissions allocated based on the SCOPEC data on gas consumption in terminals. A driver is estimated for the category of “gas production” based on the arrivals of natural gas in England and Scotland (DTI: 1991, 1996, 2000, 2001 & 2002). Other sources are minor and are covered in Table A1.2.

## 1.3	Manufacturing Industries and Construction

The drivers calculated to disaggregate regional fuel consumption from these sectors are summarised in Table A1.3.

### 1.3.1	Iron and Steel

The ISSB (1991, 1996, 1999, 2000, 2001 & 2002) provides annual reports of the detailed regional consumption of fuel by the steel industry.  Recent issues of UK Iron and Steel Statistics are less detailed than earlier ones, though the additional detail is available on request.

The consumption of coke by sinter plant is estimated as the non-blast furnace coke consumption (as this is the main other use of coke).  The consumption of coke oven gas is distributed as proportional to regional figures for coal feed to coke ovens, whilst the consumption of blast furnace gas is distributed as proportional to regional figures for coke feed to blast furnaces.  (The production of these gases should be proportional to the fuel used as feedstock.)

In the latest revision of iron & steel sector drivers, alterations have been made to the derivation of drivers for coke use in blast furnaces, as previous calculations produced negative drivers for Wales for this activity in 1995 and 1998. Review of the calculation methodology has resulted in more consistent figures for this activity, and has altered regional drivers significantly for 1995 and 1998. This new method should reflect the time trend in blast furnace coke consumption more reliably than the previous method.

In addition to these changes to calculation methodology, 2001 also saw major changes in the industry, including the closure of the Llanwern steel manufacturing facility in Wales.  This had a huge impact on energy production & consumption.

The ISSB reports the general consumption of coal, fuel oil, gas oil, LPG and natural gas by the primary iron and steel industry.  This is a narrower definition than that used by DTI which includes foundries and finishing plant, and therefore the DTI data used in the UK GHGI is higher than the ISSB data.  Nevertheless, the regional ISSB data is used as a surrogate, since the distribution of the wider steel industry is directly linked to that of the primary industry.  Moreover, the emissions from the secondary plant are considerably lower than the primary plant.

- Table A1.3a: Manufacturing Industry and Construction (Base Year – 1990)1

| IPCC Category | NAEI Sources | Activity: Fuel Consumption | 1990 |
| --- | --- | --- | --- |
| Iron & Steel | Sinter Plant | Coke-breeze | Other coke consumption, ISSB |
|  | Iron & Steel | Blast furnace gas | Coke consumed in blast furnaces, ISSB, WO |
|  |  | Coke oven gas | Coal feed to coke ovens, ISSB, WS |
|  |  | Coke | Coke consumed in blast furnaces, ISSB, WO |
|  |  | Fuel oil, gas oil, LPG, natural gas, coal | Consumption of specified fuel, ISSB |
| Other Industry | Other Industry | Burning oil, fuel oil, gas oil | Regional oil consumption, DTI, WO (Remainder calculated after other uses) |
|  |  | OPG | All such plant are located in Scotland, DTI |
|  |  | LPG | Mass balance, DTI |
|  |  | Lubricants | Regional sales, DTI |
|  |  | Natural gas | Natural gas consumed, data from Transco |
|  |  | Colliery Methane | Deep mined coal production, British Coal Authority |
|  |  | Coal, coke | Coal consumption, WO, NIO |
|  |  | Coke oven gas | Coal feed to coke ovens, ISSB, WO, WS |
|  |  | SSF | NAEI spatial inventory |
|  |  | Wood | SPRU database: non-traded fuel |
|  | Cement | Coal, oil, gas, petrocoke, tyres, waste oil | Regional cement capacity, BCA |
|  | Ammonia (combustion) | Natural Gas | All such plant are located in England |
|  | Autogenerators | Coal | All such plant are located in England |
|  |  | Natural gas | ( Data sources exactly as per “Other Industry” above) |
|  | Other-Industry: Off-road | Gas oil, petrol | Employment in construction, ONS |

- See Section 1.1.3 for abbreviations

- Table A1.3b: Manufacturing Industry and Construction (1995; 1998 to 2001)

| IPCC Category | NAEI Sources | Activity: Fuel Consumption | Data Sources / Comments |
| --- | --- | --- | --- |
| Iron & Steel | Sinter Plant | Coke-breeze | Other coke consumption, ISSB |
|  | Iron & Steel | Blast furnace gas | Coke Consumed in blast furnaces, ISSB, WO |
|  |  | Coke oven gas | Coal feed to coke ovens, ISSB, WS |
|  |  | Coke | Coke consumed in blast Furnaces, ISSB, WO |
|  |  | Fuel oil, gas oil, LPG, natural gas, coal | Consumption of specified fuel, ISSB |
| Other Industry | Other Industry | Burning oil, fuel oil, gas oil | Regional oil consumption, DTI, WO (Remainder calculated after other uses) |
|  |  | OPG | All such plant are located in Scotland, DTI |
|  |  | LPG | Mass balance, DTI |
|  |  | Lubricants | Regional sales, DTI |
|  |  | Natural gas | Natural gas consumed, data from Transco & (since 1995) from Phoenix |
|  |  | Colliery Methane | Deep mined coal production, British Coal Authority |
|  |  | Coal, coke | Coal consumption, WO, NIO |
|  |  | Coke oven gas | Coal feed to coke ovens, ISSB, WO, WS |
|  |  | SSF | NAEI spatial inventory |
|  |  | Wood | SPRU database: non-traded fuel |
|  | Cement | Coal, oil, gas, petrocoke, tyres, waste oil | Regional cement capacity, BCA |
|  | Ammonia (combustion) | Natural Gas | All such plant are located in England |
|  | Autogenerators | Coal | All such plant are located in England |
|  |  | Natural gas | ( Data sources exactly as per “Other Industry” above) |
|  | Other-Industry: Off-road | Gas oil, petrol | Employment in construction, ONS |

### 1.3.2	Other Industry

DTI (2002, 2001, 2000, 1996 & 1991) reports regional consumption of liquid fuels, but only as totals for: England and Wales combined, Scotland and Northern Ireland. WO (1998) reports liquid fuel consumption up to 1993.  The total consumption for Wales has been extrapolated from 1993 to provide the England-Wales split.

Burning oil is mainly consumed in the residential sector, but there is also a significant use by industry.  Hence industrial consumption of burning oil was distributed according to the remaining consumption after domestic consumption had been deducted.

Fuel oil is consumed widely across industry, and the regional distribution of miscellaneous industry consumption is calculated by difference, when fuel oil use identified for specific uses is deducted from the DTI total.  The consumption of fuel oil within specific uses is initially estimated through the calculation of other drivers, as discussed in other sections of this report.

The procedure for estimating the fuel oil totals for England, Scotland, Wales and Northern Ireland was revised in the 1999 Inventory.  In the collation of the 2001 regional inventory, new data was identified for 1990, 1995 and 2000, leading to a revision of the fuel oil drivers for these years.

DTI (2002) reports total fuel oil sales for England & Wales (combined), Scotland and Northern Ireland.  However, these are not consistent with the totals used in the inventory because they include fishing and navigation but exclude refineries.  The industrial consumption in 1995 is distributed according to the Science Policy Research Unit (SPRU) database.  This is a database of around 6,000 small boilers (<50 MW) based on insurance records collected in the period 1992 to 94.  The database records a grid reference, economic sector, nominal fuel consumption and fuel type, and allows an estimate for total consumption for 1995 to be made on a basis consistent with the inventory (i.e. corrected for navigation, fishing and refineries).  These 1995 totals have been extrapolated to other years based on the DTI totals for fuel oil, illustrating the time trend in fuel oil consumption more reliably than the previous method.

The driver for industrial gas oil consumption is calculated in a similar manner to the fuel oil driver.  DTI reports gas oil consumption on a similar basis to fuel oil, and the SPRU driver is used to distribute industrial gas oil use in 1995.

Some gas oil and petrol is used for off-road machinery mainly in the construction industry.  The UK estimates themselves are uncertain, and regional drivers are therefore based on construction industry employment data (ONS: 1996, 2001, 2002).

Liquid Petroleum Gas (LPG) has a number of uses, which can be more precisely characterised in other sectors such as domestic use and the growing sector of LPG use in road transport applications. Hence, industrial use of LPG has been disaggregated based on a mass balance, using total regional sales (DTI: 2001, 2002) less consumption by all other sources.

DTI (2001, 2002) estimate that 40% of lubricants sold are eventually combusted.  Hence, the driver for emissions from lubricant use is based on regional lubricant sales (DTI: 1991, 1996, 2001, 2002) with England and Wales being disaggregated based on regional manufacturing employment statistics (ONS: 2001, 2002).

Transco (2001, 2002) supplies data on natural gas sales to consumers categorised by consumer size and region in Great Britain, excluding consumption by large industrial users and power generators. Consumption data for gas use in Northern Ireland is supplied by Phoenix Natural Gas Ltd (2001, 2002) for 1999-2001.

Regional gas consumption is estimated by matching the regional Transco and Phoenix totals with the NAEI UK total. Northern Ireland gas consumption is estimated directly from the Phoenix data.  Gas consumption by “other industry” and “autogenerators” in GB is disaggregated based on a mass balance, after all other defined sector uses have been deducted from the totals.  Determination of drivers for gas consumption by other defined sectors is discussed in other sections of this appendix.  The driver determined for “other industry” is also used for “autogenerators”.

Regional coal data is difficult to obtain, and hence estimates are very uncertain.  Sectoral data is available for Northern Ireland, NIO (2002) up to the present and Wales (WO, 1998) to 1993, but published data for coal use across industry in Scotland and England are not evident.

Industrial coal consumption for 1995 and 1998 to 2001 are estimated from sales data, gathered from all major coal producers.  Using a mass balance approach, considering production, imports, stock-changes and imports, it is possible to estimate the amount of British coal not included in the sales data.  This 'unknown' coal and anthracite is allocated to industrial consumption in proportion to the 'known' sales data.  English and Welsh sales data are quite comprehensive but since 1995, Scottish data has been somewhat incomplete.  Hence the allocation of consumption has been estimated for Scotland through extrapolation of the 1995 England-Scotland coal use figures.

To provide an estimated regional split for coal use in 1990, figures have been extrapolated back from data from the WO (1998) and NIO (1996) and the 1995 England-Scotland split.

Drivers for fuel consumption in cement kilns are based on regional clinker capacity data for 1995, 1998, 2000 and 2001 supplied by the British Cement Association (BCA: 2001, 2002).  These are applied to all fuels, with a correction factor applied to Northern Ireland to account for the absence of natural gas.  Where the UK estimate of fuel consumed in cement kilns has been revised for a given year, the regional consumptions have also been revised.

The calculation methodology for the contribution of fuel consumption emissions in this sector has been altered somewhat by the move to separate out the contribution of lime kiln fuel combustion. This has led to a revision of historic drivers for cement-sector fuel combustion drivers. (see Section 1.8.1 also)

“Autogeneration” refers to electricity generation by industry for its own use.  In the case of coal, the key autogenerators are Alcan and Brunner Mond, both of which are located in England. The DTI-sourced figures for coal autogeneration were reviewed in 2001, leading to minor updates for data from 1998-2000. Gas autogeneration is not considered a large source and has therefore been distributed according to the other natural gas “other industry” driver, as discussed above.

## 1.4	Transport

The drivers used for transport are summarised in Table A1.4.

### 1.4.1	Aviation and Navigation

All domestic aviation and navigation emissions are reported as unallocated. Emissions from aircraft support equipment are distributed according to aircraft movement data at the major UK airports (DLTR, 2002).

The UK fuel statistics for coastal shipping contain oil consumption by the offshore industry (which includes on-shore terminals) and it is these emissions that are reported as navigation in England and Scotland.  Terminal emissions data are supplied by SCOPEC (2002), for 1998 to 2001 and UKOOA (1999) for 1995.  Estimates for 1990 have been calculated by extrapolating data from 1995, based on landings of natural gas and crude oil in England and Scotland.

### Road Transportation

Carbon dioxide, methane and nitrous oxide are emitted from the exhaust of all road vehicles with internal combustion engines running on petrol and diesel fuels. CO2 is the principal product of combustion and emissions are directly related to the fuel efficiency of the vehicle.  Methane is a hydrocarbon emitted as a result of the incomplete combustion of the fuel.  Nitrous oxide is a by-product of the combustion process and emitted from partial oxidation of nitrogen present in the air.

All these pollutants are emitted by different amounts from vehicles of similar size running on petrol and diesel fuel.  For example, diesel cars tend to be more fuel efficient than petrol cars of a similar size, so their carbon emissions are lower.  None of these pollutants are subject to regulatory type-approval emission limits, as are those, which have an impact on air quality.  However, their emissions are affected by technologies introduced to reduce emissions of the regulatory pollutants.  Methane emissions are lower from petrol vehicles fitted with a three-way catalyst, although the reduction in emissions of this pollutant by the catalyst is not as efficient as it is for other hydrocarbons.  Measurements also suggest that a three-way catalyst, which is efficient at reducing NOx emissions, actually increases emissions of N2O, formed as a by-product of the catalyst NOx reduction process.

Total emissions from road transport in a country or region are normally calculated from the following information:

- Emission factors for different types of vehicles.  In the case of carbon emissions, fuel consumption factors can be used because the mass of carbon emitted is proportional to the mass of fuel consumed.  Emission factors (g/km) and fuel consumption factors depend on the vehicle type and fuel type (petrol or diesel) and are influenced by the drive cycle or average speeds on the different types of roads.

- Traffic activity, including distance and average speed travelled by each type of vehicle on each type of road.

- Fleet composition in terms of the age of the fleet and the petrol/diesel mix.  The age of the fleet determines the proportion of vehicles manufactured in conformity with different exhaust emission regulations (which have been successively tightened up over the past 30 years).  One of the defining factors for the inventories in the years between 1995-2001 is the proportion of petrol cars fitted with a three-way catalyst since this became mandatory for all new cars first registered in the UK from around August 1992, in accordance with EC Directive 91/441/EEC.  The proportion of cars and vans running on diesel fuel is also an important factor.  The sensitivity to the age of the fleet will be much less for the 1990 inventory because there were very few cars then fitted with catalysts and the difference in emissions from cars made to the earlier emission standards was much smaller.

- National or regional fuel consumption statistics.  This provides a particularly good indicator of carbon dioxide emissions.

- Table A1.4a: Transport (Base Year – 1990)1

| IPCC Category | NAEI Sources | Activity: Fuel Consumption | 1990 |
| --- | --- | --- | --- |
| Civil Aviation | NA | Aviation Gasoline, Jet Gasoline | Unallocated |
| Road Transportation | Road Transport | Petrol, Diesel oil | Road fuel sales, DTI; vehicle km, DETR / DLTR Traffic data: National Traffic Census, DfT Dept of Regional Development (NI: 1990) Fuel consumption: Digest of UK Energy Statistics (1990) |
| Railways | Railways | Gas oil | Gas oil consumption, Railtrack & NIR |
| Navigation | Coastal | Gas oil, Fuel oil | Unallocated except for terminals |
| Other | Aircraft Support | Gas oil | Regional aircraft movements |

1	See Section 1.1.3 for abbreviations

- Table A1.4b: Transport (1995; 1998 to 2001)1

| IPCC Category | NAEI Sources | Activity: Fuel Consumption | Data Sources / Comments |
| --- | --- | --- | --- |
| Civil Aviation | NA | Aviation Gasoline, Jet Gasoline | Unallocated |
| Road Transportation | Road Transport | Petrol, Diesel oil, LPG | Road fuel sales, DTI; vehicle km, DETR / DLTR Emission factors: COPERT III (1990-2000), TRL (2001) Composition of fleet: Vehicle Licensing Statistics Report, DfT (GB) Dept of Regional Development (NI). Traffic data: National Traffic Census, DfT (GB: 1990-2001) Dept of Regional Development (NI: 1990-1999), Traffic Census Report (NI: 2000), extrapolation using GB figs (NI, 2001) Fuel consumption: Digest of UK Energy Statistics (1990-2001), Welsh Office fuels data (WO, 1998) |
| Railways | Railways | Gas oil | Gas oil consumption, Railtrack, Translink & NIR |
| Navigation | Coastal | Gas oil, Fuel oil | Unallocated except for terminals |
| Other | Aircraft Support | Gas oil | Regional aircraft movements |

The emission factors and methodology followed for the regional inventory of emissions from road transport were those used for the UK National Atmospheric Emissions Inventory (NAEI).  These are largely taken from the European COPERT III program and database, recommended for CORINAIR and forming the basis of the IPCC Guidelines (European Environment Agency, 2000), and from the more recent compilation of exhaust emission factors provided by TRL based on recent tests carried out on vehicles in the UK fleet (Barlow et al, 2001).

#### 1.4.2.1	Emission factors

All the emission factors were those used in the latest UK Greenhouse Gas Emissions Inventory for 2001 (Baggott et al., 2003).  The current inventory uses slightly revised methane emission factors for some vehicle categories compared with those used in the last regional inventory compilation (Salway et al., 2002a).  Speed-related factors from COPERT III (European Environment Agency, 2000) were still used, but for vehicles meeting Euro standards not covered by COPERT III (e.g. more modern vehicles), these were re-estimated by scaling them in proportion to trends in the new set of speed-emission factors for total hydrocarbons (HC) between different vehicle classes.  The new HC factors come from the set of speed-emission factor equations derived by TRL from recent emission test programmes carried out in the UK on Euro I and II vehicles, combined with a re-evaluation of some HC factors for pre-Euro I vehicles to ensure internal consistency.  The factors for N2O are unchanged (COPERT III), with emissions data for this pollutant remaining sparse and only those for petrol cars and LGVs with catalysts (Euro I and on) able to reflect a dependence on vehicle speed.  The uncertainties in these factors for CH4 and N2O can be expected to be quite large.  However, the emission factors used reflect the fact that three-way catalysts are less efficient in removing methane from the exhausts than other hydrocarbons and also lead to higher N2O emissions than non-catalyst vehicles.

Fuel consumption factors for different vehicle types and speeds have also been revised since the last regional inventory compilation (Salway et al., 2002a).  These again were calculated from equations relating fuel consumption to average speed based on the new set of tailpipe CO2, CO and total hydrocarbon (HC) emission-speed equations developed by TRL (Barlow et al, 2001).  A substantial part of the new emission measurements for Euro I and II standard vehicles come from recent test programmes funded by DfT and DEFRA and carried out in UK test laboratories between 1999 and 2001.  The data are based on various programmes of measurements of exhaust emissions from in-service vehicles made over defined drive cycles, usually on rolling roads or engine test beds.

Tables A1.4.1 to A1.4.3 show the emission and fuel consumption factors used for the inventory broken down by vehicle type, road type and emission standard which the vehicle was compliant with when manufactured and first registered.  The approximate dates of introduction of the emission standards in the UK are given in the NAEI Report (Goodwin et al, 2001). Where the original source of the factors provided them as speed-emission factor equations, emission factors were calculated at average speeds typical of the road types shown in the tables.

#### 1.4.2.2	Age and composition of the fleet

Information on the age and composition of the vehicle fleet in the regions from 1990 to 2001 were taken from vehicle licensing statistics.  For England, Scotland and Wales, the data were taken from the Vehicle Licensing Statistics Report published for Great Britain each year by DfT (2002a); this is based on the DVLA files of vehicles licensed in Great Britain at the end of each year.

Additional information had been obtained directly from DfT which showed the post-town where the vehicles were registered and the year of first registration of vehicles currently licensed in 1995 (DoT, 1996).  By grouping together the post-town data into the regions, it was possible to estimate the average age of the fleet based on registrations in England, Scotland and Wales.  This tended to show that the age of the fleet was very similar in England and Wales, but somewhat newer in Scotland.  However, because vehicles are not necessarily used on the roads in the regions where they are registered (this would be particularly true for company cars and commercial vehicles), the licensing data by post-town was not used for the regional inventories and it was assumed that the age of the fleet and petrol/diesel mix for Great Britain as a whole applied equally to England, Scotland and Wales.

For Northern Ireland, the situation was slightly different.  Vehicle licensing statistics for private and light goods vehicles (PLG) were available from the Central Statistics and Research Branch of the Department of Regional Development in Northern Ireland.  These showed a newer fleet of cars than in Great Britain (DoRDNI, 2002a).  It is likely that most of the light duty vehicles on the road in Northern Ireland will be those licensed in Northern Ireland and vice versa.  This means that a newer licensed fleet should result in a higher proportion of cars fitted with three-way catalysts on the road in Northern Ireland during 1995-2001 than in England, Scotland and Wales.

Neither the vehicle licensing statistics nor traffic flow data for Northern Ireland were able to separate private cars from light goods vehicles (LGVs) or vans.  However, the proportion of diesel vehicles in the PLG class in Northern Ireland was 37% in 2001 (DoRDNI, 2002a) compared with 20% in Great Britain (DfT, 2002a).  This was interpreted as indicating that a higher proportion of PLG vehicles in Northern Ireland were LGVs than in Great Britain.  In the absence of any additional information, it was assumed that the proportions of diesels in the private car fleet and in the LGV fleet were the same in Northern Ireland as in Great Britain (13.8% for cars and 82.2%, for LGVs in 2001 (DfT, 2002a)).  This leads to the conclusion that 38.7% of the PLG fleet and, it is assumed, the PLG vehicle kilometres in Northern Ireland in 2001 were as LGVs.

#### 1.4.2.3	Traffic data

The preferred indicators for road transport activity in emission inventories are traffic data in terms of vehicle kilometres travelled per year disaggregated by vehicle and road type.  For the UK national inventory (the NAEI), vehicle kilometre data for the road network in Great Britain are provided by DfT for each vehicle type on roads classified as trunk, principal and minor roads in built-up areas (urban) and non-built-up areas (rural) and motorways (DfT, 2002b).  These estimates are based on traffic counts from the rotating census and core census surveys.

Vehicle kilometre data for 1990 - 2001 were available from DfT separated into the road networks in England, Scotland and Wales.  However, these did not provide a good representation of vehicle kilometres on the many minor roads in these regions.  In regions like Scotland, a significant proportion of vehicle kilometres is probably made on minor roads.  Salway et al (2000) discuss how use of the minor road vehicle kilometres provided by DfT can lead to anomalies in calculated fuel consumption trends for the regions.

The NAEI provides 1 x 1 km resolution maps of emissions for several pollutants and fuel consumption across the whole of the UK.  For road transport, different methods are used to map emissions from major and minor roads.  For major roads, emissions are calculated on individual road links from emission factors and traffic flow data (annual average daily flows) provided by DfT from the national traffic census (DfT, 2002b).  Total UK emissions and fuel consumption on minor roads are calculated from estimates of the total vehicle kilometres on minor roads and are then spatially disaggregated on the UK minor road network using Ordnance Survey maps and estimated average traffic flows on minor roads in different area types.

UK maps of fuel consumption on the major and minor road network were used to estimate road transport emissions of methane and nitrous oxide in England, Scotland and Wales in 1999 - 2001.  The way the calculations were made is described below.

Vehicle kilometre data in Northern Ireland for different road classes and vehicle categories were used to calculate methane and nitrous oxide emissions in Northern Ireland.  Vehicle kilometre data were available for years up to 1999 (DoRDNI, 2002b), but for 2000 and 2001, these data were not available.  Vehicle kilometre figures were estimated for 2000 from changes in the annual average daily traffic flows on road links in Northern Ireland in 2000 covered by the Annual Traffic Census Report 2000 (DoRDNI, 2002c).  For 2001, estimates were made based on the growth rates in traffic on similar road types from 1999 to 2001 in Great Britain and assumed to apply to Northern Ireland.

#### 1.4.2.4	Fuel consumption in the regions

Fuel delivery data are available for England & Wales, Scotland and Northern Ireland from DUKES (DTI, 2002).  Separate fuel delivery data for Wales was available from the Welsh Office for 1990, but not for 1995-2001 (WO, 1998).  Therefore, the split in petrol and diesel consumed between England and Wales in 1990 was assumed to apply to the DUKES England & Wales total for 1995-2001.

In 1990, model predictions of fuel consumption in England & Wales based on traffic data were found to be very similar to actual figures provided by the Welsh Office and DUKES, based on fuel deliveries in the regions. Model predictions in 1995 indicated that the England / Wales split for 1995 would be very similar to 1990. These factors indicate that extrapolation of the England / Wales split, based on 1998 Welsh Office data, provide a reasonable estimation methodology.

#### 1.4.2.5	Method for calculating emissions of CO2, CH4 and N2O from road transport in the regions

Different methods were used to derive the regional inventories of CO2 and for CH4 and N2O.

Regional inventories of CO2 emissions from road transport were calculated directly from the petrol and diesel fuel delivery figures for the regions in accordance with IPCC guidelines on reporting Greenhouse emissions.  The calculated CO2 emissions from the four regions were normalised so that the sum added up to the reported UK total CO2 emissions from road transport.  This small normalisation correction accounts for the very small proportion of these fuels consumed by off-road vehicles and machinery.

The inventory pictures a large and continuous decline in CO2 emissions from road transport in Northern Ireland over the period between 1990 and 2000, a feature not observed in the other regions, although this seems to have levelled out as a slight growth in emissions occurred from 2000 to 2001.  The decrease reflects a large decline in petrol and diesel fuel deliveries in Northern Ireland particularly since 1995, presumably due to cross-border purchasing of fuels by motorists at filling stations in the Irish Republic where the price of fuels are lower and the price differential has become increasingly larger.

Emissions of CH4 and N2O in England, Scotland and Wales were calculated from a combination of emission factors, traffic activity and fuel consumption data.  First, fleet-average, fuel-based emission factors were calculated for the region in each year, for each main vehicle type.  To do this, CH4 and N2O emissions were calculated for each vehicle type in each region from the emission factors and uncertain traffic data figures for each region (vehicle kilometres per year) and divided by the fuel consumed calculated in the same way from fuel consumption factors and vehicle kilometre data.  The results are factors in units g pollutant/kg fuel for each vehicle type reflecting the composition of the fleet that year, for example, the proportion of cars with three-way catalysts, and relative proportion of miles done on different types of roads (e.g. urban, rural, motorway) in the region.

The fuel-based emission factors for each vehicle type were then combined with model estimates of the total amount of petrol or diesel they each consumed in the region.  For England, Scotland and Wales, the estimates for 1999 - 2001 come from the NAEI maps of fuel consumption by vehicles on major and minor roads in the UK referred to earlier.  This procedure ensures adequate coverage of vehicle activity on the minor roads, which the vehicle kilometre data from DfT do not cover well.  At present, fuel consumption maps covering each vehicle type are not available for 1990, 1995 and 1998.  For these years, the regional fuel consumption estimates were scaled relative to 1999 according to trends in the DUKES total fuel delivery figures for the regions each year relative to 1999 and then divided between each vehicle category according to their fuel consumption factors and approximate annual vehicle kilometre figures in the region.  This procedure in effect uses DTI fuel delivery data as an indicator of traffic activity in England, Scotland and Wales in earlier years relative to 1999, but will be reasonably robust if there is a fairly equitable movement of fuel bought in neighbouring regions across borders.

The procedure for calculating emissions of CH4 and N2O between 1999 and 2001 (based on NAEI fuel consumption maps) works equally well for Northern Ireland.  However, the procedure used for England, Scotland and Wales for the earlier years (1990 – 1998, based on fuel deliveries) cannot be used to estimate their emissions in Northern Ireland because of the extensive amount of cross-border purchasing of fuel in the Irish Republic, but consumed by traffic in Northern Ireland.  The fuel delivery statistics from DTI (2002) suggest a 39% reduction in petrol sales and a 51% reduction in diesel sales in Northern Ireland from 1995 to 2001 in spite of the fact that traffic had grown by almost 20% over this period.  The CH4 and N2O inventory needs to reflect the growth in traffic.  Therefore, emissions were calculated from the vehicle kilometre and traffic survey data provided by the Department of Regional Development in Northern Ireland (DoRDNI ,2002b, 2002c) and the vehicle kilometre-based emission factors.  The calculations took account of the newer car fleet in Northern Ireland, compared with Great Britain, leading to a higher proportion of cars with three-way catalysts.

Table A1.4.1	Fuel Consumption Factors for Road Transport (in g fuel/km)

Table A1.4.2	Methane Emission Factors for Road Transport (in g/km)

Table A1.4.3	N2O Emission Factors for Road Transport (in g/km)

### 1.4.3	Railways

Emissions from railway locomotives in Great Britain are disaggregated based on diesel oil consumption data supplied by Network Rail (2003). The data is reported for each railway company, whose area of operation can in most cases be allocated to one of the four regions. (Some railway company operations are spread across the regions and in these cases, fuel consumption has been allocated based on information directly from railway operators.)  Emissions from railways in Northern Ireland are based on fuel consumption data supplied by Translink (2003).

## 1.5	Other Sectors

### 1.5.1	Commercial & Institutional

The NAEI categories “public administration” and “miscellaneous” are considered together for the regional inventories, as regional statistics are not available to this level of sectoral detail.

Regional miscellaneous coal use has been reported previously for Wales (WO, 1998), whilst some industrial sector coal uses have been reported for NI (NIO, 2002). Estimates for 1990, 1995, and 1998 to 2001 are all based on the SPRU database for miscellaneous coal consumption.

Regional gas sales for the commercial sector have previously been reported by the DTI (1992), but for later years (1995 to date) the key source has been Transco data for regional gas consumption in the 73-732 MWh range. The Transco source provides the closest data available for commercial and institutional consumers, but the total is lower than UK data reported by DTI (2001, 2002). This data is used as a driver to distribute the NAEI miscellaneous and public service gas consumption in GB.  Natural gas consumption data for Northern Ireland are supplied by Phoenix Natural Gas for 1999-2001.  The commercial consumption is used as an estimate for Northern Ireland miscellaneous and public service gas consumption.

Fuel oil and gas oil use in these sectors is distributed using regional employment statistics in non-industrial sectors from the ONS employment database (ONS: 2002), as a suitable source of surrogate activity data.

Stationary combustion by the railway sector is classified as a commercial source. Consumption of burning oil, fuel oil, and coke is relatively insignificant, and has therefore been allocated according to the diesel oil driver used for locomotives. Natural gas consumption for electricity generation refers to the London Underground (Lotts Road power station – operational during 2001, but due to close).

DTI (2002) reports a small amount of solid waste (municipal, industrial & hospital) consumption for energy production in the commercial and miscellaneous sectors.  Little is known about the distribution of these installations, but the emissions have been distributed using the split derived for MSW incinerators.

- Table A1.5a: Other Sectors (Base Year – 1990)1

| IPCC Category | NAEI Sources | Activity: Fuel Consumption | 1990 |
| --- | --- | --- | --- |
| Commercial & Institutional | Miscellaneous, Public service | Coal | SPRU database: Miscellaneous; coal |
|  |  | SSF | NAEI spatial inventory |
|  |  | Natural gas | Commercial Sales, DTI. |
|  |  | Landfill gas | Landfill methane emissions |
|  |  | Sewage gas | Sewage methane recovered |
|  |  | fuel oil, gas oil | Non-Industrial Employment, ONS |
|  |  | MSW | As MSW incinerators |
|  |  | Burning oil | As other industry |
|  | Railways(Stationary) | fuel oil, burning oil, coal | Regional gas oil consumption |
|  |  | Natural gas | Assumed as all England |
| Residential | Domestic | Coal, anthracite, wood | Coal consumption / sales data, NIO & WO |
|  |  | SSF, coke | See text |
|  |  | Natural gas | Domestic Gas, DTI |
|  |  | Burning oil | Regional burning oil, DTI, WO. LRC data for NI. |
|  |  | Gas oil, LPG | Regional population, ONS, LRC data for NI |
|  |  | Fuel oil | Regional population, ONS |
|  | House & Garden | DERV, petrol | Regional population, ONS |
| Agriculture, Forestry & Fishing | Agriculture | coal, coke, fuel oil, gas oil, natural gas | Agricultural employment, MAFF |
|  |  | burning oil | Regional burning oil, DTI, WO |
|  |  | straw | Wheat production, MAFF |
|  | Agriculture Power Units | Gas oil, petrol | Agricultural employment, MAFF |
|  | Fishing | gas oil, fuel oil | Unallocated |

1	 See Section 1.1.3 for abbreviations

- Table A1.5b: Other Sectors (1995; 1998 to 2001)

| IPCC Category | NAEI Sources | Activity: Fuel Consumption | Data Sources / Comments |
| --- | --- | --- | --- |
| Commercial & Institutional | Miscellaneous, Public service | Coal | SPRU database: Miscellaneous; coal |
|  |  | SSF | NAEI spatial inventory |
|  |  | Natural gas | Natural gas consumed, Transco & Phoenix |
|  |  | Landfill gas | Landfill methane emissions |
|  |  | Sewage gas | Sewage methane recovered |
|  |  | fuel oil, gas oil | Non-Industrial Employment, ONS |
|  |  | MSW | As MSW incinerators |
|  |  | Burning oil | As other industry |
|  | Railways(Stationary) | fuel oil, burning oil, coal | Regional gas oil consumption, data from Network Rail (GB) and Translink (NI) |
|  |  | Natural gas | Assumed as all England |
| Residential | Domestic | Coal, anthracite, wood | Coal consumption / sales data, NIO & WO |
|  |  | SSF, coke | See text |
|  |  | Natural gas | Customers < 73200 kWh, DTI & Transco for GB, Phoenix for NI |
|  |  | Burning oil | Regional burning oil, DTI, WO. LRC data for NI. |
|  |  | Gas oil, LPG | Regional population, ONS, LRC data for NI |
|  |  | Fuel oil | Regional population, ONS |
|  | House & Garden | DERV, petrol | Regional population, ONS |
| Agriculture, Forestry & Fishing | Agriculture | coal, coke, fuel oil, gas oil, natural gas | Agricultural employment, MAFF / DEFRA |
|  |  | burning oil | Regional burning oil, DTI, WO |
|  |  | straw | Wheat production, MAFF / DEFRA |
|  | Agriculture Power Units | Gas oil, petrol | Agricultural employment, MAFF / DEFRA |
|  | Fishing | gas oil, fuel oil | Unallocated |

### 1.5.2	Residential

Domestic coal consumption is reported in NIO (2002) up to the present and in WO (1998) to 1993.  These data also include anthracite and (for NI) other solid smokeless fuels.  Published data for Scotland and England have not been identified, and hence estimates are uncertain. The methodology for estimating regional coal consumption has been extensively revised in order to simplify it and to obtain a more consistent time series.

Domestic coal consumption for 1995 and 1998 – 2001 is estimated from sales data, gathered from all major coal producers.  A mass balance (considering production, imports, stock-changes and imports) enables estimation of the amount of British coal that is not included in the sales data. This “unknown” coal and anthracite is allocated to domestic consumption in GB in proportion to the “known” sales data.

Industry figures suggest that only small amounts of British Coal are shipped to NI. Any consumption in NI over and above the known amount of British coal shipments, are accounted as foreign imports of anthracite and steam coal. The remaining amount of foreign anthracite imports are then allocated to England, Scotland and Wales in proportion to the “known” consumption. Any remaining amount of foreign coal imports are then assumed to be imported steam coal and allocated to England.

Estimates for 1990 are based on WO (1998) and NIO (1996).  The remaining consumption was allocated to England and Scotland on the basis of the 1995 England / Scotland split.

The regional data available is not sufficiently detailed to report consumption of manufactured solid smokeless fuels (SSF). Hence a driver has been estimated based the NAEI disaggregated inventory database. This uses the distribution of smoke control areas and assumes a distribution for areas where there is no gas consumption (i.e. population density < cut off value) and allocates the UK SSF consumption to these areas.  The NI data includes SSF imports under coal and anthracite and so a correction is applied to avoid double counting.  The same driver is used for commercial and other industry use of SSF.

Domestic natural gas consumption data is available for 1990, 1995 and 1998-2001 (DTI: 1991, 1996, 1999-2002), with additional information from Transco (2002) and Phoenix Gas for NI (2002).

Regional consumption of liquid fuels are reported by DTI (1991, 1999-2002) as totals for: England and Wales combined; Scotland and Northern Ireland. WO (1998) reported similar data on liquid fuel consumption up to 1993.

A large proportion of burning oil is consumed in the domestic sector and in the original inventory the regional totals were used as the domestic driver.  However, this resulted in a very high estimate for domestic consumption in Northern Ireland in 1998.  This could not be reconciled to the house conditions survey of the same year.  An improved procedure has been adopted whereby Northern Ireland's domestic consumption is estimated from the house conditions survey, with the remaining burning oil consumption allocated to England, Wales and Scotland according to the DTI totals. Welsh consumption is extrapolated from 1993 data.

The consumption of fuel oil by the domestic sector is a very small amount, and is distributed simply according to population (ONS: 2002).

For domestic consumption of gas oil, once again the consumption pattern across Northern Ireland must be accounted for. Data from the House Conditions Survey used in the Greater Belfast Local Inventory (LRC, 1999) suggests that most domestic oil consumption is burning oil. Hence as a simplifying assumption, domestic gas-oil consumption in the region is taken as zero. Domestic gas oil consumption for England, Scotland and Wales is then allocated by population (ONS: 2002).

The domestic consumption of liquid petroleum gas (LPG) in Northern Ireland is proportionately higher than in Great Britain due to the historical absence of mains gas supplies. The consumption for Northern Ireland is estimated using data on the consumption per household from the House Conditions Survey used in the Greater Belfast Local Inventory (LRC, 1999). Consumption for England, Scotland and Wales is once again allocated on the basis of population (ONS: 2002).

### Agriculture, Forestry & Fisheries

Regional fuel consumption by agriculture is not available.  Emissions are allocated on the basis of employment figures from DEFRA (2001, 2002). Fishing emissions are unallocated.

## 1.6	Military

Emissions from military aircraft and naval vessels are not allocated within the regional inventories.  Army vehicle emissions are included within road transport data and other army emissions are included within public service categories but aren’t clearly defined.

## 1.7	Fugitive Emissions from Fuels

### 1.7.1	Coal Mining

- Methane emissions arise from coal mining activities.  Emissions are estimated based on the amounts of deep mined and open cast coal produced.  Hence regional estimates are based on regional coal production derived from a number of sources: Coal Authority (2001, 2002), BGS (1991,1996, 2002), WO (1998), SO (1999), DTI (1996).  A small emission occurs from coal storage and transport, which is based on deep mined coal production.  Data suggests that only small amounts of coal are transported outside of the region of production and no attempt has been made to allow for this.  Hence coal storage and transport emissions are distributed according to deep-mined production.  These estimates only include emissions from working mines and exclude closed mines.  It was previously believed that emissions became negligible once a coal mine closed, particularly if the mine flooded (Williams, 1993).  However, a review sponsored by DETR, suggests that these emissions may be significant (Sage, 2001).  A number of estimates have been made ranging from 20 to 300 ktonnes methane in 1999.  DEFRA is planning more research to establish a reliable estimate of emissions from this source.

### 1.7.2	Solid Fuels Transformation

For coke ovens, three fugitive emissions are estimated:

- A ‘residual’ emission of CO2 which reflects the difference between the carbon input to the coke oven and the carbon content of the coke and coke-oven gas produced.

- Emissions from the flaring of coke-oven gas.

- Emissions of methane from the process.

These are disaggregated based on the regional consumption of coking coal discussed in Section 1.2.3.

For solid smokeless fuel (SSF) plant, the only fugitive emissions estimated are the ‘residual’ CO2 emission and some process methane.  The driver used is that for  regional consumption of coal by SSF plant (see 1.2.3).  It is known that some petroleum coke is used in SSF production but the amount is uncertain.  The same driver is applied to the petroleum coke consumption.

### 1.7.3	Oil and Natural Gas

All emissions from the offshore industry have been classified as unallocated.  However some emissions occur from on-shore oil and gas terminals in England, Wales and Scotland and from the on-shore oil and gas fields.

The estimates of terminal flaring and venting emissions are based on SCOPEC (2002) data for 1998-2001 and UKOOA (1999) for 1995.  No new data is available for a majority of onshore flaring sites in 2001 and so calculations were based mainly on 2000 figures.  Data is unavailable for 1990, so these are extrapolated based on flaring volumes for Scottish Terminals and natural gas arrivals to gas terminals in England (DTI: 1991, 1996).

The 2000-2001 UK inventories include a correction to account for flaring on onshore oil and gas fields excluded by the SCOPEC Inventory.  This is based on flaring volume data from DTI sources (DTI, 2002; 2001; 2001a; 2001b).  In the UK national inventory their contribution is negligible, but the data is more significant for the regional inventories.  Wytch Farm, which lies a few miles off the south coast of England, was classified as on-shore for this purpose.

The SCOPEC database (2002) provides data for fugitive emissions of CO2 and methane from terminals for 1998-2001. Methane emissions arise from venting, oil storage and tanker loading and unloading, whilst CO2 emissions arise from venting and processes.  A more aggregated set of data for 1995 has been provided by UKOOA (1999), whilst estimates for 1990 have been calculated by extrapolation of data of oil and gas arrivals in England and Scotland (DTI: 1991, 1996).

The UK national inventory estimates emissions of methane from leakage from the natural gas transmission system. Estimates are based on Transco data of specific leakage rates from the mains & services, and data on the stock of mains & services. The baseline estimate is for 1991, with figures for subsequent years based on the upgrading of the system.

Transco have developed a new model which produces regional leakage estimates from the low-pressure transmission system for 1998 to 2001.  The 1998 regional split was applied to the estimates of the ‘old’ model to obtain estimates for 1990 and 1995.  Whilst leakage from low-pressure mains and services accounts for most of the emission, there is also a contribution from high-pressure mains, storage and other losses.  These additional emissions are allocated using the same regional split as the low-pressure leakage.

- Table A1.7a: Fugitive Emissions from Fuels (Base Year – 1990)1

| IPCC Category | NAEI Sources | Activity: Fuel Consumption | 1990 |
| --- | --- | --- | --- |
| Coal Mining | Deep mined coal Coal storage & transport | Deep mine coal production | Regional deep mine production, British Coal Authority |
|  | Open cast coal | Open cast mine coal production. | Regional open cast mine production, British Coal Authority |
| Solid Fuel transformation | Coke production | Coke production | Coal feed to coke ovens, ISSB, WS, DTI |
|  | Flaring | Coke oven gas | Coal feed to coke ovens, ISSB, WS, DTI |
|  | SSF production | Coal, Petrocoke | Coal feed to SSF plant, DTI, WS |
| Oil | Offshore Oil & Gas | NA | Fugitive emissions from Terminals (extrapolated from 1995), |
|  | Oil Terminal Storage | NA | Have used 1998 driver |
|  | Onshore Loading | Oil loaded | Have used 1998 driver |
| Venting & Flaring | Offshore Flaring | Volume gas flared | Flaring at terminals and onshore fields, UKOOA, DTI |
|  | Offshore Venting | NA | Fugitive emissions from Terminals (extrapolated from 1995) |
| Natural Gas | Gas Leakage | Natural gas leakage | Transco estimate for LP mains |

1	 See Section 1.1.3 for abbreviations

- Table A1.7b: Fugitive Emissions from Fuels (1995; 1998 to 2001)

| IPCC Category | NAEI Sources | Activity: Fuel Consumption | Data Sources / Comments |
| --- | --- | --- | --- |
| Coal Mining | Deep mined coal Coal storage & transport | Deep mine coal production | Regional deep mine production, British Coal Authority |
|  | Open cast coal | Open cast mine coal production. | Regional open cast mine production, British Coal Authority |
| Solid Fuel transformation | Coke production | Coke production | Coal feed to coke ovens, ISSB, WS, DTI and (1999-current) PI |
|  | Flaring | Coke oven gas | Coal feed to coke ovens, ISSB, WS, DTI and (1999-current) PI |
|  | SSF production | Coal, Petrocoke | Coal feed to SSF plant, DTI, WS |
| Oil | Offshore Oil & Gas | NA | UKOOA Process emissions from Terminals, SCOPEC |
|  | Oil Terminal Storage | NA | Data from storage emissions, SCOPEC database. |
|  | Onshore Loading | Oil loaded | Data from loading emissions, SCOPEC database. |
| Venting & Flaring | Offshore Flaring | Volume gas flared | Flaring at terminals and onshore fields, UKOOA, SCOPEC, DTI |
|  | Offshore Venting | NA | Data from venting emissions, SCOPEC database. |
| Natural Gas | Gas Leakage | Natural gas leakage | Transco estimate for LP mains |

## 1.8	Industrial Processes

These sources report process and fugitive emissions from industrial processes as opposed to the emissions from fuel combustion used to provide energy to these processes.  (Table A1.2 covers combustion emissions.)  The drivers used for process and fugitive industrial releases are summarised in Table A1.8.

### 1.8.1	Minerals Industries

Large emissions of CO2 arise from the degradation of limestone used in cement and lime kilns.  Cement emissions are estimated from the production of cement clinker, with regional emission estimations based on percentage capacity data for the regions supplied by the British Cement Association (2002).  Through discussions with regulatory authorities (Environment Agency, Scottish Environmental Protection Agency, and in Northern Ireland, the Department of the Environment) it has been determined that lime calcination only occurs in England.

Limestone and dolomite are also used in iron and steel production.  Discussions with British Steel suggest that it would be impossible to identify all the different uses of limestone and dolomite in iron and steel making.  The major use is in blast furnaces, and so emissions have been disaggregated based upon regional iron production figures (ISSB, 1991; 1996; 1999 - 2002).

Limestone, dolomite and soda ash are also used in glass production.  Emissions were previously disaggregated using plant capacity and CO2 emissions data from British Glass, for 1990, 1995, 1998 and subsequently extrapolated for 1999 and 2000. However, the improvement of data supplied via the Pollution Inventory (Environment Agency, 2002) has enabled more accurate disaggregation for the years 2000 and 2001. Hence in the current revision of the regional inventories, previous 2000 data has been revised, and a new methodology used to determine current data.

The 2001 inventory also reports CO2 and methane emissions from Fletton brick production, as introduced in 2000.  These bricks are made from Fletton clay which contains a significant amount of naturally occurring carbonaceous material. Disaggregation is simple as all such production occurs in England.

### 1.8.2	Chemical Production

The UK Inventory reports emissions of carbon dioxide from ammonia production; nitrous oxide from adipic acid production and nitrous oxide from nitric acid production.  The ammonia and adipic acid plants are in England.  Nitric acid is produced in England and Northern Ireland and estimates are based on the plant capacities and estimated emission factors.

- Table A1.8a: Industrial Processes (Base Year – 1990)1

| IPCC Category | NAEI Sources | Activity Data | 1990 |
| --- | --- | --- | --- |
| Cement Production | Cement (decarbonizing) | Clinker production | Regional cement capacity, BCA |
| Lime Production | Lime (decarbonizing) | Limestone consumption | All such plant located in England |
| Limestone and Dolomite Use | Glass production | Limestone and dolomite consumption | Regional glass production, BGlass |
|  | Blast Furnaces | Limestone and dolomite consumption | Iron production, ISSB |
| Soda Ash Production and Use | Glass production | Soda Ash Consumption | Regional glass production, BGlass |
| Mineral Products: Other | Fletton Brick Production | Fletton Brick Production | All such plant located in England |
| Ammonia Production | Ammonia feedstock | Natural gas feedstock | All such plant located in England |
| Nitric Acid Production | Nitric Acid Production | Plant capacity | Regional plant capacity |
| Adipic Acid Production | Adipic Acid Production | Adipic acid made | All such plant located in England |
| Chemical Industry: Other | Methanol Production | Production of Methanol | All such plant located in England |
| Chemical Industry: Other | Ethylene Production | Production of Ethylene | Plant capacities |
| Chemical Industry: Other | Chemical Industry | NA | Extrapolated from PI data |
| Iron and Steel | Electric Arc Furnace | EAF steel production | Regional EAF production, ISSB |
|  | Flaring | Blast furnace gas | Coke consumed in blast furnaces, ISSB, WO |
| Aluminium Production | Aluminium production | Primary aluminium produced | Regional aluminium plant capacity, ALCAN |
| SF6 Used in Aluminium and Magnesium Foundries | SF6 Cover gas | NA | Regional consumption, EM |

- See Section 1.1.3 for abbreviations

- Table A1.8a: Industrial Processes (Base Year – 1990)  (continued)

| IPCC Category | NAEI Sources | Activity Data | 1990 |
| --- | --- | --- | --- |
| Halocarbon & SF6 By-Product Emissions | Halocarbon Production | NA | All such plant are located in England. |
| Refrigeration and Air Conditioning | Refrigeration | NA | Regional population, ONS |
|  | Supermarket Refrigeration | NA | Regional population, EM |
|  | Mobile Air conditioning | NA | Regional population, EM |
| Foam Blowing | Foams | NA | Regional population, ONS |
| Fire Extinguishers | Fire fighting | NA | Regional population, ONS |
| Aerosols | Metered Dose Inhalers | NA | Regional population, ONS |
|  | Aerosols (halocarbons) | NA | Regional population, ONS |
| Other | Electronics | NA | Regional electronics plant consumption, EM |
|  | Training shoes | NA | Regional population, EM |
|  | Electrical Insulation | NA | Regional electrical capacity, EM |

- Table A1.8b: Industrial Processes (1995; 1998 to 2001)

| IPCC Category | NAEI Sources | Activity Data | Data Sources / Comments |
| --- | --- | --- | --- |
| Cement Production | Cement (decarbonizing) | Clinker production | Regional cement capacity, BCA |
| Lime Production | Lime (decarbonizing) | Limestone consumption | All such plant located in England |
| Limestone and Dolomite Use | Glass production | Limestone and dolomite consumption | Regional glass production, BGlass |
|  | Blast Furnaces | Limestone and dolomite consumption | Iron production, ISSB |
| Soda Ash Production and Use | Glass production | Soda Ash Consumption | Regional glass production, BGlass |
| Mineral Products: Other | Fletton Brick Production | Fletton Brick Production | All such plant located in England |
| Ammonia Production | Ammonia feedstock | Natural gas feedstock | All such plant located in England |
| Nitric Acid Production | Nitric Acid Production | Plant capacity | Regional plant capacity, PI |
| Adipic Acid Production | Adipic Acid Production | Adipic acid made | All such plant located in England |
| Chemical Industry: Other | Methanol Production | Production of Methanol | All such plant located in England |
| Chemical Industry: Other | Ethylene Production | Production of Ethylene | Plant Capacities, PI |
| Chemical Industry: Other | Chemical Industry | NA | PI data, or extrapolated from PI data |
| Iron and Steel | Electric Arc Furnace | EAF steel production | Regional EAF production, ISSB |
|  | Flaring | Blast furnace gas | Coke Consumed in blast furnaces, ISSB, WO |
| Aluminium Production | Aluminium production | Primary aluminium produced | Regional aluminium plant capacity, ALCAN |
| SF6 Used in Aluminium and Magnesium Foundries | SF6 Cover gas | NA | Regional consumption, EM |

- Table A1.8b: Industrial Processes (1995; 1998 to 2001) (continued)

| IPCC Category | NAEI Sources | Activity Data | Data Sources / Comments |
| --- | --- | --- | --- |
| Halocarbon & SF6 By-Product Emissions | Halocarbon Production | NA | All such plant are located in England. |
| Refrigeration and Air Conditioning | Refrigeration | NA | Regional population, ONS |
|  | Supermarket Refrigeration | NA | Regional population, EM |
|  | Mobile Air conditioning | NA | Regional population, EM |
| Foam Blowing | Foams | NA | Regional population, ONS |
| Fire Extinguishers | Fire fighting | NA | Regional population, ONS |
| Aerosols | Metered Dose Inhalers | NA | Regional population, ONS |
|  | Aerosols (halocarbons) | NA | Regional population, ONS |
| Other | Electronics | NA | Regional electronics plant consumption, EM |
|  | Training shoes | NA | Regional population, EM |
|  | Electrical Insulation | NA | Regional electrical capacity, EM |

The UK inventory now reports emissions of methane from methanol production, ethylene production and the other chemical industry. The methanol plant is located in England, whilst ethylene production occurs in England, Scotland and Wales. These emissions are distributed based on data reported in the Pollution Inventory (Environment Agency, 2002) and plant capacity.  Emissions are extrapolated to 1990 and 1995 based on plant capacities.

The emissions from the “other chemical industry” sector are disaggregated to England and Wales based on the site data in the Pollution Inventory.  Data on emissions from other chemical processes are not available for Scotland.

### 1.8.3	Metal Production

In the iron and steel industry, emissions of CO2 arise from electric arc furnaces through the consumption of the graphite anodes.  Regional data on steel production from electric arc furnaces is used to determine the regional drivers for this activity (ISSB, 1991; 1996; 1999 - 2002).

The flaring of waste blast furnace gas is disaggregated according to the distribution of blast furnaces, using the driver derived for coal consumption by blast furnaces (ISSB, 1991; 1996; 1999 - 2002).

Emissions of CO2 from iron and steel making are estimated from a mass balance on the coke consumed in blast furnaces; the blast furnace gas produced; the pig iron produced; the pig iron used in steel making and the crude steel produced.  The emissions are distributed using appropriate drivers for each source and sink taken from ISSB (1991; 1996; 1999 - 2002).  These include regional data on coke consumed in blast furnaces, pig iron production and crude steel production.

The electrolytic process used to produce aluminium results in a CO2 emission as the petroleum coke anode is consumed.  Emission estimations are based on plant capacity data provided by Alcan (1998, 2000, 2002). Recent alterations to production figures have led to minor revisions for regional drivers for 1995 and 1998, with more significant data revisions required for 1999 and 2000. There have been some significant changes in the aluminium industry in recent years, with the closure of the Kinlocheven plant in 2000, and the expansion of the Lynmouth plant, and hence there has been a swing in emissions from this sector from Scotland to England.  Aluminium production also results in emissions of PFCs.

### 1.8.4	Use of Halocarbons and SF6

The UK emissions of halocarbons and sulphur hexafluoride are based on estimates from a model prepared (and updated annually) by Enviros March (2001).  For some sources, the emission is equal to the consumption of fluid (e.g. aerosols). For other sources the emissions occur during product manufacture, leakage during product lifetime, and at product disposal (e.g. refrigerators).  In these cases emissions are estimated from a time dependent model of the bank of fluid held in products, accounting for product production and disposal.

Data for HFC emissions from metered dose inhalers in the UK are also taken from the Enviros March predictive model.  The National Asthma Campaign’s National Asthma Audit (1999-2000) concluded that, ‘There is little variation in asthma prevalence among children or adults throughout Great Britain.’  Therefore, the regional split of emissions is proportional to population.

Supermarket refrigeration is regarded as sufficiently different from other refrigeration to warrant a separate study.  Emissions are based on a market review of the number and size of supermarkets in the regions, combined with discussions with gas manufacturers on the sales into this sector.  Discussions with supermarket owners also suggest that regional use could be approximately equated to sales volume, which in turn could be approximated by regional population split.

Air conditioning systems in cars began to use HFC134a from around 1993.  Data is supplied by SMMT on regional sales of new cars.  Initially, installation of air conditioning was skewed towards company cars, which are broadly distributed according to population.

PFCs and SF6 are used to cushion the soles of some training shoes.  Data have previously been gathered from discussions with Nike.  Sales figures for the devolved regions of the UK were not available, and therefore the regional split is made according to population.

Sulphur hexafluoride is used in electrical switchgear within the electricity transmission system.  Estimates are based on discussions with gas manufacturers, equipment manufacturers and the Electricity Association. The regional split is based on the total electrical capacity of a particular region.

For aerosols, the split by region is made on the basis of population, although use of these gases often have industrial applications. Making the split using population has the advantage of making the data directly comparable with the figures for the baseline years of 1990 and 1995.

Other sources such as fire extinguishers are very small and are likely to be distributed with the general population.

Most magnesium production takes place in England, with major factories in Manchester, Oxford & Birmingham.  There is no production known in Scotland or Northern Ireland.

Emissions data for regional emissions from semiconductor wafer manufacture are estimated directly from manufacturing data.

## 1.9	Agriculture

The UK inventory was disaggregated into England, Scotland, Wales and Northern Ireland.  No methodological alterations were made in terms of emissions calculations, with defaults and emission factors carried over from the national inventory.

Regional crop areas were obtained from the Defra (previously MAFF) June Agricultural Census for 1990, 1995, 1998 - 2001 (MAFF: 1991,1996,1999, 2000; DEFRA: 2001, 2002).  Crop production data is taken from Agriculture in the UK and Basic Horticultural Statistics for the UK.

Fertiliser applications were derived using regional crop areas and average application rates published in the British Survey of Fertiliser Practice for 1990, 1995, 1998 - 2001 (BSFP, 1991; 1996; 1999; 2000; 2001; 2002).  Application rates in Wales were assumed to be equivalent to those in England, and rates in Northern Ireland were assumed to be the same as Scottish applications. Where application rates were not available for particular crop types, the crop area was amalgamated with a similar crop with a known fertiliser application rate. Where annual applications were not available, fertiliser application for a different year was used.

Livestock numbers were obtained from the Defra (previously MAFF) June Agricultural Census for 1990, 1995, 1998 - 2001 with the exception of deer, which were derived from the distribution reported in Economic Report on Scottish Agriculture (SO, 1996).

As in the national inventory, the area of cultivated histosols (soils of high organic content) was assumed to be equivalent to the area of Eutric Histosols.  This was disaggregated according to a percentage split estimated by the Soil Survey and Land Research Centre (personal communication).

In general, the UK totals in the disaggregated inventory match well with those submitted in the national inventory.  Any small differences are due to the derivation of disaggregated data that was not readily available.  These small differences have been removed by normalising the regional inventories so that the sum of England, Scotland, Wales and Northern Ireland equals the UK Emission.  (For details of the normalisation procedure, see Section 1.1.2.)

## 1.10	Land Use Change and Forestry

### 1.10.1	Introduction

Appendix 6 of the UK National GHG Inventory for 1990 to 2001 describes the methods for estimating removals and emissions of carbon dioxide due to Land Use Change and Forestry (LUCF) (Baggott et al. 2003).

The estimates for Land Use Change and Forestry are from work carried out by the Centre for Ecology & Hydrology described in the scientific literature (Milne and Brown 1997, Milne et al. 1998, Cannell et al. 1999) and in Contract Reports to Defra (see Milne et al. 2003).  The data is reported under IPCC categories 5A (Changes in Forests and Other Woody Biomass), 5D (CO2 Emissions from Soils) and 5E (Other).

The methods used to make estimates for category 5A are presently under review. New information (Hargreaves et al., 2003) on losses of carbon from peatland after afforestation suggest that these are much smaller than previously assumed.  The net effect of revising category 5A data to take this new information into account will result in greater removals of carbon dioxide for each of the Devolved Administration areas.  This will be particularly the case for Scotland where the largest areas of afforestation and peatland are located.  An estimate of likely increases to removals of carbon dioxide for the United Kingdom is given in Appendix 6 of the UK National GHG Inventory for 1990 to 2001 (Baggott et al., 2003).  It is planned to introduce the revised method for inventories to be published from 2004 onwards.

No data is included for Categories 5B (Forest and Grassland Conversion) or 5C (Abandonment of Managed Lands) as these are considered to be negligible, or not occurring, in the UK.  However, studies of data on deforestation in Great Britain have recently been made (Levy and Milne, 2003) and these should allow emissions in Category 5B to be reported in future.

Here further detail is provided on: a) methods used for emissions from soils in Northern Ireland and b) disaggregation to the devolved regions of UK fluxes due to liming of agricultural land and in category 5E (Other).

### 1.10.2.	Land use change in Northern Ireland

For this region the default method of the IPCC Guidelines (IPCC, 1997a; 1997b; 1997c) continues to be used.  Cruickshank and Tomlinson (2000) provide a general description of the method as applied to Northern Ireland.

Awareness of problems associated with the land use statistics available for this default method continue (Milne et al., 2003).  A summary of the changes in land uses and the resulting changes in stored soil carbon are shown in Table A1.12.

No uncertainty analyses have been carried out for the changes on soil carbon for Northern Ireland.

### Emissions of CO2 from Soil due to Liming

Emissions of carbon dioxide from the application of limestone, chalk and dolomite to agricultural soils were estimated for the UK using the IPCC (1997a; 1997b; 1997c) default method.  Data on the use of limestone, chalk and dolomite for agricultural purposes is reported in BGS (2002). Estimates of these individual materials had to be made this year as only their total was published because of commercial confidentiality rules for small quantities.  It is assumed that all the carbon contained in the lime is released in the year of use.  For limestone and chalk, a factor of 120 t C/kt is used, and for dolomite application, 130 t C/kt.  These factors are based on the stoichiometry of the reaction and assume pure limestone and dolomite.

The UK data was disaggregated to the four devolved regions on the assumption that application is in the proportions 73 : 12 : 8 : 6 to England, Scotland, Wales and Northern Ireland.  These proportions are based on the areas of tilled and grass in each region and data from the British Survey of Fertilisers Practice for the fractions of each area that are limed in each region.

Table A1.12	Summary of changes in areas of land use and resulting changes in soil carbon in Northern Ireland using the IPCC “default” method.

| Year | Arable (kha) | Grass (kha) | Semi-natural, not peat (kha) | Semi-natural, Peat / bog (kha) | Urban (kha) | Carbon loss (kt/yr) |
| --- | --- | --- | --- | --- | --- | --- |
| 1970 | 96.9 | 740.3 | 249.9 | 149.0 | 40.4 |  |
| 1971 | 94.4 | 745.2 | 248.0 | 148.4 | 40.6 |  |
| 1972 | 84.2 | 759.5 | 233.5 | 147.8 | 47.7 |  |
| 1973 | 77.6 | 754.5 | 227.8 | 147.1 | 65.2 |  |
| 1974 | 74.8 | 761.1 | 239.1 | 146.5 | 49.6 |  |
| 1975 | 73.4 | 758.5 | 226.5 | 145.9 | 65.7 |  |
| 1976 | 79.2 | 773.9 | 226.1 | 145.3 | 44.6 |  |
| 1977 | 82.9 | 772.5 | 214.8 | 144.7 | 53.3 |  |
| 1978 | 79.9 | 776.1 | 210.1 | 144.1 | 57.4 |  |
| 1979 | 76.8 | 777.1 | 219.0 | 143.5 | 50.4 |  |
| 1980 | 77.5 | 791.2 | 212.9 | 142.9 | 43.1 |  |
| 1981 | 75.1 | 781.5 | 221.1 | 142.3 | 46.9 |  |
| 1990 | 61.5 | 797.9 | 212.6 | 136.9 | 67.7 | -353 |
| 1991 | 62.8 | 806.1 | 210.9 | 135.5 | 61.2 | -361 |
| 1992 | 64.0 | 800.3 | 202.6 | 134.9 | 70.9 | -351 |
| 1993 | 63.0 | 817.2 | 199.9 | 134.9 | 57.2 | -245 |
| 1994 | 57.6 | 814.8 | 197.2 | 134.9 | 66.6 | -366 |
| 1995 | 56.4 | 819.1 | 191.0 | 134.9 | 68.7 | -278 |
| 1996 | 56.5 | 821.0 | 188.9 | 134.9 | 67.8 | -310 |
| 1997 | 58.1 | 826.7 | 184.2 | 134.9 | 64.4 | -222 |
| 1998 | 57.4 | 832.3 | 177.7 | 134.9 | 65.3 | -290 |
| 1999 | 54.8 | 835.2 | 177.3 | 134.9 | 64.7 | -288 |
| 2000 | 52.5 | 831.0 | 175.0 | 134.9 | 74.1 | -259 |
| 2001 | 52.6 | 841.6 | 172.6 | 134.9 | 65.2 | -273 |

### 1.10.4.	Other Sources and Sinks (Table 5E)

Other considerations include the changes in crop biomass (a carbon sink) and three additional sources of carbon emissions:

- Drainage of afforested deep peat

- Drainage of lowland wetlands

- Peat extraction.

The relevant activity data and carbon fluxes for the devolved regions have been developed from estimates made for the UK.

#### 1.10.4.1 Changes in Crop Biomass

The approach taken to estimating changes in stocks of biomass in crops at the UK level is described in Appendix 6 of Baggott et al. (2003). The rates for the devolved regions were estimated from the UK data in the proportions 85 : 12 : 1 : 1 for England, Scotland, Wales and Northern Ireland, based on the areas of cropland in each region.

#### 1.10.4.2 Peat Extraction

Trends in peat extraction in Scotland and England over period 1990 to 2001 are included. In Northern Ireland no new data on use of peat for horticultural use was available and a recent survey of extraction for fuel use suggested that there is no significant trend for this purpose. The contribution of emissions due to peat extraction in Northern Ireland is therefore included as constant from 1990 to 2001. Peat extraction is negligible in Wales.  Activity data are from BGS (2002) and Cruickshank and Tomlinson (1997) provide further information.

#### 1.10.4.3 Lowland (fen) peat drainage

The trend in emissions due to changing areas of drainage is based on the work of Bradley (1997).  All of the UK emission is assumed to be in England.

#### 1.10.4.4 Upland (forestry) peat drainage

The UK total emission is allocated to each devolved region in proportion to an estimate of the area of afforested deep peat in each region (Cannell et al. 1993).

## 1.11	Waste

### 1.11.1	Solid Waste Disposal on Land

In the UK Inventory, emissions are estimated based on a model of methane production in landfill sites. The generation of methane is assumed to follow a first order model with different decay rates for different types of waste.  The model requires data on waste disposals and waste composition from 1945 onwards.

The AEA Technology model of methane generation from landfill sites used for the past four years (Brown et al, 1999) has now been updated and revised for DEFRA by the consultants Land Quality Management (LQM, 2003).

The main changes made by LQM can be summarised as follows:

- New waste inventory data for 1995 (MSW) and 1999 (commercial and industrial waste) have been used to revise the estimates of UK waste arisings.  Degradable organic carbon (DOC) parameters were revised, resulting in much higher values than those used previously, especially for paper.

- Instead of using the IPCC default of 10% for the oxidation of methane in the landfill cover, LQM have developed a model to estimate the residual methane oxidation factor based on field and laboratory observations of soil oxidation capacity and expert judgement.  Oxidation factors are estimated to be much higher than the IPCC default – up to 90%.

- The estimates of capture of landfill gas for flaring or utilisation have been increased, based on a survey of installed flare capacity.

The overall effect of these changes is to predict more methane generation, but also more capture and much more oxidation of methane.  On balance, the overall level of emissions is predicted to be similar for current and future years, but much higher (by a factor of two) for past years.

The regional estimates have been calculated by LQM based on updated waste arisings data for the regions.  Up until 1995, waste arisings data is assumed to be the same as in Brown et al (1999).  After 1995, data are taken from the England and Wales National Waste Production Survey (Environment Agency, 1999b), the Scottish Waste Data Digest (SEPA, 2001) and the Waste Management Strategy Northern Ireland (DoE NI, 2001).

The Brown et al (1999) study adjusted the proportion of waste landfilled for each region to reflect regional data on waste disposals.  For Scotland and Northern Ireland, this meant higher percentage disposals to landfill than for England and Wales.  However the LQM approach is based only on the waste arisings data (baseline scenario).  There is no adjustment for different proportions of waste disposal to landfill in each region.  It is also assumed that the composition of waste in each region is the same, and that the degree of methane recovery is the same in each region.

The LQM study provides regional estimates for Scotland and Northern Ireland but only a total aggregated figure for England and Wales.  The aggregated emissions estimate provided by LQM has therefore been split between England and Wales assuming that the same ratio applies as for the 2000 estimates based on the Brown et al (1999) model (94% England, 6% Wales).

Previous years’ (1990, 1995, 1998, 1999 and 2000) estimates have also been revised to reflect the new estimates of the LQM model.  The LQM report provides only estimates for the years 1990, 1995 and 2000.  Figures for 1998 and 1999 have therefore been estimated by linear extrapolation between 1995 and 2000.

### 1.11.2	Waste Water Handling

The UK emission of methane from sewage treatment is based on a model reported by Hobson et al (1996).  This breaks down the UK arisings of sewage into the amounts treated by up to 14 different disposal routes.  Empirical emission factors are applied to each route to estimate methane emissions.  Methane recovery is assumed for the anaerobic digestion route.  The model requires estimates of the annual arisings of sewage and the proportion treated by the disposal routes.  These data are taken from the UK Sewage Sludge Survey for 1990/91 (DOE, 1993).

A survey for 1995/96 has been released (Environment Agency, 1999), although the regional disaggregation was based on the earlier work, in line with the UK inventory.  The survey reports disposals of sewage for Scotland, Northern Ireland and England/Wales and gives data on disposal routes.  However the data is incomplete and it is necessary to make assumptions to account for all disposals.  Where assumptions had to be made, the UK defaults were adopted.  Data for Wales are taken from the later survey since the earlier work did not specifically identify Wales.  The Welsh estimates are based on data reported by Welsh Water whose area of activity does not exactly match the Welsh National Territory but should give a reasonable estimate.  Using the regional data, estimates have been made for 1990 and 1995 for Scotland, Northern Ireland and Wales.  It was assumed that the percentage mix of disposal routes did not change between 1990 and 1998 but that regional sewage disposals increased in line with regional population and with the production of sewage per head assumed by Hobson et al (1996) for the UK.

The Hobson et al (1996) study includes estimates for 2001 that take account of projected changes in sewage disposal following the cessation of sea dumping in 1998.  Some modifications were made to the initial model predictions, as it is clear that some of the disposal routes anticipated by Hobson et al have not been adopted (for example sewage sludge incineration is not widespread in Scotland).  The model also predicts the amount of sewage gas recovered for use or disposal.  This data has been used as the driver for sewage gas combustion.

### 1.11.3	Waste Incineration

The UK Inventory reports emissions from the incineration of sewage sludge, municipal solid waste and some chemical waste.  Regional estimates are based on DEFRA (2001a) which reports data for the amount sewage incinerated for Scotland, Northern Ireland and England & Wales.

Emission drivers from MSW Incinerators for 1990-1995 are based on capacity data for individual incinerators taken from RCEP (1993).  It is assumed there were no significant changes between 1990 and 1995.  Estimates for recent years are based on plant capacity data (Patel, 2000).  All of the larger MSW incinerator plant have been re-fitted during the late 1990s to generate electricity and are therefore reported as power stations in the regional inventories. A handful of smaller waste incinerators (municipal, industrial and clinical) are used for space/district heating and are reported as commercial or miscellaneous.  The disaggregation of these smaller heat-generating plant is based on the same driver as for larger MSW incinerators, as there is no specific source of information that provides a more satisfactory estimation of the regional split. The total consumption of these incinerators is reported in DTI (2001, 1996, 1991).

The 2001 inventory includes emissions from clinical waste incineration.  Emissions are allocated to the regions based on a set of plant capacity data for 1998.

The 2001 version of the UK Inventory includes emissions from chemical waste incineration for the first time.  Emissions data are available for England and Wales only, based on data taken from the Pollution Inventory (Environment Agency, 2002), and these data are used for the regional estimates also.  Some chemical waste incineration takes place in Scotland but no emissions data are available, and hence the emissions contributed from this source are currently omitted from the Scottish regional figures.  No chemical incinerators have been identified in Northern Ireland.

## 1.12 Uncertainties

### 1.12.1	Introduction

The uncertainties in the UK Inventory are estimated using a Monte Carlo simulation.  Eggleston et al (1998) and Salway et al (2001) describe this in detail.  In general this involves estimating the uncertainties in the activity data and the emission factors for all the emission source categories and then using a Monte Carlo simulation package to calculate the uncertainty in the emission totals.  In order to apply a similar approach to the regional inventories, it is necessary to estimate uncertainties for the regional activity data (i.e. fuel consumption, production data).  The same emission factors are used in the regional inventories as in the UK Inventory, so their uncertainties are known.  In the UK Inventory uncertainties in the activity data are estimated on the basis of the statistical differences between fuel supply and demand data reported in the energy statistics.  However, such data is not available for the regional data used.  Moreover, for some sources, no direct activity data is available at all, and it has been necessary to distribute the UK data using surrogate data (e.g. employment statistics).  In such cases, it is impossible to say whether the surrogate statistics are an accurate indicator of fuel consumption.

Given the difficulties inherent in estimating the uncertainties in the regional estimates, it is evident that such estimates are likely to be tentative and should be treated as indicative rather than a precise estimate of uncertainties.

### 1.12.2	Regional Uncertainty Estimation

The uncertainties in the regional inventories are also estimated using a Monte Carlo simulation. In order to simplify the calculations, the source categories are far broader than those used in the UK Inventory simulation.  In the regional simulation, the combustion categories are effectively the total consumption of a particular fuel.  This contrasts with the UK simulation where there is a further disaggregation into sectoral categories (e.g. power stations, refineries).  The rationale for this is that it is far easier to form a view of the uncertainty in the total consumption of a fuel in a region than to attempt to estimate uncertainties in diverse sectors where in some cases surrogates have been used.

For each of the broad source categories, an estimate of the activity uncertainty has been made for Scotland, Wales, Northern Ireland and Unallocated, with the aim of obtaining a factor of a similar scale to the uncertainty within the UK national inventory data.

Example: Sales data from the DTI is used to estimate burning oil consumption in Scotland and Northern Ireland. It is reasonable to assume that the uncertainty of this data is similar to that of the UK burning oil consumption figures.

For England & Wales, sales data is available only as a combined figure, and therefore the estimates for burning oil use in each of England and Wales are calculated using population as a surrogate factor to divide the available data.

This extra calculation step introduces greater uncertainty, and hence it has been estimated that the uncertainty for the Wales and England data are double that for Scotland and Northern Ireland.

It is important to note that the uncertainties in the inventories for the UK, England, Scotland, Wales, Northern Ireland, Unallocated are inter-dependent, because:-

- UK Emissions = [England + Scotland + Wales + Northern Ireland + Unallocated]

Thus, in the example above, it is necessary to check that the calculated uncertainty for England and Wales appeared reasonable and could be reconciled to the uncertainties for the other regions.

In many of the non-combustion sources (e.g. LUCF, agriculture, coal mines) the overall uncertainty is dominated by the emission factor and the uncertainty in the activity data is not a determining factor.  Moreover, it is unlikely to be any significant variation in uncertainties between regions.  In these cases, a low uncertainty for the activity data (say 1%) may be assumed for each region, whilst the UK uncertainty for the emission factor is applied.

In the case of halocarbon and SF6 emissions it is not considered feasible to attempt to assume varying uncertainties across the regions.  Hence it is assumed that the uncertainty of each regional emission is the same as that of the UK.  This is equivalent to assuming that the emissions are correlated or that the uncertainty in the regional activity data is very small.  This is clearly not the case, but given that these emissions make a small contribution to the total GWP, this seems a reasonable working approximation.

## References

Adger, N, Subak, S. (1995) Carbon fluxes resulting from Land Use Change: Land Use Data and Policy.  In: Carbon Sequestration in Vegetation and Soils (Ed. by MGR Cannell), DOE/ITE Contract EPG 1/1/3. Interim Report March 1995. Department of Environment, London.

- AES (2002), Power station fuel consumption, Personal Communication.

- Alcan (1998, 2000, 2002), Personal Communication.

Baggott, SL, Davidson, I, Dore, C, Goodwin, J, Milne, R, Murrells, TP, Rose, M, Watterson, JD, Underwood, B, (2003). UK Greenhouse Gas Inventory, 1990 to 2001 Annual Report for submission under Framework Convention on Climate Change. National Environmental Technology Centre, AEA Technology Centre AEAT/ENV/R/1396.

Barlow, TJ, Hickman, AJ, Boulter, P, “Exhaust Emission Factors 2001: Database and Emission Factors”, TRL Report PR/SE/230/00, September 2001

Bell, DM, (2002), Power Station Fuel Consumption, Personal Communication, Northern Ireland Office.

BGS (1991, 1996, 2002), United Kingdom Minerals Yearbook 1991, 1996, 2002, British Geological Survey, Natural Environment Research Council.

- BP (2002), Grangemouth Refinery, Personal Communication.

Bradley, I, (1997) Carbon loss from drained lowland fens. In: Carbon Sequestration in Vegetation and Soils (Ed. by MGR Cannell), DOE/ITE Contract EPG 1/1/3. Final Report March 1997. Department of Environment, London.

- British Cement Association, (1999, 2001, 2002), Personal Communication.

British Glass, (2001), Emissions from the UK Glass Industry. Report prepared for NETCEN.

Brown, KA, Smith, A, Burnley, SJ, Campbell, DJV, King, K, Milton, MJT, (1999) Methane Emissions from UK Landfills, AEA Technology, AEAT-20624, Culham

BSFP (1991, 1996, 1999, 2000, 2001, 2002), British Survey of Fertiliser Practice for 1990, 1995, 1998-2001.

Cannell, MGR, Dewar, RC, Pyatt, DG, (1993) Conifer plantations on drained peatland in Britain: a net gain or loss of carbon? Forestry, 66,  pp353-368.

Cannell, MGR, Milne, R, Hargreaves, KJ, Brown, TAW, Cruickshank, MM, Bradley, RI, Spencer, T, Hope, D, Billett, MF, Adger, WN & Subak, S (1999) National inventories of terrestrial carbon sources and sinks: the UK experience. Climatic Change, 42, 505-530

- Coal Authority, (2002), Regional Coal Production, Personal Communication.

Cruickshank, MM, Tomlinson, RW (1997) Carbon loss from UK peatlands for fuel and horticulture. In: Carbon Sequestration in Vegetation and Soils (Ed. by MGR Cannell), DOE, Contract EPG 1/1/3. Final Report March 1997. Department of Environment, London.

Cruickshank, MM, and Tomlinson, RW, (2000) Change in soil carbon storage in Northern Ireland: estimated by the IPCC default and matrix methods. In.: Carbon Sequestration in Vegetation and Soils (Ed. by R. Milne), DETR Contract EPG 1/1/39. Final Report April 2000. (Available at http://www.nbu.ac.uk/ukcarbon/)

DEFRA, (2000a), EPSIM, Sewage Sludge Disposal Tables produced for Digest of Environmental Statistics.

DEFRA, (2001) June Agricultural Census for 2000.  Department for Environment, Food and Rural Affairs.

DEFRA, (2002). Agricultural Census Statistics for UK, Annual Reference Tables, Labour Force. DEFRA Agricultural Statistics website.

- DETR, (1997), Digest of Environmental Statistics, The Stationary Office.

DfT (2002a), “Vehicle Licensing Statistics: 2001 data”, Transport Statistics Bulletin (SB(02) 18), 2002

DfT (2002b), “Transport Statistics Great Britain: 2002 edition”, The Stationary Office

- DLTR (2002), Aircraft Movement data, Personal communication.

- DOE, (1993), UK Sewage Sludge Survey, CES Limited, HMSO

DoRDNI (2002a), “Northern Ireland Vehicle Registrations, Transport Statistics 2001-2002”, Central Statistics and Research Branch, Department of Regional Development in Northern Ireland

(Available at  http://csrb.drdni.gov.uk/statistics/statistics.asp)

DoRDNI (2002b), “Vehicle Kilometres of Travel Survey of Northern Ireland, Annual Report 1999”, Report for Roads Service Headquarters, Department of Regional Development in Northern Ireland, 2002

DoRDNI (2002c), “Annual Traffic Census Report 2000”, Department of Regional Development in Northern Ireland, communication, 2002

- DoT (1996) Transport Statistics communication, Department of Transport

DTI, (1991, 1992, 1996, 2000, 2001, 2002), Digest of UK Energy Statistics, 1991, 1992, 1996, 1997, 1999, 2000, 2001, 2002 editions.

DTI (2001a), Development of UK Oil and Gas Resources, Department of Trade and Industry, The Stationary Office.

- DTI (2001b), Personal Communication, EPA, Department of Trade and Industry.

EFW, (2001), Energy for Waste, Current and Projected Energy for Waste Capacity in UK as at December 2000.

- Edison Mission, (2001), Personal Communication.

Eggleston, HS, Salway, AG, Charles, D, Jones, BMR, Milne, R, (1998), Treatment of Uncertainties for National Estimates of Greenhouse Gas Emissions, National Environmental Technology Centre, AEA Technology, Report AEAT - 2688.

Electricity Association.  Electricity transmission data, personal communication (see Section 1.8.4)

Environment Agency (1999), UK Sewage Sludge Survey, Regional Presentation, WRc, R&D Project Record P2/065/1

- Environment Agency, (1999a) The Pollution Inventory 1998

Environment Agency (2000, 2001, 2002) The Pollution Inventory 1999, 2000 & 2001

Enviros March, (2001), Report on the Emissions of HFCs, PFCs and SF6 in England, Scotland, Wales and N Ireland in the years 1990, 1995, 1998 and 1999.  (Selected emission estimates updates in 2001).  Enviros March. Telegraphic House, Waterfront Quay, Salford Quays, Manchester

European Environment Agency (2000), COPERT III : Computer Programme to Calculate Emissions from Road Transport - Methodology and Emission Factors (Version 2.1), L. Ntziachristos and Z Samaras, European Topic Centre on Air Emissions, European Environment Agency, November 2000

Goodwin J W L, A G Salway, T P Murrells, CJ Dore, NR Passant, KR King, PJ Coleman, MM Hobson, ST Pye and JD Watterson, (2001),“UK Emissions of Air Pollutants 1970 - 1999. A Report of the National Atmospheric Emissions Inventory”, AEAT/ENV/R/0798, ISBN 1-85580-031-4, November 2001

Hargreaves, KJ, Milne, R and Cannell, MGR, (2003). Carbon balance of afforested peatland in Scotland. Forestry. (In press)

Hobson, J, Palfrey, R, Sivil, D, Palfrey, E, Day, M, (1996) Control Measures to Limit Methane Emissions from Sewage and Sludge Treatment and Disposal, WRc, Report No DOE 4118

- Innogy (2002), Power station fuel consumption, Personal Communication.

IPCC, (1997a), IPCC Revised 1996 Guidelines for National Greenhouse Gas Inventories, Volume 1, Greenhouse Gas Inventory Reporting Instructions, IPCC WGI Technical Support Unit, Hadley Centre, Meteorological Office, Bracknell, UK.

IPCC, (1997b), IPCC Revised 1996 Guidelines for National Greenhouse Gas Inventories, Volume 2, Greenhouse Gas Inventory Workbook, IPCC WGI Technical Support Unit, Hadley Centre, Meteorological Office, Bracknell, UK.

IPCC, (1997c), IPCC Revised 1996 Guidelines for National Greenhouse Gas Inventories, Volume 3, Greenhouse Gas Inventory Reference Manual, IPCC WGI Technical Support Unit, Hadley Centre, Meteorological Office, Bracknell, UK.

ISSB, (1991, 1996, 1999, 2000, 2001, 2002) Iron and Steel Industry, Annual Statistics for the UK, 1990, 1995, 1998-2001. ISSB Limited

Levy, P and Milne, R, (2003) Deforestation rates in the United Kingdom. In: UK Emissions by Sources and Removals by Sinks due to Land Use, Land Use Change and Forestry Activities. Annual report (2003) for DEFRA Contract EPG1/1/160 (Ed. by R. Milne). (Available at http://www.nbu.ac.uk/ukcarbon/ )

- LRC(1999), Personal Communication from L Saddler, London Research Centre

LQM (2003) Methane emissions from landfill sites in the UK.  Final report.  January 2003.  Report for the UK Department for Environment, Food and Rural Affairs.  Authors: Gregory, R.G., Gillett, A.G., Bradley, D.  LQM report 443/1.  Defra contract EPG 1/1/145.

MAFF (1991, 1996,1999, 2000) June Agricultural Census for 1990, 1995, 1998 and 1999. Ministry of Agriculture, Fisheries and Food

MCG, (1999), UK Emissions of HFCs, PFCs and SF6 Potential Emission Reduction Options.  A study for the Department of the Environment carried out by March Consulting Group. Telegraphic House, Waterfront Quay, Salford Quays, Manchester

Milne, R and Brown, TA (1997) Carbon in the vegetation and soils of Great Britain. Journal of Environmental Management, 49, 413 - 433.

- Hargreaves, KJ, Milne, R and Cannell, MGR, (2003). Carbon balance of

afforested peatland in Scotland. Forestry, 76, 299 - 317.

Milne, R, Brown, TAW and Murray, TD (1998) The effect of geographical variation in planting rate on the uptake of carbon by new forests of Great Britain. Forestry, 71, 298 – 309.

Milne, R., Tomlinson, RW, and Murray, TD,  (2003) Land Use Change and Forestry: The 2001 UK Greenhouse Gas Inventory and projections to 2020.  In: UK Emissions by Sources and Removals by Sinks due to Land Use, Land Use Change and Forestry Activities. Annual report (2003) for DEFRA Contract EPG1/1/160 (Ed. by R. Milne). (Available at http://www.nbu.ac.uk/ukcarbon/ )

- Network Rail, (2003), personal communication from Rebecca Smith

- NIO (1996, 2001, 2002), Northern Ireland Abstract of Statistics

- ONS (1996, 2000, 2001, 2002), Office for National Statistics

- ONS (2000), IDBR, Employment Database

Patel, NM (2000), Personal Communication, Strategic Consultancy, AEA Technology

Phoenix Natural Gas (2002), Power station fuel consumption, Personal Communication

PowerGen (2002), Power Station Fuel Consumption, Personal Communication from H Evans.

RCEP (1993), Royal Commission on Environmental Pollution, 17th Report, Incineration of Waste, HMSO, London

Sage, PW, (2001), Methane from Abandoned Coal Mines in the UK, AEA Technology, Report AEAT/ENV/R/0500, Harwell

Salway AG, Murrells TP, Cook A, (2000), “Greenhouse Gas Inventories for England, Scotland, Wales and Northern Ireland: 1990, 1995 and 1998”, AEAT/R/ENV/0314, ISBN 0-7058-1795-4, November 2000

Salway AG, Murrells TP, Pye S, Watterson J ,Milne R, (2001), “Greenhouse Gas Inventories for England, Scotland, Wales and Northern Ireland: 1990-1999” AEAT/R/ENV/0772, ISBN 0-7058-1795-4, August 2001

Salway AG, Murrells, TP, Milne, R, Hidri S (2002a), “Greenhouse Gas Inventories for England, Scotland, Wales and Northern Ireland: 1990-2000” AEAT/R/ENV/01182, August 2002

Salway, AG, Murrells, TP, Milne, R, Ellis, S, (2002). UK Greenhouse Gas Inventory, 1990 to 2000. Annual Report for submission under the Framework Convention on Climate Change, National Environmental Technology Centre, AEA Technology Centre, AEAT/R/ENV/1000, ISBN 0-7058-1805-5

SCOPEC (2002), Environmental Database for Emissions and Discharges from Offshore Installations, Atmospheric Emissions Inventory, 2001, UKOOA.

Scottish Power plc (2002), Power Station Fuel Consumption, Personal Communication

Scottish and Southern Energy plc. (2002). Power Station Fuel Consumption, Personal Communication from A. McRae.

- SEPA (2002), Scottish Waste Data Digest 2001

- SMMT (2002), regional car sales, personal communication. (section 1.8.4)

- SO, (1996), Economic Report on Scottish Agriculture, Scottish Office, 1996

- SO, (1999), Scottish Energy Statistics. Paper supplied by G Winter, Scottish Office

Transco (2002) Historic Gas Demands by Load Category, Personal Communication.

Translink (2003), Personal Communication from I Campbell, Translink (previously NIR, Northern Ireland Railways)

- TXU (2002), Power station fuel combustion, personal communication.

UKOOA, (1999), Personal communication from P Russell, UK Offshore Operators Association Limited (Environment Committee)

UKPIA, (1998), Personal Communication from I McPherson, United Kingdom Petroleum Industry Association Ltd

UKPIA, (2002), Personal Communication, United Kingdom Petroleum Industry Association Ltd

Williams, A, (1993), Methane Emissions, Watt Committee Report Number 28, The Watt Committee on Energy, London.

- WO, (1998), Digest of Welsh Historical Statistics 1974-1996, The Welsh Office