Revision to the Method of Estimating Emissions from Aircraft in the UK Greenhouse Gas Inventory

Report to Global Atmosphere Division, DEFRA John Watterson Charles Walker Simon Eggleston

netcen/ED47052 July 2004

| Title | Revision to the Method of Estimating Emissions from Aircraft in the UK Greenhouse Gas Inventory |
| --- | --- |

| Customer | Global Atmosphere Division, DEFRA |
| --- | --- |

| Customer reference | RMP/2106 |
| --- | --- |

| Confidentiality, copyright and reproduction | This document has been prepared by Netcen in connection with a contract to supply goods and/or services and is submitted only on the basis of strict confidentiality. The contents must not be disclosed to third parties other than in accordance with the terms of the contract. |
| --- | --- |

| File reference | netcen/ED47052 |
| --- | --- |

| Reference number | /tmp/in/input.doc |
| --- | --- |

| Issue number | Issue 1.1 |
| --- | --- |

Address for	Netcen

Correspondence	Culham Science Park

Abingdon

Oxon

OX14 3ED

Telephone	0870 190 6594

Facsimile	0870 190 6607

John.D.Watterson@aeat.co.uk

Netcen is an operating division of AEA Technology plc

Netcen is certificated to ISO9001 & ISO 14001

|  | Name | Signature | Date |
| --- | --- | --- | --- |
| Authors | John Watterson Charles Walker Simon Eggleston |  | 19-07-04 |
| Reviewed by | Brian Underwood |  | 19-07-04 |
| Approved by | John Watterson |  | 30-07-04 |

Executive Summary

This report describes an improved technique for estimating emissions and fuel use for civil aircraft in the UK and presents estimates of emissions (1990 to 2002 inclusive) from this technique.  The work has been completed to improve the quality of the estimates of emissions from aviation for the UK greenhouse gas inventory.  The method estimates emissions from the number of aircraft movements of broken down by at aircraft type at each UK airport and so complies with the IPCC (Intergovernmental Panel on Climate Change) Tier 3 specification.  The emissions will be included in both the UK National Atmospheric Emissions Inventory and the UK Greenhouse Gas Inventory.  Therefore, emissions of a range of pollutants have been estimated in addition to greenhouse gases.

In comparison with earlier methods used to estimate emissions from aviation for the greenhouse gas inventory, the approach described here is much more detailed and reflects differences between airports and the aircraft that use them.  Additional emission sources (such as aircraft auxiliary power units) are also now included.

This study has utilised data from a range of airport emission inventories compiled in the last few years by Netcen.  This work includes the RASCO study (23 regional airports, with a 1999 case calculated from CAA movement data) carried out for the Department for Transport (DfT), and the published inventories for Heathrow, Gatwick and Stansted airports, commissioned by BAA and representative of the fleets at those airports in a fairly recent year.  Emissions of NOx and fuel use from the Heathrow inventory have been used to verify the results of this study.

The initial results of the work were reviewed with the Department for Transport, and the recommendations from that consultation have been used to refine the method used to estimate emissions from aviation.

Contents

1	Introduction	1

1.1	Overview of the Work	1

1.2	Structure of the report	2

1.3	Quoted accuracy of the emissions in this study	2

2	Background to the study and scope of work	3

2.1	Background to the study	3

2.2	Scope of work and pollutants covered	3

2.3	airport related and other aviation emissions not considered in this report	3

3	Emissions from aviation	5

3.1	Pollutants produced by aircraft engines	5

3.2	Fuels used by aircraft engines	5

3.3	Types of aircraft engines	5

3.4	Categories of air transport movements	5

4	The terminology of aircraft movement	7

4.1	Overview of a typical flight cycle	7

4.2	modes (or phases) of the LTO cycle	8

5	Reporting requirements of the UNFCCC and UN/ECE for emissions from aviation	9

5.1	Overview of reporting requirements	9

5.2	Aggregation of domestic and international emissions	9

5.3	FCCC reporting categories	10

5.4	UN/ECE reporting categories	10

5.5	IPCC Definitions of international and domestic aircraft movements	11

5.5.1	Current accordance of the GHGI method with the IPCC definitions	12

6	Mapping of aviation fuels between the NAEI and the GHGI	13

6.1	IPCC fuel categories for aviation	13

6.2	DTI DUKES fuel categories for aviation	13

6.3	Mapping of aviation fuel categories used in DUKES, the GHGI and the NAEI	14

7	Current data and methods used to estimate emissions from UK aviation	15

7.1	activity data	15

7.1.1	Fuel use	15

7.1.2	Movement data	15

7.2	emission factors	15

7.2.1	EFs to estimate emissions of CO2 and SO2	15

7.2.2	EFs to estimate emissions of CH4 and N20 from civil aviation	16

7.3	Methods used to estimate emissions	16

7.3.1	CO2 and SO2	16

7.3.2	Non CO2 pollutants	17

7.3.3	Limitations of these methods	17

7.4	IPCC Tier equivalent of current calculation methods	17

7.4.1	The IPCC Tier system	17

7.4.2	Classification of current methods	19

7.5	Geographical coverage of current estimates	19

8	Revised data and methods to estimate emissions from UK aviation	21

8.1	Overview of method	21

8.2	Advantages of the new calculation method	21

8.3	Activity data	22

8.3.1	Aircraft movements and distances travelled	22

8.3.2	Times in mode	22

8.4	Emission factors	22

8.4.1	LTO cycle	23

8.4.2	Cruise	24

8.5	Basic Approach to estimating emissions from the LTO cycle	24

8.5.1	Treatment of times in mode	25

8.5.2	Treatment of engine thrust settings	26

8.5.3	Assigning airports to airport classes	26

8.6	Details of calculations of emissions according to LTO Mode	26

8.6.1	Taxi-out and Taxi-in	26

8.6.2	Hold	27

8.6.3	Take-off roll and initial-climb	27

8.6.4	Climb-out	28

8.6.5	Approach	28

8.6.6	Landing-roll	28

8.6.7	APU	28

8.7	calculation of emissions in the Cruise	29

8.7.1	Non-greenhouse gases	29

8.7.2	Greenhouse gases	30

8.7.3	Adjustment to the great circle distances	30

8.7.4	Procedures to avoid double counting of emissions for domestic flights	30

8.8	reconciliation of Fuel use and emissions with Fuel sales estimates in DUKES	30

8.9	matching international/domestic split of emissions to IPCC requirements	31

8.10	Geographical coverage of the estimates of emissions	31

8.11	accordance of the EUROSTAT aviation emissions INVENTORY with the UK GHGI	32

9	Comparison of emissions with the Heathrow airport inventory	33

9.1	London Heathrow aiport emissions inventory	33

9.2	Comparison with of emissions and fuel consumptions with the Heathrow inventory	33

10	Estimates of emissions from UK aviation from 1990 to 2002	35

10.1	Time series of estimates of emissions from aviation	35

10.1.1	‘Direct’ greenhouse gases	35

10.1.2	GWP weighted emissions	35

10.1.3	‘Indirect’ greenhouse gases	35

10.2	Comments on methodology	36

10.3	Fuel reconciliation	47

11	Uncertainty analysis	49

12	Acknowledgements	50

13	References	51

Appendices

| Appendix 1 | Matching of specific and generic aircraft types |
| --- | --- |
| Appendix 2 | Times in mode |
| Appendix 3 | Emission factors and fuel flow rates |
| Appendix 4 | Consultation with the DfT |

- Document revision history

| - Issue | - Revision history |
| --- | --- |
| - Issue 1.0 | - First issue - Submitted to Prof. David Lee for expert review |
| - Issue 1.1 | - Minor typographical errors corrected |

Abbreviations, acronyms and definitions

Greenhouse gases

CH4	Methane

CO2	Carbon dioxide

N2O	Nitrous oxide

Other pollutants

HC	Hydrocarbons

NOx	Nitrogen oxides (NO+NO2)

NMVOC	Non-Methane Volatile Organic Compounds

PAHs	Polyaromatic hydrocarbons

PM10	Particulate Matter with aerodynamic diameter less than 10m

VOC	Volatile Organic Compound

…………………………………………………………………………………………………………………………………………………

General

AIA	Aerospace Industries Association (represents US aerospace industry)

Airside	Within the boundary of an airport

ANCAT	Abatement of Nuisances Caused by Air Transport (ECAC Group of Experts)

AOQ	Annual Oil Questionnaire - produced by the DTI

APU	Auxiliary Power Unit
Supplies power to the aircraft when the main engines are not running and is needed to start the main engines.

AQMA	Air Quality Management Area

AQS	Air Quality Strategy (UK)

ATMs	Air Transport Movement(s)

BA	British Airways

BOSS	BOSS database.  BAA database of aircraft movements

CAA	Civil Aviation Authority

CAEP	Committee on Aviation Environmental Protection (a standing committee of the International Civil Aviation Organization)

CFMA	Central Flight Management Unit

CFMU	Central Flow Management Unit
The Central Flow Management Unit is responsible for providing Air Traffic Flow Management services within the airspace of participating European States.  It undertakes all actions required to realise this objective in accordance with the policy defined by the authorities of the Eurocontrol Organisation.

Chapter (2/3/4)	An aircraft ranking system structured around noise levels of aircraft (by April 2002, all aircraft using Heathrow will be Chapter 3).
The Chapter system is based on International Civil Aviation Organization Chapter rules.  If ICAO adopts CAEP's recommendations, a new Chapter 4 standard will go into effect on Jan. 1, 2006.  It requires new airliners to be at least 10 decibels quieter than the existing ICAO standard.  In the U.S., new aircraft are currently certified to the FAA's "Stage 3" standard, which is based on the current ICAO "Chapter 3" rules.

Civil	For reporting to the IPCC, this would include emissions from domestic aviation and international aviation – emissions from each are reported separately.  It excludes military emissions.

DEFRA	Department of the Environment, Farming and Rural Affairs

Dft	Department for Transport

Domestic	Civil flights within the UK

DORA	Department of Operational Research and Analysis

DTI	Department of Trade and Industry

DTLR	Department of Transport, Local Government and the Local Regions

DUKES	Digest of UK Energy Statistics – an annual publication containing demand and supply of fuels

EAA	European Airlines Association

EC	European Commission

ECAC	European Civil Aviation Conference
Programme to promote uniformity in the adoption and integration of environmental recommendations and measures, as well as to discourage the proliferation of local rules at individual airports.

EF	Emission Factor
Specifies the quantity of greenhouse gas or pollutant emitted per unit activity (for example, quantity of greenhouse gas emitted per tonne of fuel burnt, or per LTO cycle flown).

Emission indices	Analogous to emission factors

ERLIG	Sub group of ECAC (European Civil Aviation Conference)

EU	European Union

EUROCONTROL	The European Organisation for the Safety of Air Navigation
EUROCONTROL has as its primary objective the development of a seamless, pan-European air traffic management (ATM) system.

FAA	Federal Aviation Administration

FIS	Flight Information System (BAA data collection tool)

General aviation	Has a specific definition from ICAO
ICAO’s ‘Manual on the ICAO Statistics Programme’ defines ‘general aviation’ as all civil operations other than scheduled air services and non-scheduled air transport operations for remuneration or hire.  For ICAO statistical purposes, the general aviation activities are classified into instructional flying, business and pleasure flying, aerial work and other flying.

GHG	Greenhouse gas

HAL	Heathrow Airport Ltd (now BAA plc)

IABC	International Business Aviation Council

IATA	International Air Travel Association

ICAO	International Civil Aviation Organisation

IEA	International Energy Agency

IFR	(Civil aviation) Instrumental Flight Rules

IPCC	Intergovernmental Panel on Climate Change

Mode	Phase of the LTO cycle
(taxi-out, hold, take-off roll, initial climb, climb out, approach, landing roll and taxi-in)

International	Flights between UK and abroad

Landside	Outside the boundary of an airport

LHR	London Heathrow

LTO	Landing and Take-Off cycle

MOS	Monthly Oil Statistics data return produced by the DTI

NAEI	National Atmospheric Emissions Inventory

NAQS	National Air Quality Strategy (UK)

NATS	National Air Traffic Services Ltd

NFR	Nomenclature For Reporting – reporting system developed by the UN/ECE TFEIP

NIR	National Inventory Report
Contains estimates of greenhouse gas emissions and the methods used to produce those emissions.  Submitted annually to the UNFCCC.

Objective (AQS)	Air quality objectives (in the UK AQS)
Are policy targets generally expressed as a maximum ambient concentration to be achieved, either without exception or with a permitted number of exceedences, within a specified timescale.

POC	Port of call

RASCO	Regional Air Services Co-ordination study

SERAS	South East and East of England Regional Air Services Study (project commissioned by the DTLR)

SN	Smoke Number – dimensionless metric of soot emissions

SNAP	CORINAIR Nomenclature - Selected Nomenclature for sources of Air Pollution

SNAP97	CORINAIR Nomenclature.
SNAP97 covers additional activities that are sources of the heavy metals and persistent organics and is fully consistent with the IPCC nomenclature (1996 Revised IPCC Guidelines for National Greenhouse Gas Inventories, WMO/IPCC, 1997) developed for reporting under the UN Framework Climate Change.

Standard (AQS)	Air quality standards (in the UK AQS)
Concentrations of pollutants in the atmosphere which can broadly be taken to achieve a certain level of environmental quality.  The standards are based on assessment of the effects of each pollutant on human health including the effects on sensitive subgroups.

Tankering	The term for loading of fuel used for subsequent flight segments (taking more fuel onboard than is required by the fuel flight plan).
The main reasons for tankering of fuel are commercial ones – for example, in cases where the cost of fuel consumed in carrying additional fuel is more than offset by the difference in the price of fuel at the departure point and a destination where the fuel could be loaded (IPPC, 1999).

UK	United Kingdom

UKPIA	UK Petroleum Industry Association (UKPIA).  Trade association which represents the oil refining and marketing industry in the United Kingdom

UN/ECE	United Nations Economic Commission for Europe

UNFCCC	United Nations Framework Convention on Climate Change

VFR	(Civil aviation) Visual Flight Rules - also called general aviation

# Introduction

## Overview of the Work

This report describes an improved technique for estimating emissions and fuel use for civil aircraft in the UK and presents estimates of emissions (1990 to 2002 inclusive) from this technique.  The work has been completed to improve the quality of the estimates of emissions from aviation for the UK greenhouse gas inventory.  The method estimates emissions from the number of aircraft movements broken down by individual aircraft at each UK airport and so complies with the IPCC (Intergovernmental Panel on Climate Change) Tier 3 specification.  The emissions will be included in both the National Atmospheric Emissions Inventory and the greenhouse gas inventory, and therefore emissions of a range of pollutants have been estimated in addition to greenhouse gases.

The method only considers emissions from the aircraft themselves.  Emissions from stationary plant on the airport and ground support vehicles are reported in separate and appropriate categories in the greenhouse gas inventory.  Emissions from the cruise phase appear as memo items in IPCC and UN/ECE reporting, and the method reported here covers this part of the aircraft’s journey also.  Cruise emissions are only calculated for aircraft departures from UK airports, thus giving a total fuel consumption compatible with recorded deliveries of aviation fuel to the UK.  Calculating the total fuel consumption including the cruise component enables a check to be made against fuel deliveries.  In addition there is considerable interest in cruise emissions given the enhanced effect on climate change compared with ground level emissions for some pollutant gases.

In comparison with earlier methods used for the greenhouse gas inventory, the approach used here is more detailed and reflects differences between airports and the aircraft that use them.  Additional emission sources (such as aircraft auxiliary power units) are also now included.

This study has utilised data from a range of airport emission inventories compiled in the last few years by Netcen.  This work includes the RASCO study (23 regional airports, with a 1999 case calculated from CAA movement data) carried out for the Department for Transport (DfT), and the published inventories for Heathrow, Gatwick and Stansted airports, commissioned by BAA and representative of the fleets at those airports in a fairly recent year.  A separate estimate of the emissions of NOx from the Heathrow inventory has been used to verify the results of this study.

The initial results of the work were reviewed with the DfT, and a summary of the findings and changes made as a result of this work are given in Appendix 4.

## Structure of the report

The report is structured as follows:

- Chapter 1	(this section) presents the structure of the report

- Chapter 2	provides the background to the study and scope of work

- Chapter 3	provides a very brief overview of the pollutants released from aircraft engines, aircraft engine technologies, and the fuels used in aviation

- Chapter 4	gives a very brief overview of the key concepts and terminology used in aviation to describe aircraft movements – these are fundamental to understanding how aviation emissions are aggregated in the greenhouse gas inventory

- Chapter 5	provides an overview of the reporting requirements of the UK to the UNFCCC and UN/ECE.  It also explains the aggregation of emissions into domestic and international categories, and the differences required for reporting to these two organisations.

- Chapter 6	gives an overview of the aviation fuels specified by the IPCC that need to be considered when estimating emissions from aviation, and explains the differences between IPCC aviation fuel terminology and the terminology used in the publication containing UK national energy statistics (DUKES).

- Chapter 7	summarises the activity data required and methods used to estimate aviation emissions in the current UK greenhouse gas inventory.  At the time this report was written, these methods were last used in the 2002 greenhouse gas inventory.

- Chapter 8	describes the revised methodology that has been used to estimate aviation emissions presented in this report, and will be used to estimate emissions in the 2003 greenhouse gas inventory onwards

- Chapter 9	describes a comparison of selected emissions from the new GHG inventory with those predicted by the latest Heathrow inventory

- Chapter 10	summarises the emissions from the revised GHG aviation method (1990 to 2002 inclusive), and compares them the emission estimates (1990 to 2001 inclusive) in the current (2002) GHG inventory

There are three Appendices that provide additional technical information.

## Quoted accuracy of the emissions in this study

In this report, emissions are quoted to 0.01 ktonne (or better) purely for convenience, to avoid the risk of rounding errors, and for convenience when taking ratios.  The number of decimal places used should not be taken as indicative of the accuracy of the estimates.

# Background to the study and scope of work

## Background to the study

This study was commissioned by Defra following a recommendation made in the peer review of CO2 from combustion in the UK greenhouse gas inventory (Simmons, 2003):

“The present modelling of emissions from aviation uses a relatively simple model of air traffic movements.  In view of the magnitude and growing importance of aircraft emissions a more detailed model is recommended using aircraft movements by flight stage and aircraft type.  This will assist in distinguishing between international and domestic flights in conformity with their definitions in the international methodology.  …”

## Scope of work and pollutants covered

The method used to estimate emissions complies with the protocols set out in international guidelines on emissions inventories developed by the IPCC, and provides appropriately aggregated emissions for national reporting purposes required by the UNFCCC.  The method used corresponds to the IPCC Tier 3 specification, which is in fact the CORINAIR ‘Detailed’ methodology (1996).  Emissions from civil aviation (domestic and international) from the LTO cycle and the cruise are covered.

The method estimates emissions of the following ‘direct’ greenhouse gases:

- CO2

- CH4

- N2O

A wide range of other pollutants are also estimated as these are required for the National Atmospheric Emissions Inventory.  These pollutants are:

- CO	(indirect GHG)

- NMVOC	(indirect GHG)

- NOx	(indirect GHG)

- PM10

- SO2	(indirect GHG)

- selected metals

- selected organic species

## airport related and other aviation emissions not considered in this report

The method used to estimate emissions from military aircraft has not been revised in this work programme.  In part, this is because there is insufficient information in the public domain about military aircraft movements and emission factors for military aircraft engines.  Emissions from military aircraft are included in the current greenhouse gas inventory, and these are reported under IPCC source category 1A5b Other Military Aircraft & Naval Vessels.

Emissions from airside ground support vehicles are not considered in this report.  Emissions from these vehicles are included in the greenhouse gas inventory, and are reported under IPCC source category 1A3e Other Transportation.

Emissions from stationary plant on the airport are reported in separate and appropriate categories in the greenhouse gas inventory.

# Emissions from aviation

This section provides a very brief overview of the pollutants released from aircraft engines, aircraft engine technologies, and the fuels used in aviation.

A considerable amount of the information in this section has been taken from the descriptions given in a paper on aircraft emissions by Rypdal (2000).  The paper was written to support the development of the IPCC Good Practice Guidance on estimating emissions for greenhouse gas inventories.

## Pollutants produced by aircraft engines

Emissions from aircraft originate from the fuel burnt in the main aircraft engines, and the engines powering the auxiliary power units (APU).  The combustion products from the engines include greenhouse gases and other pollutants.  Carbon dioxide, oxides of nitrogen, and carbon monoxide are emitted in the greatest quantities per tonne of fuel consumed, but methane, nitrous oxide, other by-product gases, and trace amounts of metals are emitted also.  The fuel use and emissions will be dependent on the fuel type, aircraft type, engine type, engine load and flying altitude.

## Fuels used by aircraft engines

Two types of fuels are used by aviation world-wide: aviation gasoline, and kerosene (sometimes referred to as jet kerosene).

Aviation gasoline is used in small piston engined aircraft only.  Most aircraft run on kerosene, and the bulk of fuel used for UK aviation is kerosene.  Chapter 6 provides further information about the definitions of fuels used in the UK and how these compare to IPCC categories of aviation fuels.

## Types of aircraft engines

In general, there are two types of engines; reciprocating piston engines, and gas turbines.  In piston engines, energy is extracted from a combustion chamber by means of a piston and crank mechanism that drives the propellers to give the aircraft momentum.  In gas turbines compressed air is heated by combustion in a combustion chamber and the major part of this is used for propulsion of the aircraft.  A part of the energy contained in the hot air flow is used to drive the turbine that in turn drives the compressor.  Turbojet engines use only energy from the expanding exhaust stream for propulsion, whereas turbofan and turboprop engines use energy from the turbine to drive a fan or propeller for propulsion.

## Categories of air transport movements

Air transport movements are often divided into four categories (EEA, 2000):

- Civil IFR (Instrumental Flight Rules) flights;

- Civil VFR (Visual Flight Rules) flights, also called general aviation;

- Civil Helicopters, and

- Operational Military flights.

Most emissions originate from aircraft flying under Civil IFR, which covers the scheduled flights of civil aircraft.

# The terminology of aircraft movement

This section provides a very brief overview of the key concepts and terminology used in aviation to describe aircraft movements.  These concepts are fundamental to understanding how aviation emissions are aggregated in the greenhouse gas inventory.

## Overview of a typical flight cycle

Operations of aircraft are usually divided into two main parts – the landing and take off cycle, and the cruise phase (EEA, 2000) – see Figure 3.1.

- The Landing/Take-off (LTO) cycle includes all aircraft activities near the airport that take place below the altitude of 3000 feet (1000 m).  Therefore, this includes taxi-in and out, take-off, climb-out, and approach and landing.  The LTO is defined in ICAO (1993).

- Cruise which here is defined as all activities that take place at altitudes above 3000 feet (1000 m).  No upper limit of altitude is given.  Cruise, in the inventory methodology, includes climb to cruise altitude, cruise, and descent from cruise altitudes.

Figure 3.1	The simplified aircraft operation cycle (taken from Rypdal (2000))

## modes (or phases) of the LTO cycle

The LTO cycle can be subdivided into eight ‘modes’, or phases:

- taxi-out;

- hold;

- take-off roll;

- initial climb (i.e. wheels-off the runway to throttleback, assumed to occur at 450 m);

- climb out (450 m to 1000 m);

- approach (1000 m to touchdown);

- landing roll

- taxi-in.

‘Taxi-out’ commences at the stand, and so includes pushback from the stand towards the runway, and ends when the aircraft joins the take-off queue.  ‘Taxi-in’ commences at the runway exit and ends when the aircraft reaches the stand.  Note Figure 4.1 simplifies some of these modes, for example, landing in the figure would be approach and landing.

# Reporting requirements of the UNFCCC and UN/ECE for emissions from aviation

This section provides an overview of the reporting requirements of the UK to the UNFCCC and UN/ECE.  It also explains the aggregation of emissions into domestic and international categories, and the differences required for reporting to these two organisations.

## Overview of reporting requirements

Estimates of emissions from aviation in the UK greenhouse gas inventory are reported to the FCCC.  Estimates of emissions from aviation in the NAEI are reported to the UN/ECE.  These two organisations had slightly different reporting requirements until 2003, although the requirements are now identical.  This is explained the sections below.

## Aggregation of domestic and international emissions

The reporting requirements to the UN/ECE have altered recently.  Table 5.1 shows the emissions included in the totals for the domestic and international aviation categories for the FCCC and the UN/ECE.

Table 5.1	Components of emissions included in reported emissions from civil aviation

| Organisation receiving emissions data | Category of emissions | LTO | Cruise |
| --- | --- | --- | --- |
| FCCC | Domestic |  |  |
|  | International | m | m |
| UN/ECE | Domestic |  | m |
|  | International | m | m |

Notes

	emissions included in national totals

m	memo items - emissions are estimated and reported, but are not included in national totals

Emissions from the LTO cycle include emissions within a 1000 m ceiling of landing.

## FCCC reporting categories

Table 5.2 shows the categories in which emissions from aviation are reported to the FCCC.  The LTO and cruise components are summed, and not reported separately.

Table 5.2	IPCC categories for reporting aviation emissions to the FCCC

| Category of aviation | IPCC category | Comment |
| --- | --- | --- |
| International | 1A3ai International Aviation | Includes emissions from the LTO and the cruise phase of the flights. Estimates of emissions from international flights are to be excluded as far as possible from national totals. International emissions are often referred to as emissions from International Bunkers. These emissions appear as ‘memo items’ at the foot of IPCC reporting tables. |
| Domestic | 1A3aii Domestic | Includes emissions from all civil domestic passenger and freight traffic movements inside a country (not used as international bunkers). Includes emissions from the LTO and the cruise phase of the flights. |
| Military | 1A5b Mobile | Emissions from the use of fuel by military aircraft. |

## UN/ECE reporting categories

Reporting of emissions from the National Atmospheric Emissions Inventory to the UN/ECE now occurs in the new CORINAIR NFR (Nomenclature For Reporting) format.  Previously, emissions were reported using SNAP codes (Selected Nomenclature for sources of Air Pollution).

Table 5.3 shows the categories in which emissions from aviation are reported to the UN/ECE.  Emissions from aviation are reported to the UN/ECE occurs in slightly more detail than to the FCCC, and emissions are split according to the section of the flight cycle (LTO and cruise).

Table 5.3	NFR categories for reporting aviation emissions to the UN/ECE

| Category of aviation | Section of the flight cycle | NFR category | Comment |
| --- | --- | --- | --- |
| International | LTO | 1A3ai (i) International Aviation (LTO) | Memo item and not included in national totals |
|  | Cruise | 1A3ai (ii) International Aviation (Cruise) | Memo item and not included in national totals |
| Domestic | LTO | 1A3aii (i) Civil Aviation (Domestic, LTO) |  |
|  | Cruise | 1A3aii (ii) Civil Aviation (Domestic, Cruise) | Memo item and not included in national totals |
| Military | - | 1A5b Other, Mobile (Including military) |  |

## IPCC Definitions of international and domestic aircraft movements

The IPCC Reference Manual (IPCC, 1997c) provides guidance on the definitions of international and domestic aircraft movements:

“If an aircraft goes from one airport in one country to another in the same country and then leaves to a third airport in another country, the first flight stage is considered a domestic trip while the second is considered an international trip.  It is not important whether the airport is a domestic or an international airport.  In addition, the type of activity (LTO, cruise, domestic, international) is independent of the nationality of the carrier.  This treatment of domestic and international differs from that recommended to states by the International Civil Aviation Organization (ICAO, 1994).  ICAO defines as domestic all flight stages flown between domestic points by an airline registered in that state and therefore excludes flights between domestic points by foreign airlines.”

This guidance was supplemented by additional guidance in the IPCC Good Practice Guidance (IPCC, 2000), and is summarized in Table 2.9 of the Good Practice Guidance.

Table 5.2	IPCC categories for reporting aviation emissions to the FCCC (Table 2.9 of the IPCC GPG “Distinction between domestic and international flights”)

| Case | Domestic | International |
| --- | --- | --- |
| Depart and arrive in same country | Yes | No |
| Depart from one country and arrive in another | No | Yes |
| Depart in one country, stop in the same country without dropping or picking up any passengers or freight, then depart again to arrive in another country | No | Yes |
| Depart in one country, stop in the same country and drop and pick up passengers or freight, then depart finally arriving in another country | Domestic stage | International stage |
| Depart in one country, stop in the same country, only pick up more passengers or freight and then depart finally arriving in another country | No | Yes |
| Departs in one country with a destination in another country, and makes an intermediate stop in the destination country where no passengers or cargo are loaded. | No | Both segments international |

The GPG notes that

“These definitions should be applied irrespective of the nationality of the carrier.  The treatment of domestic and international aviation, both in the IPCC Guidelines and in Table 2.9 of the GPG, differs from that recommended to states by the International Civil Aviation Organization for the purposes of classifying flight stages when reporting air carrier statistical data (ICAO, 1997). In this context, ICAO defines as domestic, all flight stages flown between domestic points by an air carrier whose principal place of business is in that state and therefore (i) includes flight stages between domestic points that precede a flight stage to another country, and (ii) excludes flights between domestic points by foreign carriers.”

### Current accordance of the GHGI method with the IPCC definitions

The aircraft movement data in the current greenhouse gas inventory is based on data obtained from the CAA, and from data presented in Transport Statistics UK (which is also derived from CAA data).  In these data sets, domestic flights are those entirely within the UK, Isle of Man and the Channel Isles.  International flights are those flown between the UK, Isle of Man and the Channel Isles and points within other countries.  This definition differs slightly from the IPCC definition (IPCC, 2000) in that journeys involving departure in the UK, a stop in the UK and then departure for another country would be counted as a domestic movement and an international movement.  Under the IPCC definition such a journey would be counted as domestic plus international only if passengers or cargo were dropped at the stop.  Otherwise the journey would be an international one.  However, the proportion of such journeys will be small.

# Mapping of aviation fuels between the NAEI and the GHGI

- This section provides an overview of the aviation fuels specified by the IPCC that need to be considered when estimating emissions from aviation, the slight differences between IPCC aviation fuel terminology and the terminology used in the publication containing UK national energy statistics (DUKES), and the mapping between fuels in the NAEI and the GHGI.

## IPCC fuel categories for aviation

- The IPCC Reporting Instructions (IPCC, 1997a; see table in Section 1.2 of the guidelines) give a list of fuels that should be considered when reporting national emissions, including those from aviation.  The three fuels that need to be considered are: aviation gasoline, jet gasoline, and jet kerosene.

- The glossary in the IPCC Reporting Instructions gives the definitions of these fuels:

- Aviation Gasoline.  This is motor spirit prepared especially for aviation piston engines, with an octane number suited to the engine, a freezing point of -60°C and a distillation range usually within the limits of 30°C and 180°C.

- Jet Gasoline.  (Naphtha type Jet Fuel or JPA).  A light hydrocarbon oil distilling between 100°C and 250°C for use in aviation turbine power units.  It is obtained by blending kerosenes and gasoline or naphthas in such a way that the aromatic content does not exceed 25 per cent in volume, and the vapour pressure is between 13.7 kPa and 20.6 kPa.

- Jet Kerosene.  This is a distillate used for aviation turbine power units.  It has the same distillation characteristics between 150°C and 300°C (generally not above 250°C) and flash point as kerosene.  In addition, it has particular specifications (such as freezing point) which are established by the International Air Transport Association (IATA).

## DTI DUKES fuel categories for aviation

- Estimates of the demand for petroleum products used in aviation are presented in the commodity balance tables of the DTI publication DUKES (e.g. DUKES, 2003).  There are only two aviation fuel categories in DUKES, and the definitions given of those fuels are:

- Aviation spirit (AS).  A light hydrocarbon oil product used to power piston-engined aircraft power units.

- Aviation turbine fuel (ATF).  The main aviation fuel used for powering aviation gas-turbine power units (jet aircraft engines)

- Aviation spirit is equivalent to aviation gasoline.  Aviation turbine fuel includes jet gasoline and jet kerosene.  We currently assume the majority of ATF in the UK is jet kerosene.  Emissions estimated from the use of aviation turbine fuel will therefore include emissions from both jet gasoline and jet kerosene.

## Mapping of aviation fuel categories used in DUKES, the GHGI and the NAEI

- The greenhouse gas inventory uses much of the underlying data in the NAEI, but the fuel nomenclature of the two inventories is slightly different.  Table 6.1 summarises these differences.

Table 6.1	Mapping of fuels reported in DUKES, and used in the GHGI and the NAEI

| - Category of fuel | - DTI DUKES | - GHGI | - NAEI |
| --- | --- | --- | --- |
| - Liquid | - Aviation Spirit | - Aviation Gasoline | - Aviation Spirit |
|  | - Aviation Turbine Fuel | - Jet Kerosene | - Aviation Turbine Fuel |

# Current data and methods used to estimate emissions from UK aviation

This section of the report summarises the activity data required and methods used to estimate aviation emissions in the current UK greenhouse gas inventory.  At the time this report was written, these methods were last used in the 2002 inventory, and are presented in the 2004 National Inventory Report (Baggott et al., 2004).  When the 2003 GHG inventory is compiled (for the 2005 National Inventory Report), the methods described below will be superseded with the revised methods described in Chapter 8 of this report.

## activity data

Fuel use and aircraft movements are the activity data needed to produce estimates of emissions from aviation for the current method.

### Fuel use

Total inland deliveries of aviation spirit and aviation turbine fuel to air transport are given in DTI DUKES.  This is the best approximation of aviation bunker fuel consumption available and covers international, domestic and military use.  Tankering (the term for loading of fuel used for subsequent flight segments) may mean the fuel use estimates in DUKES do not correctly reflect use by UK aviation – see Section 10.3 for further discussion about this).

Total consumption by military aviation is currently given by the MOD, and is assumed to be aviation turbine fuel.

### Movement data

The DfT reports data on arrivals and departures of aircraft at UK airports, and domestic and international aircraft km flown.  This was used to estimate total domestic and international landing and take-offs (LTOs).

## emission factors

The emissions of carbon dioxide and sulphur dioxide depend on the carbon and sulphur contents of the fuels.  The EFs of methane and nitrous oxide and other pollutants are fleet-averaged default values taken from IPCC or EMEP/CORINAIR guidelines.

### EFs to estimate emissions of CO2 and SO2

Emissions of carbon dioxide and sulphur dioxide are derived from the carbon and sulphur contents in the fuels.  The sulphur contents are updated annually from data provided by UKPIA.  The carbon contents of the fuels are currently under review.  The EFs used in the 2002 GHGI for the year 2002 are shown in Table 7.1.

Table 7.1	EFs for CO2 and SO2 from aviation fuels (kg/t)

| Fuel | CO2 | SO2 |
| --- | --- | --- |
|  | (kt/tonne)1 | (kg/tonne) |
| Aviation Turbine Fuel | 859 | 0.80 |
| Aviation Spirit | 865 | 0.80 |

Notes

1	Emission factor as kg carbon/tonne fuel

### EFs to estimate emissions of CH4 and N20 from civil aviation

Emissions factors for methane and nitrous oxide currently used are IPCC or EMEP/CORINAIR defaults; see Table 7.2.

Table 7.2	EFs for non- CO2 gases for aviation

| Type of flight | Units | CH4 | N2O | NOx | CO | NMVOC | Fuel burnt |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Domestic LTO | kg/LTO | 0.394a | 0.1b | 9.0a | 16.9a | 3.706a | 1000 |
| International LTO | kg/LTO | 6.96a | 0.2b | 23.6a | 101.3a | 65.54a | 2400 |
| Domestic Cruise | kg/t fuel | 0b | 0.1b | 11b | 7b | 0.7b | - |
| International Cruise | kg/t fuel | 0b | 0.1b | 17b | 5b | 2.7b | - |

Notes

a	EMEP/CORINAIR (1996)

b	IPCC (1997)

## Methods used to estimate emissions

The method used to estimate emissions depends on the greenhouse gas, or pollutant.  The two methods used are described below.  The methods separate out LTO from cruise, and provide an approximate estimate of the split between domestic and international fuel use.

### CO2 and SO2

The procedure used to estimate emissions of carbon dioxide and sulphur dioxide is set out below.

- Total inland deliveries of aviation spirit and aviation turbine fuel to air transport are given in DTI DUKES.  This is the best approximation of aviation bunker fuel consumption available and covers international, domestic and military use.

- Data on numbers of arrivals and departures at UK airports are reported by the DfT.  This was used to estimate total domestic and international landing and take-offs (LTO).

- Data on domestic aircraft km are reported by the DfT.

- Using IPCC default fuel consumption factors for domestic LTOs and cruise together with the LTO data and total domestic km flown, an estimate was made of the total fuel consumption of domestic flights.

- Total consumption by military aviation is given in ONS (1995) and MOD (from 1996 onwards) and is assumed to be aviation turbine fuel.

- An estimate of international fuel consumption was made by deducting military fuel and domestic fuel from the inland deliveries of aviation fuel calculated in (i).

Based on these assumptions the total consumptions of aviation turbine fuel and aviation spirit by domestic and international flights were estimated.  Hence, it was a simple matter to calculate the carbon dioxide emission using the emission factors given in IPCC Guidelines (IPCC, 1997c) and shown in Table 7.1.

### Non CO2 pollutants

Emissions from non-CO2 pollutants were calculated according to the very simple EMEP/CORINAIR/IPCC methodology described in EMEP/CORINAIR (1996) and IPCC (1997c).  The procedure was:

- Data on the annual number of domestic and international landing and takeoff cycles (LTO) (DLTR, 2001b) were used together with the default emission factors in Table 7.2 to estimate the emissions within the take-off and landing phase of the domestic and international flights.

- The fuel consumptions within the cruise phases of the domestic and international flights were then calculated by subtracting the LTO fuel consumption from the total domestic and international consumptions.

- The emissions within the cruise phase were calculated using the cruise emission factors in Table 7.2 together with the cruise fuel consumption.

### Limitations of these methods

The current methods used to estimate emissions from aviation have a number of limitations.  These are:

General limitations of the methods

- Does not make use of detailed movement and route information available in the UK, which splits activity by aircraft type.

- Does not use emission factor information for individual aircraft types.

Specific limitations of the methods

- CO2 and SO2

  - Domestic/international fuel split is based on an approximate calculation.

- Other gases

  - LTO/cruise fuel split is based on an approximate calculation.

  - Emission factors are fleet averages, or for representative types.

Other points

- The current method will overestimate fuel use and emissions from domestic aircraft.  This is because only two aircraft types are considered and the default factors used relate to older models.  It is clear, that more smaller, modern aircraft are in use on domestic and international routes.  Emissions from international aviation will be correspondingly underestimated.

## IPCC Tier equivalent of current calculation methods

### The IPCC Tier system

The IPCC methods used to estimate GHGs emissions from fuel combustion are divided into Tiers.  Tier 1 methods are based on simple estimation methods, and require basic activity data.  The Tier 2 and 3 methods are more complex methods, and need more detailed activity data.  The IPCC guidance does not include a description of a Tier 3 method for estimating emissions from aviation, and refers the reader to the CORINAIR guidance.  The IPCC Good Practice Guidance provides a decision tree which helps the GHG compiler decide which method to use (IPCC, 2000; Figure 2.7 Methodology decision tree for aircraft).

CORINAIR and ECAC (European Civil Aviation Conference) have a similar hierarchy of methods.  According to the EMEP/CORINAIR terminology, methods are classified into three types: ‘very simple’, ‘simple’ or ‘detailed’.  Under ECAC these are ANCAT 1, ANCAT 2, and ANCAT 3 (where ANCAT refers to the ECAC group of experts on the Abatement of Nuisances Caused by Air Transport).  A summary of the data requirements of the three levels of methodology are summarised in the Table 7.3.

Table 7.3	Minimum data requirements for the three levels of methodology.  (adapted from ECAC/ANCAT guidance material; after Lock, (2003))

| CORINAIR - > | Very simple | Simple | Detailed |
| --- | --- | --- | --- |
| IPCC - > | Tier 1 | Tier 2 | Tier 3 |
| ANCAT - > | ANCAT 1 | ANCAT 2 | ANCAT 3 |
| Activity data required … |  |  |  |
| Fuel sales | Total quantity of fuel sold | Total quantity of fuel sold | Total quantity of fuel sold |
| Split of fuel use | Domestic & international | Domestic & international | Domestic & international |
| Total number of movements | Domestic & international | Movements by aircraft type | Movements by aircraft type |
| Details of aircraft types | Not required | Used on domestic & international flights | Used on domestic & international flights |
| Levels of detail of movement data | Total number of domestic and international aircraft movements | Number of domestic and international LTOs, total number of aircraft movements | Details of departure and arrival airports of individual aircraft for both domestic and international flights |
| Reconciliation of estimated fuel use to nationally reported fuel sales | Not required – method should automatically lead to emissions being estimated from all fuel sales | Not required – method should automatically lead to emissions being estimated from all fuel sales | Adjustment of estimated emissions required |

The Tier 1 and Tier 2 methods are outlined in the IPCC Guidelines (1997c).  Both methods are based on distinguishing between domestic fuel use and international fuel use.  Tier 1 is purely fuel based, while the Tier 2 is based on the number of landing/take-off cycles (LTOs) and fuel use – see Table 7.3.  The estimate of CO2 emissions depends on carbon content of fuel (and the fraction oxidized) and therefore should not vary significantly with the Tier.

The IPCC guidelines provides a further split of Tier 2 into Tier 2a and Tier 2b methods.  Building on this, the IPCC Good Practice Guidance (IPCC, 2000) provides a decision tree to help inventory compilers choose the highest possible Tier to use based on the data available to them – see Figure 7.3.

Figure 7.3	Methodology decision tree for aircraft
(Figure 2.7 in the IPCC Good Practice Guidance (IPCC, 2000))

Data on fuel sold is needed in all three methods.  However, whereas in the Tier 1 and 2 methods, the fuel sold is an input parameter, in the Tier 3 method, the fuel sold is used to adjust the emissions after they have been estimated.

### Classification of current methods

The current methods lies between the Tier 1 and 2 methods in terms of complexity.  However, as LTO data are available at an aggregate level, the current method could be considered to correspond to IPCC Tier 2a.

## Geographical coverage of current estimates

According to the IPCC Guidelines, "inventories should include greenhouse gas emissions and removals taking place within national (including administered) territories and offshore areas over which the country has jurisdiction." IPCC, (1997c); (IPPC Reference Manual, Overview, Page 5).

The national estimates of aviation fuels consumed in the UK are taken from DTI DUKES.  The current (and future) methods used to estimate emissions from aviation rely on these data, and so the geographical coverage of the estimates of emissions will be determined by the geographical coverage of DUKES.

The UK DTI have confirmed that the coverage of the energy statistics in DUKES is England, Wales, Scotland and Northern Ireland plus any oil supplied from the UK to the Channel Islands and the Isle of Man.  This clarification was necessary since this information cannot be gained from UK trade statistics.  The DTI have confirmed estimates in DUKES exclude Gibraltar and the other UK overseas territories.  The DTI definition fully accords with that of the "economic territory of the United Kingdom" used by the UK Office for National Statistics (ONS), which in turn accords with the definition required to be used under the European System of Accounts (ESA95).

# Revised data and methods to estimate emissions from UK aviation

This Chapter describes the revised methodology that has been used to estimate aviation emissions presented in this report, and will be used to estimate emissions in the 2003 greenhouse gas inventory onwards.

## Overview of method

The revised method described below complies with IPCC Tier 3 requirements (the IPCC guidance for Tier 3 refers onwards to the ‘detailed’ methodology presented in the CORINAIR technical guidance).

The methodology:

- Provides estimates which are compatible with IPCC and UN/ECE reporting requirements

- Uses a common approach to estimating emissions for all UK airports.  Emissions at all UK airports that report aircraft movement to the CAA are included, and one method is applied to all airports to give a consistent approach to estimating emissions.

- Uses the latest information available on aircraft LTO emissions in the UK, by utilising data from recent studies at major UK airports, and uses a much-improved method for estimating emissions from the cruise.

For the LTO emissions at each airport, the methodology adopted here was similar to that used for the study of Heathrow airport  (Underwood et al., 2003a).  The approach takes account of reduced thrust during take-off and includes the contribution from auxiliary power units (APUs).  However, to extend the emission estimates to cover all airports and to give a consistent time series from 1990 to the 2002 inclusive, we have made some simplifications to the methodology to reflect the availability of data at these other airports.

Separate estimates have been made for emissions from the LTO cycle and the cruise phase for both domestic and international aviation.  For the LTO phase, fuel consumed and emissions per LTO cycle are based on detailed airport studies, and engine specific emission factors (from the ICAO database).  For the cruise, fuel consumed and emissions have been estimated using distances (based on great circles) travelled from each airport for a set of representative aircraft types.

## Advantages of the new calculation method

This new method has a number of advantages over the previous one:

- For CO2 and SO2 – it provides a more accurate split of domestic and international fuel used, and hence emissions.

- For other gases and pollutants – it provides a more accurate estimate of fuel use in the LTO cycle and the cruise phase.

- The method provides a more accurate estimate of total emissions through the use of aircraft-specific emissions in the LTO and in the cruise.

- The method enables the evolution of the aircraft fleet (over time) to be reflected in time series of emissions.

- New airport-specific LTO information can be easily incorporated. (As airports grow in capacity, this is likely to lead to a specific emission inventory being compiled for those airports and the specific LTO data can then be used for those airports thereafter.)

- The method allows updated UK-specific cruise emission-factor versus sector length curves to be substituted for the CORINAR defaults as they become available.

## Activity data

### Aircraft movements and distances travelled

Detailed activity data has been provided by the CAA.  These data include aircraft movements broken down by

- airport

- aircraft type

- whether the flight is international or domestic

- next/last POC (port of call) from which sector lengths (great circle) have been calculated

The CAA have provided a time series of aircraft movement data from 1990 to 2002 inclusive, and will provide a new set of annual movement data each year.  Table 8.1 lists the fields of data that are provided in the raw datasets.

Table 8.1	CAA movement data supplied in the data sets

| Field | Description |
| --- | --- |
| Year | All years 1990 to 2002 (to be extended annually) |
| Reporting airport | Airport reporting data – all significant airports in UK |
| International | Yes/No (i.e. flight starts/ends outside UK) |
| Arrival | Yes/No |
| Last/Next | Name of last (if arrival) or next (if departure) airport |
| Origin | Airport where flight originated (not used) |
| Aircraft | Aircraft type |
| Sector length | Great circle length in kilometres |
| Number | Number of flights of this type per year |

### Times in mode

The methods used to estimates the times in mode are presented in Section 8.5.

Times in mode data are presented in Appendix 2.

## Emission factors

Emission factors for the LTO cycle of aircraft operation have been taken from the ICAO database.  The cruise emissions have been taken from CORINAIR data (which are themselves developed from the same original ICAO dataset).  More information is provided in the following two sections.  Appendix 3 provides details of emission factors:

- LTO emission factor data are presented in Table A3.1a&b and Table A3.2 a&b

- APU emission factor data are presented in Table A3.3

- cruise emissions data are presented in Table A3.4

### LTO cycle

For the LTO-cycle calculations, emissions per LTO cycle are required for each of a number of representative aircraft types.  For this purpose, the GHGs and pollutants can be considered in the following categories.

#### CO2 and SO2 and metals in aviation fuels

For these pollutants, the emissions in the LTO cycle are obtained by multiplying the fuel used in the LTO cycle by a constant pollutant-specific factor.  In the case of SO2, the factor derives from the average sulphur content of the fuel.  This factor varies from year to year, and an appropriate value has been assigned for each year based on the sulphur content of fuel provided by UKPIA.  For CO2 the factor is based on the average carbon contents of aviation fuels (ignoring losses to CO, CH4 and NMVOC in this part of the calculation).  This factor may vary from year to year, but the following single values have been assigned to the fuels for all years:

- Aviation Turbine Fuel	859 kg carbon/tonne

- Aviation Spirit	865 kg carbon/tonne

These carbon contents are the ones currently used in the GHGI.  The carbon contents of aviation fuels are currently under review.

The contents of selected metals in the aviation fuels were derived from analyses of the fuels as part of the emission factor development programme for the NAEI.

#### NOx and CO

These pollutants are measured for jet engines in service as part of ICAO engine certification, so they are readily included in airport emission inventories.  All the inventories referred to above generated by Netcen include NOx as a specific output, so the emission factors for modes of the LTO cycle for representative aircraft types were readily extracted from existing work.  CO has not often been required in past airport inventories, but the data have been extracted with only a modest amount of additional work.

#### NMVOC and CH4

Total HC (expressed as methane equivalent) is also measured as part of ICAO engine certification so the total HC emissions are easily included in airport emission inventories.  Data on the speciation of VOC emissions from aircraft is sparse, but approximate factors exist to convert HC emissions (as methane equivalent) in the LTO cycle into mass of NMVOC and mass of methane in the LTO cycle.  In principle these factors are aircraft-type and engine thrust dependent, but the existing data only support one constant factor, which is applicable to all jets and turboprops and another factor for piston engined aircraft.  The factor (unitless) to convert HC (as methane) to total mass of VOC was 1.22 for jets and turboprops and 1.04 for piston engined aircraft (US EPA).  The methane fraction was 9.57% for jets and turboprops and 10.95% for piston engined aircraft (Spicer et al., 1984).  NMVOC emissions were a specific output for some, but not all, of the airport emission inventories referred to above, but the total HC emissions (and so NMVOC and methane) per LTO by aircraft type have been extracted with only a modest amount of additional work.

#### N2O

Very little information exists on the aircraft-type specific emission factors for this pollutant, and IPCC default emission factors have been used (IPPC, 1997c).

#### PM10

The ICAO databank does not contain emission factors for PM10 directly, but does include ‘Smoke Number’ (SN).  The Smoke Number is an indirect measure of particulate emissions calculated from the reflectance of a filter paper measured before and after the passage of a known volume of smoke-bearing sample.

The emission factors for PM10 are based on a method developed by Netcen and used in the Heathrow 2002 inventory (Underwood et al., 2003a).  This derived emission factors of PM10, based on a relationship between a gravimetric measure of particulate emissions and SN (Döpelheuer and Lecht, 1999).  A key area of uncertainty for this methodology was the question of applicability of the relationship, derived from data for one (old) type of combustor, to modern combustors.

#### Calculation of weighted-average LTO emission factors

The emission factors and fuel flow rates for the modes of the LTO cycle are based mainly on the ICAO database, which gives emissions for most engine types.  The UK movement data supplied by the CAA specifies just the aircraft type, and does not indicate which engine types are used on the aircraft.  Therefore, in cases where different engines might be used on a single type of aircraft, a weighted average of the emissions from the different engines was used.  For Heathrow the weighted average was based on the mix of engines at Heathrow, as used in the Heathrow Airport Study 2002 (Underwood et al., 2004).  For all other airports the weighted average was based on the mix of engines at Gatwick, as used in the Gatwick Airport study, (Underwood et al., 2003b).  For some aircraft, data were not available for at least one airport. In these cases either data from the other airport were used or data for another aircraft type were used as a surrogate.

This method gives emission factors and fuel flow rates at four standard thrust settings (7%, 30%, 85% and 100%).  Emission factors and fuel flow rates at other thrust settings are obtained by piecewise linear interpolation.

### Cruise

The cruise emissions have been taken from the CORINAIR workbook as recommended in the IPCC guidelines.  This gives emissions of NOx, HC, CO according to distance flown for a representative number of aircraft types.  By performing a linear regression on the data, the emissions can be estimated according to distance flown.

The CAA movement data contain a more detailed breakdown of specific aircraft types than the generic aircraft types of the EMEP/CORINAIR Emission Inventory Guidebook.  Therefore, each specific aircraft type in the CAA data has been assigned to a generic type.  Further details are given in Table A1.1 in Appendix 1.

NMVOC emission factors have been derived from the HC values.  The indications are that CH4 emissions are negligible at cruise altitudes, and we have assumed them to be zero.  This was the assumption in the previous aviation calculation method also.

As for the LTO cycle, emissions factors for CO2, SO2 and metals have been derived from their contents in the aviation fuels.

Very little information exists on the aircraft-type specific emission factors for N2O, and IPCC default emission factors have been used (IPPC, 1997c).

## Basic Approach to estimating emissions from the LTO cycle

The tool used to generate the emissions uses a large database.  The structure of the database had been designed so new aircraft movement data and emission factors can easily be added, or the exiting data modified.

### Treatment of times in mode

Schematically, a contribution to aircraft exhaust emissions (in kg) arising from a given ‘mode’ of aircraft operation (taxiing, for example) is given by the product of the duration (seconds) of the operation, the engine fuel flow rate at the appropriate thrust setting (kg fuel per second) and the emission factor for the pollutant of interest (kg pollutant per kg fuel).  The annual emissions total for the mode (kg per year) is obtained by summing contributions over all engines for all aircraft movements in the year.

The time in each mode of operation for each type of airport and aircraft has been taken from individual airport studies (further information is given in Section 8.5.3).  The time in mode is multiplied by an emission rate (the product of fuel flow rate and emission factor) at the appropriate engine thrust setting in order to estimate emissions for phase of the aircraft flight.  The sum of the emissions from all the modes provides the total emissions for a particular aircraft journey.  The modes are:

- Taxi-out

- Hold

- Take-off Roll (start of roll to wheels-off)

- Initial-climb (wheels-off to 450 m altitude)

- Climb-out (450 m to 1000 m altitude)

- Approach (from 1000 m altitude)

- Landing-roll

- Taxi-in

- APU use after arrival

- Auxiliary Power Unit (APU) use prior to departure

Departure movements comprise the following LTO modes: taxi-out, hold, take-off roll, initial-climb, climb-out and APU use prior to departure.  Arrivals comprise: approach, landing-roll, taxi-in and APU use after arrival.

Equation 1 gives the emissions (and fuel consumption) of pollutant p, from a particular LTO mode of operation m (excluding APU use), for one movement of a specific aircraft type s at an airport:

ELTOa,m,p,s=NsTa,m,sFa,sta,m,sIa,p,sta,m,s	Equation 1

Where:

ELTOa,m,p,s	is the emissions in mode m of pollutant p for a specific aircraft type s at airport type a (kg)

a	is the airport type

m	is the mode

p	is the pollutant

s	is the specific aircraft type

Ns	is the number of engines on aircraft type s

Ta,m,s	is the time in mode m for a specific aircraft type s at airport type a (s)

Fa,st	is the weighted average fuel flow for an engine on aircraft type s at airport type a for thrust t (kg s-1)

Ia,p,st	is the weighted average emission factor of pollutant p for an engine on aircraft type s at airport type a for thrust t (kg / kg fuel)

ta,m,s	is the engine thrust setting during mode m for aircraft type s at airport type a (%)

The fuel consumption is calculated by setting Ia,p,st to unity in Equation 1.

### Treatment of engine thrust settings

As part of engine certification, engine fuel flow and emission factors are measured at 7%, 30%, 85% and 100% thrust settings (ICAO, 1993).  These thrusts have been applied directly to some modes (e.g. 7% for taxing).  Where other thrust settings have been used, for example take-off at reduced thrust, a linear interpolation between these settings has been used.  This interpolation has been done separately for fuel flow and emission factor.

Table 8.2	Default engine thrust settings according to mode

| Mode | Default thrust setting |
| --- | --- |
| Taxi-out, Hold and Taxi-in | 7% |
| Take-off | 100%1 |
| Initial-climb | 100%1 |
| Climb-out | 85%2 |
| Approach | 30% |
| Landing-roll | 7%3 |

Notes

1	Reduced thrust can be used on take-off and data are available for Heathrow and Gatwick

2	Percentage set to reduced take-off thrust, if reduced take-off thrust is below 85%

3	Reverse thrust at a different thrust setting can be used

### Assigning airports to airport classes

We have time in mode data for the three major airports in the UK from specific airport studies.  Other airports are classed as either regional or small with times in mode based on data from the regional airport study:

- Heathrow	data from Heathrow airport study (Underwood et al., 2003a)

- Gatwick	data from Gatwick airport study (Underwood et al., 2003b)

- Stansted	data from Stansted airport study (Underwood et al., 2001b)

- Regional	data from Regional airport study (Underwood et al., 2001a)

- Small	data from Regional airport study (Underwood et al., 2001a)

As more detailed airport inventories become available, the list of airport classes can be extended.

## Details of calculations of emissions according to LTO Mode

This section presents more detail of the methods used for estimating emissions from modes of the LTO cycle.  The method used for estimating emissions from the cruise is described in Section 8.7, and the method used for estimating emissions from APU use is described in Section 8.6.7.

### Taxi-out and Taxi-in

The times taken for planes to taxi to and from the runways are used to calculate emissions for this part of the LTO cycle.  Data for Heathrow, Gatwick and Stansted are taken from the airport studies listed in Section 8.5.3.

The data from Heathrow and Gatwick enabled average taxi-in and taxi-out times to be calculated by aircraft type.  Aircraft are grouped into 13 classes (NATS categories) - 12 fixed wing aircraft and helicopters.

The Stansted data gave no information that would enable taxi times to be calculated by aircraft NATS category, so average values of taxi-in and taxi-out times were used.

There are no available data for taxi times for regional airports, so an estimate was made based on runway length:

Ta,m=0.1×Ra 	Equation 2

Where:

a	is the airport

m	is the mode (Taxi-in or Taxi-out)

Ra	is the length of the longest runway at airport a (m)

Ta,m	is the time in mode m (Taxi-in or Taxi-out) at airport a (s)

The coefficient (currently estimate to be equal to 0.1) was derived from Heathrow, Stansted and Gatwick data.

For small airports the taxi-in and taxi-out times were both set to 200 seconds.

We have assumed that aircraft use 7% thrust during taxing.

### Hold

The time spent holding at the runway head determines the emissions in this phase of the LTO cycle.  Any time spent at intermediate hold points (e.g. waiting to cross runways) is included in the taxi.  Data for Heathrow, Gatwick and Stansted are taken from the air airport studies listed in Section 8.5.3.

The data from Heathrow and Gatwick enabled average hold times to be calculated according to the NATS category.  The Stansted data gave an average hold time for all aircraft.

There are no available data for hold times for regional and small airports.  An approximate relationship was devised between the annual number of movements and average holding time based on data (albeit sparse) at the London airports for various years representing different levels of airport activity.  This relationship sets the average holding delay to zero for less than 100,000 atms, rising linearly to 10 minutes at 200,000 atms (on a single runway), and remaining constant for a higher number of atms.  The relationship is based on the assumption that measures would be taken to ensure average holding delays do not grow much beyond 10 minutes.

We have assumed that aircraft use 7% thrust during hold.

### Take-off roll and initial-climb

Take-off roll emissions are estimated from emission rate at take-off thrust multiplied by the take-off roll time (time from start of roll to wheels-off).  Initial-climb emissions are estimated from initial-climb time (time from wheels-off to 450 m altitude).

In general, aircraft will not take-off at 100% thrust but use reduced thrust.  However, some departures will be at 100% thrust if this is the airline policy or weather conditions or operational procedures require it.  The take-off thrust and times will vary by airport and aircraft.

The data from Heathrow and Gatwick enabled the fraction of take-offs at 100% thrust, the average reduced thrust setting, and the times in take-off roll and initial-climb to be calculated for each specific aircraft type.

The Stansted data gave no information on reduced thrust.  Therefore, the Gatwick take-off roll and initial climb data were used for Stansted.  Using Stansted times and Gatwick reduced thrust data would have been inconsistent as take-off roll and initial climb times are highly dependant upon the take-off thrust setting.  These assumptions have been applied to the regional and small airports also.

### Climb-out

Once aircraft reach an altitude of approximately 450 m they will reduce their thrust setting from take-off thrust to climb-out thrust - this is termed “throttle back”.

Climb-out emissions are estimated from the time to climb from 450 m to 1000 m multiplied by the emission rate at climb-out thrust.

In general, aircraft climb-out at 85% thrust.  However, if the assumed take-off thrust was less than 85%, the climb-out thrust was set equal to the take-off thrust.

The data from Heathrow and Gatwick enabled the fraction of climb-outs at 85% thrust, the average reduced climb-out thrust setting and the times in climb-out to be calculated for each specific aircraft type.  The Gatwick climb out times were also used for Stansted, as well as for regional and small airports

### Approach

Approach emissions are estimated from the time in approach from 1000 m multiplied by the emission rate at 30% thrust.  In line with the airport studies, the time in approach was assumed to be 286 seconds for large, medium and small aircraft, 312 seconds for light aircraft, and 390 seconds for helicopters.

### Landing-roll

In general, aircraft will land at idle thrust (7%).  However, most aircraft make use of reverse thrust (usually 30%) for some of the time.

The data from Heathrow and Gatwick gave the percentage of aircraft that use reverse thrust for each specific aircraft type.  The Gatwick landing roll thrust data have also been used for Stansted as well as regional and small airports.

The data from Heathrow, Gatwick and Stansted give the total time in landing-roll and the time in reverse thrust as well as the reverse thrust settings according to aircraft NATS category.  The regional airport study (Underwood et al., 2001a) provided equivalent data for the regional and small airports.

Again emissions are estimated from the times in idle and reverse thrust multiplied by the emission rate at appropriate thrust.

### APU

Auxiliary power units provide power for aircraft whilst they are on the ground with their main engines switched off.  The APU is also used to start the main aircraft engines.

The emissions from APUs is not a large source but one that is increasingly under scrutiny as these ground-level emissions can affect local air quality on and around the airport.  These emissions can be limited by providing external power and/or pre-conditioned air to the aircraft when they are at a stand.

The following equation gives the emissions (and fuel consumption) from APU use:

ELTOa,m,p,s=Ta,m,sFa,sIa,p,s	Equation 3

Where:

ELTOa,m,p,s	is the emissions in mode m of pollutant p for a specific aircraft type s at airport type a (kg)

a	is the airport type

m	is the mode (APU use prior to departure or APU use after arrival)

p	is the pollutant

s	is the specific aircraft type

Ta,m,s	is the time in mode m for a specific aircraft type s at airport type a (s)

Fa,s	is the fuel flow for an APU on aircraft type s at airport type a (kg s-1)

Ia,p,s	is the emission factor of pollutant p for an APU on aircraft type s at airport type a (kg / kg fuel)

The Heathrow and Gatwick studies identify the type of APU fitted to each plane.

The data from Heathrow and Gatwick enabled average APU running times to be calculated by NATS category.  The Stansted data gives one average APU running time prior to departure and one after arrival for all aircraft.  The regional airport study provided equivalent data for the regional and small airports.

## calculation of emissions in the Cruise

The methods used to estimate emissions during the cruise are described below.

### Non-greenhouse gases

The EMEP/CORINAIR Emission Inventory Guidebook (EMEP/CORINAIR, 1996) provides fuel consumption and emissions of non-GHGs (NOx, HC and CO) for a number of aircraft modes in the cruise.  The data are given for a selection of generic aircraft type and for a number of standard flight distances.

The breakdown of the CAA movement by aircraft type contains a more detailed list of aircraft types than in the EMEP/CORINAIR Emission Inventory Guidebook.  Therefore, each specific aircraft type in the CAA data has been assigned to a generic type in the Guidebook.  Details of this mapping are given in Appendix 1; see Table A1.1.

A linear regression has been applied to these data to give emissions (and fuel consumption) as a function of distance:

ECruised,g,p=mg,pd+cg,p	Equation 4

Where:

ECruised,g,p	is the emissions in cruise of pollutant p for generic aircraft type g and flight distance d (kg)

d	is the flight distance

g	is the generic aircraft type

p	is the pollutant (or fuel consumption)

mg,p	is the slope of regression for generic aircraft type g and pollutant p (kg / km)

cg,p	is the intercept of regression for generic aircraft type g and pollutant p (kg)

The regression does not go through the origin as the emissions include a climb from 1000 m to the cruising altitude and descent to 1000 m, where 1000 m is defined as the limit of the LTO cycle.

Emissions of SO2 and metals are derived from estimates of fuels consumed in the cruise (see Equation 4) multiplied by the sulphur and metals contents of the aviation fuels.

### Greenhouse gases

Estimates of CO2 were derived from estimates of fuel consumed in the cruise (see Equation 4) and the carbon contents of the aviation fuels.

The indications are that CH4 emissions are negligible at cruise altitudes, and the emission factors listed in EMEP/CORINAIR guidance are zero (EMEP/CORINAIR, 1996); we have also assumed them to be zero.  This was the assumption in the previous aviation calculation method also.

Estimates of N2O have been derived from a factor recommended by the IPCC (IPCC, 1997c) and the estimates of fuel consumed in the cruise (see Equation 4).

### Adjustment to the great circle distances

The movement data from the CAA provides great circle (shortest possible) distances between the airports.  However aircraft do not fly the shortest distances.  The IPCC report on “Aviation and the Global Atmosphere” (IPCC, 1999 – see Section 8.2.2.3. En Route and Oceanic Operations) notes that:

The existing worldwide route structure often imposes mileage penalties compared to the most economic routes (generally great-circle routes); it also takes into account wind, temperature, and other factors such as aircraft weight, charges, and safety. Use of a fixed-route network often results in concentration of traffic flows at major intersections, which can lead to a reduction in the number of routes and flight levels that are available. Studies on penalties to air traffic associated with the European ATS Route Network alone suggest that ATM-related problems add an average of about 9–10% to the flight track distance of all European flights en route and in terminal manoeuvring areas (TMA) (EUROCONTROL, 1992). Lack of international coordination in the development of ground ATC systems exacerbates these problems. Examples include inconsistent separation standards in radar and non-radar airspace and operation at less than optimum flight levels in oceanic airspace as a result of communication deficiencies.

In this study we have increased the distances travelled between airports by 9.5% for all years.  Clearly this may need to be reviewed, especially if airlines minimise distances flown in order to reduce emissions or fuel use.

### Procedures to avoid double counting of emissions for domestic flights

In order to avoid double counting of cruise emissions, the entire cruise emissions have been associated with the departure airport.

## reconciliation of Fuel use and emissions with Fuel sales estimates in DUKES

Using the Tier 3 methodology provides a means of calculating total fuel used directly from aircraft movement data.  The method is likely to lead to a different estimate of the total consumptions of aviation fuels from those presented in DTI DUKES.  (Note that the ATF fuel consumptions presented in DUKES will include use of both civil and military ATF, and the military ATF use must be subtracted from the DUKES total to provide an estimate of the civil consumption).  The fuel consumption data in DTI DUKES is provided by fuel sales return from the UK Petroleum Industries Association (UKPIA).  Ideally, the agreement between the two estimates for civil aviation fuel use should be exact, but in practice, there are reasons why this may not be the case.  Section 10.3 compares the estimates of fuel consumed with the fuel consumption data presented in DUKES, and this gives an estimate of the accuracy of the Tier 3 calculation method.  Comments are also made here about the possible reasons for any differences between the two estimates.

The estimates of fuel consumed in the commodity balance table in DUKES are the national statistics on fuel consumption, and the IPCC guidance states that national total emissions must be on the basis of fuel sales.  Therefore, the estimates of emissions have been re-normalised based on the results of the comparison (see Section 10.1).  This re-normalised of emissions will occur each time the aircraft movement data is modified or data for another year added.

## matching international/domestic split of emissions to IPCC requirements

The categorisation of aircraft movements into international and domestic movements will determine the international / domestic split in the total emission estimates.  The ideal IPCC definitions of an international and a domestic aircraft movement are explained in Section 5.5.

The UK CAA has provided the aircraft movement data for this work.  The definitions the CAA use to categorise whether a movement is international or domestic are (CAA, per. comm.)

- Domestic	a flight is domestic if the initial point on the service is a domestic and the final point is a domestic airport

- International		a flight is international if either the initial point or the final point on the service is an international airport

Take, for example, a flight (service) that travels the following route: Glasgow (within the UK) – Birmingham (within the UK) – Paris (outside the UK).  The airport reporting the aircraft movement in this example is Glasgow, and the final airport on the service is Paris.  The CAA categorises this flight as international, as the final point on the service is outside the UK.

Flights to the Channel Islands and the Isle of Man are considered to be within the UK in the CAA data.

By following the IPCC Good Practice guidance (IPCC, 2000), it is necessary to know whether passengers or freight are put down before deciding whether the whole journey is considered as an international flight or consisting of a (or several) domestic flight(s) and an international flight.  We feel the consequence of the difference between CAA and IPCC definitions will have a small impact on total emissions.

The CAA definitions above are also used by the CAA to generate national statistics of international and domestic aircraft movements.  Therefore, the aircraft movement data used in this updated aviation methodology are consistent with national statistical datasets on aircraft movements.

## Geographical coverage of the estimates of emissions

The geographical coverage of the old method used to estimate aviation emissions is explained in Section 7.5.  The coverage of the new method is exactly the same.

## accordance of the EUROSTAT aviation emissions INVENTORY with the UK GHGI

Estimates of fuel consumed in the commodity balance table in DUKES are the national statistics on fuel consumption.  The DTI supply these data to Eurostat, along with estimates of the split in domestic and international fuel consumed.  The DTI estimates of these splits have differed from the ones estimated in the UK GHG inventories (up to and including the 2002 GHGI, presented in the 2004 NIR).

A recent DTI study (Simmons, 2002) identified this inconsistency between energy data reported to Eurostat by the DTI, and data used in the GHGI.  These inconsistencies were explained by the adjustments made in the GHGI in the bunker fuel data to satisfy IPCC reporting requirements.

The DTI feel that the UK GHGI estimate of the split in domestic and international fuel consumed is likely to be more reliable that the UKPIA estimate the DTI receives.  To resolve this discrepancy between the Eurostat and the UK GHG inventories, the DTI propose to use the UK GHGI estimate of the split in domestic and international fuel consumed, and to provide these data to Eurostat.

# Comparison of emissions with the Heathrow airport inventory

Netcen has completed a number of UK airport emission inventories.  These inventories have been developed and refined over a number of years, and have been subject to considerable external scrutiny.  Therefore, as a simple check of the reliability of emissions produced by the revised GHG aviation method, we have compared selected emissions from the new GHG inventory with those predicted by the latest Heathrow inventory.  The Heathrow inventory has been chosen (rather than the Stansted or Gatwick inventories) as it represents emissions from the UK’s largest airport with greatest number of aircraft movements and aircraft types.

## London Heathrow aiport emissions inventory

Netcen created an emissions inventory for Heathrow Airport Limited (HAL) for 1998 (Underwood and Walker, 1999).  The inventory was then updated for 2002 (Underwood and Walker, 2003).  The inventories estimated the emissions of:

- oxides of nitrogen (NOx),

- carbon monoxide (CO),

- Non-Methane Volatile Organic Compounds (NMVOCs), and,

- particulate matter (the PM10 fraction of particulate matter).

## Comparison with of emissions and fuel consumptions with the Heathrow inventory

Both the Heathrow (LHR inventory) and the new GHG aviation inventory estimate emissions of NOx.  This is currently a pollutant of concern and is included in the UK National Air Quality Strategy, and so emissions of this pollutant have been compared between the two inventories.

The method for the new GHG aviation inventory is not as complex as that used in the HAL Heathrow emissions inventory, although both methods have many elements in common.  Therefore, we would not expect a perfect agreement between the emissions predicted by the two inventories.

The overall agreement of NOx emissions between the two inventories is extremely good, and suggests the revised GHG aviation methodology is producing reliable NOx emissions estimates for the LTO cycle (see results presented in Table 9.1).  As expected, there are some inter-mode differences in emission estimates between the two inventories.  Table 9.1 also shows the differences in the estimates of fuel consumed according to LTO mode and the overall agreement between the two inventories is also extremely good.

Table 9.1	Comparison of emission estimates of NOx (ktonnes) and fuel consumed (ktonnes) according to LTO mode between the LHR and revised GHG aviation inventories

|  | NOx | NOx |  |  | Fuel | Fuel |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | LHR1 | T3 model | Difference2 |  | LHR1 | T3 model | Difference2 |
|  | (ktonnes) | (ktonnes) |  |  | (ktonnes) | (ktonnes) |  |
| Taxi-out | 0.29 | 0.31 | +8.2% |  | 64.75 | 69.86 | +7.9% |
| Hold | 0.12 | 0.13 | +7.8% |  | 27.29 | 29.32 | +7.4% |
| Take-off Roll | 0.82 | 0.80 | -2.3% |  | 28.78 | 28.14 | -2.2% |
| Initial-climb | 0.98 | 0.96 | -2.1% |  | 34.93 | 34.22 | -2.0% |
| Climb-out | 1.28 | 1.24 | -2.7% |  | 47.60 | 46.70 | -1.9% |
| Approach | 0.75 | 0.74 | -1.4% |  | 73.68 | 72.32 | -1.8% |
| Landing-roll | 0.06 | 0.06 | +1.7% |  | 7.13 | 7.41 | +3.9% |
| Taxi-in | 0.16 | 0.18 | +7.6% |  | 36.90 | 39.67 | +7.5% |
| APU | 0.31 | 0.30 | -2.9% |  | 43.97 | 43.09 | -2.0% |
| Total LTO | 4.77 | 4.73 | -1.0% |  | 365.04 | 370.73 | +1.6% |

Notes

1	The London Heathrow Inventory (LHR) methodology includes data relating to modifications made the to engines of BA's Boeing 747-400 fleet during 2002 (HT-upgrade) and idle fuel flow data for the BA fleet.  These are not considered in the revised GHG aviation inventory.

2	Percentage difference relative to the LHR inventory estimates

# Estimates of emissions from UK aviation from 1990 to 2002

This chapter summarises the emissions from the revised GHG aviation method (1990 to 2002 inclusive), and compares them the emission estimates (1990 to 2001 inclusive) in the current (2002) GHG inventory.

## Time series of estimates of emissions from aviation

The following tables summarise the revised time series of emission estimates from UK aviation, and compare them to the estimating emissions.  Table 10.1 summarises the major changes in emissions according to pollutant.  A tick mark is used if the change in emissions between the old and revised GHG aviation model emission estimates is greater than 25% in any year from 1990 to 2002, and a brief explanation is provided for the change in emission estimates.

### ‘Direct’ greenhouse gases

Tables 10.2 to 10.4 summarise the emissions of the direct GHGs using revised GHG aviation model from 1990 to 2002.

### GWP weighted emissions

Each of the direct GHGs has an associated global warming potential:

- CO2	GWP	1

- CH4	GWP	21

- N2O	GWP	310

Emissions expressed as GWP weighted emissions are normally given in terms of emissions from the sum of the Kyoto ‘basket’ of six GHGs, which are: CO2, CH4, N2O and the three fluorinated gases (HFCs, PFCs, and SF6).  Only CO2, CH4, and N2O are released from aviation – the fluorinated gases have exclusively industrial sources.

Table 10.5 summarise the emissions as GWP weighted emissions.  Emissions of N2O and CH4 are small relative to CO2, and therefore even allowing for the greater GWPs of N2O and CH4 relative to CO2, the changes in GWP weighed emissions are dominated by changes in the emissions of CO2.

### ‘Indirect’ greenhouse gases

Tables 10.6 to 10.9 summarise the emissions of the indirect GHGs using revised GHG aviation model from 1990 to 2002.

## Comments on methodology

The revised methodology estimates emissions from detailed aircraft movement and route information, which splits activity by aircraft type.  Utilising this level of detail is critical to allow accurate emission estimates to be produced.  However, there are some limitations of the revised methodology and these are discussed next.

When estimating the time series emissions from the LTO cycle, the revised method applies operational data for a recent year (fleet mix, and times-in-mode) from Heathrow, Gatwick and Stansted airports to historic aircraft movement data.  In this context, the key limitations of this approach are that it:

- Ignores the potential evolution in the engine mix for a given aircraft type.  The current engine mixes are assumed to apply to relevant aircraft for all years from 1990 to 2002 inclusive.

- Ignores the evolution of times-in-mode.  As airports have grown busier, taxiing and holding times may increase.

These two factors are likely to be less important in terms of their effects on LTO emissions than accurately representing the changes in the numbers of flights of each type of aircraft.

The cruise emissions have been estimated using data for a representative number of aircraft types presented in the CORINAIR workbook (as recommended in the IPCC guidelines).  The number of aircraft types is not extensive, although the revised method allows updated UK-specific cruise emission-factor versus sector length curves to be substituted for the CORINAR defaults as they become available.  The revised method for estimating emissions from the cruise is a considerable improvement over the original one.

Table 10.1	Summary of major changes in emissions according to pollutant and flight cycle.
(A star symbol  is used if the change in emission estimates is greater than 25%, in any year, between the old and revised estimates from 1990 to 2002.   indicates an increase (all years 1990 to 2002) in emissions using the new aviation model over the original estimates; indicates a decrease (all years 1990 to 2002) in emissions using the new aviation model over the original estimates;  indicates increases in emissions in some years and decreases in others years.)

| Pollutant | Domestic aviation | Reason | International aviation | Reason |
| --- | --- | --- | --- | --- |
| ‘Direct’ greenhouse gases |  |  |  |  |
| CO2 |   | - Better estimates of domestic / international fuel split from the new model – domestic emissions have declined by the same amount as international emissions have increased |  |  |
| CH4 |   |  |   | - Better estimates of domestic / international fuel split from the new model; LTO EFs revised |
| N20 |   | - Better estimates of domestic / international fuel split from the new model; LTO EFs revised |  |  |
| GWP emissions |   | - Better estimates of domestic / international fuel split from the new model; LTO EFs revised; change in GWP dominated by changes in CO2 emissions |  | - Better estimates of domestic / international fuel split from the new model; LTO EFs revised; change in GWP dominated by changes in CO2 emissions |
| ‘Indirect’ greenhouse gases |  |  |  |  |
| CO |   | - Better estimates of domestic / international fuel split from the new model; LTO EFs revised |   | - Better estimates of domestic / international fuel split from the new model; LTO EFs revised |
| NMVOC |   |  |   | - Better estimates of domestic / international fuel split from the new model; LTO EFs revised |
| NOx |   | - Better estimates of domestic / international fuel split from the new model; LTO EFs revised |   |  |
| SO2 |   | - Better estimates of domestic / international fuel split from the new model – domestic emissions have declined by the same amount as international emissions have increased |  |  |

Notes

- EF – Emission Factor

Table 10.2	Emissions of CO2 from the original and revised aviation estimation methodologies (ktonnes)
(Emissions from the revised model are in italics; domestic emissions, differences and percentages are in shaded cells)

| Year | Flight cycle stage | Domestic emissions Revised GHG aviation method | International emissions Revised GHG aviation method | Domestic emissions Original method | International emissions Original method | Domestic difference (revised method – original method) | International difference (revised method – original method) | Domestic % change (see note at end of table) | International % change (see note at end of table) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (ktonnes) | (ktonnes) | (ktonnes) | (ktonnes) | (ktonnes) | (ktonnes) |  |  |
| 1990 | Total | 1,280.82 | 15,666.08 | 2,158.43 | 14,790.50 | -877.60 | 875.58 | -40.7% | 5.9% |
| 1991 | Total | 1,252.62 | 15,438.94 | 2,124.80 | 14,565.81 | -872.18 | 873.12 | -41.0% | 6.0% |
| 1992 | Total | 1,259.97 | 17,079.37 | 2,219.58 | 16,121.52 | -959.61 | 957.85 | -43.2% | 5.9% |
| 1993 | Total | 1,338.91 | 18,182.72 | 2,281.12 | 17,241.11 | -942.20 | 941.61 | -41.3% | 5.5% |
| 1994 | Total | 1,254.60 | 18,927.20 | 2,253.94 | 17,928.55 | -999.34 | 998.66 | -44.3% | 5.6% |
| 1995 | Total | 1,331.70 | 20,127.61 | 2,380.15 | 19,079.88 | -1048.46 | 1047.73 | -44.1% | 5.5% |
| 1996 | Total | 1,447.12 | 21,340.09 | 2,486.97 | 20,301.08 | -1039.86 | 1039.02 | -41.8% | 5.1% |
| 1997 | Total | 1,500.93 | 22,691.66 | 2,571.79 | 21,621.61 | -1070.86 | 1070.04 | -41.6% | 4.9% |
| 1998 | Total | 1,632.92 | 25,252.63 | 2,688.53 | 24,197.81 | -1055.61 | 1054.82 | -39.3% | 4.4% |
| 1999 | Total | 1,807.35 | 27,438.19 | 2,752.85 | 26,493.68 | -945.50 | 944.51 | -34.3% | 3.6% |
| 2000 | Total | 1,971.66 | 30,241.56 | 2,801.38 | 29,412.07 | -829.72 | 829.50 | -29.6% | 2.8% |
| 2001 | Total | 2,007.35 | 28,541.42 | 2,937.76 | 27,612.82 | -930.41 | 928.60 | -31.7% | 3.4% |
| 2002 | Total | 2,012.16 | 27,836.30 | 2,921.20 | 26,926.50 | -909.04 | 909.80 | -31.1% | 3.4% |

Notes

- Flight cycle stage – emissions are given for the total cycle i.e. the sum of the LTO cycle and the cruise

- The percentage changes are quoted for the differences between the revised aviation estimation method and the original method relative to the original aviation method

Table 10.3	Emissions of CH4 from the original and revised aviation estimation methodologies (ktonnes) 
(Emissions from the revised model are in italics; domestic emissions, differences and percentages are in shaded cells)

| Year | Flight cycle stage | Domestic emissions Revised GHG aviation method | International emissions Revised GHG aviation method | Domestic emissions Original method | International emissions Original method | Domestic difference (revised method – original method) | International difference (revised method – original method) | Domestic % change (see note at end of table) | International % change (see note at end of table) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (ktonnes) | (ktonnes) | (ktonnes) | (ktonnes) | (ktonnes) | (ktonnes) |  |  |
| 1990 | Total | 0.12 | 0.28 | 0.12 | 2.85 | 0.005 | -2.57 | 3.9% | -90.3% |
| 1991 | Total | 0.12 | 0.22 | 0.11 | 2.75 | 0.002 | -2.52 | 1.8% | -91.9% |
| 1992 | Total | 0.12 | 0.21 | 0.12 | 2.98 | 0.003 | -2.77 | 2.8% | -93.0% |
| 1993 | Total | 0.12 | 0.21 | 0.12 | 3.06 | 0.006 | -2.85 | 4.8% | -93.1% |
| 1994 | Total | 0.11 | 0.20 | 0.11 | 3.18 | -0.003 | -2.99 | -2.6% | -93.8% |
| 1995 | Total | 0.11 | 0.18 | 0.12 | 3.31 | -0.013 | -3.12 | -11.1% | -94.4% |
| 1996 | Total | 0.11 | 0.18 | 0.12 | 3.49 | -0.015 | -3.31 | -11.9% | -94.7% |
| 1997 | Total | 0.12 | 0.18 | 0.13 | 3.71 | -0.010 | -3.53 | -7.9% | -95.2% |
| 1998 | Total | 0.11 | 0.18 | 0.13 | 3.98 | -0.018 | -3.81 | -13.9% | -95.6% |
| 1999 | Total | 0.13 | 0.16 | 0.14 | 4.22 | -0.004 | -4.06 | -3.3% | -96.2% |
| 2000 | Total | 0.14 | 0.14 | 0.14 | 4.47 | 0.001 | -4.33 | 0.5% | -96.9% |
| 2001 | Total | 0.14 | 0.11 | 0.14 | 4.53 | -0.005 | -4.42 | -3.1% | -97.5% |
| 2002 | Total | 0.12 | 0.11 | 0.14 | 4.50 | -0.020 | -4.39 | -14.1% | -97.6% |

Notes

- Flight cycle stage – emissions are given for the total cycle i.e. the sum of the LTO cycle and the cruise

- The percentage changes are quoted for the differences between the revised aviation estimation method and the original method relative to the original aviation method

Table 10.4	Emissions of N2O from the original and revised aviation estimation methodologies (ktonnes) 
(Emissions from the revised model are in italics; domestic emissions, differences and percentages are in shaded cells)

| Year | Flight cycle stage | Domestic emissions Revised GHG aviation method | International emissions Revised GHG aviation method | Domestic emissions Original method | International emissions Original method | Domestic difference (revised method – original method) | International difference (revised method – original method) | Domestic % change (see note at end of table) | International % change (see note at end of table) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (ktonnes) | (ktonnes) | (ktonnes) | (ktonnes) | (ktonnes) | (ktonnes) |  |  |
| 1990 | Total | 0.04 | 0.50 | 0.069 | 0.453 | -0.03 | 0.04 | -40.6% | 9.7% |
| 1991 | Total | 0.04 | 0.49 | 0.067 | 0.447 | -0.03 | 0.04 | -41.0% | 9.7% |
| 1992 | Total | 0.04 | 0.54 | 0.070 | 0.495 | -0.03 | 0.05 | -43.2% | 9.6% |
| 1993 | Total | 0.04 | 0.58 | 0.072 | 0.530 | -0.03 | 0.05 | -41.3% | 9.0% |
| 1994 | Total | 0.04 | 0.60 | 0.072 | 0.551 | -0.03 | 0.05 | -44.3% | 9.1% |
| 1995 | Total | 0.04 | 0.64 | 0.076 | 0.587 | -0.03 | 0.05 | -44.0% | 8.9% |
| 1996 | Total | 0.05 | 0.68 | 0.079 | 0.624 | -0.03 | 0.05 | -41.8% | 8.5% |
| 1997 | Total | 0.05 | 0.72 | 0.082 | 0.665 | -0.03 | 0.06 | -41.6% | 8.3% |
| 1998 | Total | 0.05 | 0.80 | 0.085 | 0.745 | -0.03 | 0.06 | -39.2% | 7.6% |
| 1999 | Total | 0.06 | 0.87 | 0.087 | 0.817 | -0.03 | 0.05 | -34.3% | 6.6% |
| 2000 | Total | 0.06 | 0.96 | 0.089 | 0.908 | -0.03 | 0.05 | -29.6% | 5.7% |
| 2001 | Total | 0.06 | 0.91 | 0.093 | 0.851 | -0.03 | 0.06 | -31.6% | 6.5% |
| 2002 | Total | 0.06 | 0.88 | 0.093 | 0.829 | -0.03 | 0.05 | -31.1% | 6.6% |

Notes

- Flight cycle stage – emissions are given for the total cycle i.e. the sum of the LTO cycle and the cruise

- The percentage changes are quoted for the differences between the revised aviation estimation method and the original method relative to the original aviation method

Table 10.5	Change in GWP weighted emissions (includes contributions from CO2, CH4 and N2O - F-gases are not emitted from aviation) 
(Emissions from the revised model are in italics; domestic emissions, differences and percentages are in shaded cells)

| Year | Flight cycle stage | Domestic emissions Revised GHG aviation method | International emissions Revised GHG aviation method | Domestic emissions Original method | International emissions Original method | Domestic difference (revised method – original method) | International difference (revised method – original method) | Domestic % change (see note at end of table) | International % change (see note at end of table) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (ktonnes) | (ktonnes) | (ktonnes) | (ktonnes) | (ktonnes) | (ktonnes) |  |  |
| 1990 | Total | 1,296.01 | 15,826.06 | 2,182.15 | 14,990.86 | -886.14 | 835.20 | -40.6% | 5.6% |
| 1991 | Total | 1,267.39 | 15,595.55 | 2,148.10 | 14,761.94 | -880.71 | 833.61 | -41.0% | 5.6% |
| 1992 | Total | 1,274.88 | 17,251.89 | 2,243.86 | 16,337.51 | -968.99 | 914.38 | -43.2% | 5.6% |
| 1993 | Total | 1,354.71 | 18,366.14 | 2,306.06 | 17,469.59 | -951.35 | 896.55 | -41.3% | 5.1% |
| 1994 | Total | 1,269.24 | 19,117.63 | 2,278.47 | 18,166.20 | -1009.23 | 951.43 | -44.3% | 5.2% |
| 1995 | Total | 1,347.01 | 20,329.59 | 2,406.05 | 19,331.28 | -1059.04 | 998.31 | -44.0% | 5.2% |
| 1996 | Total | 1,463.64 | 21,553.99 | 2,514.03 | 20,568.03 | -1050.39 | 985.96 | -41.8% | 4.8% |
| 1997 | Total | 1,518.13 | 22,918.71 | 2,599.73 | 21,905.71 | -1081.60 | 1013.00 | -41.6% | 4.6% |
| 1998 | Total | 1,651.35 | 25,504.88 | 2,717.72 | 24,512.55 | -1066.37 | 992.33 | -39.2% | 4.0% |
| 1999 | Total | 1,827.88 | 27,711.64 | 2,782.77 | 26,835.56 | -954.89 | 876.08 | -34.3% | 3.3% |
| 2000 | Total | 1,993.97 | 30,542.12 | 2,831.83 | 29,787.49 | -837.85 | 754.63 | -29.6% | 2.5% |
| 2001 | Total | 2,030.02 | 28,824.70 | 2,969.67 | 27,971.67 | -939.65 | 853.03 | -31.6% | 3.0% |
| 2002 | Total | 2,034.54 | 28,112.54 | 2,952.94 | 27,278.13 | -918.40 | 834.41 | -31.1% | 3.1% |

Notes

- Flight cycle stage – emissions are given for the total cycle i.e. the sum of the LTO cycle and the cruise

- The percentage changes are quoted for the differences between the revised aviation estimation method and the original method relative to the original aviation method

Table 10.6	Emissions of CO from the original and revised aviation estimation methodologies (ktonnes) 
(Emissions from the revised model are in italics; domestic emissions, differences and percentages are in shaded cells)

| Year | Flight cycle stage | Domestic emissions Revised GHG aviation method | International emissions Revised GHG aviation method | Domestic emissions Original method | International emissions Original method | Domestic difference (revised method – original method) | International difference (revised method – original method) | Domestic % change (see note at end of table) | International % change (see note at end of table) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (ktonnes) | (ktonnes) | (ktonnes) | (ktonnes) | (ktonnes) | (ktonnes) |  |  |
| 1990 | Total | 30.68 | 12.85 | 7.77 | 60.06 | 22.90 | -47.21 | 294.7% | -78.6% |
| 1991 | Total | 27.99 | 11.96 | 7.59 | 58.35 | 20.40 | -46.39 | 268.7% | -79.5% |
| 1992 | Total | 28.02 | 12.76 | 7.85 | 63.86 | 20.17 | -51.10 | 256.9% | -80.0% |
| 1993 | Total | 28.40 | 13.67 | 8.06 | 66.62 | 20.34 | -52.95 | 252.4% | -79.5% |
| 1994 | Total | 31.15 | 13.41 | 7.83 | 69.32 | 23.32 | -55.91 | 297.9% | -80.7% |
| 1995 | Total | 30.76 | 13.85 | 8.26 | 72.75 | 22.50 | -58.90 | 272.5% | -81.0% |
| 1996 | Total | 34.41 | 14.53 | 8.62 | 77.06 | 25.79 | -62.52 | 299.0% | -81.1% |
| 1997 | Total | 39.03 | 15.02 | 8.87 | 81.92 | 30.15 | -66.90 | 339.9% | -81.7% |
| 1998 | Total | 39.43 | 16.29 | 9.25 | 89.54 | 30.18 | -73.24 | 326.3% | -81.8% |
| 1999 | Total | 47.16 | 17.27 | 9.51 | 96.22 | 37.65 | -78.95 | 395.8% | -82.1% |
| 2000 | Total | 54.18 | 18.34 | 9.69 | 104.07 | 44.49 | -85.73 | 459.2% | -82.4% |
| 2001 | Total | 59.01 | 16.85 | 10.13 | 101.97 | 48.88 | -85.11 | 482.6% | -83.5% |
| 2002 | Total | 53.13 | 16.45 | 10.08 | 100.52 | 43.04 | -84.07 | 426.9% | -83.6% |

Notes

- Flight cycle stage – emissions are given for the total cycle i.e. the sum of the LTO cycle and the cruise

- The percentage changes are quoted for the differences between the revised aviation estimation method and the original method relative to the original aviation method

Table 10.7	Emissions of NMVOC from the original and revised aviation estimation methodologies (ktonnes) 
(Emissions from the revised model are in italics; domestic emissions, differences and percentages are in shaded cells)

| Year | Flight cycle stage | Domestic emissions Revised GHG aviation method | International emissions Revised GHG aviation method | Domestic emissions Original method | International emissions Original method | Domestic difference (revised method – original method) | International difference (revised method – original method) | Domestic % change (see note at end of table) | International % change (see note at end of table) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (ktonnes) | (ktonnes) | (ktonnes) | (ktonnes) | (ktonnes) | (ktonnes) |  |  |
| 1990 | Total | 1.52 | 4.78 | 1.38 | 36.87 | 0.13 | -32.09 | 9.7% | -87.0% |
| 1991 | Total | 1.46 | 4.14 | 1.34 | 35.79 | 0.12 | -31.65 | 8.9% | -88.4% |
| 1992 | Total | 1.43 | 4.16 | 1.38 | 39.13 | 0.05 | -34.97 | 3.9% | -89.4% |
| 1993 | Total | 1.48 | 4.37 | 1.41 | 40.74 | 0.07 | -36.37 | 4.6% | -89.3% |
| 1994 | Total | 1.24 | 4.27 | 1.36 | 42.39 | -0.12 | -38.12 | -8.5% | -89.9% |
| 1995 | Total | 1.20 | 4.33 | 1.43 | 44.44 | -0.23 | -40.11 | -16.0% | -90.2% |
| 1996 | Total | 1.24 | 4.56 | 1.49 | 47.05 | -0.25 | -42.49 | -17.0% | -90.3% |
| 1997 | Total | 1.31 | 4.73 | 1.53 | 50.01 | -0.22 | -45.28 | -14.1% | -90.5% |
| 1998 | Total | 1.30 | 5.10 | 1.59 | 54.56 | -0.29 | -49.45 | -18.5% | -90.6% |
| 1999 | Total | 1.46 | 5.36 | 1.64 | 58.53 | -0.18 | -53.17 | -11.3% | -90.8% |
| 2000 | Total | 1.54 | 5.61 | 1.67 | 63.16 | -0.14 | -57.55 | -8.3% | -91.1% |
| 2001 | Total | 1.54 | 5.17 | 1.75 | 62.12 | -0.20 | -56.95 | -11.7% | -91.7% |
| 2002 | Total | 1.40 | 5.15 | 1.74 | 61.29 | -0.34 | -56.14 | -19.3% | -91.6% |

Notes

- Flight cycle stage – emissions are given for the total cycle i.e. the sum of the LTO cycle and the cruise

- The percentage changes are quoted for the differences between the revised aviation estimation method and the original method relative to the original aviation method

Table 10.8	Emissions of NOx (as NO2) from the original and revised aviation estimation methodologies (ktonnes) 
(Emissions from the revised model are in italics; domestic emissions, differences and percentages are in shaded cells)

| Year | Flight cycle stage | Domestic emissions Revised GHG aviation method | International emissions Revised GHG aviation method | Domestic emissions Original method | International emissions Original method | Domestic difference (revised method – original method) | International difference (revised method – original method) | Domestic % change (see note at end of table) | International % change (see note at end of table) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (ktonnes) | (ktonnes) | (ktonnes) | (ktonnes) | (ktonnes) | (ktonnes) |  |  |
| 1990 | Total | 4.48 | 75.64 | 6.93 | 72.79 | -2.46 | 2.86 | -35.4% | 3.9% |
| 1991 | Total | 4.42 | 73.16 | 6.84 | 71.83 | -2.42 | 1.33 | -35.4% | 1.8% |
| 1992 | Total | 4.44 | 80.12 | 7.16 | 79.64 | -2.71 | 0.48 | -37.9% | 0.6% |
| 1993 | Total | 4.51 | 85.15 | 7.36 | 85.50 | -2.85 | -0.34 | -38.7% | -0.4% |
| 1994 | Total | 4.17 | 88.58 | 7.30 | 88.90 | -3.13 | -0.32 | -42.8% | -0.4% |
| 1995 | Total | 4.45 | 93.94 | 7.71 | 94.80 | -3.26 | -0.86 | -42.3% | -0.9% |
| 1996 | Total | 4.85 | 100.12 | 8.06 | 100.94 | -3.21 | -0.82 | -39.8% | -0.8% |
| 1997 | Total | 4.97 | 106.36 | 8.34 | 107.53 | -3.37 | -1.18 | -40.4% | -1.1% |
| 1998 | Total | 5.60 | 117.90 | 8.72 | 120.76 | -3.12 | -2.86 | -35.8% | -2.4% |
| 1999 | Total | 6.42 | 126.57 | 8.92 | 132.57 | -2.51 | -6.00 | -28.1% | -4.5% |
| 2000 | Total | 7.18 | 138.22 | 9.08 | 147.70 | -1.90 | -9.48 | -20.9% | -6.4% |
| 2001 | Total | 7.20 | 129.63 | 9.53 | 137.84 | -2.32 | -8.21 | -24.4% | -6.0% |
| 2002 | Total | 7.03 | 125.50 | 9.47 | 134.20 | -2.44 | -8.70 | -25.8% | -6.5% |

Notes

- Flight cycle stage – emissions are given for the total cycle i.e. the sum of the LTO cycle and the cruise

- The percentage changes are quoted for the differences between the revised aviation estimation method and the original method relative to the original aviation method

Table 10.9	Emissions of SO2 from the original and revised aviation estimation methodologies (ktonnes) 
(Emissions from the revised model are in italics; domestic emissions, differences and percentages are in shaded cells)

| Year | Flight cycle stage | Domestic emissions Revised GHG aviation method | International emissions Revised GHG aviation method | Domestic emissions Original method | International emissions Original method | Domestic difference (revised method – original method) | International difference (revised method – original method) | Domestic % change (see note at end of table) | International % change (see note at end of table) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (ktonnes) | (ktonnes) | (ktonnes) | (ktonnes) | (ktonnes) | (ktonnes) |  |  |
| 1990 | Total | 0.24 | 2.98 | 0.41 | 2.82 | -0.17 | 0.17 | -40.6% | 5.9% |
| 1991 | Total | 0.32 | 3.92 | 0.54 | 3.70 | -0.22 | 0.22 | -41.0% | 6.0% |
| 1992 | Total | 0.40 | 5.42 | 0.70 | 5.12 | -0.30 | 0.30 | -43.2% | 5.9% |
| 1993 | Total | 0.34 | 4.62 | 0.58 | 4.38 | -0.24 | 0.24 | -41.3% | 5.5% |
| 1994 | Total | 0.40 | 6.01 | 0.72 | 5.69 | -0.32 | 0.32 | -44.3% | 5.6% |
| 1995 | Total | 0.34 | 5.11 | 0.60 | 4.85 | -0.27 | 0.27 | -44.0% | 5.5% |
| 1996 | Total | 0.37 | 5.42 | 0.63 | 5.16 | -0.26 | 0.26 | -41.8% | 5.1% |
| 1997 | Total | 0.48 | 7.20 | 0.82 | 6.86 | -0.34 | 0.34 | -41.6% | 4.9% |
| 1998 | Total | 0.52 | 8.02 | 0.85 | 7.68 | -0.33 | 0.33 | -39.2% | 4.4% |
| 1999 | Total | 0.40 | 6.10 | 0.61 | 5.89 | -0.21 | 0.21 | -34.3% | 3.6% |
| 2000 | Total | 0.45 | 6.91 | 0.64 | 6.72 | -0.19 | 0.19 | -29.6% | 2.8% |
| 2001 | Total | 0.51 | 7.25 | 0.75 | 7.01 | -0.24 | 0.24 | -31.6% | 3.4% |
| 2002 | Total | 0.42 | 5.83 | 0.61 | 5.64 | -0.19 | 0.19 | -31.1% | 3.4% |

Notes

- Flight cycle stage – emissions are given for the total cycle i.e. the sum of the LTO cycle and the cruise

- The percentage changes are quoted for the differences between the revised aviation estimation method and the original method relative to the original aviation method

## Fuel reconciliation

The estimates of aviation fuels consumed in the commodity balance table in DUKES are the national statistics on fuel consumption, and IPCC guidance states that national total emissions must be on the basis of fuel sales.  Therefore, the estimates of emissions have been re-normalised based on the results of the comparison (see Section 8.8).  The ATF fuel consumptions presented in DTI DUKES include the use of both civil and military ATF, and the military ATF use must be subtracted from the DUKES total to provide an estimate of the civil aviation consumption.  This estimate of civil ATF consumption has been used in the fuel reconciliation.  Emissions will be re-normalised each time the aircraft movement data is modified or data for another year added.

The data in Table 10.10 shows a comparison of the estimated consumptions of ATF and AS from the new GHG aviation model to those in presented in DTI DUKES.  The agreement between the estimate of civil ATF consumption produced by the revised GHG aviation model and the data presented in DTI DUKES is very good.  However, the new GHG aviation model only accounts for a small fraction of aviation spirit use.  A large fraction of the missing aviation spirit will be used at small airfields – aircraft movements associated with this fuel will not be reported to the CAA, and so the emissions will not have been estimated from these aircraft movements from our model.  However, the aviation spirit used has been reported in DTI DUKES.  Since the aviation spirit is mostly being used at small airports, it was therefore deemed appropriate to scale up only the LTO cycle emissions to account for the missing aviation spirit since most of the fuel will be used at low altitude.

The fuel consumption data in DTI DUKES are provided by fuel sales returns from the UK Petroleum Industries Association (UKPIA).  Ideally, the agreement between the estimate for civil aviation fuel use, produced by the revised aviation model, and that presented in DUKES should be exact, but in practice, there are reasons why this may not be the case.  There are two different types of contributions to potential discrepancies:

- Inaccuracies within the new aviation methodology – for example, the engine fuel flow rates assumed, the distances aircraft have flown etc.  These limitations would apply to almost any method of similar complexity used to estimate emissions, and are not special limitations of the current method.

- The two quantities of fuel being compared are not quantifying the same thing.  For example, if there is a substantial amount of tankering occurring and it was asymmetric between the UK and other countries then, the agreement between the two estimates of fuel use could not be perfect.

Table 10.10	Comparison of the emissions of estimates of ATF and AS consumptions produced by the new GHG aviation model to those in DTI DUKES

| Year | DUKES | DUKES | Military | Civil ATF estimate |  | New GHG aviation model | New GHG aviation model |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AS | ATF | ATF | ATF |  | AS | ATF | AS | ATF |
|  |  | (total of military and civil) |  | (DUKES – military) |  |  |  | Percent of AS accounted for by new GHG aviation model | Percent of civil ATF use accounted for by new GHG aviation model |
|  | (ktonnes) | (ktonnes) | (ktonnes) | ktonnes) |  | (ktonnes) | (ktonnes) |  |  |
| 1990 | 26 | 6,589 | 1,234 | 5,355 |  | 1.84 | 5,164 | 7.1% | 96.4% |
| 1991 | 24 | 6,176 | 901 | 5,275 |  | 1.53 | 5,024 | 6.4% | 95.2% |
| 1992 | 27 | 6,666 | 870 | 5,796 |  | 1.79 | 5,534 | 6.6% | 95.5% |
| 1993 | 27 | 7,106 | 935 | 6,171 |  | 2.13 | 5,502 | 7.9% | 89.2% |
| 1994 | 29 | 7,284 | 905 | 6,379 |  | 1.34 | 6,136 | 4.6% | 96.2% |
| 1995 | 29 | 7,660 | 876 | 6,784 |  | 1.19 | 6,407 | 4.1% | 94.4% |
| 1996 | 32 | 8,049 | 846 | 7,203 |  | 1.13 | 6,802 | 3.5% | 94.4% |
| 1997 | 37 | 8,411 | 767 | 7,644 |  | 1.05 | 7,376 | 2.8% | 96.5% |
| 1998 | 36 | 9,241 | 741 | 8,500 |  | 0.98 | 8,043 | 2.7% | 94.6% |
| 1999 | 45 | 9,939 | 699 | 9,240 |  | 0.97 | 8,476 | 2.1% | 91.7% |
| 2000 | 52 | 10,806 | 631 | 10,175 |  | 0.86 | 8,819 | 1.6% | 86.7% |
| 2001 | 59 | 10,297 | 657 | 9,640 |  | 0.77 | 8,834 | 1.3% | 91.6% |
| 2002 | 52 | 10,148 | 724 | 9,424 |  | 0.80 | 8,677 | 1.5% | 92.1% |

Notes

- ATF – Aviation Turbine Fuel

- AS – Aviation Spirit

- Estimates of military ATF use are provided by the UK Ministry of Defence

- The estimates of ATF and AS consumptions in DTI DUKES for 2002, 2001 and 2000 were taken from DUKES (2003).  Data for previous years have been taken from the latest revised data in earlier editions of DUKES

# Uncertainty analysis

Estimates of uncertainty on the emissions are produced for both the greenhouse gas inventory and the National Atmospheric Emissions Inventory.  In the light of this work, the estimates of uncertainty on both the activity data and emission factors will be reviewed, and if necessary, revised.

This work is currently still in progress (as of June 2004).

# Acknowledgements

We are grateful for the help of the following people during the course of this work programme; apologies for any omissions.

- Brian Underwood (Netcen aviation team; advice and support throughout the project)

- Martin Peirce (Netcen aviation team; acquisition of aircraft movement data from BAA, and development work on the aviation emissions database)

- Andy Lelland (Netcen aviation team; development work on the aviation emissions database)

- Claire Thomson (Netcen; initial work on the aviation database)

- Justin Goodwin (Netcen NAEI Project Director; data extraction queries from the NAEI database)

- Chris Dore (Netcen NAEI Project Manager; review of a late draft of this report)

- Roger Gardner (DfT) and Peter Newton (DTI) (for comments on this work and suggestions for improvement made during and after the DfT consultation in January 2004, DfT, London)

The aircraft movement data for 1990 to 2002 was provided by the UK CAA.

# References

Baggott, S L,  Brown, L, Milne, R, Murrells, T P, Passant, N and Watterson, J D. (2004) UK greenhouse gas inventory, 1990 to 2002.  Annual report for submission under the Framework Convention in Climate Change.  ISBN 0-9547136-2-1.  National Environmental Technology Centre (NETCEN), Culham Science Centre, Abingdon, Oxfordshire, OX14 3ED, UK.

CAA (per. comm., 2004) Civil Aviation Authority.  Telephone conversation with Debbie McLean.

Dopelheuer, A and Lecht, M (1999) Influence of engine performance on emission characteristics. RTO Meeting Proceedings 14 Gas Turbine Engine Combustion, Emissions and Alternative Fuels. RTO-MP-14 AC/323 (AVT)TP/10. NATO Research and Technology Organization.

- DTI, (2003), Digest of United Kingdom Energy Statistics 2003, London, The Stationary Office, London.

DTLR (2002b), “Transport Statistics Great Britain: 2002 edition”, The Stationary Office.

EEA (2000) UNECE/EMEP/EEA The emission inventory Guidebook.  Snap codes 080501-04 Air traffic.

EMEP/CORINAIR (1996) Atmospheric Emission Inventory Guidebook, 1st Edition, ed. G McInnes.

EUROCONTROL, 1992: “Penalties to Air Traffic Associated with the ATS Route Network in the Continental ECAC States Area”. Document no. 921016, EUROCONTROL, Brussels, Belgium, 26 pp. + appendices.

ICAO (1993) International Standards and Recommended Practices, Environmental Protection Annex 16, Volume II Aircraft Engine Emissions (second ed.) ICAO, 1993.

IPCC (1999) IPCC Special Report - Aviation And The Global Atmosphere.  A Special Report of IPCC Working Groups I and II in collaboration with the Scientific Assessment Panel to the Montreal Protocol on Substances that Deplete the Ozone Layer.  Published for the Intergovernmental Panel on Climate Change.  Cambridge University Press, CB2 2RU, UK.

- IPCC, (1997a), IPCC Revised 1996 Guidelines for National Greenhouse Gas Inventories, Volume 1, Greenhouse Gas Inventory Reporting Instructions, IPCC WGI Technical Support Unit, Hadley Centre, Meteorological Office, Bracknell, UK.

- IPCC, (1997b), IPCC Revised 1996 Guidelines for National Greenhouse Gas Inventories, Volume 2, Greenhouse Gas Inventory Workbook, IPCC WGI Technical Support Unit, Hadley Centre, Meteorological Office, Bracknell, UK.

- IPCC, (1997c), IPCC Revised 1996 Guidelines for National Greenhouse Gas Inventories, Volume 3, Greenhouse Gas Inventory Reference Manual, IPCC WGI Technical Support Unit, Hadley Centre, Meteorological Office, Bracknell, UK.

IPCC, (2000), Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories, ed. Penman, J , Kruger, D, Galbally, I, et al, IPCC National Greenhouse Gas Inventories Programme, Technical Support Programme Technical Support Unit, Institute for Global Environmental Strategies, Hayama, Kanagawa, Japan.

Lock, Graham (Eurostat) (2003).  EU experience in the estimation of aviation fuel consumption and emissions using air traffic data.  SMI-WP/2.  Paper prepared for the ICAO-UNFCCC Scoping Meeting on Aircraft Emissions Inventories, Montreal, 27 – 28, February, 2003.

ONS, (1995), UK Defence Statistics, 1994, Office for National Statistics

Rypdal, Kristin (Statistics Norway) (2000).  Aircraft Emissions.  Background paper to Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories.

Simmons, T (2003) Peer Review of the UK Greenhouse Gas Inventory - CO2 from Fuel Combustion.  Avonlog Ltd., 49 Headley Chase, Brentwood, Essex, CM14 5DH.

Simmons, T (2002) Energy Data Harmonisation (a project undertaken for the UK DTI, Avonlog Ltd – a study done for Rachael Winther at UK DTI).  Avonlog Ltd., 49 Headley Chase, Brentwood, Essex, CM14 5DH.

Spicer, C. W., et al. (1984), Battelle Columbus Laboratories, Composition and Photochemical Reactivity of Turbine Engine Exhaust, Report No. ESL-TR-84-28.  Prepared for Air Force Engineering and Services Center (RDVS), Tyndall AFB,  FL, September 1984

Underwood et al, 2001a, “Air Quality at UK Regional Airports: A report produced for DETR” B Y Underwood, S M Brightwell, J MacKenzie, M J Peirce and C T Walker, AEA Technology Report, AEAT/R/ENV/0322, UK, June 2001

Underwood et al, 2001b, “Air Quality Assessment of BAA Stansted Airport Development: A report produced for Stansted Airport Ltd” B Y Underwood and C T Walker, T Stebbings and M S Pratt, AEA Technology Report, AEAT/ENV/R/0558, UK, July 2001

Underwood et al, 2003, “Heathrow Emission Inventory 2000, A report produced for BAA Heathrow” B Y Underwood and C T Walker, AEA Technology Report, netcen/AEAT/ENV/R/1307/Issue 1, UK, June 2003

Underwood, B and Walker, C. (1999) Heathrow Emission Inventory 1998.  A report produced for Heathrow Airport Ltd.  AEAT-5568.  AEA Technology plc.  Thomson House, Warrington Rd., Risley, Warrington, Cheshire, WA3 6AT, UK.

Underwood, B. and Walker, C. (2003) Heathrow Emission Inventory 2002.  AEA Technology plc.  A report produced for BAA plc.  Thomson House, Warrington Rd., Risley, Warrington, Cheshire, WA3 6AT, UK.

Appendices

CONTENTS

| Appendix 1 | Matching of specific and generic aircraft types |
| --- | --- |
| Appendix 2 | Times in mode |
| Appendix 3 | Emission factors and fuel flow rates |
| Appendix 4 | Consultation with the DfT |

Appendix 1 Matching of specific and generic aircraft types

CONTENTS

| Table A1.1 | Matching of specific aircraft types in the CAA data to generic EMEP/CORINAIR aircraft types |
| --- | --- |

Table A1.1	Matching of specific aircraft types in the CAA data to generic EMEP/CORINAIR aircraft types

| EMEP/CORINAIR Aircraft Type | CAA Aircraft Type | Number of Engines | NATS Category | Fuel | APU |
| --- | --- | --- | --- | --- | --- |
| A310 | AIRBUS A300-600 | 2 | Group 2 | ATF | 331-200ER |
|  | AIRBUS A300B1/B2 | 2 | Group 2 | ATF | 700-4B |
|  | AIRBUS A300B4-100/200 | 2 | Group 2 | ATF | 700-4B |
|  | AIRBUS A300F4 | 2 | Group 2 | ATF | 700-4B |
|  | AIRBUS A310-202 | 2 | Group 2 | ATF | 331-200ER |
|  | AIRBUS A310-300 | 2 | Group 2 | ATF | 331-200ER |
| A320 | AIRBUS A319 | 2 | Group 5 | ATF | 36-300 |
|  | AIRBUS A320-100/200 | 2 | Group 5 | ATF | 36-300 |
|  | AIRBUS A321 | 2 | Group 5 | ATF | 36-300 |
|  | BOEING 737-800 | 2 | Group 5 | ATF | 131-9 |
|  | BOEING 737-900 | 2 | Group 5 | ATF | 131-9 |
| A330 | AIRBUS A330-200 | 2 | Group 1 | ATF | 331-350 |
|  | AIRBUS A330-300 | 2 | Group 1 | ATF | 331-350 |
| A340 | AIRBUS A340-200 | 4 | Group 1 | ATF | 331-350 |
|  | AIRBUS A340-300 | 4 | Group 1 | ATF | 331-350 |
|  | AIRBUS A340-600 | 4 | Group 1 | ATF | 331-350 |
|  | ILYUSHIN IL86 | 4 | Group 2 | ATF | 700-4B |
|  | ILYUSHIN IL96-300 | 4 | Group 2 | ATF | 700-4B |
| Antonov 26 | ANTONOV AN-24 | 2 | Group 8 | ATF | None |
|  | ANTONOV AN26B/32 | 2 | Group 8 | ATF | None |
|  | NAMC YS11 | 2 | Group 8 | ATF | None |
| ATR 42-320 | ATR42-300 | 2 | Group 11 | ATF | 36-150[ ] |
|  | ATR42-500 | 2 | Group 11 | ATF | 36-150[ ] |
|  | BRISTOL 170 FREIGHTER | 2 | Group 11 | AS | None |
|  | CONVAIR 240/340/440 | 2 | Group 11 | AS | None |
|  | GULF AMERICAN GULFSTREAM I | 2 | Group 11 | ATF | None |
|  | ILYUSHIN IL12/IL14 | 2 | Group 11 | AS | None |
| ATR 72-200 | ATR72 | 2 | Group 8 | ATF | 36-150[ ] |
|  | HANDLEY PAGE HERALD 200 | 2 | Group 8 | ATF | 36-150[ ] |
|  | HANDLEY PAGE HERALD 700 | 2 | Group 8 | ATF | 36-150[ ] |
|  | NORD 2501 NORTALAS | 2 | Group 8 | AS | 36-150[ ] |
| B727 | BOEING 727-100/100C | 3 | Group 5 | ATF | 85-129 |
|  | BOEING 727-200/200 ADVANCED | 3 | Group 5 | ATF | 85-129 |
|  | CONVAIR 880 | 4 | Group 5 | ATF | 85-129 |
| B737 100 | BOEING 737-100 | 2 | Group 5 | ATF | 85-129 |
|  | BOEING 737-200 | 2 | Group 5 | ATF | 85-129 |
|  | BOEING 737-300 | 2 | Group 5 | ATF | 85-129 |
| B737 400 | BOEING 737-400 | 2 | Group 5 | ATF | 85-129 |
|  | BOEING 737-500 | 2 | Group 5 | ATF | 85-129 |
|  | BOEING 737-600 | 2 | Group 5 | ATF | 131-9 |
|  | BOEING 737-700 | 2 | Group 5 | ATF | 131-9 |
| B747 100-300 | ANTONOV AN-124 | 4 | Group 2 | ATF | 700-4B |
|  | BOEING 747-100/100F | 4 | Group 1 | ATF | 600-4 |
|  | BOEING 747-200B | 4 | Group 1 | ATF | 600-4 |
|  | BOEING 747-200B (COMBI) | 4 | Group 1 | ATF | 600-4 |
|  | BOEING 747-200C/200F | 4 | Group 1 | ATF | 600-4 |
|  | BOEING 747-300(STRETCH UP DK) | 4 | Group 1 | ATF | 600-4 |
|  | BOEING 747-300M (COMBI) | 4 | Group 1 | ATF | 600-4 |
|  | BOEING 747SP | 4 | Group 1 | ATF | 600-4 |
| B747 400 | BOEING 747-400 | 4 | Group 1 | ATF | PW901A |
|  | BOEING 747-400F | 4 | Group 1 | ATF | PW901A |
|  | BOEING 747-400M (COMBI) | 4 | Group 1 | ATF | PW901A |
| B757 | BOEING 707 ALL SERIES | 4 | Group 4 | ATF | 85-129 |
|  | BOEING 707-120/121B | 4 | Group 4 | ATF | 85-129 |
|  | BOEING 720B | 4 | Group 4 | ATF | 85-129 |
|  | BOEING 757-200 | 2 | Group 4 | ATF | 331-200ER |
|  | BOEING 757-300 | 2 | Group 4 | ATF | 331-200ER |
|  | MCDONNELL-DOUGLAS DC8-10/50 | 4 | Group 4 | ATF | 85-129 |
|  | MCDONNELL-DOUGLAS DC861/3 71/3 | 4 | Group 4 | ATF | 85-129 |
|  | MCDONNELL-DOUGLAS DC8-62/72 | 4 | Group 4 | ATF | 85-129 |
|  | MCDONNELL-DOUGLAS DC8-71/73 | 4 | Group 4 | ATF | 85-129 |
|  | MCDONNELL-DOUGLAS DC8F 54/55 | 4 | Group 4 | ATF | 85-129 |
|  | TUPOLEV TU154A/B | 3 | Group 5 | ATF | 85-129 |
|  | TUPOLEV TU154M | 3 | Group 5 | ATF | 85-129 |
|  | TUPOLEV TU204 | 2 | Group 5 | ATF | 85-129 |
| B767 300 ER | BOEING 767-200 | 2 | Group 2 | ATF | 331-200ER |
|  | BOEING 767-200ER | 2 | Group 2 | ATF | 331-200ER |
|  | BOEING 767-300 | 2 | Group 2 | ATF | 331-200ER |
|  | BOEING 767-300ER/F | 2 | Group 2 | ATF | 331-200ER |
|  | BOEING 767-400ER | 2 | Group 2 | ATF | 331-200ER |
|  | ILYUSHIN IL62 | 4 | Group 4 | ATF | 85-129 |
|  | ILYUSHIN IL76 | 4 | Group 2 | ATF | 700-4B |
| B777 | BOEING 777-200 | 2 | Group 1 | ATF | 331-500 |
|  | BOEING 777-200ER | 2 | Group 1 | ATF | 331-500 |
|  | BOEING 777-300 | 2 | Group 1 | ATF | 331-500 |
| BAC1-11 | AEROSPATIALE CARAVELLE 10B/10R | 2 | Group 5 | ATF | 85-129 |
|  | AEROSPATIALE CARAVELLE 12 | 2 | Group 5 | ATF | 85-129 |
|  | AEROSPATIALE CARAVELLE 6/6R | 2 | Group 5 | ATF | 85-129 |
|  | BAE(BAC)111-200 | 2 | Group 5 | ATF | 85-129 |
|  | BAE(BAC)111-300/400/475 | 2 | Group 5 | ATF | 85-129 |
|  | BAE(BAC)111-500 | 2 | Group 5 | ATF | 85-129 |
|  | GA GULFSTREAM 3 | 2 | Group 5 | ATF | 36-150[RR] |
|  | GULF AMERICAN GULFSTREAM II | 2 | Group 7 | ATF | 36-150[RR] |
|  | TUPOLEV TU124 | 2 | Group 5 | ATF | 85-129 |
| BAe Jetstream 31 | BAE JETSTREAM 31/32 | 2 | Group 12 | ATF | None |
| BAe Jetstream 41 | BAE JETSTREAM 41 | 2 | Group 8 | ATF | None |
| BAe146 | AVROLINER RJ100/115 | 4 | Group 7 | ATF | 36-150[RR] |
|  | AVROLINER RJ70 | 4 | Group 7 | ATF | 36-150[RR] |
|  | AVROLINER RJ85/QT | 4 | Group 7 | ATF | 36-150[RR] |
|  | BAE 146-100 | 4 | Group 5 | ATF | 36-100 |
|  | BAE 146-200/QT | 4 | Group 5 | ATF | 36-100 |
|  | BAE 146-300 | 4 | Group 5 | ATF | 36-150[ ] |
| Beech 1900C Airliner | BEECHCRAFT 1900C/D AIRLINER | 2 | Group 8 | ATF | None |
| Beech Super King Air 200B | BEECHCRAFT 200 SUPERKING AIR | 2 | Group 12 | ATF | None |
| Beech Super King Air 350 | BEECHCRAFT 300 SUPER KING AIR | 2 | Group 12 | ATF | None |
| Cessna 208 Caravan | AEROSPATIALE SA315 LAMA | 1 | Helicopter | ATF | None |
|  | AEROSPATIALE SA341 GAZELLE | 1 | Helicopter | ATF | None |
|  | AEROSPATIALE SA350 ECUREUIL | 1 | Helicopter | ATF | None |
|  | BELL 206B JET RANGER | 1 | Helicopter | ATF | None |
|  | BELL 206L LONG RANGER | 1 | Helicopter | ATF | None |
|  | CESSNA 150 | 1 | Group 12 | AS | None |
|  | CESSNA 172 SKYHAWK | 1 | Group 12 | AS | None |
|  | CESSNA 180 SKYWAGON | 1 | Group 12 | AS | None |
|  | CESSNA 206 STATIONAIR | 1 | Group 12 | AS | None |
|  | CESSNA 208 CARAVAN I | 1 | Group 12 | ATF | None |
|  | CESSNA 210 CENTURION | 1 | Group 12 | AS | None |
|  | DE HAVILLAND TIGER MOTH | 1 | Group 12 | AS | None |
|  | DHC2 BEAVER | 1 | Group 12 | AS | None |
|  | GROB 6109B | 1 | Group 12 | AS | None |
|  | HUGHES 500 | 1 | Helicopter | ATF | None |
|  | MOONEY M20J | 1 | Group 12 | AS | None |
|  | PIAGGIO P166-DL3 | 2 | Group 12 | ATF | None |
|  | PIPER PA18 SUPER CUB | 1 | Group 12 | AS | None |
|  | PIPER PA24 COMMANCHE | 1 | Group 12 | AS | None |
|  | PIPER PA28 CHEROKEE SRS/PA32 | 1 | Group 12 | AS | None |
|  | SOCATA TB10 TOBAGO | 1 | Group 12 | AS | None |
| Dash 8 Q400 | BOMBARDIER DASH 8 Q100/200 | 2 | Group 8 | ATF | 30-54 |
|  | BOMBARDIER DASH 8 Q400 | 2 | Group 8 | ATF | 30-54 |
|  | DE HAVILLAND DASH 8-300/Q300 | 2 | Group 8 | ATF | 30-54 |
|  | DE HAVILLAND DASH 8-100 | 2 | Group 8 | ATF | 30-54 |
| DC10-30 | BAC/AEROSPATIALE CONCORDE | 4 | Group 3 | ATF | None |
|  | LOCKHEED L1011-1/100 TRISTAR | 3 | Group 2 | ATF | ST6L-73 |
|  | LOCKHEED L1011-200 TRISTAR | 3 | Group 2 | ATF | ST6L-73 |
|  | LOCKHEED L1011-500 TRISTAR | 3 | Group 2 | ATF | ST6L-73 |
|  | MCDONNELL-DOUGLAS DC10-10 | 3 | Group 2 | ATF | 700-4B |
|  | MCDONNELL-DOUGLAS DC10-30 | 3 | Group 2 | ATF | 700-4B |
|  | MCDONNELL-DOUGLAS DC10-40 | 3 | Group 2 | ATF | 700-4B |
|  | MCDONNELL-DOUGLAS MD11 | 3 | Group 2 | ATF | 700-4B |
| DC9 | ANTONOV AN72 | 2 | Group 5 | ATF | 85-129 |
|  | CANADAIR GLOBAL EXPRESS | 2 | Group 5 | ATF | 85-129 |
|  | DASSAULT-BREGUET MERCURE | 2 | Group 5 | ATF | 85-129 |
|  | GULF AMERICAN GULFSTREAM V | 2 | Group 7 | ATF | 36-150[RR] |
|  | MCDONNELL-DOUGLAS DC9-10/15 | 2 | Group 5 | ATF | 85-129 |
|  | McDONNELL-DOUGLAS DC9-20 | 2 | Group 5 | ATF | 85-129 |
|  | MCDONNELL-DOUGLAS DC9-30 | 2 | Group 5 | ATF | 85-129 |
|  | MCDONNELL-DOUGLAS DC9-40 | 2 | Group 5 | ATF | 85-129 |
|  | MCDONNELL-DOUGLAS DC9-50 | 2 | Group 5 | ATF | 85-129 |
|  | TUPOLEV TU134 | 2 | Group 5 | ATF | 85-129 |
| De Havilland Dash 7 | ARMSTRONG WHITWORTH ARGOSY | 4 | Group 8 | ATF | None |
|  | DE HAVILLAND DHC-7 DASH-7 | 4 | Group 8 | ATF | None |
|  | DOUGLAS DC4 SKYMASTER | 4 | Group 8 | AS | None |
|  | DOUGLAS DC6/6A/6B/6C | 4 | Group 8 | AS | None |
|  | VICKERS VISCOUNT 700 | 4 | Group 8 | ATF | None |
|  | VICKERS VISCOUNT 800 | 4 | Group 8 | ATF | None |
| De Havilland DHC-3 Turbo-Otter | AEROSPATIALE AS332 SUPER PUMA (L1/L2) | 2 | Helicopter | ATF | None |
|  | AEROSPATIALE AS355 ECUREUIL 2 | 2 | Helicopter | ATF | None |
|  | AEROSPATIALE SA330 PUMA | 2 | Helicopter | ATF | None |
|  | AEROSPATIALE SA365 DAUPHIN | 2 | Helicopter | ATF | None |
|  | AEROSPATIALE SA365 DAUPHIN/AMB | 2 | Helicopter | ATF | None |
|  | AGUSTA A109A | 2 | Helicopter | ATF | None |
|  | AIRSHIP INDUSTRIES SKYSHIP 500 | 2 | Airship | AS | None |
|  | AIRSHIP INDUSTRIES SKYSHIP 600 | 2 | Airship | ATF | None |
|  | BEECH KINGAIR 90 | 2 | Group 12 | ATF | None |
|  | BEECH KINGAIR C90A | 2 | Group 12 | ATF | None |
|  | BELL 212/412 | 2 | Helicopter | ATF | None |
|  | BELL 222 | 2 | Helicopter | ATF | None |
|  | BELL MODEL 214ST | 2 | Helicopter | ATF | None |
|  | BOEING-VERTOL MOD-234 CHINOOK | 2 | Helicopter | ATF | None |
|  | DE HAVILLAND DH6 TWIN OTTER | 2 | Group 12 | ATF | None |
|  | EUROCOPTER EC135T | 2 | Helicopter | ATF | None |
|  | MBB BK117 | 2 | Helicopter | ATF | None |
|  | MBB BO105 | 2 | Helicopter | ATF | None |
|  | MITSUBISHI MU2 | 2 | Group 12 | ATF | None |
|  | SIKORSKY S61N | 2 | Helicopter | ATF | None |
|  | SIKORSKY S76 SPIRIT | 2 | Helicopter | ATF | None |
|  | WESTLAND 30 SRS 100 | 2 | Helicopter | ATF | None |
| Dornier 328-110 | DORNIER 328 | 2 | Group 8 | ATF | 36-150[ ] |
| Embraer 110P2A | BEECH KINGAIR 100 | 2 | Group 12 | ATF | None |
|  | BEECHCRAFT 99/99A | 2 | Group 12 | ATF | None |
|  | DORNIER 228-100/200 | 2 | Group 11 | ATF | None |
|  | EMBRAER EMB110 BANDEIRANTE | 2 | Group 12 | ATF | None |
|  | EMBRAER EMB121 XINGU | 2 | Group 12 | ATF | None |
|  | LET 410 | 2 | Group 12 | ATF | None |
|  | PIAGGIO P.180 AVANTI | 2 | Group 12 | ATF | None |
|  | PIPER PA42 CHEYENNE III/IV | 2 | Group 12 | ATF | None |
| F100 | FOKKER 100 | 2 | Group 7 | ATF | 36-150[RR] |
|  | FOKKER 70 | 2 | Group 7 | ATF | 36-4A |
|  | GULF AMERICAN GULFSTREAM IV | 2 | Group 7 | ATF | 36-150[RR] |
| F28 | AEROSPATIALE CORVETTE | 2 | Group 9 | ATF | None |
|  | BAE(HS)125 | 2 | Group 9 | ATF | 36-100 |
|  | BAE125-1000 | 2 | Group 9 | ATF | 36-100 |
|  | BEECHCRAFT 400 BEECHJET | 2 | Group 9 | ATF | None |
|  | BOMBARDIER REGIONAL JET 100/200 | 2 | Group 7 | ATF | 36-150[RR] |
|  | BOMBARDIER REGIONAL JET RJ700 | 2 | Group 7 | ATF | 36-150[RR] |
|  | BOMBARDIER REGIONAL JET RJ700ER | 2 | Group 7 | ATF | 36-150[RR] |
|  | CANADAIR CL-600-604 CHALLENGER | 2 | Group 7 | ATF | 36-150[RR] |
|  | CESSNA 500 CITATION I | 2 | Group 10 | ATF | None |
|  | CESSNA 525 CITATIONJET | 2 | Group 10 | ATF | None |
|  | CESSNA 550 CITATION II | 2 | Group 10 | ATF | None |
|  | CESSNA 560 CITATION V | 2 | Group 10 | ATF | None |
|  | CESSNA 650 CITATION III/VI/VII | 2 | Group 10 | ATF | None |
|  | CESSNA 750 CITATION X | 2 | Group 10 | ATF | None |
|  | DASSAULT BREGUET FALCON 50 | 3 | Group 7 | ATF | 36-150[RR] |
|  | DASSAULT MYSTERE-FALCON 10 | 2 | Group 9 | ATF | None |
|  | DASSAULT MYSTERE-FALCON 20 | 2 | Group 9 | ATF | None |
|  | DASSAULT MYSTERE-FALCON 2000 | 2 | Group 9 | ATF | 36-150[RR] |
|  | DASSAULT MYSTERE-FALCON 900 | 3 | Group 7 | ATF | 36-150[RR] |
|  | DASSAULT MYSTERE-FALCON 900EX | 3 | Group 7 | ATF | 36-150[RR] |
|  | DORNIER 328 JET | 2 | Group 7 | ATF | 36-150[ ] |
|  | EMBRAER RJ135 | 2 | Group 7 | ATF | 36-150[ ] |
|  | EMBRAER RJ145 | 2 | Group 7 | ATF | 36-150[ ] |
|  | FOKKER F28-1000 | 2 | Group 7 | ATF | 36-4A |
|  | FOKKER F28-2000 | 2 | Group 7 | ATF | 36-4A |
|  | FOKKER F28-3000 | 2 | Group 7 | ATF | 36-4A |
|  | FOKKER F28-4000/6000 | 2 | Group 7 | ATF | 36-4A |
|  | GATES LEARJET 24/25D | 2 | Group 9 | ATF | None |
|  | GATES LEARJET 25B | 2 | Group 9 | ATF | None |
|  | GATES LEARJET 31 | 2 | Group 9 | ATF | None |
|  | GATES LEARJET 35A/36A | 2 | Group 9 | ATF | None |
|  | GATES LEARJET 55 LONGHORN | 2 | Group 9 | ATF | None |
|  | LEARJET 45 | 2 | Group 9 | ATF | None |
|  | LOCKHEED JETSTAR II | 4 | Group 7 | ATF | 36-150[RR] |
|  | ROCKWELL SABRELINER SERIES | 2 | Group 9 | ATF | None |
|  | YAKOVLEV YAK-40 | 3 | Group 9 | ATF | None |
| Fokker 27 Friendship | BAE (HS) 748 | 2 | Group 8 | ATF | None |
|  | FAIRCHILD HILLER FH 227B | 2 | Group 8 | ATF | None |
|  | FOKKER F27 100-400/600 | 2 | Group 8 | ATF | None |
|  | FOKKER F27-500 | 2 | Group 8 | ATF | None |
| Fokker 50 Srs 100 | BAE ATP | 2 | Group 8 | ATF | 30-54 |
|  | FOKKER 50 | 2 | Group 8 | ATF | 30-54 |
| Lockheed C-130H Hercules | AEROSPACELINES B377SUPER GUPPY | 4 | Group 6 | ATF | None |
|  | ANTONOV AN-12 | 4 | Group 6 | ATF | None |
|  | CANADAIR CL-44 | 4 | Group 6 | ATF | None |
|  | ILYUSHIN IL18 | 4 | Group 6 | ATF | None |
|  | LOCKHEED L100 HERCULES | 4 | Group 6 | ATF | None |
|  | LOCKHEED L188 ELECTRA | 4 | Group 6 | ATF | None |
|  | SHORTS BELFAST | 4 | Group 6 | ATF | None |
|  | V953C MERCHANTMAN | 4 | Group 6 | ATF | None |
| MD 82 | BOEING 717-200 | 2 | Group 5 | ATF | 85-129 |
|  | Mc DONNELL DOUGLAS MD90 | 2 | Group 5 | ATF | 85-129 |
|  | MCDONNELL-DOUGLAS MD80-MD83 | 2 | Group 5 | ATF | 85-129 |
|  | MCDONNELL-DOUGLAS MD87 | 2 | Group 5 | ATF | 85-129 |
|  | MCDONNELL-DOUGLAS MD88 | 2 | Group 5 | ATF | 85-129 |
|  | TUPOLEV TU104 | 2 | Group 5 | ATF | 85-129 |
|  | YAKOVLEV YAK-42 | 3 | Group 5 | ATF | 85-129 |
| Reims F406 Caravan II | BEAGLE B206 | 2 | Group 12 | AS | None |
|  | BEECHCRAFT 18/SUPER H18 | 2 | Group 12 | AS | None |
|  | BEECHCRAFT 50 TWIN BONANZA | 2 | Group 12 | AS | None |
|  | BEECHCRAFT 76 DUCHESS | 2 | Group 12 | AS | None |
|  | BEECHCRAFT 95 TRAVEL AIR | 2 | Group 12 | AS | None |
|  | BEECHCRAFT BARON MOD 55/58/58P | 2 | Group 12 | AS | None |
|  | BEECHCRAFT DUKE | 2 | Group 12 | AS | None |
|  | BEECHCRAFT QUEEN AIR 65/80 | 2 | Group 12 | AS | None |
|  | BELL 47G | 1 | Helicopter | AS | None |
|  | CESSNA 310 | 2 | Group 12 | AS | None |
|  | CESSNA 336/337 SKYMASTER | 2 | Group 12 | AS | None |
|  | CESSNA 337 SUPER SKYMASTER | 2 | Group 12 | AS | None |
|  | CESSNA 340 | 2 | Group 12 | AS | None |
|  | CESSNA 401/402/411/421 | 2 | Group 12 | AS | None |
|  | CESSNA 404 TITAN | 2 | Group 12 | AS | None |
|  | CESSNA 414A CHANCELLOR | 2 | Group 12 | AS | None |
|  | CESSNA 425 CONQUEST I | 2 | Group 12 | ATF | None |
|  | CESSNA 441 CONQUEST II | 2 | Group 12 | ATF | None |
|  | CESSNA T303 CRUSADER | 2 | Group 12 | AS | None |
|  | DE HAVILLAND DRAGON RAPIDE | 2 | Group 12 | AS | None |
|  | DORNIER DO28D SKYSERVANT | 2 | Group 11 | AS | None |
|  | ENSTROM F28A/F280 SHARK | 1 | Helicopter | AS | None |
|  | GAF NOMAD N22B/N24A | 2 | Group 12 | ATF | None |
|  | GRUMMAN GA-7 COUGAR | 2 | Group 12 | AS | None |
|  | HAWKER SIDDELEY DH 104 DOVE | 2 | Group 12 | AS | None |
|  | HAWKER SIDDELEY DH114 HERON | 4 | Group 12 | AS | None |
|  | HILLER UH12 | 1 | Helicopter | AS | None |
|  | HUGHES 269A | 1 | Helicopter | AS | None |
|  | PARTENAVIA P68B/C | 2 | Group 12 | AS | None |
|  | PILATUS BN-2A ISLANDER | 2 | Group 12 | AS | None |
|  | PILATUS BN-2A TRISLANDER MK3 | 3 | Group 12 | AS | None |
|  | PIPER PA23 AZTEC/APACHE | 2 | Group 12 | AS | None |
|  | PIPER PA30/PA39 TWIN COMANCHE | 2 | Group 12 | AS | None |
|  | PIPER PA31/P NAVAJO CHIEFTAIN | 2 | Group 12 | AS | None |
|  | PIPER PA31T CHEYENNE I/II | 2 | Group 12 | ATF | None |
|  | PIPER PA34 SENECA II | 2 | Group 12 | AS | None |
|  | PIPER PA44 SEMINOLE | 2 | Group 12 | AS | None |
|  | REIMS-CESSNA F406/CARAVAN II | 2 | Group 12 | ATF | None |
|  | RILEY TURBO-EXEC 400 | 2 | Group 12 | AS | None |
|  | RO TURBO COMMANDER 680T-690 | 2 | Group 12 | ATF | None |
|  | ROBINSON R22 | 1 | Helicopter | AS | None |
|  | ROCKWELL SHRIKE/COURSER/COMMAN | 2 | Group 12 | AS | None |
|  | ROCKWELL TURBO COMMANDER | 2 | Group 12 | ATF | None |
| Saab 2000 | CONVAIR 580/600/640 | 2 | Group 8 | ATF | 30-54 |
|  | SAAB 2000 | 2 | Group 8 | ATF | 30-54 |
| Saab 340B | EMBRAER EMB120 BRASILIA | 2 | Group 8 | ATF | 36-150[ ] |
|  | SAAB FAIRCHILD 340 | 2 | Group 8 | ATF | 36-150[ ] |
| Shorts 330 | AEROSPATIALE (NORD)262 | 2 | Group 8 | ATF | None |
|  | BEECHCRAFT STARSHIP MODEL 2000 | 2 | Group 12 | ATF | None |
|  | DOUGLAS DC3 C47 DAKOTA | 2 | Group 8 | AS | None |
|  | SHORTS 330 | 2 | Group 8 | ATF | None |
| Shorts 360-300 | SHORTS 360 | 2 | Group 8 | ATF | None |
| Shorts SC.7 Srs3M-200 | SHORTS SC7 SKYLINER | 2 | Group 12 | ATF | None |
|  | SHORTS SC7 SKYVAN | 2 | Group 12 | ATF | None |
| Swearingen Metro III | FAIRCHILD SA-227 METRO 23 | 2 | Group 12 | ATF | None |
|  | FAIRCHILD SA-227 METRO III | 2 | Group 12 | ATF | None |
|  | SWEARINGEN MERLIN IIA/IIB/IIIB | 2 | Group 12 | ATF | None |
|  | SWEARINGEN MERLIN IVA | 2 | Group 12 | ATF | None |
|  | SWEARINGEN METRO II | 2 | Group 12 | ATF | None |

Appendix 2 Times in mode

CONTENTS

| Table A2.1 | Times in mode by airport type and NATS category |
| --- | --- |
| Table A2.2 | Times in mode - Heathrow |
| Table A2.3 | Times in mode – Gatwick |

Table A2.1	Times in mode by airport type and NATS category

| Airport Type | NATS Category | Time In Approach | Time In Landing-roll | Time In Reverse Thrust | Reverse Thrust Setting | Time In Taxi-in | Time In Taxi-out | Time In Hold | Time APU Arrivals | Time APU Departures |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Gatwick | Group 1 | 286.00 | 70.38 | 30.00 | 30.00 | 491.48 | 872.31 | 338.11 | 1200.00 | 4306.64 |
| Gatwick | Group 10 | 312.00 | 56.30 | 0.00 | 7.00 | 411.28 | 363.92 | 302.25 | 900.00 | 3529.16 |
| Gatwick | Group 11 | 312.00 | 56.12 | 0.00 | 7.00 | 508.42 | 673.71 | 331.13 | 900.00 | 3446.43 |
| Gatwick | Group 12 | 312.00 | 56.07 | 0.00 | 7.00 | 483.69 | 602.71 | 335.53 | 900.00 | 1856.68 |
| Gatwick | Group 2 | 286.00 | 68.81 | 30.00 | 30.00 | 479.46 | 659.98 | 273.37 | 1200.00 | 3750.22 |
| Gatwick | Group 3 | 286.00 | 70.64 | 30.00 | 85.00 | 502.32 | 273.85 | 88.99 | 900.00 | 4500.00 |
| Gatwick | Group 4 | 286.00 | 65.84 | 15.00 | 30.00 | 485.32 | 556.31 | 270.48 | 900.00 | 4404.59 |
| Gatwick | Group 5 | 286.00 | 59.14 | 15.00 | 30.00 | 483.66 | 617.34 | 256.49 | 900.00 | 3610.67 |
| Gatwick | Group 6 | 286.00 | 53.49 | 15.00 | 30.00 | 491.59 | 570.96 | 344.32 | 900.00 | 2057.71 |
| Gatwick | Group 7 | 286.00 | 53.03 | 0.00 | 7.00 | 493.57 | 673.47 | 342.57 | 900.00 | 3824.90 |
| Gatwick | Group 8 | 286.00 | 56.89 | 0.00 | 7.00 | 471.08 | 551.20 | 320.09 | 900.00 | 1741.43 |
| Gatwick | Group 9 | 312.00 | 56.06 | 0.00 | 7.00 | 417.38 | 400.88 | 331.57 | 900.00 | 3679.26 |
| Gatwick | Helicopter | 390.00 | 0.00 | 0.00 | 7.00 | 375.93 | 0.00 | 0.00 | 900.00 | 1886.88 |
| Heathrow | Group 1 | 286.00 | 61.00 | 30.00 | 30.00 | 525.95 | 834.75 | 324.39 | 1200.00 | 4948.88 |
| Heathrow | Group 10 | 312.00 | 45.66 | 0.00 | 7.00 | 660.36 | 818.75 | 342.98 | 900.00 | 1800.00 |
| Heathrow | Group 11 | 312.00 | 54.48 | 0.00 | 7.00 | 707.45 | 765.76 | 330.36 | 900.00 | 1800.00 |
| Heathrow | Group 12 | 312.00 | 64.00 | 0.00 | 7.00 | 616.02 | 832.92 | 340.07 | 900.00 | 1800.00 |
| Heathrow | Group 2 | 286.00 | 55.21 | 30.00 | 30.00 | 452.63 | 789.54 | 341.80 | 1200.00 | 4958.27 |
| Heathrow | Group 3 | 286.00 | 64.00 | 30.00 | 85.00 | 710.35 | 733.61 | 342.01 | 900.00 | 1800.00 |
| Heathrow | Group 4 | 286.00 | 49.29 | 15.00 | 30.00 | 424.17 | 702.48 | 356.35 | 900.00 | 1800.00 |
| Heathrow | Group 5 | 286.00 | 46.41 | 15.00 | 30.00 | 371.08 | 780.23 | 341.20 | 900.00 | 1800.00 |
| Heathrow | Group 6 | 286.00 | 49.65 | 15.00 | 30.00 | 494.86 | 868.08 | 340.94 | 900.00 | 1800.00 |
| Heathrow | Group 7 | 286.00 | 49.65 | 0.00 | 7.00 | 401.58 | 690.28 | 342.64 | 900.00 | 1800.00 |
| Heathrow | Group 8 | 286.00 | 35.00 | 0.00 | 7.00 | 766.96 | 762.81 | 341.94 | 900.00 | 1800.00 |
| Heathrow | Group 9 | 312.00 | 45.99 | 0.00 | 7.00 | 637.63 | 837.26 | 340.93 | 900.00 | 1800.00 |
| Heathrow | Helicopter | 390.00 | 0.00 | 0.00 | 7.00 | 667.27 | 849.23 | 344.31 | 900.00 | 1800.00 |
| Regional | Group 1 | 286.00 | 70.00 | 30.00 | 30.00 | a | a | b | 1050.00 | 1804.00 |
| Regional | Group 10 | 312.00 | 60.00 | 0.00 | 7.00 | a | a | b | 1050.00 | 1804.00 |
| Regional | Group 11 | 312.00 | 60.00 | 0.00 | 7.00 | a | a | b | 1050.00 | 1804.00 |
| Regional | Group 12 | 312.00 | 60.00 | 0.00 | 7.00 | a | a | b | 1050.00 | 1804.00 |
| Regional | Group 2 | 286.00 | 70.00 | 30.00 | 30.00 | a | a | b | 1050.00 | 1804.00 |
| Regional | Group 3 | 286.00 | 70.00 | 30.00 | 85.00 | a | a | b | 1050.00 | 1804.00 |
| Regional | Group 4 | 286.00 | 60.00 | 15.00 | 30.00 | a | a | b | 1050.00 | 1804.00 |
| Regional | Group 5 | 286.00 | 60.00 | 15.00 | 30.00 | a | a | b | 1050.00 | 1804.00 |
| Regional | Group 6 | 286.00 | 60.00 | 15.00 | 30.00 | a | a | b | 1050.00 | 1804.00 |
| Regional | Group 7 | 286.00 | 60.00 | 0.00 | 7.00 | a | a | b | 1050.00 | 1804.00 |
| Regional | Group 8 | 286.00 | 60.00 | 0.00 | 7.00 | a | a | b | 1050.00 | 1804.00 |
| Regional | Group 9 | 312.00 | 60.00 | 0.00 | 7.00 | a | a | b | 1050.00 | 1804.00 |
| Regional | Helicopter | 390.00 | 0.00 | 0.00 | 7.00 | a | a | b | 1050.00 | 1804.00 |
| Small | Group 1 | 286.00 | 70.00 | 30.00 | 30.00 | 120.00 | 120.00 | b | 1050.00 | 1804.00 |
| Small | Group 10 | 312.00 | 60.00 | 0.00 | 7.00 | 120.00 | 120.00 | b | 1050.00 | 1804.00 |
| Small | Group 11 | 312.00 | 60.00 | 0.00 | 7.00 | 120.00 | 120.00 | b | 1050.00 | 1804.00 |
| Small | Group 12 | 312.00 | 60.00 | 0.00 | 7.00 | 120.00 | 120.00 | b | 1050.00 | 1804.00 |
| Small | Group 2 | 286.00 | 70.00 | 30.00 | 30.00 | 120.00 | 120.00 | b | 1050.00 | 1804.00 |
| Small | Group 3 | 286.00 | 70.00 | 30.00 | 85.00 | 120.00 | 120.00 | b | 1050.00 | 1804.00 |
| Small | Group 4 | 286.00 | 60.00 | 15.00 | 30.00 | 120.00 | 120.00 | b | 1050.00 | 1804.00 |
| Small | Group 5 | 286.00 | 60.00 | 15.00 | 30.00 | 120.00 | 120.00 | b | 1050.00 | 1804.00 |
| Small | Group 6 | 286.00 | 60.00 | 15.00 | 30.00 | 120.00 | 120.00 | b | 1050.00 | 1804.00 |
| Small | Group 7 | 286.00 | 60.00 | 0.00 | 7.00 | 120.00 | 120.00 | b | 1050.00 | 1804.00 |
| Small | Group 8 | 286.00 | 60.00 | 0.00 | 7.00 | 120.00 | 120.00 | b | 1050.00 | 1804.00 |
| Small | Group 9 | 312.00 | 60.00 | 0.00 | 7.00 | 120.00 | 120.00 | b | 1050.00 | 1804.00 |
| Small | Helicopter | 390.00 | 0.00 | 0.00 | 7.00 | 120.00 | 120.00 | b | 1050.00 | 1804.00 |
| Stansted | Group 1 | 286.00 | 94.12 | 30.00 | 30.00 | 359.01 | 440.32 | 268.00 | 1050.00 | 1804.20 |
| Stansted | Group 10 | 312.00 | 47.05 | 0.00 | 7.00 | 359.01 | 440.32 | 268.00 | 1050.00 | 1804.20 |
| Stansted | Group 11 | 312.00 | 47.05 | 0.00 | 7.00 | 359.01 | 440.32 | 268.00 | 1050.00 | 1804.20 |
| Stansted | Group 12 | 312.00 | 47.05 | 0.00 | 7.00 | 359.01 | 440.32 | 268.00 | 1050.00 | 1804.20 |
| Stansted | Group 2 | 286.00 | 82.63 | 30.00 | 30.00 | 359.01 | 440.32 | 268.00 | 1050.00 | 1804.20 |
| Stansted | Group 3 | 286.00 | 82.63 | 30.00 | 85.00 | 359.01 | 440.32 | 268.00 | 1050.00 | 1804.20 |
| Stansted | Group 4 | 286.00 | 80.54 | 15.00 | 30.00 | 359.01 | 440.32 | 268.00 | 1050.00 | 1804.20 |
| Stansted | Group 5 | 286.00 | 53.23 | 15.00 | 30.00 | 359.01 | 440.32 | 268.00 | 1050.00 | 1804.20 |
| Stansted | Group 6 | 286.00 | 50.78 | 15.00 | 30.00 | 359.01 | 440.32 | 268.00 | 1050.00 | 1804.20 |
| Stansted | Group 7 | 286.00 | 53.38 | 0.00 | 7.00 | 359.01 | 440.32 | 268.00 | 1050.00 | 1804.20 |
| Stansted | Group 8 | 286.00 | 49.58 | 0.00 | 7.00 | 359.01 | 440.32 | 268.00 | 1050.00 | 1804.20 |
| Stansted | Group 9 | 312.00 | 69.77 | 0.00 | 7.00 | 359.01 | 440.32 | 268.00 | 1050.00 | 1804.20 |
| Stansted | Helicopter | 390.00 | 0.00 | 0.00 | 7.00 | 359.01 | 440.32 | 268.00 | 1050.00 | 1804.20 |

a = runway length / 10 m

b = 0 if Number of Movements per Year / Number of Runways < 100000, 600 if Number of Movements per Year / Number of Runways > 400000 otherwise (Number of Movements per Year / Number of Runways - 100000) * 0.002

Table A2.2	Times in mode - Heathrow

| CAA Aircraft Type | Percentage At Full Take-off Thrust | Reduced Take-off Thrust Setting | Percentage At Full Climb-out Thrust | Reduced Climb-out Thrust Setting | Percentage Using Reverse Thrust | Time In Take-off Roll (see note at end of table) | Time In Initial-climb | Time In Climb-out |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AEROSPACELINES B377SUPER GUPPY | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 62.83 | 70.04 |
| AEROSPATIALE (NORD)262 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| AEROSPATIALE AS332 SUPER PUMA (L1/L2) | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| AEROSPATIALE AS355 ECUREUIL 2 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| AEROSPATIALE CARAVELLE 10B/10R | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| AEROSPATIALE CARAVELLE 12 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| AEROSPATIALE CARAVELLE 6/6R | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| AEROSPATIALE CORVETTE | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| AEROSPATIALE SA315 LAMA | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| AEROSPATIALE SA330 PUMA | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| AEROSPATIALE SA341 GAZELLE | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| AEROSPATIALE SA350 ECUREUIL | 0.00 | 85.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| AEROSPATIALE SA365 DAUPHIN | 0.00 | 85.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| AEROSPATIALE SA365 DAUPHIN/AMB | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| AGUSTA A109A | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| AIRBUS A300-600 | 10.00 | 78.20 | 10.00 | 78.20 | 100.00 |  | 41.65 | 57.70 |
| AIRBUS A300B1/B2 | 10.00 | 78.20 | 10.00 | 78.20 | 100.00 |  | 41.65 | 57.70 |
| AIRBUS A300B4-100/200 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 38.51 | 72.03 |
| AIRBUS A300F4 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 41.65 | 57.70 |
| AIRBUS A310-202 | 10.00 | 78.20 | 10.00 | 78.20 | 100.00 |  | 41.65 | 57.70 |
| AIRBUS A310-300 | 10.00 | 78.20 | 10.00 | 78.20 | 100.00 |  | 41.65 | 57.70 |
| AIRBUS A319 | 5.09 | 80.90 | 5.09 | 80.90 | 10.62 |  | 37.15 | 56.27 |
| AIRBUS A320-100/200 | 6.23 | 84.10 | 6.23 | 84.10 | 72.23 |  | 37.15 | 56.27 |
| AIRBUS A321 | 7.45 | 84.10 | 7.45 | 84.10 | 100.00 |  | 37.15 | 56.27 |
| AIRBUS A330-200 | 1.00 | 86.80 | 100.00 | 85.00 | 100.00 |  | 41.65 | 57.70 |
| AIRBUS A330-300 | 1.00 | 86.80 | 100.00 | 85.00 | 100.00 |  | 41.65 | 57.70 |
| AIRBUS A340-200 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 41.65 | 57.70 |
| AIRBUS A340-300 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 41.65 | 57.70 |
| AIRBUS A340-600 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 41.65 | 57.70 |
| AIRSHIP INDUSTRIES SKYSHIP 500 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| AIRSHIP INDUSTRIES SKYSHIP 600 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| ANTONOV AN-12 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 62.83 | 70.04 |
| ANTONOV AN-124 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 41.65 | 57.70 |
| ANTONOV AN-24 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| ANTONOV AN26B/32 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| ANTONOV AN72 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 37.78 | 69.20 |
| ARMSTRONG WHITWORTH ARGOSY | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| ATR42-300 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| ATR42-500 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| ATR72 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| AVROLINER RJ100/115 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| AVROLINER RJ70 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| AVROLINER RJ85/QT | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| BAC/AEROSPATIALE CONCORDE | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 85.00 | 120.00 |
| BAE (HS) 748 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| BAE 146-100 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 39.60 | 70.64 |
| BAE 146-200/QT | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 39.60 | 70.64 |
| BAE 146-300 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 39.60 | 70.64 |
| BAE ATP | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| BAE JETSTREAM 31/32 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BAE JETSTREAM 41 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BAE(BAC)111-200 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| BAE(BAC)111-300/400/475 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| BAE(BAC)111-500 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| BAE(HS)125 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BAE125-1000 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BEAGLE B206 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BEECH KINGAIR 100 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BEECH KINGAIR 90 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BEECH KINGAIR C90A | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BEECHCRAFT 18/SUPER H18 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BEECHCRAFT 1900C/D AIRLINER | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BEECHCRAFT 200 SUPERKING AIR | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BEECHCRAFT 300 SUPER KING AIR | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BEECHCRAFT 400 BEECHJET | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BEECHCRAFT 50 TWIN BONANZA | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BEECHCRAFT 76 DUCHESS | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BEECHCRAFT 95 TRAVEL AIR | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BEECHCRAFT 99/99A | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BEECHCRAFT BARON MOD 55/58/58P | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BEECHCRAFT DUKE | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BEECHCRAFT QUEEN AIR 65/80 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BEECHCRAFT STARSHIP MODEL 2000 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BELL 206B JET RANGER | 0.00 | 85.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| BELL 206L LONG RANGER | 0.00 | 85.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| BELL 212/412 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| BELL 222 | 0.00 | 85.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| BELL 47G | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BELL MODEL 214ST | 0.00 | 85.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| BOEING 707 ALL SERIES | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 33.02 | 48.48 |
| BOEING 707-120/121B | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 36.51 | 69.49 |
| BOEING 717-200 | 3.00 | 96.67 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BOEING 720B | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 36.51 | 69.49 |
| BOEING 727-100/100C | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 37.15 | 56.27 |
| BOEING 727-200/200 ADVANCED | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 37.15 | 56.27 |
| BOEING 737-100 | 1.00 | 96.15 | 100.00 | 85.00 | 100.00 |  | 37.78 | 69.20 |
| BOEING 737-200 | 10.00 | 83.80 | 10.00 | 83.80 | 100.00 |  | 37.15 | 56.27 |
| BOEING 737-300 | 10.01 | 83.80 | 10.01 | 83.80 | 100.00 |  | 37.15 | 56.27 |
| BOEING 737-400 | 10.00 | 83.80 | 10.00 | 83.80 | 100.00 |  | 37.15 | 56.27 |
| BOEING 737-500 | 10.00 | 83.80 | 10.00 | 83.80 | 100.00 |  | 37.15 | 56.27 |
| BOEING 737-600 | 10.00 | 83.80 | 10.00 | 83.80 | 100.00 |  | 37.15 | 56.27 |
| BOEING 737-700 | 10.00 | 83.80 | 10.00 | 83.80 | 100.00 |  | 37.15 | 56.27 |
| BOEING 737-800 | 10.00 | 83.80 | 10.00 | 83.80 | 100.00 |  | 37.15 | 56.27 |
| BOEING 737-900 | 10.00 | 83.80 | 10.00 | 83.80 | 100.00 |  | 37.15 | 56.27 |
| BOEING 747-100/100F | 3.00 | 86.50 | 100.00 | 85.00 | 100.00 |  | 57.01 | 77.62 |
| BOEING 747-200B | 3.53 | 86.50 | 100.00 | 85.00 | 100.00 |  | 57.01 | 77.62 |
| BOEING 747-200B (COMBI) | 3.00 | 86.50 | 100.00 | 85.00 | 100.00 |  | 57.01 | 77.62 |
| BOEING 747-200C/200F | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 57.01 | 77.62 |
| BOEING 747-300(STRETCH UP DK) | 4.60 | 86.50 | 100.00 | 85.00 | 100.00 |  | 57.01 | 77.62 |
| BOEING 747-300M (COMBI) | 3.00 | 86.50 | 100.00 | 85.00 | 100.00 |  | 57.01 | 77.62 |
| BOEING 747-400 | 8.37 | 86.88 | 85.60 | 84.80 | 49.12 |  | 57.01 | 77.62 |
| BOEING 747-400F | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 57.01 | 77.62 |
| BOEING 747-400M (COMBI) | 3.00 | 86.50 | 100.00 | 85.00 | 100.00 |  | 57.01 | 77.62 |
| BOEING 747SP | 3.00 | 86.50 | 100.00 | 85.00 | 100.00 |  | 57.01 | 77.62 |
| BOEING 757-200 | 10.00 | 77.29 | 10.00 | 77.29 | 100.00 |  | 33.02 | 48.48 |
| BOEING 757-300 | 10.00 | 76.10 | 10.00 | 76.10 | 100.00 |  | 33.02 | 48.48 |
| BOEING 767-200 | 5.00 | 77.00 | 5.00 | 77.00 | 100.00 |  | 41.65 | 57.70 |
| BOEING 767-200ER | 5.00 | 77.00 | 5.00 | 77.00 | 100.00 |  | 41.65 | 57.70 |
| BOEING 767-300 | 5.00 | 77.00 | 5.00 | 77.00 | 11.13 |  | 41.65 | 57.70 |
| BOEING 767-300ER/F | 5.00 | 77.00 | 5.00 | 77.00 | 93.42 |  | 41.65 | 57.70 |
| BOEING 767-400ER | 8.79 | 88.16 | 53.73 | 84.00 | 100.00 |  | 38.51 | 72.03 |
| BOEING 777-200 | 1.00 | 87.04 | 100.00 | 85.00 | 60.40 |  | 44.23 | 68.66 |
| BOEING 777-200ER | 1.00 | 86.54 | 100.00 | 85.00 | 53.27 |  | 44.23 | 68.66 |
| BOEING 777-300 | 1.00 | 86.80 | 100.00 | 85.00 | 100.00 |  | 44.23 | 68.66 |
| BOEING-VERTOL MOD-234 CHINOOK | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| BOMBARDIER DASH 8 Q100/200 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| BOMBARDIER DASH 8 Q400 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| BOMBARDIER REGIONAL JET 100/200 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| BOMBARDIER REGIONAL JET RJ700 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| BOMBARDIER REGIONAL JET RJ700ER | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| BRISTOL 170 FREIGHTER | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CANADAIR CL-44 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 62.83 | 70.04 |
| CANADAIR CL-600-604 CHALLENGER | 29.10 | 90.91 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| CANADAIR GLOBAL EXPRESS | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 37.78 | 69.20 |
| CESSNA 150 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 172 SKYHAWK | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 180 SKYWAGON | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 206 STATIONAIR | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 208 CARAVAN I | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 210 CENTURION | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 310 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 336/337 SKYMASTER | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 337 SUPER SKYMASTER | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 340 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 401/402/411/421 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 404 TITAN | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 414A CHANCELLOR | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 425 CONQUEST I | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 441 CONQUEST II | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 500 CITATION I | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 525 CITATIONJET | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 550 CITATION II | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 560 CITATION V | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 650 CITATION III/VI/VII | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 750 CITATION X | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA T303 CRUSADER | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CONVAIR 240/340/440 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CONVAIR 580/600/640 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| CONVAIR 880 | 96.04 | 92.41 | 100.00 | 85.00 | 100.00 |  | 62.83 | 70.04 |
| DASSAULT BREGUET FALCON 50 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| DASSAULT MYSTERE-FALCON 10 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| DASSAULT MYSTERE-FALCON 20 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| DASSAULT MYSTERE-FALCON 2000 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| DASSAULT MYSTERE-FALCON 900 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| DASSAULT MYSTERE-FALCON 900EX | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| DASSAULT-BREGUET MERCURE | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 37.78 | 69.20 |
| DE HAVILLAND DASH 8-300/Q300 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| DE HAVILLAND DASH 8-100 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| DE HAVILLAND DH6 TWIN OTTER | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| DE HAVILLAND DHC-7 DASH-7 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| DE HAVILLAND DRAGON RAPIDE | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| DE HAVILLAND TIGER MOTH | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| DHC2 BEAVER | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| DORNIER 228-100/200 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| DORNIER 328 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| DORNIER 328 JET | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| DORNIER DO28D SKYSERVANT | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| DOUGLAS DC3 C47 DAKOTA | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| DOUGLAS DC4 SKYMASTER | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| DOUGLAS DC6/6A/6B/6C | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| EMBRAER EMB110 BANDEIRANTE | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| EMBRAER EMB120 BRASILIA | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| EMBRAER EMB121 XINGU | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| EMBRAER RJ135 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| EMBRAER RJ145 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| ENSTROM F28A/F280 SHARK | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| EUROCOPTER EC135T | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| FAIRCHILD HILLER FH 227B | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| FAIRCHILD SA-227 METRO 23 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| FAIRCHILD SA-227 METRO III | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| FOKKER 100 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| FOKKER 50 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| FOKKER 70 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| FOKKER F27 100-400/600 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| FOKKER F27-500 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| FOKKER F28-1000 | 29.10 | 90.91 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| FOKKER F28-2000 | 29.10 | 90.91 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| FOKKER F28-3000 | 29.10 | 90.91 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| FOKKER F28-4000/6000 | 29.10 | 90.91 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| GA GULFSTREAM 3 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| GAF NOMAD N22B/N24A | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| GATES LEARJET 24/25D | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| GATES LEARJET 25B | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| GATES LEARJET 31 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| GATES LEARJET 35A/36A | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| GATES LEARJET 55 LONGHORN | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| GROB 6109B | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| GRUMMAN GA-7 COUGAR | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| GULF AMERICAN GULFSTREAM I | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| GULF AMERICAN GULFSTREAM II | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| GULF AMERICAN GULFSTREAM IV | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| GULF AMERICAN GULFSTREAM V | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| HANDLEY PAGE HERALD 200 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| HANDLEY PAGE HERALD 700 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| HAWKER SIDDELEY DH 104 DOVE | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| HAWKER SIDDELEY DH114 HERON | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| HILLER UH12 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| HUGHES 269A | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| HUGHES 500 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| ILYUSHIN IL12/IL14 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| ILYUSHIN IL18 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 62.83 | 70.04 |
| ILYUSHIN IL62 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 33.02 | 48.48 |
| ILYUSHIN IL76 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 41.65 | 57.70 |
| ILYUSHIN IL86 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 41.65 | 57.70 |
| ILYUSHIN IL96-300 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 41.65 | 57.70 |
| LEARJET 45 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| LET 410 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| LOCKHEED JETSTAR II | 2.01 | 97.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| LOCKHEED L100 HERCULES | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 37.15 | 56.27 |
| LOCKHEED L1011-1/100 TRISTAR | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 41.65 | 57.70 |
| LOCKHEED L1011-200 TRISTAR | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 38.51 | 72.03 |
| LOCKHEED L1011-500 TRISTAR | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 41.65 | 57.70 |
| LOCKHEED L188 ELECTRA | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| MBB BK117 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| MBB BO105 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| Mc DONNELL DOUGLAS MD90 | 10.00 | 83.80 | 10.00 | 83.80 | 100.00 |  | 37.15 | 56.27 |
| MCDONNELL-DOUGLAS DC10-10 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 38.51 | 72.03 |
| MCDONNELL-DOUGLAS DC10-30 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 41.65 | 57.70 |
| MCDONNELL-DOUGLAS DC10-40 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 38.51 | 72.03 |
| MCDONNELL-DOUGLAS DC8-10/50 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 36.51 | 69.49 |
| MCDONNELL-DOUGLAS DC861/3 71/3 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 36.51 | 69.49 |
| MCDONNELL-DOUGLAS DC8-62/72 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 33.02 | 48.48 |
| MCDONNELL-DOUGLAS DC8-71/73 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 36.51 | 69.49 |
| MCDONNELL-DOUGLAS DC8F 54/55 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 36.51 | 69.49 |
| MCDONNELL-DOUGLAS DC9-10/15 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 37.78 | 69.20 |
| McDONNELL-DOUGLAS DC9-20 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 37.78 | 69.20 |
| MCDONNELL-DOUGLAS DC9-30 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 37.78 | 69.20 |
| MCDONNELL-DOUGLAS DC9-40 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 37.78 | 69.20 |
| MCDONNELL-DOUGLAS DC9-50 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 37.78 | 69.20 |
| MCDONNELL-DOUGLAS MD11 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 41.65 | 57.70 |
| MCDONNELL-DOUGLAS MD80-MD83 | 10.15 | 83.80 | 10.15 | 83.80 | 100.00 |  | 37.15 | 56.27 |
| MCDONNELL-DOUGLAS MD87 | 10.00 | 83.80 | 10.00 | 83.80 | 100.00 |  | 37.15 | 56.27 |
| MCDONNELL-DOUGLAS MD88 | 10.00 | 83.80 | 10.00 | 83.80 | 100.00 |  | 37.15 | 56.27 |
| MITSUBISHI MU2 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| MOONEY M20J | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| NAMC YS11 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| NORD 2501 NORTALAS | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| PARTENAVIA P68B/C | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| PIAGGIO P.180 AVANTI | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| PIAGGIO P166-DL3 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| PILATUS BN-2A ISLANDER | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| PILATUS BN-2A TRISLANDER MK3 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| PIPER PA18 SUPER CUB | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| PIPER PA23 AZTEC/APACHE | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| PIPER PA24 COMMANCHE | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| PIPER PA28 CHEROKEE SRS/PA32 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| PIPER PA30/PA39 TWIN COMANCHE | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| PIPER PA31/P NAVAJO CHIEFTAIN | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| PIPER PA31T CHEYENNE I/II | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| PIPER PA34 SENECA II | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| PIPER PA42 CHEYENNE III/IV | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| PIPER PA44 SEMINOLE | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| REIMS-CESSNA F406/CARAVAN II | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| RILEY TURBO-EXEC 400 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| RO TURBO COMMANDER 680T-690 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| ROBINSON R22 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| ROCKWELL SABRELINER SERIES | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| ROCKWELL SHRIKE/COURSER/COMMAN | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| ROCKWELL TURBO COMMANDER | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| SAAB 2000 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| SAAB FAIRCHILD 340 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| SHORTS 330 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| SHORTS 360 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| SHORTS BELFAST | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 62.83 | 70.04 |
| SHORTS SC7 SKYLINER | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| SHORTS SC7 SKYVAN | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| SIKORSKY S61N | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| SIKORSKY S76 SPIRIT | 0.00 | 85.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| SOCATA TB10 TOBAGO | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| SWEARINGEN MERLIN IIA/IIB/IIIB | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| SWEARINGEN MERLIN IVA | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| SWEARINGEN METRO II | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| TUPOLEV TU104 | 6.33 | 93.20 | 100.00 | 85.00 | 100.00 |  | 37.78 | 69.20 |
| TUPOLEV TU124 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| TUPOLEV TU134 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 37.15 | 56.27 |
| TUPOLEV TU154A/B | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 37.15 | 56.27 |
| TUPOLEV TU154M | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 37.15 | 56.27 |
| TUPOLEV TU204 | 1.47 | 88.50 | 77.48 | 83.51 | 100.00 |  | 36.51 | 69.49 |
| V953C MERCHANTMAN | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 62.83 | 70.04 |
| VICKERS VISCOUNT 700 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| VICKERS VISCOUNT 800 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| WESTLAND 30 SRS 100 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| YAKOVLEV YAK-40 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| YAKOVLEV YAK-42 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 37.15 | 56.27 |

Notes

- Time In Take-off Roll data are commercially confidential

Table A2.3	Times in mode – Gatwick

| CAA Aircraft Type | Percentage At Full Take-off Thrust | Reduced Take-off Thrust Setting | Percentage At Full Climb-out Thrust | Reduced Climb-out Thrust Setting | Percentage Using Reverse Thrust | Time In Take-off Roll (see note at end of table) | Time In Initial-climb | Time In Climb-out |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AEROSPACELINES B377SUPER GUPPY | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 62.83 | 70.04 |
| AEROSPATIALE (NORD)262 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| AEROSPATIALE AS332 SUPER PUMA (L1/L2) | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| AEROSPATIALE AS355 ECUREUIL 2 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| AEROSPATIALE CARAVELLE 10B/10R | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| AEROSPATIALE CARAVELLE 12 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| AEROSPATIALE CARAVELLE 6/6R | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| AEROSPATIALE CORVETTE | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| AEROSPATIALE SA315 LAMA | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| AEROSPATIALE SA330 PUMA | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| AEROSPATIALE SA341 GAZELLE | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| AEROSPATIALE SA350 ECUREUIL | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| AEROSPATIALE SA365 DAUPHIN | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| AEROSPATIALE SA365 DAUPHIN/AMB | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| AGUSTA A109A | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| AIRBUS A300-600 | 2.10 | 89.98 | 100.00 | 85.00 | 100.00 |  | 38.51 | 72.03 |
| AIRBUS A300B1/B2 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 38.51 | 72.03 |
| AIRBUS A300B4-100/200 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 38.51 | 72.03 |
| AIRBUS A300F4 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 41.65 | 57.70 |
| AIRBUS A310-202 | 2.10 | 89.98 | 100.00 | 85.00 | 100.00 |  | 38.51 | 72.03 |
| AIRBUS A310-300 | 2.10 | 89.98 | 100.00 | 85.00 | 100.00 |  | 38.51 | 72.03 |
| AIRBUS A319 | 3.54 | 89.99 | 100.00 | 85.00 | 99.61 |  | 37.78 | 69.20 |
| AIRBUS A320-100/200 | 2.12 | 89.42 | 100.00 | 85.00 | 100.00 |  | 37.78 | 69.20 |
| AIRBUS A321 | 0.77 | 89.24 | 100.00 | 85.00 | 100.00 |  | 37.78 | 69.20 |
| AIRBUS A330-200 | 2.65 | 88.70 | 100.00 | 85.00 | 100.00 |  | 40.58 | 78.11 |
| AIRBUS A330-300 | 5.00 | 94.52 | 100.00 | 85.00 | 100.00 |  | 40.58 | 78.11 |
| AIRBUS A340-200 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 47.91 | 79.43 |
| AIRBUS A340-300 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 47.91 | 79.43 |
| AIRBUS A340-600 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 41.65 | 57.70 |
| AIRSHIP INDUSTRIES SKYSHIP 500 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| AIRSHIP INDUSTRIES SKYSHIP 600 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| ANTONOV AN-12 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 62.83 | 70.04 |
| ANTONOV AN-124 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 41.65 | 57.70 |
| ANTONOV AN-24 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| ANTONOV AN26B/32 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| ANTONOV AN72 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 37.78 | 69.20 |
| ARMSTRONG WHITWORTH ARGOSY | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| ATR42-300 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| ATR42-500 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| ATR72 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| AVROLINER RJ100/115 | 2.01 | 97.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| AVROLINER RJ70 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| AVROLINER RJ85/QT | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| BAC/AEROSPATIALE CONCORDE | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 85.00 | 120.00 |
| BAE (HS) 748 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| BAE 146-100 | 96.04 | 92.41 | 100.00 | 85.00 | 100.00 |  | 62.83 | 70.04 |
| BAE 146-200/QT | 28.03 | 87.47 | 51.03 | 85.00 | 100.00 |  | 62.83 | 70.04 |
| BAE 146-300 | 23.41 | 90.45 | 100.00 | 85.00 | 100.00 |  | 62.83 | 70.04 |
| BAE ATP | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| BAE JETSTREAM 31/32 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BAE JETSTREAM 41 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BAE(BAC)111-200 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| BAE(BAC)111-300/400/475 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| BAE(BAC)111-500 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| BAE(HS)125 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BAE125-1000 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BEAGLE B206 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BEECH KINGAIR 100 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BEECH KINGAIR 90 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BEECH KINGAIR C90A | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BEECHCRAFT 18/SUPER H18 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BEECHCRAFT 1900C/D AIRLINER | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BEECHCRAFT 200 SUPERKING AIR | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BEECHCRAFT 300 SUPER KING AIR | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BEECHCRAFT 400 BEECHJET | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BEECHCRAFT 50 TWIN BONANZA | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BEECHCRAFT 76 DUCHESS | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BEECHCRAFT 95 TRAVEL AIR | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BEECHCRAFT 99/99A | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BEECHCRAFT BARON MOD 55/58/58P | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BEECHCRAFT DUKE | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BEECHCRAFT QUEEN AIR 65/80 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BEECHCRAFT STARSHIP MODEL 2000 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BELL 206B JET RANGER | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| BELL 206L LONG RANGER | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| BELL 212/412 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| BELL 222 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| BELL 47G | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BELL MODEL 214ST | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| BOEING 707 ALL SERIES | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 36.51 | 69.49 |
| BOEING 707-120/121B | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 36.51 | 69.49 |
| BOEING 717-200 | 3.00 | 96.67 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| BOEING 720B | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 36.51 | 69.49 |
| BOEING 727-100/100C | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 37.78 | 69.20 |
| BOEING 727-200/200 ADVANCED | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 37.78 | 69.20 |
| BOEING 737-100 | 1.00 | 96.15 | 100.00 | 85.00 | 100.00 |  | 37.78 | 69.20 |
| BOEING 737-200 | 1.00 | 96.15 | 100.00 | 85.00 | 100.00 |  | 37.78 | 69.20 |
| BOEING 737-300 | 6.49 | 87.06 | 63.62 | 79.07 | 100.00 |  | 37.78 | 69.20 |
| BOEING 737-400 | 9.66 | 81.43 | 14.39 | 80.59 | 100.00 |  | 37.78 | 69.20 |
| BOEING 737-500 | 49.40 | 84.24 | 75.60 | 79.58 | 100.00 |  | 37.78 | 69.20 |
| BOEING 737-600 | 3.00 | 79.58 | 3.00 | 79.58 | 100.00 |  | 37.78 | 69.20 |
| BOEING 737-700 | 12.41 | 91.21 | 95.45 | 76.01 | 100.00 |  | 37.78 | 69.20 |
| BOEING 737-800 | 2.03 | 92.19 | 100.00 | 85.00 | 100.00 |  | 37.78 | 69.20 |
| BOEING 737-900 | 10.00 | 83.80 | 10.00 | 83.80 | 100.00 |  | 37.15 | 56.27 |
| BOEING 747-100/100F | 2.00 | 77.62 | 2.00 | 77.62 | 100.00 |  | 47.91 | 79.43 |
| BOEING 747-200B | 2.00 | 77.62 | 2.00 | 77.62 | 100.00 |  | 47.91 | 79.43 |
| BOEING 747-200B (COMBI) | 2.00 | 77.62 | 2.00 | 77.62 | 100.00 |  | 47.91 | 79.43 |
| BOEING 747-200C/200F | 2.00 | 77.62 | 2.00 | 77.62 | 100.00 |  | 47.91 | 79.43 |
| BOEING 747-300(STRETCH UP DK) | 2.00 | 77.62 | 2.00 | 77.62 | 100.00 |  | 47.91 | 79.43 |
| BOEING 747-300M (COMBI) | 3.00 | 86.50 | 100.00 | 85.00 | 100.00 |  | 57.01 | 77.62 |
| BOEING 747-400 | 2.91 | 77.77 | 4.08 | 77.62 | 98.16 |  | 47.80 | 79.35 |
| BOEING 747-400F | 2.00 | 77.62 | 2.00 | 77.62 | 100.00 |  | 47.91 | 79.43 |
| BOEING 747-400M (COMBI) | 3.00 | 86.50 | 100.00 | 85.00 | 100.00 |  | 57.01 | 77.62 |
| BOEING 747SP | 2.00 | 77.62 | 2.00 | 77.62 | 100.00 |  | 47.91 | 79.43 |
| BOEING 757-200 | 1.47 | 88.50 | 77.48 | 83.51 | 100.00 |  | 36.51 | 69.49 |
| BOEING 757-300 | 8.75 | 93.30 | 100.00 | 85.00 | 100.00 |  | 36.51 | 69.49 |
| BOEING 767-200 | 2.00 | 83.16 | 2.00 | 83.16 | 100.00 |  | 38.51 | 72.03 |
| BOEING 767-200ER | 32.70 | 94.70 | 100.00 | 85.00 | 100.00 |  | 38.51 | 72.03 |
| BOEING 767-300 | 2.00 | 83.16 | 2.00 | 83.16 | 100.00 |  | 38.51 | 72.03 |
| BOEING 767-300ER/F | 8.79 | 88.16 | 53.73 | 84.00 | 100.00 |  | 38.51 | 72.03 |
| BOEING 767-400ER | 8.79 | 88.16 | 53.73 | 84.00 | 100.00 |  | 38.51 | 72.03 |
| BOEING 777-200 | 0.00 | 86.93 | 100.00 | 85.00 | 100.00 |  | 40.58 | 78.11 |
| BOEING 777-200ER | 3.95 | 88.53 | 100.00 | 85.00 | 100.00 |  | 40.58 | 78.11 |
| BOEING 777-300 | 0.00 | 86.93 | 100.00 | 85.00 | 100.00 |  | 40.58 | 78.11 |
| BOEING-VERTOL MOD-234 CHINOOK | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| BOMBARDIER DASH 8 Q100/200 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| BOMBARDIER DASH 8 Q400 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| BOMBARDIER REGIONAL JET 100/200 | 29.10 | 90.91 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| BOMBARDIER REGIONAL JET RJ700 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| BOMBARDIER REGIONAL JET RJ700ER | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| BRISTOL 170 FREIGHTER | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CANADAIR CL-44 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 62.83 | 70.04 |
| CANADAIR CL-600-604 CHALLENGER | 29.10 | 90.91 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| CANADAIR GLOBAL EXPRESS | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 37.78 | 69.20 |
| CESSNA 150 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 172 SKYHAWK | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 180 SKYWAGON | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 206 STATIONAIR | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 208 CARAVAN I | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 210 CENTURION | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 310 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 336/337 SKYMASTER | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 337 SUPER SKYMASTER | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 340 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 401/402/411/421 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 404 TITAN | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 414A CHANCELLOR | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 425 CONQUEST I | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 441 CONQUEST II | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 500 CITATION I | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 525 CITATIONJET | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 550 CITATION II | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 560 CITATION V | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 650 CITATION III/VI/VII | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA 750 CITATION X | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CESSNA T303 CRUSADER | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CONVAIR 240/340/440 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| CONVAIR 580/600/640 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| CONVAIR 880 | 96.04 | 92.41 | 100.00 | 85.00 | 100.00 |  | 62.83 | 70.04 |
| DASSAULT BREGUET FALCON 50 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| DASSAULT MYSTERE-FALCON 10 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| DASSAULT MYSTERE-FALCON 20 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| DASSAULT MYSTERE-FALCON 2000 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| DASSAULT MYSTERE-FALCON 900 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| DASSAULT MYSTERE-FALCON 900EX | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| DASSAULT-BREGUET MERCURE | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 37.78 | 69.20 |
| DE HAVILLAND DASH 8-300/Q300 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| DE HAVILLAND DASH 8-100 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| DE HAVILLAND DH6 TWIN OTTER | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| DE HAVILLAND DHC-7 DASH-7 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| DE HAVILLAND DRAGON RAPIDE | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| DE HAVILLAND TIGER MOTH | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| DHC2 BEAVER | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| DORNIER 228-100/200 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| DORNIER 328 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| DORNIER 328 JET | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| DORNIER DO28D SKYSERVANT | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| DOUGLAS DC3 C47 DAKOTA | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| DOUGLAS DC4 SKYMASTER | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| DOUGLAS DC6/6A/6B/6C | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| EMBRAER EMB110 BANDEIRANTE | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| EMBRAER EMB120 BRASILIA | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| EMBRAER EMB121 XINGU | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| EMBRAER RJ135 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| EMBRAER RJ145 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| ENSTROM F28A/F280 SHARK | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| EUROCOPTER EC135T | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| FAIRCHILD HILLER FH 227B | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| FAIRCHILD SA-227 METRO 23 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| FAIRCHILD SA-227 METRO III | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| FOKKER 100 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| FOKKER 50 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| FOKKER 70 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| FOKKER F27 100-400/600 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| FOKKER F27-500 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| FOKKER F28-1000 | 29.10 | 90.91 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| FOKKER F28-2000 | 29.10 | 90.91 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| FOKKER F28-3000 | 29.10 | 90.91 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| FOKKER F28-4000/6000 | 29.10 | 90.91 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| GA GULFSTREAM 3 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| GAF NOMAD N22B/N24A | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| GATES LEARJET 24/25D | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| GATES LEARJET 25B | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| GATES LEARJET 31 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| GATES LEARJET 35A/36A | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| GATES LEARJET 55 LONGHORN | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| GROB 6109B | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| GRUMMAN GA-7 COUGAR | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| GULF AMERICAN GULFSTREAM I | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| GULF AMERICAN GULFSTREAM II | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| GULF AMERICAN GULFSTREAM IV | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| GULF AMERICAN GULFSTREAM V | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| HANDLEY PAGE HERALD 200 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| HANDLEY PAGE HERALD 700 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| HAWKER SIDDELEY DH 104 DOVE | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| HAWKER SIDDELEY DH114 HERON | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| HILLER UH12 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| HUGHES 269A | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| HUGHES 500 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| ILYUSHIN IL12/IL14 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| ILYUSHIN IL18 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 62.83 | 70.04 |
| ILYUSHIN IL62 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 36.51 | 69.49 |
| ILYUSHIN IL76 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 41.65 | 57.70 |
| ILYUSHIN IL86 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 41.65 | 57.70 |
| ILYUSHIN IL96-300 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 41.65 | 57.70 |
| LEARJET 45 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| LET 410 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| LOCKHEED JETSTAR II | 2.01 | 97.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| LOCKHEED L100 HERCULES | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 62.83 | 70.04 |
| LOCKHEED L1011-1/100 TRISTAR | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 38.51 | 72.03 |
| LOCKHEED L1011-200 TRISTAR | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 38.51 | 72.03 |
| LOCKHEED L1011-500 TRISTAR | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 38.51 | 72.03 |
| LOCKHEED L188 ELECTRA | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| MBB BK117 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| MBB BO105 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| Mc DONNELL DOUGLAS MD90 | 10.00 | 83.80 | 10.00 | 83.80 | 100.00 |  | 37.15 | 56.27 |
| MCDONNELL-DOUGLAS DC10-10 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 38.51 | 72.03 |
| MCDONNELL-DOUGLAS DC10-30 | 65.25 | 95.00 | 100.00 | 85.00 | 100.00 |  | 38.51 | 72.03 |
| MCDONNELL-DOUGLAS DC10-40 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 38.51 | 72.03 |
| MCDONNELL-DOUGLAS DC8-10/50 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 36.51 | 69.49 |
| MCDONNELL-DOUGLAS DC861/3 71/3 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 36.51 | 69.49 |
| MCDONNELL-DOUGLAS DC8-62/72 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 36.51 | 69.49 |
| MCDONNELL-DOUGLAS DC8-71/73 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 36.51 | 69.49 |
| MCDONNELL-DOUGLAS DC8F 54/55 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 36.51 | 69.49 |
| MCDONNELL-DOUGLAS DC9-10/15 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 37.78 | 69.20 |
| McDONNELL-DOUGLAS DC9-20 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 37.78 | 69.20 |
| MCDONNELL-DOUGLAS DC9-30 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 37.78 | 69.20 |
| MCDONNELL-DOUGLAS DC9-40 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 37.78 | 69.20 |
| MCDONNELL-DOUGLAS DC9-50 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 37.78 | 69.20 |
| MCDONNELL-DOUGLAS MD11 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 38.51 | 72.03 |
| MCDONNELL-DOUGLAS MD80-MD83 | 6.33 | 93.20 | 100.00 | 85.00 | 100.00 |  | 37.78 | 69.20 |
| MCDONNELL-DOUGLAS MD87 | 6.33 | 93.20 | 100.00 | 85.00 | 100.00 |  | 37.78 | 69.20 |
| MCDONNELL-DOUGLAS MD88 | 6.33 | 93.20 | 100.00 | 85.00 | 100.00 |  | 37.78 | 69.20 |
| MITSUBISHI MU2 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| MOONEY M20J | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| NAMC YS11 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| NORD 2501 NORTALAS | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| PARTENAVIA P68B/C | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| PIAGGIO P.180 AVANTI | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| PIAGGIO P166-DL3 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| PILATUS BN-2A ISLANDER | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| PILATUS BN-2A TRISLANDER MK3 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| PIPER PA18 SUPER CUB | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| PIPER PA23 AZTEC/APACHE | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| PIPER PA24 COMMANCHE | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| PIPER PA28 CHEROKEE SRS/PA32 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| PIPER PA30/PA39 TWIN COMANCHE | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| PIPER PA31/P NAVAJO CHIEFTAIN | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| PIPER PA31T CHEYENNE I/II | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| PIPER PA34 SENECA II | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| PIPER PA42 CHEYENNE III/IV | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| PIPER PA44 SEMINOLE | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| REIMS-CESSNA F406/CARAVAN II | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| RILEY TURBO-EXEC 400 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| RO TURBO COMMANDER 680T-690 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| ROBINSON R22 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| ROCKWELL SABRELINER SERIES | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| ROCKWELL SHRIKE/COURSER/COMMAN | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| ROCKWELL TURBO COMMANDER | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| SAAB 2000 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| SAAB FAIRCHILD 340 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| SHORTS 330 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| SHORTS 360 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| SHORTS BELFAST | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 62.83 | 70.04 |
| SHORTS SC7 SKYLINER | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| SHORTS SC7 SKYVAN | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| SIKORSKY S61N | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| SIKORSKY S76 SPIRIT | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| SOCATA TB10 TOBAGO | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| SWEARINGEN MERLIN IIA/IIB/IIIB | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| SWEARINGEN MERLIN IVA | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| SWEARINGEN METRO II | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| TUPOLEV TU104 | 6.33 | 93.20 | 100.00 | 85.00 | 100.00 |  | 37.78 | 69.20 |
| TUPOLEV TU124 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| TUPOLEV TU134 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 37.78 | 69.20 |
| TUPOLEV TU154A/B | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 37.15 | 56.27 |
| TUPOLEV TU154M | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 37.78 | 69.20 |
| TUPOLEV TU204 | 1.47 | 88.50 | 77.48 | 83.51 | 100.00 |  | 36.51 | 69.49 |
| V953C MERCHANTMAN | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 62.83 | 70.04 |
| VICKERS VISCOUNT 700 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| VICKERS VISCOUNT 800 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 90.00 | 120.00 |
| WESTLAND 30 SRS 100 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 0.00 | 390.00 |
| YAKOVLEV YAK-40 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 50.00 | 80.00 |
| YAKOVLEV YAK-42 | 100.00 | 100.00 | 100.00 | 85.00 | 100.00 |  | 37.15 | 56.27 |

Notes

- Time In Take-off Roll data are commercially confidential

Appendix 3 Emission factors and fuel flow rates

CONTENTS

| Table A3.1a | Emission factors and fuel flow rates – Heathrow |
| --- | --- |
| Table A3.1b | Emission factors and fuel flow rates – Heathrow |
| Table A3.2a | Emission factors and fuel flow rates – Gatwick |
| Table A3.2b | Emission factors and fuel flow rates – Gatwick |
| Table A3.3 | APU emission factors and fuel flow rates |
| Table A3.4 | Cruise emission and fuel consumptions |

Table A3.1a	Emission factors and fuel flow rates – Heathrow

| CAA Aircraft Type | HC_100 | HC_85 | HC_30 | HC_7 | CO_100 | CO_85 | CO_30 | CO_7 | NOx_100 | NOx_85 | NOx_30 | NOx_7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AEROSPACELINES B377SUPER GUPPY | 0.28 | 0.89 | 1.96 | 17.61 | 2.00 | 2.10 | 5.10 | 43.60 | 8.88 | 9.22 | 7.49 | 3.52 |
| AEROSPATIALE (NORD)262 | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| AEROSPATIALE AS332 SUPER PUMA (L1/L2) | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| AEROSPATIALE AS355 ECUREUIL 2 | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| AEROSPATIALE CARAVELLE 10B/10R | 0.09 | 0.12 | 0.18 | 3.69 | 0.12 | 0.63 | 2.65 | 31.77 | 22.70 | 17.30 | 7.20 | 3.60 |
| AEROSPATIALE CARAVELLE 12 | 0.09 | 0.12 | 0.18 | 3.69 | 0.12 | 0.63 | 2.65 | 31.77 | 22.70 | 17.30 | 7.20 | 3.60 |
| AEROSPATIALE CARAVELLE 6/6R | 0.09 | 0.12 | 0.18 | 3.69 | 0.12 | 0.63 | 2.65 | 31.77 | 22.70 | 17.30 | 7.20 | 3.60 |
| AEROSPATIALE CORVETTE | 0.09 | 0.19 | 5.15 | 40.00 | 2.10 | 3.18 | 32.00 | 97.00 | 9.23 | 8.56 | 5.29 | 2.63 |
| AEROSPATIALE SA315 LAMA | 0.26 | 0.37 | 5.18 | 20.16 | 7.80 | 9.00 | 48.60 | 97.30 | 6.60 | 5.96 | 2.24 | 1.43 |
| AEROSPATIALE SA330 PUMA | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| AEROSPATIALE SA341 GAZELLE | 0.26 | 0.37 | 5.18 | 20.16 | 7.80 | 9.00 | 48.60 | 97.30 | 6.60 | 5.96 | 2.24 | 1.43 |
| AEROSPATIALE SA350 ECUREUIL | 0.26 | 0.37 | 5.18 | 20.16 | 7.80 | 9.00 | 48.60 | 97.30 | 6.60 | 5.96 | 2.24 | 1.43 |
| AEROSPATIALE SA365 DAUPHIN | 0.26 | 0.37 | 5.18 | 20.16 | 7.80 | 9.00 | 48.60 | 97.30 | 6.60 | 5.96 | 2.24 | 1.43 |
| AEROSPATIALE SA365 DAUPHIN/AMB | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| AGUSTA A109A | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| AIRBUS A300-600 | 0.05 | 0.06 | 0.12 | 1.92 | 0.12 | 0.12 | 1.93 | 20.41 | 28.83 | 21.96 | 12.45 | 4.62 |
| AIRBUS A300B1/B2 | 0.14 | 0.15 | 0.28 | 2.99 | 0.44 | 0.44 | 3.65 | 24.88 | 29.05 | 25.50 | 10.09 | 3.39 |
| AIRBUS A300B4-100/200 | 0.14 | 0.15 | 0.28 | 2.72 | 0.45 | 0.45 | 3.71 | 24.04 | 28.97 | 25.50 | 10.16 | 3.40 |
| AIRBUS A300F4 | 0.14 | 0.15 | 0.28 | 2.72 | 0.45 | 0.45 | 3.71 | 24.04 | 28.97 | 25.50 | 10.16 | 3.40 |
| AIRBUS A310-202 | 0.25 | 0.30 | 0.37 | 5.22 | 0.80 | 0.88 | 2.68 | 26.08 | 28.87 | 25.48 | 10.97 | 3.63 |
| AIRBUS A310-300 | 0.05 | 0.06 | 0.13 | 2.37 | 0.11 | 0.13 | 2.38 | 23.08 | 24.29 | 19.65 | 11.96 | 4.42 |
| AIRBUS A319 | 0.05 | 0.05 | 0.10 | 0.46 | 0.60 | 0.70 | 2.66 | 14.42 | 24.37 | 20.67 | 8.71 | 4.46 |
| AIRBUS A320-100/200 | 0.12 | 0.12 | 1.40 | 1.18 | 0.79 | 1.20 | 6.90 | 17.76 | 25.25 | 20.72 | 8.54 | 4.45 |
| AIRBUS A321 | 0.07 | 0.10 | 0.92 | 1.66 | 0.58 | 0.80 | 4.85 | 17.03 | 35.82 | 27.12 | 10.53 | 4.93 |
| AIRBUS A330-200 | 0.00 | 0.00 | 0.02 | 1.64 | 0.25 | 0.22 | 0.98 | 18.49 | 35.18 | 27.19 | 10.76 | 4.65 |
| AIRBUS A330-300 | 0.00 | 0.00 | 0.01 | 1.46 | 0.20 | 0.16 | 0.90 | 17.94 | 34.34 | 26.41 | 10.32 | 4.71 |
| AIRBUS A340-200 | 0.01 | 0.01 | 0.06 | 5.00 | 1.00 | 0.85 | 1.40 | 30.93 | 37.67 | 29.05 | 10.67 | 4.28 |
| AIRBUS A340-300 | 0.01 | 0.01 | 0.07 | 5.38 | 0.96 | 0.82 | 1.59 | 32.64 | 34.82 | 27.22 | 10.31 | 4.23 |
| AIRBUS A340-600 | 0.00 | 0.00 | 0.00 | 0.10 | 0.02 | 0.25 | 0.46 | 10.30 | 44.91 | 32.76 | 11.78 | 6.19 |
| AIRSHIP INDUSTRIES SKYSHIP 500 | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| AIRSHIP INDUSTRIES SKYSHIP 600 | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| ANTONOV AN-12 | 0.28 | 0.89 | 1.96 | 17.61 | 2.00 | 2.10 | 5.10 | 43.60 | 8.88 | 9.22 | 7.49 | 3.52 |
| ANTONOV AN-124 | 0.12 | 0.12 | 0.20 | 0.30 | 0.35 | 0.40 | 0.90 | 6.90 | 37.00 | 31.50 | 11.80 | 5.80 |
| ANTONOV AN-24 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| ANTONOV AN26B/32 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| ANTONOV AN72 | 0.12 | 0.14 | 1.50 | 43.60 | 2.70 | 3.20 | 14.50 | 60.30 | 19.10 | 16.30 | 7.00 | 3.60 |
| ARMSTRONG WHITWORTH ARGOSY | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| ATR42-300 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| ATR42-500 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| ATR72 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| AVROLINER RJ100/115 | 0.01 | 0.01 | 0.12 | 4.72 | 0.20 | 0.30 | 4.43 | 37.83 | 14.52 | 12.02 | 6.39 | 3.28 |
| AVROLINER RJ70 | 0.01 | 0.01 | 0.12 | 4.72 | 0.20 | 0.30 | 4.43 | 37.83 | 14.52 | 12.02 | 6.39 | 3.28 |
| AVROLINER RJ85/QT | 0.01 | 0.01 | 0.12 | 4.72 | 0.20 | 0.30 | 4.43 | 37.83 | 14.52 | 12.02 | 6.39 | 3.28 |
| BAC/AEROSPATIALE CONCORDE | 2.90 | 1.70 | 11.40 | 33.40 | 29.00 | 19.90 | 52.90 | 100.10 | 9.50 | 9.30 | 3.50 | 1.70 |
| BAE (HS) 748 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| BAE 146-100 | 0.06 | 0.05 | 0.22 | 5.39 | 0.30 | 0.25 | 7.10 | 40.93 | 13.35 | 10.56 | 6.60 | 3.78 |
| BAE 146-200/QT | 0.06 | 0.05 | 0.22 | 5.39 | 0.30 | 0.25 | 7.10 | 40.93 | 13.35 | 10.56 | 6.60 | 3.78 |
| BAE 146-300 | 0.06 | 0.05 | 0.22 | 5.39 | 0.30 | 0.25 | 7.10 | 40.93 | 13.35 | 10.56 | 6.60 | 3.78 |
| BAE ATP | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| BAE JETSTREAM 31/32 | 0.11 | 0.15 | 0.64 | 79.11 | 0.80 | 1.00 | 7.00 | 61.50 | 12.36 | 11.86 | 9.92 | 2.86 |
| BAE JETSTREAM 41 | 0.11 | 0.15 | 0.64 | 79.11 | 0.80 | 1.00 | 7.00 | 61.50 | 12.36 | 11.86 | 9.92 | 2.86 |
| BAE(BAC)111-200 | 0.09 | 0.12 | 0.18 | 3.69 | 0.12 | 0.63 | 2.65 | 31.77 | 22.70 | 17.30 | 7.20 | 3.60 |
| BAE(BAC)111-300/400/475 | 0.09 | 0.12 | 0.18 | 3.69 | 0.12 | 0.63 | 2.65 | 31.77 | 22.70 | 17.30 | 7.20 | 3.60 |
| BAE(BAC)111-500 | 0.09 | 0.12 | 0.18 | 3.69 | 0.12 | 0.63 | 2.65 | 31.77 | 22.70 | 17.30 | 7.20 | 3.60 |
| BAE(HS)125 | 0.06 | 0.07 | 1.41 | 9.04 | 1.13 | 1.62 | 15.56 | 47.70 | 19.15 | 16.02 | 6.92 | 3.72 |
| BAE125-1000 | 0.06 | 0.07 | 1.41 | 9.04 | 1.13 | 1.62 | 15.56 | 47.70 | 19.15 | 16.02 | 6.92 | 3.72 |
| BEAGLE B206 | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| BEECH KINGAIR 100 | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| BEECH KINGAIR 90 | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| BEECH KINGAIR C90A | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| BEECHCRAFT 18/SUPER H18 | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| BEECHCRAFT 1900C/D AIRLINER | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| BEECHCRAFT 200 SUPERKING AIR | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| BEECHCRAFT 300 SUPER KING AIR | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| BEECHCRAFT 400 BEECHJET | 0.00 | 1.30 | 11.70 | 119.10 | 0.00 | 1.15 | 38.60 | 119.20 | 11.13 | 10.08 | 4.93 | 1.66 |
| BEECHCRAFT 50 TWIN BONANZA | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| BEECHCRAFT 76 DUCHESS | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| BEECHCRAFT 95 TRAVEL AIR | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| BEECHCRAFT 99/99A | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| BEECHCRAFT BARON MOD 55/58/58P | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| BEECHCRAFT DUKE | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| BEECHCRAFT QUEEN AIR 65/80 | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| BEECHCRAFT STARSHIP MODEL 2000 | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| BELL 206B JET RANGER | 0.26 | 0.37 | 5.18 | 20.16 | 7.80 | 9.00 | 48.60 | 97.30 | 6.60 | 5.96 | 2.24 | 1.43 |
| BELL 206L LONG RANGER | 0.26 | 0.37 | 5.18 | 20.16 | 7.80 | 9.00 | 48.60 | 97.30 | 6.60 | 5.96 | 2.24 | 1.43 |
| BELL 212/412 | 0.26 | 0.37 | 5.18 | 20.16 | 7.80 | 9.00 | 48.60 | 97.30 | 6.60 | 5.96 | 2.24 | 1.43 |
| BELL 222 | 0.26 | 0.37 | 5.18 | 20.16 | 7.80 | 9.00 | 48.60 | 97.30 | 6.60 | 5.96 | 2.24 | 1.43 |
| BELL 47G | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| BELL MODEL 214ST | 0.26 | 0.37 | 5.18 | 20.16 | 7.80 | 9.00 | 48.60 | 97.30 | 6.60 | 5.96 | 2.24 | 1.43 |
| BOEING 707 ALL SERIES | 0.50 | 0.40 | 2.10 | 123.00 | 0.89 | 1.90 | 19.50 | 138.99 | 12.69 | 9.59 | 5.30 | 2.20 |
| BOEING 707-120/121B | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| BOEING 717-200 | 0.00 | 0.02 | 0.01 | 0.21 | 0.78 | 0.75 | 3.76 | 16.27 | 23.97 | 18.65 | 11.19 | 5.37 |
| BOEING 720B | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| BOEING 727-100/100C | 0.40 | 0.50 | 1.60 | 10.60 | 1.50 | 2.00 | 10.50 | 35.50 | 17.10 | 13.50 | 5.50 | 2.70 |
| BOEING 727-200/200 ADVANCED | 0.45 | 0.46 | 1.66 | 9.61 | 1.23 | 1.63 | 9.16 | 33.33 | 18.27 | 14.35 | 5.78 | 2.91 |
| BOEING 737-100 | 0.25 | 0.27 | 1.32 | 8.39 | 0.82 | 1.06 | 7.42 | 27.91 | 18.85 | 14.67 | 6.14 | 3.04 |
| BOEING 737-200 | 0.37 | 0.43 | 1.10 | 5.37 | 0.94 | 1.09 | 4.61 | 19.91 | 18.99 | 14.62 | 6.17 | 3.09 |
| BOEING 737-300 | 0.04 | 0.05 | 0.08 | 1.91 | 0.89 | 0.92 | 3.48 | 30.95 | 19.07 | 16.53 | 8.66 | 4.08 |
| BOEING 737-400 | 0.03 | 0.04 | 0.07 | 1.43 | 0.90 | 0.90 | 3.11 | 26.90 | 20.66 | 17.77 | 9.09 | 4.29 |
| BOEING 737-500 | 0.04 | 0.05 | 0.08 | 2.21 | 0.90 | 0.95 | 3.74 | 33.79 | 17.94 | 15.69 | 8.36 | 3.93 |
| BOEING 737-600 | 0.10 | 0.10 | 0.10 | 3.05 | 0.58 | 0.50 | 3.13 | 25.60 | 20.85 | 17.62 | 9.56 | 4.32 |
| BOEING 737-700 | 0.11 | 0.11 | 0.15 | 5.59 | 0.49 | 0.63 | 2.87 | 25.58 | 23.34 | 19.10 | 9.92 | 4.43 |
| BOEING 737-800 | 0.10 | 0.10 | 0.10 | 2.20 | 0.32 | 0.60 | 1.96 | 20.72 | 26.70 | 21.30 | 10.38 | 4.52 |
| BOEING 737-900 | 0.10 | 0.10 | 0.10 | 1.90 | 0.20 | 0.60 | 1.60 | 18.80 | 28.80 | 22.50 | 10.80 | 4.70 |
| BOEING 747-100/100F | 0.00 | 0.00 | 0.60 | 25.90 | 0.90 | 0.90 | 5.80 | 68.60 | 41.70 | 31.50 | 9.10 | 3.20 |
| BOEING 747-200B | 0.21 | 0.15 | 0.30 | 3.78 | 0.84 | 0.61 | 1.83 | 16.72 | 44.45 | 32.06 | 9.14 | 3.85 |
| BOEING 747-200B (COMBI) | 0.08 | 0.08 | 0.38 | 12.39 | 0.65 | 0.65 | 4.50 | 42.98 | 36.04 | 29.67 | 9.82 | 3.36 |
| BOEING 747-200C/200F | 0.21 | 0.20 | 0.31 | 11.84 | 0.22 | 0.20 | 1.70 | 52.33 | 32.03 | 25.87 | 7.84 | 3.02 |
| BOEING 747-300(STRETCH UP DK) | 0.06 | 0.19 | 2.66 | 32.31 | 0.69 | 1.21 | 11.62 | 52.24 | 41.65 | 31.14 | 9.73 | 3.55 |
| BOEING 747-300M (COMBI) | 0.14 | 0.15 | 0.28 | 2.72 | 0.45 | 0.45 | 3.71 | 24.04 | 28.97 | 25.50 | 10.16 | 3.40 |
| BOEING 747-400 | 0.14 | 0.13 | 0.17 | 2.35 | 0.39 | 0.27 | 1.35 | 20.88 | 40.54 | 29.63 | 10.71 | 4.42 |
| BOEING 747-400F | 0.08 | 0.09 | 0.16 | 4.75 | 0.48 | 0.56 | 1.89 | 29.96 | 30.44 | 24.24 | 11.97 | 3.93 |
| BOEING 747-400M (COMBI) | 0.06 | 0.06 | 0.13 | 2.55 | 0.18 | 0.20 | 2.05 | 22.60 | 26.67 | 21.14 | 12.31 | 4.48 |
| BOEING 747SP | 0.08 | 0.08 | 1.18 | 34.35 | 0.15 | 0.15 | 7.29 | 81.03 | 39.21 | 29.01 | 7.86 | 3.12 |
| BOEING 757-200 | 0.09 | 0.04 | 0.15 | 0.60 | 0.38 | 0.29 | 2.10 | 19.93 | 25.48 | 19.62 | 7.85 | 4.14 |
| BOEING 757-300 | 0.07 | 0.00 | 0.05 | 0.14 | 0.33 | 0.26 | 2.43 | 18.24 | 25.88 | 19.30 | 8.65 | 4.58 |
| BOEING 767-200 | 0.15 | 0.12 | 0.13 | 1.26 | 0.51 | 0.48 | 1.36 | 8.94 | 38.48 | 29.98 | 9.82 | 4.10 |
| BOEING 767-200ER | 0.16 | 0.13 | 0.14 | 1.50 | 0.53 | 0.51 | 1.43 | 9.95 | 38.16 | 29.98 | 9.93 | 4.07 |
| BOEING 767-300 | 0.31 | 0.30 | 0.33 | 0.85 | 0.79 | 0.35 | 1.09 | 12.58 | 61.90 | 43.73 | 10.50 | 4.77 |
| BOEING 767-300ER/F | 0.08 | 0.08 | 0.15 | 2.61 | 0.29 | 0.31 | 1.78 | 22.06 | 33.66 | 25.37 | 12.37 | 4.42 |
| BOEING 767-400ER | 0.05 | 0.05 | 0.11 | 1.64 | 0.08 | 0.08 | 1.91 | 19.12 | 28.10 | 21.55 | 12.62 | 4.74 |
| BOEING 777-200 | 0.06 | 0.05 | 0.10 | 1.33 | 0.15 | 0.14 | 0.83 | 15.78 | 46.76 | 35.54 | 13.91 | 5.18 |
| BOEING 777-200ER | 0.03 | 0.02 | 0.03 | 0.89 | 0.21 | 0.18 | 0.77 | 14.67 | 50.35 | 36.78 | 13.71 | 5.35 |
| BOEING 777-300 | 0.01 | 0.00 | 0.00 | 0.70 | 0.28 | 0.20 | 0.57 | 13.07 | 45.70 | 33.30 | 11.58 | 5.33 |
| BOEING-VERTOL MOD-234 CHINOOK | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| BOMBARDIER DASH 8 Q100/200 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| BOMBARDIER DASH 8 Q400 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| BOMBARDIER REGIONAL JET 100/200 | 0.06 | 0.05 | 0.13 | 4.69 | 0.00 | 0.00 | 1.88 | 47.59 | 11.28 | 9.68 | 6.63 | 3.72 |
| BOMBARDIER REGIONAL JET RJ700 | 0.06 | 0.05 | 0.13 | 4.69 | 0.00 | 0.00 | 1.88 | 47.59 | 11.28 | 9.68 | 6.63 | 3.72 |
| BOMBARDIER REGIONAL JET RJ700ER | 0.02 | 0.02 | 0.06 | 0.08 | 0.41 | 0.50 | 2.91 | 24.92 | 14.67 | 12.82 | 11.10 | 4.31 |
| BRISTOL 170 FREIGHTER | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| CANADAIR CL-44 | 0.28 | 0.89 | 1.96 | 17.61 | 2.00 | 2.10 | 5.10 | 43.60 | 8.88 | 9.22 | 7.49 | 3.52 |
| CANADAIR CL-600-604 CHALLENGER | 0.06 | 0.06 | 0.13 | 3.95 | 0.00 | 0.00 | 1.90 | 42.60 | 11.61 | 10.14 | 6.86 | 3.82 |
| CANADAIR GLOBAL EXPRESS | 0.12 | 0.14 | 1.50 | 43.60 | 2.70 | 3.20 | 14.50 | 60.30 | 19.10 | 16.30 | 7.00 | 3.60 |
| CESSNA 150 | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| CESSNA 172 SKYHAWK | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| CESSNA 180 SKYWAGON | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| CESSNA 206 STATIONAIR | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| CESSNA 208 CARAVAN I | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| CESSNA 210 CENTURION | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| CESSNA 310 | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| CESSNA 336/337 SKYMASTER | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| CESSNA 337 SUPER SKYMASTER | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| CESSNA 340 | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| CESSNA 401/402/411/421 | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| CESSNA 404 TITAN | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| CESSNA 414A CHANCELLOR | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| CESSNA 425 CONQUEST I | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| CESSNA 441 CONQUEST II | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| CESSNA 500 CITATION I | 0.01 | 0.01 | 4.43 | 50.50 | 2.65 | 3.50 | 40.50 | 132.00 | 7.60 | 6.77 | 3.44 | 1.75 |
| CESSNA 525 CITATIONJET | 0.01 | 0.01 | 4.43 | 50.50 | 2.65 | 3.50 | 40.50 | 132.00 | 7.60 | 6.77 | 3.44 | 1.75 |
| CESSNA 550 CITATION II | 0.08 | 0.23 | 5.95 | 44.18 | 2.13 | 3.25 | 33.27 | 99.04 | 9.28 | 8.65 | 5.29 | 2.52 |
| CESSNA 560 CITATION V | 0.00 | 0.68 | 15.96 | 96.89 | 2.50 | 4.15 | 49.14 | 124.55 | 9.94 | 9.79 | 5.23 | 1.09 |
| CESSNA 650 CITATION III/VI/VII | 0.06 | 0.09 | 1.74 | 11.02 | 1.16 | 1.68 | 16.32 | 49.43 | 18.94 | 15.88 | 6.88 | 3.66 |
| CESSNA 750 CITATION X | 0.29 | 0.33 | 0.83 | 5.84 | 0.97 | 1.19 | 4.50 | 29.88 | 16.92 | 14.56 | 6.63 | 3.24 |
| CESSNA T303 CRUSADER | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| CONVAIR 240/340/440 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| CONVAIR 580/600/640 | 0.25 | 0.29 | 0.64 | 2.51 | 0.75 | 0.92 | 3.28 | 17.35 | 20.54 | 17.47 | 7.79 | 3.83 |
| CONVAIR 880 | 0.06 | 0.05 | 0.22 | 5.39 | 0.30 | 0.25 | 7.10 | 40.93 | 13.35 | 10.56 | 6.60 | 3.78 |
| DASSAULT BREGUET FALCON 50 | 0.06 | 0.07 | 1.41 | 9.04 | 1.13 | 1.62 | 15.56 | 47.70 | 19.15 | 16.02 | 6.92 | 3.72 |
| DASSAULT MYSTERE-FALCON 10 | 0.11 | 0.13 | 4.26 | 20.04 | 1.39 | 2.03 | 22.38 | 58.60 | 15.25 | 13.08 | 5.90 | 2.82 |
| DASSAULT MYSTERE-FALCON 20 | 0.10 | 0.10 | 1.40 | 18.00 | 22.00 | 27.00 | 62.00 | 155.00 | 5.60 | 4.40 | 1.80 | 0.90 |
| DASSAULT MYSTERE-FALCON 2000 | 0.10 | 0.10 | 1.40 | 18.00 | 22.00 | 27.00 | 62.00 | 155.00 | 5.60 | 4.40 | 1.80 | 0.90 |
| DASSAULT MYSTERE-FALCON 900 | 0.06 | 0.07 | 1.41 | 9.04 | 1.13 | 1.62 | 15.56 | 47.70 | 19.15 | 16.02 | 6.92 | 3.72 |
| DASSAULT MYSTERE-FALCON 900EX | 0.06 | 0.07 | 1.41 | 9.04 | 1.13 | 1.62 | 15.56 | 47.70 | 19.15 | 16.02 | 6.92 | 3.72 |
| DASSAULT-BREGUET MERCURE | 0.12 | 0.14 | 1.50 | 43.60 | 2.70 | 3.20 | 14.50 | 60.30 | 19.10 | 16.30 | 7.00 | 3.60 |
| DE HAVILLAND DASH 8-300/Q300 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| DE HAVILLAND DASH 8-100 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| DE HAVILLAND DH6 TWIN OTTER | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| DE HAVILLAND DHC-7 DASH-7 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| DE HAVILLAND DRAGON RAPIDE | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| DE HAVILLAND TIGER MOTH | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| DHC2 BEAVER | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| DORNIER 228-100/200 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| DORNIER 328 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| DORNIER 328 JET | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| DORNIER DO28D SKYSERVANT | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| DOUGLAS DC3 C47 DAKOTA | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| DOUGLAS DC4 SKYMASTER | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| DOUGLAS DC6/6A/6B/6C | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| EMBRAER EMB110 BANDEIRANTE | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| EMBRAER EMB120 BRASILIA | 0.26 | 0.30 | 0.72 | 3.10 | 0.79 | 0.97 | 3.80 | 22.43 | 19.78 | 16.84 | 7.22 | 3.52 |
| EMBRAER EMB121 XINGU | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| EMBRAER RJ135 | 0.25 | 0.29 | 0.68 | 2.77 | 0.77 | 0.94 | 3.51 | 19.60 | 20.20 | 17.19 | 7.54 | 3.69 |
| EMBRAER RJ145 | 0.27 | 0.31 | 0.75 | 3.33 | 0.82 | 1.00 | 4.12 | 23.19 | 19.62 | 16.70 | 7.15 | 3.49 |
| ENSTROM F28A/F280 SHARK | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| EUROCOPTER EC135T | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| FAIRCHILD HILLER FH 227B | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| FAIRCHILD SA-227 METRO 23 | 0.11 | 0.15 | 0.64 | 79.11 | 0.80 | 1.00 | 7.00 | 61.50 | 12.36 | 11.86 | 9.92 | 2.86 |
| FAIRCHILD SA-227 METRO III | 0.11 | 0.15 | 0.64 | 79.11 | 0.80 | 1.00 | 7.00 | 61.50 | 12.36 | 11.86 | 9.92 | 2.86 |
| FOKKER 100 | 0.11 | 0.22 | 0.11 | 1.23 | 0.69 | 0.48 | 1.76 | 22.36 | 16.45 | 12.94 | 5.26 | 2.50 |
| FOKKER 50 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| FOKKER 70 | 0.80 | 0.30 | 0.90 | 3.40 | 0.70 | 0.80 | 3.90 | 24.10 | 21.10 | 16.80 | 5.70 | 2.50 |
| FOKKER F27 100-400/600 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| FOKKER F27-500 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| FOKKER F28-1000 | 0.06 | 0.06 | 0.13 | 3.95 | 0.00 | 0.00 | 1.90 | 42.60 | 11.61 | 10.14 | 6.86 | 3.82 |
| FOKKER F28-2000 | 0.06 | 0.06 | 0.13 | 3.95 | 0.00 | 0.00 | 1.90 | 42.60 | 11.61 | 10.14 | 6.86 | 3.82 |
| FOKKER F28-3000 | 0.06 | 0.06 | 0.13 | 3.95 | 0.00 | 0.00 | 1.90 | 42.60 | 11.61 | 10.14 | 6.86 | 3.82 |
| FOKKER F28-4000/6000 | 0.06 | 0.06 | 0.13 | 3.95 | 0.00 | 0.00 | 1.90 | 42.60 | 11.61 | 10.14 | 6.86 | 3.82 |
| GA GULFSTREAM 3 | 0.09 | 0.12 | 0.18 | 3.69 | 0.12 | 0.63 | 2.65 | 31.77 | 22.70 | 17.30 | 7.20 | 3.60 |
| GAF NOMAD N22B/N24A | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| GATES LEARJET 24/25D | 0.11 | 0.13 | 4.26 | 20.04 | 1.39 | 2.03 | 22.38 | 58.60 | 15.25 | 13.08 | 5.90 | 2.82 |
| GATES LEARJET 25B | 0.11 | 0.13 | 4.26 | 20.04 | 1.39 | 2.03 | 22.38 | 58.60 | 15.25 | 13.08 | 5.90 | 2.82 |
| GATES LEARJET 31 | 0.11 | 0.13 | 4.26 | 20.04 | 1.39 | 2.03 | 22.38 | 58.60 | 15.25 | 13.08 | 5.90 | 2.82 |
| GATES LEARJET 35A/36A | 0.11 | 0.13 | 4.26 | 20.04 | 1.39 | 2.03 | 22.38 | 58.60 | 15.25 | 13.08 | 5.90 | 2.82 |
| GATES LEARJET 55 LONGHORN | 0.06 | 0.07 | 1.41 | 9.04 | 1.13 | 1.62 | 15.56 | 47.70 | 19.15 | 16.02 | 6.92 | 3.72 |
| GROB 6109B | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| GRUMMAN GA-7 COUGAR | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| GULF AMERICAN GULFSTREAM I | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| GULF AMERICAN GULFSTREAM II | 0.09 | 0.12 | 0.18 | 3.69 | 0.12 | 0.63 | 2.65 | 31.77 | 22.70 | 17.30 | 7.20 | 3.60 |
| GULF AMERICAN GULFSTREAM IV | 0.80 | 0.30 | 0.90 | 3.40 | 0.70 | 0.80 | 3.90 | 24.10 | 21.10 | 16.80 | 5.70 | 2.50 |
| GULF AMERICAN GULFSTREAM V | 0.02 | 0.02 | 0.05 | 1.12 | 1.04 | 0.93 | 4.81 | 28.00 | 18.73 | 15.03 | 7.67 | 4.67 |
| HANDLEY PAGE HERALD 200 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| HANDLEY PAGE HERALD 700 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| HAWKER SIDDELEY DH 104 DOVE | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| HAWKER SIDDELEY DH114 HERON | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| HILLER UH12 | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| HUGHES 269A | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| HUGHES 500 | 0.26 | 0.37 | 5.18 | 20.16 | 7.80 | 9.00 | 48.60 | 97.30 | 6.60 | 5.96 | 2.24 | 1.43 |
| ILYUSHIN IL12/IL14 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| ILYUSHIN IL18 | 0.28 | 0.89 | 1.96 | 17.61 | 2.00 | 2.10 | 5.10 | 43.60 | 8.88 | 9.22 | 7.49 | 3.52 |
| ILYUSHIN IL62 | 0.30 | 0.40 | 1.20 | 10.50 | 2.80 | 3.70 | 11.80 | 54.00 | 16.30 | 12.60 | 5.10 | 2.70 |
| ILYUSHIN IL76 | 0.50 | 0.60 | 1.20 | 52.00 | 3.90 | 4.20 | 9.30 | 54.40 | 12.80 | 12.10 | 5.10 | 2.70 |
| ILYUSHIN IL86 | 0.50 | 0.60 | 1.20 | 52.00 | 3.90 | 4.20 | 9.30 | 54.40 | 12.80 | 12.10 | 5.10 | 2.70 |
| ILYUSHIN IL96-300 | 0.12 | 0.12 | 0.20 | 0.30 | 0.35 | 0.40 | 0.90 | 6.90 | 37.00 | 31.50 | 11.80 | 5.80 |
| LEARJET 45 | 0.11 | 0.13 | 4.26 | 20.04 | 1.39 | 2.03 | 22.38 | 58.60 | 15.25 | 13.08 | 5.90 | 2.82 |
| LET 410 | 0.00 | 0.00 | 2.19 | 50.17 | 1.00 | 1.20 | 23.30 | 64.00 | 7.81 | 7.00 | 8.37 | 2.43 |
| LOCKHEED JETSTAR II | 0.01 | 0.01 | 0.12 | 4.72 | 0.20 | 0.30 | 4.43 | 37.83 | 14.52 | 12.02 | 6.39 | 3.28 |
| LOCKHEED L100 HERCULES | 0.28 | 0.89 | 1.96 | 17.61 | 2.00 | 2.10 | 5.10 | 43.60 | 8.88 | 9.22 | 7.49 | 3.52 |
| LOCKHEED L1011-1/100 TRISTAR | 0.15 | 0.25 | 5.96 | 67.75 | 0.78 | 1.68 | 20.65 | 88.99 | 37.33 | 26.89 | 8.18 | 2.86 |
| LOCKHEED L1011-200 TRISTAR | 0.15 | 0.25 | 5.96 | 67.75 | 0.78 | 1.68 | 20.65 | 88.99 | 37.33 | 26.89 | 8.18 | 2.86 |
| LOCKHEED L1011-500 TRISTAR | 0.39 | 0.26 | 0.70 | 1.95 | 0.70 | 0.33 | 1.56 | 12.39 | 52.30 | 38.20 | 9.80 | 4.20 |
| LOCKHEED L188 ELECTRA | 0.28 | 0.89 | 1.96 | 17.61 | 2.00 | 2.10 | 5.10 | 43.60 | 8.88 | 9.22 | 7.49 | 3.52 |
| MBB BK117 | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| MBB BO105 | 0.26 | 0.37 | 5.18 | 20.16 | 7.80 | 9.00 | 48.60 | 97.30 | 6.60 | 5.96 | 2.24 | 1.43 |
| Mc DONNELL DOUGLAS MD90 | 0.04 | 0.04 | 0.06 | 0.10 | 0.53 | 0.62 | 2.44 | 12.43 | 26.50 | 22.30 | 8.90 | 4.70 |
| MCDONNELL-DOUGLAS DC10-10 | 0.30 | 0.30 | 0.60 | 19.90 | 0.50 | 0.50 | 5.50 | 52.00 | 41.60 | 33.90 | 11.80 | 4.60 |
| MCDONNELL-DOUGLAS DC10-30 | 0.14 | 0.15 | 0.28 | 2.72 | 0.45 | 0.45 | 3.71 | 24.04 | 28.97 | 25.50 | 10.16 | 3.40 |
| MCDONNELL-DOUGLAS DC10-40 | 0.14 | 0.15 | 0.28 | 2.72 | 0.45 | 0.45 | 3.71 | 24.04 | 28.97 | 25.50 | 10.16 | 3.40 |
| MCDONNELL-DOUGLAS DC8-10/50 | 0.04 | 0.04 | 0.08 | 1.13 | 0.90 | 0.90 | 3.40 | 23.50 | 20.40 | 17.30 | 8.70 | 4.30 |
| MCDONNELL-DOUGLAS DC861/3 71/3 | 0.04 | 0.04 | 0.08 | 1.13 | 0.90 | 0.90 | 3.40 | 23.50 | 20.40 | 17.30 | 8.70 | 4.30 |
| MCDONNELL-DOUGLAS DC8-62/72 | 0.04 | 0.04 | 0.08 | 1.13 | 0.90 | 0.90 | 3.40 | 23.50 | 20.40 | 17.30 | 8.70 | 4.30 |
| MCDONNELL-DOUGLAS DC8-71/73 | 0.04 | 0.04 | 0.08 | 1.13 | 0.90 | 0.90 | 3.40 | 23.50 | 20.40 | 17.30 | 8.70 | 4.30 |
| MCDONNELL-DOUGLAS DC8F 54/55 | 0.04 | 0.04 | 0.08 | 1.13 | 0.90 | 0.90 | 3.40 | 23.50 | 20.40 | 17.30 | 8.70 | 4.30 |
| MCDONNELL-DOUGLAS DC9-10/15 | 0.12 | 0.14 | 1.50 | 43.60 | 2.70 | 3.20 | 14.50 | 60.30 | 19.10 | 16.30 | 7.00 | 3.60 |
| McDONNELL-DOUGLAS DC9-20 | 0.12 | 0.14 | 1.50 | 43.60 | 2.70 | 3.20 | 14.50 | 60.30 | 19.10 | 16.30 | 7.00 | 3.60 |
| MCDONNELL-DOUGLAS DC9-30 | 0.12 | 0.14 | 1.50 | 43.60 | 2.70 | 3.20 | 14.50 | 60.30 | 19.10 | 16.30 | 7.00 | 3.60 |
| MCDONNELL-DOUGLAS DC9-40 | 0.12 | 0.14 | 1.50 | 43.60 | 2.70 | 3.20 | 14.50 | 60.30 | 19.10 | 16.30 | 7.00 | 3.60 |
| MCDONNELL-DOUGLAS DC9-50 | 0.12 | 0.14 | 1.50 | 43.60 | 2.70 | 3.20 | 14.50 | 60.30 | 19.10 | 16.30 | 7.00 | 3.60 |
| MCDONNELL-DOUGLAS MD11 | 0.06 | 0.06 | 0.13 | 2.37 | 0.20 | 0.24 | 1.83 | 21.30 | 30.96 | 23.42 | 12.61 | 4.52 |
| MCDONNELL-DOUGLAS MD80-MD83 | 0.08 | 0.12 | 0.45 | 0.99 | 0.51 | 0.69 | 3.84 | 16.12 | 19.59 | 15.29 | 8.07 | 3.98 |
| MCDONNELL-DOUGLAS MD87 | 0.01 | 0.02 | 0.08 | 0.17 | 0.44 | 0.53 | 3.80 | 17.63 | 17.02 | 13.41 | 7.72 | 4.03 |
| MCDONNELL-DOUGLAS MD88 | 0.00 | 0.00 | 0.00 | 0.00 | 0.42 | 0.49 | 3.79 | 17.89 | 16.49 | 13.02 | 7.65 | 4.05 |
| MITSUBISHI MU2 | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| MOONEY M20J | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| NAMC YS11 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| NORD 2501 NORTALAS | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| PARTENAVIA P68B/C | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| PIAGGIO P.180 AVANTI | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| PIAGGIO P166-DL3 | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| PILATUS BN-2A ISLANDER | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| PILATUS BN-2A TRISLANDER MK3 | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| PIPER PA18 SUPER CUB | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| PIPER PA23 AZTEC/APACHE | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| PIPER PA24 COMMANCHE | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| PIPER PA28 CHEROKEE SRS/PA32 | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| PIPER PA30/PA39 TWIN COMANCHE | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| PIPER PA31/P NAVAJO CHIEFTAIN | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| PIPER PA31T CHEYENNE I/II | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| PIPER PA34 SENECA II | 8.95 | 9.32 | 11.05 | 136.82 | 1055.63 | 937.39 | 971.00 | 579.26 | 2.95 | 4.50 | 3.92 | 1.93 |
| PIPER PA42 CHEYENNE III/IV | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| PIPER PA44 SEMINOLE | 5.21 | 5.44 | 6.64 | 112.38 | 608.98 | 540.89 | 562.81 | 360.02 | 6.93 | 7.62 | 6.46 | 2.33 |
| REIMS-CESSNA F406/CARAVAN II | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| RILEY TURBO-EXEC 400 | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| RO TURBO COMMANDER 680T-690 | 0.11 | 0.15 | 0.64 | 79.11 | 0.80 | 1.00 | 7.00 | 61.50 | 12.36 | 11.86 | 9.92 | 2.86 |
| ROBINSON R22 | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| ROCKWELL SABRELINER SERIES | 0.06 | 0.07 | 1.41 | 9.04 | 1.13 | 1.62 | 15.56 | 47.70 | 19.15 | 16.02 | 6.92 | 3.72 |
| ROCKWELL SHRIKE/COURSER/COMMAN | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| ROCKWELL TURBO COMMANDER | 0.11 | 0.15 | 0.64 | 79.11 | 0.80 | 1.00 | 7.00 | 61.50 | 12.36 | 11.86 | 9.92 | 2.86 |
| SAAB 2000 | 0.25 | 0.29 | 0.64 | 2.51 | 0.75 | 0.92 | 3.28 | 17.35 | 20.54 | 17.47 | 7.79 | 3.83 |
| SAAB FAIRCHILD 340 | 0.63 | 0.70 | 1.86 | 13.50 | 2.00 | 2.24 | 18.55 | 56.92 | 12.69 | 10.67 | 4.06 | 2.11 |
| SHORTS 330 | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| SHORTS 360 | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| SHORTS BELFAST | 0.28 | 0.89 | 1.96 | 17.61 | 2.00 | 2.10 | 5.10 | 43.60 | 8.88 | 9.22 | 7.49 | 3.52 |
| SHORTS SC7 SKYLINER | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| SHORTS SC7 SKYVAN | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| SIKORSKY S61N | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| SIKORSKY S76 SPIRIT | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| SOCATA TB10 TOBAGO | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| SWEARINGEN MERLIN IIA/IIB/IIIB | 0.11 | 0.15 | 0.64 | 79.11 | 0.80 | 1.00 | 7.00 | 61.50 | 12.36 | 11.86 | 9.92 | 2.86 |
| SWEARINGEN MERLIN IVA | 0.11 | 0.15 | 0.64 | 79.11 | 0.80 | 1.00 | 7.00 | 61.50 | 12.36 | 11.86 | 9.92 | 2.86 |
| SWEARINGEN METRO II | 0.11 | 0.15 | 0.64 | 79.11 | 0.80 | 1.00 | 7.00 | 61.50 | 12.36 | 11.86 | 9.92 | 2.86 |
| TUPOLEV TU104 | 0.00 | 0.00 | 0.00 | 0.00 | 0.42 | 0.48 | 3.66 | 17.14 | 17.47 | 13.37 | 7.65 | 4.19 |
| TUPOLEV TU124 | 0.09 | 0.12 | 0.18 | 3.69 | 0.12 | 0.63 | 2.65 | 31.77 | 22.70 | 17.30 | 7.20 | 3.60 |
| TUPOLEV TU134 | 0.12 | 0.14 | 1.50 | 43.60 | 2.70 | 3.20 | 14.50 | 60.30 | 19.10 | 16.30 | 7.00 | 3.60 |
| TUPOLEV TU154A/B | 2.00 | 2.00 | 2.60 | 32.00 | 5.00 | 6.00 | 13.00 | 64.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| TUPOLEV TU154M | 0.40 | 0.50 | 1.90 | 12.70 | 3.00 | 3.60 | 18.20 | 77.70 | 14.50 | 11.60 | 5.10 | 2.90 |
| TUPOLEV TU204 | 0.03 | 0.00 | 0.04 | 0.27 | 0.26 | 0.29 | 2.71 | 20.26 | 22.45 | 17.63 | 8.39 | 4.40 |
| V953C MERCHANTMAN | 0.28 | 0.89 | 1.96 | 17.61 | 2.00 | 2.10 | 5.10 | 43.60 | 8.88 | 9.22 | 7.49 | 3.52 |
| VICKERS VISCOUNT 700 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| VICKERS VISCOUNT 800 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| WESTLAND 30 SRS 100 | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| YAKOVLEV YAK-40 | 0.06 | 0.07 | 1.41 | 9.04 | 1.13 | 1.62 | 15.56 | 47.70 | 19.15 | 16.02 | 6.92 | 3.72 |
| YAKOVLEV YAK-42 | 2.00 | 2.00 | 2.60 | 32.00 | 5.00 | 6.00 | 13.00 | 64.00 | 0.00 | 0.00 | 0.00 | 0.00 |

Table A3.1b	Emission factors and fuel flow rates – Heathrow

| CAA Aircraft Type | SN_100 | SN_85 | SN_30 | SN_7 | Fuel_100 | Fuel_85 | Fuel_30 | Fuel_7 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AEROSPACELINES B377SUPER GUPPY | 0.00 | 0.00 | 0.00 | 0.00 | 0.2994 | 0.2769 | 0.1436 | 0.0769 | Surrogate LOCKHEED L100 HERCULES (Gatwick) |
| AEROSPATIALE (NORD)262 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate SHORTS 360 (Gatwick) |
| AEROSPATIALE AS332 SUPER PUMA (L1/L2) | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate AEROSPATIALE SA365 DAUPHIN (Gatwick) |
| AEROSPATIALE AS355 ECUREUIL 2 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate AEROSPATIALE SA365 DAUPHIN (Gatwick) |
| AEROSPATIALE CARAVELLE 10B/10R | 21.90 | 21.90 | 21.90 | 21.90 | 0.8910 | 0.7260 | 0.2780 | 0.1270 | Surrogate GA GULFSTREAM 3 (Gatwick) |
| AEROSPATIALE CARAVELLE 12 | 21.90 | 21.90 | 21.90 | 21.90 | 0.8910 | 0.7260 | 0.2780 | 0.1270 | Surrogate GA GULFSTREAM 3 (Gatwick) |
| AEROSPATIALE CARAVELLE 6/6R | 21.90 | 21.90 | 21.90 | 21.90 | 0.8910 | 0.7260 | 0.2780 | 0.1270 | Surrogate GA GULFSTREAM 3 (Gatwick) |
| AEROSPATIALE CORVETTE | 14.10 | 14.10 | 14.10 | 14.10 | 0.1697 | 0.1430 | 0.0590 | 0.0261 |  |
| AEROSPATIALE SA315 LAMA | 0.00 | 0.00 | 0.00 | 0.00 | 0.0334 | 0.0309 | 0.0107 | 0.0079 | Surrogate AEROSPATIALE SA350 ECUREUIL (Gatwick) |
| AEROSPATIALE SA330 PUMA | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate AEROSPATIALE SA365 DAUPHIN (Gatwick) |
| AEROSPATIALE SA341 GAZELLE | 0.00 | 0.00 | 0.00 | 0.00 | 0.0334 | 0.0309 | 0.0107 | 0.0079 | Surrogate AEROSPATIALE SA350 ECUREUIL (Gatwick) |
| AEROSPATIALE SA350 ECUREUIL | 0.00 | 0.00 | 0.00 | 0.00 | 0.0334 | 0.0309 | 0.0107 | 0.0079 |  |
| AEROSPATIALE SA365 DAUPHIN | 0.00 | 0.00 | 0.00 | 0.00 | 0.0334 | 0.0309 | 0.0107 | 0.0079 |  |
| AEROSPATIALE SA365 DAUPHIN/AMB | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate AEROSPATIALE SA365 DAUPHIN (Gatwick) |
| AGUSTA A109A | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate AEROSPATIALE SA365 DAUPHIN (Gatwick) |
| AIRBUS A300-600 | 7.24 | 5.76 | 1.38 | 1.38 | 2.5362 | 2.0554 | 0.6667 | 0.2055 |  |
| AIRBUS A300B1/B2 | 10.86 | 11.61 | 1.80 | 1.59 | 2.3633 | 1.9417 | 0.6635 | 0.1651 |  |
| AIRBUS A300B4-100/200 | 10.95 | 11.72 | 1.62 | 1.40 | 2.3610 | 1.9400 | 0.6630 | 0.1630 | Surrogate AIRBUS A300B1/B2 (Gatwick) |
| AIRBUS A300F4 | 10.95 | 11.72 | 1.62 | 1.40 | 2.3610 | 1.9400 | 0.6630 | 0.1630 |  |
| AIRBUS A310-202 | 10.52 | 8.51 | 1.78 | 1.78 | 2.2287 | 1.8562 | 0.6323 | 0.1612 |  |
| AIRBUS A310-300 | 6.41 | 4.29 | 1.37 | 1.37 | 2.2281 | 1.8262 | 0.6077 | 0.1959 |  |
| AIRBUS A319 | 8.86 | 8.86 | 8.87 | 8.86 | 0.9678 | 0.8136 | 0.3079 | 0.1164 |  |
| AIRBUS A320-100/200 | 9.93 | 9.41 | 6.62 | 6.04 | 1.0758 | 0.8920 | 0.3126 | 0.1170 |  |
| AIRBUS A321 | 7.22 | 7.41 | 6.32 | 6.39 | 1.4145 | 1.1362 | 0.3788 | 0.1272 |  |
| AIRBUS A330-200 | 2.23 | 2.23 | 2.19 | 2.19 | 3.1606 | 2.5524 | 0.8440 | 0.2736 |  |
| AIRBUS A330-300 | 2.03 | 2.03 | 1.99 | 1.99 | 3.1971 | 2.5778 | 0.8492 | 0.2797 |  |
| AIRBUS A340-200 | 12.58 | 9.80 | 1.03 | 1.10 | 1.4560 | 1.1950 | 0.3860 | 0.1240 |  |
| AIRBUS A340-300 | 11.76 | 9.08 | 1.01 | 1.10 | 1.3732 | 1.1287 | 0.3692 | 0.1204 |  |
| AIRBUS A340-600 | 2.24 | 2.09 | 1.02 | 0.76 | 2.2400 | 1.8300 | 0.6200 | 0.2300 |  |
| AIRSHIP INDUSTRIES SKYSHIP 500 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 310 (Gatwick) |
| AIRSHIP INDUSTRIES SKYSHIP 600 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 310 (Gatwick) |
| ANTONOV AN-12 | 0.00 | 0.00 | 0.00 | 0.00 | 0.2994 | 0.2769 | 0.1436 | 0.0769 | Surrogate LOCKHEED L100 HERCULES (Gatwick) |
| ANTONOV AN-124 | 13.00 | 13.00 | 13.00 | 13.00 | 1.7390 | 1.4310 | 0.4890 | 0.1780 | Surrogate ILYUSHIN IL96-300 (Gatwick) |
| ANTONOV AN-24 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate FOKKER 50 (Gatwick) |
| ANTONOV AN26B/32 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate FOKKER 50 (Gatwick) |
| ANTONOV AN72 | 33.00 | 35.00 | 27.00 | 8.00 | 1.1500 | 0.9750 | 0.3500 | 0.1300 | Surrogate TUPOLEV TU134 (Gatwick) |
| ARMSTRONG WHITWORTH ARGOSY | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate ATR72 (Gatwick) |
| ATR42-300 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 |  |
| ATR42-500 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate ATR42-300 (Gatwick) |
| ATR72 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 |  |
| AVROLINER RJ100/115 | 10.30 | 10.20 | 6.90 | 6.80 | 0.3578 | 0.2961 | 0.1083 | 0.0453 |  |
| AVROLINER RJ70 | 10.30 | 10.20 | 6.90 | 6.80 | 0.3578 | 0.2961 | 0.1083 | 0.0453 |  |
| AVROLINER RJ85/QT | 10.30 | 10.20 | 6.90 | 6.80 | 0.3578 | 0.2961 | 0.1083 | 0.0453 |  |
| BAC/AEROSPATIALE CONCORDE | 19.50 | 26.10 | 26.40 | 26.40 | 6.3650 | 2.3290 | 1.1710 | 0.4210 |  |
| BAE (HS) 748 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate FOKKER 50 (Gatwick) |
| BAE 146-100 | 13.50 | 12.70 | 5.70 | 2.30 | 0.3581 | 0.2955 | 0.1034 | 0.0408 |  |
| BAE 146-200/QT | 13.50 | 12.70 | 5.70 | 2.30 | 0.3581 | 0.2955 | 0.1034 | 0.0408 |  |
| BAE 146-300 | 13.50 | 12.70 | 5.70 | 2.30 | 0.3581 | 0.2955 | 0.1034 | 0.0408 |  |
| BAE ATP | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 |  |
| BAE JETSTREAM 31/32 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0577 | 0.0515 | 0.0315 | 0.0141 |  |
| BAE JETSTREAM 41 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0577 | 0.0515 | 0.0315 | 0.0141 | Gatwick data |
| BAE(BAC)111-200 | 21.90 | 21.90 | 21.90 | 21.90 | 0.8910 | 0.7260 | 0.2780 | 0.1270 | Surrogate GA GULFSTREAM 3 (Gatwick) |
| BAE(BAC)111-300/400/475 | 21.90 | 21.90 | 21.90 | 21.90 | 0.8910 | 0.7260 | 0.2780 | 0.1270 | Surrogate GA GULFSTREAM 3 (Gatwick) |
| BAE(BAC)111-500 | 21.90 | 21.90 | 21.90 | 21.90 | 0.8910 | 0.7260 | 0.2780 | 0.1270 | Surrogate GA GULFSTREAM 3 (Gatwick) |
| BAE(HS)125 | 0.00 | 0.00 | 0.00 | 0.00 | 0.2250 | 0.1860 | 0.0720 | 0.0260 |  |
| BAE125-1000 | 0.00 | 0.00 | 0.00 | 0.00 | 0.2250 | 0.1860 | 0.0720 | 0.0260 | Surrogate BAE(HS)125 (Gatwick) |
| BEAGLE B206 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR (Gatwick) |
| BEECH KINGAIR 100 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate EMBRAER EMB110 BANDEIRANTE (Gatwick) |
| BEECH KINGAIR 90 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Gatwick data |
| BEECH KINGAIR C90A | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate BEECH KINGAIR 90 (Gatwick) |
| BEECHCRAFT 18/SUPER H18 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR (Gatwick) |
| BEECHCRAFT 1900C/D AIRLINER | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 |  |
| BEECHCRAFT 200 SUPERKING AIR | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 |  |
| BEECHCRAFT 300 SUPER KING AIR | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Gatwick data |
| BEECHCRAFT 400 BEECHJET | 18.50 | 18.50 | 18.50 | 18.50 | 0.2054 | 0.1727 | 0.0660 | 0.0296 |  |
| BEECHCRAFT 50 TWIN BONANZA | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR (Gatwick) |
| BEECHCRAFT 76 DUCHESS | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR (Gatwick) |
| BEECHCRAFT 95 TRAVEL AIR | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR (Gatwick) |
| BEECHCRAFT 99/99A | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate EMBRAER EMB110 BANDEIRANTE (Gatwick) |
| BEECHCRAFT BARON MOD 55/58/58P | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR (Gatwick) |
| BEECHCRAFT DUKE | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR (Gatwick) |
| BEECHCRAFT QUEEN AIR 65/80 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR (Gatwick) |
| BEECHCRAFT STARSHIP MODEL 2000 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate SHORTS 360 (Gatwick) |
| BELL 206B JET RANGER | 0.00 | 0.00 | 0.00 | 0.00 | 0.0334 | 0.0309 | 0.0107 | 0.0079 |  |
| BELL 206L LONG RANGER | 0.00 | 0.00 | 0.00 | 0.00 | 0.0334 | 0.0309 | 0.0107 | 0.0079 |  |
| BELL 212/412 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0334 | 0.0309 | 0.0107 | 0.0079 | Surrogate BELL 222 (Gatwick) |
| BELL 222 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0334 | 0.0309 | 0.0107 | 0.0079 |  |
| BELL 47G | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 210 CENTURION (Gatwick) |
| BELL MODEL 214ST | 0.00 | 0.00 | 0.00 | 0.00 | 0.0334 | 0.0309 | 0.0107 | 0.0079 |  |
| BOEING 707 ALL SERIES | 52.00 | 52.00 | 52.00 | 52.00 | 1.2540 | 1.0320 | 0.3890 | 0.1280 |  |
| BOEING 707-120/121B | 18.00 | 18.00 | 18.00 | 18.00 | 1.2540 | 1.0320 | 0.3890 | 0.1280 | Surrogate BOEING 707 ALL SERIES (Gatwick) |
| BOEING 717-200 | 3.24 | 2.73 | 0.97 | 0.91 | 0.8360 | 0.6900 | 0.2450 | 0.0960 | Gatwick data |
| BOEING 720B | 18.00 | 18.00 | 18.00 | 18.00 | 1.2540 | 1.0320 | 0.3890 | 0.1280 | Surrogate BOEING 707 ALL SERIES (Gatwick) |
| BOEING 727-100/100C | 22.20 | 22.20 | 22.20 | 22.20 | 0.9892 | 0.8113 | 0.2861 | 0.1291 |  |
| BOEING 727-200/200 ADVANCED | 22.80 | 22.80 | 22.80 | 22.80 | 1.0591 | 0.8592 | 0.3022 | 0.1334 |  |
| BOEING 737-100 | 21.25 | 21.25 | 21.25 | 21.25 | 1.1619 | 0.9318 | 0.3325 | 0.1445 | Surrogate BOEING 737-200 (Gatwick) |
| BOEING 737-200 | 16.38 | 16.38 | 16.38 | 16.38 | 1.1309 | 0.9113 | 0.3212 | 0.1387 |  |
| BOEING 737-300 | 5.70 | 3.20 | 2.63 | 2.37 | 1.0361 | 0.8621 | 0.3098 | 0.1181 |  |
| BOEING 737-400 | 7.65 | 3.87 | 2.50 | 2.30 | 1.1509 | 0.9516 | 0.3353 | 0.1238 |  |
| BOEING 737-500 | 4.30 | 2.61 | 2.50 | 2.21 | 0.9628 | 0.8051 | 0.2937 | 0.1148 |  |
| BOEING 737-600 | 11.54 | 11.54 | 11.54 | 11.54 | 0.9261 | 0.7711 | 0.2767 | 0.1006 |  |
| BOEING 737-700 | 13.21 | 13.21 | 13.21 | 13.21 | 1.0459 | 0.8639 | 0.3044 | 0.1065 |  |
| BOEING 737-800 | 13.44 | 13.44 | 13.44 | 13.44 | 1.1500 | 0.9455 | 0.3248 | 0.1106 |  |
| BOEING 737-900 | 14.70 | 14.70 | 14.70 | 14.70 | 1.2210 | 0.9990 | 0.3380 | 0.1130 |  |
| BOEING 747-100/100F | 16.00 | 16.00 | 16.00 | 16.00 | 2.1610 | 1.7790 | 0.6240 | 0.2320 |  |
| BOEING 747-200B | 8.57 | 8.57 | 8.57 | 8.56 | 2.3957 | 1.8808 | 0.6566 | 0.2309 |  |
| BOEING 747-200B (COMBI) | 13.19 | 13.62 | 8.01 | 7.88 | 2.3406 | 1.9231 | 0.6701 | 0.1963 |  |
| BOEING 747-200C/200F | 8.07 | 8.07 | 8.07 | 8.07 | 2.4403 | 1.9982 | 0.6799 | 0.2374 |  |
| BOEING 747-300(STRETCH UP DK) | 11.30 | 11.30 | 11.30 | 11.30 | 2.4588 | 1.9618 | 0.7063 | 0.2684 |  |
| BOEING 747-300M (COMBI) | 10.95 | 11.72 | 1.62 | 1.40 | 2.3610 | 1.9400 | 0.6630 | 0.1630 |  |
| BOEING 747-400 | 5.62 | 5.18 | 4.34 | 4.34 | 2.6301 | 2.1025 | 0.7079 | 0.2410 |  |
| BOEING 747-400F | 8.50 | 8.50 | 8.50 | 8.50 | 2.3510 | 1.9120 | 0.6430 | 0.2080 |  |
| BOEING 747-400M (COMBI) | 7.40 | 5.75 | 2.67 | 2.67 | 2.3997 | 1.9607 | 0.6478 | 0.2018 |  |
| BOEING 747SP | 12.93 | 10.61 | 4.81 | 3.32 | 2.1096 | 1.7873 | 0.6199 | 0.2146 |  |
| BOEING 757-200 | 7.52 | 7.52 | 7.52 | 7.52 | 1.8340 | 1.4902 | 0.5248 | 0.1851 |  |
| BOEING 757-300 | 7.30 | 7.30 | 7.30 | 7.30 | 2.0700 | 1.6400 | 0.5500 | 0.1900 |  |
| BOEING 767-200 | 4.63 | 4.63 | 4.63 | 4.63 | 2.0584 | 1.6804 | 0.7589 | 0.2055 |  |
| BOEING 767-200ER | 5.03 | 4.93 | 4.55 | 4.55 | 2.0777 | 1.6972 | 0.7461 | 0.2041 |  |
| BOEING 767-300 | 4.24 | 4.10 | 3.61 | 3.61 | 2.7140 | 2.1614 | 0.7068 | 0.2542 |  |
| BOEING 767-300ER/F | 7.99 | 7.31 | 5.00 | 5.00 | 2.5960 | 2.0807 | 0.6805 | 0.2141 |  |
| BOEING 767-400ER | 7.15 | 5.69 | 0.87 | 0.87 | 2.5949 | 2.1002 | 0.6817 | 0.2044 | Surrogate BOEING 767-300ER/F (Gatwick) |
| BOEING 777-200 | 5.68 | 5.40 | 5.67 | 5.54 | 3.3871 | 2.7108 | 0.8852 | 0.2750 |  |
| BOEING 777-200ER | 4.73 | 4.46 | 4.63 | 4.45 | 3.6969 | 2.9400 | 0.9523 | 0.2961 |  |
| BOEING 777-300 | 5.34 | 5.34 | 5.34 | 5.34 | 3.9100 | 3.1000 | 1.0000 | 0.3000 |  |
| BOEING-VERTOL MOD-234 CHINOOK | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate AEROSPATIALE SA365 DAUPHIN (Gatwick) |
| BOMBARDIER DASH 8 Q100/200 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate BOMBARDIER DASH 8 Q400 (Gatwick) |
| BOMBARDIER DASH 8 Q400 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 |  |
| BOMBARDIER REGIONAL JET 100/200 | 18.30 | 9.20 | 3.10 | 3.10 | 0.3991 | 0.3288 | 0.1160 | 0.0489 |  |
| BOMBARDIER REGIONAL JET RJ700 | 18.30 | 9.20 | 3.10 | 3.10 | 0.3991 | 0.3288 | 0.1160 | 0.0489 |  |
| BOMBARDIER REGIONAL JET RJ700ER | 5.33 | 1.48 | 0.00 | 0.00 | 0.6042 | 0.4941 | 0.1681 | 0.0690 | Surrogate BOMBARDIER REGIONAL JET RJ700 (Gatwick) |
| BRISTOL 170 FREIGHTER | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate ATR42-300 (Gatwick) |
| CANADAIR CL-44 | 0.00 | 0.00 | 0.00 | 0.00 | 0.2994 | 0.2769 | 0.1436 | 0.0769 | Surrogate LOCKHEED L100 HERCULES (Gatwick) |
| CANADAIR CL-600-604 CHALLENGER | 21.90 | 9.20 | 3.10 | 3.10 | 0.4070 | 0.3343 | 0.1190 | 0.0496 | Surrogate BOMBARDIER REGIONAL JET 100/200 (Gatwick) |
| CANADAIR GLOBAL EXPRESS | 33.00 | 35.00 | 27.00 | 8.00 | 1.1500 | 0.9750 | 0.3500 | 0.1300 | Surrogate TUPOLEV TU134 (Gatwick) |
| CESSNA 150 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 210 CENTURION (Gatwick) |
| CESSNA 172 SKYHAWK | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 210 CENTURION (Gatwick) |
| CESSNA 180 SKYWAGON | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 210 CENTURION (Gatwick) |
| CESSNA 206 STATIONAIR | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 210 CENTURION (Gatwick) |
| CESSNA 208 CARAVAN I | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Gatwick data |
| CESSNA 210 CENTURION | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Gatwick data |
| CESSNA 310 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 |  |
| CESSNA 336/337 SKYMASTER | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 310 (Gatwick) |
| CESSNA 337 SUPER SKYMASTER | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 310 (Gatwick) |
| CESSNA 340 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 310 (Gatwick) |
| CESSNA 401/402/411/421 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR (Gatwick) |
| CESSNA 404 TITAN | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR (Gatwick) |
| CESSNA 414A CHANCELLOR | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 |  |
| CESSNA 425 CONQUEST I | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR (Gatwick) |
| CESSNA 441 CONQUEST II | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR (Gatwick) |
| CESSNA 500 CITATION I | 15.00 | 15.00 | 15.00 | 15.00 | 0.1480 | 0.1240 | 0.0510 | 0.0230 |  |
| CESSNA 525 CITATIONJET | 15.00 | 15.00 | 15.00 | 15.00 | 0.1480 | 0.1240 | 0.0510 | 0.0230 | Surrogate CESSNA 500 CITATION I (Gatwick) |
| CESSNA 550 CITATION II | 14.32 | 14.17 | 14.32 | 14.32 | 0.1730 | 0.1457 | 0.0596 | 0.0262 |  |
| CESSNA 560 CITATION V | 17.11 | 15.13 | 17.11 | 17.11 | 0.2150 | 0.1801 | 0.0678 | 0.0276 |  |
| CESSNA 650 CITATION III/VI/VII | 0.39 | 0.34 | 0.39 | 0.39 | 0.2248 | 0.1859 | 0.0719 | 0.0260 |  |
| CESSNA 750 CITATION X | 1.00 | 0.00 | 0.00 | 0.00 | 0.2800 | 0.2340 | 0.0910 | 0.0390 |  |
| CESSNA T303 CRUSADER | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 310 (Gatwick) |
| CONVAIR 240/340/440 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate ATR42-300 (Gatwick) |
| CONVAIR 580/600/640 | 1.00 | 0.00 | 0.00 | 0.00 | 0.3770 | 0.3150 | 0.1170 | 0.0490 | Surrogate SAAB 2000 (Gatwick) |
| CONVAIR 880 | 13.50 | 12.70 | 5.70 | 2.30 | 0.3581 | 0.2955 | 0.1034 | 0.0408 | Surrogate BAE 146-100 (Gatwick) |
| DASSAULT BREGUET FALCON 50 | 0.00 | 0.00 | 0.00 | 0.00 | 0.2250 | 0.1860 | 0.0720 | 0.0260 |  |
| DASSAULT MYSTERE-FALCON 10 | 0.00 | 0.00 | 0.00 | 0.00 | 0.2050 | 0.1730 | 0.0670 | 0.0240 |  |
| DASSAULT MYSTERE-FALCON 20 | 31.60 | 26.60 | 14.00 | 7.80 | 0.3280 | 0.2760 | 0.1160 | 0.0580 |  |
| DASSAULT MYSTERE-FALCON 2000 | 31.60 | 26.60 | 14.00 | 7.80 | 0.3280 | 0.2760 | 0.1160 | 0.0580 |  |
| DASSAULT MYSTERE-FALCON 900 | 0.00 | 0.00 | 0.00 | 0.00 | 0.2250 | 0.1860 | 0.0720 | 0.0260 |  |
| DASSAULT MYSTERE-FALCON 900EX | 0.00 | 0.00 | 0.00 | 0.00 | 0.2250 | 0.1860 | 0.0720 | 0.0260 | Surrogate DASSAULT MYSTERE-FALCON 900 (Gatwick) |
| DASSAULT-BREGUET MERCURE | 33.00 | 35.00 | 27.00 | 8.00 | 1.1500 | 0.9750 | 0.3500 | 0.1300 | Surrogate TUPOLEV TU134 (Gatwick) |
| DE HAVILLAND DASH 8-300/Q300 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 |  |
| DE HAVILLAND DASH 8-100 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Gatwick data |
| DE HAVILLAND DH6 TWIN OTTER | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate BEECH KINGAIR 90 (Gatwick) |
| DE HAVILLAND DHC-7 DASH-7 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate ATR72 (Gatwick) |
| DE HAVILLAND DRAGON RAPIDE | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR (Gatwick) |
| DE HAVILLAND TIGER MOTH | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 210 CENTURION (Gatwick) |
| DHC2 BEAVER | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 210 CENTURION (Gatwick) |
| DORNIER 228-100/200 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate ATR42-300 (Gatwick) |
| DORNIER 328 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 |  |
| DORNIER 328 JET | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 |  |
| DORNIER DO28D SKYSERVANT | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR (Gatwick) |
| DOUGLAS DC3 C47 DAKOTA | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate SHORTS 360 (Gatwick) |
| DOUGLAS DC4 SKYMASTER | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate SHORTS 360 (Gatwick) |
| DOUGLAS DC6/6A/6B/6C | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate SHORTS 360 (Gatwick) |
| EMBRAER EMB110 BANDEIRANTE | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Gatwick data |
| EMBRAER EMB120 BRASILIA | 1.00 | 0.00 | 0.00 | 0.00 | 0.3800 | 0.3190 | 0.1170 | 0.0500 |  |
| EMBRAER EMB121 XINGU | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate EMBRAER EMB110 BANDEIRANTE (Gatwick) |
| EMBRAER RJ135 | 1.00 | 0.00 | 0.00 | 0.00 | 0.3783 | 0.3168 | 0.1170 | 0.0494 | Gatwick data |
| EMBRAER RJ145 | 1.09 | 0.10 | 0.10 | 0.10 | 0.3778 | 0.3172 | 0.1166 | 0.0500 |  |
| ENSTROM F28A/F280 SHARK | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 210 CENTURION (Gatwick) |
| EUROCOPTER EC135T | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate AEROSPATIALE SA365 DAUPHIN (Gatwick) |
| FAIRCHILD HILLER FH 227B | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate FOKKER 50 (Gatwick) |
| FAIRCHILD SA-227 METRO 23 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0577 | 0.0515 | 0.0315 | 0.0141 | Surrogate SWEARINGEN METRO II (Gatwick) |
| FAIRCHILD SA-227 METRO III | 0.00 | 0.00 | 0.00 | 0.00 | 0.0577 | 0.0515 | 0.0315 | 0.0141 | Surrogate SWEARINGEN METRO II (Gatwick) |
| FOKKER 100 | 9.87 | 9.87 | 9.87 | 9.87 | 0.8600 | 0.7100 | 0.2500 | 0.1200 |  |
| FOKKER 50 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 |  |
| FOKKER 70 | 21.30 | 21.30 | 21.30 | 21.30 | 0.7600 | 0.6300 | 0.2300 | 0.1100 |  |
| FOKKER F27 100-400/600 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate FOKKER 50 (Gatwick) |
| FOKKER F27-500 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate FOKKER 50 (Gatwick) |
| FOKKER F28-1000 | 21.90 | 9.20 | 3.10 | 3.10 | 0.4070 | 0.3343 | 0.1190 | 0.0496 | Surrogate BOMBARDIER REGIONAL JET 100/200 (Gatwick) |
| FOKKER F28-2000 | 21.90 | 9.20 | 3.10 | 3.10 | 0.4070 | 0.3343 | 0.1190 | 0.0496 | Surrogate BOMBARDIER REGIONAL JET 100/200 (Gatwick) |
| FOKKER F28-3000 | 21.90 | 9.20 | 3.10 | 3.10 | 0.4070 | 0.3343 | 0.1190 | 0.0496 | Surrogate BOMBARDIER REGIONAL JET 100/200 (Gatwick) |
| FOKKER F28-4000/6000 | 21.90 | 9.20 | 3.10 | 3.10 | 0.4070 | 0.3343 | 0.1190 | 0.0496 | Surrogate BOMBARDIER REGIONAL JET 100/200 (Gatwick) |
| GA GULFSTREAM 3 | 21.90 | 21.90 | 21.90 | 21.90 | 0.8910 | 0.7260 | 0.2780 | 0.1270 |  |
| GAF NOMAD N22B/N24A | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR (Gatwick) |
| GATES LEARJET 24/25D | 0.00 | 0.00 | 0.00 | 0.00 | 0.2050 | 0.1730 | 0.0670 | 0.0240 | Gatwick data |
| GATES LEARJET 25B | 0.00 | 0.00 | 0.00 | 0.00 | 0.2050 | 0.1730 | 0.0670 | 0.0240 | Surrogate GATES LEARJET 24/25D (Gatwick) |
| GATES LEARJET 31 | 0.00 | 0.00 | 0.00 | 0.00 | 0.2050 | 0.1730 | 0.0670 | 0.0240 |  |
| GATES LEARJET 35A/36A | 0.00 | 0.00 | 0.00 | 0.00 | 0.2050 | 0.1730 | 0.0670 | 0.0240 |  |
| GATES LEARJET 55 LONGHORN | 0.00 | 0.00 | 0.00 | 0.00 | 0.2250 | 0.1860 | 0.0720 | 0.0260 |  |
| GROB 6109B | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 210 CENTURION (Gatwick) |
| GRUMMAN GA-7 COUGAR | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR (Gatwick) |
| GULF AMERICAN GULFSTREAM I | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate ATR42-300 (Gatwick) |
| GULF AMERICAN GULFSTREAM II | 21.90 | 21.90 | 21.90 | 21.90 | 0.8910 | 0.7260 | 0.2780 | 0.1270 |  |
| GULF AMERICAN GULFSTREAM IV | 21.30 | 21.30 | 21.30 | 21.30 | 0.7600 | 0.6300 | 0.2300 | 0.1100 |  |
| GULF AMERICAN GULFSTREAM V | 13.96 | 11.87 | 0.27 | 0.57 | 0.7140 | 0.5950 | 0.2140 | 0.0890 |  |
| HANDLEY PAGE HERALD 200 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate ATR72 (Gatwick) |
| HANDLEY PAGE HERALD 700 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate ATR72 (Gatwick) |
| HAWKER SIDDELEY DH 104 DOVE | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR (Gatwick) |
| HAWKER SIDDELEY DH114 HERON | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR (Gatwick) |
| HILLER UH12 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 210 CENTURION (Gatwick) |
| HUGHES 269A | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 210 CENTURION (Gatwick) |
| HUGHES 500 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0334 | 0.0309 | 0.0107 | 0.0079 | Surrogate AEROSPATIALE SA350 ECUREUIL (Gatwick) |
| ILYUSHIN IL12/IL14 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate ATR42-300 (Gatwick) |
| ILYUSHIN IL18 | 0.00 | 0.00 | 0.00 | 0.00 | 0.2994 | 0.2769 | 0.1436 | 0.0769 | Surrogate LOCKHEED L100 HERCULES (Gatwick) |
| ILYUSHIN IL62 | 22.90 | 22.90 | 22.90 | 22.90 | 1.5200 | 1.3000 | 0.5000 | 0.2150 |  |
| ILYUSHIN IL76 | 22.10 | 22.10 | 22.10 | 22.10 | 2.4000 | 1.6000 | 0.5800 | 0.2100 | Surrogate ILYUSHIN IL86 (Gatwick) |
| ILYUSHIN IL86 | 22.10 | 22.10 | 22.10 | 22.10 | 2.4000 | 1.6000 | 0.5800 | 0.2100 |  |
| ILYUSHIN IL96-300 | 13.00 | 13.00 | 13.00 | 13.00 | 1.7390 | 1.4310 | 0.4890 | 0.1780 |  |
| LEARJET 45 | 0.00 | 0.00 | 0.00 | 0.00 | 0.2050 | 0.1730 | 0.0670 | 0.0240 | Surrogate GATES LEARJET 35A/36A (Gatwick) |
| LET 410 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0535 | 0.0504 | 0.0271 | 0.0145 |  |
| LOCKHEED JETSTAR II | 10.30 | 10.20 | 6.90 | 6.80 | 0.3578 | 0.2961 | 0.1083 | 0.0453 | Surrogate AVROLINER RJ100/115 (Gatwick) |
| LOCKHEED L100 HERCULES | 0.00 | 0.00 | 0.00 | 0.00 | 0.2994 | 0.2769 | 0.1436 | 0.0769 |  |
| LOCKHEED L1011-1/100 TRISTAR | 22.60 | 22.60 | 22.60 | 22.60 | 1.8770 | 1.5460 | 0.5660 | 0.2250 |  |
| LOCKHEED L1011-200 TRISTAR | 22.60 | 22.60 | 22.60 | 22.60 | 1.8770 | 1.5460 | 0.5660 | 0.2250 | Surrogate LOCKHEED L1011-1/100 TRISTAR (Gatwick) |
| LOCKHEED L1011-500 TRISTAR | 12.30 | 12.30 | 12.30 | 12.30 | 2.2100 | 1.7900 | 0.6300 | 0.2400 |  |
| LOCKHEED L188 ELECTRA | 0.00 | 0.00 | 0.00 | 0.00 | 0.2994 | 0.2769 | 0.1436 | 0.0769 | Gatwick data |
| MBB BK117 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate AEROSPATIALE SA365 DAUPHIN (Gatwick) |
| MBB BO105 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0334 | 0.0309 | 0.0107 | 0.0079 | Gatwick data |
| Mc DONNELL DOUGLAS MD90 | 9.00 | 9.00 | 9.00 | 9.00 | 1.0530 | 0.8800 | 0.3190 | 0.1280 |  |
| MCDONNELL-DOUGLAS DC10-10 | 4.00 | 3.00 | 3.00 | 4.00 | 1.8120 | 1.5020 | 0.4940 | 0.1760 | Gatwick data |
| MCDONNELL-DOUGLAS DC10-30 | 10.95 | 11.72 | 1.62 | 1.40 | 2.3610 | 1.9400 | 0.6630 | 0.1630 |  |
| MCDONNELL-DOUGLAS DC10-40 | 10.95 | 11.72 | 1.62 | 1.40 | 2.3610 | 1.9400 | 0.6630 | 0.1630 | Gatwick data |
| MCDONNELL-DOUGLAS DC8-10/50 | 7.90 | 4.20 | 2.50 | 2.00 | 1.1140 | 0.9110 | 0.3180 | 0.1300 | Surrogate MCDONNELL-DOUGLAS DC8-62/72 (Gatwick) |
| MCDONNELL-DOUGLAS DC861/3 71/3 | 7.90 | 4.20 | 2.50 | 2.00 | 1.1140 | 0.9110 | 0.3180 | 0.1300 | Surrogate MCDONNELL-DOUGLAS DC8-62/72 (Gatwick) |
| MCDONNELL-DOUGLAS DC8-62/72 | 7.90 | 4.20 | 2.50 | 2.00 | 1.1140 | 0.9110 | 0.3180 | 0.1300 |  |
| MCDONNELL-DOUGLAS DC8-71/73 | 7.90 | 4.20 | 2.50 | 2.00 | 1.1140 | 0.9110 | 0.3180 | 0.1300 | Gatwick data |
| MCDONNELL-DOUGLAS DC8F 54/55 | 7.90 | 4.20 | 2.50 | 2.00 | 1.1140 | 0.9110 | 0.3180 | 0.1300 | Surrogate MCDONNELL-DOUGLAS DC8-62/72 (Gatwick) |
| MCDONNELL-DOUGLAS DC9-10/15 | 33.00 | 35.00 | 27.00 | 8.00 | 1.1500 | 0.9750 | 0.3500 | 0.1300 | Surrogate TUPOLEV TU134 (Gatwick) |
| McDONNELL-DOUGLAS DC9-20 | 33.00 | 35.00 | 27.00 | 8.00 | 1.1500 | 0.9750 | 0.3500 | 0.1300 | Surrogate TUPOLEV TU134 (Gatwick) |
| MCDONNELL-DOUGLAS DC9-30 | 33.00 | 35.00 | 27.00 | 8.00 | 1.1500 | 0.9750 | 0.3500 | 0.1300 | Surrogate TUPOLEV TU134 (Gatwick) |
| MCDONNELL-DOUGLAS DC9-40 | 33.00 | 35.00 | 27.00 | 8.00 | 1.1500 | 0.9750 | 0.3500 | 0.1300 | Surrogate TUPOLEV TU134 (Gatwick) |
| MCDONNELL-DOUGLAS DC9-50 | 33.00 | 35.00 | 27.00 | 8.00 | 1.1500 | 0.9750 | 0.3500 | 0.1300 | Surrogate TUPOLEV TU134 (Gatwick) |
| MCDONNELL-DOUGLAS MD11 | 8.09 | 7.22 | 4.03 | 4.03 | 2.6265 | 2.1046 | 0.6872 | 0.2105 |  |
| MCDONNELL-DOUGLAS MD80-MD83 | 8.44 | 8.49 | 8.49 | 8.49 | 1.3065 | 1.0598 | 0.3704 | 0.1363 |  |
| MCDONNELL-DOUGLAS MD87 | 6.51 | 6.52 | 6.52 | 6.52 | 1.2856 | 1.0470 | 0.3639 | 0.1369 |  |
| MCDONNELL-DOUGLAS MD88 | 6.10 | 6.10 | 6.10 | 6.10 | 1.2820 | 1.0450 | 0.3630 | 0.1370 |  |
| MITSUBISHI MU2 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate BEECH KINGAIR 90 (Gatwick) |
| MOONEY M20J | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 210 CENTURION (Gatwick) |
| NAMC YS11 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate FOKKER 50 (Gatwick) |
| NORD 2501 NORTALAS | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate ATR72 (Gatwick) |
| PARTENAVIA P68B/C | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR (Gatwick) |
| PIAGGIO P.180 AVANTI | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate EMBRAER EMB110 BANDEIRANTE (Gatwick) |
| PIAGGIO P166-DL3 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 310 (Gatwick) |
| PILATUS BN-2A ISLANDER | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR (Gatwick) |
| PILATUS BN-2A TRISLANDER MK3 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR (Gatwick) |
| PIPER PA18 SUPER CUB | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 210 CENTURION (Gatwick) |
| PIPER PA23 AZTEC/APACHE | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 |  |
| PIPER PA24 COMMANCHE | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 210 CENTURION (Gatwick) |
| PIPER PA28 CHEROKEE SRS/PA32 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 210 CENTURION (Gatwick) |
| PIPER PA30/PA39 TWIN COMANCHE | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate PIPER PA31/P NAVAJO CHIEFTAIN (Gatwick) |
| PIPER PA31/P NAVAJO CHIEFTAIN | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 |  |
| PIPER PA31T CHEYENNE I/II | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate PIPER PA31/P NAVAJO CHIEFTAIN (Gatwick) |
| PIPER PA34 SENECA II | 0.00 | 0.00 | 0.00 | 0.00 | 0.0178 | 0.0135 | 0.0083 | 0.0017 |  |
| PIPER PA42 CHEYENNE III/IV | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate EMBRAER EMB110 BANDEIRANTE (Gatwick) |
| PIPER PA44 SEMINOLE | 0.00 | 0.00 | 0.00 | 0.00 | 0.0347 | 0.0296 | 0.0181 | 0.0070 | Surrogate PIPER PA34 SENECA II (Gatwick) |
| REIMS-CESSNA F406/CARAVAN II | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR (Gatwick) |
| RILEY TURBO-EXEC 400 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR (Gatwick) |
| RO TURBO COMMANDER 680T-690 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0577 | 0.0515 | 0.0315 | 0.0141 | Surrogate ROCKWELL TURBO COMMANDER (Gatwick) |
| ROBINSON R22 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 210 CENTURION (Gatwick) |
| ROCKWELL SABRELINER SERIES | 0.00 | 0.00 | 0.00 | 0.00 | 0.2250 | 0.1860 | 0.0720 | 0.0260 | Surrogate BAE(HS)125 (Gatwick) |
| ROCKWELL SHRIKE/COURSER/COMMAN | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR (Gatwick) |
| ROCKWELL TURBO COMMANDER | 0.00 | 0.00 | 0.00 | 0.00 | 0.0577 | 0.0515 | 0.0315 | 0.0141 | Gatwick data |
| SAAB 2000 | 1.00 | 0.00 | 0.00 | 0.00 | 0.3770 | 0.3150 | 0.1170 | 0.0490 |  |
| SAAB FAIRCHILD 340 | 4.90 | 4.40 | 4.40 | 4.40 | 0.2790 | 0.2380 | 0.0992 | 0.0509 | Surrogate EMBRAER EMB120 BRASILIA (Gatwick) |
| SHORTS 330 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate SHORTS 360 (Gatwick) |
| SHORTS 360 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Gatwick data |
| SHORTS BELFAST | 0.00 | 0.00 | 0.00 | 0.00 | 0.2994 | 0.2769 | 0.1436 | 0.0769 | Surrogate LOCKHEED L100 HERCULES (Gatwick) |
| SHORTS SC7 SKYLINER | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate SHORTS 360 (Gatwick) |
| SHORTS SC7 SKYVAN | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate SHORTS 360 (Gatwick) |
| SIKORSKY S61N | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Gatwick data |
| SIKORSKY S76 SPIRIT | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 |  |
| SOCATA TB10 TOBAGO | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 210 CENTURION (Gatwick) |
| SWEARINGEN MERLIN IIA/IIB/IIIB | 0.00 | 0.00 | 0.00 | 0.00 | 0.0577 | 0.0515 | 0.0315 | 0.0141 | Surrogate SWEARINGEN METRO II (Gatwick) |
| SWEARINGEN MERLIN IVA | 0.00 | 0.00 | 0.00 | 0.00 | 0.0577 | 0.0515 | 0.0315 | 0.0141 | Surrogate SWEARINGEN METRO II (Gatwick) |
| SWEARINGEN METRO II | 0.00 | 0.00 | 0.00 | 0.00 | 0.0577 | 0.0515 | 0.0315 | 0.0141 |  |
| TUPOLEV TU104 | 6.38 | 6.38 | 6.38 | 6.38 | 1.3145 | 1.0654 | 0.3735 | 0.1361 | Surrogate MCDONNELL-DOUGLAS MD80-MD83 (Gatwick) |
| TUPOLEV TU124 | 21.90 | 21.90 | 21.90 | 21.90 | 0.8910 | 0.7260 | 0.2780 | 0.1270 | Surrogate GA GULFSTREAM 3 (Gatwick) |
| TUPOLEV TU134 | 33.00 | 35.00 | 27.00 | 8.00 | 1.1500 | 0.9750 | 0.3500 | 0.1300 |  |
| TUPOLEV TU154A/B | 0.00 | 0.00 | 0.00 | 0.00 | 1.7500 | 1.3300 | 0.5500 | 0.2200 |  |
| TUPOLEV TU154M | 0.00 | 0.00 | 0.00 | 0.00 | 1.4200 | 1.1000 | 0.4200 | 0.2070 |  |
| TUPOLEV TU204 | 7.31 | 7.31 | 7.31 | 7.31 | 1.8567 | 1.5043 | 0.5209 | 0.1803 | Surrogate BOEING 757-200 (Gatwick) |
| V953C MERCHANTMAN | 0.00 | 0.00 | 0.00 | 0.00 | 0.2994 | 0.2769 | 0.1436 | 0.0769 | Surrogate LOCKHEED L100 HERCULES (Gatwick) |
| VICKERS VISCOUNT 700 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate ATR72 (Gatwick) |
| VICKERS VISCOUNT 800 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate ATR72 (Gatwick) |
| WESTLAND 30 SRS 100 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate AEROSPATIALE SA365 DAUPHIN (Gatwick) |
| YAKOVLEV YAK-40 | 0.00 | 0.00 | 0.00 | 0.00 | 0.2250 | 0.1860 | 0.0720 | 0.0260 | Surrogate DASSAULT BREGUET FALCON 50 (Gatwick) |
| YAKOVLEV YAK-42 | 0.00 | 0.00 | 0.00 | 0.00 | 1.7500 | 1.3300 | 0.5500 | 0.2200 | Surrogate TUPOLEV TU154A/B (Gatwick) |

Table A3.2a	Emission factors and fuel flow rates – Gatwick

| CAA Aircraft Type | HC_100 | HC_85 | HC_30 | HC_7 | CO_100 | CO_85 | CO_30 | CO_7 | NOx_100 | NOx_85 | NOx_30 | NOx_7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AEROSPACELINES B377SUPER GUPPY | 0.28 | 0.89 | 1.96 | 17.61 | 2.00 | 2.10 | 5.10 | 43.60 | 8.88 | 9.22 | 7.49 | 3.52 |
| AEROSPATIALE (NORD)262 | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| AEROSPATIALE AS332 SUPER PUMA (L1/L2) | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| AEROSPATIALE AS355 ECUREUIL 2 | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| AEROSPATIALE CARAVELLE 10B/10R | 0.09 | 0.12 | 0.18 | 3.69 | 0.12 | 0.63 | 2.65 | 31.77 | 22.70 | 17.30 | 7.20 | 3.60 |
| AEROSPATIALE CARAVELLE 12 | 0.09 | 0.12 | 0.18 | 3.69 | 0.12 | 0.63 | 2.65 | 31.77 | 22.70 | 17.30 | 7.20 | 3.60 |
| AEROSPATIALE CARAVELLE 6/6R | 0.09 | 0.12 | 0.18 | 3.69 | 0.12 | 0.63 | 2.65 | 31.77 | 22.70 | 17.30 | 7.20 | 3.60 |
| AEROSPATIALE CORVETTE | 0.09 | 0.19 | 5.15 | 40.00 | 2.10 | 3.18 | 32.00 | 97.00 | 9.23 | 8.56 | 5.29 | 2.63 |
| AEROSPATIALE SA315 LAMA | 0.26 | 0.37 | 5.18 | 20.16 | 7.80 | 9.00 | 48.60 | 97.30 | 6.60 | 5.96 | 2.24 | 1.43 |
| AEROSPATIALE SA330 PUMA | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| AEROSPATIALE SA341 GAZELLE | 0.26 | 0.37 | 5.18 | 20.16 | 7.80 | 9.00 | 48.60 | 97.30 | 6.60 | 5.96 | 2.24 | 1.43 |
| AEROSPATIALE SA350 ECUREUIL | 0.26 | 0.37 | 5.18 | 20.16 | 7.80 | 9.00 | 48.60 | 97.30 | 6.60 | 5.96 | 2.24 | 1.43 |
| AEROSPATIALE SA365 DAUPHIN | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| AEROSPATIALE SA365 DAUPHIN/AMB | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| AGUSTA A109A | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| AIRBUS A300-600 | 0.04 | 0.05 | 0.11 | 1.48 | 0.06 | 0.04 | 1.91 | 18.89 | 28.57 | 21.69 | 12.53 | 4.76 |
| AIRBUS A300B1/B2 | 0.14 | 0.15 | 0.28 | 2.72 | 0.45 | 0.45 | 3.71 | 24.04 | 28.97 | 25.50 | 10.16 | 3.40 |
| AIRBUS A300B4-100/200 | 0.14 | 0.15 | 0.28 | 2.72 | 0.45 | 0.45 | 3.71 | 24.04 | 28.97 | 25.50 | 10.16 | 3.40 |
| AIRBUS A300F4 | 0.14 | 0.15 | 0.28 | 2.72 | 0.45 | 0.45 | 3.71 | 24.04 | 28.97 | 25.50 | 10.16 | 3.40 |
| AIRBUS A310-202 | 0.30 | 0.37 | 0.45 | 6.28 | 1.00 | 1.10 | 2.80 | 28.20 | 29.60 | 26.60 | 10.80 | 3.40 |
| AIRBUS A310-300 | 0.06 | 0.06 | 0.13 | 2.34 | 0.13 | 0.14 | 2.18 | 22.55 | 25.82 | 20.47 | 12.15 | 4.48 |
| AIRBUS A319 | 0.18 | 0.18 | 0.61 | 5.37 | 0.86 | 0.96 | 3.27 | 27.69 | 22.40 | 18.91 | 8.73 | 3.91 |
| AIRBUS A320-100/200 | 0.13 | 0.13 | 0.88 | 1.92 | 0.75 | 0.96 | 4.32 | 17.00 | 28.67 | 23.73 | 10.03 | 4.75 |
| AIRBUS A321 | 0.07 | 0.11 | 0.24 | 1.51 | 0.60 | 0.67 | 1.67 | 13.41 | 36.80 | 28.59 | 10.98 | 5.02 |
| AIRBUS A330-200 | 0.01 | 0.01 | 0.04 | 1.78 | 0.29 | 0.26 | 1.06 | 18.88 | 46.94 | 34.10 | 8.62 | 3.58 |
| AIRBUS A330-300 | 0.03 | 0.04 | 0.15 | 3.24 | 0.71 | 0.73 | 1.73 | 23.37 | 42.19 | 33.73 | 14.55 | 4.16 |
| AIRBUS A340-200 | 0.01 | 0.01 | 0.06 | 5.00 | 1.00 | 0.85 | 1.40 | 30.93 | 37.67 | 29.05 | 10.67 | 4.28 |
| AIRBUS A340-300 | 0.01 | 0.01 | 0.08 | 5.49 | 0.95 | 0.81 | 1.65 | 33.15 | 34.00 | 26.70 | 10.18 | 4.21 |
| AIRBUS A340-600 | 0.00 | 0.00 | 0.00 | 0.10 | 0.02 | 0.25 | 0.46 | 10.30 | 44.91 | 32.76 | 11.78 | 6.19 |
| AIRSHIP INDUSTRIES SKYSHIP 500 | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| AIRSHIP INDUSTRIES SKYSHIP 600 | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| ANTONOV AN-12 | 0.28 | 0.89 | 1.96 | 17.61 | 2.00 | 2.10 | 5.10 | 43.60 | 8.88 | 9.22 | 7.49 | 3.52 |
| ANTONOV AN-124 | 0.12 | 0.12 | 0.20 | 0.30 | 0.35 | 0.40 | 0.90 | 6.90 | 37.00 | 31.50 | 11.80 | 5.80 |
| ANTONOV AN-24 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| ANTONOV AN26B/32 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| ANTONOV AN72 | 0.12 | 0.14 | 1.50 | 43.60 | 2.70 | 3.20 | 14.50 | 60.30 | 19.10 | 16.30 | 7.00 | 3.60 |
| ARMSTRONG WHITWORTH ARGOSY | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| ATR42-300 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| ATR42-500 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| ATR72 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| AVROLINER RJ100/115 | 0.01 | 0.01 | 0.12 | 4.72 | 0.20 | 0.30 | 4.43 | 37.83 | 14.52 | 12.02 | 6.39 | 3.28 |
| AVROLINER RJ70 | 0.01 | 0.01 | 0.12 | 4.72 | 0.20 | 0.30 | 4.43 | 37.83 | 14.52 | 12.02 | 6.39 | 3.28 |
| AVROLINER RJ85/QT | 0.01 | 0.01 | 0.12 | 4.72 | 0.20 | 0.30 | 4.43 | 37.83 | 14.52 | 12.02 | 6.39 | 3.28 |
| BAC/AEROSPATIALE CONCORDE | 2.90 | 1.70 | 11.40 | 33.40 | 29.00 | 19.90 | 52.90 | 100.10 | 9.50 | 9.30 | 3.50 | 1.70 |
| BAE (HS) 748 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| BAE 146-100 | 0.06 | 0.05 | 0.22 | 5.39 | 0.30 | 0.25 | 7.10 | 40.93 | 13.35 | 10.56 | 6.60 | 3.78 |
| BAE 146-200/QT | 0.06 | 0.05 | 0.22 | 5.39 | 0.30 | 0.25 | 7.10 | 40.93 | 13.35 | 10.56 | 6.60 | 3.78 |
| BAE 146-300 | 0.06 | 0.05 | 0.22 | 5.39 | 0.30 | 0.25 | 7.10 | 40.93 | 13.35 | 10.56 | 6.60 | 3.78 |
| BAE ATP | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| BAE JETSTREAM 31/32 | 0.11 | 0.15 | 0.64 | 79.11 | 0.80 | 1.00 | 7.00 | 61.50 | 12.36 | 11.86 | 9.92 | 2.86 |
| BAE JETSTREAM 41 | 0.11 | 0.15 | 0.64 | 79.11 | 0.80 | 1.00 | 7.00 | 61.50 | 12.36 | 11.86 | 9.92 | 2.86 |
| BAE(BAC)111-200 | 0.09 | 0.12 | 0.18 | 3.69 | 0.12 | 0.63 | 2.65 | 31.77 | 22.70 | 17.30 | 7.20 | 3.60 |
| BAE(BAC)111-300/400/475 | 0.09 | 0.12 | 0.18 | 3.69 | 0.12 | 0.63 | 2.65 | 31.77 | 22.70 | 17.30 | 7.20 | 3.60 |
| BAE(BAC)111-500 | 0.09 | 0.12 | 0.18 | 3.69 | 0.12 | 0.63 | 2.65 | 31.77 | 22.70 | 17.30 | 7.20 | 3.60 |
| BAE(HS)125 | 0.06 | 0.07 | 1.41 | 9.04 | 1.13 | 1.62 | 15.56 | 47.70 | 19.15 | 16.02 | 6.92 | 3.72 |
| BAE125-1000 | 0.06 | 0.07 | 1.41 | 9.04 | 1.13 | 1.62 | 15.56 | 47.70 | 19.15 | 16.02 | 6.92 | 3.72 |
| BEAGLE B206 | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| BEECH KINGAIR 100 | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| BEECH KINGAIR 90 | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| BEECH KINGAIR C90A | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| BEECHCRAFT 18/SUPER H18 | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| BEECHCRAFT 1900C/D AIRLINER | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| BEECHCRAFT 200 SUPERKING AIR | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| BEECHCRAFT 300 SUPER KING AIR | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| BEECHCRAFT 400 BEECHJET | 0.00 | 1.30 | 11.70 | 119.10 | 0.00 | 1.15 | 38.60 | 119.20 | 11.13 | 10.08 | 4.93 | 1.66 |
| BEECHCRAFT 50 TWIN BONANZA | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| BEECHCRAFT 76 DUCHESS | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| BEECHCRAFT 95 TRAVEL AIR | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| BEECHCRAFT 99/99A | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| BEECHCRAFT BARON MOD 55/58/58P | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| BEECHCRAFT DUKE | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| BEECHCRAFT QUEEN AIR 65/80 | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| BEECHCRAFT STARSHIP MODEL 2000 | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| BELL 206B JET RANGER | 0.26 | 0.37 | 5.18 | 20.16 | 7.80 | 9.00 | 48.60 | 97.30 | 6.60 | 5.96 | 2.24 | 1.43 |
| BELL 206L LONG RANGER | 0.26 | 0.37 | 5.18 | 20.16 | 7.80 | 9.00 | 48.60 | 97.30 | 6.60 | 5.96 | 2.24 | 1.43 |
| BELL 212/412 | 0.26 | 0.37 | 5.18 | 20.16 | 7.80 | 9.00 | 48.60 | 97.30 | 6.60 | 5.96 | 2.24 | 1.43 |
| BELL 222 | 0.26 | 0.37 | 5.18 | 20.16 | 7.80 | 9.00 | 48.60 | 97.30 | 6.60 | 5.96 | 2.24 | 1.43 |
| BELL 47G | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| BELL MODEL 214ST | 0.26 | 0.37 | 5.18 | 20.16 | 7.80 | 9.00 | 48.60 | 97.30 | 6.60 | 5.96 | 2.24 | 1.43 |
| BOEING 707 ALL SERIES | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| BOEING 707-120/121B | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| BOEING 717-200 | 0.00 | 0.02 | 0.01 | 0.21 | 0.78 | 0.75 | 3.76 | 16.27 | 23.97 | 18.65 | 11.19 | 5.37 |
| BOEING 720B | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| BOEING 727-100/100C | 0.40 | 0.50 | 1.60 | 10.60 | 1.50 | 2.00 | 10.50 | 35.50 | 17.10 | 13.50 | 5.50 | 2.70 |
| BOEING 727-200/200 ADVANCED | 0.47 | 0.47 | 1.73 | 10.00 | 1.24 | 1.66 | 9.43 | 34.50 | 17.92 | 14.21 | 5.64 | 2.90 |
| BOEING 737-100 | 0.25 | 0.27 | 1.32 | 8.39 | 0.82 | 1.06 | 7.42 | 27.91 | 18.85 | 14.67 | 6.14 | 3.04 |
| BOEING 737-200 | 0.25 | 0.27 | 1.32 | 8.39 | 0.82 | 1.06 | 7.42 | 27.91 | 18.85 | 14.67 | 6.14 | 3.04 |
| BOEING 737-300 | 0.03 | 0.04 | 0.07 | 1.67 | 0.90 | 0.91 | 3.32 | 29.17 | 19.77 | 17.04 | 8.83 | 4.16 |
| BOEING 737-400 | 0.03 | 0.04 | 0.07 | 1.42 | 0.90 | 0.90 | 3.10 | 26.81 | 20.69 | 17.80 | 9.10 | 4.30 |
| BOEING 737-500 | 0.03 | 0.04 | 0.07 | 1.46 | 0.90 | 0.90 | 3.13 | 27.17 | 20.55 | 17.69 | 9.06 | 4.28 |
| BOEING 737-600 | 0.10 | 0.10 | 0.10 | 3.10 | 0.60 | 0.50 | 3.20 | 25.90 | 20.50 | 17.40 | 9.50 | 4.30 |
| BOEING 737-700 | 0.10 | 0.10 | 0.10 | 2.97 | 0.58 | 0.52 | 3.04 | 25.21 | 21.09 | 17.76 | 9.61 | 4.34 |
| BOEING 737-800 | 0.10 | 0.10 | 0.10 | 1.93 | 0.22 | 0.58 | 1.64 | 19.02 | 28.73 | 22.46 | 10.75 | 4.68 |
| BOEING 737-900 | 0.10 | 0.10 | 0.10 | 1.90 | 0.20 | 0.60 | 1.60 | 18.80 | 28.80 | 22.50 | 10.80 | 4.70 |
| BOEING 747-100/100F | 0.10 | 0.10 | 1.30 | 36.10 | 0.00 | 0.00 | 7.60 | 83.60 | 38.70 | 28.50 | 7.60 | 3.10 |
| BOEING 747-200B | 0.51 | 0.25 | 0.91 | 5.72 | 1.13 | 0.43 | 2.79 | 16.34 | 56.27 | 40.92 | 10.39 | 4.26 |
| BOEING 747-200B (COMBI) | 0.59 | 0.26 | 0.70 | 1.41 | 1.24 | 0.34 | 1.56 | 9.30 | 59.35 | 43.01 | 10.63 | 4.41 |
| BOEING 747-200C/200F | 0.28 | 0.21 | 0.38 | 9.88 | 0.41 | 0.23 | 1.67 | 44.26 | 37.15 | 29.08 | 8.37 | 3.28 |
| BOEING 747-300(STRETCH UP DK) | 0.09 | 0.17 | 2.00 | 24.11 | 0.71 | 1.06 | 8.90 | 41.47 | 41.56 | 30.70 | 9.49 | 3.62 |
| BOEING 747-300M (COMBI) | 0.14 | 0.15 | 0.28 | 2.72 | 0.45 | 0.45 | 3.71 | 24.04 | 28.97 | 25.50 | 10.16 | 3.40 |
| BOEING 747-400 | 0.06 | 0.05 | 0.12 | 1.52 | 0.05 | 0.05 | 2.11 | 19.09 | 25.71 | 20.20 | 12.42 | 4.73 |
| BOEING 747-400F | 0.06 | 0.07 | 0.13 | 2.69 | 0.22 | 0.25 | 1.85 | 22.45 | 29.32 | 22.75 | 12.43 | 4.52 |
| BOEING 747-400M (COMBI) | 0.06 | 0.06 | 0.13 | 2.55 | 0.18 | 0.20 | 2.05 | 22.60 | 26.67 | 21.14 | 12.31 | 4.48 |
| BOEING 747SP | 0.18 | 0.15 | 2.04 | 37.17 | 0.68 | 0.93 | 10.25 | 79.50 | 41.53 | 30.38 | 8.51 | 3.23 |
| BOEING 757-200 | 0.03 | 0.00 | 0.04 | 0.27 | 0.26 | 0.29 | 2.71 | 20.26 | 22.45 | 17.63 | 8.39 | 4.40 |
| BOEING 757-300 | 0.07 | 0.00 | 0.05 | 0.14 | 0.33 | 0.26 | 2.43 | 18.24 | 25.88 | 19.30 | 8.65 | 4.58 |
| BOEING 767-200 | 0.15 | 0.12 | 0.13 | 1.25 | 0.51 | 0.48 | 1.36 | 8.84 | 38.50 | 30.00 | 9.80 | 4.10 |
| BOEING 767-200ER | 0.19 | 0.22 | 0.29 | 4.27 | 0.61 | 0.66 | 2.43 | 24.37 | 28.91 | 24.50 | 11.21 | 3.87 |
| BOEING 767-300 | 0.33 | 0.32 | 0.35 | 0.83 | 0.86 | 0.38 | 1.01 | 12.16 | 64.94 | 45.73 | 10.33 | 4.76 |
| BOEING 767-300ER/F | 0.05 | 0.05 | 0.11 | 1.64 | 0.08 | 0.08 | 1.91 | 19.12 | 28.10 | 21.55 | 12.62 | 4.74 |
| BOEING 767-400ER | 0.05 | 0.05 | 0.11 | 1.64 | 0.08 | 0.08 | 1.91 | 19.12 | 28.10 | 21.55 | 12.62 | 4.74 |
| BOEING 777-200 | 0.00 | 0.00 | 0.00 | 1.55 | 0.20 | 0.16 | 0.80 | 18.42 | 34.76 | 27.59 | 10.59 | 4.75 |
| BOEING 777-200ER | 0.03 | 0.03 | 0.05 | 0.53 | 0.19 | 0.17 | 0.90 | 13.34 | 49.16 | 36.34 | 15.28 | 5.63 |
| BOEING 777-300 | 0.01 | 0.00 | 0.00 | 0.70 | 0.28 | 0.20 | 0.57 | 13.07 | 45.70 | 33.30 | 11.58 | 5.33 |
| BOEING-VERTOL MOD-234 CHINOOK | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| BOMBARDIER DASH 8 Q100/200 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| BOMBARDIER DASH 8 Q400 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| BOMBARDIER REGIONAL JET 100/200 | 0.06 | 0.06 | 0.13 | 3.95 | 0.00 | 0.00 | 1.90 | 42.60 | 11.61 | 10.14 | 6.86 | 3.82 |
| BOMBARDIER REGIONAL JET RJ700 | 0.02 | 0.02 | 0.06 | 0.08 | 0.41 | 0.50 | 2.91 | 24.92 | 14.67 | 12.82 | 11.10 | 4.31 |
| BOMBARDIER REGIONAL JET RJ700ER | 0.02 | 0.02 | 0.06 | 0.08 | 0.41 | 0.50 | 2.91 | 24.92 | 14.67 | 12.82 | 11.10 | 4.31 |
| BRISTOL 170 FREIGHTER | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| CANADAIR CL-44 | 0.28 | 0.89 | 1.96 | 17.61 | 2.00 | 2.10 | 5.10 | 43.60 | 8.88 | 9.22 | 7.49 | 3.52 |
| CANADAIR CL-600-604 CHALLENGER | 0.06 | 0.06 | 0.13 | 3.95 | 0.00 | 0.00 | 1.90 | 42.60 | 11.61 | 10.14 | 6.86 | 3.82 |
| CANADAIR GLOBAL EXPRESS | 0.12 | 0.14 | 1.50 | 43.60 | 2.70 | 3.20 | 14.50 | 60.30 | 19.10 | 16.30 | 7.00 | 3.60 |
| CESSNA 150 | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| CESSNA 172 SKYHAWK | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| CESSNA 180 SKYWAGON | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| CESSNA 206 STATIONAIR | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| CESSNA 208 CARAVAN I | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| CESSNA 210 CENTURION | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| CESSNA 310 | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| CESSNA 336/337 SKYMASTER | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| CESSNA 337 SUPER SKYMASTER | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| CESSNA 340 | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| CESSNA 401/402/411/421 | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| CESSNA 404 TITAN | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| CESSNA 414A CHANCELLOR | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| CESSNA 425 CONQUEST I | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| CESSNA 441 CONQUEST II | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| CESSNA 500 CITATION I | 0.01 | 0.01 | 4.43 | 50.50 | 2.65 | 3.50 | 40.50 | 132.00 | 7.60 | 6.77 | 3.44 | 1.75 |
| CESSNA 525 CITATIONJET | 0.01 | 0.01 | 4.43 | 50.50 | 2.65 | 3.50 | 40.50 | 132.00 | 7.60 | 6.77 | 3.44 | 1.75 |
| CESSNA 550 CITATION II | 0.06 | 0.39 | 9.01 | 61.57 | 2.17 | 3.45 | 38.00 | 106.98 | 9.53 | 9.02 | 5.26 | 2.08 |
| CESSNA 560 CITATION V | 0.00 | 0.68 | 15.91 | 97.14 | 2.47 | 4.12 | 49.02 | 124.49 | 9.96 | 9.80 | 5.22 | 1.09 |
| CESSNA 650 CITATION III/VI/VII | 0.06 | 0.07 | 1.41 | 9.04 | 1.13 | 1.62 | 15.56 | 47.70 | 19.15 | 16.02 | 6.92 | 3.72 |
| CESSNA 750 CITATION X | 0.29 | 0.33 | 0.83 | 5.84 | 0.97 | 1.19 | 4.50 | 29.88 | 16.92 | 14.56 | 6.63 | 3.24 |
| CESSNA T303 CRUSADER | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| CONVAIR 240/340/440 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| CONVAIR 580/600/640 | 0.25 | 0.29 | 0.64 | 2.51 | 0.75 | 0.92 | 3.28 | 17.35 | 20.54 | 17.47 | 7.79 | 3.83 |
| CONVAIR 880 | 0.06 | 0.05 | 0.22 | 5.39 | 0.30 | 0.25 | 7.10 | 40.93 | 13.35 | 10.56 | 6.60 | 3.78 |
| DASSAULT BREGUET FALCON 50 | 0.06 | 0.07 | 1.41 | 9.04 | 1.13 | 1.62 | 15.56 | 47.70 | 19.15 | 16.02 | 6.92 | 3.72 |
| DASSAULT MYSTERE-FALCON 10 | 0.11 | 0.13 | 4.26 | 20.04 | 1.39 | 2.03 | 22.38 | 58.60 | 15.25 | 13.08 | 5.90 | 2.82 |
| DASSAULT MYSTERE-FALCON 20 | 0.09 | 0.09 | 1.40 | 15.68 | 16.59 | 20.42 | 49.96 | 127.18 | 9.11 | 7.41 | 3.13 | 1.63 |
| DASSAULT MYSTERE-FALCON 2000 | 0.10 | 0.10 | 1.40 | 18.00 | 22.00 | 27.00 | 62.00 | 155.00 | 5.60 | 4.40 | 1.80 | 0.90 |
| DASSAULT MYSTERE-FALCON 900 | 0.06 | 0.07 | 1.41 | 9.04 | 1.13 | 1.62 | 15.56 | 47.70 | 19.15 | 16.02 | 6.92 | 3.72 |
| DASSAULT MYSTERE-FALCON 900EX | 0.06 | 0.07 | 1.41 | 9.04 | 1.13 | 1.62 | 15.56 | 47.70 | 19.15 | 16.02 | 6.92 | 3.72 |
| DASSAULT-BREGUET MERCURE | 0.12 | 0.14 | 1.50 | 43.60 | 2.70 | 3.20 | 14.50 | 60.30 | 19.10 | 16.30 | 7.00 | 3.60 |
| DE HAVILLAND DASH 8-300/Q300 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| DE HAVILLAND DASH 8-100 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| DE HAVILLAND DH6 TWIN OTTER | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| DE HAVILLAND DHC-7 DASH-7 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| DE HAVILLAND DRAGON RAPIDE | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| DE HAVILLAND TIGER MOTH | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| DHC2 BEAVER | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| DORNIER 228-100/200 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| DORNIER 328 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| DORNIER 328 JET | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| DORNIER DO28D SKYSERVANT | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| DOUGLAS DC3 C47 DAKOTA | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| DOUGLAS DC4 SKYMASTER | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| DOUGLAS DC6/6A/6B/6C | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| EMBRAER EMB110 BANDEIRANTE | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| EMBRAER EMB120 BRASILIA | 0.63 | 0.70 | 1.86 | 13.50 | 2.00 | 2.24 | 18.55 | 56.92 | 12.69 | 10.67 | 4.06 | 2.11 |
| EMBRAER EMB121 XINGU | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| EMBRAER RJ135 | 0.25 | 0.29 | 0.68 | 2.77 | 0.77 | 0.94 | 3.51 | 19.60 | 20.20 | 17.19 | 7.54 | 3.69 |
| EMBRAER RJ145 | 0.25 | 0.29 | 0.67 | 2.76 | 0.77 | 0.94 | 3.50 | 19.48 | 20.22 | 17.21 | 7.55 | 3.70 |
| ENSTROM F28A/F280 SHARK | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| EUROCOPTER EC135T | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| FAIRCHILD HILLER FH 227B | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| FAIRCHILD SA-227 METRO 23 | 0.11 | 0.15 | 0.64 | 79.11 | 0.80 | 1.00 | 7.00 | 61.50 | 12.36 | 11.86 | 9.92 | 2.86 |
| FAIRCHILD SA-227 METRO III | 0.11 | 0.15 | 0.64 | 79.11 | 0.80 | 1.00 | 7.00 | 61.50 | 12.36 | 11.86 | 9.92 | 2.86 |
| FOKKER 100 | 0.11 | 0.22 | 0.11 | 1.23 | 0.69 | 0.48 | 1.76 | 22.36 | 16.45 | 12.94 | 5.26 | 2.50 |
| FOKKER 50 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| FOKKER 70 | 0.80 | 0.30 | 0.90 | 3.40 | 0.70 | 0.80 | 3.90 | 24.10 | 21.10 | 16.80 | 5.70 | 2.50 |
| FOKKER F27 100-400/600 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| FOKKER F27-500 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| FOKKER F28-1000 | 0.06 | 0.06 | 0.13 | 3.95 | 0.00 | 0.00 | 1.90 | 42.60 | 11.61 | 10.14 | 6.86 | 3.82 |
| FOKKER F28-2000 | 0.06 | 0.06 | 0.13 | 3.95 | 0.00 | 0.00 | 1.90 | 42.60 | 11.61 | 10.14 | 6.86 | 3.82 |
| FOKKER F28-3000 | 0.06 | 0.06 | 0.13 | 3.95 | 0.00 | 0.00 | 1.90 | 42.60 | 11.61 | 10.14 | 6.86 | 3.82 |
| FOKKER F28-4000/6000 | 0.06 | 0.06 | 0.13 | 3.95 | 0.00 | 0.00 | 1.90 | 42.60 | 11.61 | 10.14 | 6.86 | 3.82 |
| GA GULFSTREAM 3 | 0.09 | 0.12 | 0.18 | 3.69 | 0.12 | 0.63 | 2.65 | 31.77 | 22.70 | 17.30 | 7.20 | 3.60 |
| GAF NOMAD N22B/N24A | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| GATES LEARJET 24/25D | 0.11 | 0.13 | 4.26 | 20.04 | 1.39 | 2.03 | 22.38 | 58.60 | 15.25 | 13.08 | 5.90 | 2.82 |
| GATES LEARJET 25B | 0.11 | 0.13 | 4.26 | 20.04 | 1.39 | 2.03 | 22.38 | 58.60 | 15.25 | 13.08 | 5.90 | 2.82 |
| GATES LEARJET 31 | 0.11 | 0.13 | 4.26 | 20.04 | 1.39 | 2.03 | 22.38 | 58.60 | 15.25 | 13.08 | 5.90 | 2.82 |
| GATES LEARJET 35A/36A | 0.11 | 0.13 | 4.26 | 20.04 | 1.39 | 2.03 | 22.38 | 58.60 | 15.25 | 13.08 | 5.90 | 2.82 |
| GATES LEARJET 55 LONGHORN | 0.06 | 0.07 | 1.41 | 9.04 | 1.13 | 1.62 | 15.56 | 47.70 | 19.15 | 16.02 | 6.92 | 3.72 |
| GROB 6109B | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| GRUMMAN GA-7 COUGAR | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| GULF AMERICAN GULFSTREAM I | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| GULF AMERICAN GULFSTREAM II | 0.09 | 0.12 | 0.18 | 3.69 | 0.12 | 0.63 | 2.65 | 31.77 | 22.70 | 17.30 | 7.20 | 3.60 |
| GULF AMERICAN GULFSTREAM IV | 0.80 | 0.30 | 0.90 | 3.40 | 0.70 | 0.80 | 3.90 | 24.10 | 21.10 | 16.80 | 5.70 | 2.50 |
| GULF AMERICAN GULFSTREAM V | 0.02 | 0.02 | 0.05 | 1.12 | 1.04 | 0.93 | 4.81 | 27.99 | 18.73 | 15.03 | 7.67 | 4.67 |
| HANDLEY PAGE HERALD 200 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| HANDLEY PAGE HERALD 700 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| HAWKER SIDDELEY DH 104 DOVE | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| HAWKER SIDDELEY DH114 HERON | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| HILLER UH12 | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| HUGHES 269A | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| HUGHES 500 | 0.26 | 0.37 | 5.18 | 20.16 | 7.80 | 9.00 | 48.60 | 97.30 | 6.60 | 5.96 | 2.24 | 1.43 |
| ILYUSHIN IL12/IL14 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| ILYUSHIN IL18 | 0.28 | 0.89 | 1.96 | 17.61 | 2.00 | 2.10 | 5.10 | 43.60 | 8.88 | 9.22 | 7.49 | 3.52 |
| ILYUSHIN IL62 | 0.30 | 0.40 | 1.20 | 10.50 | 2.80 | 3.70 | 11.80 | 54.00 | 16.30 | 12.60 | 5.10 | 2.70 |
| ILYUSHIN IL76 | 0.50 | 0.60 | 1.20 | 52.00 | 3.90 | 4.20 | 9.30 | 54.40 | 12.80 | 12.10 | 5.10 | 2.70 |
| ILYUSHIN IL86 | 0.50 | 0.60 | 1.20 | 52.00 | 3.90 | 4.20 | 9.30 | 54.40 | 12.80 | 12.10 | 5.10 | 2.70 |
| ILYUSHIN IL96-300 | 0.12 | 0.12 | 0.20 | 0.30 | 0.35 | 0.40 | 0.90 | 6.90 | 37.00 | 31.50 | 11.80 | 5.80 |
| LEARJET 45 | 0.11 | 0.13 | 4.26 | 20.04 | 1.39 | 2.03 | 22.38 | 58.60 | 15.25 | 13.08 | 5.90 | 2.82 |
| LET 410 | 0.00 | 0.00 | 2.19 | 50.17 | 1.00 | 1.20 | 23.30 | 64.00 | 7.81 | 7.00 | 8.37 | 2.43 |
| LOCKHEED JETSTAR II | 0.01 | 0.01 | 0.12 | 4.72 | 0.20 | 0.30 | 4.43 | 37.83 | 14.52 | 12.02 | 6.39 | 3.28 |
| LOCKHEED L100 HERCULES | 0.28 | 0.89 | 1.96 | 17.61 | 2.00 | 2.10 | 5.10 | 43.60 | 8.88 | 9.22 | 7.49 | 3.52 |
| LOCKHEED L1011-1/100 TRISTAR | 0.15 | 0.25 | 5.96 | 67.75 | 0.78 | 1.68 | 20.65 | 88.99 | 37.33 | 26.89 | 8.18 | 2.86 |
| LOCKHEED L1011-200 TRISTAR | 0.15 | 0.25 | 5.96 | 67.75 | 0.78 | 1.68 | 20.65 | 88.99 | 37.33 | 26.89 | 8.18 | 2.86 |
| LOCKHEED L1011-500 TRISTAR | 0.51 | 0.38 | 4.51 | 45.29 | 1.71 | 2.55 | 17.99 | 74.58 | 47.58 | 33.57 | 9.76 | 3.60 |
| LOCKHEED L188 ELECTRA | 0.28 | 0.89 | 1.96 | 17.61 | 2.00 | 2.10 | 5.10 | 43.60 | 8.88 | 9.22 | 7.49 | 3.52 |
| MBB BK117 | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| MBB BO105 | 0.26 | 0.37 | 5.18 | 20.16 | 7.80 | 9.00 | 48.60 | 97.30 | 6.60 | 5.96 | 2.24 | 1.43 |
| Mc DONNELL DOUGLAS MD90 | 0.04 | 0.04 | 0.06 | 0.10 | 0.53 | 0.62 | 2.44 | 12.43 | 26.50 | 22.30 | 8.90 | 4.70 |
| MCDONNELL-DOUGLAS DC10-10 | 0.30 | 0.30 | 0.60 | 19.90 | 0.50 | 0.50 | 5.50 | 52.00 | 41.60 | 33.90 | 11.80 | 4.60 |
| MCDONNELL-DOUGLAS DC10-30 | 0.14 | 0.15 | 0.28 | 2.72 | 0.45 | 0.45 | 3.77 | 24.04 | 28.76 | 25.23 | 10.16 | 3.40 |
| MCDONNELL-DOUGLAS DC10-40 | 0.14 | 0.15 | 0.28 | 2.72 | 0.45 | 0.45 | 3.71 | 24.04 | 28.97 | 25.50 | 10.16 | 3.40 |
| MCDONNELL-DOUGLAS DC8-10/50 | 0.04 | 0.04 | 0.08 | 1.13 | 0.90 | 0.90 | 3.40 | 23.50 | 20.40 | 17.30 | 8.70 | 4.30 |
| MCDONNELL-DOUGLAS DC861/3 71/3 | 0.04 | 0.04 | 0.08 | 1.13 | 0.90 | 0.90 | 3.40 | 23.50 | 20.40 | 17.30 | 8.70 | 4.30 |
| MCDONNELL-DOUGLAS DC8-62/72 | 0.04 | 0.04 | 0.08 | 1.13 | 0.90 | 0.90 | 3.40 | 23.50 | 20.40 | 17.30 | 8.70 | 4.30 |
| MCDONNELL-DOUGLAS DC8-71/73 | 0.04 | 0.04 | 0.08 | 1.13 | 0.90 | 0.90 | 3.40 | 23.50 | 20.40 | 17.30 | 8.70 | 4.30 |
| MCDONNELL-DOUGLAS DC8F 54/55 | 0.04 | 0.04 | 0.08 | 1.13 | 0.90 | 0.90 | 3.40 | 23.50 | 20.40 | 17.30 | 8.70 | 4.30 |
| MCDONNELL-DOUGLAS DC9-10/15 | 0.12 | 0.14 | 1.50 | 43.60 | 2.70 | 3.20 | 14.50 | 60.30 | 19.10 | 16.30 | 7.00 | 3.60 |
| McDONNELL-DOUGLAS DC9-20 | 0.12 | 0.14 | 1.50 | 43.60 | 2.70 | 3.20 | 14.50 | 60.30 | 19.10 | 16.30 | 7.00 | 3.60 |
| MCDONNELL-DOUGLAS DC9-30 | 0.12 | 0.14 | 1.50 | 43.60 | 2.70 | 3.20 | 14.50 | 60.30 | 19.10 | 16.30 | 7.00 | 3.60 |
| MCDONNELL-DOUGLAS DC9-40 | 0.12 | 0.14 | 1.50 | 43.60 | 2.70 | 3.20 | 14.50 | 60.30 | 19.10 | 16.30 | 7.00 | 3.60 |
| MCDONNELL-DOUGLAS DC9-50 | 0.12 | 0.14 | 1.50 | 43.60 | 2.70 | 3.20 | 14.50 | 60.30 | 19.10 | 16.30 | 7.00 | 3.60 |
| MCDONNELL-DOUGLAS MD11 | 0.08 | 0.08 | 0.15 | 3.73 | 0.40 | 0.52 | 1.74 | 25.83 | 34.89 | 26.34 | 12.53 | 4.07 |
| MCDONNELL-DOUGLAS MD80-MD83 | 0.00 | 0.00 | 0.00 | 0.00 | 0.42 | 0.48 | 3.66 | 17.14 | 17.47 | 13.37 | 7.65 | 4.19 |
| MCDONNELL-DOUGLAS MD87 | 0.00 | 0.00 | 0.00 | 0.00 | 0.42 | 0.49 | 3.79 | 17.89 | 16.49 | 13.02 | 7.65 | 4.05 |
| MCDONNELL-DOUGLAS MD88 | 0.00 | 0.00 | 0.00 | 0.00 | 0.42 | 0.49 | 3.79 | 17.88 | 16.52 | 13.03 | 7.65 | 4.05 |
| MITSUBISHI MU2 | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| MOONEY M20J | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| NAMC YS11 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| NORD 2501 NORTALAS | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| PARTENAVIA P68B/C | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| PIAGGIO P.180 AVANTI | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| PIAGGIO P166-DL3 | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| PILATUS BN-2A ISLANDER | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| PILATUS BN-2A TRISLANDER MK3 | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| PIPER PA18 SUPER CUB | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| PIPER PA23 AZTEC/APACHE | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| PIPER PA24 COMMANCHE | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| PIPER PA28 CHEROKEE SRS/PA32 | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| PIPER PA30/PA39 TWIN COMANCHE | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| PIPER PA31/P NAVAJO CHIEFTAIN | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| PIPER PA31T CHEYENNE I/II | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| PIPER PA34 SENECA II | 5.21 | 5.44 | 6.64 | 112.38 | 608.98 | 540.89 | 562.81 | 360.02 | 6.93 | 7.62 | 6.46 | 2.33 |
| PIPER PA42 CHEYENNE III/IV | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| PIPER PA44 SEMINOLE | 5.21 | 5.44 | 6.64 | 112.38 | 608.98 | 540.89 | 562.81 | 360.02 | 6.93 | 7.62 | 6.46 | 2.33 |
| REIMS-CESSNA F406/CARAVAN II | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| RILEY TURBO-EXEC 400 | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| RO TURBO COMMANDER 680T-690 | 0.11 | 0.15 | 0.64 | 79.11 | 0.80 | 1.00 | 7.00 | 61.50 | 12.36 | 11.86 | 9.92 | 2.86 |
| ROBINSON R22 | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| ROCKWELL SABRELINER SERIES | 0.06 | 0.07 | 1.41 | 9.04 | 1.13 | 1.62 | 15.56 | 47.70 | 19.15 | 16.02 | 6.92 | 3.72 |
| ROCKWELL SHRIKE/COURSER/COMMAN | 10.00 | 8.16 | 9.70 | 49.20 | 1199.00 | 983.30 | 691.30 | 897.40 | 1.99 | 4.59 | 10.16 | 1.16 |
| ROCKWELL TURBO COMMANDER | 0.11 | 0.15 | 0.64 | 79.11 | 0.80 | 1.00 | 7.00 | 61.50 | 12.36 | 11.86 | 9.92 | 2.86 |
| SAAB 2000 | 0.25 | 0.29 | 0.64 | 2.51 | 0.75 | 0.92 | 3.28 | 17.35 | 20.54 | 17.47 | 7.79 | 3.83 |
| SAAB FAIRCHILD 340 | 0.63 | 0.70 | 1.86 | 13.50 | 2.00 | 2.24 | 18.55 | 56.92 | 12.69 | 10.67 | 4.06 | 2.11 |
| SHORTS 330 | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| SHORTS 360 | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| SHORTS BELFAST | 0.28 | 0.89 | 1.96 | 17.61 | 2.00 | 2.10 | 5.10 | 43.60 | 8.88 | 9.22 | 7.49 | 3.52 |
| SHORTS SC7 SKYLINER | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| SHORTS SC7 SKYVAN | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| SIKORSKY S61N | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| SIKORSKY S76 SPIRIT | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| SOCATA TB10 TOBAGO | 12.36 | 16.63 | 13.38 | 68.08 | 1442.10 | 1470.90 | 1261.60 | 1293.70 | 0.36 | 0.24 | 1.39 | 0.39 |
| SWEARINGEN MERLIN IIA/IIB/IIIB | 0.11 | 0.15 | 0.64 | 79.11 | 0.80 | 1.00 | 7.00 | 61.50 | 12.36 | 11.86 | 9.92 | 2.86 |
| SWEARINGEN MERLIN IVA | 0.11 | 0.15 | 0.64 | 79.11 | 0.80 | 1.00 | 7.00 | 61.50 | 12.36 | 11.86 | 9.92 | 2.86 |
| SWEARINGEN METRO II | 0.11 | 0.15 | 0.64 | 79.11 | 0.80 | 1.00 | 7.00 | 61.50 | 12.36 | 11.86 | 9.92 | 2.86 |
| TUPOLEV TU104 | 0.00 | 0.00 | 0.00 | 0.00 | 0.42 | 0.48 | 3.66 | 17.14 | 17.47 | 13.37 | 7.65 | 4.19 |
| TUPOLEV TU124 | 0.09 | 0.12 | 0.18 | 3.69 | 0.12 | 0.63 | 2.65 | 31.77 | 22.70 | 17.30 | 7.20 | 3.60 |
| TUPOLEV TU134 | 0.12 | 0.14 | 1.50 | 43.60 | 2.70 | 3.20 | 14.50 | 60.30 | 19.10 | 16.30 | 7.00 | 3.60 |
| TUPOLEV TU154A/B | 2.00 | 2.00 | 2.60 | 32.00 | 5.00 | 6.00 | 13.00 | 64.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| TUPOLEV TU154M | 0.40 | 0.50 | 1.90 | 12.70 | 3.00 | 3.60 | 18.20 | 77.70 | 14.50 | 11.60 | 5.10 | 2.90 |
| TUPOLEV TU204 | 0.03 | 0.00 | 0.04 | 0.27 | 0.26 | 0.29 | 2.71 | 20.26 | 22.45 | 17.63 | 8.39 | 4.40 |
| V953C MERCHANTMAN | 0.28 | 0.89 | 1.96 | 17.61 | 2.00 | 2.10 | 5.10 | 43.60 | 8.88 | 9.22 | 7.49 | 3.52 |
| VICKERS VISCOUNT 700 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| VICKERS VISCOUNT 800 | 1.00 | 1.10 | 3.00 | 23.90 | 3.20 | 3.50 | 33.30 | 91.40 | 5.60 | 4.50 | 0.90 | 0.70 |
| WESTLAND 30 SRS 100 | 1.75 | 2.03 | 22.71 | 101.63 | 5.10 | 6.50 | 34.80 | 115.30 | 7.98 | 7.57 | 4.65 | 1.97 |
| YAKOVLEV YAK-40 | 0.06 | 0.07 | 1.41 | 9.04 | 1.13 | 1.62 | 15.56 | 47.70 | 19.15 | 16.02 | 6.92 | 3.72 |
| YAKOVLEV YAK-42 | 2.00 | 2.00 | 2.60 | 32.00 | 5.00 | 6.00 | 13.00 | 64.00 | 0.00 | 0.00 | 0.00 | 0.00 |

Table A3.2b	Emission factors and fuel flow rates – Gatwick

| CAA Aircraft Type | SN_100 | SN_85 | SN_30 | SN_7 | Fuel_100 | Fuel_85 | Fuel_30 | Fuel_7 | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AEROSPACELINES B377SUPER GUPPY | 0.00 | 0.00 | 0.00 | 0.00 | 0.2994 | 0.2769 | 0.1436 | 0.0769 | Surrogate LOCKHEED L100 HERCULES |
| AEROSPATIALE (NORD)262 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate SHORTS 360 |
| AEROSPATIALE AS332 SUPER PUMA (L1/L2) | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate AEROSPATIALE SA365 DAUPHIN |
| AEROSPATIALE AS355 ECUREUIL 2 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate AEROSPATIALE SA365 DAUPHIN |
| AEROSPATIALE CARAVELLE 10B/10R | 21.90 | 21.90 | 21.90 | 21.90 | 0.8910 | 0.7260 | 0.2780 | 0.1270 | Surrogate GA GULFSTREAM 3 |
| AEROSPATIALE CARAVELLE 12 | 21.90 | 21.90 | 21.90 | 21.90 | 0.8910 | 0.7260 | 0.2780 | 0.1270 | Surrogate GA GULFSTREAM 3 |
| AEROSPATIALE CARAVELLE 6/6R | 21.90 | 21.90 | 21.90 | 21.90 | 0.8910 | 0.7260 | 0.2780 | 0.1270 | Surrogate GA GULFSTREAM 3 |
| AEROSPATIALE CORVETTE | 14.10 | 14.10 | 14.10 | 14.10 | 0.1697 | 0.1430 | 0.0590 | 0.0261 |  |
| AEROSPATIALE SA315 LAMA | 0.00 | 0.00 | 0.00 | 0.00 | 0.0334 | 0.0309 | 0.0107 | 0.0079 | Surrogate AEROSPATIALE SA350 ECUREUIL |
| AEROSPATIALE SA330 PUMA | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate AEROSPATIALE SA365 DAUPHIN |
| AEROSPATIALE SA341 GAZELLE | 0.00 | 0.00 | 0.00 | 0.00 | 0.0334 | 0.0309 | 0.0107 | 0.0079 | Surrogate AEROSPATIALE SA350 ECUREUIL |
| AEROSPATIALE SA350 ECUREUIL | 0.00 | 0.00 | 0.00 | 0.00 | 0.0334 | 0.0309 | 0.0107 | 0.0079 |  |
| AEROSPATIALE SA365 DAUPHIN | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 |  |
| AEROSPATIALE SA365 DAUPHIN/AMB | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate AEROSPATIALE SA365 DAUPHIN |
| AGUSTA A109A | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate AEROSPATIALE SA365 DAUPHIN |
| AIRBUS A300-600 | 6.94 | 5.36 | 0.00 | 0.00 | 2.5799 | 2.0959 | 0.6720 | 0.2050 |  |
| AIRBUS A300B1/B2 | 10.95 | 11.72 | 1.62 | 1.40 | 2.3610 | 1.9400 | 0.6630 | 0.1630 |  |
| AIRBUS A300B4-100/200 | 10.95 | 11.72 | 1.62 | 1.40 | 2.3610 | 1.9400 | 0.6630 | 0.1630 | Surrogate AIRBUS A300B1/B2 |
| AIRBUS A300F4 | 10.95 | 11.72 | 1.62 | 1.40 | 2.3610 | 1.9400 | 0.6630 | 0.1630 | Heathrow data |
| AIRBUS A310-202 | 12.00 | 10.00 | 2.00 | 2.00 | 2.2540 | 1.8850 | 0.6410 | 0.1500 |  |
| AIRBUS A310-300 | 6.86 | 4.92 | 1.54 | 1.54 | 2.3302 | 1.9032 | 0.6296 | 0.1992 |  |
| AIRBUS A319 | 5.87 | 5.87 | 5.87 | 5.87 | 0.9078 | 0.7569 | 0.2677 | 0.0976 |  |
| AIRBUS A320-100/200 | 7.00 | 6.82 | 5.97 | 5.78 | 1.1049 | 0.9160 | 0.3192 | 0.1163 |  |
| AIRBUS A321 | 7.51 | 7.51 | 7.51 | 7.51 | 1.4273 | 1.1428 | 0.3800 | 0.1275 |  |
| AIRBUS A330-200 | 11.86 | 11.86 | 11.85 | 11.85 | 2.3755 | 1.9379 | 0.6585 | 0.2604 |  |
| AIRBUS A330-300 | 3.95 | 3.95 | 3.95 | 3.95 | 2.8450 | 2.3332 | 0.7993 | 0.2225 |  |
| AIRBUS A340-200 | 12.58 | 9.80 | 1.03 | 1.10 | 1.4560 | 1.1950 | 0.3860 | 0.1240 |  |
| AIRBUS A340-300 | 11.51 | 8.86 | 1.01 | 1.10 | 1.3488 | 1.1088 | 0.3641 | 0.1193 |  |
| AIRBUS A340-600 | 2.24 | 2.09 | 1.02 | 0.76 | 2.2400 | 1.8300 | 0.6200 | 0.2300 | Heathrow data |
| AIRSHIP INDUSTRIES SKYSHIP 500 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 310 |
| AIRSHIP INDUSTRIES SKYSHIP 600 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 310 |
| ANTONOV AN-12 | 0.00 | 0.00 | 0.00 | 0.00 | 0.2994 | 0.2769 | 0.1436 | 0.0769 | Surrogate LOCKHEED L100 HERCULES |
| ANTONOV AN-124 | 13.00 | 13.00 | 13.00 | 13.00 | 1.7390 | 1.4310 | 0.4890 | 0.1780 | Surrogate ILYUSHIN IL96-300 |
| ANTONOV AN-24 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate FOKKER 50 |
| ANTONOV AN26B/32 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate FOKKER 50 |
| ANTONOV AN72 | 33.00 | 35.00 | 27.00 | 8.00 | 1.1500 | 0.9750 | 0.3500 | 0.1300 | Surrogate TUPOLEV TU134 |
| ARMSTRONG WHITWORTH ARGOSY | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate ATR72 |
| ATR42-300 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 |  |
| ATR42-500 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate ATR42-300 |
| ATR72 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 |  |
| AVROLINER RJ100/115 | 10.30 | 10.20 | 6.90 | 6.80 | 0.3578 | 0.2961 | 0.1083 | 0.0453 |  |
| AVROLINER RJ70 | 10.30 | 10.20 | 6.90 | 6.80 | 0.3578 | 0.2961 | 0.1083 | 0.0453 |  |
| AVROLINER RJ85/QT | 10.30 | 10.20 | 6.90 | 6.80 | 0.3578 | 0.2961 | 0.1083 | 0.0453 |  |
| BAC/AEROSPATIALE CONCORDE | 19.50 | 26.10 | 26.40 | 26.40 | 6.3650 | 2.3290 | 1.1710 | 0.4210 |  |
| BAE (HS) 748 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate FOKKER 50 |
| BAE 146-100 | 13.50 | 12.70 | 5.70 | 2.30 | 0.3581 | 0.2955 | 0.1034 | 0.0408 |  |
| BAE 146-200/QT | 13.50 | 12.70 | 5.70 | 2.30 | 0.3581 | 0.2955 | 0.1034 | 0.0408 |  |
| BAE 146-300 | 13.50 | 12.70 | 5.70 | 2.30 | 0.3581 | 0.2955 | 0.1034 | 0.0408 |  |
| BAE ATP | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 |  |
| BAE JETSTREAM 31/32 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0577 | 0.0515 | 0.0315 | 0.0141 |  |
| BAE JETSTREAM 41 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0577 | 0.0515 | 0.0315 | 0.0141 |  |
| BAE(BAC)111-200 | 21.90 | 21.90 | 21.90 | 21.90 | 0.8910 | 0.7260 | 0.2780 | 0.1270 | Surrogate GA GULFSTREAM 3 |
| BAE(BAC)111-300/400/475 | 21.90 | 21.90 | 21.90 | 21.90 | 0.8910 | 0.7260 | 0.2780 | 0.1270 | Surrogate GA GULFSTREAM 3 |
| BAE(BAC)111-500 | 21.90 | 21.90 | 21.90 | 21.90 | 0.8910 | 0.7260 | 0.2780 | 0.1270 | Surrogate GA GULFSTREAM 3 |
| BAE(HS)125 | 0.00 | 0.00 | 0.00 | 0.00 | 0.2250 | 0.1860 | 0.0720 | 0.0260 |  |
| BAE125-1000 | 0.00 | 0.00 | 0.00 | 0.00 | 0.2250 | 0.1860 | 0.0720 | 0.0260 | Surrogate BAE(HS)125 |
| BEAGLE B206 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR |
| BEECH KINGAIR 100 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate EMBRAER EMB110 BANDEIRANTE |
| BEECH KINGAIR 90 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 |  |
| BEECH KINGAIR C90A | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate BEECH KINGAIR 90 |
| BEECHCRAFT 18/SUPER H18 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR |
| BEECHCRAFT 1900C/D AIRLINER | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 |  |
| BEECHCRAFT 200 SUPERKING AIR | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 |  |
| BEECHCRAFT 300 SUPER KING AIR | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 |  |
| BEECHCRAFT 400 BEECHJET | 18.50 | 18.50 | 18.50 | 18.50 | 0.2054 | 0.1727 | 0.0660 | 0.0296 | Heathrow data |
| BEECHCRAFT 50 TWIN BONANZA | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR |
| BEECHCRAFT 76 DUCHESS | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR |
| BEECHCRAFT 95 TRAVEL AIR | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR |
| BEECHCRAFT 99/99A | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate EMBRAER EMB110 BANDEIRANTE |
| BEECHCRAFT BARON MOD 55/58/58P | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR |
| BEECHCRAFT DUKE | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR |
| BEECHCRAFT QUEEN AIR 65/80 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR |
| BEECHCRAFT STARSHIP MODEL 2000 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate SHORTS 360 |
| BELL 206B JET RANGER | 0.00 | 0.00 | 0.00 | 0.00 | 0.0334 | 0.0309 | 0.0107 | 0.0079 |  |
| BELL 206L LONG RANGER | 0.00 | 0.00 | 0.00 | 0.00 | 0.0334 | 0.0309 | 0.0107 | 0.0079 |  |
| BELL 212/412 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0334 | 0.0309 | 0.0107 | 0.0079 | Surrogate BELL 222 |
| BELL 222 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0334 | 0.0309 | 0.0107 | 0.0079 |  |
| BELL 47G | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 210 CENTURION |
| BELL MODEL 214ST | 0.00 | 0.00 | 0.00 | 0.00 | 0.0334 | 0.0309 | 0.0107 | 0.0079 |  |
| BOEING 707 ALL SERIES | 18.00 | 18.00 | 18.00 | 18.00 | 1.2540 | 1.0320 | 0.3890 | 0.1280 |  |
| BOEING 707-120/121B | 18.00 | 18.00 | 18.00 | 18.00 | 1.2540 | 1.0320 | 0.3890 | 0.1280 | Surrogate BOEING 707 ALL SERIES |
| BOEING 717-200 | 3.24 | 2.73 | 0.97 | 0.91 | 0.8360 | 0.6900 | 0.2450 | 0.0960 |  |
| BOEING 720B | 18.00 | 18.00 | 18.00 | 18.00 | 1.2540 | 1.0320 | 0.3890 | 0.1280 | Surrogate BOEING 707 ALL SERIES |
| BOEING 727-100/100C | 22.20 | 22.20 | 22.20 | 22.20 | 0.9892 | 0.8113 | 0.2861 | 0.1291 |  |
| BOEING 727-200/200 ADVANCED | 23.00 | 23.00 | 23.00 | 23.00 | 1.0400 | 0.8460 | 0.2980 | 0.1320 |  |
| BOEING 737-100 | 21.25 | 21.25 | 21.25 | 21.25 | 1.1619 | 0.9318 | 0.3325 | 0.1445 | Surrogate BOEING 737-200 |
| BOEING 737-200 | 21.25 | 21.25 | 21.25 | 21.25 | 1.1619 | 0.9318 | 0.3325 | 0.1445 |  |
| BOEING 737-300 | 6.51 | 3.34 | 2.50 | 2.24 | 1.0859 | 0.9011 | 0.3208 | 0.1206 |  |
| BOEING 737-400 | 7.69 | 3.90 | 2.50 | 2.30 | 1.1536 | 0.9537 | 0.3359 | 0.1240 |  |
| BOEING 737-500 | 7.52 | 3.83 | 2.50 | 2.30 | 1.1439 | 0.9461 | 0.3338 | 0.1235 |  |
| BOEING 737-600 | 11.40 | 11.40 | 11.40 | 11.40 | 0.9130 | 0.7610 | 0.2740 | 0.1000 |  |
| BOEING 737-700 | 11.54 | 11.54 | 11.54 | 11.54 | 0.9374 | 0.7798 | 0.2794 | 0.1011 |  |
| BOEING 737-800 | 14.62 | 14.62 | 14.62 | 14.62 | 1.2174 | 0.9957 | 0.3372 | 0.1130 |  |
| BOEING 737-900 | 14.70 | 14.70 | 14.70 | 14.70 | 1.2210 | 0.9990 | 0.3380 | 0.1130 | Heathrow data |
| BOEING 747-100/100F | 12.30 | 9.50 | 2.50 | 0.70 | 2.0990 | 1.7890 | 0.6190 | 0.2110 |  |
| BOEING 747-200B | 12.04 | 11.97 | 11.80 | 11.75 | 2.3496 | 1.9131 | 0.6558 | 0.2587 |  |
| BOEING 747-200B (COMBI) | 12.30 | 12.30 | 12.30 | 12.30 | 2.3400 | 1.9100 | 0.6500 | 0.2600 |  |
| BOEING 747-200C/200F | 8.86 | 8.86 | 8.86 | 8.86 | 2.4215 | 1.9817 | 0.6743 | 0.2416 |  |
| BOEING 747-300(STRETCH UP DK) | 10.10 | 10.10 | 10.10 | 10.10 | 2.4509 | 1.9400 | 0.6937 | 0.2565 |  |
| BOEING 747-300M (COMBI) | 10.95 | 11.72 | 1.62 | 1.40 | 2.3610 | 1.9400 | 0.6630 | 0.1630 | Heathrow data |
| BOEING 747-400 | 6.84 | 4.50 | 0.08 | 0.08 | 2.4290 | 1.9872 | 0.6515 | 0.2003 |  |
| BOEING 747-400F | 7.84 | 6.52 | 3.40 | 3.40 | 2.5514 | 2.0620 | 0.6754 | 0.2068 |  |
| BOEING 747-400M (COMBI) | 7.40 | 5.75 | 2.67 | 2.67 | 2.3997 | 1.9607 | 0.6478 | 0.2018 | Heathrow data |
| BOEING 747SP | 14.63 | 12.13 | 5.93 | 4.57 | 2.1855 | 1.8240 | 0.6388 | 0.2315 |  |
| BOEING 757-200 | 7.31 | 7.31 | 7.31 | 7.31 | 1.8567 | 1.5043 | 0.5209 | 0.1803 |  |
| BOEING 757-300 | 7.30 | 7.30 | 7.30 | 7.30 | 2.0700 | 1.6400 | 0.5500 | 0.1900 |  |
| BOEING 767-200 | 4.60 | 4.60 | 4.60 | 4.60 | 2.0550 | 1.6780 | 0.7593 | 0.2054 |  |
| BOEING 767-200ER | 9.13 | 7.44 | 2.44 | 2.44 | 2.2448 | 1.8570 | 0.6435 | 0.1757 |  |
| BOEING 767-300 | 4.07 | 4.07 | 4.07 | 4.07 | 2.7269 | 2.1672 | 0.7093 | 0.2588 |  |
| BOEING 767-300ER/F | 7.15 | 5.69 | 0.87 | 0.87 | 2.5949 | 2.1002 | 0.6817 | 0.2044 |  |
| BOEING 767-400ER | 7.15 | 5.69 | 0.87 | 0.87 | 2.5949 | 2.1002 | 0.6817 | 0.2044 | Surrogate BOEING 767-300ER/F |
| BOEING 777-200 | 5.34 | 5.34 | 5.34 | 5.34 | 3.2100 | 2.6600 | 0.9000 | 0.2800 |  |
| BOEING 777-200ER | 2.35 | 2.26 | 2.31 | 2.25 | 3.5024 | 2.8225 | 0.9194 | 0.2997 |  |
| BOEING 777-300 | 5.34 | 5.34 | 5.34 | 5.34 | 3.9100 | 3.1000 | 1.0000 | 0.3000 |  |
| BOEING-VERTOL MOD-234 CHINOOK | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate AEROSPATIALE SA365 DAUPHIN |
| BOMBARDIER DASH 8 Q100/200 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate BOMBARDIER DASH 8 Q400 |
| BOMBARDIER DASH 8 Q400 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 |  |
| BOMBARDIER REGIONAL JET 100/200 | 21.90 | 9.20 | 3.10 | 3.10 | 0.4070 | 0.3343 | 0.1190 | 0.0496 |  |
| BOMBARDIER REGIONAL JET RJ700 | 5.33 | 1.48 | 0.00 | 0.00 | 0.6042 | 0.4941 | 0.1681 | 0.0690 |  |
| BOMBARDIER REGIONAL JET RJ700ER | 5.33 | 1.48 | 0.00 | 0.00 | 0.6042 | 0.4941 | 0.1681 | 0.0690 | Surrogate BOMBARDIER REGIONAL JET RJ700 |
| BRISTOL 170 FREIGHTER | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate ATR42-300 |
| CANADAIR CL-44 | 0.00 | 0.00 | 0.00 | 0.00 | 0.2994 | 0.2769 | 0.1436 | 0.0769 | Surrogate LOCKHEED L100 HERCULES |
| CANADAIR CL-600-604 CHALLENGER | 21.90 | 9.20 | 3.10 | 3.10 | 0.4070 | 0.3343 | 0.1190 | 0.0496 | Surrogate BOMBARDIER REGIONAL JET 100/200 |
| CANADAIR GLOBAL EXPRESS | 33.00 | 35.00 | 27.00 | 8.00 | 1.1500 | 0.9750 | 0.3500 | 0.1300 | Surrogate TUPOLEV TU134 |
| CESSNA 150 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 210 CENTURION |
| CESSNA 172 SKYHAWK | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 210 CENTURION |
| CESSNA 180 SKYWAGON | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 210 CENTURION |
| CESSNA 206 STATIONAIR | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 210 CENTURION |
| CESSNA 208 CARAVAN I | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 |  |
| CESSNA 210 CENTURION | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 |  |
| CESSNA 310 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Heathrow data |
| CESSNA 336/337 SKYMASTER | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 310 |
| CESSNA 337 SUPER SKYMASTER | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 310 |
| CESSNA 340 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 310 |
| CESSNA 401/402/411/421 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR |
| CESSNA 404 TITAN | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR |
| CESSNA 414A CHANCELLOR | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Heathrow data |
| CESSNA 425 CONQUEST I | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR |
| CESSNA 441 CONQUEST II | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR |
| CESSNA 500 CITATION I | 15.00 | 15.00 | 15.00 | 15.00 | 0.1480 | 0.1240 | 0.0510 | 0.0230 |  |
| CESSNA 525 CITATIONJET | 15.00 | 15.00 | 15.00 | 15.00 | 0.1480 | 0.1240 | 0.0510 | 0.0230 | Surrogate CESSNA 500 CITATION I |
| CESSNA 550 CITATION II | 15.25 | 14.58 | 15.25 | 15.25 | 0.1861 | 0.1564 | 0.0622 | 0.0267 |  |
| CESSNA 560 CITATION V | 17.13 | 15.17 | 17.13 | 17.13 | 0.2149 | 0.1800 | 0.0678 | 0.0276 |  |
| CESSNA 650 CITATION III/VI/VII | 0.00 | 0.00 | 0.00 | 0.00 | 0.2250 | 0.1860 | 0.0720 | 0.0260 |  |
| CESSNA 750 CITATION X | 1.00 | 0.00 | 0.00 | 0.00 | 0.2800 | 0.2340 | 0.0910 | 0.0390 |  |
| CESSNA T303 CRUSADER | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 310 |
| CONVAIR 240/340/440 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate ATR42-300 |
| CONVAIR 580/600/640 | 1.00 | 0.00 | 0.00 | 0.00 | 0.3770 | 0.3150 | 0.1170 | 0.0490 | Surrogate SAAB 2000 |
| CONVAIR 880 | 13.50 | 12.70 | 5.70 | 2.30 | 0.3581 | 0.2955 | 0.1034 | 0.0408 | Surrogate BAE 146-100 |
| DASSAULT BREGUET FALCON 50 | 0.00 | 0.00 | 0.00 | 0.00 | 0.2250 | 0.1860 | 0.0720 | 0.0260 |  |
| DASSAULT MYSTERE-FALCON 10 | 0.00 | 0.00 | 0.00 | 0.00 | 0.2050 | 0.1730 | 0.0670 | 0.0240 |  |
| DASSAULT MYSTERE-FALCON 20 | 23.41 | 19.70 | 10.37 | 5.78 | 0.3013 | 0.2527 | 0.1046 | 0.0497 |  |
| DASSAULT MYSTERE-FALCON 2000 | 31.60 | 26.60 | 14.00 | 7.80 | 0.3280 | 0.2760 | 0.1160 | 0.0580 |  |
| DASSAULT MYSTERE-FALCON 900 | 0.00 | 0.00 | 0.00 | 0.00 | 0.2250 | 0.1860 | 0.0720 | 0.0260 |  |
| DASSAULT MYSTERE-FALCON 900EX | 0.00 | 0.00 | 0.00 | 0.00 | 0.2250 | 0.1860 | 0.0720 | 0.0260 | Surrogate DASSAULT MYSTERE-FALCON 900 |
| DASSAULT-BREGUET MERCURE | 33.00 | 35.00 | 27.00 | 8.00 | 1.1500 | 0.9750 | 0.3500 | 0.1300 | Surrogate TUPOLEV TU134 |
| DE HAVILLAND DASH 8-300/Q300 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 |  |
| DE HAVILLAND DASH 8-100 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 |  |
| DE HAVILLAND DH6 TWIN OTTER | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate BEECH KINGAIR 90 |
| DE HAVILLAND DHC-7 DASH-7 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate ATR72 |
| DE HAVILLAND DRAGON RAPIDE | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR |
| DE HAVILLAND TIGER MOTH | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 210 CENTURION |
| DHC2 BEAVER | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 210 CENTURION |
| DORNIER 228-100/200 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate ATR42-300 |
| DORNIER 328 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 |  |
| DORNIER 328 JET | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Heathrow data |
| DORNIER DO28D SKYSERVANT | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR |
| DOUGLAS DC3 C47 DAKOTA | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate SHORTS 360 |
| DOUGLAS DC4 SKYMASTER | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate SHORTS 360 |
| DOUGLAS DC6/6A/6B/6C | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate SHORTS 360 |
| EMBRAER EMB110 BANDEIRANTE | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 |  |
| EMBRAER EMB120 BRASILIA | 4.90 | 4.40 | 4.40 | 4.40 | 0.2790 | 0.2380 | 0.0992 | 0.0509 |  |
| EMBRAER EMB121 XINGU | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate EMBRAER EMB110 BANDEIRANTE |
| EMBRAER RJ135 | 1.00 | 0.00 | 0.00 | 0.00 | 0.3783 | 0.3168 | 0.1170 | 0.0494 |  |
| EMBRAER RJ145 | 1.00 | 0.00 | 0.00 | 0.00 | 0.3783 | 0.3167 | 0.1170 | 0.0494 |  |
| ENSTROM F28A/F280 SHARK | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 210 CENTURION |
| EUROCOPTER EC135T | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate AEROSPATIALE SA365 DAUPHIN |
| FAIRCHILD HILLER FH 227B | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate FOKKER 50 |
| FAIRCHILD SA-227 METRO 23 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0577 | 0.0515 | 0.0315 | 0.0141 | Surrogate SWEARINGEN METRO II |
| FAIRCHILD SA-227 METRO III | 0.00 | 0.00 | 0.00 | 0.00 | 0.0577 | 0.0515 | 0.0315 | 0.0141 | Surrogate SWEARINGEN METRO II |
| FOKKER 100 | 9.87 | 9.87 | 9.87 | 9.87 | 0.8600 | 0.7100 | 0.2500 | 0.1200 |  |
| FOKKER 50 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 |  |
| FOKKER 70 | 21.30 | 21.30 | 21.30 | 21.30 | 0.7600 | 0.6300 | 0.2300 | 0.1100 |  |
| FOKKER F27 100-400/600 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate FOKKER 50 |
| FOKKER F27-500 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate FOKKER 50 |
| FOKKER F28-1000 | 21.90 | 9.20 | 3.10 | 3.10 | 0.4070 | 0.3343 | 0.1190 | 0.0496 | Surrogate BOMBARDIER REGIONAL JET 100/200 |
| FOKKER F28-2000 | 21.90 | 9.20 | 3.10 | 3.10 | 0.4070 | 0.3343 | 0.1190 | 0.0496 | Surrogate BOMBARDIER REGIONAL JET 100/200 |
| FOKKER F28-3000 | 21.90 | 9.20 | 3.10 | 3.10 | 0.4070 | 0.3343 | 0.1190 | 0.0496 | Surrogate BOMBARDIER REGIONAL JET 100/200 |
| FOKKER F28-4000/6000 | 21.90 | 9.20 | 3.10 | 3.10 | 0.4070 | 0.3343 | 0.1190 | 0.0496 | Surrogate BOMBARDIER REGIONAL JET 100/200 |
| GA GULFSTREAM 3 | 21.90 | 21.90 | 21.90 | 21.90 | 0.8910 | 0.7260 | 0.2780 | 0.1270 |  |
| GAF NOMAD N22B/N24A | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR |
| GATES LEARJET 24/25D | 0.00 | 0.00 | 0.00 | 0.00 | 0.2050 | 0.1730 | 0.0670 | 0.0240 |  |
| GATES LEARJET 25B | 0.00 | 0.00 | 0.00 | 0.00 | 0.2050 | 0.1730 | 0.0670 | 0.0240 | Surrogate GATES LEARJET 24/25D |
| GATES LEARJET 31 | 0.00 | 0.00 | 0.00 | 0.00 | 0.2050 | 0.1730 | 0.0670 | 0.0240 |  |
| GATES LEARJET 35A/36A | 0.00 | 0.00 | 0.00 | 0.00 | 0.2050 | 0.1730 | 0.0670 | 0.0240 |  |
| GATES LEARJET 55 LONGHORN | 0.00 | 0.00 | 0.00 | 0.00 | 0.2250 | 0.1860 | 0.0720 | 0.0260 |  |
| GROB 6109B | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 210 CENTURION |
| GRUMMAN GA-7 COUGAR | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR |
| GULF AMERICAN GULFSTREAM I | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate ATR42-300 |
| GULF AMERICAN GULFSTREAM II | 21.90 | 21.90 | 21.90 | 21.90 | 0.8910 | 0.7260 | 0.2780 | 0.1270 |  |
| GULF AMERICAN GULFSTREAM IV | 21.30 | 21.30 | 21.30 | 21.30 | 0.7600 | 0.6300 | 0.2300 | 0.1100 |  |
| GULF AMERICAN GULFSTREAM V | 13.96 | 11.87 | 0.27 | 0.57 | 0.7139 | 0.5949 | 0.2140 | 0.0890 |  |
| HANDLEY PAGE HERALD 200 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate ATR72 |
| HANDLEY PAGE HERALD 700 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate ATR72 |
| HAWKER SIDDELEY DH 104 DOVE | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR |
| HAWKER SIDDELEY DH114 HERON | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR |
| HILLER UH12 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 210 CENTURION |
| HUGHES 269A | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 210 CENTURION |
| HUGHES 500 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0334 | 0.0309 | 0.0107 | 0.0079 | Surrogate AEROSPATIALE SA350 ECUREUIL |
| ILYUSHIN IL12/IL14 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate ATR42-300 |
| ILYUSHIN IL18 | 0.00 | 0.00 | 0.00 | 0.00 | 0.2994 | 0.2769 | 0.1436 | 0.0769 | Surrogate LOCKHEED L100 HERCULES |
| ILYUSHIN IL62 | 22.90 | 22.90 | 22.90 | 22.90 | 1.5200 | 1.3000 | 0.5000 | 0.2150 |  |
| ILYUSHIN IL76 | 22.10 | 22.10 | 22.10 | 22.10 | 2.4000 | 1.6000 | 0.5800 | 0.2100 | Surrogate ILYUSHIN IL86 |
| ILYUSHIN IL86 | 22.10 | 22.10 | 22.10 | 22.10 | 2.4000 | 1.6000 | 0.5800 | 0.2100 | Heathrow data |
| ILYUSHIN IL96-300 | 13.00 | 13.00 | 13.00 | 13.00 | 1.7390 | 1.4310 | 0.4890 | 0.1780 | Heathrow data |
| LEARJET 45 | 0.00 | 0.00 | 0.00 | 0.00 | 0.2050 | 0.1730 | 0.0670 | 0.0240 | Surrogate GATES LEARJET 35A/36A |
| LET 410 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0535 | 0.0504 | 0.0271 | 0.0145 | Heathrow data |
| LOCKHEED JETSTAR II | 10.30 | 10.20 | 6.90 | 6.80 | 0.3578 | 0.2961 | 0.1083 | 0.0453 | Surrogate AVROLINER RJ100/115 |
| LOCKHEED L100 HERCULES | 0.00 | 0.00 | 0.00 | 0.00 | 0.2994 | 0.2769 | 0.1436 | 0.0769 |  |
| LOCKHEED L1011-1/100 TRISTAR | 22.60 | 22.60 | 22.60 | 22.60 | 1.8770 | 1.5460 | 0.5660 | 0.2250 |  |
| LOCKHEED L1011-200 TRISTAR | 22.60 | 22.60 | 22.60 | 22.60 | 1.8770 | 1.5460 | 0.5660 | 0.2250 | Surrogate LOCKHEED L1011-1/100 TRISTAR |
| LOCKHEED L1011-500 TRISTAR | 17.29 | 13.37 | 3.75 | 2.14 | 2.3641 | 1.9227 | 0.6861 | 0.2685 |  |
| LOCKHEED L188 ELECTRA | 0.00 | 0.00 | 0.00 | 0.00 | 0.2994 | 0.2769 | 0.1436 | 0.0769 |  |
| MBB BK117 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate AEROSPATIALE SA365 DAUPHIN |
| MBB BO105 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0334 | 0.0309 | 0.0107 | 0.0079 |  |
| Mc DONNELL DOUGLAS MD90 | 9.00 | 9.00 | 9.00 | 9.00 | 1.0530 | 0.8800 | 0.3190 | 0.1280 | Heathrow data |
| MCDONNELL-DOUGLAS DC10-10 | 4.00 | 3.00 | 3.00 | 4.00 | 1.8120 | 1.5020 | 0.4940 | 0.1760 |  |
| MCDONNELL-DOUGLAS DC10-30 | 11.09 | 11.74 | 1.61 | 1.40 | 2.3428 | 1.9263 | 0.6574 | 0.1630 |  |
| MCDONNELL-DOUGLAS DC10-40 | 10.95 | 11.72 | 1.62 | 1.40 | 2.3610 | 1.9400 | 0.6630 | 0.1630 |  |
| MCDONNELL-DOUGLAS DC8-10/50 | 7.90 | 4.20 | 2.50 | 2.00 | 1.1140 | 0.9110 | 0.3180 | 0.1300 | Surrogate MCDONNELL-DOUGLAS DC8-62/72 |
| MCDONNELL-DOUGLAS DC861/3 71/3 | 7.90 | 4.20 | 2.50 | 2.00 | 1.1140 | 0.9110 | 0.3180 | 0.1300 | Surrogate MCDONNELL-DOUGLAS DC8-62/72 |
| MCDONNELL-DOUGLAS DC8-62/72 | 7.90 | 4.20 | 2.50 | 2.00 | 1.1140 | 0.9110 | 0.3180 | 0.1300 |  |
| MCDONNELL-DOUGLAS DC8-71/73 | 7.90 | 4.20 | 2.50 | 2.00 | 1.1140 | 0.9110 | 0.3180 | 0.1300 |  |
| MCDONNELL-DOUGLAS DC8F 54/55 | 7.90 | 4.20 | 2.50 | 2.00 | 1.1140 | 0.9110 | 0.3180 | 0.1300 | Surrogate MCDONNELL-DOUGLAS DC8-62/72 |
| MCDONNELL-DOUGLAS DC9-10/15 | 33.00 | 35.00 | 27.00 | 8.00 | 1.1500 | 0.9750 | 0.3500 | 0.1300 | Surrogate TUPOLEV TU134 |
| McDONNELL-DOUGLAS DC9-20 | 33.00 | 35.00 | 27.00 | 8.00 | 1.1500 | 0.9750 | 0.3500 | 0.1300 | Surrogate TUPOLEV TU134 |
| MCDONNELL-DOUGLAS DC9-30 | 33.00 | 35.00 | 27.00 | 8.00 | 1.1500 | 0.9750 | 0.3500 | 0.1300 | Surrogate TUPOLEV TU134 |
| MCDONNELL-DOUGLAS DC9-40 | 33.00 | 35.00 | 27.00 | 8.00 | 1.1500 | 0.9750 | 0.3500 | 0.1300 | Surrogate TUPOLEV TU134 |
| MCDONNELL-DOUGLAS DC9-50 | 33.00 | 35.00 | 27.00 | 8.00 | 1.1500 | 0.9750 | 0.3500 | 0.1300 | Surrogate TUPOLEV TU134 |
| MCDONNELL-DOUGLAS MD11 | 9.60 | 9.60 | 9.60 | 9.60 | 2.6232 | 2.0751 | 0.6860 | 0.2180 |  |
| MCDONNELL-DOUGLAS MD80-MD83 | 6.38 | 6.38 | 6.38 | 6.38 | 1.3145 | 1.0654 | 0.3735 | 0.1361 |  |
| MCDONNELL-DOUGLAS MD87 | 6.10 | 6.10 | 6.10 | 6.10 | 1.2820 | 1.0450 | 0.3630 | 0.1370 |  |
| MCDONNELL-DOUGLAS MD88 | 6.11 | 6.11 | 6.11 | 6.11 | 1.2828 | 1.0455 | 0.3632 | 0.1370 |  |
| MITSUBISHI MU2 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate BEECH KINGAIR 90 |
| MOONEY M20J | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 210 CENTURION |
| NAMC YS11 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate FOKKER 50 |
| NORD 2501 NORTALAS | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate ATR72 |
| PARTENAVIA P68B/C | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR |
| PIAGGIO P.180 AVANTI | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate EMBRAER EMB110 BANDEIRANTE |
| PIAGGIO P166-DL3 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 310 |
| PILATUS BN-2A ISLANDER | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR |
| PILATUS BN-2A TRISLANDER MK3 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR |
| PIPER PA18 SUPER CUB | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 210 CENTURION |
| PIPER PA23 AZTEC/APACHE | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 |  |
| PIPER PA24 COMMANCHE | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 210 CENTURION |
| PIPER PA28 CHEROKEE SRS/PA32 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 210 CENTURION |
| PIPER PA30/PA39 TWIN COMANCHE | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate PIPER PA31/P NAVAJO CHIEFTAIN |
| PIPER PA31/P NAVAJO CHIEFTAIN | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 |  |
| PIPER PA31T CHEYENNE I/II | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate PIPER PA31/P NAVAJO CHIEFTAIN |
| PIPER PA34 SENECA II | 0.00 | 0.00 | 0.00 | 0.00 | 0.0347 | 0.0296 | 0.0181 | 0.0070 |  |
| PIPER PA42 CHEYENNE III/IV | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate EMBRAER EMB110 BANDEIRANTE |
| PIPER PA44 SEMINOLE | 0.00 | 0.00 | 0.00 | 0.00 | 0.0347 | 0.0296 | 0.0181 | 0.0070 | Surrogate PIPER PA34 SENECA II |
| REIMS-CESSNA F406/CARAVAN II | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR |
| RILEY TURBO-EXEC 400 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR |
| RO TURBO COMMANDER 680T-690 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0577 | 0.0515 | 0.0315 | 0.0141 | Surrogate ROCKWELL TURBO COMMANDER |
| ROBINSON R22 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 210 CENTURION |
| ROCKWELL SABRELINER SERIES | 0.00 | 0.00 | 0.00 | 0.00 | 0.2250 | 0.1860 | 0.0720 | 0.0260 | Surrogate BAE(HS)125 |
| ROCKWELL SHRIKE/COURSER/COMMAN | 0.00 | 0.00 | 0.00 | 0.00 | 0.0130 | 0.0090 | 0.0046 | 0.0010 | Surrogate CESSNA 414A CHANCELLOR |
| ROCKWELL TURBO COMMANDER | 0.00 | 0.00 | 0.00 | 0.00 | 0.0577 | 0.0515 | 0.0315 | 0.0141 |  |
| SAAB 2000 | 1.00 | 0.00 | 0.00 | 0.00 | 0.3770 | 0.3150 | 0.1170 | 0.0490 |  |
| SAAB FAIRCHILD 340 | 4.90 | 4.40 | 4.40 | 4.40 | 0.2790 | 0.2380 | 0.0992 | 0.0509 | Surrogate EMBRAER EMB120 BRASILIA |
| SHORTS 330 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate SHORTS 360 |
| SHORTS 360 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 |  |
| SHORTS BELFAST | 0.00 | 0.00 | 0.00 | 0.00 | 0.2994 | 0.2769 | 0.1436 | 0.0769 | Surrogate LOCKHEED L100 HERCULES |
| SHORTS SC7 SKYLINER | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate SHORTS 360 |
| SHORTS SC7 SKYVAN | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate SHORTS 360 |
| SIKORSKY S61N | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 |  |
| SIKORSKY S76 SPIRIT | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 |  |
| SOCATA TB10 TOBAGO | 0.00 | 0.00 | 0.00 | 0.00 | 0.0327 | 0.0258 | 0.0125 | 0.0032 | Surrogate CESSNA 210 CENTURION |
| SWEARINGEN MERLIN IIA/IIB/IIIB | 0.00 | 0.00 | 0.00 | 0.00 | 0.0577 | 0.0515 | 0.0315 | 0.0141 | Surrogate SWEARINGEN METRO II |
| SWEARINGEN MERLIN IVA | 0.00 | 0.00 | 0.00 | 0.00 | 0.0577 | 0.0515 | 0.0315 | 0.0141 | Surrogate SWEARINGEN METRO II |
| SWEARINGEN METRO II | 0.00 | 0.00 | 0.00 | 0.00 | 0.0577 | 0.0515 | 0.0315 | 0.0141 |  |
| TUPOLEV TU104 | 6.38 | 6.38 | 6.38 | 6.38 | 1.3145 | 1.0654 | 0.3735 | 0.1361 | Surrogate MCDONNELL-DOUGLAS MD80-MD83 |
| TUPOLEV TU124 | 21.90 | 21.90 | 21.90 | 21.90 | 0.8910 | 0.7260 | 0.2780 | 0.1270 | Surrogate GA GULFSTREAM 3 |
| TUPOLEV TU134 | 33.00 | 35.00 | 27.00 | 8.00 | 1.1500 | 0.9750 | 0.3500 | 0.1300 |  |
| TUPOLEV TU154A/B | 0.00 | 0.00 | 0.00 | 0.00 | 1.7500 | 1.3300 | 0.5500 | 0.2200 | Heathrow data |
| TUPOLEV TU154M | 0.00 | 0.00 | 0.00 | 0.00 | 1.4200 | 1.1000 | 0.4200 | 0.2070 |  |
| TUPOLEV TU204 | 7.31 | 7.31 | 7.31 | 7.31 | 1.8567 | 1.5043 | 0.5209 | 0.1803 | Surrogate BOEING 757-200 |
| V953C MERCHANTMAN | 0.00 | 0.00 | 0.00 | 0.00 | 0.2994 | 0.2769 | 0.1436 | 0.0769 | Surrogate LOCKHEED L100 HERCULES |
| VICKERS VISCOUNT 700 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate ATR72 |
| VICKERS VISCOUNT 800 | 8.80 | 8.80 | 8.80 | 8.80 | 0.1780 | 0.1570 | 0.0814 | 0.0517 | Surrogate ATR72 |
| WESTLAND 30 SRS 100 | 0.00 | 0.00 | 0.00 | 0.00 | 0.0643 | 0.0596 | 0.0344 | 0.0185 | Surrogate AEROSPATIALE SA365 DAUPHIN |
| YAKOVLEV YAK-40 | 0.00 | 0.00 | 0.00 | 0.00 | 0.2250 | 0.1860 | 0.0720 | 0.0260 | Surrogate DASSAULT BREGUET FALCON 50 |
| YAKOVLEV YAK-42 | 0.00 | 0.00 | 0.00 | 0.00 | 1.7500 | 1.3300 | 0.5500 | 0.2200 | Surrogate TUPOLEV TU154A/B |

Table A3.3	APU emission factors and fuel flow rates

| APU | Fuel (kg s-1) | HC (g / kg fuel) | CO (g / kg fuel) | NOx (g / kg fuel) |
| --- | --- | --- | --- | --- |
| 131-9 | 0.0321 | 0.37 | 4.88 | 6.64 |
| 30-54 | 0.0087 | 4.31 | 36.30 | 3.57 |
| 331-200ER | 0.0338 | 0.43 | 4.13 | 9.51 |
| 331-350 | 0.0571 | 0.23 | 1.86 | 9.90 |
| 331-500 | 0.0675 | 0.20 | 1.89 | 11.41 |
| 36-100 | 0.0184 | 0.57 | 31.10 | 5.33 |
| 36-150[ ] | 0.0188 | 0.61 | 6.45 | 5.10 |
| 36-150[RR] | 0.0231 | 0.49 | 7.26 | 5.29 |
| 36-300 | 0.0278 | 0.15 | 2.05 | 10.10 |
| 36-4A | 0.0169 | 0.36 | 13.47 | 5.10 |
| 600-4 | 0.0966 | 0.28 | 8.65 | 5.33 |
| 700-4B | 0.0583 | 0.37 | 3.73 | 8.23 |
| 85-129 | 0.0296 | 1.03 | 17.99 | 4.75 |
| 85-98CK | 0.0296 | 1.03 | 17.99 | 4.75 |
| None | 0.0000 | 0.00 | 0.00 | 0.00 |
| PW901A | 0.1087 | 1.50 | 16.78 | 3.15 |
| ST6L-73 | 0.0554 | 0.02 | 0.05 | 8.90 |

Table A3.4	Cruise emission and fuel consumptions

| EMEP/CORINAIR Aircraft Type | CO Slope | CO Intercept | Fuel Slope | Fuel Intercept | HC Slope | HC Intercept | NOx Slope | NOx Intercept |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A310 | 2.50E-03 | 1.31E+00 | 4.91E+00 | -2.23E+02 | 6.15E-04 | 1.60E-01 | 5.90E-02 | 9.39E+00 |
| A320 | 1.86E-03 | 9.99E-01 | 2.64E+00 | 3.26E+02 | 4.58E-04 | 5.60E-02 | 3.17E-02 | 1.25E+01 |
| A330 | 8.07E-03 | 3.81E+00 | 6.46E+00 | 1.25E+02 | 6.09E-03 | 6.52E-01 | 8.00E-02 | 2.98E+01 |
| A340 | 5.19E-03 | 1.67E+01 | 7.21E+00 | -1.22E+03 | 3.18E-03 | 1.75E+01 | 1.20E-01 | -1.96E+01 |
| Antonov 26 | 9.35E-02 | 4.02E-01 | 1.43E+00 | 1.69E+01 | 3.25E-02 | 5.70E+00 | 1.36E-03 | 3.15E-01 |
| Airship | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 |
| ATR 42-320 | 7.70E-03 | 3.98E-01 | 8.41E-01 | 2.42E+01 | 0.00E+00 | 0.00E+00 | 6.68E-03 | 3.65E-01 |
| ATR 72-200 | 4.89E-03 | 3.02E-01 | 9.25E-01 | 2.00E+00 | 0.00E+00 | 0.00E+00 | 8.68E-03 | 4.33E-01 |
| B727 | 6.69E-03 | 2.38E+00 | 4.42E+00 | 1.14E+02 | 1.62E-03 | 8.72E-01 | 3.93E-02 | 1.90E+00 |
| B737 100 | 2.89E-03 | 2.65E+00 | 2.75E+00 | 2.41E+02 | 1.47E-03 | 8.26E-01 | 2.05E-02 | 6.33E+00 |
| B737 400 | 3.36E-03 | 2.27E+00 | 3.03E+00 | -1.20E+01 | 1.50E-04 | 1.66E-01 | 2.76E-02 | 2.42E+00 |
| B747 100-300 | 8.06E-03 | 1.41E+01 | 1.22E+01 | -1.93E+03 | 2.33E-03 | 6.07E+00 | 2.20E-01 | -3.56E+01 |
| B747 400 | 7.97E-03 | 1.71E+01 | 1.12E+01 | -2.54E+03 | 1.74E-03 | 6.55E+00 | 1.62E-01 | -3.97E+01 |
| B757 | 4.71E-03 | 1.94E+00 | 3.75E+00 | 3.04E+02 | 3.54E-03 | 5.10E-01 | 4.59E-02 | 2.47E+01 |
| B767 300 ER | 5.38E-03 | 3.32E+00 | 5.36E+00 | -5.72E+02 | 2.76E-03 | -1.07E+00 | 6.95E-02 | 7.77E-01 |
| B777 | 6.01E-03 | 1.83E+01 | 7.74E+00 | -7.19E+02 | 4.17E-03 | 1.60E+01 | 1.20E-01 | 3.19E+00 |
| BAC1-11 | 1.82E-03 | 1.57E+00 | 2.49E+00 | 1.20E+02 | 3.27E-04 | 1.59E-01 | 2.62E-02 | 2.57E+00 |
| BAe Jetstream 31 | 3.02E-03 | 2.98E-01 | 5.01E-01 | 1.30E+01 | 1.99E-04 | 3.13E-02 | 5.39E-03 | 3.21E-02 |
| BAe Jetstream 41 | 3.37E-03 | 6.36E-01 | 7.35E-01 | -4.25E+00 | 1.51E-04 | 1.04E-01 | 7.42E-03 | -1.29E-01 |
| BAe146 | 2.32E-03 | 1.16E+00 | 2.76E+00 | 9.39E+00 | 8.40E-04 | 1.82E-01 | 1.85E-02 | 3.60E+00 |
| Beech 1900C Airliner | 2.24E-02 | -4.11E-01 | 4.80E-01 | 1.28E+01 | 4.61E-03 | 1.72E-01 | 1.83E-03 | 1.52E-01 |
| Beech Super King Air 200B | 1.20E-02 | -1.02E+00 | 3.92E-01 | 7.33E+00 | 2.19E-03 | -2.09E-01 | 1.41E-03 | 1.47E-01 |
| Beech Super King Air 350 | 1.90E-02 | -1.76E-01 | 4.43E-01 | 5.74E+00 | 2.13E-03 | 5.61E-02 | 1.59E-03 | 8.04E-02 |
| Cessna 208 Caravan | 1.18E-03 | -9.34E-02 | 3.11E-01 | -1.06E+01 | 2.09E-05 | 1.16E-02 | 1.80E-03 | -5.56E-02 |
| Dash 8 Q400 | 7.55E-03 | 7.04E-02 | 1.64E+00 | 3.90E+01 | 0.00E+00 | 0.00E+00 | 1.99E-02 | 2.46E+00 |
| DC10-30 | 6.51E-03 | 1.42E+01 | 9.22E+00 | -1.39E+03 | 4.14E-03 | 1.74E+01 | 1.68E-01 | -2.62E+01 |
| DC9 | 3.21E-03 | 2.22E+00 | 3.04E+00 | 1.05E+02 | 1.57E-03 | 3.12E-01 | 2.58E-02 | 3.48E+00 |
| De Havilland Dash 7 | 7.87E-03 | 1.10E+00 | 1.05E+00 | -1.35E-01 | 1.72E-04 | 3.28E-01 | 5.22E-03 | 1.35E-01 |
| De Havilland DHC-3 Turbo-Otter | 4.18E-03 | -1.69E-01 | 3.13E-01 | -2.74E+00 | 2.86E-04 | -1.21E-02 | 1.48E-03 | 3.20E-02 |
| Dornier 328-110 | 5.98E-03 | 6.23E-02 | 7.41E-01 | 1.19E+01 | 0.00E+00 | 0.00E+00 | 6.05E-03 | 3.43E-01 |
| Embraer 110P2A | 1.98E-03 | -4.94E-02 | 5.13E-01 | -1.30E+01 | 1.11E-04 | -2.83E-03 | 2.95E-03 | -5.79E-02 |
| F100 | 2.16E-03 | 1.50E+00 | 2.47E+00 | 1.59E+02 | 7.64E-04 | 2.60E-01 | 1.49E-02 | 7.06E+00 |
| F28 | 9.97E-04 | 2.42E+00 | 2.34E+00 | 1.35E+02 | 2.19E-03 | 1.76E+00 | 2.15E-02 | 3.71E+00 |
| Fokker 27 Friendship | 5.37E-02 | 1.95E+00 | 9.88E-01 | -1.28E+01 | 6.21E-03 | 2.14E+00 | 7.96E-04 | 2.01E-01 |
| Fokker 50 Srs 100 | 4.91E-03 | 7.18E-01 | 1.10E+00 | 4.86E+01 | 0.00E+00 | 0.00E+00 | 1.23E-02 | 1.27E+00 |
| Helicopter | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 |
| Lockheed C-130H Hercules | 5.75E-03 | 1.38E+00 | 3.71E+00 | -3.91E+01 | 5.18E-04 | 1.03E+00 | 3.78E-02 | 1.21E+00 |
| Lockheed P-3B Orion | 7.06E-03 | 1.76E+00 | 2.81E+00 | 6.32E+01 | 1.32E-03 | 1.24E+00 | 2.45E-02 | 2.55E+00 |
| MD 82 | 4.03E-03 | 1.34E+00 | 3.30E+00 | 4.25E+02 | 1.41E-03 | 4.18E-01 | 3.55E-02 | 1.37E+01 |
| Reims F406 Caravan II | 5.21E-03 | -4.98E-01 | 3.14E-01 | 6.01E-01 | 4.52E-04 | -4.96E-02 | 1.44E-03 | 5.85E-02 |
| Saab 2000 | 5.41E-03 | 1.14E-01 | 1.46E+00 | -6.84E+00 | 2.35E-05 | 2.81E-02 | 1.55E-02 | 1.40E-01 |
| Saab 340B | 2.76E-03 | 3.15E-01 | 7.34E-01 | 1.33E+01 | 1.63E-03 | 9.31E-02 | 7.65E-03 | 6.28E-02 |
| Shorts 330 | 4.70E-03 | 1.75E-01 | 6.92E-01 | 1.74E+01 | 6.29E-20 | 1.32E-01 | 3.50E-03 | 3.27E-01 |
| Shorts 360-300 | 1.81E-02 | 9.34E-01 | 7.77E-01 | 2.18E+01 | 2.43E-03 | 4.07E-01 | 3.68E-03 | 3.16E-01 |
| Shorts SC.7 Srs3M-200 | 4.81E-03 | -3.12E-01 | 7.38E-01 | -5.14E+00 | 7.02E-04 | -7.04E-02 | 7.37E-03 | -7.56E-02 |
| Swearingen Metro III | 3.87E-03 | -1.82E-01 | 4.27E-01 | 2.58E+00 | 2.72E-04 | -6.25E-03 | 4.23E-03 | 2.51E-02 |

Appendix 4 Consultation with the DfT

CONTENTS

Purpose of the consultation with the Department for Transport

Location

Aims of the meeting

People present at the meeting

Summary of the recommendations and actions taken from the consultation

Purpose of the consultation with the Department for Transport

A consultation on the initial results of this work took place with the Department for Transport in January 2004.  This consultation was part of the work package to improve the method used to estimate emissions from aviation, and was designed to allow comments from the DfT to guide the final stages of the work.

Location

The meeting took place at the Department for Transport, London, on 19th January, 2004.

Aims of the meeting

The meeting concentrated on examining technical aspects of the work to improve estimates of emissions from aviation within the UK GHG inventory.  A background briefing note on the method was provided by Simon Eggleston, and preliminary revised estimates of greenhouse gas and other pollutants from aviation.

People present at the meeting

- Netcen

- John Watterson	Manager, UK Greenhouse Gas Inventory

- Simon Eggleston	Project Manager, GHG Aviation Tier 3 project

- DfT

- Roger Gardiner	Noise & Emissions technical branch

- DTI

- Peter Newton	Aviation and environmental expert

Charles Walker (Netcen) completed the technical work started by Simon Eggleston).

Summary of the recommendations and actions taken from the consultation

| Person | Recommended action | Action taken |
| --- | --- | --- |
| Simon Eggleston (now Charles Walker) | - To provide information about the fleet disaggregation used in the model (into the 12 types of aircraft mentioned in the new model) | - Information provided in the Appendices of this report |
|  | - To check with Brian Underwood if reduced take-off thrust data are readily available for airports other than Heathrow and Gatwick | - In progress |
|  | - To provide more information about the possible reasons for differences between new GHG aviation emissions model and the HEI | - Information provided |
| Roger Gardiner | - To contact Chris Vandenburg at Eurocontrol and obtain modifier to the great circle distances used (time series from 1990 to 2002, inclusive, if possible) | - Done by John Watterson at Workshop on Emissions of GHGs from Aviation and Navigation, Copenhagen, May 2004. JW requested if data were available from Andrew Watt and Ted Eliff of Eurocontrol – but no such data are available from at the moment |
|  | - To contact Kevin Morris at BAA to request operational practices regarding take-off thrust for a selection of aircraft operators | - In progress. The work already accounts for operator specific procedures at Heathrow and Gatwick |
|  | - To check with Defra what profile this aviation work has with respect to carbon trading | - Done |
|  | - Discuss what information the technical report should contain with Defra (TM). Martin Meadows (Defra, AEQ should be consulted also) | - Martin Meadows has been informed about the changes to non-GHGs emissions as a result of this work |
| John Watterson | - To provide definitions of the IPCC reporting requirements in the technical report, and to state which emissions from which modes are included in the various IPCC sectors | - Included in this report |
|  | - Find an independent auditor for the final report | - Done. Prof. David Lee (Manchester Metropolitan University) will be the independent auditor |

Particles that pass through the selective size inlet of a specified measuring instrument with 50% efficiency at 10m aerodynamic diameter, where the ‘aerodynamic diameter’ of a particle is the diameter of a spherical particle of unit relative density that would have the same gravitational settling velocity as the particle of interest.

The term general aviation has a specific definition from ICAO.  The following definition is cited in IPCC (2000).  “ICAO’s ‘Manual on the ICAO Statistics Programme’ defines ‘general aviation’ as all civil operations other than scheduled air services and non-scheduled air transport operations for remuneration or hire.  For ICAO statistical purposes, the general aviation activities are classified into instructional flying, business and pleasure flying, aerial work and other flying”.

Note the IPCC refer to altitudes of 914 m (rather than 1000 m) and 3000 feet (IPCC, 2000)

In 2001 the UN/ECE TFEIP developed the NFR (Nomenclature For Reporting) source sector classification system for the Reporting Guidelines.  In the development of NFR a correlation was established between the SNAP, NFR and CRF/IPCC reporting source categories.  Following experiences from the 2001 reporting round the system has been revised in 2002 according to the final draft CLRTAP/EMEP 2002 Reporting Guidelines (described in eb.air.ge.1.2002.7.pdf).  This correlation is included in the contents/index for the current version of the Guidebook (AIND) and codes are also included in the Guidebook’s technical chapters. National reporters of emission inventories have used this format for the first time to report emissions in the 2002 reporting round (due 31 January 2003).  See http://reports.eea.eu.int/EMEPCORINAIR3/en/BNPA%20v3.1.pdf

Air Transport Movements

There is a difference between the emissions reported in the UK NIR and those calculated by the IEA.  This results from a mismatch on data sources between Netcen (who compile the UK GHGI) and the IEA on the domestic / international split of aviation fuel use.  The IEA use data as provided by the DTI in the Monthly Oil Statistics (MOS) data return and Annual Oil Questionnaire (AOQ) for their calculations of UK emissions.  MOS and AOQ returns are also provided to Eurostat.  DTI base the MOS (& AOQ) returns on the same data as used in their routine publications such as Energy Trends and the Digest of UK Energy Statistics.  The MOS / AOQ returns also include an unpublished split of domestic / international aviation fuel use based on collated company data.  DTI is concerned about the quality of this split since it is unclear how oil companies can record eventual usage after sale.  As result DTI is keen to switch to using the ratio as calculated by Netcen in the MOS & AOQ return as it is more accurate.

Heathrow, 2002; Gatwick, 2002/3; and Stansted, 1999

The term for loading of fuel used for subsequent flight segments (taking more fuel onboard than is required by the fuel flight plan).  The main reasons for tankering of fuel are commercial ones – for example, in cases where the cost of fuel consumed in carrying additional fuel is more than offset by the difference in the price of fuel at the departure point and a destination where the fuel could be loaded (IPCC, 1999).