ISBN 0-9547136-6-4

| Greenhouse Gas Inventories for England, Scotland, Wales and Northern Ireland: 1990 - 2003 |  |
| --- | --- |
| Main authors | SL Baggott, L Brown, R Milne, TP Murrells, N Passant, DG Thistlethwaite |
| With contributions from | MK Downes, S Grice, Y Li, A Wagner, JD Watterson, A Thomson, D Mobbs |
| September 2005 |  |
| This work forms part of the Global Atmosphere Research Programme of the Department for Environment, Food and Rural Affairs |  |

| Greenhouse Gas Inventories for England, Scotland, Wales and Northern Ireland: 1990 - 2003 |  |
| --- | --- |
| Main authors | SL Baggotta, L Brownb, R Milnec, TP Murrellsa, N Passanta, DG Thistlethwaitea |
| With contributions from | MK Downesa, S Gricea, Y Lia, A Wagnera, JD Wattersona, A Thomsonc, D Mobbsc |
| September 2005 |  |
| a National Environmental Technology Centre (NETCEN), AEA Technology plc, Culham Science Centre, Abingdon, Oxon., OX14 3ED, UK. b Institute of Grassland and Environmental Research (IGER), North Wyke Research Station, Okehampton, Devon, EX20 2SB, UK. c Centre for Ecology and Hydrology (CEH), Bush Estate, Pennicuik, Midlothian, EH26 OQB, UK. |  |

| Title | Greenhouse Gas Inventories, for England, Scotland, Wales and Northern Ireland: 1990 - 2003 |
| --- | --- |
| Customer | Department for Environment, Food and Rural Affairs The Scottish Executive, The National Assembly for Wales, The Northern Ireland Department of Environment. |
| Customer reference | RMP/2106 |
| Confidentiality, copyright and reproduction | Copyright AEA Technology plc 2004. All rights reserved. Enquiries about copyright and reproduction should be addressed to the Commercial Manager, AEA Technology plc. |
| File reference | 48209301/2003 |
| Report number | AEAT/ENV/R/2037 |
| NAEI contacts database reference | CD3662/GT |
| Report status | Issue 1 |
| ISBN number | ISBN 0-9547136-6-4 |
|  | National Environmental Technology Centre AEA Technology plc, B551, Harwell, Didcot, Oxfordshire, OX11 OQJ Telephone 0870 190 6594 Facsimile 0870 190 6607 |
|  | AEA Technology is the trading name of AEA Technology plc AEA Technology is certified to BS EN ISO 9001: 2000, The TickIT Guide Issue 5 and ISO 14001: 1996. |

|  | Name | Signature |  | Date |
| --- | --- | --- | --- | --- |
| Authors | SL Baggott, L Brown, R Milne, TP Murrells, N Passant, DG Thistlethwaite |  |  |  |
| Reviewed by | J Watterson DG Thistlethwaite |  |  |  |
| Approved by | J Goodwin |  |  |  |

- Preface

- This is the Greenhouse Gas Inventory Report for England, Scotland, Wales and Northern Ireland submitted in the year 2005 to the UK Department for Environment, Food & Rural Affairs, the Scottish Executive, the National Assembly for Wales and the Department of Environment for Northern Ireland.  It contains greenhouse gas emission inventories for the constituent countries of the UK for the period 1990 to 2003, and the descriptions of the methods used to produce the estimates.  Tabular data in the Common Reporting Format (CRF) covering greenhouse gas emissions from England, Scotland, Wales, Northern Ireland and the UK for the period 1990 to 2003 are presented in Appendix 2 of this report.

These greenhouse gas inventories are based on the same datasets used by the UK Greenhouse Gas Inventory (GHGI) and the National Atmospheric Emissions Inventory (NAEI) for reporting atmospheric emissions under other international agreements and are consistent with the GHGI and the NAEI where they overlap.

- This inventory is compiled on behalf of the UK Department for Environment, Food & Rural Affairs (Global Atmosphere Division) and the Devolved Administrations, by the National Environmental Technology Centre (Netcen).  We acknowledge the positive support and advice from DEFRA and the Devolved Administrations throughout the work, and we are grateful for the help of all those who have contributed to this report.

- Units and Conversions

Emissions of greenhouse gases presented in this report are given in Gigagrammes (Gg), Million tonnes (Mt) and Teragrammes (Tg).  GWP weighted emissions are also provided.  To convert between the units of emissions, use the conversion factors given below.

Prefixes and multiplication factors

| Multiplication factor | Abbreviation | Prefix | Symbol |
| --- | --- | --- | --- |
| 1,000,000,000,000,000 | 1015 | peta | P |
| 1,000,000,000,000 | 1012 | tera | T |
| 1,000,000,000 | 109 | Giga | G |
| 1,000,000 | 106 | mega | M |
| 1,000 | 103 | kilo | k |
| 100 | 102 | hecto | h |
| 10 | 101 | deca | da |
| 0.1 | 10-1 | deci | d |
| 0.01 | 10-2 | centi | c |
| 0.001 | 10-3 | milli | m |
| 0.000,001 | 10-6 | micro |  |

1 kilotonne (kt)		=	103 tonnes	=	1,000 tonnes

1 Million tonne (Mt)	=	106 tonnes	=	1,000,000 tonnes

1 Gigagramme (Gg)	=	1 kt

1 Teragramme (Tg)	=	1 Mt

Conversion of carbon emitted to carbon dioxide emitted

To convert emissions expressed in weight of carbon, to emissions in weight of carbon dioxide, multiply by 44/12.

Conversion of Gg of greenhouse gas emitted into Gg CO2 equivalent

Gg (of GHG) * GWP = Gg CO2 equivalent

- The GWP is the Global Warming Potential of the greenhouse gas.  The GWPs of the greenhouse gases are given in Table 1.1 of Chapter 1.

- Abbreviations for Greenhouse Gases and Chemical Compounds

| Type of greenhouse gas | Formula or abbreviation | Name |
| --- | --- | --- |
| Direct | CH4 | Methane |
| Direct | CO2 | Carbon dioxide |
| Direct | N2O | Nitrous oxide |
| Direct | HFCs | Hydrofluorocarbons |
| Direct | PFCs | Perfluorocarbons |
| Direct | SF6 | Sulphur hexafluoride |
| Indirect | CO | Carbon monoxide |
| Indirect | NMVOC | Non-methane volatile organic compound |
| Indirect | NOX | Nitrogen oxides (reported as nitrogen dioxide) |
| Indirect | SO2 | Sulphur dioxide |

HFCs, PFCs and SF6 are collectively known as the ‘F-gases’.

Direct greenhouse gases have a direct effect on radiative forcing within the atmosphere. Reactive gases such as carbon monoxide (CO), volatile organic compounds (VOC), nitrogen oxides (NO & NO2) and sulphur dioxide (SO2), are termed indirect greenhouse gases. These pollutants are not significant direct greenhouse gases, but through atmospheric chemistry they impact upon the abundance of the direct greenhouse gases.

- Executive Summary

This report presents estimates of greenhouse gas emission inventories for the constituent countries of the UK.  Separate greenhouse gas emission inventories have been estimated for England, Scotland, Wales and Northern Ireland for the years 1990, 1995, 1998 to 2003.  The greenhouse gases reported are:

- Carbon dioxide	(CO2)

- Methane	(CH4)

- Nitrous oxide	(N2O)

- Hydrofluorocarbons	(HFCs)

- Perfluorocarbons	(PFCs)

- Sulphur hexafluoride	(SF6)

The estimates are consistent with the United Nations Framework Convention on Climate Change (FCCC) reporting guidelines and the 2003 UK Greenhouse Gas Inventory (Baggott et al., 2005).  Some emissions, mainly mobile and offshore sources, cannot be allocated to any country, so an unallocated category is used to report these.

The study shows that the UK distribution of regional greenhouse gas emissions in 2003, expressed in terms of global warming potentials (GWP), is:

- England	75.8%

- Scotland	9.7%

- Wales	7.6%

- Northern Ireland	3.3%

- Unallocated	3.7%

Table ES1 (below) presents emissions of the six greenhouse gases in more detail.

UK trends in emissions of greenhouse gases over recent years are as follows:

- Carbon dioxide: Overall UK emissions have fallen by 5.6% between 1990 and 2003, mainly driven by the installation of combined cycle gas turbines (CCGT) in the power generation sector in England and reductions in CO2 emissions from industry in England, Scotland and Wales.

- Methane: Overall UK emissions have fallen by 47.6% between 1990 and 2003, due primarily to significant reductions in methane emissions from waste disposal and coal mining sources across all constituent countries.

- Nitrous oxide: Overall UK emissions have fallen by 40.5% between 1990 and 2003, driven predominantly by a large reduction in emissions following the installation of abatement measures at an adipic acid plant in England.  This overall downward trend is offset to a small degree by a rise across all constituent countries in nitrous oxide emissions from the transport sector over the period due to increased use of three-way catalytic converters.

- HFCs: Overall UK emissions have fallen by 30.9% between 1995 and 2003, mainly due to a big reduction in emissions following the installation of improved abatement equipment at an HCFC plant in England.  However, there is a rising trend in emissions across all countries from sources such as losses from refrigeration and air conditioning equipment and emissions from industrial aerosols and metered dose inhalers.

- PFCs: Overall UK emissions have fallen by 17.6% between 1995 and 2003, mainly due to improved control measures in aluminium production in England and Wales and a reduction in aluminium production capacity in Scotland.

- SF6: Overall UK emissions have risen by 20.8% between 1995 and 2003, due to increased magnesium production in England and Wales, greater use of SF6 in training shoes and electrical switch gear across all constituent countries, and greater use of SF6 in the electronics industry mainly in Scotland and Wales.

In the compilation of GHG inventories for the constituent countries of the UK, where possible the same methodology has been used to calculate emission estimates as for the UK Inventory.  However, for many emission sources the data available for constituent country emissions are less detailed than for the UK as a whole, and for some sources country-level data are not available at all.

In particular, complete sets of fuel consumption data are not available for England, Wales and Scotland.  In order to make emission estimates for fuel consumption, therefore, the available data has been supplemented with surrogate statistics including: plant capacities, boiler capacities, employment statistics and production of industrial products.  These activity statistics have been used to provide an estimated disaggregation of total UK emissions across the countries.

For other key emission sources (such as industrial processes, agriculture, land-use change and forestry, waste disposal) there are more reliable and complete country-level datasets available, although some of these are less detailed than data used for the UK Inventory.

As a result of these data availability issues, it should be noted that the emission estimates for the England, Scotland, Wales and Northern Ireland inventories are subject to greater uncertainty than the equivalent UK estimates. Chapter 7 of this report outlines significant changes to inventory methodology that have impacted upon the DA-level estimates for GHG emissions, including changes to LUCF emission and removal estimates.

Revisions and Updates to the Greenhouse Gas Inventories

Each year, the greenhouse gas inventories for England, Scotland, Wales and Northern Ireland are extended and updated.

The time series of the inventories are extended by including a new inventory year – i.e. the previous inventory (published in October 2004) covered the years up to and including 2002, whilst this report gives emission estimates for the years up to and including 2003.

The inventories are also updated to take account of any amendments to core activity or emission factor data, and these amendments may result in revisions to emission estimates for a given year.  Core energy statistics (mainly provided by the DTI via their annual publication “The Digest of UK Energy Statistics”) are revised annually and hence the data provided (e.g. for “coal used in energy generation in 2001”) may be different in the latest edition of the Digest, compared to that used in the compilation of the previous inventory report.  In addition, since the previous inventory report, a more representative emission factor for one or more greenhouse gases may have been derived for a given process.  Use of a new emission factor in emission estimation calculations may lead to revisions of historic data.  The nature of emission inventories is such that ongoing improvements to data collection or estimation techniques will inevitably lead to some revisions of historic data.

Therefore, it is not appropriate to use data from previous reports and compare them with the figures in this report, without taking account of any changes to either the emission estimation methodology or the source data.  There is normally a comment in the report to indicate where such changes have occurred.

Notes on Table ES1 (Below)

#1995 is used as the Base Year for emissions of HFCs, PFCs and SF6 in the UK’s Climate Change Programme, in accordance with Article 3.8 of the Kyoto Protocol.

All of the CO2 data are based on the total emissions of CO2, excluding removals of CO2 in Land Use, Land Use Change and Forestry sectors.

Table ES1	Summary of Greenhouse Gas Emission Trends for UK and Constituent Countries (as GWP-Equivalent Mass of Carbon)

Contacts

This work forms part of the Global Atmosphere Research Programme of the Department for Environment, Food and Rural Affairs (Contract RMP/2106).  The land use, land use change and forestry estimates were provided by the Centre for Ecology and Hydrology (Edinburgh) (Contract CPEG 1).  The Institute of Grassland and Environmental Research (IGER) provide the estimates of agricultural emissions.

Science policy enquiries should be directed to Dr Jim Penman, Global Atmosphere Division, Department for Environment, Food and Rural Affairs, 3rd Floor, Ashdown House, 123 Victoria Street, London, SW1E 6DE, UK, Tel: +44 (0) 20 7944 5232, Fax: +44 (0) 20 7944 5219, E-mail: jim.penman@defra.gsi.gov.uk

Technical enquiries should be directed to Dr JD Watterson, Netcen, AEA Technology plc, E5 Culham Science Centre, Abingdon, Oxfordshire, OX14 3ED, UK, Tel: +44 (0) 870 190 6594, Fax: +44 (0) 870 190 6607, E-mail: john.d.watterson@aeat.co.uk

Technical enquiries on land use, land use change and forestry should be addressed to Dr R Milne, Centre for Ecology and Hydrology (Edinburgh), Bush Estate, Penicuik, EH26 OQB, UK, Tel: +44 (0) 131 445 8575, Fax +44 (0) 131 445 3943, E-mail: rmilne@ceh.ac.uk

Technical enquiries on agriculture should be addressed to Dr. L Brown at the Institute of Grassland and Environmental Research (IGER), North Wyke, Devon, EX20 2SB, UK, Tel: +44 (0) 1837 883509, Fax: + 44 (0) 1837 82139, E-mail: lorna.brown@bbsrc.ac.uk

A copy of this report and related data may be found on the website maintained by Netcen for Defra: http://www.naei.org.uk

Further copies of this report are available from:

- Defra Publications

Admail 6000

London

SW1A 2XX

Tel: 08459 556000

Fax: 020 8957 5012

- Email: defra@iforcegroup.com

Contents

1.	Introduction	1

1.1	Background to Inventory Development For the Devolved Administrations	1

1.2	Report Structure	2

1.3	Global Warming Potential	2

1.4	Revisions and updates to the Greenhouse Gas inventories	3

2.	Emissions in England	4

2.1	carbon dioxide	4

2.2	Methane	6

2.3	Nitrous Oxide	8

2.4	Hydrofluorocarbons	9

2.5	Perfluorocarbons	10

2.6	Sulphur Hexafluoride	10

3.	Emissions in Scotland	11

3.1	Carbon Dioxide	11

3.2	Methane	13

3.3	Nitrous Oxide	14

3.4	Hydrofluorocarbons	16

3.5	Perfluorocarbons	16

3.6	Sulphur Hexafluoride	16

4.	Emissions in Wales	17

4.1	Carbon Dioxide	17

4.2	Methane	19

4.3	Nitrous Oxide	21

4.4	hydrofluorocarbons	22

4.5	Perfluorocarbons	22

4.6	Sulphur Hexafluoride	22

5.	Emissions in Northern Ireland	23

5.1	Carbon Dioxide	23

5.2	Methane	25

5.3	Nitrous Oxide	26

5.4	Hydrofluorocarbons	27

5.5	Perfluorocarbons	27

5.6	Sulphur Hexafluoride	27

6.	Unallocated Emissions	28

7.	Availability of Data & Changes to Inventory Methodologies	30

7.1	Availability of Data by Sector	30

7.1.1	Fuel Consumption	30

7.2	Significant Changes to Inventory Methodology	32

7.2.1	Review of UK Carbon Emission Factors	32

7.2.2	UK Method Improvement: Land Use, Land Use Change and Forestry (LULUCF)	33

7.2.3	UK Method Improvement: Aviation	33

7.2.4	UK Method improvement: Iron and Steel and Fuel Transformation	35

7.2.5	UK Method Improvement: Oil and Gas Industry	36

7.2.6	UK Method Improvement: Waste Lubricant Combustion	37

7.2.7	New UK Source: CO2 Emissions from Flue Gas Desulphurisation in the Power Generation Sector	37

7.2.8	UK Data Revisions: Methane & Nitrous Oxide Emissions	37

7.2.9	Revisions to Regional Disaggregation Methodologies	38

8.	Uncertainty in the Inventories	39

9.	Summary Tables	40

10.	References	50

11.	Acknowledgements	56

Tables

Table ES1	Summary of Greenhouse Gas Emission Trends for UK and Constituent Countries (expressed as GWP-weighted equivalent mass of carbon)

Table 1.1	GWP of Greenhouse Gases on a 100-Year Horizon (t CO2 equiv/t gas)

Table 2.1	Key Categories: England CO2 Emissions in 2003

Table 2.2	Key Categories: England CH4 Emissions in 2003

Table 2.3	Key Categories: England N2O Emissions in 2003

Table 3.1	Key Categories: Scotland CO2 Emissions in 2003

Table 3.2	Key Categories: Scotland CH4 Emissions in 2003

Table 3.3	Key Categories: Scotland N2O Emissions in 2003

Table 4.1	Key Categories: Wales CO2 Emissions in 2003

Table 4.2	Key Categories: Wales CH4 Emissions in 2003

Table 4.3	Key Categories: Wales N2O Emissions in 2003

Table 5.1	Key Categories: Northern Ireland CO2 Emissions in 2003

Table 5.2	Key Categories: Northern Ireland CH4 Emissions in 2003

Table 5.3	Key Categories: Northern Ireland N2O Emissions in 2003

Table 9.1	Estimated Uncertainties in the Regional Inventories in 2003

Table 10.1	Summary of Greenhouse Gas Emissions as GWP-weighted Equivalent Mass of Carbon (Mt C)

Table 10.2	Summary of Greenhouse Gas Emissions as GWP-weighted Equivalent Mass of CO2 (Mt)

Table 10.3	Summary of Greenhouse Gas Emissions (Mass of Gas Emitted)

## Figures

Figure 7.2.2		Effects of UK Method Change to LULUCF Sector

Figure 8.2	Road Transport CO2 Emission Trends 1990-2003: Current Methodology

Figure 8.3	Road Transport CO2 Emission Trends 1990-2003: Alternative Methodology

Figure 8.4	Road Transport CO2 Emission Trends 1990-2003: UK Comparison between methods

Figure 1		Emissions of Carbon Dioxide

Figure 2		Emissions of Methane

Figure 3		Emissions of Nitrous Oxide

Figure 4		Emissions of HFCs

Figure 5		Emissions of PFCs

Figure 6		Emissions of SF6

## Appendices

Appendix 1	Estimation Methodology

Appendix 2	Greenhouse Gas Inventories for England, Scotland, Wales and Northern Ireland, 1990-2003, presented in Common Reporting Format

APPENDIX 3	Summary Tables of Greenhouse Gas Emission Trends for England, Scotland, Wales and Northern Ireland, 1990-2003

## Document Revision History

| Version | Comment |
| --- | --- |
| Issue 1 | First issue of finalised report, following drafting by Netcen and feedback from Defra, the Devolved Administrations, and co-authors from CEH and IGER. |

# Introduction

## Background to Inventory Development For the Devolved Administrations

The United Nations Framework Convention on Climate Change (FCCC) was ratified by the United Kingdom in December 1993 and came into force in March 1994.  Parties to the Convention are committed to develop, publish and regularly update national emission inventories of greenhouse gases (GHG).

Following devolution, a national UK inventory continues to be needed to ensure the UK fulfils its reporting requirements under the FCCC and to monitor the legally binding commitments under the Kyoto Protocol to reduce greenhouse gas emissions.  However, some of the measures to deliver GHG emission reductions are devolved and information on the emissions from the four individual countries is needed to support action in each country.

Therefore, DEFRA agreed with the Scottish Executive (SE), the National Assembly for Wales (NAW) and in Northern Ireland, the Department of the Environment, to carry out a joint research project to provide first estimates of GHG emissions inventories for England, Scotland, Wales and Northern Ireland.  The results of this study were published in Greenhouse Gas Inventories for England, Scotland, Wales and Northern Ireland: 1990 and 1995: A Scoping Study, AG Salway et al (1999).  Subsequently these studies have been updated for 1998 to 2002 (Baggott et al 2004).

This report updates and revises the earlier studies and presents separate GHG Inventories for England, Scotland, Wales and Northern Ireland for the years 1990, 1995, and 1998 to 2003.  Emissions of the six direct greenhouse gases are reported, namely:

- Carbon dioxide	(CO2)

- Methane	(CH4)

- Nitrous oxide	(N2O)

- Hydrofluorocarbons	(HFCs)

- Perfluorocarbons	(PFCs)

- Sulphur hexafluoride	(SF6)

These inventories are reported using Intergovernmental Panel on Climate Change (IPCC) Sectoral Tables which are a subset of the IPCC Common Reporting Format (CRF) and are consistent with the most recent UK Greenhouse Gas Inventory (Baggott et al, 2005).  This report follows the convention used in Baggott et al (2005) of reporting carbon dioxide emissions and removals as separate totals.  This differs from the approach used in the CRF tables submitted to UNFCCC where the carbon dioxide data are reported as net emissions (i.e. emissions minus removals).  Hence the UK National Total Emission quoted is higher than that quoted on the CRF basis.

Certain emissions cannot be allocated to a country and are reported in a table for unallocated emissions.  These emission sources are:

- Shipping

- Aviation

- Military Aviation

- Naval

- Offshore Oil and Gas.

## Report Structure

- This report is structured as follows:

Main body of the report: This part of the report presents and discusses the inventories for England, Scotland, Wales and Northern Ireland, providing greenhouse gas emissions data for the years 1990, 1995, and 1998 to 2003.  The reasons for any significant trends in emissions, issues regarding data availability and uncertainty estimates are provided for each inventory.  Tables 10.1 to 10.3 present the summary data for these years and include global warming potential (GWP) weighted emissions as well as actual emissions. Figures 1 to 6 provide country and sectoral breakdowns for each greenhouse gas.

Appendix 1: This appendix describes in detail the methodology of the estimates and how the Devolved Administration inventories relate to the UK Greenhouse Gas Inventory.

Appendix 2: This appendix provides IPCC Sectoral Tables for 1990 and 2003 for England, Scotland, Wales and Northern Ireland.  Summary tables (IPCC Sectoral Table 7A) are provided for 1995, 1998 to 2002 for England, Scotland, Wales and Northern Ireland.  UK summary tables are also reported.  Table 3 of the Sectoral Tables has been omitted because this reports Volatile Organic Compounds (VOCs) which are not relevant to this study.  In IPCC Tables, emissions are reported in Gigagrammes (Gg).  A full set of IPCC Sectoral Tables for 1990, 1995, and 1998 to 2003 is provided on a CD-ROM attached to this report.

## Global Warming Potential

Greenhouse gases all have different degrees of effectiveness in global warming.  The Global Warming Potential (GWP) is an attempt to provide a simple measure of the relative radiative effects of the emissions of the various gases.  The index is defined as the cumulative radiative forcing between the present and some chosen time horizon caused by a unit mass of gas emitted now, expressed relative to that of CO2.  It is necessary to define a time horizon because the gases have different lifetimes in the atmosphere.  Table 1.1 shows GWPs defined on a 100-year horizon (IPCC, 1996).  A range of GWP values is shown for HFCs and PFCs because these refer to a number of species, each with its own GWP.  By weighting the emission of a gas with its GWP it is possible to estimate the total contribution to global warming of UK greenhouse gas emissions.  The GWP weighted emissions in Tables 10.1 and 10.2 are expressed in equivalent masses of carbon and CO2, respectively.

- Table 1.1	GWP of Greenhouse Gases on a 100-year Horizon (t CO2 equiv/ t gas)

| Greenhouse Gas | Global Warming Potential (t CO2 equiv / t gas) |
| --- | --- |
| Carbon Dioxide | 1 |
| Methane | 21 |
| Nitrous Oxide | 310 |
| HFCs | 140-11700 |
| PFCs | 6500-9200 |
| SF6 | 23900 |

## Revisions and updates to the Greenhouse Gas inventories

Each year, the greenhouse gas inventories for England, Scotland, Wales and Northern Ireland are extended and updated.

The time series of the inventories are extended by including a new inventory year – i.e. the previous report covered the years up to and including 2002, whilst this report gives emission estimates for the years up to and including 2003.

The inventories are also updated to take account of any amendments to core activity or emission factor data, and these amendments may result in revisions to emission estimates for a given year. Core energy statistics (mainly provided by the DTI via their annual publication “The Digest of UK Energy Statistics”) are revised annually and hence the data provided (e.g. for “coal used in energy generation in 2001”) may be different in the latest edition of the Digest, compared to that used in the compilation of the previous inventory report. In addition, since the previous inventory report, a more representative emission factor for one or more greenhouse gases may have been derived for a given process. Use of a new emission factor in emission estimation calculations may lead to revisions of historic data.  The nature of emission inventories is such that ongoing improvements to data collection or estimation techniques will inevitably lead to some revisions of historic data.

Therefore, it is not appropriate to use data from previous reports and compare them with the figures in this report, without taking account of any changes to either the emission estimation methodology or the source data.  There is normally a comment in the report to indicate where such changes have occurred.

# Emissions in England

## carbon dioxide

Figure 1 (see Chapter 9) shows the emissions of CO2 for 1990 to 2003 broken down by major IPCC source category and Table 2.1 (below) highlights the most significant IPCC sectors in 2003.  Total emissions of CO2 in England accounted for 76.2% of the UK total in 2003 and have declined by 6.9% since 1990.

Table 2.1	Key Categories: England CO2 Emissions in 2003

| IPCC Sector | % of total 2003 England CO2 Emissions | Change against 1990 baseline (%) |
| --- | --- | --- |
| 1A1a – Power Generation | 31.6 | - 20.2 |
| 1A1b – Petroleum Refining | 2.9 | + 9.0 |
| 1A1c – Manufacture of Solid Fuels | 0.8 | + 45.3 |
| 1A2a – Manufacturing Industry & Construction: Iron & Steel | 2.7 | - 7.8 |
| 1A2f – Manufacturing Industry & Construction: Other | 13.3 | - 4.6 |
| 1A3b – Road Transport | 22.8 | + 7.5 |
| 1A4a – Commercial / Institutional | 4.7 | - 15.0 |
| 1A4b - Residential | 16.3 | + 13.2 |
| 2A1 – Cement Production | 1.0 | - 17.8 |
| 5D – Emissions from Soils | 1.3 | - 24.8 |

The largest source of CO2 emissions in England is from Energy Industries and in 2003 this contributed 35.4% of the total CO2 for the country, down from 36.9% of the England total of CO2 emissions in 1990. This sector includes power generation, refineries, solid fuel transformation processes and the oil and gas industry. Power generation in England contributed 31.6% of the total English CO2 emission, which is slightly higher than the UK proportion of 30.5%.

The mix of generation capacity is different in England from the rest of the UK: there is a much higher proportion of combined cycle gas turbines (CCGT) stations; a lower proportion of conventional fossil fuel stations; a lower proportion of nuclear generation and no hydroelectricity.  Emissions from Energy Industries in England have decreased by 17.5% since 1990. At the UK level, the reduction in CO2 emissions from Energy industries over 1990-2003 is only 9.8%.  This difference can be explained, in part, by the installation of CCGTs in England and increased nuclear capacity and utilisation in England over the period. The CCGTs have higher efficiency than conventional thermal stations and produce lower emissions per GWh electricity generated.

Petroleum refining constitutes 2.9% of CO2 emissions in England in 2003, lower than the UK mean contribution of 3.2% of total CO2 emissions from refineries in 2003.  Refinery CO2 emissions in England have increased by 9% since 1990.  The other energy emissions are relatively small and are mostly gas consumption at oil and gas terminals, gas separation plant, coking and solid fuel production.  Only emissions arising from on-shore installations in England have been included.  Other energy emissions have however increased by 45.3% from 1990 to 2003 as a result of an increase in gas consumption by the oil and gas industry, although the 1990 figures are highly uncertain.

Combustion emissions from Manufacturing Industry and Construction (IPCC Sector 1A2) account for 16.0% of the English total.  The iron and steel industry in England accounts for 61.2% of UK Iron and Steel combustion emissions.  The ‘Other industry’ category (IPCC sector 1A2f) for England contributes 85% towards the UK ‘Other industry’ total.

Road Transport is the largest single source after power generation, contributing 22.8% to the English total CO2 emission. The contribution of English Road Transport to UK Road Transport CO2 emissions is 83.2%, which is slightly less than that which would be expected from England's population (84% of UK). The emission has risen by 7.5% from 1990 to 2003 compared with an 8.2% rise for the UK. (See also the discussion regarding road transport emission estimation methodology in Appendix 1.)

Other combustion emissions arise from the domestic (residential), commercial and public sectors.  The emission estimations from these sectors are subject to quite significant uncertainty due to the absence of comprehensive, detailed fuel use data, particularly for solid and liquid fuels.  CO2 emissions from domestic combustion sources are estimated to account for 16.3% of the English total.  As a proportion of UK domestic emissions they are estimated to represent 82.8%, which is slightly less than would be expected from England’s population (84%).

Fugitive emissions from fuels arise mainly from flaring of coke oven gas and flaring at terminals and are not significant.

Industrial processes produce emissions from non-combustion sources such as the use of limestone in cement and glass making.  The largest contribution is from Cement Production constituting 1.0% of the English total with smaller emissions from glass, ammonia, aluminium, iron and steel production contributing a further 1.0% of the English total in 2003.  England emits all of the UK’s emissions from lime production and ammonia production, but these emissions are not significant in terms of the English total.  It should be noted that these emissions are non-combustion emissions - combustion emissions from industry are covered by category 1A2. Carbon dioxide emissions from waste incineration are not significant.  Since 1997 all waste incinerators were converted to generate electricity and hence their emissions are now reported within IPCC sector 1A1a: Power Generation.

Data are calculated and presented in this report for emissions and removals of carbon dioxide from Land Use Change and Forestry (LUCF). Emissions from LUCF in 2003 constitute 1.7% of the total English emissions and contribute 49.9% of the UK LUCF emission. England has remained a net source of carbon dioxide from LUCF activities but the size of the source has diminished by 41% from 5.6 to 3.3 Mt CO2 between 1990 and 2003. This is due to a reduction in LUCF emissions, primarily the reduction in category 5D CO2 Emissions and Removals from Soils (cultivation of mineral soils and liming of soils). Emissions from lowland drainage (5E) have also diminished over time. Category 5A Changes in Forests and Other Woody Biomass Stocks (Harvested Wood) was a sink of CO2 in 1990 but diminished over time to 2002, and became a source of CO2 in 2003. The English LUCF removals represent 25% of UK removals.  The LUCF data for 1990 to 2002 in this report have been extensively revised in comparison with last year’s publication.

## Methane

Figure 2 (see Chapter 9) shows the emissions of methane for 1990 to 2003 broken down by major IPCC source category and Table 2.2 (below) highlights the most significant IPCC sectors in 2003.  Total emissions of methane in England accounted for 70.5% of the UK total in 2003 and have declined by 52.5% since 1990. The major sources of methane in England are waste disposal, coal mining, leakage from the gas distribution system and agriculture.

Table 2.2	Key Categories: England CH4 Emissions in 2003

| IPCC Sector | % of total 2003 England CH4 | Change against 1990 baseline (%) |
| --- | --- | --- |
| 1A1a – Power Generation | 2.4 | + 548 |
| 1B1a – Mining activities | 15.9 | - 72.5 |
| 1B2b – Transmission / Distribution (Gas leakage from distribution grid) | 15.9 | - 38.5 |
| 4A1 – Enteric Fermentation (Cattle) | 24.0 | - 15.8 |
| 4A3 - Enteric Fermentation (Sheep) | 5.4 | - 21.9 |
| 4B1 – Manure Management (Cattle) | 4.0 | - 16.1 |
| 6A1 – Managed Waste Disposal on Land | 24.5 | - 66.4 |
| 6B2 – Wastewater Handling | 2.4 | + 5.0 |

The largest source of methane emissions in England is agriculture. This contributes 36% to the overall CH4 emissions in England, with cattle responsible for 78% of the agricultural emissions.  Emissions from agriculture are largely dependent on the numbers of livestock and have fallen by 19% from 1990 to 2003 resulting from a decline in cattle and sheep numbers.  England accounts for around 55% of UK agricultural methane emissions.  Of the total emission from agriculture in England, 84% is due to enteric fermentation.

The next largest source of methane emissions in England is waste disposal.  This contributes around 26.9% to England’s emissions and is overwhelmingly landfill methane with a small contribution from wastewater treatment.  The landfill emission is around 87% of the UK landfill emission, which is slightly higher than would be expected from England’s population (84%).  Estimates are based on data on disposal of municipal solid waste and sewage sludge in England, using UK data for waste composition and the percentage of MSW disposed to landfill.  Also it has been assumed that the proportion of methane recovered in England reflects that recovered in the UK.  Since 1990, landfill emissions in England have declined by 66% due to the increasing use of methane recovery systems.  This is the same trend as is observed for the UK as a whole.  Emissions from wastewater treatment are around 2.4% of the English total methane emissions and comprise 86% of UK wastewater emissions.

The category Fugitive Emissions from Fuels (IPCC Sector 1B) reports emissions of methane from coal mining, coking, the oil and gas industry and natural gas distribution.  The combined emission is around 32% of the English total methane emission compared with the UK average of 27%. The higher English emission is due to the greater contribution of coal mining and leakage from the gas transmission system in England than elsewhere in the UK.  Of these fugitive methane emissions, coal mining contributes 15.9%, natural gas distribution 15.9% and oil and gas terminals 0.1% of the English total.  Coal mining emissions have declined by 73% from 1990 to 2003 due to the decline in the coal industry.  Gas leakage from the gas transmission system is reducing as the mains and services are renewed.  The reduction in leakage between 1990 and 2003 is around 39%.

Fuel combustion activities account for 4.9% of England’s emissions of methane, mostly from domestic coal combustion and road transport.

## Nitrous Oxide

Figure 3 (see Chapter 9) shows the emissions of nitrous oxide for 1990 to 2003 broken down by major IPCC source category and Table 2.3 (below) highlights the most significant IPCC sectors in 2003. Total emissions of nitrous oxide in England accounted for 71% of the UK total in 2003 and have declined by 48% since 1990. The major sources of nitrous oxide in England are combustion, agriculture and chemical processes. Emission estimates are typically subject to significant error, particularly those from agriculture, primarily due to uncertainties associated with certain emission factors.

Table 2.3	Key Categories: England N2O Emissions in 2003

| IPCC Sector | % of total 2003 England N2O | Change against 1990 baseline (%) |
| --- | --- | --- |
| 1A1a – Power Generation | 6.2 | + 7.7 |
| 1A2f – Manufacturing Industry & Construction | 1.9 | - 32.4 |
| 1A3b – Road Transport | 14.0 | + 370 |
| 2B2 – Nitric Acid Production | 9.1 | - 21.9 |
| 2B3 – Adipic Acid Production | 2.0 | - 98 |
| 4B12 – Solid Storage & Drylot | 2.2 | - 21.4 |
| 4D – Agricultural Soils | 58.6 | - 16.4 |
| 6B2 – Wastewater Handling | 3.6 | + 9.1 |

Of the total English emission of 93 kt N2O in 2003, 57 kt N2O of this was from agriculture, representing 61% of the total.  Most of these were emissions arising from the agricultural soils category as a result of processes in the soil arising from, in order of magnitude:

[Note: numbers in brackets show the percentage of the total agricultural N2O emission (Source: IGER 2005, Personal Communication)]

- synthetic fertiliser application (27%)

- leaching of fertiliser nitrogen and applied animal manures to ground and surface water (26%)

- wastes from grazing animals (13%)

- ploughing in crop residues (13%)

- manure used as fertiliser (8%)

- atmospheric deposition of ammonia (NH3) and oxides of nitrogen (NOx) (6%)

- cultivation of legumes (2%)

- cultivation of histosols (i.e. high organic content soils) (0.7%)

- biological fixation in improved grass (0.5%)

A relatively small proportion (2.6kt N2O) is emitted from the management of animal manure (emissions related to handling of manure before it is added to the soil).  English agricultural nitrous oxide emissions have decreased by 17% in the period 1990-2003 and in 2003 represent around 65% of UK agricultural emissions.

Up until 1998, a substantial proportion of England’s nitrous oxide emissions was produced by chemical processes, namely adipic acid production and to a lesser extent nitric acid production.  In 1998, these processes constituted around 40% of England’s N2O emissions and 98% of UK industrial process N2O emissions.  In October 1998 an N2O abatement unit was commissioned on the one adipic acid production plant in England and emissions from this source were significantly reduced. In 2003, the sum of the English emissions from the nitric acid and adipic acid production is around 10.3 kt N2O, equivalent to 7.9% of the total UK N2O emission.

The remaining emissions of nitrous oxide in England result from road transport (14%) and other combustion sources (10%) - mainly combustion in power generation and industry. Note that emissions of nitrous oxide from road transport have risen by several hundred percent over the period as a result of the increasing use of catalytic converters on motor vehicles.

## Hydrofluorocarbons

Historically the largest source of HFCs is fugitive emissions from the manufacture of HCFCs and HFCs.  All such production is located in England and in 1998 this source contributed 75% of HFC emissions (as CO2 equivalent) in England and 72% of total UK HFC emissions (as CO2 equivalent).

Over recent years, HFC emissions from the manufacture of HCFCs and HFCs have been reduced through the installation of improved abatement systems on HCFC production plant. In 2003, HCFC and HFC production in England contributed only 23% of total English HFC emissions (as CO2 equivalent) and 20% of total UK HFC emissions (as CO2 equivalent).

In 2003, refrigeration contributed 45% of total English HFC emissions (as CO2 equivalent) due to losses from refrigeration and air conditioning equipment during its manufacture and lifetime. Aerosols contributed 24% to the total English HFC GWP emission in 2003, the main sources being industrial aerosols and medical use of metered dose inhalers. The remaining emission sources (foams, fire-fighting and solvents) contributed 7.9% of total English HFC emissions.

In 1998, total emissions of HFCs in England had increased by 71% since 1990 due to the increasing use of HFCs in aerosols and refrigeration and the increased production of HCFCs and HFCs.  HFC emission reductions across several sectors after 1998 mean that emissions of HFCs in England in 2003 are down to 82% of their 1990 levels.

## Perfluorocarbons

The largest source of perfluorocarbons in England is aluminium production, which accounts for 59% of total English PFC emissions (as CO2 equivalent) in 2003.  Aluminium plants are also located in Scotland and Wales and overall England contributes 66% to the total UK emissions from aluminium production.

All PFC production is located in England.  Fugitive emissions from the production of PFCs are the second largest source of PFC emissions in England accounting for 30% (as CO2 equivalent) of the 2003 emissions.

The remaining emissions of PFC in England arise from the IPCC category 2F6 ‘Other’.  This category accounts for 11% of the 2003 English total (as CO2 equivalent).  Emissions in this category consist mostly of emissions from the electronics industry.

## Sulphur Hexafluoride

The main source of sulphur hexafluoride emissions is from use as a cover gas in magnesium production.  This accounted for around 60% of total England SF6 emissions in 2003.  Magnesium production is largely concentrated in England and accounts for 93% of the UK magnesium production emission.  Sulphur hexafluoride is also emitted from other sources: electrical switchgear used in power transmission, electronics applications and leakage from the soles of certain brands of training shoes.  The sum of these emissions accounts for around 40% of total English emissions in 2003.

The sum of SF6 emissions from England represents 90% of the UK total in 2003.  Emissions of SF6 in England have increased by 44% since 1990 due to increases in emissions from trainers, magnesium production and switchgear.

# Emissions in Scotland

## Carbon Dioxide

Figure 1 (see Chapter 9) shows the emissions of carbon dioxide for 1990 to 2003 broken down by major IPCC source category and Table 3.1 (below) highlights the most significant IPCC sectors in 2003.  Total emissions of carbon dioxide in Scotland accounted for 9.5% of the UK total in 2003 and have declined by 7.7 % since 1990.

Table 3.1	Key Categories: Scotland CO2 Emissions in 2003

| IPCC Sector | % of total 2003 Scotland CO2 Emissions | Change against 1990 baseline (%) |
| --- | --- | --- |
| 1A1a – Power Generation | 33.2 | + 21.1 |
| 1A1b – Petroleum Refining | 3.8 | - 28.4 |
| 1A1c – Manufacture of Solid Fuels | 3.9 | + 33.4 |
| 1A2f – Manufacturing Industry & Construction: Other | 10.3 | - 23.6 |
| 1A3b – Road Transport | 18.0 | + 7.0 |
| 1A4a – Commercial / Institutional | 3.8 | - 35.3 |
| 1A4b - Residential | 12.9 | + 3.2 |
| 1B2aii - Oil Production | 1.2 | - 44.4 |
| 5A2 – Changes in Temperate Forest Stocks | - 18.5 (SINK) | + 33.4 |
| 5D – Emissions from Soils | 9.2 | - 4.4 |

The largest source of CO2 emissions in Scotland is Energy Industries, which includes power generation, refineries, solid fuel transformation processes and the oil and gas industry.  In 2003, power generation (IPCC category 1A1a) contributed around 33% of the total Scottish CO2 emission, which is somewhat higher than the UK average of 30.5%. Scottish emissions from 1A1a have increased by 21% since 1990 in contrast with a fall of 15% in UK emissions.

These observations may be due to Scotland generating electricity that is subsequently exported and used elsewhere in the UK. The mix of generation capacity in Scotland is different from the rest of the UK, there being a higher proportion of nuclear and hydro-electricity plant, and hence a lower carbon dioxide emission may be expected.  On the other hand, the fossil fuel generation in Scotland is from conventional coal and gas fired stations, whilst in England and Wales there has been increased commissioning and utilisation of combined cycle gas turbines (CCGT) over the period that exhibit higher generation efficiencies than conventional thermal plant.

Petroleum refining constitutes a larger proportion of national emissions in Scotland at 3.8 % compared with 3.2% for the UK due to the greater incidence of oil and gas landings in Scotland from offshore facilities compared to the UK average.  The other energy emissions account for around 3.9% of Scottish emissions, mostly from gas consumption at oil & gas terminals and gas separation plant.  Only those emissions arising from on-shore installations in Scotland have been included.

CO2 emissions from Manufacturing Industry and Construction (IPCC Sector 1A2) account for 10.5% of the Scottish total compared with 15.3% for the UK. Between 1990 and 2003, emissions have declined over the period by 54%, mainly due to the closure of the Ravenscraig Steel Plant.

Road transport is the largest single source of CO2 after power generation and comprises around 18% of the Scottish total. Scotland’s contribution to UK road transport emissions is 8.2%, which is slightly lower than would be expected from Scotland’s population (8.5%).  The emission has risen by 7% over the period (1990-2003) compared with an 8% rise for the UK.  (See also the discussion regarding road transport emission estimation methodology in Appendix 1.)

Other combustion emissions arise from the domestic, commercial and public sectors. The emission estimations from these sectors are subject to quite significant uncertainty due to the absence of comprehensive, detailed fuel use data, particularly for solid and liquid fuels. CO2 emissions from domestic combustion sources are estimated to account for around 12.9% of the Scottish total.  As a proportion of UK domestic emissions they are 8.2% which is consistent with Scotland’s population (8.5%).

Around 1.8% of emissions arise from oil and gas fugitives, mainly oil and gas flaring.  Between 1990 to 2003, emissions from flaring have fallen by 69% due to greater focus on gas flaring reduction through environmental regulations.

Industrial processes produce emissions from non-combustion sources such as the use of limestone in cement and glass making.  The largest contribution is from cement with smaller emissions from glass and aluminium production.  Together these processes emitted around 0.8% of the Scottish total in 2003 and have decreased by 35% over 1990-2003. Since 1990 emissions from iron and steel processes have become negligible (due to the Ravenscraig plant closure).

- Carbon dioxide emissions from waste incineration are not significant.

Data are calculated and presented in this report for emissions and removals of carbon dioxide from Land Use Change and Forestry (LUCF). Emissions from LUCF in 2003 constitute 9.5% of the total Scottish emission and contribute 35.1% of the UK LUCF emission. Scotland is a net sink for carbon dioxide from LUCF activities and the size of this sink has increased by 61% from 3.0 to 4.9 Mt CO2 from 1990 to 2003. This is due to a small reduction in overall LUCF emissions and a 20% increase in LUCF removals, primarily through forest growth under category 5A Changes in Forests and Other Woody Biomass Stocks (Temperate Forests). Category 5A Changes in Forests and Other Woody Biomass Stocks (Harvested Wood) was a sink of CO2 in 1990 but diminished over time to 2002, and became a source of CO2 in 2003. The Scottish removals represent 62% of UK removals.  The LUCF data for 1990 to 2002 in this report have been extensively revised in comparison with last year’s publication.

## Methane

Figure 2 (see Chapter 9) shows the emissions of methane for 1990 to 2003 broken down by major IPCC source category and Table 3.2 (below) highlights the most significant IPCC sectors in 2003.  Total emissions of methane in Scotland accounted for 10.7% of the UK total in 2003 and have declined by 35% since 1990. The major sources of methane in Scotland are waste disposal, coal mining, leakage from the gas distribution system and agriculture.

Table 3.2	Key Categories: Scotland CH4 Emissions in 2003

| IPCC Sector | % of total 2003 Scotland CH4 | Change against 1990 baseline (%) |
| --- | --- | --- |
| 1A1a – Power Generation | 1.2 | + 937 |
| 1B1a – Mining activities | 1.1 | - 91.5 |
| 1B2b – Transmission / Distribution | 8.2 | - 40.5 |
| 4A1 – Enteric Fermentation (Cattle) | 46.3 | - 3.9 |
| 4A3 - Enteric Fermentation (Sheep) | 18.8 | -17.6 |
| 4B1 – Manure Management (Cattle) | 5.8 | - 4.5 |
| 6A1 – Managed Waste Disposal on Land | 11.6 | - 61.9 |
| 6B2 – Wastewater Handling | 1.4 | + 369 |

The largest source of methane emissions in Scotland is agriculture.  Around 73% of Scottish emissions arise from agriculture with cattle responsible for 71% and sheep 26% of Scottish agricultural emissions. Scottish agricultural emissions arise from enteric fermentation in livestock (90%) and the handling of their wastes (10%).  Emissions are largely dependent on the numbers of livestock and have fallen by 8% over the period 1990-2003, due to a decline in cattle and sheep numbers.  Scotland accounts for around 17% of UK agricultural emissions.

The next largest source of methane in Scotland in 2003 is waste disposal.  This contributes around 13% to Scotland’s emissions and is overwhelmingly landfill methane with a small contribution from wastewater treatment. Scotland contributes 6.3% to the UK’s landfill emission, which is slightly lower than its 8.5% share of the UK population total.  Estimates were based on data on arisings of municipal solid waste and sewage sludge in Scotland but using UK data for their composition and the proportion of Municipal Solid Waste disposed of to landfill.  It has been assumed that the degree of methane recovery from Scottish landfills reflects that of the UK.  Landfill emissions have fallen by 62% since 1990 due an increase in the use of methane recovery systems, though this reduction assumes the UK trend.  Emissions from wastewater treatment are estimated to be around 7.9% of UK wastewater treatment emissions.  They have increased significantly since 1998 when sea dumping ended and other disposal routes were adopted.

The category Fugitive Emissions from Fuels (IPCC Sector 1B) reports emissions of methane from coal mining, the oil and gas industry and natural gas distribution.  The combined emission is 10.3% of the Scottish total.  This is a lower proportion compared with the UK as a whole, where fugitives are around 27% of the total emissions.  This is as a result of the greater contribution of coal mining and leakage from the gas transmission system elsewhere in the UK. Of these emissions, those from coal mining contributed 1.1%, oil and gas terminals 0.9% and natural gas distribution 8.2% of the Scottish total.  Coal mining emissions have declined by 92% over the period due to the decline in the coal industry. Emissions from the oil & gas industry have fallen by 88% over the same period due to tighter regulation of environmental emissions. Gas leakage from the gas transmission system was reduced by 41% over 1990-2003 as the mains and services are renewed.  The estimate of gas leakage from the gas transmission system is based on Transco data.

Fuel combustion activities account for 2.9 % of Scottish methane emissions.  These emissions stem primarily from power generation and domestic fuel combustion.

## Nitrous Oxide

Figure 3 (see Chapter 9) shows the emissions of nitrous oxide for 1990 to 2003 broken down by major IPCC source category and Table 3.3 (below) highlights the most significant IPCC sectors in 2003. Total emissions of nitrous oxide in Scotland accounted for 12.7% of the UK total in 2003 and have declined by 16.2% since 1990. The major sources of nitrous oxide in Scotland are combustion and agriculture. Emission estimates are typically subject to significant error, particularly those from agriculture, primarily due to uncertainties associated with certain emission factors.

Table 3.3	Key Categories: Scotland N2O Emissions in 2003

| IPCC Sector | % of total 2003 Scotland N2O | Change against 1990 baseline (%) |
| --- | --- | --- |
| 1A1a – Power Generation | 4.1 | + 70.1 |
| 1A2f – Manufacturing Industry & Construction | 1.2 | -53.9 |
| 1A3b – Road Transport | 7.7 | + 360 |
| 4B12 – Solid Storage & Drylot | 3.6 | - 1.8 |
| 4D – Agricultural Soils | 79.1 | - 17.4 |
| 6B2 – Wastewater Handling | 1.9 | + 387 |

Of the total Scottish emission of 17kt N2O in 2003, around 14kt N2O of this was from agriculture, representing 83% of the total.  Most of these were emissions arising from the agricultural soils category as a result of processes in the soil arising from, in order of magnitude:

[Note: numbers in brackets show the percentage of the total agricultural N2O emission (Source: IGER 2005, Personal Communication)]

- leaching of fertiliser nitrogen and applied animal manures to ground and surface water (27%)

- synthetic fertiliser application (26%)

- wastes from grazing animals (20%)

- manure used as fertiliser (8%)

- ploughing in crop residues (7%)

- atmospheric deposition of ammonia (NH3) and oxides of nitrogen (NOx) (6%)

- biological fixation in improved grass (0.7%)

- cultivation of histosols (i.e. high organic content soils) (0.2%)

- cultivation of legumes (<0.1%)

- field burning (discontinued in 1993)

A relatively small proportion (0.7kt N2O) is emitted from animal manure management.  Scottish agricultural nitrous oxide emissions have fallen by 17% between 1990 and 2003, and in 2003 represent around 16% of UK agricultural emissions.

The remaining emissions of nitrous oxide result from fuel combustion (2.5 kt N2O). Just over half (1.3 kt N2O) of this is from road transport with the remainder arising from stationary combustion such as power generation and industrial sources.  Note that emissions of nitrous oxide from road transport have risen by several hundred percent over the period as a result of the increasing use of catalytic converters on motor vehicles.

In 1990 around 1.3 kt of nitrous oxide were emitted from a nitric acid plant in Leith, however by 1995 this had been dismantled and moved to Dublin.  This is a major component of the reduction in Scottish emissions.

## Hydrofluorocarbons

- Scottish emissions of HFCs were 6.6% of the UK total (as CO2 equivalent) in 2003.  The main sources are aerosols and refrigeration.

In 2003, refrigeration contributed 57% of total Scottish HFC emissions (as CO2 equivalent) due to losses from refrigeration and air conditioning equipment during its manufacture and lifetime. Aerosols contributed 32% to the total Scottish HFC emission in 2003, the main sources being industrial aerosols and medical use of metered dose inhalers. The remaining emission sources (foams, fire-fighting and solvents) contributed 10.6% of total Scottlish HFC emissions.

The total emission has increased from virtually zero in 1990 to 708 kt in 2003 due to the greater use of these substances to replace alternative banned species such as CFCs.

## Perfluorocarbons

The largest source of perfluorocarbons in Scotland is consumption by the electronics industry.  In 2003, this contributed around 90% to the total Scottish PFC emission (as CO2 equivalent).  The other main source of PFCs in Scotland is aluminium production and this contributes 10% to the total emissions of PFCs from Scotland.

Overall, Scottish PFC emissions account for 21.5% of the UK total (as CO2 equivalent) and have decreased by 28% over 1990-2003 as the decreases in emissions from the aluminium production have out-weighed the increase from the electronics industry.

## Sulphur Hexafluoride

All emissions of SF6 in Scotland occur in the IPCC category 2F6.  This category includes emissions from the electronics industry, as well as leakage from electrical switchgear and from the soles of certain brands of training shoes.  Overall emissions in 2003 are 3.3% of the UK total and in Scotland the emissions of SF6 have increased by 159% over 1990-2003.

# Emissions in Wales

## Carbon Dioxide

Figure 1 (see Chapter 9) shows the emissions of carbon dioxide for 1990 to 2003 broken down by major IPCC source category and Table 4.1 (below) highlights the most significant IPCC sectors in 2003.  Total emissions of carbon dioxide in Wales accounted for 7.4% of the UK total in 2003 and have increased by 0.2% since 1990.

Table 4.1	Key Categories: Wales CO2 Emissions in 2003

| IPCC Sector | % of total 2003 Wales CO2 Emissions | Change against 1990 baseline (%) |
| --- | --- | --- |
| 1A1a – Power Generation | 32.3 | + 21.8 |
| 1A1b – Petroleum Refining | 7.9 | - 7.0 |
| 1A1c – Manufacture of Solid Fuels | 0.8 | - 48.0 |
| 1A2a – Manufacturing Industry & Construction: Iron & Steel | 17.6 | - 5.3 |
| 1A2f – Manufacturing Industry & Construction: Other | 6.8 | - 23.9 |
| 1A3b – Road Transport | 14.0 | + 7.3 |
| 1A4a – Commercial / Institutional | 2.4 | - 29.8 |
| 1A4b - Residential | 10.8 | + 13.6 |
| 2A1 – Cement Production | 1.0 | - 19.6 |
| 5A2 – Changes in Temperate Forest Stocks | -3.3 (SINK) | + 4.2 |
| 5D – Emissions from Soils | 2.8 | - 10.7 |

The largest source of CO2 emissions in Wales is Energy Industries, which includes power generation, refineries and solid fuel transformation processes.  Electricity generation contributed around 32.3% of the total Welsh carbon dioxide emissions in 2003, which is slightly higher than the UK proportion of 30.5%.  Emissions from electricity generation in Wales have increased by 22% compared with a fall of 15% in UK emissions over 1990 to 2003.

There is now only one nuclear power station in Wales whilst there has been a growth of Combined Cycle Gas Turbines (CCGTs) to replace the generating capacity from Trawsfynydd Nuclear Station which closed in 1991. The remaining fossil fuel generation is from two conventional coal stations.

There has been an increase in generation capacity in Wales from the opening of several CCGTs: a 500 MW CCGT at Deeside in 1994, a 1,420 MW CCGT at Connahs Quay in 1996, a 250 MW CCGT at Barry in 1998, and a 575 MW CCGT at Baglan Bay in 2002.  The quantity of coal consumed at Aberthaw power station has declined over the period 1990 to 2003.  Two power stations have closed: the oil-fired station at Pembroke, and the coal-fired station at Uskmouth. Uskmouth has subsequently re-opened as Fifoots after being upgraded and fitted with Flue Gas Desulphurisation.

Petroleum refining constitutes 7.9% of Welsh CO2 emissions in 2003 compared with 3.2% for the UK.  The other energy emissions are mostly combustion emissions from coke ovens and solid fuel plant and account for 0.8% of the 2003 Welsh carbon dioxide total emission.  There are no significant emissions from oil and gas production.

Combustion emissions from Manufacturing Industries and Construction (IPCC Sector 1A2) account for 24.4% of the Welsh total compared with 15.3% for the UK. The high contribution from industry can be explained by the high concentration of iron and steel plant in Wales.  This accounts for 38% of UK Iron and Steel combustion emissions in 2003.  The Welsh ‘other industry’ category is 8.2% of the UK total.

Iron & Steel Sector in Wales: Recent Trends

Over recent years there have been significant fluctuations in the CO2 emissions from the iron and steel sector in Wales, and these have had a big impact on the total CO2 emissions trends. Both iron & steel production and coke use statistics dipped in 2002, primarily due to the explosion at Port Talbot blast furnace #5 in November 2001  (the furnace provides just under half of Port Talbot production and didn't come back on line until January 2003). Hence Wales CO2 emissions dipped in 2002 as the production of pig iron at Port Talbot during 2002 was limited to the 2 Mtonnes capacity of #4 blast furnace, whereas during 2001 and 2003 there was additional production of approximately 1-1.5 Mtonnes from the old and new #5 furnaces respectively.

More generally, pig iron production in Wales increased throughout the 1990s, but then in 2001 the Llanwern blast furnaces were closed with the result that pig iron production fell from 5.2 Mtonnes in 2000 to 3.4 Mtonnes in 2001.  The loss of #5 at Port Talbot then led to a further fall to 1.9 Mtonnes in 2002 before the new #5 came on stream in 2003 and production of pig iron increased again to 3.3 Mtonnes, leading to a significant growth in CO2 emissions from this source over 2002-2003.

Road transport is the largest single source after power generation and iron and steel, and comprises 14% of the total Welsh carbon dioxide emission in 2003. The contribution of Welsh Road Transport to UK Road Transport CO2 emissions is 5.0%, which is consistent with Wales’ population (4.9% of UK population). The emission has risen by 7.3% from 1990 to 2003 compared with an 8.2% rise for the UK. (See also the discussion regarding road transport emission estimation methodology in Appendix 1.)

Other combustion emissions arise from the domestic, commercial and public sectors. The emission estimations from these sectors are subject to quite significant uncertainty due to the absence of comprehensive, detailed fuel use data, particularly for solid and liquid fuels. CO2 emissions from domestic combustion sources are estimated to account for 10.8% of the Welsh total in 2003. As a proportion of UK domestic emissions they are estimated to represent 5.4%, which is consistent with the relative populations.

Industrial processes also produce emissions from non-combustion sources such as the use of limestone in cement and glass making. In Wales the largest contribution is from the Iron & Steel Industry from a range of sources: limestone use in blast furnaces, flaring of blast furnace gas and electric arc furnaces. Other industrial processes include cement, aluminium and glass production.  Together these processes emitted 3.3% of the Welsh total in 2003.  The Welsh industrial process emission is 13% of the UK due to the high proportion of iron & steel and aluminium production in Wales.

Data are calculated and presented in this report for emissions and removals of carbon dioxide from Land Use Change and Forestry (LUCF). Emissions from LUCF in 2003 constitute 2.9% of the total Welsh emission and contribute 8.2% of the UK LUCF emission. Wales is a net sink of carbon dioxide from LUCF activities but the size of this sink has diminished from 0.3 to 0.1 Mt CO2 since 1990. LUCF emissions have decreased over time, but LUCF removals have also decreased. Category 5A Changes in Forests and Other Woody Biomass Stocks (Harvested Wood) was a sink of CO2 in 1990 but diminished over time to 2002, and became a source of CO2 in 2003. The Welsh removals represent 8% of UK removals.  The LUCF data for 1990 to 2002 in this report have been extensively revised in comparison with last year’s publication.

There are no municipal waste incinerators in Wales but a small emission from clinical incineration is reported.

## Methane

Figure 2 (see Chapter 9) shows the emissions of methane for 1990 to 2003 broken down by major IPCC source category and Table 4.2 (below) highlights the most significant IPCC sectors in 2003. Total emissions of methane in Wales accounted for 9.4% of the UK total in 2003 and have declined by 34.5% since 1990. The major sources of methane in Wales are waste disposal, coal mining, leakage from the gas distribution system and agriculture.

Table 4.2	Key Categories: Wales CH4 Emissions in 2003

| IPCC Sector | % of total 2003 Wales CH4 | Change against 1990 baseline (%) |
| --- | --- | --- |
| 1A1a – Power Generation | 1.2 | + 1004 |
| 1A2a – Manufacturing Industry & Construction: Iron & Steel | 1.3 | + 12.8 |
| 1B1a – Mining activities | 4.8 | - 84.0 |
| 1B2b – Transmission / Distribution | 6.0 | - 35.1 |
| 4A1 – Enteric Fermentation (Cattle) | 39.0 | - 1.7 |
| 4A3 - Enteric Fermentation (Sheep) | 26.6 | - 6.6 |
| 4B1 – Manure Management (Cattle) | 6.1 | - 2.6 |
| 6A1 – Managed Waste Disposal on Land | 11.0 | - 66.1 |

The largest source of methane emissions in Wales is agriculture.  Around 73% of Wales’ methane emissions arise from agriculture, with cattle responsible for 61% and sheep 37% of Welsh agricultural emissions.  Welsh agricultural emissions arise from enteric fermentation in livestock (90%) and the management of their wastes (10%).  Emissions are largely dependent on the numbers of livestock and have decreased by 3.5% over the period 1990-2003, due to a decline in cattle and sheep numbers. Wales accounts for around 15% of UK agricultural emissions.

The next largest source of methane is waste, which contributes 11.6% to Wales’ emissions in 2003.  Of this, 95% is landfill methane, with a small contribution from wastewater treatment (5%).  The landfill emission is 5.2% of UK landfill emissions which is consistent with the respective populations.  Estimates were based on data on arisings of municipal solid waste and sewage sludge in Wales but using UK data for their composition and the proportion of MSW disposed of to landfill.  The degree of methane recovery on Welsh landfills was assumed to reflect that of the rest of the UK.  On this basis, landfill emissions have fallen by 66% since 1990 because of increasing use of methane recovery systems reflecting the UK trend.  Emissions from Welsh wastewater treatment are 3.1% of UK wastewater treatment emissions and are dependent on the data on sewage disposals and disposal routes used.

The category Fugitive Emissions from Fuels (IPCC Sector 1B) reports emissions of methane from coal mining, coke production and natural gas distribution.  The combined emission is 10.9% of the 2003 Welsh total, compared to a UK average of 27%. This is due to the greater contribution of coal mining, oil and gas production and leakage from the gas transmissions system elsewhere in the UK.  Of these fugitive emissions, coal mining contributes 4.8% and natural gas distribution 6.0% to the 2003 Welsh total methane emission.  Coal mining emissions have declined by 84% over 1990-2003 due to the decline in the coal industry, whilst emissions from gas leakage from the gas transmission system have reduced by 35% as the mains and services have been renewed.

Fuel combustion activities account for 4.0% of total Welsh methane emissions. These emissions stem primarily from sintering processes in the iron and steel industry, power generation and domestic combustion.

## Nitrous Oxide

Figure 3 (see Chapter 9) shows the emissions of nitrous oxide for 1990 to 2003 broken down by major IPCC source category and Table 4.3 (below) highlights the most significant IPCC sectors in 2003. Total emissions of nitrous oxide in Wales accounted for 8.3% of the UK total in 2003 and have declined by 5.7% since 1990. The major sources of nitrous oxide in Wales are combustion and agriculture. Emission estimates are typically subject to significant error, particularly those from agriculture, primarily due to uncertainties associated with certain emission factors.

Table 4.3	Key Categories: Wales N2O Emissions in 2003

| IPCC Sector | % of total 2003 Wales N2O | Change against 1990 baseline (%) |
| --- | --- | --- |
| 1A1a – Power Generation | 5.7 | + 88.7 |
| 1A2a – Manufacturing Industry & Construction: Iron & Steel | 1.0 | - 7.2 |
| 1A3b – Road Transport | 7.5 | + 380 |
| 4B12 – Solid Storage & Drylot | 3.5 | - 2.8 |
| 4D – Agricultural Soils | 78.1 | - 13.9 |
| 6B2 – Wastewater Handling | 1.1 | + 22.6 |

Of the total Welsh emission of 11kt N2O in 2003, 8.9 kt N2O (82%) of this was from agriculture. Most of these were emissions arising from the agricultural soils category as a result of processes in the soil arising from, in order of magnitude:

[Note: numbers in brackets show the percentage of the total agricultural N2O emission (Source: IGER 2005, Personal Communication)]

- wastes from grazing animals (31%)

- leaching of fertiliser nitrogen and applied animal manures to ground and surface water (27%)

- synthetic fertiliser application (18%)

- manure used as fertiliser (9%)

- atmospheric deposition of ammonia (NH3) and oxides of nitrogen (NOx) (7%)

- ploughing in crop residues (1%)

- biological fixation in improved grass (1%)

- cultivation of histosols (i.e. high organic content soils) (0.4%)

- cultivation of legumes (<0.1%)

- field burning (discontinued in 1993)

A relatively small proportion (0.4 kt N2O) is emitted from the treatment of agricultural wastes (manure management). Welsh agricultural nitrous oxide emissions have fallen by 14% between 1990 and 2003, and in 2003 represent around 10% of UK agricultural emissions.

The remaining 1.8 kt (16.8 % of the 2003 Welsh total) of nitrous oxide emissions results from fuel combustion activities.  The main sources are power generation, road transport and manufacturing industry.  Note that emissions of nitrous oxide from road transport have risen by several hundred percent over the period as a result of the increasing use of catalytic converters on motor vehicles.

## hydrofluorocarbons

- In 2003 the total HFC emission in Wales was 3.6% of the UK HFC total (as CO2 equivalent).  Refrigeration is the largest source and contributes 54% to the total Welsh HFC emission (as CO2 equivalent) due to losses from refrigeration and air conditioning equipment during its manufacture and lifetime.  Aerosols contribute 34% to the total Welsh HFC emission (as CO2 equivalent), the main sources being industrial aerosols and medical use of metered dose inhalers. The remaining emission sources (foams, fire-fighting and solvents) contributed 11.4% to the total Welsh HFC emission (as CO2 equivalent) in 2003.

## Perfluorocarbons

The largest emission source of perfluorocarbons in Wales is the aluminium industry, which contributes 92% of the total Welsh PFC emission (as CO2 equivalent) in 2003, and 30% of UK aluminium emissions. The electronics industry also makes an important contribution accounting for 7.8% of Wales’ PFC emission (as CO2 equivalent) and 5% of UK electronics emissions.  The remaining sources are fire extinguishers and refrigeration.  Overall, Welsh PFC emissions are 17.8% of the UK PFC total (as CO2 equivalent).  Emissions of PFC have decreased by 78.5% from 1990 to 2003 mainly as a result of improved control measures in the aluminium industry.

## Sulphur Hexafluoride

Welsh emissions of sulphur hexafluoride are estimated at 6.6 % of the UK total in 2003.  The largest source of emissions is from use as a cover gas in magnesium production. These account for around 62% of total Welsh emissions and comprise 7% of the UK magnesium production emission.  The other source of SF6 in Wales is from IPCC category 2F6 and this accounts for 38% of emissions.  This category includes leakage from the soles of certain brands of training shoes, emissions from the electrical switchgear used in electricity transmission and emissions from the electronics industry.

# Emissions in Northern Ireland

## Carbon Dioxide

Figure 1 (see Chapter 9) shows the emissions of carbon dioxide for 1990 to 2003 broken down by major IPCC source category and Table 5.1 (below) highlights the most significant IPCC sectors in 2003.  Total emissions of carbon dioxide in Northern Ireland accounted for 2.8% of the UK total in 2003 and have declined by 3.4% since 1990.

Table 5.1	Key Categories: Northern Ireland CO2 Emissions in 2003

| IPCC Sector | % of total 2003 N. Ireland CO2 Emissions | Change against 1990 baseline (%) |
| --- | --- | --- |
| 1A1a – Power Generation | 31.2 | - 8.8 |
| 1A2f – Manufacturing Industry & Construction: Other | 11.7 | + 24.2 |
| 1A3b – Road Transport | 27.1 | + 34.7 |
| 1A4a – Commercial / Institutional | 1.7 | - 39.1 |
| 1A4b - Residential | 19.6 | - 20.3 |
| 2A1 – Cement Production | 1.1 | - 53.4 |
| 5A2 – Changes in Temperate Forest Stocks | - 4.6 (SINK) | + 2.3 |
| 5D – Emissions from Soils | 3.2 | -26.8 |
| 5E – Other Land Use Change Emissions | 3.0 | 0.0 |

Energy Industries is the largest source of carbon dioxide emissions in Northern Ireland, and in Northern Ireland this is entirely power generation as there are no refineries, collieries, solid fuel transformation plant or oil and gas processing sources.  Electricity generation contributed 31.2% of the total emission, which is similar to the UK proportion of 30.5%.  The mix of generation capacity is quite different from the rest of the UK and from 1990 to 1995 consisted entirely of coal and oil fired stations.  In 1996, the largest power station in Northern Ireland, Ballylumford, was converted from oil to use natural gas. The lack of nuclear and renewable generation up to 1996, together with the lack of natural gas contributed to the proportionately high emission from electricity generation.  Moreover, the non-availability of natural gas led to a proportionately higher consumption of electricity than in the rest of the UK, also increasing emissions.  The emission of CO2 per unit energy produced is lower for natural gas than other fossil fuels.  Natural gas has been supplied to some industrial, commercial and domestic users since 1999 and gas use continues to grow as the supply infrastructure is developed. Emissions from electricity generation decreased by 8.8% over the 1990-2003, largely due to the conversion to natural gas.

Road transport is the next largest source of carbon dioxide emissions in Northern Ireland and comprises 27.1% of the 2003 total.  The contribution of Northern Irish Road Transport to UK Road Transport CO2 emissions is 3.6%, which is slightly higher than that which would be expected from Northern Ireland’s population (2.9% of UK). The emission has risen by 34.7% from 1990 to 2003 compared with an 8.2% rise for the UK. (See also the discussion regarding road transport emission estimation methodology in Appendix 1.)

Combustion emissions from Manufacturing Industry and Construction (IPCC Sector 1A2) account for 11.7% of the total Northern Ireland carbon dioxide emission compared with 15.3% for the UK.  There is no iron and steel production in Northern Ireland, so the category is entirely ‘Other Industry’.  The Other Industry category (IPCC sector 1A2f) for Northern Ireland contributes 2.7% towards the UK Other Industry total, and has increased by approximately 24% over the period 1990-2003.

Other combustion emissions arise from the domestic, commercial and public sectors.  The emission estimations from these sectors are subject to quite significant uncertainty due to the absence of comprehensive, detailed fuel use data, particularly for solid and liquid fuels.  CO2 emissions from domestic combustion sources are estimated to account for 19.6% of the Northern Irish total.  As a proportion of UK domestic emissions they are estimated to represent 3.7%, which is slightly higher than would be expected from Northern Ireland’s population (2.9% of UK). The reason for this is the very limited availability of natural gas resulting in the high consumption of coal, burning oil and gas oil in the domestic sector.  Northern Ireland has a proportionately higher consumption of LPG (bottled gas) than the rest of the UK, but in absolute terms this is not a significant source of carbon dioxide emissions.  The high consumption of coal and oil result in a higher emission per unit energy use than in the rest of the UK.

Industrial process emissions (primarily from mineral processing sources) in Northern Ireland contribute 1.1% to the total 2003 CO2 emission.  Waste incineration is not a significant source of CO2 in Northern Ireland.

Data are calculated and presented in this report for emissions and removals of carbon dioxide from Land Use Change and Forestry (LUCF). Emissions from LUCF in 2003 constitute 6.2% of the total Northern Irish emission and contribute 6.7% of the UK LUCF emission. Northern Ireland is a net source of carbon dioxide from LUCF activities but the size of this source has diminished from 0.4 Mt CO2 to 0.2 Mt CO2 between 1990 and 2003.  This is due to the reduction in LUCF emissions from category 5D CO2 Emissions and Removals from Soils. In contrast to the other countries, in Northern Ireland the category 5A Changes in Forests and Other Woody Biomass Stocks (Harvested Wood) fluctuates over time but remains a sink of CO2. The Northern Irish removals represent 5% of UK removals.  The LUCF data for 1990 to 2002 in this report have been extensively revised in comparison with last year’s publication.

## Methane

Figure 2 (see Chapter 9) shows the emissions of methane for 1990 to 2003 broken down by major IPCC source category and Table 5.2 (below) highlights the most significant IPCC sectors in 2003. Total emissions of methane in Northern Ireland accounted for 6.7% of the UK total in 2003 and have declined by 4.0% since 1990. The main sources of methane in Northern Ireland are waste disposal and agriculture.

Table 5.2	Key Categories: Northern Ireland CH4 Emissions in 2003

| IPCC Sector | % of total 2003 N. Ireland CH4 | Change against 1990 baseline (%) |
| --- | --- | --- |
| 1A1a – Power Generation | 0.5 | + 413 |
| 1A4b – Residential | 2.2 | - 59.7 |
| 4A1 – Enteric Fermentation (Cattle) | 69.7 | + 10.2 |
| 4A3 - Enteric Fermentation (Sheep) | 8.3 | - 19.5 |
| 4B1 – Manure Management (Cattle) | 10.3 | + 13.7 |
| 4B8 – Manure Management (Swine) | 1.0 | - 36.9 |
| 4B9 – Manure Management (Poultry) | 1.1 | + 65.9 |
| 6A1 – Managed Waste Disposal on Land | 4.6 | - 57.2 |
| 6B2 – Wastewater handling | 1.0 | + 12.8 |

The largest source of methane emissions in Northern Ireland is agriculture.  Around 91% of Northern Ireland’s emissions arise from agriculture with cattle responsible for 88% of agricultural emissions.  Agricultural emissions arise from enteric fermentation in livestock (86%) and the management of their wastes (14%).  Emissions are dependent on the numbers of livestock and have increased by 6% over the period (1990 to 2003) resulting from an increase in cattle.  Northern Ireland accounts for around 13% of UK agricultural emissions.

Note that two of the key agricultural sectors that influence Northern Ireland’s methane emissions are ‘Dairy Cattle emissions from enteric fermentation’ and ‘Emissions from Dairy Cattle Wastes’ in IPCC sectors 4A1 and 4B1 respectively. The UK emissions for these sources have undergone a significant time-series revision in the latest inventory compilation. For further details see section 7.2.8.

Waste disposal contributes 5.6% to Northern Ireland’s 2003 CO2 emissions totally, from landfill methane (4.6%) and wastewater treatment (1%). The landfill emission is 1.6% of UK landfill emissions compared to a share of the UK population of 2.9%.  Estimates are based on data on arisings of municipal solid waste and sewage sludge in Northern Ireland using UK data for waste composition, percentage of MSW disposed to landfill, and proportion of methane recovery. On this basis, landfill emissions have fallen by 57% due to increasing use of methane recovery systems, reflecting the UK trend.  Emissions from wastewater treatment are 3.4% of UK wastewater treatment emissions.

Combustion emissions are not a large source of methane, accounting for only 3.1% of the total Northern Irish methane emission, mostly from domestic combustion of coal and anthracite.

## Nitrous Oxide

Figure 3 (see Chapter 9) shows the emissions of nitrous oxide for 1990 to 2003 broken down by major IPCC source category and Table 5.3 (below) highlights the most significant IPCC sectors in 2003. Total emissions of nitrous oxide in Northern Ireland account for 6.8% of the UK total in 2003 and have declined by 10.1% since 1990. The major sources of nitrous oxide in Northern Ireland are agriculture and road transport. Emission estimates are typically subject to significant error, particularly those from agriculture, primarily due to uncertainties associated with certain emission factors.

Table 5.3	Key Categories: Northern Ireland N2O Emissions in 2003

| IPCC Sector | % of total 2003 N. Ireland N2O | Change against 1990 baseline (%) |
| --- | --- | --- |
| 1A1a – Power Generation | 2.3 | + 119 |
| 1A3b – Road Transport | 6.6 | + 425 |
| 4B12 – Solid Storage & Drylot | 5.7 | - 2.5 |
| 4D – Agricultural Soils | 80.9 | - 3.7 |
| 6B2 – Wastewater Handling | 1.5 | + 17.2 |

Of the total Northern Irish emission of 8.9kt N2O in 2003, around 7.8 kt N2O (88%) was from agriculture. Most of these were emissions from the agricultural soils category as a result of processes in the soil arising from, in order of magnitude:

[Note: numbers in brackets show the percentage of the total agricultural N2O emission (Source: IGER 2005, Personal Communication)]

- leaching of fertiliser nitrogen and applied animal manures to ground and surface water (28%)

- wastes from grazing animals (22%)

- synthetic fertiliser application (20%)

- manure used as fertiliser (14%)

- atmospheric deposition of ammonia (NH3) and oxides of nitrogen (NOx) (7%)

- ploughing in crop residues (1%)

- improved grass (1%)

- histosols (i.e. high organic content soils) (0.2%)

- cultivation of legumes (<0.1%)

- field burning (discontinued in 1993)

A relatively small emission (0.6 kt N2O) comes from the treatment of animal wastes (Manure Management).  Northern Irish agricultural nitrous oxide emissions have fallen by 4% between 1990 and 2003, and in 2003 represent around 9% of UK agricultural emissions.

Fuel combustion contributes 12.4% (1.1 kt N2O) to the overall N2O total in Northern Ireland, with 0.6kt from road transport and the remainder from stationary combustion in other sectors.  Note that emissions of nitrous oxide from road transport have risen by several hundred percent over the period as a result of the increasing use of catalytic converters on motor vehicles.

Wastewater treatment accounts for 1.5% of the total N2O emissions in Northern Ireland in 2003, having increased by 17.2% over 1990-2003.

## Hydrofluorocarbons

Total Northern Irish emissions of HFCs in 2003 were 2.1% of the UK Total (as CO2 equivalent).  The largest source was refrigeration (including air conditioning) contributing 55.7% of the Northern Ireland HFC total due to losses from refrigeration and air conditioning equipment during its manufacture and lifetime. Aerosols contributed 33.2% to the total Northern Irish HFC emission in 2003, the main sources being industrial aerosols and medical use of metered dose inhalers. The remaining emission sources (foams, fire-fighting and solvents) contributed 11.1% of total Northern Irish HFC emissions. The total emission has increased from virtually zero in 1990 to 230kt CO2 equivalent in 2003.

## Perfluorocarbons

There are no estimated emissions of PFCs in Northern Ireland.

## Sulphur Hexafluoride

Northern Ireland SF6 emissions accounted for 0.4% of the UK total in 2003.  The main sources of sulphur hexafluoride emissions are leakage from the electrical switching gear used in electricity transmission and the soles of certain brands of training shoes.  The use of SF6 in the electronics industry in Northern Ireland is negligible.

# Unallocated Emissions

The following emissions sources are not allocated to England, Scotland, Wales and Northern Ireland inventories as they pertain to the UK as a whole and are predominantly offshore emissions:

- offshore oil and gas installations

- fishing

- coastal shipping

- domestic aviation

- naval vessels

- military aircraft

As a proportion of the 2003 UK totals they account for the following:

- Carbon dioxide		4.0%		(up 15% since 1990)

- Methane		2.6%		(down 42% since 1990)

- Nitrous oxide		1.1%		(up 15% since 1990)

There are no unallocated emissions of halocarbons and sulphur hexafluoride.

The breakdown of key source categories for unallocated GHG emissions is as follows:

Table 6.1	Key Categories: Unallocated CO2 Emissions in 2003

| IPCC Sector | % of total 2003 Unallocated CO2 | Change against 1990 baseline (%) |
| --- | --- | --- |
| 1A1cii – Other Energy Industries | 59.9 | + 63.7 |
| 1A4ciii - Fishing | 1.4 | - 34.4 |
| 1A5b – Other mobile | 12.1 | - 47.2 |
| 1B2a – Oil Production | 1.4 | - 58.1 |
| 1B2ciii - Flaring | 15.3 | - 0.4 |

The increases in 1A1cii are due to increased activity in the offshore oil & gas sector and stem primarily from gas separation plant and own gas combustion by offshore operators. Reductions in 1A5b are due to decreases in emissions due to naval activity and military aircraft.

Table 6.2	Key Categories: Unallocated CH4 Emissions in 2003

| IPCC Sector | % of total 2003 Unallocated CH4 | Change against 1990 baseline (%) |
| --- | --- | --- |
| 1A1cii – Other Energy Industries | 13.1 | + 267 |
| 1B2a – Oil Production | 14.8 | - 81.4 |
| 1B2ciii - Flaring | 27.9 | - 28.2 |
| 1B2ciii - Venting | 43.9 | - 12.7 |

The increases in 1A1cii are due to increased activity in the offshore oil & gas sector, from gas separation plant and other sources, whilst industry efforts to reduce the flaring and venting of petroleum gases have led to significant reductions in emissions from those sources.

Table 6.3	Key Categories: Unallocated N2O Emissions in 2003

| IPCC Sector | % of total 2003 Unallocated N2O | Change against 1990 baseline (%) |
| --- | --- | --- |
| 1A1cii – Other Energy Industries | 81.7 | + 43.7 |
| 1A4ciii - Fishing | 1.5 | - 34.4 |
| 1A5b – Other mobile | 7.8 | - 50.3 |
| 1B2ciii - Flaring | 8.0 | - 27.8 |

The increases in 1A1cii are due to increased activity in the offshore oil & gas sector, from gas separation plant and other sources, whilst industry efforts to reduce the flaring of petroleum gases have led to significant reductions in emissions from those sources.

Reductions in 1A5b are due to decreases in emissions due to naval activity and military aircraft.

# Availability of Data & Changes to Inventory Methodologies

In order to estimate a complete greenhouse gas inventory for each constituent country of the UK, it would be necessary to have a complete set of activity data for each country to the same level of detail as that used for the UK Inventory.  A complete set of such data is not available; the precise availability of data across all sub-sectors of the regional inventories is discussed in Appendix 1.

## Availability of Data by Sector

Generally, sufficient country-specific data are available for the following sectors:

- Agriculture (Defra)

- Land Use Change and Forestry (Centre for Ecology and Hydrology)

- Industrial Processes (for most of these, country data are available from producers, trade associations, the Environment Agency's Pollution Inventory, the Scottish Environmental Protection Agency’s EPER inventory and the Northern Ireland Department of Environment’s ISR inventory.)

### Fuel Consumption

The availability of data across this wide-ranging sector of activity is very variable.  The basis for all of the UK NAEI fuel consumption data are the Digest of UK Energy Statistics (DTI, 2004), and this publication does include some regional data such as coal production, domestic gas consumption and consumption of liquid fuels.  The liquid fuel data consist of totals of different types of liquid fuel for Northern Ireland, Scotland and England & Wales combined.

Northern Ireland produces a complete annual set of fuel statistics, though this only gives sectoral consumption for coal and total consumption for oil products.

Up until 1994, the Welsh Office produced a fairly detailed set of fuel statistics based on DTI estimates.  However this has been discontinued since the privatisation of the energy industries, owing to the problems of reporting potentially commercial data.

Scotland does not publish fuel statistics. Limited data on coal production and gas consumption in 1990 has previously been provided and forms the basis of some extrapolated data estimations.

Transco provides gas sales statistics disaggregated by region and consumer size and Phoenix Gas provides data for natural gas consumption in NI disaggregated by type of consumer.

Fuel consumption within the iron & steel industry is well covered by Iron and Steel Industry Statistics (ISSB, 2004) although in the latest editions some of the more detailed regional data are not reported; equivalent data are available on request.  The ISSB data deal with primary iron and steel production but excludes most secondary processes.

Fuel consumption data for power generation is obtained directly from the major power generators and is supported by emissions data within the Pollution Inventory (England & Wales) and point source data obtained directly from SEPA and DoE NI.

Fuel use within the cement & lime industry is estimated from data on plant-specific cement production capacities, provided by the British Cement Association.

Carbon emissions data are available for refineries from UKPIA and the Environment Agency’s Pollution Inventory.

Detailed data are available for the offshore industry from the UKOOA EEMS database which includes installation and process-specific data for 1995, and 1998 to 2003 of varying coverage; earlier years in the UKOOA dataset are more sparsely populated and appear to be of lower consistency across the industry. All 1990 sector splits have been based on extrapolating back the 1995 sector splits.

Hence the main sources where fuel use data have been estimated are:

- Domestic coal & oil

- Miscellaneous/Commercial sector coal & oil

- Agriculture sector coal & oil

- All fuel use within the “Other Manufacturing Industry” sector (excluding cement and autogeneration)

Various surrogates are used to estimate these sources:

- Emissions from commercial and other manufacturing industry are estimated from the Science Policy Research Unit (SPRU) database of boiler capacities, which reports boiler size, fuel type and location for the period 1992 to 94.

- The regional disaggregation of agricultural sector fuel combustion emissions and oil consumption in the commercial and public service sectors are based on employment statistics.

- Limited data on coal consumption have been collected for England and Wales in 1995, and 1998 to 2002 from coal producers.

- For England, Wales and Scotland, domestic oil use is estimated based on population, whilst for Northern Ireland, housing survey data have been used.

## Significant Changes to Inventory Methodology

A number of changes have been made to the estimates since the last study (Baggott et al 2004) due to revisions to:

- carbon, methane and nitrous oxide emission estimates in the UK inventory; and

- disaggregation methodologies to derive DA inventories from the UK data.

The most significant changes are described below.

### Review of UK Carbon Emission Factors

During autumn 2004, a comprehensive review of the timeseries of carbon emission factors (CEFs) from combustion of UK fuels was conducted to improve the accuracy of the UK Greenhouse Gas Inventory prior to the commencement of the Kyoto Protocol First Commitment Period, and to ensure that factors sourced from the inventory for use in the EU Emission Trading Scheme were representative of UK fuels. A range of CEFs were selected for review, based on the contribution of those fuels to the national total carbon emission. The time series of CEFs were reviewed and revised as necessary.  The review considered the following CEFs:

- Aviation spirit		(revised)

- Aviation turbine fuel	(unchanged)

- Coal				(revised)

- Coke				(revised)

- Diesel				(revised)

- Fuel oil			(revised)

- Gas oil			(revised)

- Naptha			(revised)

- Natural gas			(revised)

- Other petroleum gas 	(revised)

- Petcoke			(revised)

- Blast Furnace Gas (BFG)	(revised)

- Coke Oven Gas (COG)	(revised)

- Petrol				(unchanged)

The overall effect of applying the new CEFs was to increase carbon emissions, assuming that the quantities of fuel burnt remained the same.

Coal CEFs pertinent to the Electricity Supply Industry were reviewed and increased after consultation with the power generation sector.  The increase in emission factors reflects the increased use of imported coals burnt in UK power stations, which tend to have higher carbon contents than UK produced coal.  Non-power station CEFs of coal were scaled to Gross Calorific Value (GCV) data from the Digest of UK Energy Statistics (DUKES) 2004 (DTI, 2004).

Natural gas emission factors were reviewed by Transco and updated using data from measurements of the composition of representative samples of gas.

### UK Method Improvement: Land Use, Land Use Change and Forestry (LULUCF)

The method providing estimates of the GHG emissions has been revised by the UK inventory LULUCF experts at the Centre for Ecology and Hydrology in Edinburgh. A summary of the changes is as follows:

- Data on rates of change of land use improved in quality and consistency. Effects of set aside now included with other land use change.

- Data on soil carbon density extensively revised to improve quality and treatment of variation with depth.

- Calculations of uncertainty of estimates of the effect of land use change on soil carbon stocks now take into account uncertainty in land use change rates and soil carbon density as well as uncertainty in rates of change of soil carbon stock where a specific plot of land changes use.

- Estimates of variation in deforestation since 1990 now included.

- Net emissions due to disturbance of soil for afforestation now based on field measurements. Hence removals to soil carbon in conifer forest better estimated.

- Pattern of early growth of forest better modelled.

- Recent forest planting data revised.

The LUCF emissions and removals in the 2002 and 2003 GHG inventories, and the difference in the LUCF emissions between the two inventories are given below. Note that these changes have had the most significant effect in the GHG Inventory for Scotland.

### UK Method Improvement: Aviation

The method used to estimate UK emissions from aviation has been revised, and the new method corresponds to an IPPC Tier 3 approach.  The old methodology overestimated fuel use and hence emissions from domestic aircraft since only two aircraft types were considered, and the default emission factors used related to older models.  It is clear that smaller and more modern aircraft are used on domestic and international routes.  Emissions from international aviation will have been correspondingly underestimated.

In comparison with earlier methods used to estimate emissions from aviation for the greenhouse gas inventory, the approach now used is much more detailed and reflects differences between airports and the aircraft that use them.  Additional emission sources (such as aircraft auxiliary power units) are also now included.

This study has utilised data from a range of airport emission inventories compiled in the last few years by Netcen.  This work includes the RASCO study (23 regional airports, with a 1999 case calculated from CAA movement data) carried out for the Department for Transport (DfT), and the published inventories for Heathrow, Gatwick and Stansted airports, commissioned by BAA and representative of the fleets at those airports in a fairly recent year.  Emissions of NOx and fuel use from the Heathrow inventory have been used to verify the results of this study.

Figure 7.2.2: Effects of UK Method Change to LULUCF Sector

Emissions in the LUCF sector from the 2002 GHG inventory (ktonnes CO2)

|  | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 | 2003 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sum of emissions | 19,606 | 19,794 | 19,432 | 17,943 | 16,929 | 16,474 | 16,840 | 16,589 | 16,814 | 16,558 | 15,292 | 15,090 | 13,585 | n/a |
| Sum of removals | -10,556 | -10,666 | -10,846 | -11,073 | -11,286 | -11,528 | -11,612 | -11,557 | -11,528 | -11,539 | -11,653 | -11,616 | -11,682 | n/a |

Emissions in the LUCF sector from the 2003 GHG inventory (ktonnes CO2)

|  | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 | 2003 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sum of emissions | 17,558 | 17,650 | 17,424 | 16,536 | 16,585 | 16,737 | 16,478 | 16,034 | 15,463 | 15,376 | 15,038 | 14,866 | 14,520 | 14,734 |
| Sum of removals | -14,913 | -15,061 | -15,348 | -15,637 | -15,900 | -15,950 | -15,859 | -15,823 | -15,849 | -15,926 | -15,826 | -15,852 | -16,008 | -16,270 |

Difference: 2003 minus 2002 (ktonnes CO2)

|  | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 | 2003 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | -2,048 | -2,144 | -2,008 | -1,407 | -344 | 263 | -362 | -555 | -1,351 | -1,182 | -254 | -224 | 935 | n/a |

Using the IPCC Tier 3 methodology provides a means of calculating total fuel used directly from aircraft movement data.  The method is likely to lead to a different estimate of the total consumptions of aviation fuels from those presented in DUKES.

(Note that the ATF fuel consumptions presented in DUKES will include use of both civil and military ATF, and the military ATF use must be subtracted from the DUKES total to provide an estimate of the civil consumption).

The fuel consumption data in DUKES is provided by fuel sales return from the UK Petroleum Industries Association (UKPIA). Ideally, the agreement between the two estimates for civil aviation fuel use should be exact, but in practice, there are reasons why this may not be the case.

The estimates of fuel consumed in the commodity balance table in DUKES are the national statistics on fuel consumption, and the IPCC guidance states that national total emissions must be on the basis of fuel sales.  Therefore, the estimates of emissions have been re-normalised based on the results of the comparison.  This re-normalised of emissions will occur each time the aircraft movement data are modified or data for another year added.

In terms of estimates of CO2 emissions, the current approach leads to better estimates of the domestic / international fuel split and correspondingly a better estimate of the split in carbon emissions.  Note that aviation emissions are unallocated and therefore these changes do not directly affect the greenhouse gas inventories for the constituent countries of the UK.

### UK Method improvement: Iron and Steel and Fuel Transformation

The method of estimating emissions from iron and steel (I&S) production and the associated fuel transformation has been improved.  Several points need to be considered as part of this revision:

- The method of estimating emissions has been improved, but it is still based on a carbon balance with the I&S sector.

- There has been a re-allocation of emissions between various IPPC sectors, including the transfer of emissions from benzols and tars (produced in the fuel transformation process) to the waste sector

- The CEF review has affected the emissions from both the I&S and fuel transformation sectors.

The new method updates the existing approach and is still based on a carbon balance for integrated steelwork processes in the UK, including the associated fuel transformation.  There has been a re-allocation of certain fuels which affects the emissions in IPCC sectors 1A2a, 1A2f and 2C1.  The implementation of the new method has essentially re-allocated emissions and neither increased nor decreased emissions (within a margin of error of approximately 10 to 20 ktonnes). The method has removed the negative carbon emission factors that were used as artificial devices as part of the previous I&S carbon balance, and that had been criticised following external review of the GHG inventory.

Benzols and tars are by-products of the fuel transformation process and are produced during the transformation of coal to coke. In previous inventories, emissions from the combustion of these compounds was included as part of the emission from coal used in coke ovens.  However these compounds are not burnt in this sector, and are used mainly as wood preservatives.  Emissions from the decomposition of these fossil fuel derived preservatives have now been allocated to a waste sector – IPCC sector 6A1. Ongoing discussions with Corus aim to identify more precisely the final utilisation of these by-products.

The CEF revision of the GHG inventory has affected the estimates of emissions from the I&S sector. Emissions from the associated fuel transformation have been affected also.  Revised carbon factors have been used for blast furnace gas (BFG) and coke oven gas (COG).  These data were provided indirectly by Corus and were been judged to be more reliable than those previously used in the GHGI.  The revised factor for BFG is higher than the previous value, while the new factor for COG is lower than the previous value. Discussions with Corus are ongoing to further refine these emission factors, in light of greater availability of measured data.

### UK Method Improvement: Oil and Gas Industry

The UK Offshore Oil and Gas Association (UKOOA) provides emissions of carbon from the activities of the onshore and offshore oil and gas industry via their EEMS database of installation and process-specific emissions. In 2004-5 UKOOA revised the format and content of the emissions data, with some revisions to the format and values of historic emissions data of carbon, following studies to derive a sector-wide National Allocation Plan (NAP) for emissions trading.

For combustion processes over 20MW capacity (offshore & onshore), UKOOA have submitted revised data for CO2 emissions for 1998 to 2002 inclusive, to agree with their NAP agreements.

The format of the data supplied has been changed; previously combustion emissions were allocated according to fuel (diesel combustion, gas combustion, fuel oil combustion), but the new data are provided according to plant thermal capacity (combustion < 20 MW, combustion > 20 MW). The pre-1995 UKOOA data are split into fewer categories compared to data in 1995 and later.  Therefore, to produce a consistent time series of data, we have used the split in 1995 (the earliest year with detailed activity data) and applied this to activity data from 1994 back to 1990.

This has produced an improvement in time-series consistency for some offshore oil & gas source sectors such as flaring and venting, with some re-allocations of emissions from “Unallocated” to individual countries. The work to improve the UKOOA dataset is ongoing, as further inconsistencies in the reported dataset have been identified as a result of the recent focus of work in this sector.

### UK Method Improvement: Waste Lubricant Combustion

Assumptions about the percentage of waste oils and lubricants that are disposed by combustion have been revised. Previously it was estimated that around 40% of waste lubricants was burnt across the inventory time-series. From a review of recent studies into waste oil treatment and disposal options it is apparent that a much greater percentage is in fact burnt, and a time-series of estimates (varying but approximately 80%) has now been applied.

Emissions of lubricants consumed in vehicle engines are now included in GHG inventory, and this has significantly increased the emissions of carbon reported in IPCC sector 1A3b Road Transportation. There is currently limited information about where the emissions of this lubricating oil actually occur, and the current view is that these emissions are best assigned to the road transport sector.  This assumption is subject to ongoing review.

### New UK Source: CO2 Emissions from Flue Gas Desulphurisation in the Power Generation Sector

Carbon dioxide is released when gypsum is produced by FGD units fitted to power stations in the UK. In previous GHG inventories, emissions of CO2 from FGD units were not included in the inventory as estimates suggested this source would be small  (~20 kt).  Until recently, only Drax and Ratcliffe had FGD units fitted.  FGD has now been introduced (or is currently being fitted) at other power stations (Cottam, Eggborough and West Burton) and so an estimate of the emissions of CO2 from FGD has now been introduced into the inventory to prevent under-reporting of the national carbon totals.

The emissions of CO2 from this source are reported under IPCC category 2A3.  (Note – all this activity is currently in England.)

### UK Data Revisions: Methane & Nitrous Oxide Emissions

Several of the methodology changes indicated above for carbon have also impacted upon methane and nitrous oxide estimations for the UK inventory in 2003, including changes to:

- the iron & steel sector

- the offshore oil & gas sector – revision of UKOOA dataset

- the civil aviation sector

- the NAEI road transport model and combustion of lubricating oils

In addition, there have been a range of emission factor revisions that have impacted upon the UK estimates for methane and nitrous oxide:

Methane emission factor revisions have impacted upon estimates from:

- Offshore oil & gas (many sub-sectors)

- Civil aviation

- Road Transport

- Railways

- Agricultural livestock - Dairy cattle enteric fermentation

- Agricultural livestock - Dairy cattle wastes

Nitrous Oxide emission factor revisions have impacted upon estimates from:

- Offshore oil & gas (many sub-sectors)

- Civil aviation

- Road Transport

- Agricultural Soils - use of fertiliser

### Revisions to Regional Disaggregation Methodologies

A number of methods have been revised regarding the split of UK data to produce GHG inventories for England, Scotland, Wales and Northern Ireland. We have an ongoing process of improvement in this regard and increasingly are working towards harmonisation of our approach with other inventory products such as mapping grids and local inventory models.

The key changes in the latest inventory compilation are:

- Road Transport: The use of vehicle kilometre data to provide the split across the constituent countries was introduced in the 1990-2002 inventory, but inconsistencies in the Northern Ireland data were identified. These have been partially resolved through agreement with the NI DoE. The data used within the finalised inventories is consistent with the UK emissions totals, which are determined (in accordance with IPCC requirements) using fuel sales data. An alternative method whereby the DA inventory data use simply a bottom-up approach from the vehicle kilometre data has been presented separately. This produces an inconsistency with the UK totals, but provides a more consistent trend in road transport emissions across the time-series for the DAs. See Appendix 1, section 1.4 of this report for more details.

- Offshore Oil & Gas: The revisions of the UKOOA dataset have led to a review of specific emission sources, leading to some alterations to allocations from Unallocated to the constituent countries for sector 1B2a.

- National Navigation: Previously this sector (1A3d) was disaggregated using activity data from the oil & gas industry to provide an indication of coastal shipping activity. This was identified as a poor choice of parameter which introduced a bias towards Scotland in particular and has therefore been replaced by the use of port movement data.

# Uncertainty in the Inventories

A study (Eggleston et al, 1998) estimated the uncertainty in the UK Inventory.  These estimates have been revised to account for changes in the 2003 national inventory (Baggott et al, 2005) and are given in Table 9.1

As a result of the activity data gaps in the Devolved Administration inventories, the estimates will be more uncertain than for the UK inventory. The uncertainties in the emission totals have been estimated using a Monte Carlo simulation.  It is difficult to estimate the uncertainties in some of the activity data used in the DA-level inventories due to the data gaps, since it is unknown how closely the surrogate data reflect actual fuel consumption.  Hence, in the simulation it has been necessary to make fairly speculative assumptions on the uncertainties in the country-specific activity data. The approach adopted is discussed in Appendix 1. The uncertainty estimates are reported in Table 9.1. The N2O distribution is heavily skewed, so that 2.5% and 97.5% confidence limits are quoted.

- Table 9.1	Estimated Uncertainties1 in the DA Inventories in 2003

| GHG |  | England | Scotland | Wales | N Ireland | UK |
| --- | --- | --- | --- | --- | --- | --- |
| CO2 |  % | 2.3 | 6.6 | 2.5 | 3.0 | 2.2 |
| Methane |  % | 14.8 | 14.6 | 14.5 | 16.4 | 13.0 |
| N2O | Lower kt Upper kt | 20 321 | 2 72 | 2 47 | 1 40 | 27 482 |
| HFC |  % | 24 | 21 | 21 | 21 | 21 |
| PFC |  % | 22 | 13 | 13 | Note 2 | 13 |
| SF6 |  % | 18 | 16 | 16 | 16 | 16 |
| Total |  % | 12 | 23 | 19 | 38 | 14 |

Notes

- Uncertainty is defined as  2(standard deviation)/mean %, which closely approximates the 95% confidence interval

- Emissions of PFC in Northern Ireland are zero.

The relatively high uncertainties in the Scottish CO2 and GWP inventories reflect the large contribution made by Land Use, Land Use Change & Forestry (LULUCF) to the Scottish CO2 inventory. The high uncertainty in the GWP inventory for Northern Ireland and Wales is a consequence of the relatively large contributions of methane and agricultural N2O.  The GWP inventory for England has lower uncertainty as a consequence of the relatively low contributions to the English inventories from high uncertainty sources such as LULUCF and agricultural N2O.

# Summary Tables

Detailed summaries of the greenhouse gas emissions for the years 1990, 1995, and 1998 to 2003 for the UK and the Devolved Administration are shown in Tables 9.1 to 9.3.  They are reported as GWP-weighted equivalent mass of carbon (Table 9.1); GWP-weighted equivalent mass of CO2 (Table 9.2) and the mass of each gas emitted (Table 9.3).

Note that all of the CO2 data are based on the total emissions of CO2, excluding removals of CO2 in Land Use, Land Use Change and Forestry sectors.

The summary data and time-series trends presented in these summary tables are also presented in more detailed country-specific tables in Appendix 3, including a breakdown of total greenhouse gas emissions by the following IPCC Source Categories:

- Energy

- Industrial Processes

- Agriculture

- Land Use Change & Forestry (Emissions)

- Land Use Change & Forestry (Removals)

- Waste

Figures 1 to 6 illustrate trends in emissions of greenhouse gases, by sector, over 1990-2003.

Table 9.1	Summary of Greenhouse Gas (GHG) Emissions as GWP-Weighted  Equivalent Mass of Carbon (MtC)

Table 9.2	Summary of Greenhouse Gas Emissions as GWP-Weighted Equivalent Mass of CO2 (Mt)

Table 9.3		Summary of Greenhouse Gas Emissions (Mass of Gas)

# References

Adger, N, Subak, S. (1995) “Carbon fluxes resulting from Land Use Change: Land Use Data and Policy.”  In: Carbon Sequestration in Vegetation and Soils (Ed. by MGR Cannell), DOE/ITE Contract EPG 1/1/3. Interim Report March 1995. Department of Environment, London.

- AES Drax (2004), Power station fuel consumption, Personal Communication.

- Alcan (1998, 2000, 2002, 2004), Personal Communication.

Baggott, S. L., Brown, L., Milne, R., Murrells, T.P., Passant, N. and Watterson, J.D. (2005) “UK Greenhouse Gas Inventory, 1990 to 2003 Annual Report for submission under Framework Convention on Climate Change.” National Environmental Technology Centre, AEA Technology, April 2005.

Baggott SL, Milne R, Brown L, Murrells TP, Thistlethwaite G, Watterson J (2004),  “Greenhouse Gas Inventories for England, Scotland, Wales and Northern Ireland: 1990-2002.”

Barlow, TJ, Hickman, AJ, Boulter, P, “Exhaust Emission Factors 2001: Database and Emission Factors”, TRL Report PR/SE/230/00, September 2001

Bell, DM (2004), Power Station Fuel Consumption, Personal Communication, Northern Ireland Department of Environment.

BGS (1991, 1996, 2004), United Kingdom Minerals Yearbook, British Geological Survey, Natural Environment Research Council.

- BP (2004), Grangemouth Refinery, Personal Communication.

Bradley, I (1997) “Carbon loss from drained lowland fens.” In: Carbon Sequestration in Vegetation and Soils (Ed. by MGR Cannell), DOE/ITE Contract EPG 1/1/3. Final Report March 1997. Department of Environment, London.

- British Cement Association (2004), Personal Communication.

British Glass (2001), Emissions from the UK Glass Industry. Report prepared for NETCEN.

British Glass (2004), Production Statistics for UK Glass Industry, Personal Communication.

Brown, KA, Smith, A, Burnley, SJ, Campbell, DJV, King, K, Milton, MJT (1999) “Methane Emissions from UK Landfills”, AEA Technology, AEAT-20624, Culham

BSFP (1991, 1996, 2004), British Survey of Fertiliser Practice for 1990, 1995, 1998-2003.

Cannell, MGR, Dewar, RC, Pyatt, DG, (1993) “Conifer plantations on drained peatland in Britain: a net gain or loss of carbon?” In: Forestry 66, 353-368.

Cannell, MGR, Milne, R, Hargreaves, KJ, Brown, TAW, Cruickshank, MM, Bradley, RI, Spencer, T, Hope, D, Billett, MF, Adger, WN & Subak, S (1999) “National inventories of terrestrial carbon sources and sinks: the UK experience.” In: Climatic Change 42, 505-530

- Coal Authority (2004), Regional Coal Production, Personal Communication.

Cruickshank, MM, Tomlinson, RW (1997) “Carbon loss from UK peatlands for fuel and horticulture.” In: Carbon Sequestration in Vegetation and Soils (Ed. by MGR Cannell), DOE, Contract EPG 1/1/3. Final Report March 1997. Department of Environment, London.

Cruickshank, MM, and Tomlinson, RW, (2000) “Change in soil carbon storage in Northern Ireland: estimated by the IPCC default and matrix methods.” In.: Carbon Sequestration in Vegetation and Soils (Ed. by R. Milne), DETR Contract EPG 1/1/39. Final Report April 2000. (Available at http://www.nbu.ac.uk/ukcarbon/)

DANI (1991, 1996, 1999), The Agricultural Census in Northern Ireland: Results for June 1990, 1995, 1998.

DARDNI (2000, 2001), The Agricultural Census in Northern Ireland: Results for June 1999, 2000.

DEFRA (2003a), Sewage Sludge Disposal Tables produced for Digest of Environmental Statistics.

DEFRA (2001) June Agricultural Census for 2000.  Department for Environment, Food and Rural Affairs.

DEFRA (2004), Agricultural Census Statistics for UK, Annual Reference Tables, Labour Force. DEFRA Agricultural Statistics website.

- DETR (1997), Digest of Environmental Statistics, The Stationary Office.

DfT (2004a) “Vehicle Licensing Statistics: 2003 data” Transport Statistics Bulletin

DfT (2004b) “Transport Statistics Great Britain: 2004 edition”, The Stationary Office

DfT (2004c) “Road Traffic Statistics: 2003 data” Transport Statistics Bulletin

- DLTR (2004), Aircraft Movement data, Personal communication.

- DOE, (1993), UK Sewage Sludge Survey, CES Limited, HMSO

DoRDNI (2004a) “Northern Ireland Transport Statistics 2003”, Central Statistics and Research Branch, Department of Regional Development in Northern Ireland. (Available at http://csrb.drdni.gov.uk/statistics/statistics.asp)

DoRDNI (2004b), “Traffic and Travel Information Incorporating the Annual Traffic Census and Vehicle Kilometres of Travel”, Department of Regional Development in Northern Ireland

- DoT (1996, 2004) Transport Statistics communication, Department of Transport

DTI (2004), “Digest of UK Energy Statistics” Department of Trade and Industry, HMSO.

DTI (2001a), Development of UK Oil and Gas Resources, Department of Trade and Industry, The Stationary Office.

- DTI (2004b), Personal Communications, Department of Trade and Industry.

EFW (2001), Energy for Waste, Current and Projected Energy for Waste Capacity in UK as at December 2000.

Eggleston, HS, Salway, AG, Charles, D, Jones, BMR, Milne, R (1998), “Treatment of Uncertainties for National Estimates of Greenhouse Gas Emissions”, National Environmental Technology Centre, AEA Technology, Report AEAT - 2688.

Electricity Association (2004), Electricity transmission data, Personal Communication.

Environment Agency (1999), UK Sewage Sludge Survey, Regional Presentation, WRc, R&D Project Record P2/065/1

Environment Agency (1999a, 2000-2004), database of emissions to atmosphere from authorised processes in England & Wales, as reported to the Pollution Inventory (data for 1998-2003).

European Environment Agency (2000), “COPERT III: Computer Programme to Calculate Emissions from Road Transport - Methodology and Emission Factors (Version 2.1)”, L. Ntziachristos and Z Samaras, European Topic Centre on Air Emissions, European Environment Agency, November 2000

Hargreaves, KJ, Milne, R and Cannell, MGR, (2003). “Carbon balance of afforested peatland in Scotland” In: Forestry 76, 299 - 317

Haydock et al (2003) “Emissions & Projections of HFCs, PFCs and SF6 for the UK and Constituent Countries”, AEA Technology.

Hobson, J, Palfrey, R, Sivil, D, Palfrey, E, Day, M, (1996) “Control Measures to Limit CH4 Emissions from Sewage and Sludge Treatment and Disposal”, WRc, Report No DOE 4118.

IPCC, (1997a), IPCC Revised 1996 Guidelines for National Greenhouse Gas Inventories, Volume 1, Greenhouse Gas Inventory Reporting Instructions, IPCC WGI Technical Support Unit, Hadley Centre, Meteorological Office, Bracknell, UK.

IPCC, (1997b), IPCC Revised 1996 Guidelines for National Greenhouse Gas Inventories, Volume 2, Greenhouse Gas Inventory Workbook, IPCC WGI Technical Support Unit, Hadley Centre, Meteorological Office, Bracknell, UK.

IPCC, (1997c), IPCC Revised 1996 Guidelines for National Greenhouse Gas Inventories, Volume 3, Greenhouse Gas Inventory Reference Manual, IPCC WGI Technical Support Unit, Hadley Centre, Meteorological Office, Bracknell, UK.

IPCC, (1996), Climate Change 1995. The Science of Climate Change. Contribution of Working Group 1 to the Second Assessment Report of the Intergovernmental Panel on Climate Change. Ed. Houghton, JT, Cambridge University Press.

ISSB, (2004) Iron & Steel Industry Annual Statistics for the UK.

Levy, P and Milne, R, (2003) Deforestation rates in the United Kingdom. In: UK Emissions by Sources and Removals by Sinks due to Land Use, Land Use Change and Forestry Activities. Annual report (2003) for DEFRA Contract EPG1/1/160 (Ed. by R. Milne). (Available at http://www.nbu.ac.uk/ukcarbon/ )

Levy, P.E. and Milne, R. (2004) Estimation of deforestation rates in Great Britain. Forestry, 77, 9 –16.

LQM (2003) Methane emissions from landfill sites in the UK.  Final report.  January 2003.  Report for the UK Department for Environment, Food and Rural Affairs.  Authors: Gregory, R.G., Gillett, A.G., Bradley, D.  LQM report 443/1.  DEFRA contract EPG 1/1/145.

- LRC (1999), Personal Communication from L Saddler, London Research Centre

MAFF (1991, 1996,1999, 2000) June Agricultural Census for 1990, 1995, 1998 and 1999. Ministry of Agriculture, Fisheries and Food

MCG, (1999), UK Emissions of HFCs, PFCs and SF6 Potential Emission Reduction Options.  A study for the Department of the Environment carried out by March Consulting Group. Telegraphic House, Waterfront Quay, Salford Quays, Manchester

Milne, R and Brown, TA (1997) Carbon in the vegetation and soils of Great Britain. Journal of Environmental Management, 49, 413 - 433.

Milne, R., Tomlinson, RW, Mobbs, D. and Murray, TD,  (2004) Land Use Change and Forestry: The 2002 UK Greenhouse Gas Inventory and projections to 2020.  In: UK Emissions by Sources and Removals by Sinks due to Land Use, Land Use Change and Forestry Activities. Annual report (2004) for DEFRA Contract CEPG1/GA01054 (Ed. by R. Milne & D. Mobbs). (Available at http://www.nbu.ac.uk/ukcarbon/ )

Milne, R, Brown, TAW and Murray, TD (1998) The effect of geographical variation in planting rate on the uptake of carbon by new forests of Great Britain. Forestry, 71, 298 – 309.

- NIO (1996, 2001, 2002, 2003, 2004), Northern Ireland Abstract of Statistics

Office of The Deputy Prime Minister (2004) Land Use Change Statistics Index.  http://www.planning.odpm.gov.uk/luc/lucindex.htm

- ONS (2004), Datasets available pertaining to a wide range of industrial and population-related activities via the publications “Annual Abstract of Statistics” and “Regional Trends”, Office for National Statistics.

- ONS (2000a), IDBR, Employment Database

Patel, NM (2000), Personal Communication, Strategic Consultancy, AEA Technology

Phoenix Natural Gas (2004), Power station fuel consumption, Personal Communication

RCEP (1993), Royal Commission on Environmental Pollution, 17th Report, Incineration of Waste, HMSO, London

- RWE Innogy (2004), Power station annual fuel consumption, Personal Communication.

Sage, PW, (2001), Methane from Abandoned Coal Mines in the UK, AEA Technology, Report AEAT/ENV/R/0500, Harwell

Scottish Executive Rural Affairs Department (2000).  Economic Report on Scottish Agriculture: 2000 edition.

Scottish Office (1991, 1996, 1999, 2000).  Economic Report on Scottish Agriculture 1990, 1995, 1998, 1999.

Scottish Power plc (2004), annual Power Station Fuel Consumption, Personal Communication

Scottish and Southern Energy plc. (2004). Power Station Fuel Consumption, Personal Communication.

- SEPA (2002), Scottish Waste Data Digest 2001

- SEPA (2004), Annual atmospheric emissions data for authorised processes in Scotland, from SEPA’s Pollution Inventory, Personal Communication.

- SMMT (2002), Regional car sales, Personal Communication.

- SO (1996), Economic Report on Scottish Agriculture, 1996

- SO, (1999), Scottish Energy Statistics. Paper supplied by G Winter, Scottish Office.

Transco / National Grid Transco (2004) Historic Gas Demands by Load Category, Personal Communication.

Translink (2004), Annual fuel consumption, Personal Communication.

UKOOA, (1999), Personal communication from P Russell, UK Offshore Operators Association Limited (Environment Committee)

UKOOA (2004), Environmental Database for annual Emissions and Discharges from Offshore Installations, from the EEMS reporting inventory.

UKPIA (2004), Personal Communication, United Kingdom Petroleum Industry Association Ltd

Williams, A, (1993), Methane Emissions, Watt Committee Report Number 28, The Watt Committee on Energy, London.

WO, (1998), Digest of Welsh Historical Statistics 1974-1996, The Welsh Office

# Acknowledgements

The Netcen Greenhouse Gas Inventory team are very grateful for the help received from a number of people during this study, most notably:-

- Jim Penman, Susan Donaldson & Jo Halliday of Defra, Global Atmosphere Division.

- Philip Andrews & Phil Goodliffe of Defra, Sustainable Agriculture Strategy Division.

- Duncan Beamish, Brian Winning, Guy Winter & Alistair Montgomery of the Scottish Executive.

- Barry McAuley, Hazel Campbell & Rodger Lightbody of the Northern Ireland Department of Environment.

- Havard Prosser, Rhodri Griffiths & Barry Dare of the Welsh Assembly.

- Geoff Salway of RWE Innogy.

- The DTI Energy Statistics team (especially Martin Young, Julian Prime, Mike Janes, James Achur, Clive Evans & Ian Corrie).

- Ronnie Milne, Deena Mobbs and Amanda Thomson of the Centre for Ecology & Hydrology.

- Lorna Brown of the Institute of Grassland & Environmental Research.

Appendices

CONTENTS

| Appendix 1 | Estimation Methodology |
| --- | --- |
| Appendix 2 | Greenhouse Gas Inventories for England, Scotland, Wales and Northern Ireland, 1990-2003, presented in Common Reporting Format |
| Appendix 3 | Summary Tables of Greenhouse Gas Emission Trends for England, Scotland, Wales and Northern Ireland, 1990-2003 |

Percentages shown are the country-specific contributions to the UK total of GHG emissions, excluding removals; the data are NOT representative of NET GHG emissions.

The percentages presented in these figures are rounded to one decimal place, but are calculated from emission estimates calculated at full precision.

Base years for UK Greenhouse Gas Emissions are: 1990 for carbon dioxide, methane and nitrous oxide, 1995 for the fluorinated gases.

One Gigagramme (Gg) equals one thousand tonnes, or one kt

Percentages are quoted a s a proportion of the total emissions, excluding removals; the data are NOT percentages of the NET emissions total for each country.

Where population percentages are quoted throughout this report, they are taken from ONS data for 2003.

Data pertaining to agriculture emissions are provided by the Institute of Grassland and Environmental Research.

Percentages are quoted a s a proportion of the total emissions, excluding removals; the data are NOT percentages of the NET emissions total for each country.

Category 5A2 is a SINK, not a SOURCE of CO2 emissions, and therefore the +33.4 % figure represents an increase in the store of carbon rather than an increase in an emission of CO2.

Percentages are quoted a s a proportion of the total emissions, excluding removals; the data are NOT percentages of the NET emissions total for each country.

Category 5A2 is a SINK, not a SOURCE of CO2 emissions, and therefore the +4.2 % figure represents an increase in the store of carbon rather than an increase in an emission of CO2.

Percentages are quoted a s a proportion of the total emissions, excluding removals; the data are NOT percentages of the NET emissions total for each country.

The inventory methodology for determining emissions from iron & steel plant has to account for the fuel transformation processes that occur within an integrated steelworks, whereby primary fuels are inputted (coal, fuel oil etc) but converted to secondary fuels (coke, coke oven gas, blast furnace gas) and then burnt. Therefore a carbon-balance approach is used to ensure that the inputs and outputs balance and to avoid any loss or double-counting of carbon in the system.

The upper and lower estimates do not lie at an equal distance from the mean and therefore these limits are given separately for N2O as it is inappropriate to quote a single % figure.

GWP = Global Warming Potential. See Table 1.1 for details.