# Base Cations

## Introduction

Base cation emission estimates for the UK are presented in this chapter. The emission estimates cover the period 1990-2003 for Calcium (Ca), Magnesium (Mg), Sodium (Na) and Potassium (K). These estimates are highly uncertain.

A base cation is essentially a positively charged ion from group 1 or 2 of the periodic table (the alkali metals or alkaline earth metals). The most environmentally abundant of these are Na, K, Ca and Mg. Base cations are important in the environment for two reasons.

- First, their deposition has an impact on the surface soil pH. The deposition of base cations increases the alkalinity of the surface; the effect in the environment is to buffer or neutralise the effects of the acidity generated by S and N deposition (which in their mobile anionic form, SO42- and NO3-, leach Ca and Mg from the soil). Therefore one of the primary uses of these emission estimates is to use them to generate spatially resolved emission maps, which enable deposition maps to be calculated.

- Secondly, their emissions to air have an impact on atmospheric chemistry. It is important to understands the spatial distribution of emissions so that any impact on atmospheric chemistry , and resulting changes to pollutant transport can be accounted for in atmospheric chemistry and transport models.

## Background

A “critical load” approach is often taken to predict the maximum levels of acidity or alkalinity that an ecosystem can tolerate.  The base cations (Na+, K+, Ca2+, Mg2+) are known to be present in ambient air and in precipitation.  The deposition of these base cations to ecosystems will offset to some extent the acidification resulting from the deposition of oxides of sulphur, oxides of nitrogen and NH3.

The Review Group on Acid Rain (1997) reported on the decline in base cation deposition that has been observed in Europe and N America since the early 1970’s and how such a decline may offset some of the benefits of reductions in SO2 emissions.  Interest in the deposition and acid neutralising effects of base cations is mainly confined to Ca, K and Mg.  It has long been assumed that the major source of these base cations in air is dust from soil erosion, but patterns of concentrations in air and precipitation also suggest significant emissions from urban and industrial sources.  The concentrations of Ca, K and Mg in air and in precipitation measured at three rural sites in the UK declined dramatically between 1970 and 1991 (Lee and Pacyna, 1999).  It has been suggested that the decrease in base cation deposition which has been observed is due to the reduction in emissions from urban and industrial sources.  Concentrations of Na in air and rain have shown much smaller decreases over this period, consistent with its mainly marine origin as sea-salt.

The NAEI has estimated emissions from the following sources:

- Stationary combustion of fossil fuels: mainly in the fly ash from solid fuel combustion

- Mineral extraction processes: e.g. limestone quarrying

- Processes in the mineral products industry: e.g. cement manufacture and concrete batching

- Industrial processes using limestone, dolomite and soda lime:

- iron and steel manufacture

- glass manufacture

- Agricultural use: e.g. liming of soils and dust due to cultivation.

- Construction and demolition activities

- Mobile sources: mostly in the form of dust resuspension by traffic and exhaust emissions of potassium from lead replacement petrol (LRP).

There are likely to be base cation emissions from other sources, for example incineration.  Currently, these are not included in the estimates as such sources are likely to be much smaller than the sources listed above, and the levels of uncertainty in estimating the above sources do not yet warrant investigation of other sources.

## Stationary combustion of fossil fuels

The base cations emitted from stationary combustion arise from the trace concentrations of the cations found in the fuels.  The base cations will enter the atmosphere contained in the primary particulate matter (PM) which is emitted from the combustion source.  Calcium has been found in large amounts in the fine particle size fraction collected from combustion sources.

The NAEI currently estimates PM10 emissions from large combustion plant for power generation using total PM emissions data submitted by the operators to the Environment Agency and the Scottish Environmental Protection Agency.  Where reported data are incomplete, PM emission factors for the appropriate fuel are derived and combined with the amount of fuel used by the combustion plant to estimate the total mass of PM emitted.

The mass content of cations in coal has been estimated from the Turner-Fairbank Highway Research Centre (US Transport Department) figures for fly ash from bituminous coal.  Data regarding the composition of fuel oil is given in the Marine Exhaust Research Programme.

## Mineral extraction processes

Limestone quarrying is a major source of atmospheric emissions of base cations, principally calcium.  Quarrying of dolomite (CaCO3 MgCO3), rock salt (NaCl) and potash (KCl) are the principle sources of magnesium, sodium and potassium respectively.

The NAEI currently estimates PM10 emissions from quarrying using USEPA emission factors combined with UK mineral statistics on the production of each type of aggregate.  The dust emitted from limestone quarrying will be mainly particles of limestone (CaCO3) itself.   These particulates will be mainly in the coarse particle size range (>2.5 m) and will be deposited close to their source.  The quantities of these minerals extracted in the UK are given in the Minerals Yearbook (2002).

## Processes in the mineral products industry

Emissions of calcium from the mineral products industry are estimated from total PM10 emissions using emission factors from Lee and Pacyna (1999) or AEAT estimates of PM10 composition.

## Industrial processes using limestone, dolomite and soda ash

Processes involving limestone, dolomite and soda ash include iron and steel production and glass manufacturing.  Emissions of base cations from the iron and steel industry and the glass industry are based on the PM10 inventory combined with emission factors for cations taken from Lee and Pacyna (1999) or based on AEAT estimates of PM10 composition.

## Soil liming and cultivation in agriculture

The practice of soil liming in agriculture will lead to the emission of Ca when the lime is applied to the ground.  Statistics are available on the quantity of limestone used each year for liming (UK Minerals Yearbook 1990-2002) and an emission is estimated using an emission factor for non-metallic particles given by the USEPA. The average quantities of re-suspended dust, as a result of land cultivation, may be estimated from data reported in the MAFF Report CSG 15 (2000).  Emissions are estimated from the average chemical abundance of each cation in UK soil (Lindsay, 1979).

## Construction activities

The NAEI currently uses a USEPA emission factor combined with UK construction activity statistics to estimate fugitive emissions of PM10 from these processes.  A modified PM10 emission factor based on the fraction of total aggregate used in construction (UK Minerals Yearbook 1990-2002) that is limestone, dolomite or chalk, is used to estimate the base cation emissions.

## Mobile sources

Emissions of base cations from mobile sources will mainly arise from the resuspension of road dust by traffic.  Recently, Nicholson (2000) has made an estimate of the total PM10 emission from UK roads.  Using this information with data on the average chemical composition of road dust (Sloss and Smith, 2000) Na, K and Ca emissions have been estimated.  There are insignificant quantities of Mg in road dust.

Potassium compounds are the primary additives in Lead Replacement Petrol (LRP).  LRP has been available since Autumn 1999 and is the main source of potassium emissions from vehicle exhausts.  Emissions have been estimated from UK LRP sales in 1999 (calculated as a fraction of leaded petrol sales) to 2003 given by the Digest of United Kingdom Energy Statistics (DTI, 2004).

## Calcium

Combustion in industry and production processes contribute the most emissions of calcium. Within the latter sector quarrying, cement production and lime production are tha most significant contributors. However, more recently the introduction of abatement measures have ensured reductions in the emissions.

Table 7.1 UK Emissions of Calcium by UN/ECE Source Category (ktonnes)

|  | 1990 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 | 2003 | 2003% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BY UN/ECE CATEGORY |  |  |  |  |  |  |  |  |  |  |  |
| Combustion in Energy Prod |  |  |  |  |  |  |  |  |  |  |  |
| Public Power | 3.2 | 1.7 | 1.5 | 1.0 | 1.1 | 0.8 | 0.9 | 0.7 | 0.4 | 0.4 | 13.6% |
| Petroleum Refining Plants | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.2% |
| Other Combustion & Trans. | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0% |
| Combustion in Comm./Inst/ Res |  |  |  |  |  |  |  |  |  |  |  |
| Comm., Public & Agriculture | 0.2 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.6% |
| Residential Plants | 1.4 | 0.6 | 0.7 | 0.7 | 0.7 | 0.8 | 0.5 | 0.6 | 0.5 | 0.2 | 8.2% |
| Combustion in Industry | 2.7 | 2.5 | 2.5 | 2.6 | 2.6 | 2.0 | 1.7 | 1.6 | 1.4 | 1.3 | 46.9% |
| Production Processes | 1.1 | 1.0 | 1.0 | 1.0 | 1.0 | 0.9 | 0.9 | 0.8 | 0.7 | 0.8 | 28.9% |
| Shipping | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0% |
| Non livestock agriculture | 0.3 | 0.1 | 0.1 | 0.2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 1.2% |
| TOTAL | 12.6 | 9.6 | 9.4 | 9.1 | 9.0 | 7.6 | 6.6 | 6.1 | 5.2 | 4.9 | 100% |

Figure 7.1 Time Series of Calcium Emissions (tonnes)

## magnesium

The largest single source of magnesium emissions is from the quarrying of dolomite, used as an aggregate. This emission falls within the production processes sector. Quarrying in general is a significant source. Domestic coal burning is responsible for 62 tonnes, and coal burning power stations released 122 tonnes.

Estimates of emissions of magnesium from coal combustion at Alcan’s Ashington power station have been revised downwards.  Estimates for slag cement grinding, fireworks and burning of waste lubricants have all been included in the emission estimates.

Table 7.2 UK Emissions of Magnesium by UN/ECE Source Category (ktonnes)

|  | 1990 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 | 2003 | 2003% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BY UN/ECE CATEGORY |  |  |  |  |  |  |  |  |  |  |  |
| Combustion in Energy Prod |  |  |  |  |  |  |  |  |  |  |  |
| Public Power | 1.4 | 0.54 | 0.50 | 0.34 | 0.35 | 0.27 | 0.29 | 0.24 | 0.12 | 0.12 | 21.6% |
| Other Combustion & Trans. | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0% |
| Combustion in Comm./Inst/ Res |  |  |  |  |  |  |  |  |  |  |  |
| Residential Plant | 0.06 | 0.02 | 0.03 | 0.03 | 0.02 | 0.02 | 0.01 | 0.01 | 0.01 | 0.00 | 12.9% |
| Comm., Public & Agriculture | 0.47 | 0.21 | 0.22 | 0.21 | 0.22 | 0.25 | 0.16 | 0.20 | 0.16 | 0.07 | 0.8% |
| Combustion in Industry | 0.23 | 0.22 | 0.21 | 0.20 | 0.19 | 0.18 | 0.14 | 0.16 | 0.15 | 0.13 | 22.5% |
| Production Processes | 0.34 | 0.19 | 0.18 | 0.17 | 0.14 | 0.14 | 0.16 | 0.15 | 0.14 | 0.15 | 26.5% |
| Non livestock agriculture | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 2.1% |
| TOTAL | 2.2 | 1.3 | 1.2 | 1.0 | 1.0 | 1.0 | 0.9 | 0.8 | 0.7 | 0.6 | 100% |

Figure 7.2 Time Series of Magnesium Emissions (tonnes)

## sodium

Iron and steel production from sinter plants is responsible for the greatest single emission of sodium with 253 tonnes in 2003. Domestic coal burning emissions contribute 61 tonnes and coal burning power stations 119 tonnes. Other sources include glass manufacture and the food manufacture/preparation sectors.

Table 7.3 UK Emissions of Sodium by UN/ECE Source Category (ktonnes)

|  | 1990 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 | 2003 | 2003% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BY UN/ECE CATEGORY |  |  |  |  |  |  |  |  |  |  |  |
| Combustion in Energy Prod |  |  |  |  |  |  |  |  |  |  |  |
| Public Power | 1.0 | 0.5 | 0.5 | 0.3 | 0.3 | 0.3 | 0.3 | 0.2 | 0.1 | 0.1 | 16.9% |
| Petroleum Refining Plants | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.6% |
| Other Combustion & Trans. | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0% |
| Combustion in Comm./Inst/ Res |  |  |  |  |  |  |  |  |  |  |  |
| Comm., Public & Agriculture | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.7% |
| Residential Plants | 0.5 | 0.2 | 0.2 | 0.2 | 0.2 | 0.3 | 0.2 | 0.2 | 0.2 | 0.1 | 11.1% |
| Combustion in Industry | 0.5 | 0.4 | 0.4 | 0.4 | 0.4 | 0.4 | 0.3 | 0.3 | 0.3 | 0.3 | 44.5% |
| Production Processes | 0.2 | 0.2 | 0.2 | 0.2 | 0.1 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 22.5% |
| Shipping | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0. | 0% |
| Non livestock agriculture | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0% |
| TOTAL | 2.9 | 2.1 | 2.1 | 1.9 | 1.8 | 1.7 | 1.5 | 1.5 | 1.3 | 1.2 | 100% |

Figure 7.3 Time Series of Sodium Emissions (tonnes)

## potassium

As with sodium, iron and steel productions is responsible for the majority of potassium emissions, 366 tonnes, followed by domestic coal burning with 51 tonnes. Fireworks are the third main contributor, causing the emissions of 332 tonnes of potassium in 2003.

Estimates of emissions of potassium from coal combustion at Alcan’s Ashington power station have been revised downwards.  Estimates for slag cement grinding, fireworks and burning of waste lubricants have all been added for the first time. The net impact of these changes is that there has been very little change to the total emission quoted in previous versions of the inventory.

Table 7.4 UK Emissions of Potassium by UN/ECE Source Category (ktonnes)

|  | 1990 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 | 2003 | 2003% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BY UN/ECE CATEGORY |  |  |  |  |  |  |  |  |  |  |  |
| Combustion in Energy Prod |  |  |  |  |  |  |  |  |  |  |  |
| Public Power | 0.9 | 0.4 | 0.4 | 0.3 | 0.3 | 0.2 | 0.2 | 0.2 | 0.1 | 9.4 | 9.4% |
| Other Combustion & Trans. | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0% |
| Combustion in Comm./Inst/ Res |  |  |  |  |  |  |  |  |  |  |  |
| Comm., Public & Agri. | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.4% |
| Residential Plants | 0.4 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.1 | 9.3% |
| Combustion in Industry | 0.6 | 0.6 | 0.6 | 0.6 | 0.5 | 0.5 | 0.5 | 0.4 | 0.4 | 0.4 | 40.1% |
| Production Processes | 0.0 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 6.2% |
| Non livestock agriculture | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 1.8% |
| TOTAL | 2.9 | 1.7 | 1.7 | 1.5 | 1.5 | 1.4 | 1.3 | 1.3 | 1.1 | 1.1 | 100% |

Figure 7.4 Time Series of Potassium Emissions (tonnes)

## Accuracy of Emission Estimates Of Base Cations

Quantitative estimates of the uncertainties in emission inventories have been based on calculations made using a direct simulation technique, which corresponds to the IPCC Tier 2 approach recommended for greenhouse gases and also the methodology proposed in draft guidance produced by the UN/ECE Taskforce on Emission Inventories and Projections.  This work is described in detail by Passant (2002b).  The estimates are shown in Table 7.5.

Table 7.5 Uncertainty of the Emission Inventories for Base Cations

| Pollutant | Estimated Uncertainty % |
| --- | --- |
| Calcium | -50% to +200% |
| Magnesium | -40% to +90% |
| Sodium | -40% to +100% |
| Potassium | -60% to +200% |

Inventories for base cations continue to undergo improvement and development. However, many of the emission estimates are still subject to significant uncertainty.  This is because they are based on emission estimates for PM10 (which are themselves highly uncertain), coupled with estimates of the chemical composition of the PM10 which add further uncertainty.

# References

APARG (1996), The Abatement of Toxic Organic Micropollutants (TOMPS) from Stationary Sources.  Report to the Air Pollution Abatement Review Group.

APEG (1999), Source Apportionment of Airborne Particulate Matter in the United Kingdom. Report of the Airborne Particles Expert Group. ISBN 0-7058-1771-7.

Baggott SL, Brown L, Milne R, Murrells TP, Passant N, Thistlethwaite G, Watterson JD (2005), UK Greenhouse Gas Inventory, 1990 to 2003. Netcen, AEA Technology, Harwell, Oxfordshire. ISBN 0-9547136-2-1.

Buckingham, C (1997),  Atmospheric Emissions Inventory for Greater Manchester.  LRC, London.

Clayton P, Coleman P, Leonard A, Loader A, Marlowe I, Mitchell D, Richardson S, Scott D, (1991),  Review of Municipal Solid Waste Incineration in the UK. Warren Spring Laboratory Report LR776(PA).

Coleman P, Bush T, Conolly C, Irons S, Murrells T, Vincent K, Watterson J (2001), Assessment of Benzo[a]pyrene atmospheric concentrations in the UK to support the establishment of a national PAH objective. AEAT/ENV/R/0620 Issue 5.1.1. Netcen, AEA Technology, Harwell, Oxfordshire.

Coleman P, Conolly, C, Dyke, P & Watterson, J (2001), Toxic Organic Micropollutant Moitoring 1999 to 2000, AEA Technology, Report No AEAT/R/ENV/0301 Issue 1

Couling SB, Passant NR, Richardson SR, Marlowe I, Eggleston HS, Salway AG, (1994), UK Emissions to Atmosphere of Heavy Metals covered by the Hague Declaration. AEA/CS/16419044/Z003. Netcen, AEA Technology, Harwell, Oxfordshire.

DETR (1998a), Review of the United Kingdom National Air Quality Strategy - A Consultation Document. 98EP0541/A. The Stationary Office, Norwich.

DETR (2000), The Air Quality Strategy for England, Scotland, Wales and Northern Ireland- Working Together for Clean Air. The Stationary Office, Norwich.

Defra (2001), The Air Quality Strategy for England, Scotland, Wales and Northern Ireland - A Consultation Document on Proposals for Air Quality Objectives for Particles, Benzene, Carbon Monoxide and PAHs. HMSO.

Defra (2003), The Air Quality Strategy for England, Scotland, Wales and Northern Ireland: Addendum. HMSO.

DOE (1989), Sources of HCH discharge to the aquatic environment (Draft).

DOE (1994), Impacts of Nitrogen Deposition on Terrestrial Ecosystems, Report of : The UK Review Group on Impacts of Atmospheric Nitrogen. ISBN 1-870393-22-8.

DOE (1997), The United Kingdom National Air Quality Strategy. CM3587. The Stationary Office, London.

Dore CJ, Goodwin JWL, Watterson JD, Murrells TP, Passant NR, Hobson MM, Haigh KE, Baggott S L, Thistlethwaite G, Pye ST, Coleman PJ, King KR (2003), UK Emissions of Air Pollutants 1970-2001. Netcen, AEA Technology, Harwell, Oxfordshire. Report AEAT/ENV/R/1593. ISBN 1-85580-033-0.

Dore CJ, Watterson JD, Goodwin JWL, Murrells TP, Passant NR, Hobson MM, Baggott S L, Thistlethwaite G, Coleman PJ, King KR, Adams M, Cumine P R (2004), UK Emissions of Air Pollutants 1970-2002. Netcen, AEA Technology, Harwell, Oxfordshire. Report AEAT/ENV/R/1933. ISBN 0954713648.

Dragosits U and Sutton MA (2003), Update on Ammonia emissions from non-agricultural sources for the NAEI 1970-2002.

DTI (2004), Digest of United Kingdom Energy Statistics 2003. London. The Stationary Office.

Eggleston HS (1988), UK Emissions of Air Pollutants 1970-1986. LR 659 (AP)M. Warren Spring Laboratory, Stevenage.

Eggleston HS, Munday PK (1989), UK Emissions of Air Pollutants 1970-1987. LR733 (AP)M. Warren Spring Laboratory, Stevenage.

Eggleston HS, Charles D, Salway AG, Jones BMR, Milne R (1998), Treatment of Uncertainties for National Estimates of Greenhouse Gas Emissions. AEAT-2688-1.

EMEP/CORINAIR (1996), Atmospheric Emission Inventory Guidebook. ed McInnes G, F.

Environment Agency (2003), Pollution Inventory.

Environment Agency (2003), Agricultural Waste Survey: A study of the management of non-natural agricultural waste on farms.

EPAQS (1995), Expert Panel on Air Quality Standards – Particles. HMSO. London.

EPAQS (2001), Expert Panel on Air Quality Standards - Airborne Particles: What is the Appropriate Measurement on Which to Base a Standard? HMSO. London.

EU (2000), Diphenylether, pentabromo derivative (Pentabromodiphenly ether): Summary of risk assessment report. European Union risk assessment report. ISBN 92-894-6479-5.

Gillham CA, Munday PK, Eggleston HS (1992), UK Emissions of Air Pollutants 1970-1990. LR887 (AP)M. Warren Spring Laboratory, Stevenage.

Gillham CA, Couling S, Leech PK, Eggleston HS, Irwin JG (1994), UK Emission of Air Pollutants, 1970-1991 (Including Methodology Update). LR961 (AP)M. Warren Spring Laboratory, Stevenage.

Goodwin JWL, Salway AG, Murrells TP, Dore C J, Eggleston HS (1999), UK Emissions of Air Pollutants 1970-1997. Netcen, AEA Technology, Harwell, Oxfordshire. Report AEAT-EEQC/0015 Issue 1.

Goodwin JWL, Salway AG, Murrells TP, Dore C J, Passant NR Eggleston HS (2000), UK Emissions of Air Pollutants 1970-1998. Netcen, AEA Technology, Harwell, Oxfordshire. Report AEAT/R/EN/0270. ISBN 0-7058-1794-6.

Goodwin JWL, Salway AG, Murrells TP, Dore C J, Passant NR, King KR, Coleman PJ, Hobson MM, Pye ST, Watterson JD (2001), UK Emissions of Air Pollutants 1970-1999. Netcen, AEA Technology, Harwell, Oxfordshire. Report AEAT/ENV/R/0798. ISBN 1-85580-031-4.

Goodwin JWL, Salway AG, Dore C J, Murrells TP, Passant NR, King KR, Coleman PJ, Hobson MM, Pye ST, Watterson JD, Haigh KE, Conolly CM (2002), UK Emissions of Air Pollutants 1970-2000. Netcen, AEA Technology, Harwell, Oxfordshire. Report AEAT/ENV/R/1283. ISBN 1-85580-031-0.

Haydock H, Adams M, Bates J, Passant N, Pye S, Salway G, Smith A (2004), Emissions and Projections of HFCs, PFCs and SF6 for the UK and Constituent Countries. Netcen, AEA Technology, Harwell, Oxfordshire. Report AEAT/ED50090/R02.

HMIP (1995), A Review of Dioxin Emissions in the UK, Report No DOE/HMIP/RR/95/004.

IARC (1987)- International Agency for Research on Cancer. Overall Evaluations of Carcinogenicity: An Updating of IARC Monographs Volumes 1 to 42.

IPCC (1996), Climate Change 1995: The Science of Climate Change. Contribution of Working Group 1 to the Second Assessment Report of the Intergovernmental Panel on Climate Change.  ed. Houghton JT. Cambridge University Press.

IPCC (1997), Revised 1996 IPCC Guidelines for National Greenhouse Gas Inventories, Vols. 1,2,3.  ed. Houghton JT. IPCC WG1 Technical Support Unit, Hadley Centre, Meteorological Office, Bracknell, UK.

Jacoff FS, Scarberry R, Rossa D (1986), Source Assessment of Hexachlorobenzene from the Organic Chemical Manufacturing Industry Proc. Int. Symp. International Agency for research on Cancer, Lyon 24-28 Jun. 1985 . IARC Sci Pub. No. 77 p31.

Jockel W, Hartje J (1991), Datenerhebung uber die Emissionen Umweltgefahrdender Schermetalle, Technischer Uberwachungs-Verein Rheinland e.V, Institut fur Umweltschutz und Energietechnik.

Lee DS and JM Pacyna (1999), An industrial emissions inventory of calcium for Europe. Atmospheric Environment, 33, 1687 – 1697.

Leech PK (1991), UK Emissions of Air Pollutants 1970-1989. LR 826 (AP). Warren Spring Laboratory, Stevenage.

Leech PK (1993), UK Atmospheric Emissions of Metals and Halides 1970-1991. LR 923. Warren Spring Laboratory, Stevenage.

Lindsay WL (1979), Chemical Equilibria in Soils.  John Wiley and Sons, p. 7.

Lloyds Register (1995),  Marine Exhaust Emissions Research Programme.

MAFF (1991a), Pesticide Usage Survey Report 101: Vegetables for Human Consumption.

MAFF (1991b), Pesticide Usage Survey Report 102: Protected Crops (Edible and Ornamental).

MAFF (1991c), Pesticide Usage Survey Report 103: Mushrooms.

MAFF (1992a), Pesticide Usage Survey Report 107: Aerial Applications, Great Britain.

MAFF (1992b), Pesticide Usage Survey Report 108: Arable Farm Crops in Great Britain.

MAFF (1992c), Pesticide Usage Survey Report 114: Potato Stores in Great Britain.

MAFF (1992d), Pesticide Usage Survey Report 115: Orchards and Fruit Stores in Great Britain.

Milne R (2003),  Centre for Ecology and Hydrology, Personal Communication.

Misselbrook TH, Chadwick DR, Chambers BJ, Smith KA, Webb J, Demmers T, Sneath RW (2003), Inventory of Ammonia Emissions from UK Agriculture 2002.

Mumma CE and Lawless EW (1975), Survey of Industrial Processing Data. Task 1.  Hexachlorobenzene and Hexachlorobutadiene pollution from Chlorocarbon Processing.  USEPA Office of Toxic Substances. EPA 560-3-75-003 (PB-243-641).

Munday PK (1990), UK Emissions of Air Pollutants 1970-1988. LR764 (AP)M. Warren Spring Laboratory, Stevenage.

NATO/CCMS (1988),  International Toxicity Equivalent Factor (I-TEF) method of risk assessment for complex mixtures of dioxins and related compounds.  Pilot Study on international information exchange on dioxins and related compounds.  Report No 176. North Atlantic Treaty Organisation, Committee on Challenges of Modern Society.

Nicholson (2000), Re-suspension from roads: Initial estimate of emission factors. AEAT Internal Report.

Pacyna JM (1999), “Monitoring and modelling of lead, cadmium and mercury transboundary transport in the atmosphere of Europe” in Joint report of EMEP Centres: MSC-E and CCC MSC-E Alexey Ryaboshapko, Ilia Ilyin, Alexey Gusev, Olga Afinogenova Torunn Berg, Anne-Gunn Hjellbrekke Report 3/99.

Passant NR, Peirce M, Rudd HJ, Scott DW, Marlowe IT and Watterson JD (2002a), UK Particulate and Heavy Metal Emissions from Industrial Processes. Netcen, AEA Technology, Harwell, Oxfordshire, Report No AEAT-6270. Issue 2.

Passant NR (2002b), Estimation of Uncertainties in the National Atmospheric Emissions Inventory. AEA Technology. Report No. AEAT/ENV/R/1039 Draft.

Passant, N R (2003/4), Emission factors programme: Task 1 – Summary of simple desk studies, AEA Technology Plc, Report No AEAT/ENV/R/1715/Issue 1.

Passant, N R, Thistlethwaite, G, & Roberts  I (2004), NAEI Ad-hoc Emission Factors Project, Task 2.4:  Estimation of Particulate Emissions from Quarrying & Construction Processes, AEA Technology Plc, Report No AEAT/ENV/R/1711 Final Draft.

Passant, N R  & Woodfield, M J (2004), NAEI Ad-hoc Improvement Programme: PM Emissions from Fires and Natural, Agricultural and Accidental Sources, AEA Technology Plc, Report No AEAT/ENV/R/1809 Issue 1.

Perry, R E (2002), Report on a project to measure emissions and calculate emission factors for a range of pollutants released to air during the combustion of solid fuels on an open fire, CPL Ltd, Report Reference 02/aeat/1.

QUARG (1993),  Urban Air Quality in the United Kingdom, First Report of the Quality of Urban Air Review Group. Department of the Environment.  ISBN 0-95207-7116.

QUARG (1996), Airborne Particulate Matter in the United Kingdom, Third report of the Quality of Urban Air Review Group. Department of the Environment.  ISBN 0-9520771-3-2.

Ratcliffe M (1999), DETR: UK Submission to Oslo and Paris Commissions (OSPARCOM).

RGAR (1997), Acid Deposition in the United Kingdom 1992 to 1994, Fourth Report of the United Kingdom Review Group on Acid Rain. Department of the Environment. Publication Sales, Ruislip.

Salway AG, Goodwin JWL, Eggleston HS (1996), UK Emissions of Air Pollutants 1970-1993. Netcen, AEA Technology, Harwell, Oxfordshire. Report AEA/RAMP/16419127/R/001.

Salway AG, Goodwin JWL, Eggleston HS, Murrells TP (1996a), UK Emissions of Air Pollutants 1970-1994. Netcen, AEA Technology, Harwell, Oxfordshire. Report EA/RAMP/20090001/R/003.

Salway AG, Goodwin JWL, Eggleston HS, Murrells TP (1997), UK Emissions of Air Pollutants 1970-1995. Netcen, AEA Technology, Harwell, Oxfordshire. Report AEAT-1746 Issue 1.

Salway AG, Eggleston HS, Goodwin JWL, Berry JE, Murrells TP (1999), UK Emissions of Air Pollutants 1970-1996. Netcen, AEA Technology, Harwell. Report AEAT-3092 Issue 1.

Salway AG (2000), UK Greenhouse Gas Emission Inventory 1990 to 1998. Netcen, AEA Technology, Harwell. Oxfordshire.  ISBN 0-7058-1796-2.

Sloss L and Smith I (2000), Trace Element Emissions. IEA Coal Research – The Clean Coal Centre.

Smith IM (1987), Trace elements from coal combustion: emissions IEA Coal Research. IEACR/01.

Smyllie RJ (1996), Abatement Methods and Costs for Heavy Metal Emissions to Air. Entech Report No 96047.

Stewart, H E , Hewitt, C N , Bunce, R G H , Steingbrecher, R , Smiatek, G  & Schoenemeyer, T (2003), A highly spatially and temporally resolved inventory for biogenic isoprene and monoterpene emissions: Model description and application to Great Britain, J. Geophys. Res., 108(D20), 4644, doi:10.1029/2002JD002694.

Strandman T et al (2000), Polybrominated diphenly ethers (PBDE’s) in placenta and human milk. Organohologen Compounds Vol. 47.

Sullivan (1991), Orimulsion a review. DNV Technica.

Sullivan, R (1992), Orimulsion - A Review, DNV Technica Ltd, Report No DNV-T\I909.

Thistlethwaite, G (2001), Determination of Atmospheric Pollutant Emission Factors at a Small Coal-Fired Heating Boiler, AEA Technology, Report No AEAT/R/ENV/0517.

Thistlethwaite, G (2004), NAEI Ad-hoc Improvement Programme, Task 2.1: Estimation of Emissions from Small Combustion Processes, AEA Technology, Report No AEAT/R/ENV/1713.

TNO (1997),  Particulate Matter Emissions (PM10, PM2.5, PM<0.1) in Europe in 1990 and 1993. TNO Report TNO-MEP-R96/472. Netherlands.

United Kingdom Minerals Yearbook 1990-2001 (2002), British Geological Survey.

USEPA (1995), United States Environmental Protection Agency, Compilation of Air Pollutant Emission Factors. Vol 1, 5th Edition, AP-42, North Carolina.

USEPA (1988), Second Supplement to the Compendium of Methods for the Determination of Toxic Organic Compounds in Ambient Air.  EPA-600/4-89-018.  pp TO-13 to TO-97.

US EPA (2004), Compilation of Air Pollutant Emission Factors, AP-42.

van der Most PFJ, Veldt C (1989), TNO 89-036 Emissions into the atmosphere of PAH's, PCB's, Lindane and HCB in Europe.

van der Most PFJ, Veldt C (1992), Emission Factors Manual PARCOM-ATMOS, Emission factors for air pollutants 1992, TNO report ref 92-235. TNO Institute of Environmental and Energy Technology, Apeldoorn, The Netherlands.

Watson M (2003), Personal communications regarding sulphur content of liquid fuels, UKPIA.

Wenborn MJ, Coleman PJ, Passant NR, Lymberidi E, and Weir RA (1999), Speciated PAH Inventory for the UK. Netcen, AEA Technology, Harwell, Oxfordshire. Report AEAT3512/REMC/20459131 Issue1.

WHO (1998), Van den Berg M, Birnbaum L, Bosveld ATC, Brunström B, Cook P, Feeley M, Giesy JP, Hanberg A, Hasegawa R, Kennedy SW, Kubiak T, Larsen JC, van Leeuwen FXR, Liem AKD, Nolt C, Peterson RE, Poellinger L, Safe S, Schrenk D, Tillitt D, Tyskland M, Younes M, Wærn F and Zacharewski T. Toxic Equivalency Factors (TEFs) for PCBs, PCDDs, PCDFs for humans and wildlife. Environ Health Perspect, 106: 775-792.

Wild H (1992), Pentachlorophenol in the UK Environment 1: A Budget and Source Inventory (Chemosphere 24 No.7 833-845).

Williams A (1994), Methane Emissions. Watt Committee Report Number 28. The Watt Committee on Energy, London.

Wood P (1996), Heavy Metal Contents of Liquid Fuels. Netcen, AEA Technology, Harwell, Oxfordshire. Report AEA/WMES/20058001/R/001.

Annex 1 Definitions of UN/ECE Source Sectors

CONTENTS

1. UN/ECE Classification of Emission Sources

Table A1 Mapping of NAEI Base Categories to CORINAIR SNAP 1994

1. UN/ECE Classification of Emission Sources

For this report, the NAEI emission estimates have been reported according to the UNECE/CORINAIR SNAP 97 eleven sector classification. This is the last year which this reporting structure will be used because official UK data submissions are now required in a different format.

The NAEI reports emissions from the combustion of fuels and non-combustion emissions from a range of sectors.  The fuels data are taken from the Digest of UK Energy Statistics (DUKES), (DTI, 2004).  Hence the fuel definitions and the choice of base sector categories used in the NAEI often reflect those in the DUKES publications.  The choice of non-combustion sources generally reflects the availability of data on the emissions from specific activities.

The  SNAP 97 reporting format used in this report is less detailed than the NAEI base categories.  A listing of the NAEI base categories and their mapping onto the UNECE/CORINAIR SNAP 97 categories is shown in Table A1 below. In some cases the NAEI categories shown here are aggregates of more detailed emission sectors. The sectors are presented in this way in Table A1 to allow an in depth understanding of the source sector content without presenting an excessive amount of information. In addition, the names of several NAEI categories have been changed to enable a fuller understanding by those not familiar with some of the nomenclature.

Since 2000, the structure under which the UK inventory must be reported to the UN/ECE has changed. However, the SNAP categories (as explained above) are still favoured by emission inventory compilers, and has therefore been retained as the primary compilation structure of the UK emissions inventory. Output data is then formatted to the required reporting structure for submission to international bodies.

At the least detail level, there are 11 categories for the UN/ECE emission inventory format. These are as follow:-

01 Combustion in Energy Production & Transformation

02 Combustion in Commercial, Institutional & Residential & Agriculture

03 Combustion in Industry

04 Production Processes

05 Extraction & Distribution of Fossil Fuels

06 Solvent Use

07 Road Transport

08 Other Transport & Mobile Machinery

09 Waste Treatment & Disposal

10 Agriculture Forestry & Land Use Change

11 Nature

The allocation of a particular source to one of these sectors is well defined and given in more detail in Table A1. The majority of allocations are easy to understand. For example, emissions arising from fuel combustion in the manufacture of glass will come under “03 Combustion in Industry”. Emission arising from the manufacture/handling of the glass material itself and the raw materials will come under “04 Production Processes”.

Table A1 Mapping of NAEI Base Categories to CORINAIR SNAP 94

| CORINAIR SNAP 94 Source Category | NAEI Class |
| --- | --- |
| 01 Combustion in Energy Production & Transformation | Coke Production |
|  | Collieries |
|  | Gas Production |
|  | Nuclear Fuel Production |
|  | Offshore- own gas use |
|  | Power Stations |
|  | Refineries- Combustion activities only |
|  | Solid Smokeless Fuel Production |
|  | Town Gas Production |
| 02 Combustion in Commercial, Institutional & Residential & Agriculture | Accidental Fires |
|  | Agricultural Combustion |
|  | Domestic Combustion |
|  | Miscellaneous |
|  | Other Non-Industrial Combustion |
|  | Public Services (Public buildings etc.) |
|  | Railways (Stationary sources only) |
| 03 Combustion in Industry | Ammonia Combustion |
|  | Power Station supplying to industrial activities only |
|  | Cement & Lime Production- Combustion activities |
|  | Cement (combustion for non-decarbonising activities) |
|  | Ceramic manufacture |
|  | Glass Production |
|  | Iron and Steel (Blast Furnaces) |
|  | Iron and Steel (Combustion) |
|  | Iron and Steel (Grey Iron Foundry) |
|  | Iron and Steel (Sinter Plant) |
|  | Lead Crystal Glass Production |
|  | Lime Production (Combustion) |
|  | Nickel Production |
|  | Primary Copper Production |
|  | Primary Lead Production |
|  | Primary Lead-Zinc Production |
|  | Secondary Aluminium Smelting |
|  | Secondary Copper |
|  | Secondary Lead |
|  | Other industry- Combustion activities |

Table A1 Continued..

| CORINAIR SNAP 94 Source Category | NAEI Class |
| --- | --- |
| 04 Production Processes | Adipic Acid Production |
|  | Ammonia Production and use of feedstocks |
|  | Bread Baking |
|  | Brewing |
|  | Cement and Concrete Batching |
|  | Cement (decarbonising activities) |
|  | Chemical industry (Organic Chemicals) |
|  | Chloroalkali Industry |
|  | Cider Manufacture |
|  | Coke Production (Door Leakage) |
|  | Building/Construction and Road Construction |
|  | Glass Production (non fuel combustion emissions) |
|  | Iron and Steel (Ni-Cr Use, Steel Pickling, Furnaces, Flaring) |
|  | Lead Mining (Dewatering) |
|  | Lime Production (Decarbonising) |
|  | Magnesium alloying |
|  | Nitric Acid Production |
|  | Non-Ferrous Metals (Other non-ferrous metals) |
|  | Aluminium Production (General) |
|  | Primary Zinc Production |
|  | Tin Production |
|  | Other Food (animal feed;cakes, biscuits, cereals; coffee, margerine and other solid fats; meat ,fish and poultry; sugar) |
|  | Asphalt Manufacture |
|  | Other Industrial Processes |
|  | Phosphate Fertiliser Production |
|  | Pigments & Stabilisers |
|  | Quarrying |
|  | Refineries (drainage, process & tankage) |
|  | Rolling Mills (Hot & Cold Rolling) |
|  | Solid Smokeless Fuel Production |
|  | Spirit Manufacture |
|  | Sugar Beet processing |
|  | Sulphuric Acid Production |
|  | Textile Coating |
|  | Wine Manufacture |
|  | Wood Impregnation |

Table A1 Continued….

| CORINAIR SNAP 94 Source Category | NAEI Class |
| --- | --- |
| 05 Extraction & Distribution of Fossil Fuels | Coal Storage & Transport |
|  | Deep-Mined Coal |
|  | Gas Leakage from Pipes etc. |
|  | Gasification Processes |
|  | Offshore Loading of fuel |
|  | Offshore Oil & Gas |
|  | Oil Terminal Storage |
|  | Onshore loading of fuel |
|  | Open-Cast Coal |
|  | Petrol distribution (storage, dispatch, petrol stations, delivery tankers) |
|  | Petroleum Processes |
| 06 Solvent Use | Aerosols (Car care, Cosmetics & toiletries, household products) |
|  | Agrochemicals use |
|  | Coating manufacture (glue, ink, paint) |
|  | Creosote use |
|  | Decorative paint |
|  | Dry cleaning |
|  | Film coating |
|  | Industrial adhesives |
|  | Industrial coatings (automotive, coil coating, commercial vehicles, heavy duty, marine, metal and plastic, metal packaging, vehicle refinishing, wood) |
|  | Leather degreasing |
|  | Leather coating |
|  | non aerosol products (household, automotive, cosmetics & toiletries, adhesives and sealants, paint thinner or remover) |
|  | Other rubber products |
|  | Other solvent use |
|  | Paper Coating |
|  | Printing |
|  | Seed Oil Extraction |
|  | Surface cleaning |
|  | Textile coating |
|  | Tyre Manufacture |
|  | Wood impregnation |

Table A1 Continued….

| CORINAIR SNAP 94 Source Category | NAEI Class |
| --- | --- |
| 07 Road Transport | Road transport (buses) |
|  | Road transport (cars) |
|  | Road transport (cars CAT) |
|  | Road transport (cars non CAT) |
|  | Road transport (HGV Articulated) |
|  | Road transport (HGV rigid) |
|  | Road transport (LGVs) |
|  | Road transport (LGVs CAT) |
|  | Road transport (LGVs non CAT) |
|  | Road transport (motorcycles) |
| 08 Other Transport & Mobile Machinery | Agricultural Power Units |
|  | Aircraft (Military) |
|  | Aircraft Support vehicles etc. |
|  | Aircraft Take-Off and Landing (domestic) |
|  | Aircraft Take-Off and Landing (international) |
|  | Shipping (UK shipping up to 12 km of coastline) |
|  | Domestic House and Garden machinery |
|  | Fishing |
|  | Other Industry Off-Road (e.g. forklifts etc.) |
|  | Railways (Freight) |
|  | Railways (Intercity) |
|  | Railways (regional) |
|  | Shipping Naval |
| 09 Waste Treatment & Disposal | Batteries |
|  | Electrical Equipment |
|  | Incineration |
|  | Landfill |
|  | Lighting |
|  | Measurement and Control Equipment |
|  | Offshore Flaring |
|  | Refineries (flaring) |
|  | Regeneration of Activated Carbon |
|  | Sewage Sludge Disposal |
| 10 Agriculture Forestry & Land Use Change | Agricultural Soils |
|  | Agricultural Pesticides |
|  | Agricultural Livestock |
|  | Field Burning |
| 11 Nature | Forests |
|  | Natural Fires |