| Review of Carbon Emission Factors in the UK Greenhouse Gas Inventory |  |
| --- | --- |
|  | Baggott SL Lelland A Passant NP Watterson J |
| November 2004 |  |
| This work forms part of the Global Atmosphere Research Programme of the Department for Environment, Food and Rural Affairs. |  |

| Title | Review of Carbon Emission Factors in the UK Greenhouse Gas Inventory |
| --- | --- |

| Customer | Department for Environment, Food and Rural Affairs |
| --- | --- |

| Customer reference | RMP/2106 |
| --- | --- |

| Confidentiality, copyright and reproduction | Copyright AEA Technology plc. All rights reserved. Enquiries about copyright and reproduction should be addressed to the Commercial Manager, AEA Technology plc. Published by AEA Technology plc. |
| --- | --- |

| File reference | /tmp/in/input.doc |
| --- | --- |
| NAEI contacts database reference | 05452/2002/CD4545/SB |

| Reference number | AEAT/ENV/R/2347 |
| --- | --- |
| ISBN | n/a |

| Issue number | Issue One, Version 1.3.2 |
| --- | --- |

- AEA Energy and Environment

The Gemini Building

Fermi Avenue

Harwell International Business Centre

Didcot, Oxfordshire,

OX11 0QR

- Telephone		0870 190 6409

- Facsimile 		0870 190 6607

- AEA Technology is the trading name of

- AEA Technology plc

- AEA Technology is certified to ISO9001

| Report Manager | Name | S L Baggott |
| --- | --- | --- |

| Approved by | Name | J Watterson |
| --- | --- | --- |

|  | Signature |  |
| --- | --- | --- |

|  | Date | 2 November 2006 |
| --- | --- | --- |

- Units and conversions

Emissions of greenhouse gases presented in this report are given in Million tonnes (Mt) and kilotonnes (kt).  To convert between the units of emissions, use the conversion factors given below.

Prefixes and multiplication factors

| Multiplication factor | Abbreviation | Prefix | Symbol |
| --- | --- | --- | --- |
| 1,000,000,000,000,000 | 1015 | peta | P |
| 1,000,000,000,000 | 1012 | tera | T |
| 1,000,000,000 | 109 | giga | G |
| 1,000,000 | 106 | mega | M |
| 1,000 | 103 | kilo | k |
| 100 | 102 | hecto | h |
| 10 | 101 | deca | da |
| 0.1 | 10-1 | deci | d |
| 0.01 | 10-2 | centi | c |
| 0.001 | 10-3 | milli | m |
| 0.000,001 | 10-6 | micro |  |

1 kilotonne (kt)	=	103 tonnes	=	1,000 tonnes

1 Million tonne (Mt)	=	106 tonnes	=	1,000,000 tonnes

1 Gigagramme (Gg)	=	1 kt

1 Teragramme (Tg)	=	1 Mt

Conversion of carbon emitted to carbon dioxide emitted

To covert emissions expressed in weight of carbon, to emissions in weight of carbon dioxide, multiply by 44/12.

- ES.1

This report set out the results of a work programme to review and update the carbon emission factors in the 2002 UK greenhouse gas inventory.  This was part of a wider programme of inventory review and improvement to prepare the greenhouse gas inventory to deliver the UK’s Assigned Amount under the Kyoto Protocol.

Table ES1 shows the overall change in carbon emissions using the updated carbon factors for solid, liquid and gaseous fuels.

Table ES1:	Overall change in carbon emissions (MtC)

| Year | 2002 GHG inventory | Emissions using updated factors | Difference | % difference |
| --- | --- | --- | --- | --- |
| 1990 | 164.63 | 165.88 | 1.26 | 0.76 |
| 1991 | 165.66 | 167.43 | 1.77 | 1.07 |
| 1992 | 161.62 | 163.02 | 1.40 | 0.86 |
| 1993 | 157.45 | 158.62 | 1.17 | 0.75 |
| 1994 | 156.27 | 157.31 | 1.05 | 0.67 |
| 1995 | 153.83 | 154.17 | 0.34 | 0.22 |
| 1996 | 159.35 | 160.31 | 0.96 | 0.60 |
| 1997 | 152.65 | 153.84 | 1.19 | 0.78 |
| 1998 | 153.46 | 155.03 | 1.57 | 1.02 |
| 1999 | 151.13 | 152.07 | 0.93 | 0.62 |
| 2000 | 152.17 | 152.92 | 0.76 | 0.50 |
| 2001 | 155.75 | 157.09 | 1.33 | 0.86 |
| 2002 | 150.26 | 151.79 | 1.52 | 1.02 |
| % change 1990-2002 | -8.73% | -8.49% |  |  |

- Table of Contents

1	Introduction	1

1.1	Background to this Project	1

1.2	General Approach to the CEF Review	2

1.3	Fuels Reviewed	2

1.4	QUOTED ACCURACY OF THE EMISSIONS IN THIS STUDY	3

2	Overview of the changes in estimated GHG emissions due to the revisions in the selected CEFs	4

2.1	Original and revised carbon emission factors	4

2.2	contributions of Fuels to carbon emissions in inventory	6

2.3	Units of CEFs used in the UK GHG inventory	6

3	Solid fuels	8

3.1	Introduction	8

3.2	CEFs of solid fuels Used in the 2002 GHG inventory	8

3.2.1	Coal	8

3.2.2	Coke	9

3.2.3	Anthracite	10

3.2.4	Petroleum coke	10

3.3	Revised CEFs	10

3.3.1	Fraction of fuel unoxidised after combustion	10

3.3.2	Coal – ESI sector	10

3.3.3	Coal – Other sectors	12

3.3.3.1	Other sectors – excluding cement production	12

3.3.3.2	Cement production	17

3.3.4	Coke	17

3.3.5	Anthracite	17

3.3.5	Petroleum coke	17

3.3.5.1	Non-cement industry consumers	17

3.3.5.2	Cement industry	18

3.4	Effect of Revising the CEFs for Solid Fuels on Caron Emissions	18

3.4.1	Coal – ESI sector	18

3.4.2	Coal, anthracite, coke - other sectors	20

3.4.3	Petroleum coke	22

3.4.4	Overall change to carbon emissions resulting from revisions to the CEFs of the solid fuels	22

4	Gaseous fuels	23

4.1	Introduction	23

4.2	CEFs OF Gaseous Fuels Used in the 2002 GHG inventory	23

4.2.1	Natural Gas	23

4.2.2	Coke oven gas	24

4.2.3	Blast furnace gas	24

4.2.4	Other petroleum gas (OPG)	24

4.3	Revised CEFs	24

4.3.1	Fraction of fuel unoxidised after combustion	24

4.3.2	Natural gas	24

4.3.3	Coke oven gas	26

4.3.4	Blast furnace gas	26

4.3.5	Other petroleum gas	26

4.4	Effect of Revising the CEFs for Gaseous Fuels on Carbon Emissions	26

4.4.1	Natural gas	26

4.4.1.1	Database	27

4.4.2	Blast furnace gas	28

4.4.3	Coke oven gas	28

4.4.4	Other petroleum gas (OPG)	29

4.4.5	Overall change to carbon emissions resulting from revisions to the CEFs of the gaseous fuels	29

5	Liquid fuels	30

5.1	Introduction	30

5.2	CEFs OF LIQUID FUELS USED IN THE 2002 GHG INVENTORY	31

5.3	Revised CEFs	31

5.3.1	Fraction of fuel unoxidised after combustion	33

5.4	Effect of Revising the CEFs for Liquid Fuels on Carbon Emissions	33

5.4.1	Naphtha	33

5.4.2	Aviation spirit	34

5.4.3	Fuel Oil	34

5.4.4	Gas oil	35

5.4.5	Diesel oil	35

5.4.6	Overall change to carbon emissions resulting from revisions to the CEFs of the liquid fuels	35

6	Overall effect on carbon emissions of the revised CEFs	37

7	References	41

8	Acknowledgements	43

Appendix 1	44

A1.1	Bunker Emissions	44

A1.2	Gross Calorific Values	46

A1.3	Conversion Factors for Gross to Net Energy Consumption	49

Appendix 2	50

A2.1	Review of fractions of carbon left unoxidised after combustion in liquid fuels	50

Appendix 3	52

A3.1	Project Meetings	52

- Document revision history

| - Issue | - Version | - Revision history |
| --- | --- | --- |
| - Draft |  | - Issued to UK Defra for internal review and comment - Comments incorporated |
| - 1.0 | - 1.3 | - First issue |
| - 1.0 | - 1.3.1 | - Minor typographical errors corrected |
| - 1.0 | - 1.3.2 | - Minor typographical errors corrected in the headers and footers |

Abbreviations, acronyms and definitions

Greenhouse gases

CH4	Methane

CO2	Carbon dioxide

N2O	Nitrous oxide

General

AEP	Association of Electricity Providers
www.aepuk.com

AS	Aviation Spirit

ATF	Aviation Turbine Fuel

BCA	British Cement Association

BFG	Blast Furnace Gas

C	Carbon

CEF	Carbon Emission Factor

COG	Coke Oven Gas

CORUS	The owner of the many of the integrated steel works in the UK

DTI	UK Department of Trade and Industry

DERV	Diesel Engined Road Vehicle
fuel used in internal combustion engines that are compression ignition engines

DUKES	Digest of United Kingdom Energy Statistics
www.dti.gov.uk/energy/statistics

EA	Environment Agency
for England and Wales

EU	European Union

ESI	Electricity Supply Sector

ETS	Emissions Trading Scheme

GCV	Gross Calorific Value

GHG	Greenhouse gas

GHGI	Greenhouse gas inventory

IPCC	Intergovernmental Panel on Climate Change

JEP	Joint Environmental Programme
funded by the Association of Electricity Producers (AEP)

kt	kilotonne

LPG	Liquid Petroleum Gas
composed of a variable mixture of propane and butane

Mt	Mega tonne

NG	Natural gas

OPG	Other Petroleum Gas

SSF	Solid Smokeless Fuel

SEPA	Scottish Environmental Protection Agency

TRANSCO	Gas transportation company
Ceased to exists in 2005, and now part of National Grid UK

UK	United Kingdom

UKPIA	UK Petroleum Industry Association
Trade association which represents the oil refining and marketing industry in the United Kingdom
www.ukpia.com

UN/ECE	United Nations Economic Commission for Europe

UNFCCC	United Nations Framework Convention on Climate Change

# Introduction

## Background to this Project

The United Nations Framework Convention on Climate Change (UNFCCC) was ratified by the United Kingdom in December 1993 and came into force in March 1994.  Parties to the Convention are committed to develop, publish and regularly update national emission inventories of greenhouse gases (GHGs).  The UK has been compiling and submitting GHG inventories to the FCCC since 1994.

AEA Energy & Environment, on behalf of UK Defra, has prepared the latest (at the time this report was written) greenhouse gas inventory (GHGI) and associated tables of emissions in CRF format according to UNFCCC guidelines contained in FCCC/CP/2002/8 (Baggott et. al., 2004).  The estimates within the inventory have been generated by following the methods and procedures set out in the IPCC Revised 1996 Guidelines for National Greenhouse Gas Inventories (IPCC, 1997a, b, c) and Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories (IPCC, 2000).

The UK GHGI is subject to a range of external reviews.  At the time this report was written, these reviews include reviews by experts from the FCCC during desk, centralised and in-country reviews, and, by invited independent examiners during a process of peer review.  The Intergovernmental Panel on Climate Change (IPCC) Good Practice Guidance (IPCC, 2000) defines expert peer review as follows:

‘Expert peer review consists of a review of calculations or assumptions by experts in relevant technical fields.  This procedure is generally accomplished by reviewing the documentation associated with the methods and results, but usually does not include rigorous certification of data or references such as might be undertaken in an audit.  The objective of the expert peer review is to ensure that the inventory's results, assumptions, and methods are reasonable as judged by those knowledgeable in the specific field.’

The Good Practice Guidance requires that key sources should be subjected to external peer review.  Key sources are those source categories with a significant influence on a country’s total inventory of direct greenhouse gases in terms of the absolute level of emissions, the trend in emissions, or both.

The first peer review of the GHGI considered CO2 emissions from fuel combustion (Simmons, 2002).  This topic was chosen for peer review since IPCC sector 1A1 was identified as a key source because of the magnitude of emissions of carbon dioxide (CO2) from the combustion of solid, liquid and gaseous fuels in that sector.  In addition, fossil fuel combustion is the major source of UK emissions of carbon dioxide.

A key recommendation of this peer review was that the carbon emission factors (CEFs) currently used in the inventory should be reviewed:

‘Many of the carbon emission factors used in the inventory date from 1989.  Whilst they may still be accurate for the fuels used today there have been changes to the sources of fuels over the period of the inventory and changes in their qualities to reflect environmental requirements.  It is recommended that the carbon content factors be reviewed, together with related factors accounting for incomplete oxidation of the fuels during combustion.’

This report presents the results of a review of the CEFs used in the 2004 UK National Inventory Report, containing the 2002 GHG inventory (1990 to 2002 inclusive).  This work is important to the overall quality of the GHG estimates since accurate estimates of the carbon contents of fuels determine the accuracy of the estimated CO2 emissions.

## General Approach to the CEF Review

A programme of work was developed to review CEFs of the fuels in the UK greenhouse gas inventory.  The review concentrated on a selection of key fuels that contribute to the greatest proportions of CO2 emissions.  A range of stakeholders were consulted during the review process, including industry experts within the UK electricity supply industry, the petroleum industry, and the gas supply industry.  To support this work, a selection of liquid fuels were analysed, on behalf of UKPIA, to determine their carbon contents.  Analysis of gas composition data from TRANSCO was also considered.  In addition, data presented in a range of reports (including data on UK CEFs, calorific values, fractions of fuels left unoxidised after combustion, and fuel compositions) were reviewed.  A range of parties examined the initial recommendations for revised CEFs including industry experts and government representatives of the UK Energy Trading Scheme (UK ETS).  Their recommendations were considered before the final set of CEFs was selected for the GHGI.

Following this review, CEFs were introduced into the 2002 greenhouse gas inventory, and the changes in the carbon emissions were quantified.

## Fuels Reviewed

An analysis of the quantities of fuel consumed according to IPCC sector was completed in order to identify which fuels contributed the most to emissions of CO2 in the UK.  This analysis clearly showed that the main effort of the work should be concentrated on revising the CEFs for natural gas, coal and the liquid fuels.  The CEFs of other fuels were reviewed during the course of this work; the reason for selecting these additional fuels was to satisfy demands for revised CEFs for the UK ETS.

## QUOTED ACCURACY OF THE EMISSIONS IN THIS STUDY

In this report, emissions are quoted to 0.01 ktonne (or better) purely for convenience, to avoid the risk of rounding errors, and for convenience when taking ratios.  The number of decimal places used should not be taken as indicative of the accuracy of the estimates.

# Overview of the changes in estimated GHG emissions due to the revisions in the selected CEFs

This chapter of the report presents a summary of the changes in estimated GHG emissions due to the revisions in the selected CEFs.  Table 2.1 shows the values of the original CEFs used in the 2002 GHGI for 1990 and 2002, and the revisions to those CEFs.  The changes in emissions are summarised according to fuel, and IPCC reporting sector.  Table 2.1 shows the overall effect on the revised CEFs on the national GHG totals.

## Original and revised carbon emission factors

Table 2.1:	Original and revised CEFs values used in the 2002 GHGI for 1990 and 2002 (KtC/Mt fuel for solid and liquid fuels, and Kt/Mtherm for gaseous fuels)

| Fuel Type | Consumer | IPCC Fuel | Original CEF 1990 | Revised CEF 1990 | Original CEF 2002 | Revised CEF 2002 |
| --- | --- | --- | --- | --- | --- | --- |
| Solid |  | Petcoke | 800 | 930 | 800 | 930 |
|  |  | Petcoke (cement) | 800 | 829.6 | 800 | 829.6 |
|  | Agriculture | Other bituminous coal | 659.6 | 656.6 | 659.6 | 650.4 |
|  | Autogenerators | Other bituminous coal | 659.6 | 659.6 | 659.6 | 628.7 |
|  | Cement (combustion) | Other bituminous coal | 659.6 | 757.7 | 659.6 | 725.5 |
|  | Collieries | Other bituminous coal | 659.6 | 659.6 | 659.6 | 684.9 |
|  | Domestic | Other bituminous coal | 676.8 | 676.8 | 676.8 | 696.9 |
|  | Domestic | Anthracite | 813.4 | 813.4 | 813.4 | 820.6 |
|  | ESI | Other bituminous coal | 588.2 | 605 | 588.2 | 627.2 |
|  | Iron & Steel blast furnaces | Coking coal | - | - | 710 | 694 |
|  | Iron & Steel (combustion) | Other bituminous coal | 659.6 | 659.6 | 659.6 | 693.8 |
|  | Lime production (combustion) | Other bituminous coal | 659.6 | 659.6 | 659.6 | 628.7 |
|  | Miscellaneous | Other bituminous coal | 659.6 | 659.6 | 659.6 | 721.9 |
|  | Other industry | Other bituminous coal | 659.6 | 659.6 | 659.6 | 633.5 |
|  | Public services | Other bituminous coal | 659.6 | 659.6 | 659.6 | 721.9 |
|  | Railways (stationary sources) | Other bituminous coal | 659.6 | 659.6 | 659.6 | 721.9 |
| Liquid |  | Aviation spirit | 850 | 853 | 865 | 853 |
|  |  | Diesel Oil | 857 | 864 | 857 | 863 |
|  |  | Fuel Oil | 850 | 867 | 850 | 879 |
|  |  | Gas Oil | 857 | 869 | 857 | 870 |
|  |  | Naphtha | 940 | 854 | 940 | 854 |
|  |  | Residual Fuel Oil | 850 | 867 | 850 | 879 |
| Gaseous |  | Natural Gas | 1.50 | 1.45 | 1.50 | 1.48 |
|  |  | Blast Furnace Gs | 6.27 | 7.46 | 6.27 | 7.46 |
|  |  | Other Petroleum Gas | 1.68 | 1.64 | 1.63 | 1.64 |

- The original CEFs are taken from the 2002 GHGI

- ESI – Electricity Supply Industry

- Emissions of the gaseous fuels are gross

- The CEFs for solid fuels in this table account for the incomplete oxidation of the fuels following combustion.  Oxidation factors are considered in Chapters 3, 4 and 5.

## contributions of Fuels to carbon emissions in inventory

Different fuels contribute different amounts to overall carbon emissions in the UK.  It is those fuels that contribute the most to emissions that will have the most significant effect on emissions if their factors are altered.  Table 2.2 list the major contributors to carbon emissions both in 1990 and 2002.

Table 2.2:	Percentage contributions of fuels to overall carbon emissions in the UK in 1990 and 2002

| Fuel | 1990 | 2002 |
| --- | --- | --- |
| Coal | 35 | 21 |
| Natural Gas | 19 | 38 |
| Petrol | 13 | 11 |
| Fuel oil | 7 | 2 |
| DERV | 6 | 10 |
| Gas Oil | 4 | 4 |
| Blast Furnace Gas | 3 | 2 |
| OPG | 1 | 1 |
| Coke | 1 | 1 |
| Clinker Production | 1 | 1 |
| Burning Oil | 1 | 2 |
| Aviation Turbine Fuel | 1 | 1 |
| Anthracite | 1 | - |
| SSF | 1 | - |
| Coke Oven Gas | 1 | - |
| LPG | 1 | 1 |
| Limestone | 1 | - |
| Petroleum Coke | - | 1 |

The fuels listed in the above table contribute 97% to overall carbon emissions in 1990 and 2002.  In each year, the remaining 3% is made up of a number of other fuels, each contributing 1% or less to the overall total.

In both 1990 and 2002 the top three contributors remain the same – natural gas, coal and petrol, although the relative contribution from coal decreases and the importance of natural gas increases in 2002, relative to 1990.

## Units of CEFs used in the UK GHG inventory

- The emission factors used in the 2002 UK GHG inventory are expressed in terms of kg pollutant/tonne for solid and liquid fuels, and g/TJ gross for gases.  This differs from the IPCC approach which expresses emission factors as tonnes pollutant/TJ based on the net calorific value of the fuel.  For gases the factors used in the UK GHG inventory are based on the gross calorific value of the fuel.  This approach is used because the gas consumption data in commodity balance tables produced by the UK DTI are reported in terms of energy content on a gross basis.

- Emissions factors for solid and liquid fuels are in terms of mass.  These mass based factors can then be applied to consumptions of UK fuels expressed on a mass basis published by the DTI.  Emissions factors for natural gas are in terms of the energy content of the gas consumed (g carbon/GJ gross).  These energy based factors can then be applied to consumptions of UK fuels expressed on a energy basis published by the DTI.

# Solid fuels

## Introduction

Within the UK, the main solid fuels that are used are coal, anthracite, coke and petroleum coke.  In 1990 coal was the major contributor to carbon greenhouse gas emissions, with 34% of emissions arising from this fuel.  This declined to 21% in 2002.  The main user of coal is the power station sector, which accounted for 88.5% of emissions in 2002.  Table 3.1 displays the main sectors that use coal in the UK and their percentage contribution to overall carbon emissions

Table 3.1	 Significant sources of coal in the UK

| Source | % contribution to overall coal emission |
| --- | --- |
| Power Stations | 88.5 |
| Coke Production | 1.2 |
| Public Services | 0.3 |
| Domestic | 2.2 |
| Autogenerators | 3.4 |
| Other Industry combustion | 3.2 |
| Cement fuel combustion | 0.8 |
| Lime production - combustion | 0.2 |

Minor emissions also occur from the Iron and steel industry, agriculture, railways, collieries and miscellaneous sources.

Coke use in the UK accounted for 1% of overall carbon emissions in the UK in 2002.  Coke is mainly used by the iron and steel industry and in 2002, emissions from this sector accounted for 69% of emissions.  Other significant emissions arise from combustion in other industry and domestic use.

Petroleum coke emissions accounted for 1% of overall carbon emissions in the UK in 2002.  Petroleum coke is used mainly by the refineries sector (48% of emissions in 2002) and the cement industry (40% of emissions in 2002).

## CEFs of solid fuels Used in the 2002 GHG inventory

### Coal

The current carbon emission factors for coal were supplied to AEA Technology by British Coal in 1989.  Since then, the emission factors used have not been altered.  The production of coal within the UK declined steadily during the 1990s and gradually more coal has been imported from elsewhere.  Due to this change in the mix of home/imported coal now used in the UK, the factors were reviewed.

All the coal CEFs in the inventory take into account the proportion of un-oxidised carbon that remains in the ash after combustion.  Table 3.2 shows the assumed oxidation factors for each type of coal and the corresponding un-oxidised carbon emission factor.

Table 3.2:	Carbon retention in ash figures for coal use in UK in the 2004 GHG inventory

| Coal use | Un-oxidised factor (kt/Mt) | C retention in ash (%) | Oxidation factor | Oxidised factor (kt/Mt) |
| --- | --- | --- | --- | --- |
| Power Stations | 590 | 0.3 | 0.997 | 588.2 |
| Other | 680 | 3 | 0.97 | 659.6 |
| Domestic | 720 | 6 | 0.940 | 676.8 |

Table 3.3 displays the sources of coal emissions in the inventory and their corresponding emission factors.

Table 3.3:	Current coal emission factors (kt/Mt)

| Source | Current coal emission factor (kt/Mt) |
| --- | --- |
| Power Stations | 588.2 |
| Public Services | 659.6 |
| Domestic | 676.9 |
| Autogenerators | 659.6 |
| Other Industry combustion | 659.6 |
| Cement fuel combustion | 659.6 |
| Lime production - combustion | 659.6 |

Minor emissions also occur from the iron and steel industry, agriculture, railways, collieries and other miscellaneous sources.  The emission factor used for iron and steel blast furnaces is currently 710 kt/Mt and for other sources mentioned, the emission factor used is 659.6 kt/Mt.

### Coke

Coke emissions in the inventory arise from a variety of sources and overall contribute 1% to carbon emissions in the UK.  Currently, as with coal, the factors were supplied by British Coal in 1989.

As with coal, assumptions were made about the proportion of un-oxidised carbon left after combustion.  Table 3.4 shows the assumed oxidation factors for coke and the corresponding un-oxidised carbon emission factor along with the oxidised factor that is currently used in the UK GHGI.

Table 3.4:	Carbon retention in ash figures for coke use in UK

| Coke use | Un-oxidised factor (kt/Mt) | C retention in ash (%) | Oxidation factor | Oxidised factor (kt/Mt) |
| --- | --- | --- | --- | --- |
| Domestic | 820 | 2 | 0.98 | 803.6 |
| Power stations | 820 | 3 | 0.97 | 795.4 |
| Other | 820 | 3 | 0.97 | 795.4 |

Table 3.5 displays the sources of coke emissions in the inventory and their corresponding emission factors.

Table 3.5:	Sources of coke emissions and corresponding emission factors

| Source | Current coke emission factor (kt/Mt) |
| --- | --- |
| Domestic | 803.6 |
| SSF production Coke production | 820 |
| Iron and Steel (sinter) Other industry (combustion) Iron and steel (combustion) Lime production (combustion) Power stations Railway (stationary sources) Public services Agriculture Miscellaneous | 795.4 |

Emissions from iron and steel blast furnaces are calculated from a mass balance approach (see Section 3.4.2).

### Anthracite

Carbon emissions from anthracite occur from the domestic sector only.  The current carbon content of anthracite was supplied by British Coal in 1989 and has a value of 859 kt/Mt.  This has a correction applied to it for the un-burnt fraction of carbon of 5.3%, giving the carbon emission factor of 813.4 kt/Mt.

### Petroleum coke

The current factor for petroleum coke was supplied by the UK Petroleum Institute Association (UKPIA) in 1989.  The value is 800 ktC/Mt and applies to all sectors burning petroleum coke.

## Revised CEFs

### Fraction of fuel unoxidised after combustion

The CEFs in this section account for the carbon left unoxidised after combustion.

### Coal – ESI sector

Power stations are the major contributor to carbon emissions from coal combustion, and so obtaining updated emission factors for this sector was considered a priority.

A time series (1990-2003) of updated carbon emission factors (Table 4.6) was obtained from Powertech (Quick, 2004).  It was advised that data in this time series was based on Powergen operated sites only but would be fairly representative of the UK as a whole.  These updated emission factors took into account the increase in the UK’s use of imported coals as well as a carbon in ash content (i.e. un-burnt carbon).

Table 3.6:	Carbon emission factors provided by Quick (2004)

| Year | % C in coal | % C in Coal corrected for 5% C in ash, 13% ash coal | Kte C/Mte coal burnt |
| --- | --- | --- | --- |
| 1990 | 60.37 | 59.69 | 596.86 |
| 1991 | 61.05 | 60.37 | 603.66 |
| 1992 | 60.78 | 60.10 | 600.96 |
| 1993 | 60.78 | 60.10 | 600.96 |
| 1994 | 60.63 | 59.95 | 599.46 |
| 1995 | 59.73 | 59.05 | 590.46 |
| 1996 | 60.92 | 60.24 | 602.36 |
| 1997 | 61.74 | 61.06 | 610.56 |
| 1998 | 62.35 | 61.67 | 616.66 |
| 1999 | 61.94 | 61.26 | 612.56 |
| 2000 | 61.76 | 61.08 | 610.76 |
| 2001 | 62.54 | 61.86 | 618.56 |
| 2002 | 62.99 | 62.31 | 623.06 |
| 2003 | 62.58 | 61.90 | 618.96 |

A working paper from the Association of Electricity Producers (AEP) (Joslin et al, 2004) detailing their recommended carbon content of fuel and oxidation factor for 2003 was also obtained.  This paper recommended a carbon content of 0.64 and an oxidation factor of 0.98.  After discussion with both the AEP and Powergen it was established that the slightly higher carbon content quoted by the AEP was due to the influence of Aberthaw power station which burns coal with a higher carbon content.  There was also a difference in the oxidation factor between the Powertech and AEP studies, and this occurs for two primary reasons.  Firstly, the influence of Aberthaw, which due to its unique nature has high carbon content in ash.  Secondly, Powergen stations have tended to be run as ‘base-load’ stations more than others, which invariably results in better combustion performance and lower carbon in ash (Quick, 2004b).  It was therefore agreed by all parties that the time series of emission factors provided by Quick (2004) should be scaled to the AEP carbon content for 2003.  It was also agreed that the oxidation factor of 0.98 should be used.  Both these assumptions were considered to provide representative parameters from coal use within the UK.

Taking these changes into account, proposed new carbon emission factors for the Greenhouse Gas Inventory are shown in Table 3.7.  Emission factors are scaled as follows:

A * (0.64/0.6258) = Carbon Emission Factor

Where A = Carbon emission factor provided by Quick

So in 2003, Carbon emission factor = 0.6258 * (0.64/0.6258) = 0.64

Applying the oxidation factor of 0.98 provides a carbon emission factor of 0.6272.

Table 3.7:	AEP scaled Carbon emission factors

| Year | % C in coal | % C in coal corrected for 2% C in ash | kte/Mte coal burnt |
| --- | --- | --- | --- |
| 1990 | 61.74 | 60.51 | 605.05 |
| 1991 | 62.44 | 61.19 | 611.87 |
| 1992 | 62.16 | 60.92 | 609.16 |
| 1993 | 62.16 | 60.92 | 609.16 |
| 1994 | 62.01 | 60.77 | 607.66 |
| 1995 | 61.09 | 59.86 | 598.64 |
| 1996 | 62.30 | 61.06 | 610.56 |
| 1997 | 63.14 | 61.88 | 618.78 |
| 1998 | 63.76 | 62.49 | 624.89 |
| 1999 | 63.35 | 62.08 | 620.79 |
| 2000 | 63.16 | 61.90 | 618.80 |
| 2001 | 63.96 | 62.68 | 626.80 |
| 2002 | 64.42 | 63.13 | 631.31 |
| 2003 | 64.00 | 62.72 | 627.20 |

### Coal – Other sectors

#### Other sectors – excluding cement production

Table 3.1 shows that the ESI sector is an important contributor to carbon emissions from coal combustion in the UK.  Other sectors that use coal contribute just over 10% to overall emissions, with most of these sectors individually contributing less than 1% to totals.  A full review and consultation with all the industry representatives in these sectors was not considered to be a wise use of resources in this project, and so an approach was developed to scale existing CEFs to GCVs reported in DTI (2003) relative to 1990.  Carbon emission factors and GCVs are closely related and so an increase or decrease in a GCV is likely to reflect similarly in the carbon content of the fuel.  Tables 3.8 to 3.11 display the results of the simple method used to calculate revised emission factors for the sectors consuming smaller quantities of coal.  The cement sector is considered later.

Table 3.8:	Carbon emission factors used in the 2002 GHG inventory

| Source | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Agriculture | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 |
| Coke Production | 41.87 | 38.29 | 31.42 | 34.08 | 34.31 | 40.61 | 36.74 | 45.21 | 44.84 | 58.09 | 42.85 | 58.57 | 58.54 |
| Collieries | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 |
| Domestic | 676.8 | 676.8 | 676.8 | 676.8 | 676.8 | 676.8 | 676.8 | 676.8 | 676.8 | 676.8 | 676.8 | 676.8 | 676.8 |
| I&S (blast furnace) |  |  |  |  | 710 | 710 | 710 | 710 | 710 | 710 | 710 | 710 | 710 |
| I&S (combustion) | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 |
| Lime Production (combustion) | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 |
| Miscellaneous | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 |
| Other Industry (combustion) | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 |
| Public services | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 |
| Railways (stationary sources) | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 |
| SSF production | 127.22 | 79.47 | 77.52 | 49.59 | 23.56 | 33.43 | 93.15 | 135.47 | 46.64 | 36.45 | 27.39 | 37.33 | 33.87 |
| Autogenerators | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 |
| Cement (Fuel combustion) | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 | 659.6 |

Table 3.9:	GCVs reported in the DTI DUKES publication

| Source | DUKES class | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Agriculture | Agriculture | 28.9 | 28.9 | 28.9 | 28.7 | 28.8 | 29 | 28.9 | 29.1 | 28.5 | 28.9 | 29.2 | 29.8 | 28.5 |
| Collieries | Collieries | 28.6 | 28 | 26.8 | 26.8 | 28.2 | 28.2 | 26.2 | 27.8 | 29.6 | 29.3 | 29.6 | 29.8 | 29.7 |
| Domestic | House Coal | 30.2 | 30.2 | 30.2 | 30.2 | 29.8 | 30.4 | 30.6 | 30.6 | 30.9 | 30.9 | 30.9 | 30.9 | 31.1 |
| I&S (blast furnace) | Coke Ovens: Home & Imports | 31.2 | 31.4 | 31.45 | 31.46 | 31.43 | 31.4 | 32 | 32 | 32 | 30.5 | 30.4 | 30.5 | 30.5 |
| I&S (combustion) | Iron and steel industry | 28.9 | 28.9 | 28.9 | 28.7 | 29.4 | 31.5 | 31.3 | 31.3 | 31.3 | 30.7 | 30.7 | 29.4 | 30.4 |
| Lime Production (combustion) | Other industries | 27.8 | 27.8 | 27.8 | 28.1 | 28.8 | 27.7 | 27.3 | 27 | 26.9 | 26.6 | 26.8 | 26.7 | 26.5 |
| Miscellaneous | Other Consumers | 27.5 | 27.5 | 27.5 | 28.7 | 28.8 | 28.9 | 30.4 | 29.3 | 29.2 | 29.1 | 29.2 | 29.2 | 30.1 |
| Other Industry (combustion) | Other Industries | 27.8 | 27.8 | 27.8 | 28.1 | 28.8 | 27.7 | 27.3 | 27 | 26.9 | 26.6 | 26.8 | 26.7 | 26.5 |
| Public services | Other Consumers | 27.5 | 27.5 | 27.5 | 28.7 | 28.8 | 28.9 | 30.4 | 29.3 | 29.2 | 29.1 | 29.2 | 29.2 | 30.1 |
| Railways (stationary sources) | Other Consumers | 27.5 | 27.5 | 27.5 | 28.7 | 28.8 | 28.9 | 30.4 | 29.3 | 29.2 | 29.1 | 29.2 | 29.2 | 30.1 |
| Autogenerators | Other Industries | 27.8 | 27.8 | 27.8 | 28.1 | 28.8 | 27.7 | 27.3 | 27 | 26.9 | 26.6 | 26.8 | 26.7 | 26.5 |
| Cement (Fuel combustion) | Other Industries | 27.8 | 27.8 | 27.8 | 28.1 | 28.8 | 27.7 | 27.3 | 27 | 26.9 | 26.6 | 26.8 | 26.7 | 26.5 |

Table 3.10:	Ratios of GCVs relative to 1990

| Source | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Agriculture | 1.00 | 1.00 | 1.00 | 0.99 | 1.00 | 1.00 | 1.00 | 1.01 | 0.99 | 1.00 | 1.01 | 1.03 | 0.99 |
| Collieries | 1.00 | 0.98 | 0.94 | 0.94 | 0.99 | 0.99 | 0.92 | 0.97 | 1.04 | 1.02 | 1.04 | 1.04 | 1.04 |
| Domestic | 1.00 | 1.00 | 1.00 | 1.00 | 0.99 | 1.01 | 1.01 | 1.01 | 1.02 | 1.02 | 1.02 | 1.02 | 1.03 |
| I&S (blast furnace) | 1.00 | 1.01 | 1.01 | 1.01 | 1.01 | 1.01 | 1.03 | 1.03 | 1.03 | 0.98 | 0.97 | 0.98 | 0.98 |
| I&S (combustion) | 1.00 | 1.00 | 1.00 | 0.99 | 1.02 | 1.09 | 1.08 | 1.08 | 1.08 | 1.06 | 1.06 | 1.02 | 1.05 |
| Lime Production (combustion) | 1.00 | 1.00 | 1.00 | 1.01 | 1.04 | 1.00 | 0.98 | 0.97 | 0.97 | 0.96 | 0.96 | 0.96 | 0.95 |
| Miscellaneous | 1.00 | 1.00 | 1.00 | 1.04 | 1.05 | 1.05 | 1.11 | 1.07 | 1.06 | 1.06 | 1.06 | 1.06 | 1.09 |
| Other Industry (combustion) | 1.00 | 1.00 | 1.00 | 1.01 | 1.04 | 1.00 | 0.98 | 0.97 | 0.97 | 0.96 | 0.96 | 0.96 | 0.96 |
| Public services | 1.00 | 1.00 | 1.00 | 1.04 | 1.05 | 1.05 | 1.11 | 1.07 | 1.06 | 1.06 | 1.06 | 1.06 | 1.09 |
| Railways (stationary sources) | 1.00 | 1.00 | 1.00 | 1.04 | 1.05 | 1.05 | 1.11 | 1.07 | 1.06 | 1.06 | 1.06 | 1.06 | 1.09 |
| Autogenerators | 1.00 | 1.00 | 1.00 | 1.01 | 1.04 | 1.00 | 0.98 | 0.97 | 0.97 | 0.96 | 0.96 | 0.96 | 0.95 |

Table 3.11:	Revised carbon emission factors for coal

| Source | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Agriculture | 659.60 | 659.60 | 659.60 | 655.04 | 657.32 | 661.88 | 659.60 | 664.16 | 650.47 | 659.60 | 666.45 | 680.14 | 650.47 |
| Collieries | 659.60 | 645.76 | 618.09 | 618.09 | 650.37 | 650.37 | 604.25 | 641.15 | 682.66 | 675.74 | 682.66 | 687.28 | 684.97 |
| Domestic | 676.80 | 676.80 | 676.80 | 676.80 | 667.84 | 681.28 | 685.76 | 685.76 | 692.49 | 692.49 | 692.49 | 692.49 | 696.97 |
| I&S (blast furnace) | - | - | - | - | 715.23 | 714.55 | 728.21 | 728.21 | 728.21 | 694.07 | 691.79 | 694.07 | 694.07 |
| I&S (combustion) | 659.60 | 659.60 | 659.60 | 655.04 | 671.01 | 718.94 | 714.38 | 714.38 | 714.38 | 700.38 | 700.38 | 671.01 | 693.84 |
| Lime Production (combustion) | 659.60 | 659.60 | 659.60 | 666.72 | 683.33 | 657.23 | 647.74 | 640.62 | 638.25 | 631.13 | 635.87 | 633.50 | 628.76 |
| Miscellaneous | 659.60 | 659.60 | 659.60 | 688.38 | 690.78 | 693.18 | 729.16 | 702.77 | 700.38 | 697.98 | 700.38 | 700.38 | 721.96 |
| Other Industry (combustion) | 659.60 | 659.60 | 659.60 | 666.72 | 683.33 | 657.23 | 647.74 | 640.62 | 638.25 | 631.13 | 635.87 | 633.50 | 633.50 |
| Public services | 659.60 | 659.60 | 659.60 | 688.38 | 690.78 | 693.18 | 729.16 | 702.77 | 700.38 | 697.98 | 700.38 | 700.38 | 721.96 |
| Railways (stationary sources) | 659.60 | 659.60 | 659.60 | 688.38 | 690.78 | 693.18 | 729.16 | 702.77 | 700.38 | 697.98 | 700.38 | 700.38 | 721.96 |
| Autogenerators | 659.60 | 659.60 | 659.60 | 666.72 | 683.33 | 657.23 | 647.74 | 640.62 | 638.25 | 631.13 | 635.87 | 633.50 | 628.76 |

#### Cement production

We contacted the trade association for the UK cement producers - the British Cement Association (BCA) – and the National Energy Manager for Lafarge cement to gather information about the current usage and combustion conditions of coal in the cement sector.  Cement kilns operate at high temperatures (for example, temperatures in the sintering zone of rotary kilns are around 1,450°C) and so fuels will be almost completely oxidised, and effectively 100% oxidation is achieved.  The BCA indicated that the UK cement industry tends to now source their coal from abroad, whereas the current GHG inventory assumed UK coal is consumed.  Therefore, the approach of directly scaling the current carbon factors to GCVs was felt to be unsuitable for this industry.

The time series of carbon emission factors (see Table 3.12) for coal used in the cement sector was provided by Rushworth (2004) and was based on the empirical relationship between calorific values and carbon emissions established using actual data for the years 2000 to 2004.  Calorific values for mineral products for the years 1990 to 1999 were taken from DUKES (DTI, 2004) Table A2.  These calorific values are supplied to the DTI by the BCA and therefore are consistent with the empirical relationship derived.

Table 3.12:	Carbon emission factors and GCV for coal used by the UK cement industry

|  | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Emission factor kt/Mt | 757.8 | 757.8 | 757.8 | 792.7 | 760.4 | 728.2 | 736.3 | 725.5 | 714.8 | 717.5 | 725.5 | 725.5 | 725.5 | 749.7 | 744.3 |
| GCV GJ per tonne | 28.2 | 28.2 | 28.2 | 29.5 | 28.3 | 27.1 | 27.4 | 27.0 | 26.6 | 26.7 | 27.0 | 27.0 | 27.0 | 27.9 | 27.7 |

### Coke

Calorific values for coke remained constant throughout the time series and so the factors remained unaltered.

### Anthracite

In the UK inventory, carbon emissions from anthracite occur from domestic sources only.  Anthracite emission factors were scaled to GCVs as described for coal.  Table 4.13 presents the carbon emission factors found in the current inventory, corresponding GCVs, and revised of emission factors.

Table 3.13:	Current and proposed emission factors for anthracite (kt/Mt)

|  | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Current EF | 813.4 | 813.4 | 813.4 | 813.4 | 813.4 | 813.4 | 813.4 | 813.4 | 813.4 | 813.4 | 813.4 | 813.4 | 813.4 |
| GCV (GJ/t) | 33.6 | 33.6 | 33.5 | 33.5 | 33.6 | 34.0 | 33.9 | 33.9 | 34.1 | 33.5 | 33.6 | 33.9 | 33.9 |
| Proposed EF | 813.4 | 813.4 | 811.0 | 811.0 | 813.4 | 823.1 | 820.7 | 820.7 | 825.5 | 811.0 | 813.4 | 820.7 | 820.7 |

### Petroleum coke

#### Non-cement industry consumers

The UK Petroleum Industry Association (UKPIA) provided an updated estimate for the carbon factor for UK petroleum coke (UKPIA, 2004).  The estimate was based on analytical results of the carbon contents of UK petroleum coke.  UKPIA recommended that the new factor should be 930 ktC/Mt across the whole time series, but this factor was not used for petroleum coke used by the cement sector.

#### Cement industry

The cement industry use petroleum coke, and we consulted the BCA about the sources and carbon contents of the fuel consumed.  The BCA reported that all the petroleum coke consumed is not sourced from UK refineries, but is imported, and so the carbon emission factor supplied by UKPIA would not be applicable to the petroleum coke used by the UK cement industry.  The BCA provided measured carbon contents for petroleum coke for 2000 to 2004.  These data were averaged, and applied across the time series, from 1990 to 2004.  The industry normally tightly controls the energy content of its fuels and the industry’s European database shows emission factors across samples to be very similar.  Therefore, applying a constant CEF to the time series seems a reasonable assumption.

The BCA recommended a carbon emission factor of 829.6 ktC/Mt for the cement sector and this has been adopted.

## Effect of Revising the CEFs for Solid Fuels on Caron Emissions

### Coal – ESI sector

Tables 3.14 to 3.16 present the effects of introducing the revised coal CEFs for the ESI into the 2002 GHG inventory.  Table 3.14 displays the difference between overall UK carbon emissions from coal combustion before and after the CEF update for the ESI sector.  Table 3.15 shows the difference in the ESI sector only and finally Table 3.16 displays the overall differences observed in carbon emissions (from the combustion of all fuels) in the UK.

Table 3.14:	Changes in carbon emissions from coal (total for all sources consuming coal) after applying the revised coal CEF to the ESI (Mt)

| Year | Emissions in the original 2002 GHG inventory | Emissions using revised CEFs | % change to coal emissions |
| --- | --- | --- | --- |
| 1990 | 57.25 | 58.64 | 2.43 |
| 1991 | 56.95 | 58.89 | 3.41 |
| 1992 | 53.60 | 55.21 | 3.01 |
| 1993 | 43.31 | 46.65 | 2.96 |
| 1994 | 42.20 | 43.38 | 2.79 |
| 1995 | 39.71 | 40.31 | 1.52 |
| 1996 | 36.49 | 37.68 | 3.27 |
| 1997 | 31.72 | 31.11 | 4.36 |
| 1998 | 31.97 | 33.68 | 5.35 |
| 1999 | 27.90 | 29.19 | 4.62 |
| 2000 | 29.34 | 30.72 | 4.69 |
| 2001 | 33.09 | 34.99 | 5.75 |
| 2002 | 30.68 | 32.66 | 6.48 |

Table 3.15:	Changes in carbon emissions from the ESI sector after applying the revised coal CEF to the ESI (Mt)

| Year | Emissions in the original 2002 GHG inventory | Emissions using revised CEFs | % change to ESI coal emissions |
| --- | --- | --- | --- |
| 1990 | 48.56 | 49.95 | 2.86 |
| 1991 | 48.25 | 50.19 | 4.02 |
| 1992 | 45.29 | 46.90 | 3.56 |
| 1993 | 37.70 | 39.04 | 3.56 |
| 1994 | 35.68 | 36.85 | 3.30 |
| 1995 | 34.08 | 34.69 | 1.77 |
| 1996 | 31.43 | 32.62 | 3.80 |
| 1997 | 26.66 | 28.04 | 5.19 |
| 1998 | 27.43 | 29.14 | 6.23 |
| 1999 | 23.28 | 24.57 | 5.54 |
| 2000 | 26.33 | 27.71 | 5.23 |
| 2001 | 28.99 | 30.90 | 6.56 |
| 2002 | 27.14 | 29.13 | 7.32 |

Table 3.16:	Changes to overall UK carbon emissions after applying the revised coal CEF to the ESI (Mt)

| Year | Emissions in the original 2002 GHG inventory | Emissions using revised CEFs | % change to emissions |
| --- | --- | --- | --- |
| 1990 | 164.63 | 166.02 | 0.84 |
| 1991 | 165.66 | 167.60 | 1.17 |
| 1992 | 161.62 | 163.23 | 1.00 |
| 1993 | 157.45 | 158.79 | 0.85 |
| 1994 | 156.26 | 157.44 | 0.75 |
| 1995 | 153.83 | 154.43 | 0.39 |
| 1996 | 159.35 | 160.54 | 0.75 |
| 1997 | 152.65 | 154.03 | 0.91 |
| 1998 | 153.46 | 155.17 | 1.11 |
| 1999 | 151.13 | 152.42 | 0.85 |
| 2000 | 152.17 | 153.54 | 0.90 |
| 2001 | 155.75 | 157.65 | 1.22 |
| 2002 | 150.26 | 152.25 | 1.32 |

Table 3.17 displays the overall percentage changes in carbon emissions between 1990 and 2002.  It shows the decline in overall carbon emissions from the 2002 GHG inventory (from 1990 to 2002), and the decline in the carbon emissions after the revised CEF has been applied in the ESI sector.  The effect of introducing the new CEF into the ESI sector is to reduce the decline in carbon emissions, although the effect is small.

Table 3.17:	Percentage change in overall carbon emissions between 1990 and 2002

| GHG inventory | % change |
| --- | --- |
| Original 2002 GHG inventory | -8.73 |
| 2002 GHG inventory and updated coal ESI factors | -8.29 |

### Coal, anthracite, coke - other sectors

Tables 3.18 to 3.20 present the effects of introducing the revised coal (non-ESI sectors), anthracite and coke CEFs into the GHG inventory.

Table 3.18:	Changes in carbon emissions from total coal consumption (Mt)

| Year | Emissions in the original 2002 GHG inventory | Emissions using scaled CEFs | % change to coal emissions |
| --- | --- | --- | --- |
| 1990 | 57.25 | 57.76 | 0.88 |
| 1991 | 56.95 | 57.39 | 0.77 |
| 1992 | 53.60 | 53.94 | 0.64 |
| 1993 | 45.31 | 45.73 | 0.92 |
| 1994 | 42.20 | 42.66 | 1.08 |
| 1995 | 39.71 | 39.98 | 0.69 |
| 1996 | 36.49 | 36.86 | 1.02 |
| 1997 | 31.72 | 32.06 | 1.07 |
| 1998 | 31.97 | 32.30 | 1.03 |
| 1999 | 27.90 | 27.89 | -0.04 |
| 2000 | 29.34 | 29.34 | -0.02 |
| 2001 | 33.09 | 33.09 | -0.02 |
| 2002 | 30.68 | 30.67 | -0.03 |

Table 3.19:	Changes in carbon emissions from total anthracite consumption (Mt)

| Year | Emissions in the original 2002 GHG inventory | Emissions using scaled CEFs | % change to anthracite emissions |
| --- | --- | --- | --- |
| 1990 | 0.97 | 0.97 | 0.00 |
| 1991 | 1.32 | 1.32 | 0.00 |
| 1992 | 1.06 | 1.06 | -0.30 |
| 1993 | 1.57 | 1.57 | -0.30 |
| 1994 | 1.62 | 1.62 | 0.00 |
| 1995 | 1.18 | 1.19 | 1.19 |
| 1996 | 1.12 | 1.13 | 0.89 |
| 1997 | 1.04 | 1.05 | 0.89 |
| 1998 | 0.78 | 0.79 | 1.49 |
| 1999 | 0.73 | 0.73 | -0.28 |
| 2000 | 0.75 | 0.75 | 0.00 |
| 2001 | 0.90 | 0.91 | 0.89 |
| 2002 | 0.65 | 0.66 | 0.89 |

Table 3.20:	Changes in carbon emissions from total coke consumption (Mt)

| Year | Emissions in the original 2002 GHG inventory | Emissions using scaled factors | % change to coke emissions |
| --- | --- | --- | --- |
| 1990 | 2.17 | 1.38 | -36.10 |
| 1991 | 1.81 | 1.04 | -42.51 |
| 1992 | 1.88 | 1.16 | -38.47 |
| 1993 | 1.63 | 0.90 | -44.54 |
| 1994 | 1.45 | 0.70 | -51.70 |
| 1995 | 1.24 | 0.46 | -62.82 |
| 1996 | 1.32 | 0.50 | -62.17 |
| 1997 | 1.10 | 0.25 | -77.11 |
| 1998 | 1.09 | 0.27 | -74.72 |
| 1999 | 1.39 | 0.62 | -55.60 |
| 2000 | 1.36 | 0.64 | -53.04 |
| 2001 | 1.30 | 0.70 | -46.24 |
| 2002 | 1.01 | 0.48 | -52.72 |

Although the CEFs for coke remained unaltered from those given in the 2002 GHG inventory, emissions have changed due to the assumptions made about the carbon contents of other fuels.  The change arises for coke used in blast furnaces and also for coal use in coke ovens.  The important point about these source/fuel combinations is that they are artificial devices introduced into the UK GHG inventory many years ago to ensure that carbon balances for processes in steelworks balanced correctly.  For example, coke ovens involve carbon input as coal and carbon output as coke, breeze and coke oven gas.  The carbon content of these inputs/outputs was always fixed and the carbon would never balance.  A correction was therefore applied in the form of the two source/fuel combinations, one for the coke oven carbon balance and one for the blast furnace carbon balance.  These corrections are, effectively the difference between two large but similar numbers and so even a small percentage change in one of these large numbers leads to a large percentage change in the correction.

This methodology will be updated in future versions of the GHG inventory in order to eliminate the need for these corrections.  Adopting this approach would not alter the overall UK carbon emission but would change the detailed allocation of emissions to individual sources, providing more accurate allocation.

### Petroleum coke

Table 3.21 displays the changes in emissions caused by the update of the petroleum coke carbon emission factors.

Table 3.21:	Changes in carbon emissions from total petroleum coke consumption (Mt)

| Year | Emissions in the original 2002 GHG inventory | Emissions using UKPIA and cement industry (2004) CEFs | % change |
| --- | --- | --- | --- |
| 1990 | 0.78 | 0.91 | 16.25 |
| 1991 | 0.83 | 0.97 | 16.25 |
| 1992 | 1.28 | 1.44 | 12.70 |
| 1993 | 1.38 | 1.55 | 12.23 |
| 1994 | 1.39 | 1.55 | 11.27 |
| 1995 | 1.62 | 1.81 | 11.41 |
| 1996 | 1.76 | 1.95 | 10.64 |
| 1997 | 1.72 | 1.91 | 11.35 |
| 1998 | 1.55 | 1.74 | 12.05 |
| 1999 | 1.35 | 1.53 | 13.44 |
| 2000 | 1.27 | 1.44 | 12.78 |
| 2001 | 0.93 | 1.04 | 11.38 |
| 2002 | 1.09 | 1.22 | 11.18 |

### Overall change to carbon emissions resulting from revisions to the CEFs of the solid fuels

Table 3.22 displays the gross overall emission changes in carbon emissions caused by updates to the solid fuel factors in the 2002 GHG inventory.  The effect of introducing the revised solid fuel CEFs is to increase emissions of carbon from the inventory, across the time series, by approximately 1% for each year.

Table 3.22:	Overall emission changes (gross) caused by revisions to the CEFs of the solid fuels (MtC)

| Year | Emissions in the original 2002 GHG inventory | Emissions using updated solid fuel factors | % change |
| --- | --- | --- | --- |
| 1990 | 164.63 | 165.87 | 0.75 |
| 1991 | 165.66 | 167.40 | 1.05 |
| 1992 | 161.62 | 163.01 | 0.86 |
| 1993 | 157.45 | 158.64 | 0.76 |
| 1994 | 156.26 | 157.31 | 0.67 |
| 1995 | 153.83 | 154.12 | 0.19 |
| 1996 | 159.35 | 160.29 | 0.59 |
| 1997 | 152.65 | 153.73 | 0.71 |
| 1998 | 153.46 | 154.89 | 0.93 |
| 1999 | 151.13 | 151.82 | 0.45 |
| 2000 | 152.17 | 152.98 | 0.53 |
| 2001 | 155.75 | 157.16 | 0.91 |
| 2002 | 150.26 | 151.84 | 1.05 |

# Gaseous fuels

## Introduction

Within the UK the main gaseous fuel used is natural gas and this accounts for 38% of total UK carbon emissions.  Smaller emissions arise from blast furnace gas (2% of total UK carbon emissions), Other Petroleum Gas (OPG) (1% of total UK carbon emissions) and Coke Oven Gas (less than 1% of total carbon emissions).  Table 4.1 shows the sources of natural gas within the inventory and their overall percentage contribution to natural gas emissions in 2002.

Table 4.1:	Contribution of sources to natural gas emissions in 2002

| Source | % contribution to overall natural gas emission |
| --- | --- |
| Domestic | 34 |
| Power stations | 26 |
| Other industry (combustion) | 14 |
| Miscellaneous | 6 |
| Public services | 4 |
| Autogenerators | 3 |
| Iron and steel (combustion) | 2 |
| Gas production | 1 |
| Ammonia and feedstock | 1 |
| Cement (fuel combustion) Refineries(combustion) Railways (stationary sources) Agriculture Lime production(combustion) Iron and steel (blast furnaces) SSF production Nuclear fuel production, Coke production Collieries | Minor emissions of less than 1% each arise from these sources |

The main source of emissions for both blast furnace gas and coke oven gas is the iron and steel industry.  This industry accounts for 96% of carbon emissions from blast furnace gas and 52% of emissions from coke oven gas.  The majority of the remaining emissions from coke oven gas arise from coke production.

The main source of emissions from OPG is from combustion at refineries.  This accounts for 88% of emissions from OPG.

## CEFs OF Gaseous Fuels Used in the 2002 GHG inventory

### Natural Gas

The current carbon emission factor for natural gas is 1.501 kt/Mth (52.16 t CO2/TJ gross) and was supplied by British Gas in 1992.  This emission factor is applied across the time series from 1990 to 2002 for all the sources listed in Table 4.1.

### Coke oven gas

The current coke oven gas carbon emission factor was supplied by British coal in 1989.  The factor used is 1.599 kt/Mt and is applied across the whole time series of emissions.

### Blast furnace gas

The current blast furnace gas carbon emission factor was supplied by British Coal in 1989.  The factor used is 6.273 kt/Mt and is applied across the whole time series of emissions.

### Other petroleum gas (OPG)

The current OPG carbon emission factor was supplied by the UK Petroleum Institute Association (UKPIA) in 1989.  The factor used is 1.627 kt/Mtherm and is applied across the whole time series of emissions.

## Revised CEFs

### Fraction of fuel unoxidised after combustion

The CEFs in this section account for the carbon left unoxidised after combustion.  The assumption is that the oxidation is 100%.

### Natural gas

A time series of carbon emission factors (CEF) for natural gas was obtained from Transco (2004).  Data was provided for the years 1990, 1994, 1996, 1998, 1999, 2000, 2002, 2003 and is displayed in Table 4.2.

Table 4.2:	Natural gas emission factors (Transco, 2004)

| Year | Mean Carbon Emission Factor tCO2/TJ(gross) |
| --- | --- |
| 1990 | 50.37 |
| 1994 | 50.76 |
| 1996 | 50.99 |
| 1998 | 51.25 |
| 1999 | 51.48 |
| 2000 | 51.28 |
| 2002 | 51.39 |
| 2003 | 51.45 |

Details of the calculation procedures can be found in Transco (2004b-MPR039).  Transco (2004a-MPR046) state that there are two main factors that contribute to the variability in annual mean CEFs.  Firstly, the increasing influence over time from gas entering the supply network at the St Fergus entry point in Scotland, and secondly, the increasing CEF at this point.  It is suggested that the gas quality changes observed at this point reflect changes in the economics of ethylene production and prevailing international factors (e.g. 1990 Gulf War).

The inventory requires emission factors for every year from 1990-2002.  We concluded that the best method to derive CEFs for the years 1991, 92, 93, 95, 97 and 2001, was to linearly interpolate between the data for the years provided.  This decision was made based on the observation that there was a statistically significant correlation between the year and the CEF measured in that year; see Figure 4.1.

Figure 4.1:	Variation in the natural gas CEF with time (data supplied by Transco, 2004a)

Linearly interpolating between the data supplied by Transco provided the following CEFs for the intervening years:

Table 4.3:	Linearly interpolated emission factors for natural gas

| Year | CEF (tCO2/TJ (gross)) |
| --- | --- |
| 1991 | 50.47 |
| 1992 | 50.57 |
| 1993 | 50.66 |
| 1995 | 50.88 |
| 1997 | 51.12 |
| 2001 | 51.34 |

Within the inventory, data is held in ktC/Mtherm for natural gas.  The CEFs were converted from TJ to Therms using the following expression:

CEF ktC/Mth = ((A*(12/44))/0.009478)/1000

where

A = CEF tCO2/TJ (gross)

0.009478 = Conversion factor for TJ to Mtherms (DTI, 2003)

The time series resulting from this calculation is shown in Table 4.4.

Table 4.4:	CEFs for natural gas (ktC/Mtherm)

| Year | CEF ktC/Mtherm |
| --- | --- |
| 1990 | 1.4494 |
| 1991 | 1.4522 |
| 1992 | 1.4550 |
| 1993 | 1.4578 |
| 1994 | 1.4606 |
| 1995 | 1.4639 |
| 1996 | 1.4672 |
| 1997 | 1.4710 |
| 1998 | 1.4747 |
| 1999 | 1.4813 |
| 2000 | 1.4756 |
| 2001 | 1.4772 |
| 2002 | 1.4787 |
| 2003 | 1.4805 |

### Coke oven gas

Updated emission factors for coke oven gas were obtained from Corus (2004).  The factor recommended for use across the time series was 1.203 kt/Mt.

### Blast furnace gas

Updated emission factors for blast furnace gas were obtained from Corus (2004).  The factor recommended for use across the time series was 7.459 kt/Mt.

### Other petroleum gas

An updated emission factor for other petroleum gas was obtained from UKPIA (2004).  The factor recommended for use across the time series was 1.64398 kt/Mtherm.

## Effect of Revising the CEFs for Gaseous Fuels on Carbon Emissions

### Natural gas

Tables 4.5 to 4.7 display the results of introducing the new CEFs (Transco, 2004) for natural gas into the 2002 GHG inventory.  Table 4.5 shows the difference between natural gas emissions before and after the CEF update with Table 4.7 showing the overall effect the update had on overall carbon emissions in the UK.

Table 4.5:	Changes in carbon emissions after applying the revised natural gas CEF, from natural gas only (Mt)

| Year | Emissions in the original 2002 GHG inventory | Emissions using Transco (2004) factors | % change |
| --- | --- | --- | --- |
| 1990 | 30.35 | 29.39 | -3.17 |
| 1991 | 32.65 | 31.67 | -3.01 |
| 1992 | 32.81 | 31.90 | -2.77 |
| 1993 | 36.94 | 36.00 | -2.55 |
| 1994 | 39.46 | 38.53 | -2.36 |
| 1995 | 41.17 | 41.26 | -2.16 |
| 1996 | 48.91 | 47.94 | -1.98 |
| 1997 | 49.90 | 49.01 | -1.78 |
| 1998 | 52.19 | 51.37 | -1.57 |
| 1999 | 55.53 | 54.87 | -1.19 |
| 2000 | 57.13 | 56.24 | -1.54 |
| 2001 | 57.03 | 56.20 | -1.45 |
| 2002 | 56.47 | 55.70 | -1.35 |

Table 4.6:	Changes to overall UK carbon emissions after applying the revised natural gas CEF (Mt)

| Year | Emissions in the original 2002 GHG inventory | Emissions using Transco (2004) factors | % change |
| --- | --- | --- | --- |
| 1990 | 161.75 | 160.79 | -0.60 |
| 1991 | 162.75 | 161.77 | -0.60 |
| 1992 | 158.66 | 157.76 | -0.57 |
| 1993 | 154.43 | 153.49 | -0.61 |
| 1994 | 153.19 | 152.25 | -0.61 |
| 1995 | 150.68 | 149.77 | -0.60 |
| 1996 | 156.18 | 155.21 | -0.62 |
| 1997 | 149.49 | 148.60 | -0.60 |
| 1998 | 150.32 | 149.50 | -0.55 |
| 1999 | 147.99 | 147.33 | -0.45 |
| 2000 | 148.99 | 148.10 | -0.59 |
| 2001 | 152.58 | 151.76 | -0.54 |
| 2002 | 147.08 | 146.31 | -0.52 |

Table 4.7 displays the overall percentage changes in carbon emissions between 1990 and 2002, and the change in the carbon emissions after the revised natural gas CEF has been applied.  The effect of introducing the new CEF is negligible.

Table 4.7:	Percentage change in overall carbon emissions between 1990 and 2002 after introducing the revised natural gas CEF

| #### Database | % change |
| --- | --- |
| Original 2002 GHG inventory | -9.07% |
| 2002 GHG inventory and updated natural gas CEF factors | -9.00% |

The tables in the following sections set out the effects of introducing the CEFs for selected gaseous fuels into the 2002 GHG inventory.

### Blast furnace gas

Table 4.8:	Changes to carbon emissions following revision of the blast furnace gas CEF (MtC)

| Year | Emissions in the original 2002 GHG inventory | Emissions using Corus (2004) CEFs | % change |
| --- | --- | --- | --- |
| 1990 | 4.14 | 4.92 | 18.91 |
| 1991 | 4.08 | 4.85 | 18.91 |
| 1992 | 3.82 | 4.54 | 18.91 |
| 1993 | 3.83 | 4.55 | 18.91 |
| 1994 | 3.97 | 4.72 | 18.91 |
| 1995 | 4.13 | 4.91 | 18.91 |
| 1996 | 4.34 | 5.16 | 18.91 |
| 1997 | 4.47 | 5.32 | 18.91 |
| 1998 | 4.29 | 5.11 | 18.91 |
| 1999 | 4.10 | 4.87 | 18.91 |
| 2000 | 3.82 | 4.54 | 18.91 |
| 2001 | 3.18 | 3.78 | 18.91 |
| 2002 | 2.83 | 3.36 | 18.91 |

### Coke oven gas

Table 4.9:	Changes to carbon emissions following revision of the coke oven gas CEF (MtC)

| Year | Emissions in the original 2002 GHG inventory | Emissions using Corus (2004) CEFs | % change |
| --- | --- | --- | --- |
| 1990 | 0.95 | 0.72 | -24.18 |
| 1991 | 0.87 | 0.66 | -24.20 |
| 1992 | 0.79 | 0.60 | -24.19 |
| 1993 | 0.75 | 0.57 | -24.15 |
| 1994 | 0.74 | 0.56 | -24.18 |
| 1995 | 0.71 | 0.54 | -24.03 |
| 1996 | 0.78 | 0.60 | -23.13 |
| 1997 | 0.77 | 0.58 | -23.78 |
| 1998 | 0.76 | 0.58 | -23.12 |
| 1999 | 0.71 | 0.54 | -24.33 |
| 2000 | 0.73 | 0.55 | -24.63 |
| 2001 | 0.64 | 0.48 | -24.28 |
| 2002 | 0.53 | 0.41 | -24.24 |

### Other petroleum gas (OPG)

Table 4.10:	Changes to carbon emissions following revision of the OPG CEF (MtC)

| Year | Emissions in the original 2002 GHG inventory | Emissions using UKPIA (2004) CEFs | % change |
| --- | --- | --- | --- |
| 1990 | 2.39 | 2.42 | 1.04 |
| 1991 | 2.37 | 2.40 | 1.04 |
| 1992 | 2.35 | 2.38 | 1.04 |
| 1993 | 2.52 | 2.54 | 1.04 |
| 1994 | 2.61 | 2.63 | 1.04 |
| 1995 | 2.73 | 2.75 | 1.04 |
| 1996 | 2.74 | 2.77 | 1.04 |
| 1997 | 2.53 | 2.56 | 1.04 |
| 1998 | 2.40 | 2.43 | 1.04 |
| 1999 | 2.36 | 2.38 | 1.04 |
| 2000 | 2.39 | 2.41 | 1.04 |
| 2001 | 2.27 | 2.29 | 1.04 |
| 2002 | 2.03 | 2.05 | 1.04 |

### Overall change to carbon emissions resulting from revisions to the CEFs of the gaseous fuels

Table 4.11 shows the overall effect of updating all the CEFs for gaseous fuels on the carbon emissions from the 2002 GHG inventory.  The effect of introducing the revised gaseous fuel CEFs is to slightly decrease emissions of carbon from the inventory, across the time series, by no more than 0.25% with the greatest effect in the early and later years of the inventory.  The effect of revising the gaseous fuel CEFs is much smaller than revising the CEFs of the solid fuels.

Table 4.11:	Overall effect on UK carbon emission totals using updated gaseous fuels CEFs (MtC)

| Year | Emissions in the original 2002 GHG inventory | Emissions using updated gaseous factors | % change |
| --- | --- | --- | --- |
| 1990 | 161.75 | 161.36 | -0.24 |
| 1991 | 162.75 | 162.35 | -0.24 |
| 1992 | 178.66 | 158.31 | -0.22 |
| 1993 | 154.43 | 154.05 | -0.24 |
| 1994 | 153.19 | 152.85 | -0.22 |
| 1995 | 150.68 | 150.41 | -0.18 |
| 1996 | 156.18 | 155.88 | -0.19 |
| 1997 | 149.49 | 149.29 | -0.13 |
| 1998 | 150.32 | 150.15 | -0.11 |
| 1999 | 147.99 | 147.95 | -0.02 |
| 2000 | 148.99 | 148.67 | -0.21 |
| 2001 | 152.58 | 152.23 | -0.23 |
| 2002 | 147.08 | 146.74 | -0.23 |

# Liquid fuels

## Introduction

There are a number of liquid fuels used within the UK.  The major liquid fuels that are used are petrol, DERV, gas oil, burning oil and fuel oil.  The percentage contribution to UK emissions listed in Table 2.2.  Other fuels that are used include Aviation Turbine Fuel (ATF), premium burning oil, waste oils, aviation spirit, LPG and naphtha.  Table 5.1 displays the main sectors using each fuel and their percentage contribution to overall emissions from that fuel.

Table 5.1:	Sources of carbon from liquid fuels in the UK

| Fuel | Source | % contribution to overall fuel emission in UK reported in national totals to the IPCC |
| --- | --- | --- |
| Petrol | Road transport | 99 |
|  | Other industry (off road) | 1 |
| DERV | Road transport | 99.9 |
|  | Domestic house and garden | 0.1 |
| Fuel oil | Power Stations | 20 |
|  | Refineries (combustion) | 53 |
|  | Public Services | 4 |
|  | Miscellaneous | 2 |
|  | Other industry (combustion) | 14 |
|  | Iron and Steel (combustion) | 5 |
|  | Coastal | 1 |
| Gas oil | Refineries | 2 |
|  | Miscellaneous | 6 |
|  | Public services | 9 |
|  | Domestic | 3 |
|  | Agriculture | 2 |
|  | Other industry (combustion) | 32 |
|  | Iron and Steel (combustion) | 3 |
|  | Shipping, Naval | 4 |
|  | Railways | 4 |
|  | Coastal | 8 |
|  | Fishing | 1 |
|  | Agricultural power units | 4 |
|  | Other industry off-road | 19 |
| Burning oil | Domestic | 71 |
|  | Other Industry (Combustion) | 28 |
|  | Railways | 0.3 |
|  | Public Services | 0.3 |
|  | Miscellaneous | 0.3 |
| Aviation turbine fuel | Domestic Aviation | 55 |
|  | Military aviation | 45 |
| Aviation spirit | Domestic Aviation | 100 |
| Burning oil (premium) | Domestic | 100 |
| LPG | Refineries (combustion) | 1 |
|  | Gas Separation Plant (combustion) | 1 |
|  | Domestic | 29 |
|  | Other Industry (combustion) | 61 |
|  | Road Transport | 8 |
| Naphtha | Refineries (combustion) | 100 |
| Waste oils | Cement (fuel combustion) | 100 |

## CEFs OF LIQUID FUELS USED IN THE 2002 GHG INVENTORY

All the current factors for liquid fuels were supplied to AEA Technology by the United Kingdom Petroleum Industry Association (UKPIA) in 1989.  Table 5.2 shows each liquid fuel in the inventory and its corresponding emission factor.  These emission factors apply across the time series of emissions from 1990 to 2002 inclusive.

Table 5.2:	Carbon emission factors for liquid fuels in the 2002 GHG inventory

| Fuel | Current carbon emission factor (ktC/Mt) |
| --- | --- |
| Aviation spirit | 859 |
| Aviation turbine fuel | 859 |
| Burning oil | 859 |
| Burning oil premium | 859 |
| DERV | 857 |
| Fuel oil | 850 |
| Gas oil | 857 |
| LPG | 1.712ktC/Mtherm |
| Naphtha | 940 |
| Petrol | 855 |
| Waste oils | 859 |

These factors are applied across all sources for each fuel (as listed in Table 5.1).

## Revised CEFs

Updated emission factors for liquid fuels were obtained from UKPIA (2004) who carried out a survey of the carbon contents of selected fuels.  Table 5.3 shows the updated factors that were supplied.  The CEFs of fuels not included in this list remain unchanged and were left equal to the values in the 2002 GHG inventory.  A summary of the reasons for changes to the factors is given in the notes below the table and more detailed information is given in UKPIA (2004).

Table 5.2:	Updated emission factors for liquid fuels (KteC/Mte), as supplied by UKPIA (2004)

| Fuel | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 | 2003 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Naphtha | 854 | 854 | 854 | 854 | 854 | 854 | 854 | 854 | 854 | 854 | 854 | 854 | 854 | 854 |
| Aviation spirit | 853 | 853 | 853 | 853 | 853 | 853 | 853 | 853 | 853 | 853 | 853 | 853 | 853 | 853 |
| Diesel | 864 | 864 | 864 | 864 | 864 | 864 | 864 | 864 | 864 | 863 | 863 | 863 | 863 | 863 |
| Gas oil | 869 | 869 | 869 | 869 | 869 | 869 | 869 | 870 | 870 | 870 | 870 | 870 | 870 | 870 |
| Fuel oil | 867 | 868 | 865 | 865 | 865 | 865 | 865 | 868 | 871 | 874 | 875 | 875 | 877 | 879 |

Notes:

- Naphtha and aviation spirit assumed to be constant and based on UKPIA 2004 survey results

- Petrol is assumed to be constant and unchanged from current CEF assumed, and these assumptions are supported by UKPIA 2004 survey results

- Diesel is corrected for changes in sulphur content and density over time using smoothed sulphur contents from the annual UKPIA survey of sulphur contents and density from DTI DUKES

- Gas oil and fuel oil corrected for changes in sulphur content over time using smoothed sulphur contents from the annual UKPIA survey of sulphur contents

### Fraction of fuel unoxidised after combustion

The CEFs in this section account for the carbon left unoxidised after combustion.

A short review was completed to determine what a suitable oxidation factor should be.  During this review, we

- contacted the UK Petroleum Industries Association (UKPIA) to seek their view on the appropriate oxidation factors;

- considered the views of experts involved in the mobile combustion chapter of the 2006 IPPC guidelines;

- consulted with our transport sector, Tim Murrells, for his views on the mobile combustion.

The details of this review are given in Appendix 2.  We concluded that the fraction oxidised for liquid fuels should remain at 100% across the time series, primarily because of the uncertainty in the estimates of the fractions of fuels unoxidised and the difficulties of generating an accurate and defensible time series of unoxidised fractions for each fuel used in the GHG inventory from 1990 to the most current year.

## Effect of Revising the CEFs for Liquid Fuels on Carbon Emissions

The tables in the following sections set out the effects of introducing the CEFs for liquid fuels into the 2002 GHG inventory.

### Naphtha

Table 5.4:	Changes to carbon emissions following revisions to the naptha CEF (MtC)

| Year | Emissions in the original 2002 GHG inventory | Emissions using UKPIA (2004) naptha CEF | % change |
| --- | --- | --- | --- |
| 1990 | 0.02 | 0.02 | -9.14 |
| 1991 | 0.02 | 0.02 | -9.14 |
| 1992 | 0.02 | 0.02 | -9.14 |
| 1993 | 0.02 | 0.02 | -9.14 |
| 1994 | 0.01 | 0.01 | -9.14 |
| 1995 | 0.00 | 0.00 | -9.14 |
| 1996 | 0.00 | 0.00 | -9.14 |
| 1997 | 0.01 | 0.01 | -9.14 |
| 1998 | 0.01 | 0.01 | -9.14 |
| 1999 | 0.02 | 0.01 | -9.14 |
| 2000 | 0.01 | 0.01 | -9.14 |
| 2001 | 0.05 | 0.05 | -9.14 |
| 2002 | 0.02 | 0.02 | -9.14 |

### Aviation spirit

Table 56.5:	Changes to carbon emissions following revisions to the aviation spirit CEF (MtC)

| Year | Emissions in the original 2002 GHG inventory | Emissions using UKPIA (2004) aviation spirit CEF | % change |
| --- | --- | --- | --- |
| 1990 | 0.02 | 0.02 | -1.38 |
| 1991 | 0.02 | 0.02 | -1.38 |
| 1992 | 0.02 | 0.02 | -1.38 |
| 1993 | 0.02 | 0.02 | -1.38 |
| 1994 | 0.02 | 0.02 | -1.38 |
| 1995 | 0.02 | 0.02 | -1.38 |
| 1996 | 0.02 | 0.02 | -1.38 |
| 1997 | 0.03 | 0.03 | -1.38 |
| 1998 | 0.03 | 0.03 | -1.38 |
| 1999 | 0.03 | 0.03 | -1.38 |
| 2000 | 0.04 | 0.04 | -1.38 |
| 2001 | 0.05 | 0.05 | -1.38 |
| 2002 | 0.04 | 0.04 | -1.38 |

### Fuel Oil

Table 5.6:	Changes to carbon emissions following revisions to the fuel oil CEF (MtC)

| Year | Emissions in the original 2002 GHG inventory | Emissions using UKPIA (2004) fuel oil CEF | % change |
| --- | --- | --- | --- |
| 1990 | 11.7 | 12.0 | 2.00 |
| 1991 | 11.6 | 11.9 | 2.12 |
| 1992 | 10.5 | 10.7 | 1.76 |
| 1993 | 9.95 | 10.1 | 1.76 |
| 1994 | 8.92 | 9.08 | 1.76 |
| 1995 | 7.66 | 7.80 | 1.76 |
| 1996 | 7.04 | 7.16 | 1.76 |
| 1997 | 5.31 | 5.43 | 2.12 |
| 1998 | 4.54 | 4.66 | 2.47 |
| 1999 | 3.78 | 3.89 | 2.82 |
| 2000 | 2.80 | 2.89 | 2.94 |
| 2001 | 3.45 | 3.55 | 2.94 |
| 2002 | 3.24 | 3.35 | 3.18 |

### Gas oil

Table 5.7:	Changes to carbon emissions following revisions to the gas oil CEF (MtC)

| Year | Emissions in the original 2002 GHG inventory | Emissions using UKPIA (2004) gas oil CEF | % change |
| --- | --- | --- | --- |
| 1990 | 7.27 | 7.37 | 1.30 |
| 1991 | 7.27 | 7.38 | 1.40 |
| 1992 | 7.11 | 7.21 | 1.35 |
| 1993 | 6.99 | 7.09 | 1.36 |
| 1994 | 6.73 | 6.83 | 1.39 |
| 1995 | 6.51 | 6.60 | 1.40 |
| 1996 | 6.87 | 6.96 | 1.37 |
| 1997 | 6.64 | 6.75 | 1.48 |
| 1998 | 6.50 | 6.59 | 1.18 |
| 1999 | 6.02 | 6.11 | 1.48 |
| 2000 | 5.90 | 5.99 | 1.52 |
| 2001 | 5.88 | 5.97 | 1.52 |
| 2002 | 5.54 | 5.63 | 1.52 |

### Diesel oil

Table 5.8:	Changes to carbon emissions following revisions to the diesel oil CEF (MtC)

| Year | Emissions in the original 2002 GHG inventory | Emissions using UKPIA (2004) diesel oil CEF | % change |
| --- | --- | --- | --- |
| 1990 | 9.13 | 9.20 | 0.82 |
| 1991 | 9.16 | 9.24 | 0.82 |
| 1992 | 9.54 | 9.62 | 0.82 |
| 1993 | 10.1 | 10.2 | 0.82 |
| 1994 | 11.0 | 11.1 | 0.82 |
| 1995 | 11.5 | 11.6 | 0.82 |
| 1996 | 12.3 | 12.4 | 0.82 |
| 1997 | 12.8 | 12.9 | 0.82 |
| 1998 | 12.9 | 13.0 | 0.82 |
| 1999 | 13.2 | 13.3 | 0.70 |
| 2000 | 13.3 | 13.4 | 0.70 |
| 2001 | 14.0 | 14.1 | 0.70 |
| 2002 | 15.1 | 15.2 | 0.70 |

### Overall change to carbon emissions resulting from revisions to the CEFs of the liquid fuels

Table 5.9 displays the overall effect that updating the liquid fuel CEFs has on overall inventory carbon emissions from the 2002 GHG inventory.  Introducing the revised liquid fuel CEFs slightly increases the emissions of carbon from the inventory across the time series, by no more than 0.25%, with the greatest effect in the early years of the inventory.

Table 5.9:	Overall effect on UK carbon emissions using updated liquid fuel CEFs (MtC)

| Year | Emissions in the original 2002 GHG inventory | Emissions using UKPIA (2004) CEFs | % change |
| --- | --- | --- | --- |
| 1990 | 164.6 | 165.0 | 0.24 |
| 1991 | 165.6 | 166.0 | 0.25 |
| 1992 | 161.6 | 161.9 | 0.22 |
| 1993 | 157.4 | 157.8 | 0.22 |
| 1994 | 156.2 | 156.6 | 0.22 |
| 1995 | 153.8 | 154.1 | 0.21 |
| 1996 | 159.3 | 159.6 | 0.20 |
| 1997 | 152.6 | 152.9 | 0.21 |
| 1998 | 153.4 | 153.7 | 0.20 |
| 1999 | 151.1 | 151.4 | 0.19 |
| 2000 | 152.1 | 152.4 | 0.17 |
| 2001 | 155.7 | 156.0 | 0.18 |
| 2002 | 150.2 | 150.5 | 0.19 |

# Overall effect on carbon emissions of the revised CEFs

The following table displays the overall emissions as a result of revisions to all the solid, liquid and gaseous fuels documented in the report.  Emissions are presented in MtC and are gross values.  The overall effect is to increase the carbon emissions across the time series, by 1.3% in 1990 (the base year for the sources and fuels considered in this report), and by 1.5% in 2002.

Table 6.1:	Overall effect on UK carbon emissions after applying all the revised CEFs (MtC)

| Year | Emissions in the original 2002 GHG inventory | Emissions using updated factors | % change | Actual change |
| --- | --- | --- | --- | --- |
| 1990 | 164.6 | 165.8 | 0.76 | 1.25 |
| 1991 | 165.6 | 167.4 | 1.07 | 1.77 |
| 1992 | 161.6 | 163.0 | 0.86 | 1.40 |
| 1993 | 157.4 | 158.6 | 0.75 | 1.17 |
| 1994 | 156.2 | 157.3 | 0.67 | 1.05 |
| 1995 | 153.8 | 154.1 | 0.22 | 0.34 |
| 1996 | 159.3 | 160.3 | 0.60 | 0.96 |
| 1997 | 152.6 | 153.8 | 0.78 | 1.19 |
| 1998 | 153.4 | 155.0 | 1.02 | 1.57 |
| 1999 | 151.1 | 152.0 | 0.62 | 0.93 |
| 2000 | 152.1 | 152.9 | 0.50 | 0.76 |
| 2001 | 155.7 | 157.0 | 0.86 | 1.34 |
| 2002 | 150.2 | 151.7 | 1.02 | 1.52 |
| % change 1990-2002 | -8.73% | -8.50% |  |  |

Tables 6.2 and 6.3 display the emissions, according to fuel type, for the original 2002 GHG inventory, and for 2002 GHG inventory with the updated carbon emission factors.  Table 6.4 displays the percentage change overall, and by fuel type.

The overall effect of introducing the revised CEFs into the inventory was to slightly increase carbon emission across the time series.  Emissions of carbon in 1990 increased by 0.8%, and by 1.0% in 2002.

Table 6.2:	Carbon emissions in the original 2002 GHG inventory (MtC)

| IPCC Fuel | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Anthracite | 0.97 | 1.32 | 1.06 | 1.57 | 1.62 | 1.18 | 1.12 | 1.04 | 0.78 | 0.73 | 0.75 | 0.90 | 0.65 |
| Aviation Gasoline | 0.02 | 0.02 | 0.02 | 0.02 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.04 | 0.04 | 0.05 | 0.04 |
| Blast Furnace Gas | 4.14 | 4.08 | 3.82 | 3.83 | 3.97 | 4.13 | 4.34 | 4.47 | 4.29 | 4.10 | 3.82 | 3.18 | 2.83 |
| Coke Oven Coke | 2.17 | 1.81 | 1.88 | 1.63 | 1.45 | 1.24 | 1.32 | 1.10 | 1.09 | 1.39 | 1.36 | 1.30 | 1.01 |
| Coke Oven Gas | 0.95 | 0.87 | 0.79 | 0.75 | 0.74 | 0.71 | 0.78 | 0.77 | 0.76 | 0.71 | 0.73 | 0.64 | 0.53 |
| Coking Coal | 0.45 | 0.38 | 0.28 | 0.29 | 0.29 | 0.35 | 0.32 | 0.40 | 0.39 | 0.49 | 0.37 | 0.46 | 0.38 |
| Colliery Methane | 0.06 | 0.05 | 0.05 | 0.04 | 0.04 | 0.03 | 0.03 | 0.03 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| Gas/Diesel Oil | 16.40 | 16.44 | 16.65 | 17.11 | 17.80 | 18.04 | 19.18 | 19.48 | 19.47 | 19.31 | 19.30 | 19.95 | 20.67 |
| Gas Works Gas | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Jet Gasoline | 1.63 | 1.33 | 1.33 | 1.40 | 1.37 | 1.38 | 1.38 | 1.33 | 1.34 | 1.31 | 1.26 | 1.31 | 1.37 |
| Liquefied Petroleum Gas | 0.88 | 1.02 | 0.91 | 0.93 | 0.99 | 0.97 | 1.09 | 1.00 | 1.00 | 0.96 | 0.92 | 1.01 | 0.88 |
| Lubricants | 0.28 | 0.26 | 0.27 | 0.28 | 0.28 | 0.31 | 0.30 | 0.30 | 0.28 | 0.27 | 0.28 | 0.28 | 0.29 |
| Motor Gasoline | 20.79 | 20.54 | 20.56 | 20.32 | 19.53 | 18.77 | 19.16 | 19.02 | 18.68 | 18.63 | 18.47 | 17.90 | 16.89 |
| Municipal Solid Waste | 0.22 | 0.22 | 0.22 | 0.22 | 0.22 | 0.22 | 0.22 | 0.14 | 0.20 | 0.19 | 0.20 | 0.22 | 0.24 |
| Naphtha | 0.02 | 0.03 | 0.03 | 0.02 | 0.02 | 0.00 | 0.00 | 0.02 | 0.02 | 0.02 | 0.02 | 0.05 | 0.03 |
| Natural Gas | 30.35 | 32.65 | 32.81 | 36.94 | 39.46 | 42.17 | 48.91 | 49.90 | 52.19 | 55.53 | 57.13 | 57.03 | 56.47 |
| Non-Fuel Combustion | 10.63 | 9.83 | 9.56 | 9.09 | 9.66 | 8.95 | 9.15 | 9.01 | 9.07 | 8.75 | 8.17 | 7.95 | 7.45 |
| Non IPCC | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.07 |
| Orimulsion | 0.09 | 0.25 | 0.77 | 0.85 | 0.73 | 0.76 | 0.52 | 0.11 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Other Bituminous Coal | 56.80 | 56.57 | 53.31 | 45.02 | 41.91 | 39.35 | 36.17 | 31.33 | 31.58 | 27.41 | 28.97 | 32.63 | 30.29 |
| Other Kerosene | 1.77 | 2.05 | 2.12 | 2.25 | 2.28 | 2.38 | 2.87 | 2.87 | 3.07 | 3.12 | 3.30 | 3.65 | 3.31 |
| Other Oil | 0.02 | 0.03 | 0.04 | 0.05 | 0.09 | 0.09 | 0.11 | 0.07 | 0.06 | 0.07 | 0.09 | 0.11 | 0.11 |
| Patent Fuel | 0.94 | 0.93 | 0.84 | 0.90 | 0.75 | 0.60 | 0.67 | 0.50 | 0.50 | 0.45 | 0.41 | 0.38 | 0.33 |
| Petroleum Coke | 0.78 | 0.83 | 1.28 | 1.38 | 1.39 | 1.62 | 1.76 | 1.72 | 1.55 | 1.35 | 1.27 | 0.93 | 1.09 |
| Refinery Gas | 2.39 | 2.37 | 2.35 | 2.52 | 2.61 | 2.73 | 2.74 | 2.53 | 2.40 | 2.36 | 2.39 | 2.27 | 2.03 |
| Residual Fuel Oil | 11.78 | 11.68 | 10.59 | 9.95 | 8.92 | 7.66 | 7.04 | 5.31 | 4.54 | 3.78 | 2.80 | 3.45 | 3.24 |
| Scrap Tyres | 0.00 | 0.00 | 0.00 | 0.00 | 0.05 | 0.06 | 0.09 | 0.09 | 0.06 | 0.05 | 0.02 | 0.02 | 0.02 |
| Total emissions (ktC) | 164.6 | 165.7 | 161.6 | 157.4 | 156.3 | 153.8 | 159.3 | 152.6 | 153.5 | 151.1 | 152.2 | 155.8 | 150.3 |

Table 6.3:	Carbon emissions after introducing CEFs for solid, liquid and gaseous fuels (MtC) into the 2002 GHG inventory

| IPCC Fuel | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Anthracite | 0.97 | 1.32 | 1.06 | 1.57 | 1.62 | 1.19 | 1.13 | 1.05 | 0.79 | 0.73 | 0.75 | 0.91 | 0.66 |
| Aviation Gasoline | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.03 | 0.03 | 0.03 | 0.04 | 0.04 | 0.05 | 0.04 |
| Blast Furnace Gas | 4.92 | 4.85 | 4.54 | 4.55 | 4.72 | 4.91 | 5.16 | 5.32 | 5.11 | 4.87 | 4.54 | 3.78 | 3.36 |
| Coke Oven Coke | 1.38 | 1.04 | 1.16 | 0.9 | 0.7 | 0.46 | 0.5 | 0.25 | 0.27 | 0.62 | 0.64 | 0.7 | 0.48 |
| Coke Oven Gas | 0.72 | 0.66 | 0.6 | 0.57 | 0.56 | 0.54 | 0.6 | 0.58 | 0.58 | 0.54 | 0.55 | 0.48 | 0.41 |
| Coking Coal | 0.74 | 0.65 | 0.52 | 0.52 | 0.52 | 0.56 | 0.65 | 0.73 | 0.73 | 0.52 | 0.39 | 0.49 | 0.41 |
| Colliery Methane | 0.06 | 0.05 | 0.05 | 0.04 | 0.04 | 0.03 | 0.03 | 0.03 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| Gas/Diesel Oil | 16.57 | 16.62 | 16.82 | 17.29 | 17.98 | 18.23 | 19.37 | 19.69 | 19.68 | 19.49 | 19.48 | 20.14 | 20.86 |
| Gas Works Gas | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Jet Gasoline | 1.63 | 1.33 | 1.33 | 1.4 | 1.37 | 1.38 | 1.38 | 1.33 | 1.34 | 1.31 | 1.26 | 1.31 | 1.37 |
| Liquefied Petroleum Gas | 0.88 | 1.02 | 0.91 | 0.93 | 0.99 | 0.97 | 1.09 | 1 | 1 | 0.96 | 0.92 | 1.01 | 0.88 |
| Lubricants | 0.28 | 0.26 | 0.27 | 0.28 | 0.28 | 0.31 | 0.3 | 0.3 | 0.28 | 0.27 | 0.28 | 0.28 | 0.29 |
| Motor Gasoline | 20.79 | 20.54 | 20.56 | 20.32 | 19.53 | 18.77 | 19.16 | 19.02 | 18.68 | 18.63 | 18.47 | 17.9 | 16.89 |
| Municipal Solid Waste | 0.22 | 0.22 | 0.22 | 0.22 | 0.22 | 0.22 | 0.22 | 0.14 | 0.2 | 0.19 | 0.2 | 0.22 | 0.24 |
| Naphtha | 0.02 | 0.03 | 0.02 | 0.02 | 0.01 | 0 | 0 | 0.0.1 | 0.01 | 0.02 | 0.02 | 0.05 | 0.02 |
| Natural Gas | 29.39 | 31.67 | 31.9 | 36 | 38.53 | 41.26 | 47.94 | 49.01 | 51.37 | 54.87 | 56.24 | 56.2 | 55.7 |
| Non-Fuel Combustion | 10.63 | 9.83 | 9.56 | 9.09 | 9.66 | 8.95 | 9.15 | 9.01 | 9.07 | 8.75 | 8.17 | 7.95 | 7.45 |
| Non IPCC | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.07 |
| Orimulsion | 0.09 | 0.25 | 0.77 | 0.85 | 0.73 | 0.76 | 0.52 | 0.11 | 0 | 0 | 0 | 0 | 0 |
| Other Bituminous Coal | 58.41 | 58.69 | 55.03 | 46.55 | 43.32 | 40.02 | 37.4 | 32.72 | 33.28 | 28.66 | 30.32 | 34.5 | 32.25 |
| Other Kerosene | 1.77 | 2.05 | 2.12 | 2.25 | 2.28 | 2.38 | 2.87 | 2.87 | 3.07 | 3.12 | 3.3 | 3.65 | 3.31 |
| Other Oil | 0.02 | 0.03 | 0.04 | 0.05 | 0.09 | 0.09 | 0.11 | 0.07 | 0.06 | 0.07 | 0.09 | 0.11 | 0.11 |
| Patent Fuel | 0.94 | 0.93 | 0.84 | 0.9 | 0.75 | 0.6 | 0.67 | 0.5 | 0.5 | 0.45 | 0.41 | 0.38 | 0.33 |
| Petroleum Coke | 0.91 | 0.97 | 1.44 | 1.55 | 1.55 | 1.81 | 1.95 | 1.91 | 1.74 | 1.53 | 1.44 | 1.04 | 1.22 |
| Refinery Gas | 2.42 | 2.4 | 2.38 | 2.54 | 2.63 | 2.75 | 2.77 | 2.56 | 2.43 | 2.38 | 2.41 | 2.29 | 2.05 |
| Residual Fuel Oil | 12.02 | 11.93 | 10.78 | 10.13 | 9.08 | 7.8 | 7.16 | 5.43 | 4.66 | 3.89 | 2.89 | 3.55 | 3.35 |
| Scrap Tyres | 0 | 0 | 0 | 0 | 0.05 | 0.06 | 0.09 | 0.09 | 0.06 | 0.05 | 0.02 | 0.02 | 0.02 |
| Total emissions (ktC) | 165.9 | 167.4 | 163.0 | 158.6 | 157.3 | 154.2 | 160.3 | 153.8 | 155.0 | 152.1 | 152.9 | 157.1 | 151.8 |

Table 6.3:	Percentage change in carbon emissions, according to fuel, after introducing the revised CEFs into the 2002 GHG inventory

| IPCC Fuel | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Anthracite | 0.00 | 0.00 | -0.29 | -0.29 | 0.00 | 1.19 | 0.89 | 0.89 | 1.49 | -0.30 | 0.00 | 0.89 | 0.89 |
| Aviation Gasoline | -1.38 | -1.38 | -1.38 | -1.38 | -1.38 | -1.38 | -1.38 | -1.38 | -1.39 | -1.39 | -1.39 | -1.39 | -1.39 |
| Blast Furnace Gas | 18.90 | 18.90 | 18.90 | 18.90 | 18.90 | 18.90 | 18.90 | 18.90 | 18.91 | 18.91 | 18.91 | 18.91 | 18.91 |
| Coke Oven Coke | -36.09 | -42.51 | -38.47 | -44.53 | -51.70 | -62.81 | -62.17 | -77.11 | -74.72 | -55.60 | -53.04 | -46.24 | -52.72 |
| Coke Oven Gas | -24.18 | -24.20 | -24.19 | -24.14 | -24.17 | -24.03 | -23.12 | -23.77 | -24.12 | -24.33 | -24.62 | -24.28 | -24.24 |
| Coking Coal | 63.22 | 68.63 | 84.31 | 78.27 | 75.87 | 59.96 | 104.92 | 84.47 | 85.50 | 6.59 | 4.45 | 6.03 | 6.31 |
| Colliery Methane | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Gas/Diesel Oil | 1.03 | 1.07 | 1.04 | 1.04 | 4.03 | 1.02 | 1.01 | 1.04 | 1.04 | 0.94 | 0.95 | 0.94 | 0.92 |
| Gas Works Gas | 0.00 | 0.00 | - | - | - | - | - | - | - | - | - | - | - |
| Jet Gasoline | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Liquefied Petroleum Gas | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Lubricants | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Motor Gasoline | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Municipal Solid Waste | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Naphtha | -9.14 | -9.14 | -9.14 | -9.14 | -9.14 | - | - | -9.14 | -9.15 | -9.15 | -9.15 | -9.15 | -9.15 |
| Natural Gas | -3.16 | -3.00 | -2.76 | -2.55 | -2.36 | -2.15 | -1.98 | -1.78 | -1.57 | -1.19 | -1.54 | -1.45 | -1.35 |
| Non-Fuel Combustion | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Non IPCC | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Orimulsion | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | - | - | - | - | - |
| Other Bituminous Coal | 2.82 | 3.74 | 3.21 | 3.40 | 3.36 | 1.69 | 3.40 | 4.43 | 5.40 | 4.55 | 4.68 | 5.73 | 6.45 |
| Other Kerosene | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Other Oil | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Patent Fuel | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Petroleum Coke | 16.25 | 16.25 | 12.70 | 12.23 | 11.27 | 11.41 | 10.64 | 11.34 | 12.05 | 13.44 | 12.78 | 11.38 | 11.18 |
| Refinery Gas | 1.044 | 1.044 | 1.044 | 1.044 | 1.044 | 1.044 | 1.044 | 1.044 | 1.04 | 1.04 | 1.04 | 1.04 | 1.04 |
| Residual Fuel Oil | 2.00 | 2.11 | 1.76 | 1.76 | 1.76 | 1.76 | 1.76 | 2.11 | 2.47 | 2.82 | 2.94 | 2.94 | 3.18 |
| Scrap Tyres | - | - | - | - | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Overall % change | 0.76% | 1.06% | 0.86% | 0.74% | 0.67% | 0.22% | 0.60% | 0.78% | 1.02% | 0.62% | 0.50% | 0.86% | 1.02% |

# References

Baggott, S. L., Brown, L., Milne, R., Murrells, T.P., Passant, N. and Watterson, J.D. (2004). UK Greenhouse Gas Inventory, 1990 to 2002.   Annual Report for submission under Framework Convention on Climate Change. National Environmental Technology Centre, AEA Technology Centre, AEAT/ENV/R/1702.

British Gas (1992) Personal communication with Geoff Salway.

British Coal (1989) Personal communication with Simon Eggleston.  British Coal Research, Bretby, Derbyshire, UK.

Corus (2004) Personal communication.  Peter Quinn.

DTI DUKES (2004), Digest of United Kingdom Energy Statistics 2004, TSO, London 123 Kingsway London, WC2B 6PQ, UK.

IPCC, (1997a), IPCC Revised 1996 Guidelines for National Greenhouse Gas Inventories, Volume 1, Greenhouse Gas Inventory Reporting Instructions, IPCC WGI Technical Support Unit, Hadley Centre, Meteorological Office, Bracknell, UK.

IPCC, (1997b), IPCC Revised 1996 Guidelines for National Greenhouse Gas Inventories, Volume 2, Greenhouse Gas Inventory Workbook, IPCC WGI Technical Support Unit, Hadley Centre, Meteorological Office, Bracknell, UK.

IPCC, (1997c), IPCC Revised 1996 Guidelines for National Greenhouse Gas Inventories, Volume 3, Greenhouse Gas Inventory Reference Manual, IPCC WGI Technical Support Unit, Hadley Centre, Meteorological Office, Bracknell, UK.

IPCC, (2000), Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories, ed. Penman, J, Kruger, D, Galbally, I, et al, IPCC National Greenhouse Gas Inventories Programme, Technical Support Programme Technical Support Unit, Institute for Global Environmental Strategies, Hayama, Kanagawa, Japan.

Joslin, A., Rodgers, I., Salway, G. and Tabberer, R. (2004) Relating fuel consumption to CO2 emissions.  Review and recommendations for power station fuel and plant parameters for use in the DTI power station model.  AEP Report EncC 159/04.

Transco (2004a) Measurement and Process Report: MPR046.  Estimation of Carbon Dioxide Emission Factors for UK Natural Gas 1990-2003.  Safety & Engineering report.  Dave Lander, Standards Manager (Gas Quality), UK Distribution, National Grid (formerly Transco), National Grid House, Warwickshire, CV34 6DA, UK.

Transco (2004b) Measurement and Process Report: MPR039.  Estimation of Carbon Dioxide Emission Factors for UK Natural Gas 2002 and 2003.  Safety & Engineering report.  Dave Lander, Standards Manager (Gas Quality), UK Distribution, National Grid (formerly Transco), National Grid House, Warwickshire, CV34 6DA, UK.

Quick (2004).  Personal communication.  Will Quick, Joint Environmental Programme (JEP), funded by the Association of Electricity Producers (AEP).  Power Technology, Ratcliffe on Soar, Nottingham, NG11 0EE, UK.

Rushworth (2004) Personal communication.  Jim Rushworth, National Energy Manager, Lafarge Cement UK, East Aberthaw, Barry, Vale of Glamorgan, Wales, UK.

Simmons (2002) Review of the UK Greenhouse Gas Inventory CO2 from Fuel Combustion 1990-1999.  Avonlog Ltd.

UKPIA (2004) Carbon Content Of Fuels Used In The UK.  Ian McPherson, Assistant Director, UK Petroleum Industry Association, 9 Kingsway, London, WC2B 6XF, UK.

UKPIA (1989) Personal communication with Simon Eggleston.  UK Petroleum Industry Association, 9 Kingsway, London, WC2B 6XF, UK.

# Acknowledgements

We are grateful for the help of the following people during the course of this work programme; apologies for any omissions.

- Achur, James (UK DTI)

- Allen, Chris (TRANSCO - now National Grid UK)

- Austwick, Caroline (Defra)

- Boyle, Stephen (SEPA)

- Cunningham, Mike (SEPA)

- Dodwell, Chris (UK Defra)

- Donaldson, Susan (UK Defra, Climate, Energy, Science and Analysis)

- Gemill, Rob (EA)

- Hitchin, Roger (Buildings Research Establishment, BRE)

- JEP programme members

- Jones, Haydn (Netcen, EA)

- Lampert, Roger (DTI)

- Lander, Dave (TRANSCO - now National Grid UK)

- Maier, Margaret (DTI)

- McGlen, Chris (UK Coal)

- McPherson, Ian (UKPIA)

- Penman, Jim (UK Defra, Climate, Energy, Science and Analysis)

- Prior, Phil (Defra)

- Pryor, Phil (TRANSCO - now National Grid UK)

- Quick, Will (Joint Environmental Programme, E.ON Group)

- Rushworth, Jim (Lafarge Cement)

- Salway, Geoff (RWE Innogy plc)

- Tauhid, Sayeeda (UK Defra)

- Watson, Malcolm (UKPIA)

- Wilson, Dave (DTI)

- Wilson, Dave (UK DTI)

- Young, Martin (UK DTI)

# Appendix 1

## A1.1	Bunker Emissions

International shipping and aviation emissions are not included in national total emissions, but are submitted to the IPCC annually.  Modifications to the CEFs will affect the emissions from fuels consumed by international shipping and aviation, and for completeness, the revised bunker emissions are presented in this Appendix.

Table A1.1.1:	International bunker emissions (MtC)

| IPCC | IPCC Fuel | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Emissions in 2002 Greenhouse Gas Inventory |  |  |  |  |  |  |  |  |  |  |  |  |
| International Marine | Gas/Diesel Oil | 0.59 | 0.63 | 0.70 | 0.66 | 0.75 | 0.64 | 0.72 | 0.66 | 0.96 | 0.73 | 0.72 |
| International Marine | Residual Fuel Oil | 1.18 | 1.09 | 1.11 | 1.12 | 0.94 | 1.15 | 1.24 | 1.53 | 1.43 | 1.00 | 0.79 |
| Aviation Bunkers | Jet Gasoline | 4.03 | 3.97 | 4.39 | 4.70 | 4.88 | 5.20 | 5.53 | 5.89 | 6.59 | 7.22 | 8.02 |
| Emissions in Carbon Emissions Factor modified Greenhouse Gas Inventory |  |  |  |  |  |  |  |  |  |  |  |  |
| International Marine | Gas/Diesel Oil | 0.60 | 0.64 | 0.70 | 0.67 | 0.73 | 0.65 | 0.73 | 0.67 | 0.97 | 0.74 | 0.74 |
| International Marine | Residual Fuel Oil | 1.21 | 1.12 | 1.13 | 1.14 | 0.96 | 1.17 | 1.26 | 1.56 | 1.46 | 1.03 | 0.82 |
| Aviation Bunkers | Jet Gasoline | 4.03 | 3.97 | 4.39 | 4.70 | 4.88 | 5.20 | 5.53 | 5.89 | 6.59 | 7.22 | 8.02 |

## A1.2	Gross Calorific Values

Table A1.2.1: Gross Calorific Values for coal (GJ per tonne)

| Source | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Agriculture | 28.9 | 28.9 | 28.9 | 28.7 | 28.8 | 29 | 28.9 | 29.1 | 28.5 | 28.9 | 29.2 | 29.8 | 28.5 |
| Autogenerators | 27.8 | 27.8 | 27.8 | 28.1 | 28.8 | 27.7 | 27.3 | 27 | 26.9 | 26.6 | 26.8 | 26.7 | 26.5 |
| Cement (fuel combustion) | 27.8 | 27.8 | 27.8 | 28.1 | 28.8 | 27.7 | 27.3 | 27 | 26.9 | 26.6 | 26.8 | 30.4 | 29.6 |
| Coke Production | 31.2 | 31.4 | 31.4 | 31.5 | 31.4 | 31.4 | 32 | 32 | 32 | 30.5 | 30.4 | 30.5 | 30.5 |
| Collieries | 28.6 | 28 | 26.8 | 26.8 | 28.2 | 28.2 | 26.2 | 27.8 | 29.6 | 29.3 | 29.6 | 29.8 | 29.7 |
| Domestic | 30.2 | 30.2 | 30.2 | 30.2 | 29.8 | 30.4 | 30.6 | 30.6 | 30.9 | 30.9 | 30.9 | 30.9 | 31.1 |
| Iron & Steel (blast furnaces) | 31.2 | 31.4 | 31.5 | 31.5 | 31.4 | 31.4 | 32 | 32 | 32 | 30.5 | 30.4 | 30.5 | 30.5 |
| Iron & Steel (combustion) | 28.9 | 28.9 | 28.9 | 28.7 | 29.4 | 31.5 | 31.3 | 31.3 | 31.3 | 30.7 | 30.7 | 29.4 | 30.4 |
| Lime Production (combustion) | 27.8 | 27.8 | 27.8 | 28.1 | 28.8 | 27.7 | 27.3 | 27 | 26.9 | 26.6 | 26.8 | 26.7 | 26.5 |
| Miscellaneous | 27.5 | 27.5 | 27.5 | 28.7 | 28.8 | 28.9 | 30.4 | 29.3 | 29.2 | 29.1 | 29.2 | 29.2 | 30.1 |
| Other Industry (combustion) | 27.8 | 27.8 | 27.8 | 28.1 | 28.8 | 27.7 | 27.3 | 27 | 26.9 | 26.6 | 26.8 | 26.7 | 26.7 |
| Public Services | 27.5 | 27.5 | 27.5 | 28.7 | 28.8 | 28.9 | 30.4 | 29.3 | 29.2 | 29.1 | 29.2 | 29.2 | 30.1 |
| Railways (stationary sources | 27.5 | 27.5 | 27.5 | 28.7 | 28.8 | 28.9 | 30.4 | 29.3 | 29.2 | 29.1 | 29.2 | 29.2 | 30.1 |
| SSF Production | 29.2 | 29.5 | 29.4 | 29.5 | 29.4 | 29.9 | 30 | 30.4 | 30.5 | 30.1 | 30.3 | 30.3 | 30.5 |
| Town Gas Production | 31.2 | 31.5 | 31.4 | 31.5 | 31.4 | 31.4 | 32 | 32 | 32 | 30.5 | 30.4 | 30.5 | 30.5 |
| Power Stations | 24.8 | 25 | 25 | 25 | 24.8 | 25.4 | 25.4 | 25.6 | 25.8 | 25.9 | 25.9 | 25.9 | 26.1 |

Table A1.2.2:	Coal GCV for the cement industry (GJ per tonne)

| SOURCE | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cement (combustion) | 28.2 | 28.2 | 28.2 | 29.5 | 28.3 | 27.1 | 27.4 | 27 | 26.6 | 26.7 | 27 | 27 | 27 | 27.9 | 27.7 |

- 1990-93 and 1994-99 - DUKES GCV - Mineral products

- 1994 - average of 93 and 95 DUKES data

- 2000-2004 - Data supplied by Cement industry

Table A1.2.3:	Coke GCV (GJ per tonne)

| 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29.8 | 29.8 | 29.8 | 29.8 | 29.8 | 29.8 | 29.8 | 29.8 | 29.8 | 29.8 | 29.8 | 29.8 | 29.8 |

Table A1.2.4:	Petroleum Coke GCV (GJ per tonne)

| 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 39.5 | 39.5 | 39.5 | 39.5 | 39.5 | 39.5 | 39.5 | 39.5 | 39.5 | 39.5 | 39.5 | 39.5 | 39.5 |

Table A1.2.5:	Liquid fuel GCVs (GJ per tonne)

| Fuel | Dukes class | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Gas oil | Gas/Diesel Oil (incl DERV) | 45.4 | 45.4 | 45.4 | 45.4 | 45.4 | 45.4 | 45.4 | 45.4 | 45.5 | 45.6 | 45.6 | 45.6 | 45.6 |
| Fuel oil | Fuel Oil | 43.2 | 43.2 | 43.2 | 43.2 | 43.2 | 43.2 | 43.2 | 43.3 | 43.2 | 43.2 | 43.1 | 43.5 | 43.4 |
| Petrol | Motor Spirit | 47 | 47 | 47 | 47 | 47 | 47 | 47 | 47 | 47 | 47.1 | 47 | 47.1 | 47.1 |
| DERV | Gas/Diesel Oil (incl DERV) | 45.4 | 45.4 | 45.4 | 45.4 | 45.4 | 45.4 | 45.4 | 45.4 | 45.5 | 45.6 | 45.6 | 45.6 | 45.6 |
| Aviation Spirit | Aviation Spirit & Wide Cut Gasoline | 47.3 | 47.3 | 47.3 | 47.3 | 47.3 | 47.3 | 47.3 | 47.3 | 47.3 | 47.3 | 47.3 | 47.3 | 47.3 |
| Aviation Turbine fuel | Aviation Turbine Fuel | 46.2 | 46.2 | 46.2 | 46.2 | 46.2 | 46.2 | 46.2 | 46.2 | 46.2 | 46.2 | 46.2 | 46.2 | 46.2 |

Table A1.2.6:	Natural Gas GCVs from DUKES and TRANSCO (2004) (MJ/m3)

| Year | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 | 2003 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DUKES | 37.97 | 37.80 | 38.52 | 38.80 | 39.09 | 39.6 | 39.1 | 39.1 | 39.1 | 39.5 | 39.4 | 39.8 | 39.8 | 39.6 |
| Transco | 38.61 | - | - | - | 38.89 | - | 39.27 | - | 39.52 | 39.76 | 39.41 | - | 39.51 | 39.5 |

## A1.3	Conversion Factors for Gross to Net Energy Consumption

Table A1.3.1 displays the conversion factors used for Gross to Net Energy consumption as listed in the UK Greenhouse Gas Inventory 1990-2002, (Baggott et al, 2004).

Table A1.3.1:	Conversion factors for Gross to Net energy consumption

| - Fuel | - Conversion factor |
| --- | --- |
| - Other Gaseous Fuels | - 0.9 |
| - Solid and Liquid Fuels | - 0.95 |
| - LPG and OPG | - 0.92 |
| - Blast Furnace Gas | - 1.0 |

# Appendix 2

## A2.1	Review of fractions of carbon left unoxidised after combustion in liquid fuels

Background

Emissions of carbon can be estimated from the quantities of fuel burnt, their carbon contents, and the fraction of carbon that is not oxidised during the combustion.

The current assumption in the UK Greenhouse Gas inventory is that 100% of the liquid fuels and gaseous fuels are oxidised.  The EU ETS default is 99.5%.

Review

To review the assumptions currently used, we have

- contacted the UK Petroleum Industries Association (UKPIA) to seek their view on the appropriate oxidation factors

- considered the views of experts involved in the mobile combustion chapter of the 2006 IPPC guidelines

- consulted with our transport sector, Tim Murrells, for his views on the mobile combustion

Response from UKPIA

From an analysis of data provided by Ian McPherson of UKPIA, Malcolm Watson, UKPIA, has suggested that for large combustion plant in refineries the carbon oxidation is 99.9%.  This analysis ignores any soot deposited in and around the combustion chamber (JW - but this is likely to negligible).

Views of authors involved in the 2006 IPCC GLs

A table of oxidation factors were produced for the IPCC 2006 GLs by Kainou Kazanuri from a range of sources, including the IPCC EF database, and data from the IEA.  This covered all the IPCC fuels.

Jos Olivier (RIVM, Netherlands) makes the point that in general the non-oxidised fraction (i.e. that remains as particles following combustion) is very small, and is much smaller than the uncertainty in the emission factors and certainly much smaller than the accuracy of the activity data.  He considers this correction term more of an 'academic' character (i.e. in principle OK), than of an topic of any significance for accurately estimating CO2 emissions.  Jos recommended to apply values other than 1 in cases where there is a significant fraction carbon non-oxidised.

Greg Marland (Oak Ridge National Laboratory) notes that there is an analysis within the US EPA that suggests 100% oxidation of fuel for modern automobiles.  Greg notes that he does not know if this assumption can be extended to the full mobile sector.

View of Tim Murrells (NAEI transport sector expert)

This is an issue we regularly have to consider when calculating fuel consumption and CO2 factors for road transport by specific vehicle types.

When data sources provide fuel consumption factors in g fuel/km for different fuels and types of vehicles, they actually derive these from measurements of CO2 tailpipe emissions and work back to calculate fuel consumption from measured tailpipe CO2 emissions and the measured tailpipe emissions of other carbon containing species CO, hydrocarbons and PM.  There are text-book equations to do this using the carbon contents of these other species.  These are based on the assumption that all PM emitted from vehicle exhausts is in the form of elemental carbon and they assume an average carbon content of the emitted hydrocarbons.

The terminology that is used is 'tailpipe CO2' (the amount of CO2 actually emitted) and 'ultimate CO2', (the amount that has the potential of forming CO2 based on the total amount of carbon emitted).  It is ultimate CO2 that is related to fuel consumption rates.

The actual amount of carbon emitted in forms other than CO2 is time-variable, depending on the make-up of the fleet.  Old petrol cars without three-way catalysts emitted a significant amount of carbon as CO and hydrocarbons.  But with improvements in engines and abatement technology, this has reduced significantly.  Recent test data we have just been analysing for DfT shows that for modern cars, about 99.4 - 99.7% of carbon is emitted as CO2 from the tailpipe of petrol cars and 99.6 - 99.9% of carbon is emitted as CO2 from diesel cars.

This doesn't make any difference to us if it is assumed (as we do) that regardless of what the carbon is emitted as it has the potential (and will) form CO2 in the atmosphere.  For CO and hydrocarbons that is a fair assumption, but I have never been comfortable with the idea that emitted PM can potentially form CO2 and to this end I think we ought to subtract the amount of exhaust PM emitted as carbon (from our calculation of PM emissions) from DUKES fuel consumption stats.

The 0.3-0.6% of carbon that is not emitted as CO2 from petrol vehicles is virtually all emitted as CO and HC, so there are no worries here in using the DUKES figures and carbon content factors as this means all the carbon in the fuel is emitted in a form that will go on to form CO2.  For diesel vehicles, though, a significant amount of the 0.1-0.4% of carbon that is not emitted as CO2 is emitted as PM, so that ought to be taken into account in the calculations from DUKES figures on diesel consumption.

Some calculations I've done show that for modern vehicles, <0.01% of carbon in petrol is emitted as PM and 0.05-0.1% of carbon in diesel is emitted as PM.  I should stress though the figures for diesel are time-varying, as cleaner engines and technologies penetrate the fleet, the amount of PM emitted is decreasing.  In 1990, the allowance for diesel emitted PM in the carbon calculations will be higher.  Also, don't take this figure for diesel as generic - for gas-oil consumption in diesel trains, machinery, shipping etc, these are less well-designed engines with no PM abatement, and a more significant fraction of carbon in the diesel consumed by these engines will be emitted as PM, so a bigger correction in the DUKES figures ought in theory to be made.

I could develop a "fleet-weighted" correction factor based on diesel PM emissions that can be applied to the road transport CO2 calculations from DUKES diesel consumption, but it depends what you intend to do with the information.  There could be a tricky issue in that the PM emissions for road transport are calculated from traffic data and the carbon emissions being calculated from DUKES figures exclude the contribution of 'fuel tourism' vehicles that contribute to the UK traffic and therefore contribute to our PM inventory, but not our carbon inventory.

Recommendation

Our view is that the fraction oxidised for liquid fuels should remain at 100% across the time series, primarily because of the uncertainty in the estimates of the fractions of fuels unoxidised and the difficulties of generating an accurate and defensible time series of unoxidised fractions for each fuel used in the GHG inventory from 1990 to 2004.

Adopting this approach will mean that the base year emissions will be unaffected.  This will result in a slightly conservative estimate of carbon emissions – possibly more so in 1990 where combustion technology was not so advanced and the fraction of carbon unoxidised would have been slightly greater.

# Appendix 3

## A3.1	Project Meetings

| Date | Meeting | Attendees |
| --- | --- | --- |
| 01.07.2004 | Gas emission factors and calorific values for use in ETS and NAEI Defra, Ashdown House, London | Jim Penman Defra Sayeeda Tauhid Defra Caroline Doble Defra Sarah Baggott Defra/AEAT-netcen John Watterson AEAT-netcen Howard Rudd AEAT-FES David Wilson DTI Rob Gemmill EA Dave Lander Transco Chris Allen Transco Phil Pryor NG Transco Mike Cunningham SEPA |
| 10.08.2004 | Natural Gas Calorific Values and Emission Factors for Use in the NAEI and the EU ETS Defra, Ashdown House, London | DEFRA – Chris Dodwell, Caroline Austwick, Sayeeda Tauhid NETCEN – John Watterson, Sarah Baggott FES – Philip Wright Transco – Dave Lander, Phil Prior DTI – Margaret Maier, Roger Lampert EA – Haydn Jones SEPA – Mike Cunningham, Stephen Boyle |

Emissions from fuels combusted by the fuel extraction or energy producing industries.