UK air quality modelling for annual reporting 2004 on ambient air quality assessment under Council Directives 96/62/EC and 2002/3/EC relating to ozone in ambient air

A report to the Department for Environment Food and Rural Affairs, Welsh Assembly Government, The Scottish Executive and the Department of the Environment for Northern Ireland Tony Bush Jaume Targa John Stedman

AEAT/ENV/R/2053 January 2006

| Title | UK air quality modelling for annual reporting 2004 on ambient air quality assessment under Council Directives 96/62/EC and 2002/3/EC relating to ozone in ambient air |
| --- | --- |

| Customer | The Department for Environment, Food and Rural Affairs, Welsh Assembly Government, The Scottish Executive and the Department of the Environment for Northern Ireland |
| --- | --- |

| Customer reference | CPEA15 |
| --- | --- |

| Confidentiality, copyright and reproduction | Copyright AEA Technology plc All rights reserved. Enquiries about copyright and reproduction should be addressed to the Commercial Manager, AEA Technology plc. |
| --- | --- |

| File reference | ED47154004 DD3_mapsresp2004v1.doc |
| --- | --- |

| Reference number | AEAT/ENV/R/2053 |
| --- | --- |

Address for	Netcen

Correspondence	B551 Harwell

Didcot

Oxon

OX11 0QJ

Telephone	0870 190 6423

Facsimile 	0870 190 6607

Tony.bush@aeat.co.uk

netcen is a operating division of AEA Technology plc

netcen is certificated to ISO9001 & ISO 14001

|  | Name | Signature | Date |
| --- | --- | --- | --- |
| Author | Tony Bush Jaume Targa John R. Stedman |  |  |
| Reviewed by | John R. Stedman |  |  |
| Approved by | Garry D Hayman |  |  |

Executive Summary

Directive 96/62/EC on Ambient Air Quality Assessment and Management (the Framework

Directive) establishes a framework under which the EU sets limit values or target values for the concentrations of specified air pollutants. Directive 2002/3/EC (the third Daughter Directive) sets target values and long-term objectives to be achieved for ozone.

2004 is the first year for which an annual air quality assessment for the third Daughter Directive pollutants is required. A questionnaire has been completed for submission to the EU containing the results of this air quality assessment along with those required for the first and second Daughter Directives. The assessment takes the form of comparisons of measured and modelled air pollutant concentrations with the target values and long-term objectives set out in the Directives. Air quality modelling has been carried out to supplement the information available from the UK national air quality monitoring networks.

This report provides a summary of key results from the questionnaire and additional technical information on the modelling methods that have been used to assess ozone throughout the UK. This includes:

- Details of modelling methods

- Information on the verification of the models used and comparisons with data quality objectives

- Detailed modelling results and comparison with target values and long-term objectives.

Maps of background ozone concentrations in 2004 on a 1 km x 1 km grid have been prepared. The following metrics set out by the third Daughter Directive have been modelled;

- Number of days above 120 gm-3 in 2004

- Number of days above 120 gm-3 per year averaged over three years 2002-2004

- AOT40 wheat crops in 2004

- AOT40 wheat crops averaged over five years 2000-2004

The models used in this assessment have been selected based on a critical appraisal of the techniques available within the UK.

The UK has been divided into 43 zones for air quality assessment. There are 28 agglomeration zones (large urban areas) and 15 non-agglomeration zones. The status of the zones status has been determined from a combination of monitoring data and model results. The results of this assessment are summarised in Tables E1 and E2 in terms of exceedences of Target Values (TV) and Long-term Objectives (LTO).

Table E1 Summary results of air quality assessment relative to the Target Values for ozone for 2010

| Target Value | Number of zones exceeding |
| --- | --- |
| Max Daily 8-hour mean Target Value | none |
| AOT40 Target Value | none |

Table E2 Summary results of air quality assessment relative to the Long-term Objectives for ozone

| Long-term Objective | Number of zones exceeding |
| --- | --- |
| Max Daily 8-hour mean Long-term Objective | 43 zones (36 measured + 7 modelled) |
| AOT40 Long-term Objective | 7 zones (5 measured + 2 modelled) |

Contents

1	Introduction	1

1.1	The Framework and Daughter Directives	1

1.2	This report	1

1.3	Preliminary assessments and definition of zones	2

1.4	Monitoring sites	7

2	Mapping Methods	8

2.1	Modelling the number of days exceeding  120 g m-3 metric	8

2.1.1	Verification of mapped number of days > 120 g m-3 values	13

2.1.2	Detailed comparison of modelling results with Target Values and  Long-term Objectives	14

2.2	Modelling the AOT40 wheat crops metric	16

2.2.1	Verification of mapped AOT40 values	20

2.2.2	Detailed comparison of modelling results with limit values	21

3	Lists of zones in relation to Target Values and Long-term objectives	23

3.1	Results for 2004	23

3.2	measured exceedences of Target Values and long-term objectives	25

4	Acknowledgements	36

Appendices

Appendix 1 	National Network Monitoring Sites

Appendix 2 	Monitoring Sites Used To Verify The Mapped Estimates

# Introduction

## The Framework and Daughter Directives

Directive 96/62/EC on Ambient Air Quality Assessment and Management (the Framework

Directive (Council Directive 96/62/EC)) establishes a framework under which the EU sets limit values or target values for the concentrations of specified air pollutants in ambient air. Directive 1999/30/EC (the first Daughter Directive, AQDD1 (Council Directive 1999/30/EC)) sets the limit values to be achieved for sulphur dioxide, nitrogen dioxide and oxides of nitrogen, particles and lead. Directive 2000/69/EC (the second Daughter Directive, AQDD2) sets out the limit values to be achieved for benzene and carbon monoxide. Directive 2002/3/EC (the third Daughter Directive, AQDD3) sets target values (TVs) and long-term objectives (LTOs) to be achieved for ozone.

The Framework Directive includes a requirement for Member States to undertake preliminary assessments of ambient air quality, prior to the implementation of the Daughter Directives under Article 5 this Directive. The objectives of these assessments are to establish estimates for the overall distribution and levels of pollutants, and to identify additional monitoring required to fulfil obligations within the Framework Directive. Reports describing the preliminary assessment for the UK for AQDD1, AQDD2 and AQDD3 have been prepared,,. Then Daughter Directives define the number of air quality monitoring sites required on the basis of the concentrations of pollutants and population statistics. The number of monitoring sites required is significantly reduced if other means of assessment, in addition to fixed monitoring sites, are also available.  Air quality modelling has therefore been carried out to supplement the information available from the UK national air quality monitoring networks and contribute to the assessments required by the Framework and subsequent Daughter Directives.

## This report

The first and second Daughter Directives make provision for an annual air quality assessment for NO2, PM10, SO2, CO and benzene. 2004 is the first year for which an annual air quality assessment is required for ozone as specified in the third Daughter Directive. This report details the results of annual air quality assessments undertaken to satisfy the UK’s obligation under the Daughter Directives and focuses on the modelling methodology for ozone. The air quality assessments for NO2, PM10, SO2, CO and benzene are covered in a separate report that can be found on the National Air Quality Archive.

This report focuses on the modelling methodology for reporting under the AQDD3. Section 2 describe the modelling procedures used for estimation of ozone. These include:

- Information on the calibration and verification of the models

- Background ambient concentration maps

- Detailed model results and identification of exceedences of TV

The following metrics relevant to the annual reporting of data to the Commission have been investigated:

- Number of days above 120 gm-3 in 2004

- Number of days above 120 gm-3 per year averaged over three years 2002-2004

- AOT40 wheat crops in 2004

- AOT40 wheat crops averaged over five years 2000-2004

The definitions of the metrics presented above and the Target Values and Long-term Objectives are given in Annex I of Directive. In addition, Annex II of the Directive presents Alert and Information Thresholds designed to inform the public and organisations representing sensitive population groups on occasions when there is increased a risk to human health from brief exposure to elevated levels of ozone. Annex I

and II are presented below.

## Preliminary assessments and definition of zones

The preliminary assessment carried for AQDD1 defined a set of zones to be used for air quality assessments in the UK based on population and urban areas data from the 1991 UK Census. These data have now been updated using information on population from the 2001 Census and land-use data from the Devolved Administrations. Updated zones are listed in Table 1.1 and illustrated in Figure 1.1. Information on the definition of zones is included in form 2 of the questionnaire. The zone codes listed in Table 1.1 are used throughout the questionnaire. The population and area of each zone is also shown. The zones are of two types: agglomeration zones (continuous urban areas with a population in excess of 250,000) and non-agglomeration zones. There are 28 agglomeration zones and 15 non-agglomeration zones, giving a total of 43 zones in the UK. The non-agglomeration zones in England correspond to the Government Office Regions, while those in Scotland, Wales and Northern Ireland were defined in conjunction with the Devolved Administrations.

The preliminary assessment for ozone also defined the monitoring and modelling requirements for each zone based on an assessment of concentrations in relation to target values ands long-term objectives specified by AQDD3. The minimum monitoring requirement for ozone and NOx in the majority of zones was found to be at least one monitoring site per zone, with the monitoring results to be supplemented with information from modelling studies.

Source; Directive 2002/3/EC

Source; Directive 2002/3/EC

Figure 1.1 UK zones and agglomerations for 2004

Table 1.1  Zones for AQDD3 reporting

| Zone | Zone code | Ag or nonag* | Area (km2) | Population |
| --- | --- | --- | --- | --- |
| Greater London Urban Area | UK0001 | ag | 1628 | 8278251 |
| West Midlands Urban Area | UK0002 | ag | 594 | 2284093 |
| Greater Manchester Urban Area | UK0003 | ag | 557 | 2244931 |
| West Yorkshire Urban Area | UK0004 | ag | 363 | 1499465 |
| Tyneside | UK0005 | ag | 217 | 879996 |
| Liverpool Urban Area | UK0006 | ag | 184 | 816216 |
| Sheffield Urban Area | UK0007 | ag | 165 | 640720 |
| Nottingham Urban Area | UK0008 | ag | 169 | 666358 |
| Bristol Urban Area | UK0009 | ag | 142 | 551066 |
| Brighton/Worthing/Littlehampton | UK0010 | ag | 97 | 461181 |
| Leicester Urban Area | UK0011 | ag | 102 | 441213 |
| Portsmouth Urban Area | UK0012 | ag | 91 | 442252 |
| Teesside Urban Area | UK0013 | ag | 111 | 365323 |
| The Potteries | UK0014 | ag | 91 | 362403 |
| Bournemouth Urban Area | UK0015 | ag | 113 | 383713 |
| Reading/Wokingham Urban Area | UK0016 | ag | 97 | 369804 |
| Coventry/Bedworth | UK0017 | ag | 76 | 336452 |
| Kingston upon Hull | UK0018 | ag | 80 | 301416 |
| Southampton Urban Area | UK0019 | ag | 77 | 304400 |
| Birkenhead Urban Area | UK0020 | ag | 88 | 319675 |
| Southend Urban Area | UK0021 | ag | 64 | 269415 |
| Blackpool Urban Area | UK0022 | ag | 63 | 261088 |
| Preston Urban Area | UK0023 | ag | 58 | 264601 |
| Glasgow Urban Area | UK0024 | ag | 366 | 1168270 |
| Edinburgh Urban Area | UK0025 | ag | 117 | 452194 |
| Cardiff Urban Area | UK0026 | ag | 72 | 327706 |
| Swansea Urban Area | UK0027 | ag | 84 | 270506 |
| Belfast Urban Area | UK0028 | ag | 193 | 580276 |
| Eastern | UK0029 | nonag | 19113 | 4850132 |
| South West | UK0030 | nonag | 23506 | 3980991 |
| South East | UK0031 | nonag | 18645 | 6016677 |
| East Midlands | UK0032 | nonag | 15491 | 3084598 |
| North West & Merseyside | UK0033 | nonag | 13149 | 2826622 |
| Yorkshire & Humberside | UK0034 | nonag | 14787 | 2514947 |
| West Midlands | UK0035 | nonag | 12192 | 2271650 |
| North East | UK0036 | nonag | 8282 | 1269803 |
| Central Scotland | UK0037 | nonag | 9305 | 1813314 |
| North East Scotland | UK0038 | nonag | 18587 | 1001499 |
| Highland | UK0039 | nonag | 38269 | 380062 |
| Scottish Borders | UK0040 | nonag | 11145 | 254690 |
| South Wales | UK0041 | nonag | 12221 | 1578773 |
| North Wales | UK0042 | nonag | 8368 | 720022 |
| Northern Ireland | UK0043 | nonag | 13579 | 1104991 |
| Total |  |  | 242698 | 59211755 |

* ag = agglomeration zone, nonag = non-agglomeration zone

## Monitoring sites

The monitoring stations operating during 2004 for the purpose of AQDD3 are listed in Table A1.1 in Appendix 1. This information is included in form 3 of the questionnaire.  Not all sites had sufficient data capture during 2004 for data to be reported. The data quality objective (DQO) for AQDD measurements is 90 % data capture. We have included all measurements with at least 75% data capture in the analysis in order to ensure that we can make maximum use of data from the monitoring sites operational during 2004 for reporting purposes. Table A1.2 in Appendix 1 lists the data capture rates for all monitoring stations used in the calibration of models for reporting under AQDD1-3.

Measurement data from monitoring stations not in the UK’s Automatic Urban and Rural Network have been used as an independent check on the performance of model outputs. These verification data have been sourced from Netcen’s Calibration Club customers and from the Hertfordshire & Bedfordshire Air Pollution Monitoring Network and the Kent and Medway Air Quality Monitoring Network and area widely regarded as high quality and reliable. Monitoring stations used in this verification process are presented in Table A2.1 of Appendix 2.

# Mapping Methods

This section of the report presents the methods used to map ozone throughout the UK. Following recommendations made by a study comparing the relative performance of available techniques for modelling ozone within the UK, an empirical mapping approach has been used for predicting ozone concentrations in 2004.

The empirical approaches draw upon measurements from the 85 monitoring stations in the AURN (2004) to produce functions describing ground-level ozone based upon wind velocity, topography and local emissions of NOx. These functions are capable of predicting ozone levels at a resolution of 1 x 1 km2 and the methods are briefly described in the following sections, full details can be sourced from the cited references. The methods are based upon those presented by Coyle et al, NEGTAP and PORG 1998.

## Modelling the number of days exceeding 
120 g m-3 metric

At rural locations in the UK exceedences of 120 g m-3 as a maximum daily 8-hour mean are broadly representative of wide spatial areas. As a result, measured exceedences from rural monitoring stations have been interpolated for the whole UK to represent the likely exceedences of this metric in the absence of NOx titration effect arising from emissions from combustion sources.

The resultant interpolated maps, however, will overestimate values in urban areas, where nitric oxide emissions from combustion sources deplete ozone concentrations. Hence, an urban influence term UI has been defined for monitoring stations in the AURN and the interpolated rural map to correct for the depletion of ozone in areas close to sources of NO. UI is defined as follows;

UI = (mapped rural – measured) / mapped rural

UI is closely related to annual mean NOx concentration. In this study, we have used the map of NOx concentrations in the UK provided by Stedman et al (2005). The relationship between UI and empirically modelled NOx concentrations for 2004 and 2002-2004 averaged metrics is presented in Figures 2.1 and 2.2 below. Ozone concentrations averaged over the three years 2002-2004 were typically higher than those in 2004 due to the photochemical episodes during 2003. The relationship between UI and NOx concentrationis therefore more clearly seen in Figure 2.2.

The intercept of the graphs represents the NOx concentration at which there is effectively no further UI. For 2004, the value of the intercept was determined empirically as 
4.63 g m-3. It has been assumed that value of the intercept for 2002-2004 averaged maps is also 4.63 g m-3.  A map of UI can subsequently be calculated for the UK at a 1 x 1 km2 resolution;

UI = gradient * ([NOx map]- concentration @ which UI is zero)

UI is then used to correct the interpolated rural days above 120 g m-3 maps;

Corrected days above 120 g m-3 map  = interpolated rural map * (1 – UI)

Maps of modelled number of days with maximum daily 8-hour mean zone concentrations greater than 120 g m-3 for comparison with the Target Value (2004) and Long-term Objective (averaged 2002 to 2004) are presented in Figures 2.3 and 2.4 respectively.

Figure 2.1 Relationship between UI 2004 and NOx concentration [NOx] (g m-3 as NO2)

Figure 2.2 Relationship between UI 2002-4 and NOx concentration [NOx] (g m-3 as NO2)

Figure 2.3  Estimated number of days above 120 gm-3, 2004

Figure 2.4  Estimated average number of days above 120 gm-3, 2002 to 2004

### Verification of mapped number of days > 120 g m-3 values

Figures 2.5 and 2.6 show comparisons of modelled and measured number of days with maximum daily 8-hour mean ozone concentrations greater than 120 g m-3 in 2004 and averaged 2002-4 at background locations. Both the national network sites used to calibrate the models and the verification sites are shown. Lines representing y = x – 50 % and y = x + 50% are also shown (this is the AQDD3 data quality objective for modelled ozone concentrations).

Figure 2.5  Verification of background number of days > 120 g m-3 model 2004

Figure 2.6  Verification of background number of days > 120 g m-3 model

2002 – 2004

Summary statistics for the comparison between modelled and measured ozone concentrations are listed in Table 2.1. Figures 2.5 and 2.6 show that the number of days with maximum daily 8-hour mean ozone concentrations greater than 120 g m-3 are systematically at the verification sites when compared with measurements from the AURN. This observation may be related to the majority of verification sites being located in the southeast of England which experiences generally higher levels of ozone when compared with the rest of the UK.

Table 2.1  Summary statistics for comparison between modelled and measured number of days exceeding 120 g m-3 as a maximum daily 8-hour mean

|  |  | Mean of measurements (days) | Mean of model estimates (days) | r2 | % outside data quality objectives | No. sites |
| --- | --- | --- | --- | --- | --- | --- |
| National Network | 2004 | 4.0 | 3.9 | 0.71 | 33 | 69 |
| Verification Sites | 2004 | 7.0 | 5.9 | 0.35 | 47 | 20 |
| National Network | 2002-4 | 6.4 | 6.5 | 0.71 | 23 | 61 |
| Verification Sites | 2002-4 | 10.3 | 8.5 | 0.48 | 18 | 21 |

### Detailed comparison of modelling results with Target Values and 
Long-term Objectives

The modelling results, in terms of a comparison of modelled concentrations with the TV and LTO by zone, are summarised in Table 3.3. These data have also been presented in form 19g of the questionnaire. Method D in this table refers to the modelling method described in this report.

Estimates of area and population exposed have been derived from the background maps only.

Table 2.2  Tabular results of and methods used for supplementary assessment (1999/30/EC Article 7(3) and Annex VIII(II), 2000/69/EC Article 5(3) and Annex VI(II) and 2002/3/EC Article 9(1) and Annex VII(II))

|  |  | Above TV for health |  |  |  | Above LTO for health |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Area |  | Population exposed |  | Area |  | Population exposed |  |
| Zone | Zone code | km2 | Method | Number | Method | km2 | Method | Number | Method |
| Greater London Urban Area | UK0001 | 0 | D | 0 | D | 1617 | D | 7728198 | D |
| West Midlands Urban Area | UK0002 | 0 | D | 0 | D | 594 | D | 2083891 | D |
| Greater Manchester Urban Area | UK0003 | 0 | D | 0 | D | 557 | D | 1846479 | D |
| West Yorkshire Urban Area | UK0004 | 0 | D | 0 | D | 363 | D | 1150737 | D |
| Tyneside | UK0005 | 0 | D | 0 | D | 217 | D | 714326 | D |
| Liverpool Urban Area | UK0006 | 0 | D | 0 | D | 184 | D | 696998 | D |
| Sheffield Urban Area | UK0007 | 0 | D | 0 | D | 165 | D | 521984 | D |
| Nottingham Urban Area | UK0008 | 0 | D | 0 | D | 169 | D | 558935 | D |
| Bristol Urban Area | UK0009 | 0 | D | 0 | D | 142 | D | 488798 | D |
| Brighton/Worthing/Littlehampton | UK0010 | 0 | D | 0 | D | 97 | D | 386373 | D |
| Leicester Urban Area | UK0011 | 0 | D | 0 | D | 102 | D | 374314 | D |
| Portsmouth Urban Area | UK0012 | 0 | D | 0 | D | 91 | D | 354964 | D |
| Teesside Urban Area | UK0013 | 0 | D | 0 | D | 111 | D | 298093 | D |
| The Potteries | UK0014 | 0 | D | 0 | D | 91 | D | 266188 | D |
| Bournemouth Urban Area | UK0015 | 0 | D | 0 | D | 113 | D | 338103 | D |
| Reading/Wokingham Urban Area | UK0016 | 0 | D | 0 | D | 97 | D | 305786 | D |
| Coventry/Bedworth | UK0017 | 0 | D | 0 | D | 76 | D | 277475 | D |
| Kingston upon Hull | UK0018 | 0 | D | 0 | D | 80 | D | 260201 | D |
| Southampton Urban Area | UK0019 | 0 | D | 0 | D | 77 | D | 262379 | D |
| Birkenhead Urban Area | UK0020 | 0 | D | 0 | D | 88 | D | 265019 | D |
| Southend Urban Area | UK0021 | 0 | D | 0 | D | 64 | D | 217874 | D |
| Blackpool Urban Area | UK0022 | 0 | D | 0 | D | 63 | D | 212909 | D |
| Preston Urban Area | UK0023 | 0 | D | 0 | D | 58 | D | 180687 | D |
| Glasgow Urban Area | UK0024 | 0 | D | 0 | D | 366 | D | 1083323 | D |
| Edinburgh Urban Area | UK0025 | 0 | D | 0 | D | 117 | D | 428762 | D |
| Cardiff Urban Area | UK0026 | 0 | D | 0 | D | 72 | D | 261383 | D |
| Swansea Urban Area | UK0027 | 0 | D | 0 | D | 84 | D | 190040 | D |
| Belfast Urban Area | UK0028 | 0 | D | 0 | D | 193 | D | 513811 | D |
| Eastern | UK0029 | 0 | D | 0 | D | 19113 | D | 4903503 | D |
| South West | UK0030 | 0 | D | 0 | D | 23506 | D | 4035009 | D |
| South East | UK0031 | 0 | D | 0 | D | 18645 | D | 6157547 | D |
| East Midlands | UK0032 | 0 | D | 0 | D | 15491 | D | 3261288 | D |
| North West & Merseyside | UK0033 | 0 | D | 0 | D | 13149 | D | 3469390 | D |
| Yorkshire & Humberside | UK0034 | 0 | D | 0 | D | 14787 | D | 3003552 | D |
| West Midlands | UK0035 | 0 | D | 0 | D | 12192 | D | 2624016 | D |
| North East | UK0036 | 0 | D | 0 | D | 8282 | D | 1443387 | D |
| Central Scotland | UK0037 | 0 | D | 0 | D | 9314 | D | 1875411 | D |
| North East Scotland | UK0038 | 0 | D | 0 | D | 18595 | D | 972129 | D |
| Highland | UK0039 | 0 | D | 0 | D | 38404 | D | 334226 | D |
| Scottish Borders | UK0040 | 0 | D | 0 | D | 11145 | D | 250175 | D |
| South Wales | UK0041 | 0 | D | 0 | D | 12221 | D | 1696999 | D |
| North Wales | UK0042 | 0 | D | 0 | D | 8368 | D | 702304 | D |
| Northern Ireland | UK0043 | 0 | D | 0 | D | 8066 | D | 726508 | D |

## Modelling the AOT40 wheat crops metric

Annex I of the Directive describes AOT40 (expressed in (µg m-³)·hours) as the sum of the difference between hourly concentrations greater than 80 µg m-³ (= 40 parts per billion) and 80 µg m-³ over a given period using only the 1 hour values measured between 8:00 and 20:00 Central European Time each day May to July.

The AOT40 wheat crops metrics for 2004 and the averaged values 2000-2004 were calculated from measured data at rural monitoring stations in the AURN during the “well-mixed” period of the day (hours 1200 UTC to 1800 UTC).

To derive an estimate of the AOT40 for the timeframe specified by the Directive, the “well-mixed” metrics from the rural AURN stations were interpolated at a 1 x 1 km2 resolution. The resultant maps of AOT40 were then adjusted to take account of the diurnal cycle in ozone concentrations using a variable ∆O3, where ∆O3 describes the difference between the AOT40 “well-mixed” and that in the between 08:00 and 20:00 during 2004. For the purposes of this study the components of ∆O3 were described as follows and were derived from measured values in 2004;

∆O3 = 0.00063.altitude + 1.276

An urban influence term UI was subsequently defined for monitoring stations in the AURN and the interpolated rural map to correct for the depletion of ozone in areas close to sources of NO. As for the days above 120 g m-3 metric, UI is closely related to annual mean NOx concentration, and has been defined in a similar fashion. The relationship between UI and empirically modelled NOx concentrations for 2004 and 2000-2004 averaged metrics is presented in Figures 2.6 and 2.7 below.

UI is then used to correct the interpolated rural “well-mixed” AOT40 maps;

Corrected AOT40 map  = interpolated rural map * (1 – UI)

Maps of modelled AOT40 for comparison with the Target Value (2004) and Long-term Objective (averaged 2000 to 2004) are presented in Figures 2.8 and 2.9.

Figure 2.6  Relationship between UI and NOx concentration in 2004 for the AOT40 metric

[NOx] (g m-3 as NO2)

Figure 2.7  Relationship between UI and NOx concentration in 2000 - 2004 for the AOT40 metric

[NOx] (g m-3 as NO2)

Figure 2.8  Estimated AOT40 wheat crops, 2004 (g m-3.hours)

Figure 2.9  Estimated AOT40 wheat crops, averaged 2000 – 2004 (g m-3.hours)

### Verification of mapped AOT40 values

Figures 2.10 and 2.11 show comparisons of modelled and measured AOT40 metrics in 2004 and averaged 2000-4 at background locations. Both the national network sites used to calibrate the models and the verification sites are shown. Lines representing 
y = x – 50 % and y = x + 50% are also shown (this is the AQDD3 data quality objective for modelled ozone concentrations).

Figure 2.10  Verification of background AOT40 wheat crops model, 2004

Figure 2.11  Verification of background AOT40 wheat crops model, 2000-2004

Summary statistics for the comparison between modelled and measured ozone concentrations are listed in Table 2.1.

Table 2.3  Summary statistics for comparison between modelled and measured AOT40 wheat crops

|  |  | Mean of measurements (days) | Mean of model estimates (days) | r2 | % outside data quality objectives | No. sites |
| --- | --- | --- | --- | --- | --- | --- |
| National Network | 2004 | 2881 | 2678 | 0.69 | 22 | 72 |
| Verification Sites | 2004 | 3681 | 4024 | 0.23 | 42 | 19 |
| National Network | 2002-4 | 3979 | 3320 | 0.70 | 11 | 62 |
| Verification Sites | 2002-4 | 5794 | 4847 | 0.30 | 13 | 16 |

### Detailed comparison of modelling results with limit values

The modelling results, in terms of a comparison of modelled concentrations with the target value by zone, are summarised in Table 2.4. These data have also been presented in form 19g of the questionnaire. Method D in this table refers to the modelling method described in this report.

Estimates of area and population exposed have been derived from the background maps only.

Table 2.2  Tabular results of and methods used for supplementary assessment (1999/30/EC Article 7(3) and Annex VIII(II), 2000/69/EC Article 5(3) and Annex VI(II) and 2002/3/EC Article 9(1) and Annex VII(II))

|  |  | Above TV for ecosystems |  |  |  | Above LTO for ecosystems |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Area |  | Ecosystem area exposed |  | Area |  | Ecosystem area exposed |  |
| Zone | Zone code | km2 | Method | km2 | Method | km2 | Method | km2 | Method |
| Greater London Urban Area | UK0001 | 0 | D | 0 | D | 0 | D | 0 | D |
| West Midlands Urban Area | UK0002 | 0 | D | 0 | D | 0 | D | 0 | D |
| Greater Manchester Urban Area | UK0003 | 0 | D | 0 | D | 0 | D | 0 | D |
| West Yorkshire Urban Area | UK0004 | 0 | D | 0 | D | 0 | D | 0 | D |
| Tyneside | UK0005 | 0 | D | 0 | D | 0 | D | 0 | D |
| Liverpool Urban Area | UK0006 | 0 | D | 0 | D | 0 | D | 0 | D |
| Sheffield Urban Area | UK0007 | 0 | D | 0 | D | 0 | D | 0 | D |
| Nottingham Urban Area | UK0008 | 0 | D | 0 | D | 0 | D | 0 | D |
| Bristol Urban Area | UK0009 | 0 | D | 0 | D | 0 | D | 0 | D |
| Brighton/Worthing/Littlehampton | UK0010 | 0 | D | 0 | D | 0 | D | 0 | D |
| Leicester Urban Area | UK0011 | 0 | D | 0 | D | 0 | D | 0 | D |
| Portsmouth Urban Area | UK0012 | 0 | D | 0 | D | 0 | D | 0 | D |
| Teesside Urban Area | UK0013 | 0 | D | 0 | D | 0 | D | 0 | D |
| The Potteries | UK0014 | 0 | D | 0 | D | 0 | D | 0 | D |
| Bournemouth Urban Area | UK0015 | 0 | D | 0 | D | 0 | D | 0 | D |
| Reading/Wokingham Urban Area | UK0016 | 0 | D | 0 | D | 0 | D | 0 | D |
| Coventry/Bedworth | UK0017 | 0 | D | 0 | D | 0 | D | 0 | D |
| Kingston upon Hull | UK0018 | 0 | D | 0 | D | 0 | D | 0 | D |
| Southampton Urban Area | UK0019 | 0 | D | 0 | D | 0 | D | 0 | D |
| Birkenhead Urban Area | UK0020 | 0 | D | 0 | D | 0 | D | 0 | D |
| Southend Urban Area | UK0021 | 0 | D | 0 | D | 0 | D | 0 | D |
| Blackpool Urban Area | UK0022 | 0 | D | 0 | D | 0 | D | 0 | D |
| Preston Urban Area | UK0023 | 0 | D | 0 | D | 0 | D | 0 | D |
| Glasgow Urban Area | UK0024 | 0 | D | 0 | D | 0 | D | 0 | D |
| Edinburgh Urban Area | UK0025 | 0 | D | 0 | D | 0 | D | 0 | D |
| Cardiff Urban Area | UK0026 | 0 | D | 0 | D | 0 | D | 0 | D |
| Swansea Urban Area | UK0027 | 0 | D | 0 | D | 0 | D | 0 | D |
| Belfast Urban Area | UK0028 | 0 | D | 0 | D | 0 | D | 0 | D |
| Eastern | UK0029 | 0 | D | 0 | D | 6972 | D | 5685 | D |
| South West | UK0030 | 0 | D | 0 | D | 214 | D | 214 | D |
| South East | UK0031 | 0 | D | 0 | D | 452 | D | 156 | D |
| East Midlands | UK0032 | 0 | D | 0 | D | 489 | D | 367 | D |
| North West & Merseyside | UK0033 | 0 | D | 0 | D | 0 | D | 0 | D |
| Yorkshire & Humberside | UK0034 | 0 | D | 0 | D | 0 | D | 0 | D |
| West Midlands | UK0035 | 0 | D | 0 | D | 0 | D | 0 | D |
| North East | UK0036 | 0 | D | 0 | D | 0 | D | 0 | D |
| Central Scotland | UK0037 | 0 | D | 0 | D | 0 | D | 0 | D |
| North East Scotland | UK0038 | 0 | D | 0 | D | 0 | D | 0 | D |
| Highland | UK0039 | 0 | D | 0 | D | 0 | D | 0 | D |
| Scottish Borders | UK0040 | 0 | D | 0 | D | 0 | D | 0 | D |
| South Wales | UK0041 | 0 | D | 0 | D | 0 | D | 0 | D |
| North Wales | UK0042 | 0 | D | 0 | D | 0 | D | 0 | D |
| Northern Ireland | UK0043 | 0 | D | 0 | D | 0 | D | 0 | D |

# Lists of zones in relation to Target Values and Long-term objectives

## Results for 2004

The table presented in this section is from form 9 of the questionnaire. Exceedence (or otherwise of the target value (TV) and Long-term Objective (LTO) where this exists are indicated by a ‘y’ for measured exceedences and with an ‘m’ for modelled exceedences. If both measurements and model estimates show that a threshold has been exceeded then the measurements are regarded as the primary basis for compliance status and ‘y’ is therefore used. An ‘m’ in the columns marked >TV, TV; >LTO indicates that modelled concentrations were higher than measured concentrations or on rare occasions that measurements were not available for that zone and modelled values were therefore used. Modelled concentration may be higher than measured concentrations because the modelling studies provide estimates of concentrations over the entire zone. It is possible that the locations of the monitoring sites do not correspond to the location of the highest concentration in the zone. An ‘m’ in the columns marked LTO indicates that measurements were not available for that zone and modelled values were therefore used.

Table 3.1  Form 9 List of zones and agglomerations where levels exceed or do not exceed target values or long term objective

| Zone | Zone code | Thresholds for health |  |  | Thresholds for vegetation |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | >TV | ≤TV; >LTO | ≤LTO | >TV | ≤TV; >LTO | ≤LTO |
| Greater London Urban Area | UK0001 |  | y |  |  | y |  |
| West Midlands Urban Area | UK0002 |  | y |  |  |  | y |
| Greater Manchester Urban Area | UK0003 |  | y |  |  |  | y |
| West Yorkshire Urban Area | UK0004 |  | y |  |  |  | y |
| Tyneside | UK0005 |  | y |  |  |  | y |
| Liverpool Urban Area | UK0006 |  | y |  |  |  | y |
| Sheffield Urban Area | UK0007 |  | y |  |  |  | y |
| Nottingham Urban Area | UK0008 |  | y |  |  |  | y |
| Bristol Urban Area | UK0009 |  | m |  |  |  | y |
| Brighton/Worthing/Littlehampton | UK0010 |  | m |  |  |  | m |
| Leicester Urban Area | UK0011 |  | y |  |  |  | y |
| Portsmouth Urban Area | UK0012 |  | y |  |  |  | y |
| Teesside Urban Area | UK0013 |  | y |  |  |  | y |
| The Potteries | UK0014 |  | y |  |  |  | y |
| Bournemouth Urban Area | UK0015 |  | y |  |  |  | y |
| Reading/Wokingham Urban Area | UK0016 |  | y |  |  | y |  |
| Coventry/Bedworth | UK0017 |  | y |  |  |  | y |
| Kingston upon Hull | UK0018 |  | y |  |  |  | y |
| Southampton Urban Area | UK0019 |  | m |  |  |  | y |
| Birkenhead Urban Area | UK0020 |  | y |  |  |  | y |
| Southend Urban Area | UK0021 |  | y |  |  | y |  |
| Blackpool Urban Area | UK0022 |  | y |  |  |  | y |
| Preston Urban Area | UK0023 |  | y |  |  |  | y |
| Glasgow Urban Area | UK0024 |  | m |  |  |  | y |
| Edinburgh Urban Area | UK0025 |  | y |  |  |  | y |
| Cardiff Urban Area | UK0026 |  | y |  |  |  | y |
| Swansea Urban Area | UK0027 |  | y |  |  |  | y |
| Belfast Urban Area | UK0028 |  | m |  |  |  | y |
| Eastern | UK0029 |  | y |  |  | y |  |
| South West | UK0030 |  | y |  |  | m |  |
| South East | UK0031 |  | y |  |  | y |  |
| East Midlands | UK0032 |  | y |  |  | m |  |
| North West & Merseyside | UK0033 |  | y |  |  |  | y |
| Yorkshire & Humberside | UK0034 |  | y |  |  |  | y |
| West Midlands | UK0035 |  | y |  |  |  | y |
| North East | UK0036 |  | m |  |  |  | m |
| Central Scotland | UK0037 |  | y |  |  |  | y |
| North East Scotland | UK0038 |  | y |  |  |  | y |
| Highland | UK0039 |  | y |  |  |  | y |
| Scottish Borders | UK0040 |  | y |  |  |  | y |
| South Wales | UK0041 |  | y |  |  |  | y |
| North Wales | UK0042 |  | y |  |  |  | y |
| Northern Ireland | UK0043 |  | m |  |  |  | y |

## measured exceedences of Target Values and long-term objectives

Forms 13a-c of the questionnaire requires reasons associated with the measured exceedences of the LTO, Alert Threshold and Information Threshold to be documented. This information is summarised in Tables 3.2 to 3.4 for monitoring stations in the UK at which exceedences of the LTO, Alert and Information Thresholds were measured. Measured annual statistics for ozone are presented in form 15 of the questionnaire (see Table 3.5). Form 14a-b relate to measured exceedence of the TVs of which there are none.

The Reason Code ‘S10’ refers to the ‘Transport of air pollution originating from sources outside the Member State’.

Table 3.2  Form 13a Individual exceedences of ozone thresholds (2002/3/EC, Article 10(2b) and Annex III)

| - Form 13a Exceedence of ozone information threshold value |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Station name | Zone code | EoI station code | Month | Day of month | Max 1-hour mean ozone concentration (mg/m3) during exceedence period | Reason code(s) | Starting time of the exceedence period | Total number of exceedence hours | 1-hour mean NO2 concentration (mg/m3) during maximum ozone concentration |
| High Muffles | UK0034 | GB0014R | 8 | 8 | 182 | S10 | 20 | 1 | 5 |
| High Muffles | UK0034 | GB0014R | 8 | 8 | 182 | S10 | 23 | 1 | 2.5 |
| Hull Freetown | UK0018 | GB0776A | 8 | 8 | 184 | S10 | 19 | 1 | 19 |
| Lullington Heath | UK0031 | GB0038R | 9 | 5 | 184 | S10 | 15 | 1 | 7.6 |
| Middlesbrough | UK0013 | GB0583A | 8 | 8 | 182 | S10 | 21 | 1 |  |
| Northampton | UK0032 | GB0738A | 7 | 29 | 182 | S10 | 15 | 1 | 8 |
| Norwich Centre | UK0029 | GB0684A | 8 | 8 | 188 | S10 | 17 | 2 | 11 |
| Portsmouth | UK0012 | GB0733A | 9 | 5 | 182 | S10 | 18 | 1 | 10 |
| Sibton | UK0029 | GB0039R | 8 | 8 | 212 | S10 | 15 | 4 |  |
| Southend-on-Sea | UK0021 | GB0728A | 7 | 29 | 186 | S10 | 14 | 1 | 17 |
| Weybourne | UK0029 | GB0745A | 8 | 8 | 202 | S10 | 16 | 4 |  |
| Wicken Fen | UK0029 | GB0045R | 7 | 29 | 182 | S10 | 16 | 1 | 7.1 |

Table 3.3  Form 13b Individual exceedences of ozone thresholds (2002/3/EC, Article 10(2b) and Annex III)

| - Form 13b Exceedence of ozone alert threshold value |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Zone code | EoI station code | Month | Day of month | Maximum 1-hour mean ozone concentration (g/m3) during exceedence period | Reason code(s) | Starting time of the exceedence period | Total number of exceedence hours | 1-hour mean NO2 concentration (g/m3) during maximum ozone concentration |
| No exceedences |  |  |  |  |  |  |  |  |

Table 3.4  Form 13c Individual exceedences of ozone thresholds (2002/3/EC, Article 10(2b) and Annex III)

| - Form 13c Exceedence of ozone long term objective for health protection |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Station name | Zone code | EoI station code | Month | Day of month | Daily maximum8-hour mean concentration (mg/m3) | Reason code(s) |
| Aberdeen | UK0038 | GB0729A | April | 29 | 130 | S10 |
| Aberdeen | UK0038 | GB0729A | April | 30 | 132 | S10 |
| Aberdeen | UK0038 | GB0729A | August | 8 | 127 | S10 |
| Aberdeen | UK0038 | GB0729A | August | 9 | 139 | S10 |
| Aberdeen | UK0038 | GB0729A | August | 10 | 128 | S10 |
| Aberdeen | UK0038 | GB0729A | August | 11 | 123 | S10 |
| Aberdeen | UK0038 | GB0729A | August | 12 | 123 | S10 |
| Aston Hill | UK0042 | GB0031R | July | 29 | 126 | S10 |
| Aston Hill | UK0042 | GB0031R | August | 1 | 123 | S10 |
| Aston Hill | UK0042 | GB0031R | August | 2 | 133 | S10 |
| Aston Hill | UK0042 | GB0031R | August | 3 | 121 | S10 |
| Barnsley Gawber | UK0034 | GB0681A | June | 8 | 130 | S10 |
| Barnsley Gawber | UK0034 | GB0681A | August | 8 | 129 | S10 |
| Birmingham Centre | UK0002 | GB0569A | April | 25 | 122 | S10 |
| Birmingham Centre | UK0002 | GB0569A | June | 8 | 126 | S10 |
| Birmingham Centre | UK0002 | GB0569A | July | 29 | 128 | S10 |
| Birmingham Centre | UK0002 | GB0569A | August | 8 | 130 | S10 |
| Blackpool | UK0022 | GB0727A | July | 29 | 126 | S10 |
| Blackpool | UK0022 | GB0727A | August | 1 | 129 | S10 |
| Bolton | UK0003 | GB0654A | July | 29 | 133 | S10 |
| Bolton | UK0003 | GB0654A | August | 8 | 140 | S10 |
| Bottesford | UK0032 | GB0032R | June | 8 | 125 | S10 |
| Bottesford | UK0032 | GB0032R | July | 29 | 144 | S10 |
| Bottesford | UK0032 | GB0032R | August | 7 | 123 | S10 |
| Bottesford | UK0032 | GB0032R | August | 8 | 155 | S10 |
| Bottesford | UK0032 | GB0032R | August | 9 | 137 | S10 |
| Bournemouth | UK0015 | GB0741A | June | 7 | 128 | S10 |
| Bournemouth | UK0015 | GB0741A | August | 1 | 141 | S10 |
| Bournemouth | UK0015 | GB0741A | August | 2 | 139 | S10 |
| Bournemouth | UK0015 | GB0741A | August | 3 | 136 | S10 |
| Bournemouth | UK0015 | GB0741A | August | 8 | 127 | S10 |
| Bournemouth | UK0015 | GB0741A | September | 5 | 148 | S10 |
| Bradford Centre | UK0004 | GB0689A | August | 8 | 128 | S10 |
| Bush Estate | UK0037 | GB0033R | August | 8 | 151 | S10 |
| Bush Estate | UK0037 | GB0033R | August | 9 | 133 | S10 |
| Cardiff Centre | UK0026 | GB0580A | April | 25 | 125 | S10 |
| Cardiff Centre | UK0026 | GB0580A | August | 1 | 153 | S10 |
| Cardiff Centre | UK0026 | GB0580A | August | 2 | 124 | S10 |
| Coventry Memorial Park | UK0017 | GB0739A | April | 25 | 123 | S10 |
| Coventry Memorial Park | UK0017 | GB0739A | June | 8 | 144 | S10 |
| Coventry Memorial Park | UK0017 | GB0739A | July | 20 | 129 | S10 |
| Coventry Memorial Park | UK0017 | GB0739A | July | 29 | 165 | S10 |
| Coventry Memorial Park | UK0017 | GB0739A | August | 1 | 131 | S10 |
| Coventry Memorial Park | UK0017 | GB0739A | August | 2 | 139 | S10 |
| Coventry Memorial Park | UK0017 | GB0739A | August | 5 | 128 | S10 |
| Coventry Memorial Park | UK0017 | GB0739A | August | 8 | 160 | S10 |
| Cwmbran | UK0041 | GB0744A | August | 1 | 138 | S10 |
| Edinburgh St Leonards | UK0025 | GB0839A | August | 1 | 125 | S10 |
| Edinburgh St Leonards | UK0025 | GB0839A | August | 8 | 151 | S10 |
| Edinburgh St Leonards | UK0025 | GB0839A | August | 9 | 140 | S10 |
| Eskdalemuir | UK0040 | GB0002R | August | 8 | 137 | S10 |
| Glazebury | UK0033 | GB0034R | July | 29 | 136 | S10 |
| Glazebury | UK0033 | GB0034R | August | 1 | 124 | S10 |
| Glazebury | UK0033 | GB0034R | August | 8 | 140 | S10 |
| Great Dun Fell | UK0033 | GB0035R | May | 1 | 123 | S10 |
| Great Dun Fell | UK0033 | GB0035R | May | 2 | 128 | S10 |
| Great Dun Fell | UK0033 | GB0035R | August | 8 | 133 | S10 |
| Great Dun Fell | UK0033 | GB0035R | August | 9 | 121 | S10 |
| Harwell | UK0031 | GB0036R | April | 25 | 124 | S10 |
| Harwell | UK0031 | GB0036R | April | 26 | 134 | S10 |
| Harwell | UK0031 | GB0036R | June | 8 | 136 | S10 |
| Harwell | UK0031 | GB0036R | July | 29 | 160 | S10 |
| Harwell | UK0031 | GB0036R | July | 31 | 125 | S10 |
| Harwell | UK0031 | GB0036R | August | 1 | 161 | S10 |
| Harwell | UK0031 | GB0036R | August | 2 | 147 | S10 |
| Harwell | UK0031 | GB0036R | August | 8 | 134 | S10 |
| High Muffles | UK0034 | GB0014R | May | 2 | 132 | S10 |
| High Muffles | UK0034 | GB0014R | July | 29 | 139 | S10 |
| High Muffles | UK0034 | GB0014R | August | 7 | 147 | S10 |
| High Muffles | UK0034 | GB0014R | August | 8 | 171 | S10 |
| High Muffles | UK0034 | GB0014R | August | 9 | 172 | S10 |
| Hull Freetown | UK0018 | GB0776A | August | 7 | 149 | S10 |
| Hull Freetown | UK0018 | GB0776A | August | 8 | 166 | S10 |
| Hull Freetown | UK0018 | GB0776A | August | 9 | 153 | S10 |
| Ladybower | UK0032 | GB0037R | April | 25 | 128 | S10 |
| Ladybower | UK0032 | GB0037R | May | 1 | 124 | S10 |
| Ladybower | UK0032 | GB0037R | May | 2 | 126 | S10 |
| Ladybower | UK0032 | GB0037R | June | 8 | 131 | S10 |
| Ladybower | UK0032 | GB0037R | July | 29 | 128 | S10 |
| Ladybower | UK0032 | GB0037R | August | 8 | 152 | S10 |
| Ladybower | UK0032 | GB0037R | August | 9 | 138 | S10 |
| Leamington Spa | UK0035 | GB0643A | June | 8 | 133 | S10 |
| Leamington Spa | UK0035 | GB0643A | July | 29 | 133 | S10 |
| Leamington Spa | UK0035 | GB0643A | August | 1 | 122 | S10 |
| Leamington Spa | UK0035 | GB0643A | August | 8 | 140 | S10 |
| Leicester Centre | UK0011 | GB0597A | July | 29 | 125 | S10 |
| Leicester Centre | UK0011 | GB0597A | August | 2 | 122 | S10 |
| Leicester Centre | UK0011 | GB0597A | August | 7 | 122 | S10 |
| Leicester Centre | UK0011 | GB0597A | August | 8 | 149 | S10 |
| Leicester Centre | UK0011 | GB0597A | August | 9 | 132 | S10 |
| Liverpool Speke | UK0006 | GB0777A | August | 8 | 129 | S10 |
| London Bexley | UK0001 | GB0608A | June | 8 | 135 | S10 |
| London Bexley | UK0001 | GB0608A | July | 28 | 121 | S10 |
| London Bexley | UK0001 | GB0608A | July | 29 | 148 | S10 |
| London Bexley | UK0001 | GB0608A | August | 4 | 132 | S10 |
| London Bexley | UK0001 | GB0608A | August | 8 | 142 | S10 |
| London Bloomsbury | UK0001 | GB0566A | July | 29 | 121 | S10 |
| London Brent | UK0001 | GB0616A | June | 8 | 122 | S10 |
| London Brent | UK0001 | GB0616A | July | 29 | 131 | S10 |
| London Brent | UK0001 | GB0616A | September | 2 | 125 | S10 |
| London Brent | UK0001 | GB0616A | September | 5 | 135 | S10 |
| London Eltham | UK0001 | GB0586A | June | 8 | 136 | S10 |
| London Eltham | UK0001 | GB0586A | July | 28 | 127 | S10 |
| London Eltham | UK0001 | GB0586A | July | 29 | 148 | S10 |
| London Eltham | UK0001 | GB0586A | July | 31 | 123 | S10 |
| London Eltham | UK0001 | GB0586A | August | 8 | 140 | S10 |
| London Hackney | UK0001 | GB0650A | June | 8 | 130 | S10 |
| London Hackney | UK0001 | GB0650A | July | 29 | 131 | S10 |
| London Hackney | UK0001 | GB0650A | August | 8 | 129 | S10 |
| London Haringey | UK0001 | GB0638A | June | 8 | 141 | S10 |
| London Haringey | UK0001 | GB0638A | July | 29 | 136 | S10 |
| London Haringey | UK0001 | GB0638A | August | 1 | 126 | S10 |
| London Haringey | UK0001 | GB0638A | August | 8 | 129 | S10 |
| London Harlington | UK0001 | GB0837A | June | 8 | 122 | S10 |
| London Harlington | UK0001 | GB0837A | July | 29 | 134 | S10 |
| London Harlington | UK0001 | GB0837A | August | 1 | 121 | S10 |
| London Harlington | UK0001 | GB0837A | August | 8 | 132 | S10 |
| London N. Kensington | UK0001 | GB0620A | June | 8 | 142 | S10 |
| London N. Kensington | UK0001 | GB0620A | July | 29 | 155 | S10 |
| London N. Kensington | UK0001 | GB0620A | July | 31 | 125 | S10 |
| London N. Kensington | UK0001 | GB0620A | August | 8 | 136 | S10 |
| London Teddington | UK0001 | GB0644A | June | 8 | 157 | S10 |
| London Teddington | UK0001 | GB0644A | July | 28 | 130 | S10 |
| London Teddington | UK0001 | GB0644A | July | 29 | 155 | S10 |
| London Teddington | UK0001 | GB0644A | July | 31 | 133 | S10 |
| London Teddington | UK0001 | GB0644A | August | 1 | 126 | S10 |
| London Teddington | UK0001 | GB0644A | August | 2 | 128 | S10 |
| London Teddington | UK0001 | GB0644A | August | 7 | 131 | S10 |
| London Teddington | UK0001 | GB0644A | August | 8 | 140 | S10 |
| London Teddington | UK0001 | GB0644A | September | 2 | 126 | S10 |
| London Westminster | UK0001 | GB0743A | June | 8 | 128 | S10 |
| London Westminster | UK0001 | GB0743A | July | 29 | 143 | S10 |
| Lullington Heath | UK0031 | GB0038R | June | 7 | 128 | S10 |
| Lullington Heath | UK0031 | GB0038R | June | 8 | 131 | S10 |
| Lullington Heath | UK0031 | GB0038R | July | 28 | 152 | S10 |
| Lullington Heath | UK0031 | GB0038R | July | 29 | 152 | S10 |
| Lullington Heath | UK0031 | GB0038R | August | 2 | 144 | S10 |
| Lullington Heath | UK0031 | GB0038R | August | 3 | 131 | S10 |
| Lullington Heath | UK0031 | GB0038R | August | 8 | 144 | S10 |
| Lullington Heath | UK0031 | GB0038R | August | 9 | 123 | S10 |
| Lullington Heath | UK0031 | GB0038R | September | 5 | 147 | S10 |
| Manchester Piccadilly | UK0003 | GB0613A | August | 8 | 123 | S10 |
| Market Harborough | UK0032 | GB0838A | June | 8 | 144 | S10 |
| Market Harborough | UK0032 | GB0838A | July | 29 | 150 | S10 |
| Market Harborough | UK0032 | GB0838A | August | 5 | 121 | S10 |
| Market Harborough | UK0032 | GB0838A | August | 7 | 129 | S10 |
| Market Harborough | UK0032 | GB0838A | August | 8 | 163 | S10 |
| Market Harborough | UK0032 | GB0838A | August | 9 | 139 | S10 |
| Middlesbrough | UK0013 | GB0583A | May | 2 | 132 | S10 |
| Middlesbrough | UK0013 | GB0583A | July | 29 | 143 | S10 |
| Middlesbrough | UK0013 | GB0583A | August | 7 | 129 | S10 |
| Middlesbrough | UK0013 | GB0583A | August | 8 | 169 | S10 |
| Middlesbrough | UK0013 | GB0583A | August | 9 | 170 | S10 |
| Newcastle Centre | UK0005 | GB0568A | August | 8 | 127 | S10 |
| Newcastle Centre | UK0005 | GB0568A | August | 9 | 136 | S10 |
| Northampton | UK0032 | GB0738A | April | 26 | 121 | S10 |
| Northampton | UK0032 | GB0738A | July | 28 | 133 | S10 |
| Northampton | UK0032 | GB0738A | July | 29 | 171 | S10 |
| Northampton | UK0032 | GB0738A | August | 1 | 134 | S10 |
| Northampton | UK0032 | GB0738A | August | 2 | 158 | S10 |
| Northampton | UK0032 | GB0738A | August | 3 | 139 | S10 |
| Northampton | UK0032 | GB0738A | August | 5 | 131 | S10 |
| Northampton | UK0032 | GB0738A | August | 7 | 135 | S10 |
| Northampton | UK0032 | GB0738A | August | 8 | 167 | S10 |
| Northampton | UK0032 | GB0738A | August | 9 | 147 | S10 |
| Norwich Centre | UK0029 | GB0684A | June | 8 | 125 | S10 |
| Norwich Centre | UK0029 | GB0684A | July | 28 | 123 | S10 |
| Norwich Centre | UK0029 | GB0684A | July | 29 | 126 | S10 |
| Norwich Centre | UK0029 | GB0684A | August | 6 | 124 | S10 |
| Norwich Centre | UK0029 | GB0684A | August | 8 | 175 | S10 |
| Norwich Centre | UK0029 | GB0684A | August | 9 | 142 | S10 |
| Nottingham Centre | UK0008 | GB0646A | June | 8 | 122 | S10 |
| Nottingham Centre | UK0008 | GB0646A | August | 8 | 131 | S10 |
| Plymouth Centre | UK0030 | GB0687A | September | 5 | 147 | S10 |
| Plymouth Centre | UK0030 | GB0687A | September | 8 | 121 | S10 |
| Port Talbot | UK0027 | GB0651A | May | 26 | 122 | S10 |
| Port Talbot | UK0027 | GB0651A | August | 1 | 132 | S10 |
| Port Talbot | UK0027 | GB0651A | August | 2 | 124 | S10 |
| Portsmouth | UK0012 | GB0733A | July | 28 | 128 | S10 |
| Portsmouth | UK0012 | GB0733A | July | 29 | 122 | S10 |
| Portsmouth | UK0012 | GB0733A | August | 1 | 138 | S10 |
| Portsmouth | UK0012 | GB0733A | August | 2 | 138 | S10 |
| Portsmouth | UK0012 | GB0733A | August | 8 | 141 | S10 |
| Portsmouth | UK0012 | GB0733A | August | 9 | 125 | S10 |
| Portsmouth | UK0012 | GB0733A | September | 5 | 170 | S10 |
| Portsmouth | UK0012 | GB0733A | September | 6 | 125 | S10 |
| Preston | UK0023 | GB0731A | August | 8 | 128 | S10 |
| Reading New Town | UK0016 | GB0840A | April | 25 | 126 | S10 |
| Reading New Town | UK0016 | GB0840A | April | 26 | 132 | S10 |
| Reading New Town | UK0016 | GB0840A | May | 28 | 122 | S10 |
| Reading New Town | UK0016 | GB0840A | June | 7 | 133 | S10 |
| Reading New Town | UK0016 | GB0840A | June | 8 | 141 | S10 |
| Reading New Town | UK0016 | GB0840A | July | 29 | 148 | S10 |
| Reading New Town | UK0016 | GB0840A | July | 31 | 126 | S10 |
| Reading New Town | UK0016 | GB0840A | August | 1 | 141 | S10 |
| Reading New Town | UK0016 | GB0840A | August | 2 | 146 | S10 |
| Reading New Town | UK0016 | GB0840A | August | 8 | 143 | S10 |
| Redcar | UK0013 | GB0679A | May | 31 | 129 | S10 |
| Redcar | UK0013 | GB0679A | July | 29 | 154 | S10 |
| Redcar | UK0013 | GB0679A | July | 30 | 124 | S10 |
| Redcar | UK0013 | GB0679A | August | 1 | 126 | S10 |
| Redcar | UK0013 | GB0679A | August | 7 | 138 | S10 |
| Redcar | UK0013 | GB0679A | August | 8 | 155 | S10 |
| Redcar | UK0013 | GB0679A | August | 9 | 165 | S10 |
| Rochester | UK0031 | GB0617A | May | 29 | 132 | S10 |
| Rochester | UK0031 | GB0617A | June | 8 | 145 | S10 |
| Rochester | UK0031 | GB0617A | July | 28 | 149 | S10 |
| Rochester | UK0031 | GB0617A | July | 29 | 155 | S10 |
| Rochester | UK0031 | GB0617A | July | 31 | 137 | S10 |
| Rochester | UK0031 | GB0617A | August | 1 | 138 | S10 |
| Rochester | UK0031 | GB0617A | August | 2 | 138 | S10 |
| Rochester | UK0031 | GB0617A | August | 7 | 133 | S10 |
| Rochester | UK0031 | GB0617A | August | 8 | 143 | S10 |
| Rochester | UK0031 | GB0617A | September | 2 | 122 | S10 |
| Rotherham Centre | UK0007 | GB0677A | August | 8 | 136 | S10 |
| Rotherham Centre | UK0007 | GB0677A | August | 9 | 124 | S10 |
| Salford Eccles | UK0003 | GB0660A | August | 8 | 124 | S10 |
| Sandwell West Bromwich | UK0002 | GB0698A | April | 25 | 129 | S10 |
| Sandwell West Bromwich | UK0002 | GB0698A | June | 8 | 131 | S10 |
| Sandwell West Bromwich | UK0002 | GB0698A | July | 29 | 133 | S10 |
| Sandwell West Bromwich | UK0002 | GB0698A | August | 1 | 122 | S10 |
| Sandwell West Bromwich | UK0002 | GB0698A | August | 8 | 136 | S10 |
| Sheffield Centre | UK0007 | GB0615A | August | 8 | 129 | S10 |
| Sibton | UK0029 | GB0039R | June | 8 | 142 | S10 |
| Sibton | UK0029 | GB0039R | July | 28 | 147 | S10 |
| Sibton | UK0029 | GB0039R | July | 29 | 144 | S10 |
| Sibton | UK0029 | GB0039R | August | 1 | 131 | S10 |
| Sibton | UK0029 | GB0039R | August | 2 | 132 | S10 |
| Sibton | UK0029 | GB0039R | August | 6 | 136 | S10 |
| Sibton | UK0029 | GB0039R | August | 8 | 185 | S10 |
| Sibton | UK0029 | GB0039R | August | 9 | 142 | S10 |
| Sibton | UK0029 | GB0039R | September | 3 | 121 | S10 |
| Sibton | UK0029 | GB0039R | September | 5 | 121 | S10 |
| Somerton | UK0030 | GB0044R | April | 26 | 122 | S10 |
| Somerton | UK0030 | GB0044R | August | 1 | 140 | S10 |
| Somerton | UK0030 | GB0044R | August | 2 | 132 | S10 |
| Southend-on-Sea | UK0021 | GB0728A | May | 29 | 133 | S10 |
| Southend-on-Sea | UK0021 | GB0728A | June | 8 | 145 | S10 |
| Southend-on-Sea | UK0021 | GB0728A | July | 28 | 152 | S10 |
| Southend-on-Sea | UK0021 | GB0728A | July | 29 | 154 | S10 |
| Southend-on-Sea | UK0021 | GB0728A | July | 31 | 144 | S10 |
| Southend-on-Sea | UK0021 | GB0728A | August | 1 | 134 | S10 |
| Southend-on-Sea | UK0021 | GB0728A | August | 2 | 136 | S10 |
| Southend-on-Sea | UK0021 | GB0728A | August | 3 | 126 | S10 |
| Southend-on-Sea | UK0021 | GB0728A | August | 7 | 127 | S10 |
| Southend-on-Sea | UK0021 | GB0728A | August | 8 | 158 | S10 |
| St Osyth | UK0029 | GB0754A | June | 8 | 126 | S10 |
| St Osyth | UK0029 | GB0754A | July | 28 | 130 | S10 |
| St Osyth | UK0029 | GB0754A | July | 29 | 137 | S10 |
| St Osyth | UK0029 | GB0754A | July | 31 | 124 | S10 |
| St Osyth | UK0029 | GB0754A | August | 1 | 124 | S10 |
| St Osyth | UK0029 | GB0754A | August | 2 | 132 | S10 |
| St Osyth | UK0029 | GB0754A | August | 8 | 149 | S10 |
| St Osyth | UK0029 | GB0754A | September | 5 | 124 | S10 |
| Stoke-on-Trent Centre | UK0014 | GB0658A | April | 25 | 122 | S10 |
| Stoke-on-Trent Centre | UK0014 | GB0658A | August | 8 | 125 | S10 |
| Strath Vaich | UK0039 | GB0015R | April | 30 | 122 | S10 |
| Strath Vaich | UK0039 | GB0015R | August | 10 | 128 | S10 |
| Strath Vaich | UK0039 | GB0015R | August | 11 | 123 | S10 |
| Strath Vaich | UK0039 | GB0015R | August | 13 | 123 | S10 |
| Swansea | UK0027 | GB0609A | April | 25 | 123 | S10 |
| Swansea | UK0027 | GB0609A | August | 1 | 136 | S10 |
| Thurrock | UK0029 | GB0645A | June | 8 | 138 | S10 |
| Thurrock | UK0029 | GB0645A | July | 28 | 133 | S10 |
| Thurrock | UK0029 | GB0645A | July | 29 | 139 | S10 |
| Thurrock | UK0029 | GB0645A | July | 31 | 127 | S10 |
| Thurrock | UK0029 | GB0645A | August | 1 | 127 | S10 |
| Thurrock | UK0029 | GB0645A | August | 3 | 127 | S10 |
| Thurrock | UK0029 | GB0645A | August | 4 | 127 | S10 |
| Thurrock | UK0029 | GB0645A | August | 8 | 138 | S10 |
| Thurrock | UK0029 | GB0645A | September | 5 | 132 | S10 |
| Weybourne | UK0029 | GB0745A | April | 30 | 127 | S10 |
| Weybourne | UK0029 | GB0745A | May | 1 | 126 | S10 |
| Weybourne | UK0029 | GB0745A | May | 2 | 128 | S10 |
| Weybourne | UK0029 | GB0745A | May | 17 | 121 | S10 |
| Weybourne | UK0029 | GB0745A | May | 29 | 131 | S10 |
| Weybourne | UK0029 | GB0745A | June | 8 | 152 | S10 |
| Weybourne | UK0029 | GB0745A | June | 9 | 125 | S10 |
| Weybourne | UK0029 | GB0745A | July | 28 | 136 | S10 |
| Weybourne | UK0029 | GB0745A | July | 31 | 138 | S10 |
| Weybourne | UK0029 | GB0745A | August | 1 | 142 | S10 |
| Weybourne | UK0029 | GB0745A | August | 2 | 131 | S10 |
| Weybourne | UK0029 | GB0745A | August | 6 | 135 | S10 |
| Weybourne | UK0029 | GB0745A | August | 7 | 123 | S10 |
| Weybourne | UK0029 | GB0745A | August | 8 | 178 | S10 |
| Weybourne | UK0029 | GB0745A | August | 9 | 162 | S10 |
| Weybourne | UK0029 | GB0745A | September | 3 | 127 | S10 |
| Wicken Fen | UK0029 | GB0045R | April | 26 | 135 | S10 |
| Wicken Fen | UK0029 | GB0045R | June | 8 | 158 | S10 |
| Wicken Fen | UK0029 | GB0045R | July | 28 | 149 | S10 |
| Wicken Fen | UK0029 | GB0045R | July | 29 | 173 | S10 |
| Wicken Fen | UK0029 | GB0045R | August | 1 | 128 | S10 |
| Wicken Fen | UK0029 | GB0045R | August | 2 | 146 | S10 |
| Wicken Fen | UK0029 | GB0045R | August | 3 | 122 | S10 |
| Wicken Fen | UK0029 | GB0045R | August | 4 | 148 | S10 |
| Wicken Fen | UK0029 | GB0045R | August | 5 | 130 | S10 |
| Wicken Fen | UK0029 | GB0045R | August | 6 | 129 | S10 |
| Wicken Fen | UK0029 | GB0045R | August | 7 | 151 | S10 |
| Wicken Fen | UK0029 | GB0045R | August | 8 | 164 | S10 |
| Wicken Fen | UK0029 | GB0045R | August | 9 | 143 | S10 |
| Wicken Fen | UK0029 | GB0045R | September | 3 | 125 | S10 |
| Wirral Tranmere | UK0020 | GB0730A | August | 8 | 125 | S10 |
| Wolverhampton Centre | UK0002 | GB0614A | July | 29 | 128 | S10 |
| Wolverhampton Centre | UK0002 | GB0614A | August | 8 | 122 | S10 |
| Yarner Wood | UK0030 | GB0013R | April | 25 | 131 | S10 |
| Yarner Wood | UK0030 | GB0013R | April | 26 | 129 | S10 |
| Yarner Wood | UK0030 | GB0013R | May | 25 | 126 | S10 |
| Yarner Wood | UK0030 | GB0013R | August | 1 | 126 | S10 |
| Yarner Wood | UK0030 | GB0013R | September | 5 | 121 | S10 |

Table 3.5 Form 15 Annual statistics of ozone (2002/3/EC, Article 10(2b) and Annex III)

| Form 15 Annual statistics of ozone (2002/3/EC, Article 10(2b) and Annex III) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Station name | Zone code | EoI station code | AOT40 for vegetation protection (μg/m3.h) |  | AOT40 for forest protection (μg/m3.h) |  | Annual average |
|  |  |  | Value | Number of valid data | Value | Number of valid data |  |
| Aberdeen | UK0038 | GB0729A | 2387 | 1104 | 5917 | 2174 | 54 |
| Aston Hill | UK0042 | GB0031R | 2502 | 1082 | 7542 | 1779 | 66 |
| Barnsley Gawber | UK0034 | GB0681A | 3756 | 1027 | 5703 | 2042 | 48 |
| Belfast Centre | UK0028 | GB0567A | 438 | 1082 | 977 | 2152 | 43 |
| Birmingham Centre | UK0002 | GB0569A | 1896 | 1093 | 3859 | 1998 | 40 |
| Blackpool | UK0022 | GB0727A | 3475 | 1082 | 5889 | 2152 | 56 |
| Bolton | UK0003 | GB0654A | 2472 | 1093 | 4687 | 2108 | 44 |
| Bottesford | UK0032 | GB0032R | 2726 | 1093 | 6631 | 2174 | 46 |
| Bournemouth | UK0015 | GB0741A | 5016 | 1071 | 11133 | 2152 | 53 |
| Bradford Centre | UK0004 | GB0689A | 925 | 1104 | 1761 | 2152 | 35 |
| Bristol Centre | UK0009 | GB0585A | 781 | 1060 | 1497 | 2020 | 40 |
| Bury Roadside | UK0003 | GB0652A | 144 | 1060 | 296 | 2130 | 21 |
| Bush Estate | UK0037 | GB0033R | 769 | 1082 | 2347 | 2130 | 55 |
| Cardiff Centre | UK0026 | GB0580A | 1908 | 1093 | 4718 | 1932 | 41 |
| Coventry Memorial Park | UK0017 | GB0739A | 5045 | 1060 | 10887 | 2152 | 46 |
| Cwmbran | UK0041 | GB0744A | 1864 | 1104 | 4517 | 2174 | 51 |
| Derry | UK0043 | GB0673A | 1224 | 1082 | 3617 | 2130 | 57 |
| Edinburgh St Leonards | UK0025 | GB0839A | 2639 | 1060 | 5194 | 2108 | 53 |
| Eskdalemuir | UK0040 | GB0002R | 1473 | 1016 | 3205 | 2042 | 53 |
| Exeter Roadside | UK0030 | GB0640A | 188 | 1060 | 1027 | 2064 | 38 |
| Glasgow Centre | UK0024 | GB0641A | 395 | 1060 | 950 | 2130 | 37 |
| Glazebury | UK0033 | GB0034R | 3705 | 983 | 5792 | 2064 | 43 |
| Great Dun Fell | UK0033 | GB0035R | 1326 | 1082 | 3288 | 2152 | 62 |
| Harwell | UK0031 | GB0036R | 5950 | 1071 | 13732 | 2152 | 57 |
| High Muffles | UK0034 | GB0014R | 5552 | 1093 | 11318 | 2152 | 62 |
| Hull Freetown | UK0018 | GB0776A | 2686 | 1071 | 5804 | 2086 | 45 |
| Ladybower | UK0032 | GB0037R | 4927 | 1082 | 11506 | 1625 | 63 |
| Leamington Spa | UK0035 | GB0643A | 2240 | 1093 | 5855 | 2152 | 40 |
| Leeds Centre | UK0004 | GB0584A | 849 | 960 | 1381 | 1559 | 36 |
| Leicester Centre | UK0011 | GB0597A | 1240 | 1060 | 3766 | 2130 | 35 |
| Liverpool Speke | UK0006 | GB0777A | 1671 | 1093 | 3584 | 2152 | 48 |
| London Bexley | UK0001 | GB0608A | 3612 | 1071 | 7343 | 2152 | 39 |
| London Bloomsbury | UK0001 | GB0566A | 517 | 1016 | 1568 | 2108 | 24 |
| London Brent | UK0001 | GB0616A | 1405 | 1071 | 3304 | 2108 | 35 |
| London Eltham | UK0001 | GB0586A | 3364 | 1093 | 6467 | 2174 | 39 |
| London Hackney | UK0001 | GB0650A | 1457 | 1093 | 3855 | 1867 | 30 |
| London Haringey | UK0001 | GB0638A | 3085 | 1027 | 5731 | 2108 | 34 |
| London Harlington | UK0001 | GB0837A | 2347 | 1104 | 4800 | 2152 | 33 |
| London Hillingdon | UK0001 | GB0642A | 1566 | 1016 | 2959 | 1911 | 27 |
| London Lewisham | UK0001 | GB0672A | 570 | 1093 | 1142 | 2174 | 29 |
| London Marylebone Road | UK0001 | GB0682A | 8 | 1060 | 12 | 2152 | 15 |
| London N. Kensington | UK0001 | GB0620A | 4273 | 1093 | 7916 | 2130 | 36 |
| London Southwark | UK0001 | GB0656A | 869 | 1093 | 2217 | 2152 | 30 |
| London Teddington | UK0001 | GB0644A | 6742 | 1016 | 13097 | 2064 | 49 |
| London Wandsworth | UK0001 | GB0622A | 883 | 1093 | 1455 | 2174 | 26 |
| London Westminster | UK0001 | GB0743A | 1800 | 1082 | 3848 | 1976 | 31 |
| Lullington Heath | UK0031 | GB0038R | 5400 | 1049 | 14139 | 2086 | 61 |
| Manchester Piccadilly | UK0003 | GB0613A | 892 | 1093 | 1646 | 2152 | 30 |
| Manchester South | UK0003 | GB0649A | 524 | 1093 | 822 | 2064 | 35 |
| Market Harborough | UK0032 | GB0838A | 3317 | 1093 | 8843 | 1998 | 53 |
| Middlesbrough | UK0013 | GB0583A | 2278 | 1104 | 5292 | 2174 | 49 |
| Newcastle Centre | UK0005 | GB0568A | 885 | 1093 | 2188 | 1932 | 41 |
| Northampton | UK0032 | GB0738A | 3162 | 983 | 11561 | 1647 | 47 |
| Norwich Centre | UK0029 | GB0684A | 3001 | 1093 | 6206 | 2152 | 41 |
| Nottingham Centre | UK0008 | GB0646A | 1145 | 1082 | 1922 | 2130 | 31 |
| Plymouth Centre | UK0030 | GB0687A | 516 | 1093 | 2174 | 2130 | 38 |
| Port Talbot | UK0027 | GB0651A | 2349 | 1093 | 5026 | 2152 | 53 |
| Portsmouth | UK0012 | GB0733A | 2656 | 1082 | 8163 | 2174 | 46 |
| Preston | UK0023 | GB0731A | 1280 | 1093 | 2546 | 2174 | 44 |
| Reading New Town | UK0016 | GB0840A | 6212 | 1071 | 11986 | 2152 | 44 |
| Redcar | UK0013 | GB0679A | 4839 | 1049 | 8892 | 2064 | 52 |
| Rochester | UK0031 | GB0617A | 6662 | 1093 | 14021 | 2174 | 49 |
| Rotherham Centre | UK0007 | GB0677A | 1103 | 1071 | 2396 | 1779 | 34 |
| Salford Eccles | UK0003 | GB0660A | 875 | 1082 | 1929 | 1954 | 33 |
| Sandwell West Bromwich | UK0002 | GB0698A | 3077 | 1071 | 5841 | 2152 | 45 |
| Sheffield Centre | UK0007 | GB0615A | 1647 | 1093 | 2636 | 2130 | 39 |
| Sibton | UK0029 | GB0039R | 5138 | 1027 | 12753 | 2108 | 55 |
| Somerton | UK0030 | GB0044R | 3456 | 1038 | 8454 | 2108 | 55 |
| Southampton Centre | UK0019 | GB0598A | 802 | 1093 | 1841 | 2152 | 36 |
| Southend-on-Sea | UK0021 | GB0728A | 6478 | 1071 | 12785 | 2130 | 45 |
| St Osyth | UK0029 | GB0754A | 5092 | 1093 | 10422 | 2152 | 53 |
| Stoke-on-Trent Centre | UK0014 | GB0658A | 1000 | 1104 | 2256 | 2152 | 42 |
| Strath Vaich | UK0039 | GB0015R | 4382 | 1027 | 12456 | 1779 | 76 |
| Swansea | UK0027 | GB0609A | 1246 | 1082 | 3678 | 2130 | 41 |
| Thurrock | UK0029 | GB0645A | 3654 | 1082 | 8527 | 2152 | 39 |
| Weybourne | UK0029 | GB0745A | 9322 | 1104 | 17462 | 2196 | 64 |
| Wicken Fen | UK0029 | GB0045R | 8153 | 883 | 18970 | 1932 | 52 |
| Wirral Tranmere | UK0020 | GB0730A | 1450 | 1093 | 3170 | 2174 | 49 |
| Wolverhampton Centre | UK0002 | GB0614A | 1185 | 1093 | 2403 | 2152 | 41 |
| Yarner Wood | UK0030 | GB0013R | 5615 | 1038 | 11859 | 2086 | 63 |

# Acknowledgements

This work was funded by the UK Department for Environment, Food and Rural Affairs, Welsh Assembly Government, the Scottish Executive and the Department of the Environment in Northern Ireland. Permission to include monitoring data and detailed information on site locations for the verification sites were kindly provided by the Local Authorities and companies listed in Table A.2.1 in Appendix 2. The authors would also like to thank CEH Edinburgh for providing information upon mapping days exceeding and AOT40 metrics.

APPENDIX 1. NATIONAL NETWORK MONITORING SITES

Table A1.1. Monitoring sites operating during 2004 for AQDD3 reporting.

| EoI station code | Local station code | Zone code | Type of station | Use in relation to Directive 2002/3/EC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | O3 | NO2 | NOx |
| GB0729A | Aberdeen | UK0038 | U | y | y | y |
| GB0031R | Aston Hill | UK0042 | R | y | y | y |
| GB0681A | Barnsley Gawber | UK0034 | U | y | y | y |
| GB0567A | Belfast Centre | UK0028 | U | y | y | y |
| GB0569A | Birmingham Centre | UK0002 | U | y | y | y |
| GB0595A | Birmingham East | UK0002 | U | y | y | y |
| GB0851A | Birmingham Tyburn | UK0002 | U | y | y | y |
| GB0727A | Blackpool | UK0022 | U | y | y | y |
| GB0654A | Bolton | UK0003 | U | y | y | y |
| GB0032R | Bottesford | UK0032 | S | y |  |  |
| GB0741A | Bournemouth | UK0015 | U | y | y | y |
| GB0689A | Bradford Centre | UK0004 | U | y | y | y |
| GB0860A | Brighton Preston Park | UK0010 | U | y | y | y |
| GB0585A | Bristol Centre | UK0009 | U | y | y | y |
| GB0652A | Bury Roadside | UK0003 | U | y | y | y |
| GB0033R | Bush Estate | UK0037 | R | y | y | y |
| GB0580A | Cardiff Centre | UK0026 | U | y | y | y |
| GB0739A | Coventry Memorial Park | UK0017 | U | y | y | y |
| GB0744A | Cwmbran | UK0041 | U | y | y | y |
| GB0673A | Derry | UK0043 | U | y | y | y |
| GB0839A | Edinburgh St Leonards | UK0025 | U | y | y | y |
| GB0002R | Eskdalemuir | UK0040 | R | y | y | y |
| GB0640A | Exeter Roadside | UK0030 | U | y | y | y |
| GB0641A | Glasgow Centre | UK0024 | U | y | y | y |
| GB0034R | Glazebury | UK0033 | S | y | y | y |
| GB0035R | Great Dun Fell | UK0033 | RB | y |  |  |
| GB0036R | Harwell | UK0031 | R | y | y | y |
| GB0014R | High Muffles | UK0034 | R | y | y | y |
| GB0776A | Hull Freetown | UK0018 | U | y | y | y |
| GB0037R | Ladybower | UK0032 | R | y | y | y |
| GB0643A | Leamington Spa | UK0035 | U | y | y | y |
| GB0584A | Leeds Centre | UK0004 | U | y | y | y |
| GB0597A | Leicester Centre | UK0011 | U | y | y | y |
| GB0777A | Liverpool Speke | UK0006 | U | y | y | y |
| GB0608A | London Bexley | UK0001 | S | y | y | y |
| GB0566A | London Bloomsbury | UK0001 | U | y | y | y |
| GB0616A | London Brent | UK0001 | U | y | y | y |
| GB0586A | London Eltham | UK0001 | S | y | y | y |
| GB0650A | London Hackney | UK0001 | U | y | y | y |
| GB0638A | London Haringey | UK0001 | U | y |  |  |
| GB0837A | London Harlington | UK0001 | U | y | y | y |
| GB0642A | London Hillingdon | UK0001 | S | y | y | y |
| GB0672A | London Lewisham | UK0001 | U | y | y | y |
| GB0682A | London Marylebone Road | UK0001 | U | y | y | y |
| GB0620A | London N. Kensington | UK0001 | U | y | y | y |
| GB0656A | London Southwark | UK0001 | U | y | y | y |
| GB0644A | London Teddington | UK0001 | U | y | y | y |
| GB0622A | London Wandsworth | UK0001 | U | y | y | y |
| GB0743A | London Westminster | UK0001 | U | y | y | y |
| GB0006R | Lough Navar | UK0043 | RB | y |  |  |
| GB0038R | Lullington Heath | UK0031 | R | y | y | y |
| GB0613A | Manchester Piccadilly | UK0003 | U | y | y | y |
| GB0649A | Manchester South | UK0003 | S | y | y | y |
| GB0838A | Market Harborough | UK0032 | R | y | y | y |
| GB0583A | Middlesbrough | UK0013 | U | y | y | y |
| GB0043R | Narberth | UK0041 | RB | y | y | y |
| GB0568A | Newcastle Centre | UK0005 | U | y | y | y |
| GB0738A | Northampton | UK0032 | U | y | y | y |
| GB0684A | Norwich Centre | UK0029 | U | y | y | y |
| GB0646A | Nottingham Centre | UK0008 | U | y | y | y |
| GB0687A | Plymouth Centre | UK0030 | U | y | y | y |
| GB0651A | Port Talbot | UK0027 | U | y | y | y |
| GB0733A | Portsmouth | UK0012 | U | y | y | y |
| GB0731A | Preston | UK0023 | U | y | y | y |
| GB0840A | Reading New Town | UK0016 | U | y | y | y |
| GB0679A | Redcar | UK0013 | S | y | y | y |
| GB0617A | Rochester | UK0031 | R | y | y | y |
| GB0677A | Rotherham Centre | UK0007 | U | y | y | y |
| GB0660A | Salford Eccles | UK0003 | U | y | y | y |
| GB0698A | Sandwell West Bromwich | UK0002 | U | y | y | y |
| GB0615A | Sheffield Centre | UK0007 | U | y | y | y |
| GB0039R | Sibton | UK0029 | RB | y |  |  |
| GB0044R | Somerton | UK0030 | R | y | y | y |
| GB0598A | Southampton Centre | UK0019 | U | y | y | y |
| GB0728A | Southend-on-Sea | UK0021 | U | y | y | y |
| GB0754A | St Osyth | UK0029 | R | y | y | y |
| GB0658A | Stoke-on-Trent Centre | UK0014 | U | y | y | y |
| GB0015R | Strath Vaich | UK0039 | RB | y |  |  |
| GB0863A | Sunderland Silksworth | UK0036 | U | y | y | y |
| GB0609A | Swansea | UK0027 | U | y | y | y |
| GB0645A | Thurrock | UK0029 | U | y | y | y |
| GB0745A | Weybourne | UK0029 | R | y |  |  |
| GB0045R | Wicken Fen | UK0029 | R | y | y | y |
| GB0864A | Wigan Centre | UK0033 | U | y | y | y |
| GB0736A | Wigan Leigh | UK0033 | U | y | y | y |
| GB0730A | Wirral Tranmere | UK0020 | U | y | y | y |
| GB0614A | Wolverhampton Centre | UK0002 | U | y | y | y |
| GB0013R | Yarner Wood | UK0030 | R | y | y | y |

Table A1.2  Data capture rates for sites used in model calibrations for AQDD1-3, 2004

| Site | Data Capture (%) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SO2 | NO2 | PM10 | CO | Benzene | Lead | O3 | PM2,5 |
| Aberdeen | 93 | 90 | 93 | 93 | nm | nm | 94 | nm |
| Aston Hill | nm | 87 | nm | nm | nm | nm | 89.4 | nm |
| Avonmouth BZL | nm | nm | nm | nm | nm | 33.3 | nm | nm |
| Barnsley 12 | 98.9 | nm | nm | nm | nm | nm | nm | nm |
| Barnsley Gawber | 96.6 | 95.9 | nm | 96.5 | nm | nm | 96.2 | nm |
| Barnsley Gawber HC | nm | nm | nm | nm | 99.9 | nm | nm | nm |
| Bath Roadside | nm | 98.3 | nm | 99 | nm | nm | nm | nm |
| Belfast Centre | 95.1 | 91.8 | 95 | 96.4 | nm | nm | 96.4 | nm |
| Belfast Centre HC | nm | nm | nm | nm | 98 | nm | nm | nm |
| Belfast Clara St | nm | nm | 93 | nm | nm | nm | nm | nm |
| Belfast East | 96.5 | nm | nm | nm | nm | nm | nm | nm |
| Belfast Roadside | nm | nm | nm | nm | 95.7 | nm | nm | nm |
| Billingham | nm | 99.1 | nm | nm | nm | nm | nm | nm |
| Birmingham Centre | 86 | 88.9 | 94 | 91.6 | nm | nm | 88.4 | nm |
| Birmingham East | 58.4 | 53.8 | 57 | 58.4 | nm | nm | 58 | nm |
| Birmingham Roadside | nm | nm | nm | nm | 99.4 | nm | nm | nm |
| Birmingham Tyburn | 37.5 | 36.5 | 38 | 37.6 | nm | nm | 37.6 | nm |
| Blackpool | 60.4 | 77.7 | 83 | 82.4 | nm | nm | 82.6 | nm |
| Bolton | 97.6 | 94.1 | 96 | 97.5 | nm | nm | 97.6 | nm |
| Bottesford | nm | nm | nm | nm | nm | nm | 98.7 | nm |
| Bournemouth | 97.6 | 96.4 | 95 | 98.6 | nm | nm | 98.9 | nm |
| Bournemouth HC | nm | nm | nm | nm | 99.7 | nm | nm | nm |
| Bradford Centre | 96 | 95.8 | 95 | 94.5 | nm | nm | 96.8 | nm |
| Brentford Roadside | nm | 92 | nm | 95.2 | nm | nm | nm | nm |
| Brighton Preston Park | nm | 15.7 | nm | nm | nm | nm | 14.4 | nm |
| Brighton Roadside | nm | 98.9 | nm | 98.9 | nm | nm | nm | nm |
| Brighton Roadside PM10 | nm | nm | 94 | nm | nm | nm | nm | nm |
| Bristol Centre | 95.8 | 96.7 | 97 | 97.7 | nm | nm | 91.3 | nm |
| Bristol Old Market | nm | 98.7 | nm | 71.9 | nm | nm | nm | nm |
| Bristol Old Market HC | nm | nm | nm | nm | 99.4 | nm | nm | nm |
| Brookside Metals | nm | nm | nm | nm | nm | 100 | nm | nm |
| Bury Roadside | 75.9 | 91.6 | 91 | 93.6 | nm | nm | 91.9 | nm |
| Bush Estate | nm | 93.6 | nm | nm | nm | nm | 98.4 | nm |
| BZL Ltd., Hallen Village | nm | nm | nm | nm | nm | 100 | nm | nm |
| Cambridge Roadside | nm | 96.5 | nm | nm | nm | nm | nm | nm |
| Camden Kerbside | nm | 39.4 | 96 | nm | nm | nm | nm | nm |
| Canterbury | nm | 96.9 | 98 | nm | nm | nm | nm | nm |
| Cardiff | nm | nm | nm | nm | nm | 100 | nm | nm |
| Cardiff Centre | 97 | 97.5 | 93 | 96 | nm | nm | 91.1 | nm |
| Cardiff Centre HC | nm | nm | nm | nm | 91 | nm | nm | nm |
| Coventry Memorial Park | 98.3 | 97.6 | 92 | 98.3 | nm | nm | 98 | nm |
| Coventry Memorial Park HC | nm | nm | nm | nm | 99.7 | nm | nm | nm |
| Cwmbran | 96.9 | 99.4 | 100 | 98 | nm | nm | 99.5 | nm |
| Cwmbran HC | nm | nm | nm | nm | 100 | nm | nm | nm |
| Derry | 95.8 | 92 | 96 | 97.4 | nm | nm | 97.5 | nm |
| Dumfries | nm | 96.6 | 92 | 98.5 | nm | nm | nm | nm |
| Edinburgh Med. Sch. | nm | nm | nm | nm | nm | nm | nm | nm |
| Edinburgh Med. Sch. HC | nm | nm | nm | nm | 99.7 | nm | nm | nm |
| Edinburgh St Leonards | 98.5 | 91 | 99 | 98.1 | nm | nm | 93.9 | nm |
| Eskdalemuir | nm | 5.9 | nm | nm | nm | nm | 90.5 | nm |
| Eskdalemuir | nm | nm | nm | nm | nm | 100 | nm | nm |
| Exeter Roadside | 84.9 | 95.8 | nm | 87.4 | nm | nm | 97 | nm |
| Glasgow | nm | nm | nm | nm | nm | 100 | nm | nm |
| Glasgow Centre | 86.6 | 88.5 | 66 | 92.1 | nm | nm | 97.5 | nm |
| Glasgow City Chambers | nm | 98 | nm | 98.9 | nm | nm | nm | nm |
| Glasgow Kerbside | nm | 96 | 93 | 98.2 | nm | nm | nm | nm |
| Glasgow Kerbside HC | nm | nm | nm | nm | 83 | nm | nm | nm |
| Glazebury | nm | 87.3 | nm | nm | nm | nm | 95.9 | nm |
| Grangemouth | 98.6 | 98.5 | 98 | 81.2 | nm | nm | nm | nm |
| Grangemouth HC | nm | nm | nm | nm | 100 | nm | nm | nm |
| Great Dun Fell | nm | nm | nm | nm | nm | nm | 99 | nm |
| Haringey Roadside | nm | 98.2 | 99 | nm | nm | nm | nm | nm |
| Haringey Roadside HC | nm | nm | nm | nm | 100 | nm | nm | nm |
| Harwell | 96.4 | 95.7 | 96 | nm | nm | nm | 90.2 | 96.2 |
| Harwell HC | nm | nm | nm | nm | 75 | nm | nm | nm |
| High Muffles | nm | 70.1 | nm | nm | nm | nm | 99.2 | nm |
| Hove Roadside | 96 | 94.3 | nm | 98.5 | nm | nm | nm | nm |
| Hove Roadside HC | nm | nm | nm | nm | 98.3 | nm | nm | nm |
| Hull Freetown | 88.1 | 89.3 | 96 | 95.9 | nm | nm | 93.9 | nm |
| Hull Freetown HC | nm | nm | nm | nm | 100 | nm | nm | nm |
| IMI Refiners, Walsall | nm | nm | nm | nm | nm | 100 | nm | nm |
| Inverness | nm | 98.1 | 95 | 98.5 | nm | nm | nm | nm |
| Ladybower | 97.1 | 89.9 | nm | nm | nm | nm | 85.1 | nm |
| Leamington Spa | 98.4 | 93.8 | 98 | 88.8 | nm | nm | 98.7 | nm |
| Leamington Spa HC | nm | nm | nm | nm | 94.8 | nm | nm | nm |
| Leed Centre HC | nm | nm | nm | nm | 99.7 | nm | nm | nm |
| Leeds | nm | nm | nm | nm | nm | 100 | nm | nm |
| Leeds Centre | 85.5 | 92.1 | 98 | 79.4 | nm | nm | 82.7 | nm |
| Leeds Roadside | nm | nm | nm | nm | nm | nm | nm | nm |
| Leeds Roadside | nm | nm | nm | nm | 98.6 | nm | nm | nm |
| Leicester Centre | 97.3 | 85.5 | 95 | 84.9 | nm | nm | 97.8 | nm |
| Leicester Centre | nm | nm | nm | nm | 98.1 | nm | nm | nm |
| Liverpool Speke | 97.9 | 98.1 | 95 | 98.2 | nm | nm | 97.8 | nm |
| Liverpool Speke | nm | nm | nm | nm | 69.3 | nm | nm | nm |
| London A3 Roadside | nm | 96.8 | 98 | 96.9 | nm | nm | nm | nm |
| London Bexley | 95.8 | 96.1 | 92 | 94.9 | nm | nm | 95.5 | nm |
| London Bloomsbury | 97.6 | 97.5 | 98 | 97 | nm | nm | 97.1 | 98 |
| London Bloomsbury HC | nm | nm | nm | nm | 100 | nm | nm | nm |
| London Brent | 88.7 | 91 | 94 | 86.2 | nm | nm | 95 | nm |
| London Brent | nm | nm | nm | nm | nm | 100 | nm | nm |
| London Bromley | nm | 98.4 | nm | 96.1 | nm | nm | nm | nm |
| London Cromwell | nm | nm | nm | nm | nm | 100 | nm | nm |
| London Cromwell Road 2 | 98.6 | 98.7 | nm | 97.7 | nm | nm | nm | nm |
| London Eltham | 99 | 97.2 | 91 | nm | nm | nm | 96.4 | nm |
| London Eltham | nm | nm | nm | nm | 82.3 | nm | nm | nm |
| London Hackney | nm | 99.4 | nm | 94.7 | nm | nm | 88.4 | nm |
| London Haringey | nm | nm | nm | nm | nm | nm | 94.2 | nm |
| London Harlington | nm | 99 | 100 | 92.2 | nm | nm | 94.7 | nm |
| London Hillingdon | 97.9 | 97.6 | 98 | 97.8 | nm | nm | 92.6 | nm |
| London Horseferry | nm | nm | nm | nm | nm | 100 | nm | nm |
| London Lewisham | 97.8 | 97.8 | nm | nm | nm | nm | 87.8 | nm |
| London Marylebone Road | 92 | 98.3 | 99 | nm | nm | nm | 98.1 | 95.9 |
| London Marylebone Road HC | nm | nm | nm | nm | 85 | nm | nm | nm |
| London N. Kensington | 97.3 | 98.9 | 95 | 98.9 | nm | nm | 97.9 | nm |
| London Southwark | 94.8 | 88.1 | nm | 94.6 | nm | nm | 94.8 | nm |
| London Teddington | 96.1 | 93.8 | nm | nm | nm | nm | 96.1 | nm |
| London Wandsworth | nm | 99.3 | nm | nm | nm | nm | 99.3 | nm |
| London Westminster | 90.7 | 78.3 | 94 | 90.4 | nm | nm | 93.7 | nm |
| Lough Navar | nm | nm | 100 | nm | nm | nm | 74.8 | nm |
| Lullington Heath | 89.1 | 92.8 | nm | nm | nm | nm | 95.6 | nm |
| Manchester | nm | nm | nm | nm | nm | 100 | nm | nm |
| Manchester Piccadilly | 95 | 93.8 | 97 | 97.2 | nm | nm | 97.6 | nm |
| Manchester Piccadilly HC | nm | nm | nm | nm | 98 | nm | nm | nm |
| Manchester South | 85.8 | 87.4 | nm | nm | nm | nm | 95.4 | nm |
| Manchester Town Hall | nm | 94.7 | nm | 81 | nm | nm | nm | nm |
| Market Harborough | nm | 90.4 | nm | 93.2 | nm | nm | 93.5 | nm |
| Middlesbrough | 98.6 | 64.5 | 97 | 91.7 | nm | nm | 99.1 | nm |
| Middlesbrough HC | nm | nm | nm | nm | 100 | nm | nm | nm |
| Motherwell | nm | nm | nm | nm | nm | 100 | nm | nm |
| Narberth | 90.5 | 89.4 | 55 | nm | nm | nm | nm | nm |
| Newcastle | nm | nm | nm | nm | nm | 100 | nm | nm |
| Newcastle Centre | 83.7 | 81.9 | 92 | 85 | nm | nm | 90.3 | nm |
| Newcastle Centre HC | nm | nm | nm | nm | 96.4 | nm | nm | nm |
| Northampton | 89.3 | 87.1 | 89 | 90.4 | nm | nm | 87.2 | nm |
| Northampton HC | nm | nm | nm | nm | 99.7 | nm | nm | nm |
| Northampton PM10 | nm | nm | 84 | nm | nm | nm | nm | nm |
| Norwich Centre | 96.4 | 90.7 | 96 | 94.2 | nm | nm | 97.2 | nm |
| Norwich Centre HC | nm | nm | nm | nm | 99.9 | nm | nm | nm |
| Norwich Roadside | nm | 97.7 | nm | nm | nm | nm | nm | nm |
| Nottingham Centre | 95.7 | 90.9 | 96 | 91.3 | nm | nm | 97.1 | nm |
| Nottingham Centre HC | nm | nm | nm | nm | 97.7 | nm | nm | nm |
| Oxford Centre | 99.1 | 86.5 | nm | 97.2 | nm | nm | nm | nm |
| Oxford Centre HC | nm | nm | nm | nm | 98.7 | nm | nm | nm |
| Plymouth Centre | 67.6 | 89.1 | 97 | 88.8 | nm | nm | 97.8 | nm |
| Plymouth Centre HC | nm | nm | nm | nm | 91.7 | nm | nm | nm |
| Port Talbot | 97.2 | 83.9 | 96 | nm | nm | nm | 97.2 | nm |
| Portsmouth | 90.7 | 98.1 | 94 | 96.7 | nm | nm | 98.9 | nm |
| Portsmouth HC | nm | nm | nm | nm | 99.3 | nm | nm | nm |
| Preston | 98.2 | 94.3 | 98 | 93 | nm | nm | 98.3 | nm |
| Reading HC | nm | nm | nm | nm | 100 | nm | nm | nm |
| Reading New Town | 88.9 | 93.2 | 96 | 94.4 | nm | nm | 86 | nm |
| Redcar | 97.7 | 97.7 | 98 | 97 | nm | nm | 96.3 | nm |
| Rochester | 98.6 | 96.4 | 98 | nm | nm | nm | 98.6 | 98.7 |
| Rotherham Centre | 75.5 | 96.9 | nm | nm | nm | nm | 90 | nm |
| Runcorn Eston Point | nm | nm | nm | nm | nm | 33.3 | nm | nm |
| Salford Eccles | 90.1 | 96.2 | 94 | 93.9 | nm | nm | 87.4 | nm |
| Sandwell West Bromwich | 98.1 | 98.2 | nm | 97.8 | nm | nm | 97.9 | nm |
| Scunthorpe | 20.8 | nm | nm | nm | nm | nm | nm | nm |
| Scunthorpe Town | 55.5 | nm | 54 | nm | nm | nm | nm | nm |
| Sheffield | nm | nm | nm | nm | nm | 100 | nm | nm |
| Sheffield Centre | 80.1 | 97.1 | 97 | 98.1 | nm | nm | 98 | nm |
| Sheffield Centre HC | nm | nm | nm | nm | 96 | nm | nm | nm |
| Sheffield Tinsley | nm | 95.7 | nm | 97.3 | nm | nm | nm | nm |
| Sibton | nm | nm | nm | nm | nm | nm | 96.3 | nm |
| Somerton | nm | 88.8 | nm | nm | nm | nm | 95.6 | nm |
| Southampton Centre | 96.4 | 95.2 | 96 | 91 | nm | nm | 90.9 | nm |
| Southampton Centre HC | nm | nm | nm | nm | 95.3 | nm | nm | nm |
| Southend-on-Sea | 95.2 | 91.5 | 95 | 51.6 | nm | nm | 97.4 | nm |
| Southend-on-Sea HC | nm | nm | nm | nm | 99.7 | nm | nm | nm |
| Southwark Roadside | 95.4 | 75.4 | nm | 98.6 | nm | nm | nm | nm |
| St Osyth | nm | 91 | nm | 98.9 | nm | nm | 99 | nm |
| Stockport Shaw Heath | 45.8 | 90.9 | 87 | 78.3 | nm | nm | nm | nm |
| Stockton-on-Tees Yarm | nm | 98.7 | 95 | 93.9 | nm | nm | nm | nm |
| Stockton-on-Tees Yarm HC | nm | nm | nm | nm | 100 | nm | nm | nm |
| Stoke-on-Trent Centre | 86.6 | 93.2 | 78 | 94.2 | nm | nm | 98 | nm |
| Stoke-on-Trent Centre | nm | nm | nm | nm | 91.5 | nm | nm | nm |
| Strath Vaich | nm | nm | nm | nm | nm | nm | 83.9 | nm |
| Sunderland | 92.1 | nm | nm | nm | nm | nm | nm | nm |
| Sunderland Silksworth | nm | 6.3 | nm | nm | nm | nm | 6.3 | nm |
| Swansea | 97.6 | 91.5 | nm | 97.7 | nm | nm | 97 | nm |
| Swansea Lead | nm | nm | nm | nm | nm | 33.3 | nm | nm |
| Thurrock | 97.8 | 89.8 | 95 | 96.1 | nm | nm | 98.4 | nm |
| Tower Hamlets Roadside | nm | 96.3 | nm | 83.8 | nm | nm | nm | nm |
| Walsall Alumwell | nm | 93 | nm | nm | nm | nm | nm | nm |
| Walsall Willenhall | nm | 92.1 | nm | nm | nm | nm | nm | nm |
| West London | nm | 98.8 | nm | 98.8 | nm | nm | nm | nm |
| Weybourne | nm | nm | nm | nm | nm | nm | 97.1 | nm |
| Wicken Fen | 93.5 | 73.1 | nm | nm | nm | nm | 93.2 | nm |
| Wigan Centre | 22.2 | 22.2 | nm | 23 | nm | nm | 23.1 | nm |
| Wigan Centre HC | nm | nm | nm | nm | 25.7 | nm | nm | nm |
| Wigan Leigh | 51.4 | 71.4 | 72 | 72 | nm | nm | 70.2 | nm |
| Wigan Leigh HC | nm | nm | nm | nm | 49.3 | nm | nm | nm |
| Wirral Tranmere | 95.8 | 94 | 98 | 94.9 | nm | nm | 98.4 | nm |
| Wolverhampton Centre | 97.8 | 80.4 | 98 | 93.4 | nm | nm | 97.7 | nm |
| Wrexham | 89 | 95.7 | 94 | 98.4 | nm | nm | nm | nm |
| Yarner Wood | nm | 98.5 | nm | nm | nm | nm | 97.5 | nm |

nm = not measured

APPENDIX 2. MONITORING SITES USED TO VERIFY THE MAPPED ESTIMATES

Table A2.1. Monitoring sites used to verify the mapped estimates

| Site | Site Type | Authority | CO | NO2 | O3 | PM10 | SO2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Abingdon | URBAN BACKGROUND | Vale of White Horse DC |  | y | y | y | y |
| Antrim Greystone Estate | URBAN BACKGROUND | Antrim BC |  |  |  |  | y |
| Ards | URBAN BACKGROUND | Ards BC |  |  |  | y | y |
| Armagh Dobbin Street | URBAN BACKGROUND | Armagh BC |  |  |  |  | y |
| Ashford Roadside ZA2 | ROADSIDE | Ashford BC |  | y |  | y |  |
| Ballymena Ballykeel | URBAN BACKGROUND | Ballymena BC |  |  |  |  | y |
| Ballymena North Road | ROADSIDE | Ballymena BC |  | y |  |  |  |
| Ballymoney | URBAN BACKGROUND | Ballymoney BC |  |  |  | y |  |
| Barnsley A628 Roadside | ROADSIDE | Barnsley MBC |  | y |  |  |  |
| Barnsley Cudworth | URBAN BACKGROUND | Barnsley MBC |  | y |  | y | y |
| Barnsley Royston | URBAN BACKGROUND | Barnsley MBC |  | y |  | y | y |
| Basingstoke Eastrop | URBAN BACKGROUND | Basingstoke & Deane DC |  | y |  | y |  |
| Bedford Stewartby BF1 | RURAL | Mid Bedforshire DC |  |  | y |  | y |
| Bracknell Bagshot Road | ROADSIDE | Bracknell Forest BC |  | y |  | y |  |
| Bracknell Foxhill | URBAN BACKGROUND | Bracknell Forest BC |  | y | y | y |  |
| Bracknell Yorktown Road Sandhurst | ROADSIDE | Bracknell Forest BC |  | y |  | y |  |
| Broxbourne BB1 | ROADSIDE | Broxbourne DC |  | y | y | y | y |
| Bury Prestwich | ROADSIDE | Bury MBC |  | y |  | y |  |
| Bury Radcliffe | ROADSIDE | Bury MBC |  | y |  | y |  |
| Bury Town Centre | URBAN CENTRE | Bury MBC |  |  | y | y |  |
| Caerphilly | URBAN BACKGROUND | Caerphilly County BC |  | y | y |  |  |
| Caerphilly Ton-y-Felin Roadside | ROADSIDE | Caerphilly County BC |  | y | y | y |  |
| Cambridge Gonville Place | ROADSIDE | Cambridge City Council |  | y |  | y |  |
| Cambridge Newmarket Road | URBAN BACKGROUND | Cambridge City Council |  | y |  | y |  |
| Cambridge Parker Street | ROADSIDE | Cambridge City Council |  | y |  | y |  |
| Cambridge Silver Street | ROADSIDE | Cambridge City Council |  | y |  | y |  |
| Canterbury Background ZY1 | URBAN BACKGROUND | Canterbury City Council |  | y | y | y | y |
| Canterbury Roadside ZY2 | ROADSIDE | Canterbury City Council | y | y |  |  |  |
| Cardiff Briardene | URBAN BACKGROUND | Cardiff City Council | y | y | y | y | y |
| Carrickfergus Rosebrook Avenue | URBAN BACKGROUND | Carrickfergus BC |  |  |  | y | y |
| Castlereagh Espie Way | URBAN BACKGROUND | Castlereagh BC |  |  |  | y | y |
| Castlereagh Lough View Drive | ROADSIDE | Castlereagh BC |  | y |  | y |  |
| Chatham Roadside ZC1 | ROADSIDE | Medway DC |  | y | y | y | y |
| Chepstow | ROADSIDE | Monthmouthshire CC |  | y |  | y |  |
| Chesterfield Birdholme School | URBAN BACKGROUND | Chesterfield BC |  | y |  | y | y |
| Chesterfield Whittington Moor | ROADSIDE | Chesterfield BC |  | y |  | y | y |
| Chesterfield Whittington Roadside | ROADSIDE | Chesterfield BC |  | y |  | y | y |
| Craigavon Castle Lane | URBAN BACKGROUND | Craigavon BC |  | y |  |  |  |
| Craigavon Lord Lurgan Park | URBAN BACKGROUND | Craigavon BC |  |  |  | y | y |
| Derry Brandywell | URBAN BACKGROUND | Derry City Council |  |  |  | y | y |
| Derry Dale's Corner | ROADSIDE | Derry City Council |  | y |  |  |  |
| Dover Background ZD3 | URBAN BACKGROUND | Dover DC |  |  |  |  | y |
| Dover Background ZD4 | URBAN BACKGROUND | Dover DC |  |  |  |  | y |
| Dover Roadside ZD1 | ROADSIDE | Dover DC |  | y |  |  |  |
| Dover Roadside ZD2 | ROADSIDE | Dover DC |  | y | y | y | y |
| East Herts Sawbridgeworth EH2 | ROADSIDE | East Hertfordshire DC |  | y | y | y | y |
| East Herts Sawbridgeworth EH3 | URBAN BACKGROUND | East Hertfordshire DC |  | y |  | y |  |
| East Herts Sawbridgeworth EH4 | URBAN BACKGROUND | East Hertfordshire DC |  | y |  |  |  |
| East Herts Sawbridgeworth EH5 | ROADSIDE | East Hertfordshire DC |  | y |  |  |  |
| Flintshire | ROADSIDE | Flintshire CC |  | y |  | y |  |
| Folkestone ZF1 | SUBURBAN | Shepway DC |  | y | y | y | y |
| Guildford Gyratory | ROADSIDE | Guildford BC | y | y |  | y |  |
| Hertsmere Borehamwood HM1 | URBAN BACKGROUND | Hertsmere BC | y | y | y | y |  |
| Ipswich Piper's Court | ROADSIDE | Ipswich BC |  | y |  |  |  |
| King's Lynn South Quay | URBAN BACKGROUND | Kings Lynn & West Norfolk BC |  |  |  | y |  |
| Larne | URBAN BACKGROUND | Larne BC |  |  |  | y | y |
| Limavady Coolessan | URBAN BACKGROUND | Limavady BC |  |  |  | y |  |
| Lisburn Dunmurry High School | URBAN BACKGROUND | Lisburn City Council |  |  |  | y |  |
| Lisburn Island Civic Centre | URBAN BACKGROUND | Lisburn City Council |  |  |  | y | y |
| Lisburn Lagan Valley Hospital | ROADSIDE | Lisburn City Council |  | y |  | y |  |
| Liverpool Islington | ROADSIDE | Liverpool City Council |  | y |  |  | y |
| Luton Background ZL1 | URBAN BACKGROUND | Luton BC | y | y | y | y | y |
| Luton LN1 | URBAN BACKGROUND | Luton BC | y | y | y | y | y |
| Macclesfield Disley | ROADSIDE | Macclesfield BC |  | y |  | y |  |
| Maidstone Roadside ZM2 | ROADSIDE | Maidstone BC | y | y |  | y | y |
| Maidstone Rural ZM3 | RURAL | Maidstone BC |  | y | y | y | y |
| Marchlyn Mawr | REMOTE | Gwyneth Council |  | y | y |  |  |
| Monmouth | URBAN BACKGROUND | Monmouthshire CC |  | y |  | y |  |
| Newham Cam Road | ROADSIDE | London Borough of Newham | y | y | y | y | y |
| Newham Wren Close | URBAN BACKGROUND | London Borough of Newham | y | y | y | y | y |
| Newport Malpas Depot | URBAN BACKGROUND | Newport County BC |  | y | y |  |  |
| Newport St Julians Comp School | URBAN BACKGROUND | Newport County BC |  | y |  |  |  |
| Newry Kilkeel | URBAN BACKGROUND | Newry and Mourne DC |  |  |  | y |  |
| Newry Monaghan Row | URBAN BACKGROUND | Newry and Mourne DC |  | y |  | y | y |
| Newry Trevor Hill | ROADSIDE | Newry and Mourne DC |  | y |  | y | y |
| Newtownabbey Mallusk | URBAN BACKGROUND | Newtownabbey BC |  | y |  |  |  |
| Newtownabbey Shore Road | ROADSIDE | Newtownabbey BC |  | y |  |  |  |
| North Down Bangor | URBAN BACKGROUND | North Down BC |  |  |  | y | y |
| North Down Holywood A2 | ROADSIDE | North Down BC |  | y |  | y |  |
| North Herts Breechwood Green NH4 | URBAN BACKGROUND | North Hertfordshire DC |  | y |  | y |  |
| North Lincs Keadby | URBAN BACKGROUND | North Lincolnshire Council |  | y |  | y | y |
| North Lincs Killingholme | URBAN BACKGROUND | North Lincolnshire Council |  | y |  | y | y |
| Oldham West End House | URBAN BACKGROUND | Oldham MBC | y | y | y | y | y |
| Omagh Tamlaght | URBAN BACKGROUND | Omagh BC |  |  |  | y |  |
| Oxford High St | ROADSIDE | Oxford City Council |  | y |  |  |  |
| Oxford St Ebbes | URBAN BACKGROUND | Oxford City Council |  | y |  |  |  |
| Rhondda-Cynon-Taf Nantgarw | ROADSIDE | Rhondda-Cynon-Taf CBC |  |  |  | y |  |
| Rhondda-Cynon-Taf Pontypridd | URBAN BACKGROUND | Rhondda-Cynon-Taf CBC | y |  | y | y |  |
| Ribble Valley Chatburn | URBAN INDUSTRIAL | Ribble Valey BC |  | y |  | y | y |
| Ribble Valley Lillands | URBAN BACKGROUND | Ribble Valey BC |  | y |  | y | y |
| S Cambs Bar Hill | ROADSIDE | South Cambridgeshire DC |  | y |  | y |  |
| S Cambs Barrington Challis Grn | RURAL | South Cambridgeshire DC |  |  |  |  | y |
| S Cambs Barrington Fruit Farm | RURAL | South Cambridgeshire DC |  |  |  |  | y |
| S Cambs Impington | ROADSIDE | South Cambridgeshire DC |  | y |  | y |  |
| Salford M60 | ROADSIDE | Salford MBC | y | y | y | y | y |
| Sevenoaks 2 - Greatness ZV1 | URBAN BACKGROUND | Sevenoaks DC | y | y | y | y | y |
| Slough Colnbrook | URBAN BACKGROUND | Slough BC |  | y |  | y |  |
| Slough Town Centre A4 | ROADSIDE | Slough BC |  | y |  | y |  |
| South Beds Dunstable SB1 | URBAN BACKGROUND | South Bedfordshire DC |  | y |  | y |  |
| South Bucks Gerrards Cross | URBAN BACKGROUND | South Bucks DC |  | y |  | y |  |
| South Holland | RURAL | South Holland DC |  | y |  |  |  |
| St Albans Fleetville SA1 | URBAN BACKGROUND | St Albans DC | y | y | y | y | y |
| Stevenage SE1 | ROADSIDE | Stevenage BC |  | y |  | y |  |
| Stockport Bredbury | URBAN BACKGROUND | Stockport MBC |  | y |  | y |  |
| Strabane Springhill Park | URBAN BACKGROUND | Strabane DC |  |  |  | y | y |
| Swale Background ZW2 | URBAN BACKGROUND | Swale DC |  | y |  | y | y |
| Swale Roadside ZW1 | ROADSIDE | Swale DC |  | y |  | y |  |
| Swansea Morfa Roadside | ROADSIDE | Swansea CC | y | y |  | y | y |
| Swansea Morriston Roadside | ROADSIDE | Swansea CC | y | y | y | y | y |
| Tameside Two Trees School | URBAN BACKGROUND | Tameside MBC | y | y | y | y | y |
| Thanet Airport ZH3 | AIRPORT | Thanet DC |  | y |  |  |  |
| Three Rivers Rickmansworth TR1 | URBAN BACKGROUND | Three Rivers DC |  | y |  | y |  |
| Tonbridge Roadside ZT1 | ROADSIDE | Tonbridge & Malling DC |  | y |  |  |  |
| Trafford | URBAN BACKGROUND | Trafford MBC |  | y |  | y | y |
| Trafford A56 | ROADSIDE | Trafford MBC |  | y |  | y |  |
| Tunbridge Wells Background ZT2 | URBAN BACKGROUND | Tunbridge Wells DC |  | y |  |  |  |
| V Glamorgan Fonmon | RURAL | Vale of Glamorgan Council |  | y | y |  | y |
| Watford Roadside WF1 | ROADSIDE | Watford BC | y | y | y | y |  |
| Welwyn Hatfield WGC WH1 | URBAN BACKGROUND | Welwyn and Hatfield Council |  | y | y |  |  |
| Wokingham Winnersh | URBAN BACKGROUND | Wokingham DC |  | y |  | y |  |
| Wycombe Stokenchurch | URBAN BACKGROUND | Wycombe DC |  | y |  |  |  |

Bush T (2000). Article 5 Assessment of Nitrogen Dioxide, PM10, sulphur dioxide and lead in the UK. Report to the Department for Environment, Food and Rural Affairs, the Scottish Executive, Welsh Assembly Government and the Department of the Environment in Northern Ireland. AEA Technology, Netcen report AEAT/R/ENV/0165.

http://www.defra.gov.uk/environment/airquality/article5/index.htm

Bush T (2002) Preliminary Assessment of benzene and carbon monoxide levels in the UK. Report to the Department for Environment, Food and Rural Affairs, the Scottish Executive, Welsh Assembly Government and the Department of the Environment in Northern Ireland. AEA Technology, Netcen report AEAT/ENV/R/1333/Issue 1 http://www.airquality.co.uk/archive/reports/cat09/art5_dd2_v3aeat.pdf

Bush T and Kent A (2003). Preliminary Assessment of ozone levels in the UK . Report to the Department for Environment, Food and Rural Affairs, the Scottish Executive, Welsh Assembly Government and the Department of the Environment in Northern Ireland. AEA Technology, Netcen report AEAT/ENV/R/1528/Issue 1. http://www.airquality.co.uk/archive/reports/cat09/0506130933_o3dd1_art5_rep2.pdf

Stedman et al (2005). UK air quality modelling for annual reporting 2004 on ambient air quality assessment under Council Directives 96/62/EC, 1999/30/EC and 2000/69/EC. Report to the Department for Environment, Food and Rural Affairs, the Scottish Executive, Welsh Assembly Government and the Department of the Environment in Northern Ireland. AEA Technology, Netcen report. AEAT/ENV/R/2052/Issue 1.

The Hertfordshire & Bedfordshire Air Pollution Monitoring Network http://www.hertsbedsair.org.uk

The Kent and Medway Air Quality Monitoring Network http://www.kentair.org.uk

T Bush  and J Targa, 2005. Ozone Mapping Techniques for the 3rd Daughter Directive; OSRM vs Empirical modelling Comparison Report. A report to The Department for Environment, Food and Rural Affairs, Welsh Assembly Government, The Scottish Executive and the Department of the Environment for Northern Ireland. AEA Technology plc, Netcen, Harwell. Report AEAT/ENV/R/2053

Coyle M, Smith R, Stedman J, Weston K and Fowler D, 2002. Quantifying the spatial distribution of surface

ozone concentration in the UK. Atmospheric Environment, 36 (2002) 1013-1024.

NEGTAP 2001, Transboundary Air Pollution: Acidification, Eutrophication and Ground-level ozone in the UK. Prepared by the National Expert Group on Transboundary Air Pollution (NEGTAP) on behalf of the Department for Environment, Food and Rural Affairs, the Scottish Executive, Welsh Assembly Government and the Department of the Environment in Northern Ireland. ISBN 1 870393 61 9.

PORG (UK Photochemical Oxidants Review Group), 1998. Ozone in the UK. 4th report of the UK Photochemical Oxidants Review Group, 1st Edition. The Department of the Environment Transport and the Regions.

Stedman, J R, Bush, T J, Vincent K J, Kent, A J, Grice, S and Abbott, J. (2005). UK air quality modelling for annual reporting 2004 on ambient air quality assessment under Council Directives 96/62/EC, 1999/30/EC and 2000/69/EC. AEA Technology, National Environmental Technology Centre. Report AEAT/ENV/R/1790.  http://www.airquality.co.uk/archive/reports/cat05/0501121424_dd12003mapsrep4.pdf

Coyle M, Smith R, Stedman J, Weston K and Fowler D, 2002. Quantifying the spatial distribution of surface

ozone concentration in the UK. Atmospheric Environment, 36 (2002) 1013-1024.