Appendix 7 Waste

CONTENTS

1	LANDFILL	2

2	WASTE WATER TREATMENT	3

3	WASTE  INCINERATION	5

4	REFERENCES	7

# Landfill

The NAEI category Landfill maps directly on to 6A1 Landfills for methane emissions.  Emissions are reported from managed landfills only, as open dumps and unmanaged landfills are considered insignificant in the UK.

The AEA Technology model of methane generation from landfill sites used for the past four years (Brown et al, 1999) has now been updated and revised for DEFRA by the consultants Land Quality Management (LQM 2003).

The UK method conforms to good practice since a first order decay (Tier 2) methodology is used based on estimates of historical data on waste quantities, composition and disposal practices over several decades.

The UK method is based on equations 4 and 5 in the Revised 1996 IPCC guidelines (IPCC, 1997) (pp 6.10-6.11) which are compatible with equations 5.1 and 5.2 in the Good Practice Guidance (IPCC, 2000).  A slightly different version of equation 5.1 is used, which takes into account the fact that the model uses a finite time interval (one year).  The full derivation of the equations used is given in Appendix 2 of Brown et al (1999).

The UK method divides the waste stream into four categories of waste: rapidly degrading, moderately degrading, slowly degrading and inert waste.  These categories each have a separate decay rate.  The decay rates have been revised slightly for the 2002 version of the model (LQM 2003). They now range from 0.046 (slowly degrading waste) to 0.116 (rapidly degrading waste), within the range of 0.030 to 0.200 quoted in the Good Practice Guidance.

The model extends back to 1945, which gives a time period of around 4 half lives for the slowest of the three decay rates (0.05, half life 14 years).  This lies within the range of 3 to 5 half lives recommended by the Good Practice Guidance.

As recommended, the model attempts to take into account changes in landfill practice over past decades by modifying the gas collection rate over time where appropriate.

The estimates of historical waste disposal and composition data are based on various data sources, described fully in Brown et al (1999) and LQM (2003).  As recommended in the Good Practice Guidance, estimates for municipal waste are based on population where data are absent.

As recommended in the Good Practice Guidance, the estimates of waste disposal quantities include commercial and industrial waste, demolition and construction waste and sewage sludge, as well as municipal waste.  For industrial and commercial waste, the data are based on national estimates from a recent survey. The data were extrapolated to cover past years based on employment rates in the industries concerned.

All sites in the UK are managed, and therefore have a methane correction factor of 1.0.

Degradable organic carbon (DOC) was estimated assuming that the DOC arises solely from the cellulose and hemi-cellulose content of the waste.

The fraction of degradable organic carbon dissimilated (DOCF) is derived from an analysis of a laboratory study by Barlaz et al (1997).  It varies from 50% to 98% depending on the waste fraction.  The new IPCC recommended range quoted in the Guidance is 50-60%.

The fraction of CH4 in landfill gas is generally taken to be 50%, which is in line with the Guidance.  For old shallow sites it is taken to be 30% to reflect a higher degree of oxidation.

The fraction of methane recovered was derived from a survey of statistics on gas use for power generation, and a survey of installed flare capacity, assuming that flares operate at full capacity except for 15% downtime.  In 2002 the estimates were that 24% of generated methane was utilised and 45% was flared.  The estimates are not derived from metering data, as recommended by the Guidance, as such data were not readily available at the time of the study.

The oxidation factor is based on a model which is parameterised using a survey of empirical field and laboratory observations of soil methane oxidation capacity, together with expert judgement.  Oxidation factors are estimated to be much higher than the IPCC default of 10% – the LQM model predicts that 10% of methane escapes through fissures nearly all the remainder - up to 90% is oxidised in the cover material (LQM, 2003).

Recovered methane is subtracted before applying the oxidation factor.  This is in line with the Guidance.

The emissions of pollutants from the flare stacks were not estimated but those from electricity generation and heat generation were.  Emissions from electricity generation are considered under Power Stations and emissions from heat generation are included under Miscellaneous and are discussed in Appendix 1.

An estimate of NMVOC emissions from landfills was made using an emission factor of 0.01 t NMVOC/ t methane produced which is equivalent to 5.65g NMVOC/ m3 landfill gas (Passant, 1993).

Neither the GHGI nor the NAEI reports carbon dioxide emissions from the anaerobic decay of landfilled waste since this is considered to be part of the carbon cycle and is not a net source.

The estimates include the contribution of sewage disposed of to landfill.

# Waste Water Treatment

The NAEI category Sewage is mapped on to the IPCC category 6B2 Domestic and Commercial Wastewater.  There is no estimate made of emissions from private wastewater treatment plants operated by companies prior to discharge to the public sewage system or rivers.  The NAEI estimate is based on the work of Hobson et al (1996) who estimated emissions of methane for the years 1990-95.  Subsequent years are extrapolated on the basis of population.  Sewage disposed of to landfill is included in landfill emissions.

The methodology of the UK model differs in some respects from the IPCC default methodology.  The main differences are that it considers wastewater and sewage together rather than separately.  It also considers domestic, commercial and industrial wastewater together rather than separately. Emissions are based on empirical emission factors derived from the literature expressed in kg CH4/tonne dry solids rather than the BOD default factors used by IPCC.  The model however complies with the IPCC Good Practice Guidance as a national model (IPCC, 2000).

The basic activity data are the throughput of sewage sludge through the public system.  The estimates are based on the UK population connected to the public sewers and estimates of the amount of sewage per head generated.  From 1995 onwards the per capita production is a projection (Hobson et al, 1996).  The main source of sewage activity data is the UK Sewage Survey (DOE, 1993).  Emissions are calculated by disaggregating the throughput of sewage into 14 different routes.  The routes consist of different treatment processes each with its own emission factor.  The treatment routes and emission factors are shown in Table 1.  The allocation of sludge to the treatment routes is reported for each year on the CRF tables attached to this report as a CD ROM.

The model accounts for recovery of methane and its subsequent utilization and flaring by estimating the proportion of anaerobic digester emissions that are recovered.

Table 1	Specific Methane Emission Factors for Sludge Handling kg Ch4/Mg dry solids, Hobson et al (1996)

| Sludge Handling System | Gravity Thick-ening1 | Long term storage | Anaerobic Digestion2 | Agricul -tural Land | Landfill |
| --- | --- | --- | --- | --- | --- |
| Anaerobic digestion to agriculture | 0.72 |  | 143 | 5 |  |
| Digestion, drying, agriculture | 0.72 |  | 143 | 5 |  |
| Raw sludge, dried to agriculture | 0.72 |  |  | 20 |  |
| Raw sludge, long term storage (3m) ,agriculture | 0.72 | 36 |  | 20 |  |
| Raw sludge, dewatered to cake, to agriculture | 0.72 |  |  | 20 |  |
| Digestion, to incinerator | 0.72 |  | 143 |  |  |
| Raw sludge, to incinerator | 0.72 |  |  |  |  |
| Digestion , to landfill | 0.72 |  | 143 |  | 0 |
| Compost, to agriculture | 0.72 |  |  | 5 |  |
| Lime raw sludge, to agriculture | 0.72 |  |  | 20 |  |
| Raw Sludge , to landfill | 0.72 |  |  |  | 0 |
| Digestion , to sea disposal | 0.72 |  | 143 |  |  |
| Raw sludge to sea disposal | 0.72 |  |  |  |  |
| Digestion to beneficial use(e.g. land reclamation) | 0.72 |  | 143 | 5 |  |

- An emission factor of 1 kg/tonne is used for gravity thickening, Around 72% of

- sludge is gravity thickened hence an aggregate factor of 0.72 kg CH4/Mg is used.

- The factor refers to  methane production, however it is assumed that 121.5 kg CH4/Mg

is recovered or flared

Nitrous oxide emissions from the treatment of human sewage are based on the IPCC (1997c) default methodology.  The average protein consumption per person is based on the National Food Survey (DEFRA, 2001).  These range from 22.7 to 23.7 g protein/person/day.  The food survey is based on household consumption of food and so may give a low estimate.

# Waste Incineration

The NAEI estimates emissions from the categories municipal solid waste (MSW) incineration, clinical waste incineration, cremation, and sewage incineration.  Included in the inventory for the first time are the categories chemical waste incineration and animal carcass incineration.  However the coverage of some of these waste incineration sources is incomplete due to a lack of emission factor data.  The waste incineration categories are mapped onto the single NAEI category 6C Waste Incineration.  The emission factors used are shown in Table 3.

Table 3	 Emission Factors for Waste Incineration (kg/t waste)

|  |  | C1 | CH4 | N2O | NOx | CO | NMVOC | SO2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MSW(old) | kg/t | 75a | 0.0008d | 0.03f | 1.8c | 0.709c | 0.0231d | 1.36c |
| MSW(new) | kg/t | 75a | 0.0008d | 0.03f | 1.52g | 0.165g | 0.027g | 0.103g |
| Cremation | kg/body | 0 | NE | NE | 0.308h | 0.141h | 0.013h | 0.0544h |
| Clinical | kg/t | 228f | 0.0008d | 0.03f | 1.78h | 1.48h | 0.018i | 1.09h |
| Sewage | kg/t | 0 | 0.39b | 0.8f | 2.5b | 15.5b | 0.84b | 2.3e |
| Chemical | kg/tj | 1030g | NE | NE | 1.35g | 0.375g | 0.846g | 1.13g |
| Animal carcass | ktonnes emitted | 32.5 | NE | NE | 0.245 | 3.3 | 2.0 | 0.165 |

1	Emission factor as kg carbon/ t waste

a	Royal Commission on Environmental Pollution (1993)

b	EMEP/CORINAIR (1996).

c	Clayton et al. (1991)

d	Estimated from THC data in CRI (Environment Agency, 1997) assuming 3.3% methane split given in EMEP/CORINAIR (1996)

e	EMEP/CORINAIR (1996). A factor of 14 kt/Mt is used prior to 1996.

f	IPCC (2000)

g	Emission factor for 2001, Environment Agency (2002)

h	EMEP/CORINAIR (1999)

i	Based on estimate for MSW incineration based on 2000 data from Environment Agency (2001)

j	Emission factors are calculated from emissions given in the Pollution Inventory using a nominal figure for waste burnt.  These emission factors are therefore not necessarily very accurate.

NE	Not estimated

The arisings of waste and their method of disposal are not all known with any reliability.  The estimates of municipal solid waste disposed of to incinerators are taken from DUKES  (DTI, 2002).  The amounts of sewage sludge incinerated are reported in DEFRA (2001a).  Data on cremations are published by the Cremation Society of Great Britain (CSGB, 2001).  Activities for clinical and chemical waste incineration are taken from on Smyllie et al (1996).

Under IPCC guidelines, incineration refers only to plant that do not generate electricity or heat.  From the end of 1996, MSW incinerators in the UK had to meet new standards.  As a result, many incinerators closed down, were renovated or new ones built.  From 1997 onwards all MSW incinerators generated electricity or heat and are classified as either power stations or under 1A4 Other Combustion so no emissions are reported under Incineration: MSW.  In previous inventories emissions from incinerators included some plant which generated heat.  In the new inventory, these plant are reported under miscellaneous or 1A4 Other Combustion.  Emission factors for modern incinerators based on 2001 data are reported as MSW (new) for comparison with the emission factors used for old incinerators.  The emission factors given for MSW (old) pertain to old incinerators prior to 1993.  Emission factors for the years 1993-2001 were derived from the Pollution Inventory (Environment Agency, 2002).  The reported emissions of carbon dioxide exclude those deriving from recently photosynthesised carbon.  It was assumed that the proportion of recently photosynthesised carbon was 75% of the total carbon content of the waste (Brown, 1995) and this assumption is reflected in the factors in Table 3.  Emissions of carbon dioxide from clinical waste incineration have been included in the Inventory.  The emission factor is a default taken from IPCC (2000).

Emission estimates for animal carcass incineration are taken from AEAT (2002).

# References

AEAT (2002) Atmospheric Emissions From Small Carcass Incinerators.  A report for the Department for Environment, Food and Rural Affairs.  DEFRA contract DEFRA/WA0806.  AEAT report number AEAT/ENV/R/0920. AEA Technology Environment, Culham Science Centre, Abingdon, Oxfordshire, OX14 3ED

Brown, K, (1995), Personal Communication, ETSU, AEA Technology.

Brown, KA, Smith, A, Burnley, SJ, Campbell, DJV, King, K, Milton, MJT, (1999) Methane Emissions from UK Landfills, AEA Technology, AEAT-5217, Culham

Clayton, P, Coleman, P, Leonard, A, Loader, A, Marlowe, Mitchell, D, Richardson, S, Scott, D, (1991), Review of Municipal Solid Waste Incineration in the UK, LR 776 (PA), Warren Spring Laboratory, Stevenage.

CSGB, (2001), The Cremation Society of Great Britain, Personal Communication.

DEFRA, (2001), The National Food Survey, 1990-00, Personal Communication.

DEFRA, (2001a), Digest of Environmental Statistics, No.24.

DOE, (1993), UK Sewage Sludge Survey, CES Limited, HMSO

EFW, (2001), Energy for Waste, Current and Projected Energy For Waste Capacity in UK as at Dec 2000.  www.efw.org.uk

EMEP/CORINAIR, (1996), Atmospheric Emission Inventory Guidebook, 1st Edition, ed. G McInnes

EMEP/CORINAIR, (1999), Atmospheric Emission Inventory Guidebook, 2nd Edition, ed. S Richardson

Environment Agency, (1997). The Chemical Release Inventory, 1996, Personal Communication

Environment Agency, (2001). The Pollution Inventory, 2000, Personal Communication

Hobson, J, Palfrey, R, Sivil, D, Palfrey, E, Day, M, (1996) Control Measures to Limit Methane Emissions from Sewage and Sludge Treatment and Disposal, WRc , Report No DOE 4118

IPCC, (1997), IPCC Revised 1996 Guidelines for National Greenhouse Gas Inventories, Volume 3, Greenhouse Gas Inventory Reference Manual, IPCC WGI Technical Support Unit, Hadley Centre, Meteorological Office, Bracknell, UK.

IPCC, (2000), Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories, ed. Penman, J, Kruger, D, Galbally, I, et al, IPCC National Greenhouse Gas Inventories Programme, Technical Support Programme Technical Support Unit, Institute of Global Environmental Strategies, Hayama, Kanegawa, Japan.

LQM (2003) Land Quality Management report to DEFRA, January 2003, “Methane emissions from landfill sites in the UK”, LQM report 443/1.

Passant, NR, (1993), Emissions of Volatile Organic Compounds from Stationary Sources in the UK: A review of Emission Factors by Species and Process (September 1993 Update), Warren Spring Laboratory, LR 990.

Patel, N, (2000), Personal Communication, NETCEN, Culham.

Royal Commission on Environmental Pollution, (1993), 17 the Report, Incineration of Waste, HMSO, London.

Appendix 8 Uncertainties

CONTENTS

1	ESTIMATION OF UNCERTAINTY BY SIMULATION	2

1.1	Carbon Dioxide Emission Uncertainties	3

1.2	Methane Emission Uncertainties	5

1.3	Nitrous Oxide Emission Uncertainties	7

1.4	Halocarbons and SF6	8

1.5	GWP Weighted Emissions	8

2	ESTIMATION OF UNCERTAINTY BY TIER 1 APPROACH	11

3	KEY SOURCE ANALYSIS	14

4	REFERENCES	16

# Estimation of Uncertainty by Simulation

Quantitative estimates of the uncertainties in the emissions were calculated using Monte Carlo simulation.  This corresponds to the IPCC Tier 2 approach discussed in the Good Practice Guidance (IPCC, 2000).  This work is described in detail by Eggleston et al (1998) though the estimates reported here have been revised to reflect changes in the 2001 Inventory.  This section gives a brief summary of the methodology, assumptions and results of the simulation.

The general computational procedure was:

- An uncertainty distribution was allocated to each emission factor and activity rate.  The distributions used were mostly normal, log-normal or uniform.  The parameters of the distributions were set by analysing the available data on emission factors and activity data or by expert judgement.

- A calculation was set up to estimate the emission of each gas, carbon dioxide sink, and the global warming potential for the years 1990 and 2001.  Using the software tool @RISK™, each uncertainty distribution was sampled 20,000 times and the emission calculations performed to formulate a converged output distribution.

- It was assumed that the distribution of errors in the parameter values was normal.  The quoted range of possible error of uncertainty is taken as 2s, where s is the standard deviation.  If the expected value of a parameter is E and the standard deviation is s, then the uncertainty is quoted as 2s/E expressed as a percentage.  For a normal distribution the probability of the parameter being less than E-2s is 0.025 and the probability of the emission being less than E+2s is 0.975.

- For methane and nitrous oxide, it was assumed that there was no correlation between emission factors for the same fuels applied to different sources.  For CO2 emission factors for natural gas, gas oil,  burning oil, fuel oil, petrol, DERV, LPG, orimulsion,  MSW and aviation fuel were correlated with those for the same fuel used in different sources.

- The uncertainties used for the fuel activity data were estimated from the statistical difference between supply and demand for each fuel.  This means that the quoted uncertainty in Table 1 refers to the total fuel consumption rather than the consumption by a particular sector, e.g. residential coal.  Hence, to avoid underestimating uncertainties, it was necessary to correlate the uncertainties used for the same fuel in different sectors.  A further refinement was to correlate the data used for the same fuels to calculate emissions of carbon dioxide, methane and nitrous oxide.  These modifications to the methodology were introduced in the 2000 inventory.

- The uncertainty in the trend between 1990 and 2001 was also estimated.  This will be influenced by the degree of correlation of activity data and emission factors between 1990 and 2001.  Generally it was assumed that activity data from different years were not correlated, but certain emission factors were.  These correlations are discussed in subsequent sections.

- To simplify the calculations the uncertainties for total halocarbon and SF6 emissions were taken from Eggleston et al (1998).

## Carbon Dioxide Emission Uncertainties

It was necessary to estimate the uncertainties in the activity data and the emission factors for the main sources and then combine them.

The uncertainties in the fuel activity data for major fuels were estimated from the statistical differences data in DTI (1996).  These are effectively the residuals when a mass balance is performed on the production, imports, exports and consumption of fuels.  For solid and liquid fuels both positive and negative results are obtained indicating that these are uncertainties rather than losses.  For gaseous fuels these figures include losses and tended to be negative.  For natural gas, a correction was made to take account of leakage from the gas transmission system but for other gases this was not possible.  The uncertainties in activity data for minor fuels (colliery methane, orimulsion, SSF, petroleum coke) and non-fuels (limestone, dolomite and clinker) were estimated based on judgement comparing their relative uncertainty with that of the known fuels.  The high uncertainty in the aviation fuel consumption reflects the uncertainty in the split between domestic and international aviation fuel consumption.

The uncertainties in the emission factors were based largely on expert judgement.  It was possible to compare the coal emission factors used in the inventory with measurements (Fynes, 1994).  Also, Transco (1998) data allowed an estimate of the uncertainty in the carbon content of natural gas.  The time series data of the gross calorific value of fuels used in the UK (DTI, 1996) would also give some indication of the relative variability in the carbon contents.  Thus the uncertainties in the fuel emission factors were based on judgements on whether they were likely to be similar or less than those of coal or natural gas.

In the case of non-fuel sources, the uncertainty depended on the purity of limestone or the lime content of clinker so the uncertainties estimated were speculative.

The uncertainties in certain sources were estimated directly.  Offshore flaring uncertainties were estimated by comparing the SCOPEC (2001) flaring time series data with the flaring volumes reported by DTI (2001).  The uncertainty in the activity data was found to be around 16%.  This uncertainty will be an over estimate since it was assumed that the flaring volume data reported by DTI should be in a fixed proportion to the mass data reported by SCOPEC.  The uncertainty in the carbon emission factor was estimated by the variation in the time series to be around 6%.  Again this will be an over estimate since it was assumed that the carbon emission factor is constant.  Uncertainties for fuel gas combustion were estimated in a similar way.  Uncertainties in the land use change sources were recalculated (Milne, 1999) for the revised source categories in the IPCC 1996 Guidelines using data from Eggleston et al (1998).  A new carbon source Fletton bricks has been added, and the uncertainty based on expert assessment of the data used to make the estimate.  There has been a very slight revision to the uncertainty used for cement production, based on the estimates reported in IPCC (2000).  Clinical waste incineration was assumed to have the same uncertainty as MSW incineration.

The overall uncertainty was estimated as around 2.7% in 1990 and 2.2% in 2001.  In spite of the revisions to the methodology, there appears to have been little change in uncertainty from that given for the 2000 inventory.

The uncertainty in the trend between 1990 and 2001 was also estimated.  In running this simulation it was necessary to make assumptions about the degree of correlation between sources in 1990 and 2001.  If source emission factors are correlated this will have the effect of reducing the trend uncertainty.  The assumptions were:

- Activity data are uncorrelated

- Emission factors of similar fuels are correlated (i.e. gas oil with gas oil, coke with coke etc)

- Land Use Change and forestry emissions are correlated (i.e. 5A with 5A etc)

- Offshore emissions are not correlated since they are based on separate studies using emission factors appropriate for the time.

- Process emissions from blast furnaces, coke ovens and ammonia plant were not correlated.

The trend was found to range between -4.2% and -6.9%

Table 1	Estimated Uncertainties in Carbon Dioxide Inventory1 (only major sources are listed)

| Source | Activity Uncertainty % | Emission Factor Uncertainty % | Uncertainty in Emission % |
| --- | --- | --- | --- |
| Coal (including derived gases) | 1.2 | 6 | ‡ |
| Coke | 5.6 | 3 | ‡ |
| Petroleum Coke | 5 | 3 | ‡ |
| SSF | 3 | 3 | ‡ |
| Burning Oil | 6 | 2 | ‡ |
| Fuel Oil | 4 | 2 | ‡ |
| Gas Oil/Diesel Oil | 1.4 | 2 | ‡ |
| Motor Spirit (Gasoline) | 0.8 | 2 | ‡ |
| Orimulsion | 1 | 2 | ‡ |
| Aviation Fuel (Domestic) | 50 | 2 | ‡ |
| Lubricants | 25 | 5 | ‡ |
| Natural Gas | 2.4 | 1 | ‡ |
| Colliery Methane | 5 | 5 | ‡ |
| LPG | 24 | 3 | ‡ |
| OPG | 1.1 | 3 | ‡ |
| Scrap Tyres | 15 | 10 | ‡ |
| Waste Oils | 15 | 5 | ‡ |
| Ammonia Production | - | - | 5 |
| Cement | 1 | 2.2 | ‡ |
| Lime/Limestone/Dolomite | 1 | 5 | ‡ |
| Soda Ash Use | 15 | 2 | ‡ |
| Fletton Bricks | 20 | 70 | ‡ |
| Flaring | 16 | 6 | ‡ |
| Other Offshore | - | - | 28 |
| Natural Gas (offshore) | 2.4 | 6 | ‡ |
| Iron & Steel Processes | 1 | 20 | ‡ |
| Aluminium Production | 1 | 5 | ‡ |
| Waste (MSW and Clinical) | 7 | 20 | 21 |
| 5A Forest Biomass Change2 | - | - | 30 |
| 5D Soils2 | - | - | 60 |
| 5E Other2 | - | - | 50 |

1	Expressed as 2s/E

2	Uniform distribution used

‡	Input parameters were uncertainties of activity data and emission factors.

## Methane Emission Uncertainties

In the methane inventory, combustion sources are a minor source of emissions.  The uncertainty in methane combustion emission factors will outweigh the activity errors so an uncertainty of 50% was assumed for combustion sources as a whole.  The errors in the major sources are listed in Table 2.  These are mainly derived from the source documents for the estimates or from the Watt Committee Report (Williams, 1993).  The uncertainty in offshore emissions was revised for the 2000 inventory using improved estimates of the activity data .  The methane factors were assumed to have an uncertainty of 20% since the flaring factors are based on test measurements.

Table 2	 Estimated Uncertainties in the Methane Inventory (only major sources are listed)

| Source | Reference | Activity % | Emission Factor % | Source Uncertainty % |
| --- | --- | --- | --- | --- |
| Fuel Combustion | ‡ |  | 50 |  |
| Field Burning | ‡ | - | - | 50 |
| Landfill | Brown et al 1999 | - | - | ~481 |
| Livestock: enteric | Williams, 1993 | 0.1 | 20 |  |
| Livestock: wastes | Williams, 1993 | 0.1 | 30 |  |
| Coal Mining | Bennett et al, 1995 | 1.2 | 13 |  |
| Offshore | ‡ | 16 | 20 |  |
| Gas Leakage | Williams, 1993 | - | - | 17-752 |
| Chemical Industry | ‡ | 20 | 20 |  |
| Fletton Bricks | ‡ | 20 | 100 |  |
| Sewage Sludge | Hobson et al, 1996 | - | - | 50 |

1	Skewed distribution

2	Various uncertainties for different types of main and service

‡	See text

- Input parameters were uncertainties of activity data and emission factors.

The sources quoted in Table 2 are assumed to have normal distributions of uncertainties with the exception of landfills.  Brown et al (1999) estimated the uncertainty distribution for landfill emissions using Monte Carlo analysis and found it to be skewed.  For normal distributions there is always a probability of negative values of the emission factors arising.  For narrow distributions this probability is negligible; however with wide distributions the probability may be  significant.  In the original work (Eggleston et al, 1998) this problem was avoided by using truncated distributions.  However, it was found that this refinement made very little difference to the final estimates, so in these estimates normal distributions were used rather than truncated normal.

The total emission of methane in 2001 was estimated as 2196 Gg.  The Monte Carlo analysis suggested that 95% of trials were between 1950 Gg and 2617 Gg.  The total uncertainty was around 14%.  The emission of methane in 1990 was estimated as 3661 Gg.  The Monte Carlo analysis suggested that 95% of trials were between 3128 Gg and 4606Gg.  The uncertainty was around 19%.

The uncertainty in the trend between 1990 and 2001 was also estimated.  In running this simulation it was necessary to make assumptions about the degree of correlation between sources in 1990 and 2001.  If source emission factors are correlated this will have the effect of reducing the emissions.  The assumptions were:

- Activity data are uncorrelated between years, but activity data for major  fuels were correlated in the same year in a similar manner to that described above for carbon.

- Emission factors  for animals are correlated across years for a given species.

- Landfill emissions were partly correlated across years in the simulation.  It is likely that the emission factors used in the model will be correlated, and also the historical estimates of waste arisings will be correlated since they are estimated by extrapolation from the year of the study.  However, the reduction in emissions is due to flaring and utilisation systems installed since 1990 and this is unlikely to be correlated.  As a crude estimate it was assumed that the degree of correlation should reflect the reduction.  Emissions have reduced by 43% hence the degree of correlation was 57%.

- Offshore emissions are not correlated across years since they are based on separate studies using emission factors that reflected the processes in use at the time.

- Gas leakage emissions were fully correlated across years.

- Emissions from deep mines were not correlated across years as they were based on different studies, and a different selection of mines.  Open cast and coal storage and transport were correlated since they are based on default emission factors.

95% of the values found for the trend lay between  -31% and -49%.

## Nitrous Oxide Emission Uncertainties

The analysis of the uncertainties in the nitrous oxide emissions is particularly difficult because emissions arise from a diverse collection of sources and few data are available to form an assessment of the uncertainties in each source.  Emission factor data for the combustion sources are scarce and for some fuels are not available.  The parameter uncertainties are shown in Table 3.  The uncertainty assumed for agricultural soils uses a lognormal distribution since the range of possible values is so high.  Here it is assumed that the 97.5 percentile is greater by a factor of 100 than the 2.5 percentile based on advice from the Land Management Improvement Division of DEFRA.  The uncertainty distribution of the calculated emission was heavily skewed with a mean emission of 137Gg in 2001 with95% of the values found to lie between 36 Gg and  485 Gg N2O.

The uncertainty in the trend between 1990 and 2001 was also estimated.  In running this simulation it was necessary to make assumptions about the degree of correlation between sources in 1990 and 2001.  If sources are correlated this will have the effect of reducing the emissions.  The assumptions were:

- Activity data are uncorrelated between years, but similar fuels are correlated in the same year.

- Emissions from agricultural soils were correlated

- The emission factor used for sewage treatment was assumed to be correlated, though the protein consumption data used as activity data were assumed not to be correlated.

- Nitric acid production emission factors were assumed not to be correlated, for reasons explained in the 2000 report.

- Adipic acid emissions were assumed not to be correlated because of the large reduction in emissions due to the installation of abatement plant in 1998.

95% of the values for the trend were found to lie between  -17% and -73%.

Table 3: Estimated Uncertainties in the Nitrous Oxide Emissions1 (only major sources are listed)

|  | Emission Factor Uncertainty % | Activity Rate Uncertainty % |
| --- | --- | --- |
| Agricultural Soils | Log-normal2 | 0 |
| Wastewater Treatment | Log-normal2 | 10 |
| Adipic Acid | 15 | 0.5 |
| Nitric Acid | 230 | 10 |
| Coal | 195 | 1.2 |
| Anthracite | 387 | 1.2 |
| Coke | 118 | 5.6 |
| Patent Fuel | 118 | 3 |
| Burning Oil | 140 | 6 |
| Gas Oil | 140 | 1.4 |
| Fuel Oil | 140 | 4 |
| Gasoline | 170 | 0.8 |
| Auto Diesel | 170 | 1.4 |
| Orimulsion | 140 | 1 |
| LPG | 110 | 24 |
| OPG | 110 | 1.1 |
| Aviation Fuel (domestic) | 170 | 50 |
| Natural Gas | 110 | 2.4 |
| Colliery Methane | 110 | 5 |
| Lubricants | 140 | 25 |
| Biogas | 110 | 5 |
| Offshore Sources | 110 | 1 |
| Field burning | 230 | 25 |
| Poultry Litter | 230 | 7 |
| Scrap Tyres, Waste Oils | 140 | 15 |
| Sewage, MSW and Clinical Incineration | 230 | 7 |
| Wood | 230 | 30 |
| Straw | 230 | 50 |

1	Expressed as 2s/E

2	With 97.5 percentile 100 times the 2.5 percentile

## Halocarbons and SF6

The uncertainties in the emissions of HFCs, PFCs and SF6 were taken from Eggleston et al (1998).  The uncertainties were estimated as 25% for HFCs, 19% for PFCs and 13% for SF6 and were assumed uncorrelated between 1990 and 2001.  Trend uncertainties are reported in Table 4.

## GWP Weighted emissions

The uncertainty in the combined GWP weighted emission of all the greenhouse gases in 2001 was estimated as 13% and in 1990 as 15%.  The trend in the total GWP is -12%, with 95% of the values found to lie within the range -10% and -15%.  The uncertainty estimates for all gases are summarised in Table 4.  The source which makes the major contribution to the overall uncertainty is 4D Agricultural Soils.  This source shows little change over the years, but other sources have fallen since 1990.  Hence the increase in uncertainty since 1990.

Table 4: Summary of Tier 2 Uncertainty Estimates

| A | B | C | D | E | F | G | H | I | J |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IPCC | Gas | 1990 | 2001 | Uncertainty in 2001 emissions |  |  | Uncertainty | % change in | Range of likely % change |  |  |
| Source |  | Emissions | Emissions | as % of emissions |  |  | Introduced | emissions | between 2001 and 1990 |  |  |
| Category |  |  |  | in category |  | in national total | between 2001 |  |  |
|  |  |  |  |  |  | in 2001a | and base year |  |  |
|  |  |  |  | 2.5 percentile | 97.5 percentile |  |  | 2.5 percentile | 97.5 percentile |
|  |  | Gg CO2 | Gg CO2 | Gg CO2 | Gg CO2 | % | % | % | % |
|  |  | equivalent | equivalent | equivalent | equivalent |  |  |  |  |
| TOTAL | CO2 | 593807 | 560849 | 549098 | 572683 | 2.2% | -5.6% | -6.9% | -4.2% |
|  | CH4 | 76885 | 46106 | 40946 | 54956 | 14% | -40% | -49% | -31% |
|  | N2O | 67864 | 42356 | 11290 | 150281 | 204% | -38% | -73% | -17% |
|  | HFC | 11374 | 8678 | 6529 | 10806 | 25% | -24% | -47% | 9% |
|  | PFC | 2281 | 706 | 572.8 | 833.2 | 19% | -69% | -76% | -59% |
|  | SF6 | 724 | 1762 | 1535.8 | 1986.3 | 13% | 143% | 103% | 192% |
|  | All | 752934 | 660456 | 624151 | 769720 | 13% | -12% | -15% | -10% |

a	Calculated as 2s/E where s is the standard deviation and E is the mean, calculated in the simulation.

b	Not quoted because distribution is highly skewed.

# Estimation of Uncertainties using a Tier 1 Approach

The IPCC Good Practice Guidance (IPCC, 2000) defines Tier 1 and Tier 2 approaches to estimating uncertainties in national greenhouse gas inventories.  The Monte Carlo approach described above corresponds to Tier 2 whilst Tier 1 provides for a simplified calculation method based on the error propagation equations.  The results of the Tier 1 approach are shown in Table 5.  In the Tier 1 approach the emission sources are aggregated up to a level broadly similar to the IPCC Summary Table 7A.  Uncertainties are then estimated for these categories.  The uncertainties used in the Tier 2 approach are not exactly the same as those used in the Monte Carlo Simulation since the Tier 1 source categorisation is far less detailed.  However, the values used were chosen to agree approximately with those used in the Monte Carlo Simulation.  The Tier 2 approach is only able to model normal distributions. This presented a problem in how to estimate a normal distribution approximation of the lognormal distribution used for agricultural soils and wastewater treatment.  The approach adopted was to use a normal distribution with the same mean as the lognormal distribution.  The standard deviation was then estimated as (97.5 percentile –mean)/2.

The Tier 1 approach suggests an uncertainty of 17% in the combined GWP total emission in 2001.  The analysis also estimates an uncertainty of 2% in the trend between 1990 and 2000.

# Key Source Analysis

The Good Practice Guidance (2000) requires that a key source analysis be made to identify the key source categories in the inventory.  The results of the analysis are reported in Table 6.  The analysis is based on the Tier 2 level analysis and trend analysis.  The key source analysis was performed on the data shown in Table 5 using the same categorisation and the same estimates of uncertainty. The table indicates whether a key source arises from the level assessment or the trend assessment.  The factors which make a source a key source are:

- A high contribution to the total

- A high contribution to the trend

- High uncertainty.

For example: auto oil combustion is a key source of carbon dioxide because it is large; landfill methane is key because it is large, has a high uncertainty and shows a significant trend.  Sources and sinks from land use change and forestry have been included in the key source analysis since they are significant and have a high uncertainty.

# References

Bennet, S, Kershaw, S, Burrell, R, (1995), Control Measures for Methane Emissions from Coal Production, ETSU N/01/00006/REP

Brown, KA, Smith, A, Burnley, SJ, Campbell, DJV, King, K, Milton, MJT, (1999) Methane Emissions from UK Landfills, AEA Technology, AEAT-5217, Culham

DTI, (1996), Digest of United Kingdom Energy Statistics 1996, London, The Stationary Office.

DTI, (2002), Digest of United Kingdom Energy Statistics 2001, London, The Stationary Office.

Eggleston, HS, Salway, AG, Charles, D, Jones, BMR, Milne, R, (1998), Treatment of Uncertainties for National Estimates of Greenhouse Gas Emissions, National Environmental Technology Centre, AEA Technology, Report AEAT - 2688.

Fynes, G, Sage, PW,(1994), Emissions of Greenhouse Gases from Coal Fired Plant,  British Coal, Coal Research Establishment, CERCHAR, DMT, Delft University of Technology, TPS Termiska Processer AB, CONTRACT NO JOUF 0047-C(SMA)

Hobson, J, Palfrey, R, Sivil, D, Palfrey, E, Day, M, (1996) Control Measures to Limit Methane Emissions from Sewage and Sludge Treatment and Disposal, WRc , Report No DOE 4118

IPCC, (2000),  Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories, ed. Penman, J, Kruger, D, Galbally, I, et al, IPCC National Greenhouse Gas Inventories Programme, Technical Support Programme Technical Support Unit, Institute for Global Environmental Strategies, Hayama, Kanagawa, Japan.

SCOPEC, (2001), Environmental Database for Emissions & Discharges from Offshore Installations, SCOPEC, Private Communication (and previous editions)

Transco, (1998), Personal Communication, I Jones, System Control.

Milne (1999), Centre for Ecology and Hydrology, (Edinburgh), Personal Communnication.

UKOOA (1993), Atmospheric Emissions from UK Oil and Gas Exploration and Production Facilities in the UK Continental Shelf Area. Prepared for United Kingdom Offshore Operators Association Limited, March 1993, Ref HN08-007.REP, Brown & Root Environmental, Leatherhead, Surrey

Williams, A, (1993), Methane Emissions, Watt Committee Report Number 28, The Watt Committee on Energy, London.

Appendix 9 Quality Assurance and Quality Control

CONTENTS

1	INTRODUCTION	2

2	THE UK NATIONAL INVENTORY SYSTEM	2

3	QA/QC SYSTEM USED FOR THE UK GREENHOUSE
GAS INVENTORY	5

3.1	Description of the Current System	5

3.2	Special QA/QC Activities Undertaken in 2000/2001	7

4	FUTURE DEVELOPMENT OF THE QA/QC SYSTEM	10

4.1	Improvements to the System	10

5	REFERENCES	13

# Introduction

This Appendix summarizes the UK national inventory system and the current QA/QC system used in the compilation of the National Atmospheric Emissions Inventory (NAEI) and the UK Greenhouse Gas Inventory (GHGI).  The current system complies with the Tier 1 procedures outlined in the Good Practice Guidance (IPCC, 2000).  Plans are underway to develop the system and extend the range of activities so that the system complies with Tier 2.

# The UK National Inventory System

The Department for Environment, Food and Rural Affairs (DEFRA) is responsible for submitting the UK's greenhouse gas inventory (GHGI) to the UNFCCC and the EU Monitoring Mechanism.  AEA Technology compiles the GHGI on behalf of DEFRA and the devolved administrations.

Figure 1 summarises the UK national inventory system.

The GHGI is compiled using the same database as the National Atmospheric Emissions Inventory (NAEI).  The NAEI is the air emissions inventory for the UK and includes emission estimates for a wide range of important pollutants.  These include: greenhouse gases, regional pollutants leading to acid deposition and photochemical pollution, persistent organic pollutants and other toxic pollutants such as heavy metals.  The NAEI is available at http://www.naei.org.uk/

Energy statistics required for compilation of the NAEI and the GHGI are obtained from the Digest of UK Energy Statistics (DUKES), compiled and published by the Department of Trade and Industry (DTI).  DUKES is available at www.dti.gov.uk/epa/dukes.htm

Information on industrial processes is provided either directly to AEA Technology by the individual sectors responsible for emissions or from the Environment Agency's Pollution Inventory (PI).  The PI is the only statutory part of the national system.  Large companies are required to report emissions of key pollutants to the Environment Agency (a non-departmental public body). The PI is also used to help confirm some information provided voluntarily by companies directly to AEA Technology.

The Institute for Grassland and Environmental Research (IGER) compiles the inventory for agricultural emissions using agricultural statistics from DEFRA.

The Centre for Ecology and Hydrology (CEH) compiles estimates of emissions and removals from LUCF using DEFRA land-use data and information on forestry from the Forestry Commission (a non-departmental public body).

DEFRA also funds research contracts to provide emissions estimates for certain sources.  For example, AEA Technology provides landfill methane emissions estimates.  Emissions of HFCs, PFCs and SF6 were compiled by an independent consultancy (EnvirosMarch) in consultation with industry.

The GHGI is compiled according to IPCC Good Practice.  Each year the inventory is updated to include the latest data available.  Improvements to the methodology are made and are backdated to ensure a consistent time series.  Methodological changes are made to take account of new data sources, or new methodologies developed by IPCC or CORINAIR, or new research, or specific research programmes sponsored by DEFRA.

# QA/QC System used for the UK Greenhouse Gas Inventory

## Description of the Current System

The National Atmospheric Emissions Inventory and the UK Greenhouse Gas Inventory are compiled and maintained by the National Environmental Technology Centre of AEA Technology plc.  Whilst significant parts of the inventory (i.e. agriculture, land use change and forestry and halocarbons and sulphur hexafluoride emissions) are compiled by other agencies and contractors, NETCEN is the Inventory Agency responsible for co-ordinating QA/QC activities.

The system has developed over the years.  A new on-line database system was adopted for the 1997 Inventory in 1998 and since then developments have proceeded to build QA/QC procedures into the on-line system.  The database consists essentially of a table of activity data and a table of emission factors for the NAEI base source categories.  These are then multiplied together to produce emissions according to the IPCC and CORINAIR formats to be generated.

The Inventory has been subject to ISO 9000 since 1994 and is liable to audit by Lloyds and the AEAT internal QA auditors.  The NAEI has been audited favourably by Lloyds on two occasions in the last four years.  The emphasis of these audits was on authorisation of personnel to work on inventories, document control, data tracking and spreadsheet checking.  As part of the Inventory management structure there is a nominated officer responsible for the QA/QC system –the QA/QC Co-ordinator.

The system incorporates the following activities which are carried out each year as the inventory is compiled:

1.	Documentation

- Data received by NETCEN are logged, numbered and should be traceable back to their source from anywhere in the system.

- The inventory is held as an Access database of activity data and emission factors.  Within the database these data fields are referenced to the data sources, or the spreadsheet used to calculate the data.  For fuel consumption data, the DUKES (DTI, 2002) table numbers are identified.

- There is an on-line system of manuals, which defines timetables, procedures for updating the database, document control, checking procedures and procedures for updating the methodology manual.

- There is an on-line methodology manual which is updated as the inventory data are entered.  This contains details of the methodology used, emission factor and activity data sources, discussion of the rationale for choice of methodology and emission factors.

- An annual report outlining the methodology of the inventory, data sources and changes made is produced.

2.	Database

- The classification of source categories is controlled by a formatting table in the database which is used to generate emissions tabulated in the IPCC format.  Other simple queries can be used to dump all emissions data contained in the database.  These can be compared against the tabulated output to check that all sources are output and that the totals are correct.  A consistency check between IPCC output and CORINAIR formatted output is made.  Data in the CRF reporting tool are checked against the database totals.

- All fields in the database are labelled automatically with an NAEI source/fuel category, the CORINAIR SNAP code and the units used.  A comment field linked to each data entry provides further description and the data source or spreadsheet used to calculate it.

3.	Checking

- ISO 6000 requires that spreadsheet calculations are checked and the checks applied are described.  Also the data sources used for calculations must be referenced on the spreadsheet.

- Data entry into the database is checked.  It is not always possible for all data entries to be checked by a second person.  However, a major proportion of the activity data are entered and checked by third persons.

- A system has been developed to check the fuel entries in the database.  Queries will abstract and total the fuel conssumptions for each fuel.  These totals are then checked against the totals reported in DTI (2002).

- The final checks on the inventory involve a consistency check against the previous inventory for the same year.  A designated auditor identifies sources where there have been significant changes or new sources.  Inventory staff are required to explain these changes in the inventory to satisfy the auditor.

- A further final check is made on the inventory comparing the emissions of the latest year with those of the previous year (within the same version).  A designated checker identifies sources where there have been significant changes.  Inventory staff are required to explain these changes in the inventory to satisfy the checker.  This is some what more detailed than the recalculation explanations required by Table 8 in the CRF, because it is based on the more disaggregated source sectors used in the NAEI database.

4.	Recalculation

- When revisions are made to the methodologies of the estimates, emissions for all previous years are recalculated as a matter of course.

5.	Uncertainties

- Estimates are made of the uncertainties in the estimates according to Tier 1 and Tier 2 procedures.

- A ranking exercise is performed according to Tier 2 procedures to identify key source categories.

6.	Archiving

- At the end of each reporting cycle, all the database files, spreadsheets, on-line manual, electronic source data, paper source data, output files are in effect frozen and archived.  An annual report outlining the methodology if the inventory and data sources is produced.  Electronic information is stored on hard disks which are regularly backed up.  Paper information is being archived in a Lektreiver ® system and a database of all items in the archive is being compiled

The system outlined above complies with the Tier 1 procedures outlined in Table 8.1 of the Good Practice Guidance (IPCC, 2000).  However, following the release of the Good Practice Guidance a review of the QA/QC procedures has been carried out (Salway, 2001) and a QA/QC plan has been developed to extend the current procedures to comply with Tier 2.  This involves extending some of the existing procedures and adopting new ones.  The QA/QC plan is discussed further in Section 3.

## special qa/qc activities undertaken in 2002-2003

This section describes certain specific activities relating to QA/QC that were carried out in the last year.  These will in future arise from the QA/QC plan but are not necessarily carried out on an annual basis.  The activities were:

Verification of the inventory

In order to provide some verification of the UK Greenhouse Gas Inventory (GHGI), the Global Atmosphere Division of DEFRA has established continuous high-frequency observations of the Kyoto gases under the supervision of Professor Peter Simmonds of the University of Bristol at the Mace Head Atmospheric Research Station on the Atlantic Ocean coastline of Ireland.  The Met Office employs the Lagrangian dispersion model NAME (Numerical Atmospheric dispersion Modelling Environment) driven by 3D synoptic meteorology from the Unified Model (Cullen, 1993) to sort the observations made at Mace Head into those that represent Northern Hemisphere baseline air masses and those that represent regionally-polluted air masses arriving from Europe.  The Lagrangian dispersion model is then used to estimate the magnitude and spatial distribution of the European emissions that best support the observations. The technique has been applied to 3-yearly rolling subsets of the data.  The results from this study are given in Appendix 10.

Revision of halocarbon estimates

The current UK emissions and projections of HFCs, PFCs and SF6 used in this study are based on work carried out on behalf of DEFRA by March Consulting Group (now Enviros March), published in 1999.  Selective updates are incorporated each year.  Emissions data for 1990 to 1997 were based on statistical information provided by fluid manufacturers and distributors.  Projections were based on anticipated market developments and technological changes over the period to 2020.  DEFRA commissioned an updated inventory emission estimates for 1990 to 2002 and projections to 2025 of fluorinated gases (AEAT, 2002), taking into account latest information on emission factors and the adoption in some sectors of abatement measures.  In addition, this study included emission estimates and projections for NF3, though these are not included in the national totals submitted to the UNFCCC.  The results from this study will be incorporated into the 2002 inventory, to be submitted in April 2004.

Expert peer review of fuel combustion sources of carbon dioxide

An expert peer review of the key fuel combustion sources of carbon dioxide has been completed (Simmons, 2002).  The review considered the report UK Greenhouse Gas Inventory, 1990 to 1999  (ISBN 0-7058-1797-0), March 2001).  The summary of the peer review report is presented below.

"The present review examines the methods and estimates for emissions of CO2 from fuel combustion and the conformity of the methods with international recommendations for the accuracy, transparency and comparability of greenhouse gas inventories.  The inventories show the emissions by source and removals by sinks of the greenhouse gases.

The fuel combustion part of the UK Greenhouse Gas Inventory (GHGI) is soundly constructed and prepared in a manner consistent with the IPCC Guidelines and Good Practice Guidance.

The National Inventory Report (NIR) provides a comprehensive presentation and explanation of the UK Greenhouse Gas Inventory (GHGI).  The GHGI is prepared by the National Environmental Technology Centre (NETCEN) and reflects the national and international development of emissions methodology over many years and the maintenance of the National Atmospheric Emissions Inventory (NAEI).  The NAEI has provided many of the estimation methods used in the GHGI and is the source of the nomenclature and classification of activities in the GHGI.

The CO2 fuel combustion inventory is thorough in scope and covers all emission sources identified in the IPCC Guidelines.  Some recommendations listed in this review identify possible emissions arising from fuel uses for which data are not readily available.  Other recommendations concern the clarification of the origins and derivation of carbon emission factors and further explanation of methodology."

The recommendations for additions or changes to methodology or emission factors, which may lead to changes in the levels of estimated CO2 emissions, will be considered when the next inventory is developed.

Revision of estimates of methane emissions from landfill

The AEA Technology model of methane generation from landfill sites used for the past four years (Brown et al, 1999) has now been updated and revised for DEFRA by the consultants Land Quality Management (LQM 2003).

The main changes made by LQM can be summarised as follows:

- New waste inventory data for 1995 (MSW) and 1999 (commercial and industrial waste) was used to revise the estimates of waste arisings.

- DOC and dissimilable DOC parameters were revised, resulting in much higher values than those used previously, especially for paper.

- Instead of using the IPCC default of 10% for oxidation rate, LQM have developed a model to estimate the residual methane oxidation factor based on field and laboratory observations of soil oxidation capacity and expert judgement.  Oxidation factors are estimated to be much higher than the IPCC default – up to 90%.

- The estimates of capture of landfill gas for flaring or utilisation have been increased, based on a survey of installed flare capacity.

- Methane generation rate constants were revised slightly.

The overall effect of these changes was to predict more methane generation, but also more capture and much more oxidation. On balance, the overall level of emissions was predicted to be similar for current and future years, but much higher (by a factor of two) for past years.

In depth review of the UK National Inventory by the UNFCCC, September 2002

The Conference of the Parties (COP), by its decisions 6/CP.5 and 34/CP.7, requested the secretariat to conduct individual reviews of greenhouse gas (GHG) inventories submitted by Parties included in Annex I to the Convention (Annex I Parties), according to the UNFCCC guidelines for the technical review of GHG inventories from Annex I Parties, hereinafter referred to as the review guidelines.  The principle objectives of the review of the GHG inventories is to ensure that the COP has adequate information on GHG inventories and GHG emission trends, and to examine the information submitted by Annex I Parties in accordance with the UNFCCC reporting guidelines for consistency with those guidelines.

The desk review of the United Kingdom of Great Britain and Northern Ireland (the United Kingdom) took place from 9 to 27 September 2002.  The desk review was carried out by a team of nominated experts from the roster of experts, working in their own countries.  The assignments of the experts were as follows:  generalists – Mr. Moussa Cisse (Senegal) and Mr. Riccardo De Lauretis (Italy), energy – Ms. Anke Herold (Germany) and Mr. Eilev Gjerald (Norway), industrial processes – Mr. Philip Acquah (Ghana) and Ms. Marian Van Pelt (USA), agriculture – Mr. Mingxing Wang (China) and Ms. Penny Reyenga (Australia), land-use change and forestry – Mr. Wojciech Galinski (Poland) and Mr. Mikhail Gytarsky (Russian Federation), waste – Mr. Eduardo Calvo (Peru) and Mr. Carlos Lopez (Cuba).  Ms. Anke Herold and Mr. Moussa Cisse were the lead reviewers for this desk review.  The review was coordinated by Ms. Sevdalina Todorova-Brankova (UNFCCC secretariat).

Key findings of the review were:

- The national inventory submitted by the United Kingdom is in conformity with the UNFCCC reporting guidelines.  The methodology for estimating the GHG emissions is consistent with the Revised 1996 IPCC Guidelines for National Greenhouse Gas Inventories, hereinafter referred to as the IPCC Guidelines, and the IPCC good practice guidance.  The information submitted in the NIR is consistent with the CRF.

- Information reported in the NIR and complementary information reported in the CRF submitted by the United Kingdom provides a high level of transparency.  The use of notation keys and documentation boxes of the CRF contributes to the transparency of the inventory reporting.  The inventory submission explains in detail the methodologies used to estimate emissions, to reference activity data and emission factors and in reporting gaps in the inventory.  We plan to incorporate the recommendations of the review team from the 2004 inventory submission onwards.

Review of the estimation of emissions from aviation in the UK greenhouse gas inventory

An approach has been proposed to upgrade the current methodology for calculating the GHG emissions from aircraft to be compatible with the CORINAIR ‘Detailed’ (Tier3) methodology, for both domestic and international contributions.  This work will be developed the 2003/2004 reporting cycle.

Compliance check of the current UK National Inventory Report structure to the proposed requirements of the UNFCCC

We have produced a “compliance table” which tries to match the current contents of the NIR with the proposed requirements of the UNFCCC as quoted in FCCC/SBSTA/2002/L.5/Add.1.  The Proposed contents list was derived from: "Guidelines for the preparation of national communications by Parties included in Annex I to the Convention, Part I: UNFCCC reporting guidelines on annual inventories."  The NIR, to be published in April 2004, will conform to the new reporting structure.

# Future Development of the QA/QC System

The review discussed in the previous section identified areas where the current QA/QC system could be improved and the need for additional activities to comply with Tier 2.  These developments are now included in the QA/QC plan, elements of which are described in this Section.  The QA/QC plan will be included in the on-line manual system.

## improvements to the system

### Compliance of National Statistical Agencies

Many of the data received by NETCEN come from other government departments, agencies, research establishments or consultants.  Some of these organisations (e.g. DTI, MAFF and BGS) would qualify as the National Statistical Agencies referred to in the Guidance.  Other organisations (e.g. CEH, Enviros March) compile significant parts of the Inventory.  Currently the QA/QC procedures in use at NETCEN do not extend to QA/QC procedures used at these data suppliers.  The Good Practice Guidance defines as good practice that the Inventory Agency should confirm that National Statistical Agencies have implemented adequate QC procedures along the lines indicated in Table 8.1 of the Good Practice Guidelines.  Hence, we have contacted these organisations and inviting them to show how their systems comply with IPCC Good Practice Guidance.  The QA procedures used by DEFRA for agricultural data collection and archiving are outlined in Appendix 5 Agriculture and those used by the Centre for Ecology and Hydrology for preparing the land use change and forestry estimates are discussed in Appendix 6.

### Documentation and Review

The Inventory is documented both by the on-line Manual and the Annual Report.  The on-line Manual tends to include more detail which is inappropriate to the annual report.  The Good Practice Guidance highlights the need for review of methodologies during inventory compilation.  Hence the on-line manual is being developed along these lines:

- Completeness.  The manual will be extended to include material on potential emission sources, which are not estimated in the Inventory.  This will include reasons for not including these sources and some assessment of their magnitude.

- Source Review Documentation.  The manual tends to describe the methodologies in use, past revisions, emission factors and activity data sources.  It is intended that the scope should be expanded to include more detail on the choice of methodology and the choice of emission factors.  This will include evidence that internal review of emission sources takes place.

### External Peer Review

Tier 2 of the Good Practice Guidance requires that key sources should be subjected to external peer review.  During 2002, the UK implemented a programme of peer reviews by experts outside of the organisation responsible for the estimates.  The first peer review on CO2 emissions from coal, oil and gas combustion has been completed.  The programme for the external peer review is shown in Table 1.

Table 1.  QA/QC Activities Schedule

|  | 2001/2002 | 2002/2003 | 2003/2004 |
| --- | --- | --- | --- |
| Special Activities | Energy Data Harmonisation Review | Update Halocarbon Inventory Update Landfill (CH4) (including QA/QC procedures) |  |
| On-going Activities | On-Going Tier 1 Activities | On Going Tier 1 Activities | On-Going Tier 1 Activities |
|  | Carbon Factor Review(update) | Carbon Factor Review(update) | Carbon Factor Review(update) |
|  | Acid Plant Documentation (update) | Acid Plant Documentation (update) | Acid Plant Documentation (update) |
|  | Document Completeness |  |  |
|  | Document Source Reviews | Document Source Reviews | Document Source Reviews |
|  | External Agencies QA/QC | External Agencies QA/QC follow up |  |
| External Peer Review | UNFCCC In-Depth Review National Report Combustion Sources- coal, oil and gas (CO2) | National Report Agricultural Soils (N2O) Manure Management (N2O) Enteric Fermentation (CH4) | National Report Nitric Acid (N2O) LUCF 5A, 5D & 5E (CO2) Wastewater (N2O) Landfill (CH4) |

- Refers to period between inventory submission i.e. April to April

# References

AEAT (2002) Haydock, H et al. Emissions and Projections of HFCs, PFCs and SF6 for the UK and Constituent Countries (reference EPG 1/1/128).  Study by AEA Technology Environment on behalf of Global Atmosphere Division (GAD) of the Department for Environment, Food and Rural Affairs (DEFRA).

Brown, KA, Smith, A, Burnley, SJ, Campbell, DJV, King, K, Milton, MJT, (1999) Methane Emissions from UK Landfills, AEA Technology, AEAT-5217, Culham

IPCC, (2000), Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories, ed. Penman, J, Kruger, D, Galbally, I, et al, IPCC National Greenhouse Gas Inventories Programme, Technical Support Programme Technical Support Unit, Institute for Global Environmental Strategies, Hayama, Kanagawa, Japan.

LQM (2003) Land Quality Management report to DEFRA, January 2003, “Methane emissions from landfill sites in the UK”, LQM report 443/1.

Sage, PW, (2001), Methane from Abandoned Coal Mines in the UK, AEA Technology, Report AEAT/ENV/R/0500, Harwell

Salway, AG, Hill, N, Sully, J, (2001), Development and Implementation of a Methodology to Verify Projected UK Emissions of Industrial Gases, AEAT/EMV/R/0495, NETCEN, AEA Technology, Culham.

Salway, AG, (2001), Accordance of UK Greenhouse Gas Inventory with IPCC Good Practice Guidance, Note prepared for DETR, NETCEN, AEA Technology, Culham.

Simmons, T, (2002), Peer Review of the UK Greenhouse Gas Inventory CO2 from Fuel Combustion 1990 - 1999. Avonlog Ltd, 49 Headley Chase, Brentwood, Essex, CM14 5DH.

Simmons, T, (2002), Energy Data Harmonisation, A project undertaken for the Department of Trade and Industry, Avonlog Ltd.

Smith, A, (2001), Comments on conformity of UK Calculation of Methane Emissions from Landfill with IPCC Good Practice Guidance, NETCEN, AEA Technology, Culham

Appendix 10 Verification of the UK Estimates of the Kyoto Gases

CONTENTS

1	INTRODUCTION	2

2	METHANE	2

3	NITROUS OXIDE	3

4	HYDROFLUOROCARBONS	3

4.1	HFC-134A	3

4.2	HFC-152A	4

4.	REFERENCES	4

# Introduction

In order to provide some verification of the UK Greenhouse Gas Inventory (GHGI), the Global Atmosphere Division of DEFRA has established continuous high-frequency observations of the Kyoto gases under the supervision of Professor Peter Simmonds of the University of Bristol at the Mace Head Atmospheric Research Station on the Atlantic Ocean coastline of Ireland (Simmonds et al. 1996).  The Met Office employs the Lagrangian dispersion model NAME (Numerical Atmospheric dispersion Modelling Environment) (Ryall et al. 1998) driven by 3D synoptic meteorology from the Unified Model (Cullen, 1993) to sort the observations made at Mace Head into those that represent Northern Hemisphere baseline air masses and those that represent regionally-polluted air masses arriving from Europe.  The Lagrangian dispersion model is then used to estimate the magnitude and spatial distribution of the European emissions that best support the observations (Manning et al. 2003). The technique has been applied to 3-yearly rolling subsets of the data.

The best-fit technique, simulated annealing, is used to fit the model emissions to the observations. It assumes that the emissions from each grid box are uniform in both time and space over the duration of the data. This implies that the release is independent of meteorological factors such as temperature and diurnal or annual cycles, and that in its industrial production and use there is no definite cycle or intermittency.

# Methane

In Table 1, the comparison is made between the emission estimates made for the UK with the Lagrangian dispersion model and the GHGI emission estimates for the period 1995-2001.

Methane has a natural (biogenic) component, it is estimated that 22% of the annual global emission (Nilsson et al. 2001) is released from wetlands.  Usually natural emissions are strongly dependent on a range of meteorological factors such as temperature and diurnal / annual and growth / decay cycles. Such non-uniform emissions will add to the uncertainties (estimated to be 150 Gg yr-1) associated with the NAME-derived emission estimates.

Table 1	Verification of the UK emission inventory estimates for methane in Gg yr-1 for 1995-2001 (three-year averages)

| Gas | 1995-1997 | 1996-1998 | 1997-1999 | 1998-2000 | 1999-2001 |
| --- | --- | --- | --- | --- | --- |
| Methane (CH4) – NAME | 2760 | 2430 | 2560 | 2490 | 2940 |
| Methane (CH4) - GHGI | 2962 | 2837 | 2675 | 2504 | 2341 |

# Nitrous Oxide

The main activities in Europe resulting in the release of nitrous oxide are; agricultural soils (~60%), chemical industry (~20%) and combustion (~15%) (UNFCCC 1998 figures).  The amount emitted from soils has significant uncertainty and has a diurnal and seasonal release cycle. It is driven by the availability of nitrogen, temperature and the soil moisture content.

Late in 1998, DuPont introduced abatement technology at its adipic acid plant in Wilton, north east England.  It has been estimated that this has cut its emissions of N2O by 90% from 46 thousand tonne yr-1 (DEFRA, 2000).

Table 2 shows the NAME and the GHGI emission estimates for the UK for nitrous oxide for the period 1995-2001 as 3-year rolling averages.  The NAME estimates show a decline of 19 Gg yr-1 from a high in 1996-1998 to a low in 1999-2001. The GHGI estimates show a decline of 49 Gg yr-1 during the same period.

The nature of the nitrous oxide emissions challenges the NAME technique assumption of uniformity of release both in time and space. The uncertainty of the estimates is calculated to be 7 Gg yr-1.

Table 2	Verification of the UK emission inventory estimates for nitrous oxide in Gg yr-1 for 1995-2001 (three-year averages)

| Gas | 1995-1997 | 1996-1998 | 1997-1999 | 1998-2000 | 1999-2001 |
| --- | --- | --- | --- | --- | --- |
| Nitrous oxide (N2O) – NAME | 167 | 179 | 168 | 164 | 160 |
| Nitrous oxide (N2O) - GHGI | 190 | 191 | 176 | 159 | 142 |

# Hydrofluorocarbons

## HFC-134a

Table 3 shows the NAME and the GHGI emission estimates for the UK for CFC-134a for the period 1995-2001 as 3-year rolling averages. The GHGI shows a much steeper increase in emission compared to the NAME estimates, with the former showing a 120% and the latter a 41% rise in emissions during the period.

Table 3	Verification of the UK emission inventory estimates for HFC-134a in Gg yr-1 for 1995-2001 (three-year averages)

| Gas | 1995-1997 | 1996-1998 | 1997-1999 | 1998-2000 | 1999-2001 |
| --- | --- | --- | --- | --- | --- |
| HFC-134a - NAME | 0.92 | 0.94 | 1.15 | 1.06 | 1.30 |
| HFC-134a - GHGI | 0.84 | 1.11 | 1.26 | 1.58 | 1.86 |

## HFC-152a

Table 4 shows the NAME and the GHGI emission estimates for the UK for CFC-152a for the period 1995-2001 as 3-year rolling averages. Both estimates show similar trends of increase but the NAME estimates are 0.02-0.04 Gg yr-1 lower than the estimates in the GHGI.

Table 4	Verification of the UK emission inventory estimates for HFC-152a in Gg yr-1 for 1995-2001 (three-year averages)

| Gas | 1995-1997 | 1996-1998 | 1997-1999 | 1998-2000 | 1999-2001 |
| --- | --- | --- | --- | --- | --- |
| HFC-152a – NAME | 0.02 | 0.03 | 0.05 | 0.05 | 0.08 |
| HFC-152a – GHGI | 0.06 | 0.07 | 0.07 | 0.09 | 0.10 |

# References

Cullen, M.J.P., The Unified Forecast/Climate Model, Meteorological Magazine UK, 1449, 81-94, 1993

Department of the Environment, Food and Rural Affairs (DEFRA), Climate Change: The UK Programme, Nov, 2000, www.defra.gov.uk/environment/climatechange/07.htm

Manning, A.J., D.B. Ryall, R.G. Derwent, P. G. Simmonds, S. O’Doherty, Estimating european emissions of ozone-depleting and greenhouse gases using observations and a modelling back-attribution technique, in press J. of  Geophysical Research (atmospheres) 2003.

Nilsson, M., C. Mikkela, I. Sundh, G. Granberg, B. H. Svensson, and B. Ranneby, Methane emission from Swedish mires: Natural and regional budgets and dependence on mire vegetation, J. of Geophys. Res., 106, 20847-20860, 2001.

Ryall, D.B. and R. H. Maryon, Validation of the UK Met Office's NAME model against the ETEX dataset, Atmospheric Environment, 32, 4265-4276, 1998.

Simmonds, P.G., Derwent, R.G., McCulloch, A., O’Doherty, S., Gaudry, A., Long-term trends in concentrations of halocarbons and radiatively-active trace gases in Atlantic and European air masses monitored at Mace Head, Ireland from 1987-1994. Atmospheric Environment, 30, 4041-4063, 1996.

United Nations Framework Convention on Climate Change (UNFCCC) www.unfccc.int

The Environment Agency for England and Wales

For the UNFCCC review guidelines and decision 6/CP.5 see document FCCC/CP/1999/7, pages 109 to 114 and 121 to 122, respectively.

For the objectives of the review of GHG inventories see document FCCC/CP/1999/7, page 109, paragraph 2.

The guidelines for the preparation of national communications by Parties included in Annex I to the Convention, Part I: UNFCCC reporting guidelines on annual inventories (FCCC/CP/1999/7), are referred to in this report as the UNFCCC reporting guidelines.

The IPCC Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories.