Costs and benefits for the UK of Complying with the EC National Emissions Ceilings and Ozone Directive and the UNECE multi-pollutant, multi-effect protocol

Report produced for Department of the Environment, Transport and the Regions

April 2000

Costs and benefits for the UK of Complying with the EC National Emissions Ceilings and Ozone Directive and the UNECE multi-pollutant, multi-effect protocol

Report produced for Department of the Environment, Transport and the Regions

April 2000

| Title | Costs and Benefits for the UK of Complying with the EC National Emissions Ceilings and Ozone Directive and the UNECE Multi-Pollutant, Multi-Effect Protocol |
| --- | --- |
| Customer | Department of the Environment, Transport and the Regions |
| Customer reference |  |
| Confidentiality, copyright and reproduction | This document has been prepared by AEA Technology plc in connection with a contract to supply goods and/or services. The views expressed in this report are those of AEA Technology and do not, necessarily, represent the views of the funding organisation |
| File reference | ED46016 |
| Report number | AEAT/R/ENV/0035 |
| Report status | Final |
|  | Dr Deborah Buckley-Golder AEA Technology plc Culham Abingdon Oxfordshire OX14 3ED Telephone +44 (0)1235 463571 Facsimile +44 (0)1235 463005 |
|  | AEA Technology is the trading name of AEA Technology plc AEA Technology is certificated to BS EN ISO9001:(1994) |

|  | Name | Signature | Date |
| --- | --- | --- | --- |
| Lead author | M Holland |  |  |
| Reviewed by | G P Marsh |  |  |
| Approved by | D H Buckley-Golder |  |  |

Authors

| AEA Technology | Deborah Buckley-Golder Justin Goodwin Neil Passant Tony Bush Dan Forster Mike Holland Katie King Charles Walker | Project manager Emissions inventory Analysis of the costs of abatement for SO2, NOx and VOCs Article 5 Assessment Quantification of the benefits of pollution abatement |
| --- | --- | --- |
| Imperial College | Helen ApSimon David Cowell Rachel Warren | ASAM, RAINS modelling Description of exposure to secondary particulates Analysis of abatement costs for ammonia |
| Institute of Terrestrial Ecology | Keith Bull Jane Hall Jackie Ullyett | Critical loads analysis |
| Meteorological Office | Dick Derwent | HARM, ELMO modelling |
| University of Edinburgh | Sarah Metcalfe | HARM, ELMO modelling |
| University of Lancaster | Duncan Whyatt | HARM, ELMO modelling |

Executive Summary

This report describes work undertaken to provide the UK Government with an understanding of the costs and benefits for the UK of complying with a number of international measures for addressing trans-boundary air pollution:

- the European Commission’s proposed National Emissions Ceilings Directive

- the closely related Ozone Directive and

- the UNECE’s proposed multi-pollutant, multi-effect (also called the 2nd NOx) Protocol.

Information presented here will allow DETR to prepare Regulatory Impact Assessments (RIAs) for the UK, in respect of the proposed measures.  It will also form the basis for the preparation of Impact Assessments by the Scottish Executive, the National Assembly for Wales and the Northern Ireland Office for their respective countries.

Previous analysis of UK emissions and control costs has been undertaken by IIASA, using the RAINS model, with an analysis of the corresponding benefits by AEA Technology, using the ALPHA model.  The accuracy of the analysis carried out for the EC and UNECE using these two models was necessarily limited, because of the broad geographical area subject to analysis.  The same is true of the ASAM model, developed by Imperial College, which has previously been used in uncertainty analysis of the RAINS model results.  These earlier analyses have been conducted with a resolution of 150x150 km.  The work reported here has aimed to provide a more detailed analysis, using the latest available datasets and specifically UK-based models, which have been used previously in work undertaken on behalf of the EC, UNECE and UK Government and have, thus, been widely reviewed.  These models offer much higher resolution of the implications of emissions abatement as they operate on a scale of 10x10 km.

The work has drawn upon the expertise of AEA Technology (NETCEN), Imperial College, the Universities of Edinburgh and Lancaster, ITE Monkswood and the Meteorological Office.  The models employed include the UK National Atmospheric Emissions Inventory (NAEI 1997), HARM atmospheric dispersion/deposition model, ELMO ozone model, MARACCAS ammonia model and the ALPHA-UK benefits evaluation model.

The scenarios of the future selected for analysis were as follows:

- Reference scenario (UKREF), providing baseline emission projections for 2010, the reference year for the study.  Under UKREF, UK emissions are based on the 1997 NAEI escalated for future years using provisional projections of industrial growth and fuel use provided by DTI.

- J1, which was selected by the UNECE as the guiding scenario for Protocol negotiations;

- H1, selected by the EC as the basis for the draft National Emissions Ceilings Directive;

- a further scenario which was added towards the end of the study, following the publication of the emissions ceilings negotiated at the 31st Session of the UNECE Working Group on Strategies, on 2 September 1999.  This was termed WGS31c.  Full analysis could not be carried out on this scenario in the time available, although an indication of the associated costs and benefits is presented.

The regional allocation of emissions of nitrogen oxides (NOx), sulphur dioxide (SO2), volatile organic compounds (VOCs) and ammonia (NH3) under each of these scenarios is shown in Tables E-1 to E-4.

The estimated costs of reducing UK emissions in line with the selected scenarios are shown in Table E-5, and the regional allocation of costs is shown in Table E-6.  It should be noted that the estimated costs of achieving the J1 and H1 scenarios are considerably lower than earlier estimates made by IIASA, which were £447M and £899M, respectively.

Table E-1.  Regional allocation of emissions of NOx (kt)

| Scenario | England | Scotland | Wales | N. Ireland |
| --- | --- | --- | --- | --- |
| UKREF | 992 | 102 | 59 | 32 |
| WGS31c/ J1/H1 | 987 | 101 | 58 | 32 |

Table E-2.  Regional allocation of emissions of SO2  (kt)

| Scenario | England | Scotland | Wales | N. Ireland |
| --- | --- | --- | --- | --- |
| UKREF | 602 | 57 | 100 | 25 |
| WGS31c | 468 | 43 | 94 | 20 |
| J1 | 362 | 32 | 89 | 16 |
| H1 | 360 | 32 | 89 | 16 |

Table E-3.  Regional allocation of emissions of VOC (kt)

| Scenario | England | Scotland | Wales | N. Ireland | Offshore |
| --- | --- | --- | --- | --- | --- |
| UKREF | 883 | 207 | 55 | 26 | 129 |
| WGS31c | 852 | 184 | 47 | 26 | 92 |
| J1 | 773 | 174 | 43 | 23 | 87 |
| H1 | 689 | 155 | 39 | 21 | 61 |

Table E-4.  Regional allocation of emissions of NH3  (kt)

| Scenario | England | Scotland | Wales | N. Ireland |
| --- | --- | --- | --- | --- |
| UKREF/WGS31c | 202 | 42 | 24 | 30 |
| J1/H1 | 182 | 36 | 20 | 26 |

Table E-5 Costs of emission reductions for the UK (£million/year) arising from movement from the UKREF scenario to the scenarios shown.

| Scenario | SO2 | NOx | VOC | NH3* | Total |
| --- | --- | --- | --- | --- | --- |
| WGS31c | 34 | 9 | 18 | 0 | 61 |
| J1 | 61 | 9 | 64 | 11 – 24 | 145 - 158 |
| H1 | 61 | 9 | 435 | 11 - 24 | 516 - 529 |

*Abatement costs for ammonia emissions are shown as a range because of uncertainty about the acceptability of urea substitution, the higher end of the range assuming that it is not adopted.

- Table E-6 Regionalised costs (£M/year).

| Scenario | England | N. Ireland | Scotland | Wales | Offshore |
| --- | --- | --- | --- | --- | --- |
| WGS31c | 41 | 1 | 8 | 3 | 7 |
| J1 | 108 – 116 | 4 – 6 | 16 – 18 | 7 – 9 | 8 |
| H1 | 339 - 347 | 11 – 13 | 68 - 71 | 19 - 21 | 77 |

These costs must be set against the wide range of benefits attributable to abatement of the four pollutants.  This analysis deals not just with direct effects of the four, but also the effects of associated pollutants chemically derived from these emissions in the atmosphere, particularly secondary aerosols such as ammonium sulphate, and photo-oxidants such as ozone.  The effects considered include the following:

- effects on health (premature death, hospital admissions, exacerbation of asthma, etc.);

- impacts on crops and forests (yield change);

- impacts on building materials (change in frequency of repair);

- change in ecological sustainability (measured here against critical loads for acidification and eutrophication).

Benefits estimation is often criticised for being subject to a high degree of uncertainty.  In this study the analysis is carried out in a structured and transparent manner, that retains information on these uncertainties.  Uncertainties, of course, also affect the other half of the analysis – the estimation of abatement costs.  A recent review suggests that these costs tend to be overestimated in modelling exercises.  This results from the manner in which cost models tend to focus on end-of-pipe measures and, hence, do not account for some potentially cheaper options, and also an inability to account for technological change in the future.  It is, clearly, very important to understand the limitations associated with any inputs to an exercise such as this.

The following table provides some of the main results of the benefits assessment.  Benefits are split roughly evenly between NOx, SO2 and ozone.  NH3 plays a lesser role, reflecting its limited abatement under the scenarios considered.

- Table E-7.  Key benefits of each scenario, with results showing the total annual benefit to the UK of abatement across the UNECE region (except for H1, where only EU countries participate).  All monetised values in 1990£/year.

|  | 1990 | UKREF | J1 | H1 | WGS31c |
| --- | --- | --- | --- | --- | --- |
| Ecosystem area (%) subject to critical load exceedence for: |  |  |  |  |  |
| Acidification | 43% | 11% | 5.4% | 6.2% | 9.2% |
| Eutrophication | 11.2% | 2.0% | 1.0% | 1.1% | 1.3% |
| Change from UKREF in damage to: |  |  |  |  |  |
| Crops (£million) |  |  | £10M | £17M |  |
| Materials (£million) |  |  | £15M | £15M | £5M |
| Premature mortality (cases) |  |  | 540 | 530 | 160 |
| Respiratory hospital admissions |  |  | 350 | 380 | 110 |

Table E-7 takes the analysis to the same point as the assessment carried out for the National Air Quality Strategy (NAQS).  However, it is possible to go further, quantifying more effects on health, damage to forests and so on, and valuing them, although this extension of the analysis is subject to a higher level of uncertainty.  This was carried out and, to account for uncertainty, ranges were ascribed to the key variables.  Overall this led to a wide range in the benefits.  Costs tended to lie between the upper and lower bounds – a clearly inconclusive result.  However assessment of benefit-cost ratios (Table E-8) suggests that costs for J1 tend to be towards the lower end of the range, in other words, there appears a reasonable likelihood that the real benefit would exceed the estimated costs.  This is not the case for H1, however, where the bias is in the other direction, with costs towards the upper end of the range.

Table E-8.  Benefit-cost ratios, with all monetised effects taken into account (excludes damage to cultural heritage and natural ecosystems).  Positive numbers denote benefits in excess of costs by the factors quoted.  Negative numbers denote costs in excess of quantified benefits by the factors quoted.  Columns identify the source of emission and the receptor for benefits of abatement (in both cases this is either UK or full UNECE).  Costs applied are the costs to the UK in all cases.

|  | UK to UK | UK to UNECE | UNECE (incl. UK) to UK |
| --- | --- | --- | --- |
| J1 |  |  |  |
| Lower bound | -2.38 | -1.35 | -1.11 |
| Upper bound | 4.01 | 8.04 | 8.20 |
| H1 |  |  |  |
| Lower bound | -7.14 | -3.85 | -4.17 |
| Upper bound | 1.26 | 2.77 | 2.15 |
| WGS31c |  |  |  |
| Lower bound | Not quantified | Not quantified | Not quantified |
| Upper bound | Not quantified | Not quantified | Not quantified |

Contents

PART 1: Background to the study

1	Introduction	3

1.1	Approach	3

1.2	Methodology	4

1.3	Structure of the report	6

2	BACKGROUND	7

2.1	Scenarios for emissions abatement	7

2.2	Earlier analyses	7

2.3	Issues in benefits analysis and CosT-Benefit analysis (cba)	9

Part 2: ANALYSIS OF EMISSIONS AND COSTS

3	Methods - Costs	13

3.1	Development of cost curves	13

3.1.1	Input data	13

3.1.2	Development of the Cost Curves	14

3.1.3	Development of regional cost curves	15

3.1.4	Uncertainties in the cost curves	15

3.2	Mapping of emission totals	17

4	Results - Costs	19

4.1	Overall costs	19

4.2	Regional costs	23

PART 3: QUANTIFICATION OF BENEFITS AND THE COMPARISON OF COSTS AND BENEFITS

5	Methods - Benefits	27

5.1	Overview	27

5.2	Effects of the Pollutants	28

5.3	Sources of data for the analysis	28

5.4	Characterising uncertainties	28

6	Results - Benefits	31

6.1	Pollutant exposures	31

6.2	Critical loads exceedence	32

6.2.1	England and Wales	36

6.2.2	Northern Ireland	36

6.2.3	Scotland	36

6.2.4	United Kingdom	36

6.3	Results for impacts on crops, materials, mortality and respiratory hospital admissions	42

6.4	Application of additional exposure-response functions identified by COMEAP	44

6.5	Application of EAHEAP Valuations	46

6.6	Valuation of additional effects for which COMEAP provided functions	50

6.7	Chronic effects on mortality	50

6.8	Application of other Functions and valuations used in the studies for UNECE and the EC	51

6.9	Comparison of Costs and Benefits	51

pART 4: CONCLUSIONS

7	Conclusions	57

part 5:	aPPENDICES

Appendix 1: Abbreviations and terminology

Appendix 2: Scenario data from RAINS

Appendix 3: Monitoring for the proposed Ozone Directive

Appendix 4: Analysis using HARM and ELMO

Appendix 5: Analysis using RAINS and ASAM

Appendix 6: Detailed results for acidification and eutrophication

Appendix 7: Inputs to the benefits analysis

Appendix 8: Full results from the cost-benefit analysis

Appendix 9: References

PART I

Background to the Study

# Introduction

The European Commission announced in its Acidification Strategy (COM(97)88 final) its intention to put forward a proposal for a Directive establishing national emission ceilings for sulphur dioxide (SO2), nitrogen oxides (NOx), volatile organic compounds (VOCs) and ammonia (NH3) for 2010.  In preparation for the Directive, the Commission has developed a series of emission control scenarios designed to attain given environmental targets in the most cost-effective way for the EU as a whole.  These targets are set with respect to three effects: acidification and both the health and vegetation related impacts of ground-level ozone.  Eutrophication is also taken into account.

The Air Quality Framework Directive (96/62/EC) establishes a system under which the European Commission will agree air quality limit values for specified pollutants in a series of daughter Directives.  The proposed Ozone Directive is the third daughter Directive to be put forward by the European Commission.  This sets target values, to be achieved by 2010, and long-term objectives for the protection of human health and vegetation.  Given the obvious linkages that are present, the development of the Ozone Directive and the National Emission Ceilings Directive (NECD) have proceeded hand in hand.

In parallel to the above initiatives the past two years have also seen much activity in development of a new Protocol under the United Nations Economic Commission for Europe (UNECE) Convention on Long-Range Transboundary Air Pollution.  This Protocol, formally called the Protocol to Abate Acidification, Eutrophication and Ground Level Ozone in Europe has a similar scope to the EU Directives in terms of pollutants, but applies to all European countries, not just the EU Member States.  The Protocol was signed in Gothenburg in December 1999.  [Note that it is also referred to as the Multi-pollutant, Multi-effect Protocol, the 2nd NOx Protocol or the Gothenburg Protocol.]

This report describes work undertaken to provide the UK Government with an understanding of the costs and benefits of the proposed legislation, to the UK as a whole and to England, Northern Ireland, Scotland and Wales individually.  Information has been furnished which will allow the DETR to prepare Regulatory Impact Assessments (RIAs) for the UK, in respect of the proposed measures, and form the basis for the preparation of Impact Assessments by the Scottish Executive, National Assembly for Wales and Department of the Environment for Northern Ireland.

## Approach

Previous analyses of UK emissions and control costs have been undertaken by the International Institute of Applied Systems Analysis (IIASA), using the RAINS model, with an analysis of the corresponding benefits by AEA Technology, using the ALPHA model.  The accuracy of the analysis carried out for the EC and UNECE using these two models was necessarily limited, because of the broad geographical area subject to analysis.  The same is true of the ASAM model, developed by Imperial College, which has previously been used in uncertainty analysis of the RAINS model results.  These earlier analyses have been conducted at a resolution of 150x150 km.  The work reported here has aimed to provide a more detailed analysis for the UK, using the latest available datasets and UK-based models.  These have been used previously in work undertaken on behalf of the EC, UNECE and UK Government and have thus been widely reviewed.  These models offer much higher resolution of the implications of emissions abatement as they operate on a scale of 10x10 km.

The work has drawn upon the expertise of AEA Technology, Imperial College, the Universities of Edinburgh and Lancaster, ITE Monkswood and the Meteorological Office.  The models employed include the UK National Atmospheric Emissions Inventory (NAEI 1997), HARM atmospheric dispersion/deposition model, ELMO ozone model, MARACCAS ammonia model, and ALPHA-UK benefits evaluation model.  These are described in greater detail in the sections that follow.

The costs and benefits of the Directives and Protocol should not be regarded in isolation from other legislation that is currently agreed but yet to take full effect, and for which the overall consequences are, as yet, uncertain.  These include:

- the IPPC Directive

- the Kyoto Protocol

- the National Air Quality Strategy (NAQS)

The NAQS raises particular issues with respect to the negotiating position taken by the UK.  It was noted in the review of the NAQS (DETR, 1999) that the targets for PM10 and ozone will be extremely difficult to meet without significant action in other European countries.

## Methodology

The methodology for estimating the costs and benefits to the UK of meeting specified emissions abatement targets follows a number of clearly defined steps:

- compilation and mapping of ‘baseline’ emissions projections disaggregated by sector for the target year (2010);

- identification of cost-effective abatement techniques and estimation of costs associated with the application of these to reduce projected emissions to the prescribed target levels;

- mapping of emissions disaggregated by sector for alternative emission scenarios for the target year;

- modelling of the dispersion and chemical transformation of the mapped emissions to generate maps of associated air quality and deposition for each alternative scenario;

- comparison of the deposition maps against critical loads maps to determine the degree of ecological protection attributable to the prescribed level of emissions abatement;

- assessment of the exposure of sensitive receptors (people, buildings, etc.) for each scenario, by combining maps of pollution levels with maps of (e.g.) population distribution;

- quantifying impacts by combining data on exposures with dose-response functions to provide estimates of changes in hospital admissions, premature deaths, rate of material degradation, crop yields, etc.

- comparison of results between the baseline and alternative scenarios to assess the incremental change in impacts arising from emissions abatement;

- estimation of the monetary value of the benefits attributed to emissions abatement.

Estimates of UK emissions of SO2, NOx, VOCs and NH3 were based on the National Atmospheric Emissions Inventory (NAEI) and, for the purposes of mapping, were disaggregated between Scotland, Wales, Northern Ireland and England using existing information held within the NAEI database on the location of point and area sources.  Many of the important sources, such as power stations, large combustion plant, ports, airports, and process plant, were defined as point sources and, therefore, could easily be assigned to one of the four countries.  Emissions from road transport on major roads were defined as line sources and could also be assigned to individual countries.  Remaining sources, such as domestic and small industrial combustion plant, agriculture, off-road vehicles, and military vehicles were treated as area sources, and were disaggregated using surrogate statistics, such as regional population, fuel use or employment.

Emissions data were presented in map form using a 10 km x 10 km grid, to act as input for the modelling of their dispersion and chemical transformation within the atmosphere.  This was undertaken using the HARM model, to determine sulphur and nitrogen deposition and concentrations, and the ELMO model, to determine ground level ozone concentrations.  These models take account of the complex chemical reactions taking place in the atmosphere, the changing chemical composition of the air as it moves away from one source area and into another, and the impact of a range of meteorological parameters.

The output from these models is also in map format, providing data on the change in pollution concentration and deposition across the UK as a result of emissions abatement.  The maps of sulphur and nitrogen deposition were used with geographically disaggregated data on critical loads to produce maps of critical loads exceedance for the UK.  By comparing exceedance maps for baseline and abated emissions, estimates are made of the reduction in critical loads exceedance and, hence, the threat of damage to sensitive ecosystems attributable to the prescribed level of emissions abatement.

The benefits analysis for impacts on health, crops and materials was based around the ALPHA and ALPHA-UK models, used previously in analysis of the Gothenburg Protocol and the NECD, and also the National Air Quality Strategy.  Deposition/concentration maps were overlaid onto data showing the distribution of ‘stock at risk’ (people, buildings, crops etc.) and information on the sensitivity of the stock (death rates, age structure of the population, type of ecosystem etc.).  This provided information on exposure, to which could be applied exposure-response functions, to derive estimates of the impact of the changes in pollution concentration and deposition, in biological and physical terms.

Impacts were monetised to the extent possible.  For some effects, such as changes in crop yield, this can simply be done using data from relevant markets.  In other cases, particularly for effects on human health, alternative methods were necessary, based, for example, on the contingent valuation technique.  The current UK government position is against monetisation of health effects arising from exposure to air pollution.  However, an extended sensitivity analysis is supplied to indicate how the balance of costs and benefits changes under different sets of assumptions, not just on monetisation, but also at other stages of the analysis.

## Structure of the report

Part 1 of this report (sections 1+2) provides information on the background to the study, the scenarios considered, earlier analyses, etc.  In Part 2 (sections 3+4) the development of the UKREF Scenario and the estimation of the costs to the UK as a whole, and England, Northern Ireland, Scotland and Wales individually, of meeting the emissions ceilings required by the scenarios are described.  Additional, non-abatement costs attributable to the Ozone Directive are also identified.

Part 3 (sections 5+6) describes the quantification of the benefits attributable to achievement of the emissions ceilings.  This includes a description of the modelling of emissions, air quality and deposition, as well as the analysis of ecological protection and reductions in human exposure and effect.  The cost-benefit analysis is presented in a structured manner that seeks to retain understanding of the uncertainties of the analysis.

Part 4 (section 7) presents the conclusions of the study.

Additional detail of the analysis is given in a series of appendices:

Appendix 1: Abbreviations and terminology

Appendix 2: Scenario data from RAINS

Appendix 3: Monitoring for the proposed Ozone Directive

Appendix 4: Analysis using HARM and ELMO

Appendix 5: Analysis using RAINS and ASAM

Appendix 6: Detailed results for acidification and eutrophication

Appendix 7: Inputs to the benefits analysis

Appendix 8: Full results from the cost-benefit analysis

Appendix 9: References

# BACKGROUND

## Scenarios for emissions abatement

The scenarios used in this analysis were as follows:

UKREF: UK emissions based on the 1997 NAEI escalated for future years using provisional projections of industrial growth and fuel use provided by DTI, following revision of data from Energy Paper 65 (EP65). The Central (growth) / High (fuel price) scenario was used here. Emissions for other countries were based on IIASA’s Reference scenario (Amann et al, 1999a).

J1:  Emissions for all countries in the UNECE domain based on IIASA’s J1 scenario (Amann et al, 1999a).  This scenario is also referred to as G5/2rev in some reports.

H1:  Emissions for all countries in the EU set to IIASA’s H1 scenario (Amann et al, 1999b).  Emissions for other countries are set to IIASA Reference for 2010.

A fourth scenario was added towards the end of the study, following the publication of the emissions ceilings negotiated at the 31st Session of the UNECE Working Group on Strategies, on 2 September 1999, and subsequent developments prior to the signing of the Protocol.  This was termed WGS31c (after the 31st meeting of the Working Group on Strategies).  Full analysis could not be carried out on this scenario in the time available, although an indication is presented of the associated costs and benefits.

Emissions for the UK under these scenarios are summarised in Table 1.  Appendix 2 shows emissions for all UNECE countries in mainland Europe, Scandinavia, Russia, the Baltic States and the British Isles under the IIASA Reference, H1 and J1 scenarios from the RAINS analysis carried out by Amann et al (1999a, b).

Table 1. Annual emissions of each pollutant for the UK in each scenario from 2010 on (kt)

| Scenario | SO2 | NOx | VOC | NH3 |
| --- | --- | --- | --- | --- |
| UKREF | 784 | 1187 | 1300 | 319 |
| WGS31c | 625 | 1181 | 1200 | 297 |
| J1 | 499 | 1181 | 1101 | 264 |
| H1 | 497 | 1181 | 964 | 264 |

## Earlier analyses

IIASA have produced a series of reports for the European Commission and the UNECE relating to the Directives and the Protocol (the most relevant to the position in force at the time of writing being Amann et al, 1999a, b).  These studies took a series of environmental quality objectives for acidification, eutrophication and ozone, and assessed the reduction in national emissions required to meet them in a cost-effective manner.  Table 2 provides estimated emissions and costs under each of the RAINS scenarios for the UK.  The UKREF scenario used in this study is shown for comparison, demonstrating a significant reduction to the IIASA Reference case for SO2 by 20%.

Table 2.  Estimates of emissions and costs for the UK in the RAINS scenarios (from Amann et al).

| Scenario | SO2 | NH3 | NOx | VOC | Total cost (£M/year) |
| --- | --- | --- | --- | --- | --- |
| Emissions (kt) |  |  |  |  |  |
| IIASA Reference | 980 | 297 | 1186 | 1351 |  |
| UKREF | 784 | 319 | 1187 | 1336 |  |
| WGS31c | 625 | 297 | 1181 | 1200 |  |
| J1 | 499 | 264 | 1181 | 1101 |  |
| H1 | 497 | 264 | 1181 | 964 |  |
| Costs* (£M/year) |  |  |  |  |  |
| WGS31c |  |  |  |  | 209 |
| J1 | 197 | 15 | 235(NOx and VOC) |  | 447 |
| H1 | 200 | 15 | 684(NOx and VOC) |  | 899 |

* Incremental to IIASA Reference, and not UKREF

£ = 1.5 Euro

This assessment of the benefits of abatement of SO2, NOx, NH3 and VOCs, and their comparison with the costs of abatement, follows a series of cost-benefit studies carried out by the same team.  This earlier work was undertaken in the context of both the UNECE Protocol and the proposed EU Directives on national emission ceilings and ozone (AEA Technology, 1998a,b, 1999a,b,c).  Funding was provided by several sources; DETR, DGXI of the European Commission and the Dutch Ministry of Housing, Spatial Planning and Environment (MVROM).  The studies were widely reviewed by:

- UK Government Departments and associated experts;

- Government departments in other European countries;

- European Commission DGXI, its Steering Group on Ambient Air Quality, its ad hoc Working Group on Ozone and various other DGs;

- the UNECE’s Task Force on Economic Aspects of Abatement Strategies, and its Working Groups on Effects and on Strategies;

- industry, including CONCAWE, National Power and PowerGen;

- NGOs, represented by the European Environment Bureau.

Based on a set of core assumptions, the results of the earlier work suggested that benefits would exceed costs for both the H1 and the J1 scenarios for the UK and the whole of the EC/UNECE (AEA Technology, 1999a,c).  However, under some alternative sets of assumptions it was found that the quantifiable benefits would not exceed the costs, though a number of unquantifiable effects remained outside the analysis.  These assumptions related, in particular, to the health effects assessment and include the following:

- that there is no threshold for the general population for health impacts from exposure to air pollution;

- that all fractions of PM10 are equally potent;

- that chronic exposures to PM10 are causally linked to life expectancy;

- that errors in assessment of ozone damages to crops tend to cancel out.

Obviously, it is necessary to also consider the benefits for those aspects that were not quantifiable, such as effects on ecosystems and cultural heritage.

Acceptance of this work increased substantially during the series of studies, largely as a consequence of the effort made to account for uncertainty.  A major part of the uncertainty assessment involved the development of a confidence-ranking scheme.  The ranking was determined from a survey of interested parties in UK Government, and experts in ecology, integrated assessment modelling, economics and health.  A similar scheme has been followed in this report, but based more on recent reports produced through DETR and the Department of Health (DETR, 1998, 1999; COMEAP, 1998; EAHEAP, 1999).

## Issues in benefits analysis and CosT-Benefit analysis (cba)

The quantification of monetised benefits of environmental policy remains controversial, particularly with respect to:

- perception of the ethical implications of the use of monetisation for describing effects on health and natural ecosystems, and

- the uncertainties that affect benefits analysis.

Ethical concerns regarding monetisation were answered succinctly in the recent EAHEAP report for the Department of Health:

“Two hard facts confront those who have to make decisions about the appropriate level of provision of public safety.  First, safety is usually not costless; and second, society has limited resources.  Consequently, a responsible decision about any proposed public safety improvement will require a judgement as to whether the resulting reduction in risk is large enough to justify incurring the cost of implementing the improvement.  Put another way: is the reduction in risk …worth more than whatever good things could be provided if those resources were diverted elsewhere?”

In its proper context ‘money’ is therefore a simple metric for weighting different types of benefit.  It is a metric that everyone is familiar with, and allows direct comparison of benefits with the costs of the measures required for achieving policy objectives.  Further to this, the use of CBA can increase the transparency of the decision making process through explicit demonstration of the effect of applying societally based valuations to various environmental goods.  Even if policy makers feel that alternative courses of action are preferable to those suggested through cost-benefit analysis, the fact that analysis has been carried out provides a baseline against which final decisions can be gauged.

The presence of uncertainty is more difficult to deal with.  These uncertainties are both numerous and significant and affect all stages of the analysis – not just monetisation.  At the present time there is dispute as to how they should be dealt with.  In this report a pragmatic approach is taken that allows full account to be taken of the uncertainties present.  It is important to distinguish between those aspects that can be quantified with a good level of confidence and those subject to a higher level of uncertainty.

It is often said that unquantified effects are forgotten in the final comparison of costs and benefits, with the effect that anything other than a full quantification provides a systematic bias to underestimation.  On the other hand, it has been said that studies that go beyond the Department of Health’s position, as given by COMEAP/EAHEAP, introduce a danger of over-estimation.  Either way, there is a danger of generating a misleading answer which may, perhaps, be worse than having no answer at all, if it provides an artificial impression of comfort.  What is needed to resolve these problems is a methodology that promotes a better understanding of the overall consequences of uncertainties.  This report proposes such a methodology.

A recent study (SEI, 1999) highlights problems in the assessment of abatement costs, largely because the options included in the models tend to be restricted to end-of-pipe solutions.  Available data suggest that this leads to overestimation of abatement costs, as a consequence of the failure to account for alternative measures that may well be cheaper, and a failure to account for the effects of technological change.  Representatives of industry, however, tend to say that costs are underestimated (e.g. Cocks and Rodgers, 1997).  Clearly, however, it is important to be aware of the limitations of the analysis on both sides of the cost-benefit equation.

PART 2:
Analysis of Emissions and Costs

# Methods - Costs

## Development of cost curves

UK cost curves were developed by AEA Technology for SO2, NOx and VOCs, and by the Imperial College Centre of Environmental Technology (ICCET) for NH3.

### Input data

- UK inventory data

The starting point for the cost curves was emission estimates for 1990.  For SO2, NOx and VOCs these were taken from the most up-to-date version of the UK’s National Atmospheric Emissions Inventory (NAEI 1997), and from both the NAEI and MARACCAS for NH3.

- UK growth rates

The 1990 emission estimates were projected forward to 2010 using provisional projections of industrial growth rates provided by DTI.  Growth rates for fuel use were available for the major industrial and final-use sectors individually.  The Central (growth), High (fuel price) scenario was used in all cases.

The projected NAEI data were then converted into the emission categories used in the RAINS model developed by IIASA.  In a number of cases there is poor correspondence between NAEI source categories and those used in RAINS.  For example, whereas IIASA splits industrial combustion into boilers and other devices, the NAEI does not.  The conversion of NAEI sectors to those used by IIASA, therefore, involved some subjectivity.  The emission estimates derived in this manner represent emissions in 2010 assuming no change in the levels of control from 1990 onwards, and will be referred to hereafter as the ‘uncontrolled emissions’.

- UK forecasts

The cost curves also make use of UK forecasts of the impact of current legislation.  The forecasts take account of the following legislation:

- Environmental Protection Act

- Solvents Directive

- Petrol Distribution (Stage 1) Directive

- Sulphur in Liquid Fuels Directive

- Large Combustion Plant Directive

- Directives on transport emissions up to and including the legislation from Auto-Oil I: 91/441/EEC, 94/12/EC, 93/59/EEC, 91/542/EEC (Stage 1 and 2), the European Conciliation Committee and Common Position agreements on vehicle emissions (Auto-Oil) Euro III (2000) and Euro IV (2005) and European Directives Relating to the Quality of Petrol and Diesel Fuels (2000 and 2005).

These forecasts provide estimated emissions in 2010, taking into account control measures that have already been agreed, and are used in estimating UKREF emissions.

- IIASA cost data

Cost data for use in the cost curves were taken from the methodology developed by IIASA.  The UK has previously critiqued the IIASA data, leading to extensive changes to the IIASA methodology to take account of UK comments.  In the case of the VOC and SO2 curves, some minor modifications have been made for this report to better reflect the UK situation.

### Development of the Cost Curves

Each cost curve starts with the uncontrolled emission.  The first part of the cost curve includes those measures that are the most cost-effective means of achieving the UKREF emission.  For example, the uncontrolled VOC emission for industrial adhesives is 86.7 kt, whilst the UKREF emission is 42.7 kt, a decrease of 44 kt.  There are three options for the industrial adhesives sector - good housekeeping, the additional use of substitution, and the additional use of incineration, in ascending order of cost per tonne of VOC abated.  The reduction in emissions from the uncontrolled emission to the UKREF emission can be achieved, as shown in Table 3.

Table 3.  Development of the VOC cost curve for the industrial adhesives sector.

| Measure | Marginal cost | Efficiency | Uptake | Reduction |
| --- | --- | --- | --- | --- |
|  | euro/tonne | % | % | kt |
| Housekeeping | 10 | 10.5 | 100 | 9.1 |
| Substitution | 400 | 53.9 | 93 | 34.9 |
| Substitution | 400 | 53.9 | 0 | - |
| Incineration | 6000 | 57.9 | 0 | - |

The housekeeping measure is the most cost effective and is applied fully.  However, this reduces emissions by only 9.1 kt.  The substitution measure is the next most cost effective, but only needs to be applied in 93% of cases in order to provide the remaining 34.9 kt of reduction necessary to give a total reduction from the sector of 44.0 kt.  Because of the need for only partial rather than complete implementation of substitution, it appears twice in the list, the extra allowing an additional 7% reduction beyond that required.  Finally, incineration is the least cost-effective measure.

Figure 1 shows a typical cost curve.  Starting from the uncontrolled emission, emissions are reduced at increasing marginal cost (indicated by the gradient) until the)UKREF emission is reached.  The marginal cost of abatement just above this emission level is lower than the marginal cost of abatement just before this point is reached.  This is because the cost curve is constrained to the controls that will come in under)UKREF, as a result of existing legislation.  Thus, some of the most cost-effective measures might not be used until after the baseline emission is reached, because they address sectors that are not controlled in the)UKREF scenario.

### Development of regional cost curves

The 1997 NAEI comprises spatially disaggregated (or regional) inventories of individual pollutants and regional cost curves were, therefore, developed by using these regional inventories rather than a UK inventory.  In all other respects, the cost curve was identical, i.e. the same cost data were used and the same growth rates were assumed.  The regional inventories were developed using the same data as used to map emissions. (see section 3.2 below)

### Uncertainties in the cost curves

There are a number of areas of uncertainty in the cost curves.  These may be divided into uncertainties regarding uncontrolled emissions, uncertainties regarding the UKREF emissions and uncertainties regarding the costs and effectiveness of the control options used in the cost curves.

The UKREF emissions data for the cost curves are taken from the 1997 NAEI.  The NAEI has been developed over a period of more than ten years, allowing the emissions data to achieve what is believed to be a good level of accuracy.  The inventory for SO2 is probably accurate to 10% while the NOx and VOC inventories are perhaps accurate to 25%. However, the NAEI is continuously being updated and improved, as the emission sources and processes are better understood or more accurate data become available. This allows historic data to be revised retrospectively and it is to be expected that the 1990 data in the NAEI published in coming years might vary from the data used in this study.

The UKREF emissions are derived by making assumptions about the percentage reduction in emissions from each emitting sector as a result of legislation.  Although these assumptions are made on the basis of a wide body of information, the approach is simple and there is scope for considerable uncertainty.  Risk analysis, carried out on an earlier set of VOC forecasts, suggested that the uncertainty in the reduction achieved was likely to be of the order of 10%.  A similar figure could be assumed for NOx and SO2.  It should be said that the emission forecasts used to generate the UKREF emissions are fairly conservative, in that no account is taken of improvements in technology, except where required by environmental legislation.  Thus, it might be expected that the UKREF emissions are more likely to be overestimated rather than underestimated.

Figure 1.  Typical cost curve for the abatement of air pollutants subject to existing legislation.

The final source of uncertainty is the cost effectiveness data derived from the IIASA methodology.  This methodology has undergone extensive peer review throughout Europe and is considered by the authors to be the most comprehensive and accurate set of cost effectiveness data available.  Nonetheless, the data will be subject to quite large uncertainty.  The effectiveness of control measures is likely, if anything, to be overstated, since IIASA often assume that control measures can be used for all processes within a sector whereas this is not always likely to be true.

No attempt has been made to quantify the likely impact of these uncertainties in numeric terms.  However a qualitative assessment is made below (Table 4).  In all cases there are some biases to overestimation and some to underestimation.

Table 4.  Sources of uncertainty in abatement cost estimates, and potential effects on the outcome of the analysis.  ++ and -- denote cases where the real costs could be significantly higher or lower respectively than those estimated here.  + and – identify cases where uncertainties may still influence the results, but less significantly.

| Source of uncertainty | Impact on cost of reaching a given emission ceiling |
| --- | --- |
| - Scenario uncertainties |  |
| Uncontrolled emissions data | +/- |
| UKREF | +/-- |
| - Option uncertainties |  |
| Effectiveness of control measures | ++/- |
| Cost of control measures | +/- |

## Mapping of emission totals

UK emissions of NOx, SO2, and VOC were mapped using existing data available from the NAEI.  These include point source data for power stations and other large combustion plant, incineration plant, iron and steel processes, cement and lime processes, chemical processes, refineries, large solvent users and whisky distilleries.  Road transport emissions were mapped using information on traffic on major roads.  Other sources were treated as area sources and were distributed using regional statistics, such as fuel use or population.

Emission estimates for ammonia for the UK were obtained from three sources: the RAINS and MARACCAS models and the NAEI.  Total estimates from MARACCAS (which was taken as the main source of data for NH3 emissions and costs in this study) and the NAEI agreed well.  However, significant differences were noted when these estimates were compared with RAINS, although this was not surprising, given that knowledge of ammonia emissions is improving rapidly, in response to the demands of legislation, such as the proposed Protocol and Directive.

Ammonia was mapped taking account of both agricultural and non-agricultural sources.  The principal source of data for this was the ITE land cover database.  Ammonia emissions are dominated by livestock production.  Non-agricultural sources are numerous; including industry, emissions from catalytic converters, pets, horses, wild animals etc.  There is continuing uncertainty concerning non-agricultural emissions of ammonia for most of the EU countries.

# Results - Costs

## Overall costs

Cost data for each of the scenarios are shown in Table 5.

Table 5. Costs of emission reductions for the UK (£million/year) arising from movement from the UKREF scenario to the scenarios shown.

| - Scenario | SO2 | NOx | VOC | NH3* | Total |
| --- | --- | --- | --- | --- | --- |
| WGS31c | 34 | 9 | 18 | 0 | 61 |
| J1 | 61 | 9 | 64 | 11 – 24 | 145 – 158 |
| H1 | 61 | 9 | 435 | 11 - 24 | 516 – 529 |

*Abatement costs for ammonia emissions are shown as a range because of uncertainty about the acceptability of urea substitution, the higher end of the range assuming that it is not adopted.

For SO2 all of the scenarios can be met through the fitting of wet flue gas desulphurisation (WFGD) to coal fired power stations.  However, it should be noted that this is based on the assumption that existing UK power stations can be fitted with WFGD, which can operate at high efficiency typical of state-of-the-art systems (95%).  This may not be possible if power stations are operated at low load factors.  In 1997, UK coal fired power stations were operated at an average 45% annual load factor and the trend in recent years has been steadily downward. If the SO2 cost curve is modified so that the efficiency of the WFGD is 75%, a value more typical of stations operating at low load factors, the cost per tonne of SO2 abated by WFGD rises significantly, and the use of low sulphur coal is then a more cost–effective option for UK power stations.  However, this is not the most cost-effective abatement option when all sources of SO2  are taken into account, and other measures are required in order to meet the various emissions targets. This is illustrated in Table 6, where the measures required to meet each of the specified targets are listed in order of cost-effectiveness (marginal cost per tonne abated) and, therefore, in order of their applications to the system as a whole. Costs also rise above those shown in Table 5, to £43 million for WSG31b; £92 million for J1 and £94 million for H1.

The control measures required to achieve reductions in NOx emissions are shown in Table 7 and those required for VOC in Table 8.

The VOC reductions include measures that would affect small businesses.  If the control measures which affect these sectors are removed from the cost curve, the cost of reaching the WSG31b scenario is not affected, but the cost of reaching J1 rises above that shown in Table 5 to £71 million and H1 cannot be reached.

Table 6.  Measures used to achieve reductions in SO2 emissions assuming WFGD can be operated at 75% efficiency only.

| Scenario | Sector | Measure | Cumulative emission (ktonnes) | Cumulative Cost (£ million) | Marginal cost (£/tonne) |
| --- | --- | --- | --- | --- | --- |
| WGS31c | Industrial processes Industrial boilers Power stations burning coal | Various methods, 40% efficiency Low sulphur coal Low sulphur coal | 709 677 625 | 22 30 43 | 233 245 245 |
| J1 | Power stations burning coal Residential combustion Residential combustion Industrial boilers Fuel conversion industry Oil fired power stations | Low sulphur coal Low sulphur coal Low sulphur coke Low sulphur coke Low sulphur coke Low sulphur fuel oil | 622 608 605 604 603 602 | 44 47 48 48 49 49 | 245 260 260 262 262 288 |
|  | Industrial boilers Residential combustion Shipping Fuel conversion Process industry Process industry Power stations Fuel conversion Off-road transport Shipping Fuel conversion – coal | Low sulphur fuel oil Low sulphur fuel oil Low sulphur fuel oil Low sulphur fuel oil Various methods, 70% efficiency Various methods, 80% efficiency Low sulphur gas oil Low sulphur gas oil Low sulphur DERV Low sulphur gas oil Limestone injection | 593 588 586 566 536 521 521 521 521 500 499 | 52 53 54 59 70 83 83 83 83 104 105 | 288 288 288 288 366 837 1029 1029 1029 1029 1127 |
| H1 | Fuel conversion – coke Fuel conversion – fuel oil | Wet flue gas desulphurisation Wet flue gas desulphurisation | 498 497 | 106 108 | 1260 1647 |

Table 7. Measures used to achieve reductions in NOx emissions

| Scenario | Sector | Measure | Cumulative emission (ktonnes) | Cumulative Cost (£ million) | Marginal cost (£/tonne) |
| --- | --- | --- | --- | --- | --- |
| All scenarios | Fuel conversion Industrial furnaces | Combustion modifications Combustion modifications | 1182 1181 | 9 9 | 150 243 |

Table 8. Measures used to achieve reductions in VOC emissions.

| Scenario | Sector | Measure | Cumulative emission (ktonnes) | Cumulative Cost (£ million) | Marginal cost (£/tonne) |
| --- | --- | --- | --- | --- | --- |
| WGS31c | Other industrial solvent use Domestic wood fired boilers Refineries Oil production and distribution | New agrochemical formulations New appliances Covered oil-water separators and flaring Vapour balancing during loading | 1299 1297 1254 1200 | 0 0 8 18 | 0 116 179 183 |
| J1 | Oil production and distribution Domestic coal fired boilers Other industrial solvent usea Industrial adhesivesa Refineries Dry cleaninga Organic chemicals manufacture Off-road machinery Motorcycles Industrial paintsa | Vapour balancing during loading New appliances Control of seed oil extraction & leather coating Housekeeping & substitution Incineration Conventional closed circuit machines Incineration Oxidation catalysts Oxidation catalysts Substitution | 1193 1181 1179 1177 1173 1172 1139 1127 1108 1100 | 19 22 22 23 24 25 39 46 57 64 | 218 271 276 319 364 439 600 600 803 |
| H1 | Industrial paintsa Rubber processinga Retail decorative paints Offset printing Trade decorative paints Degreasing (new installations)a Publication gravure Oil production and distribution Petrol distribution, stage 1 Vehicle refinishinga Degreasing (new installations)a Organic chemicals manufacture Dry cleaning (new installations)a Dry cleaninga Domestic solvent use Packaging printing (new installations) Packaging printing Oil production and distribution Organic chemicals storage Degreasing (new installations)a Degreasinga Screen printinga Industrial adhesivesa Screen printing (new installations)a Wood preservation (new installations)a Automobile coating (new installations) Retail decorative paints | Substitution End-of-pipe abatement Substitution with waterborne paints Incineration Substitution with waterborne paints Carbon adsorption Enclosure and carbon adsorption Minimisation of venting Single stage vapour recovery Substitution Conveyorised degreasing with carbon adsorption Monthly leak detection and repair New generation closed circuit machines New generation closed circuit machines Reformulation of products Incineration End-of-pipe abatement Minimisation of flaring Single stage vapour recovery unit Water based systems or plasma processes Water based systems and conveyorised degreasing Enclosure and end-of-pipe abatement Incineration Enclosure and end-of-pipe abatement Carbon adsorption End-of-pipe abatement Substitution with waterborne and high solids paints | 1092 1090 1085 1084 1081 1079 1078 1063 1062 1062 1056 1053 1051 1050 1015 1015 1014 991 990 980 976 976 973 972 966 964 964 | 71 72 78 79 83 86 86 108 110 110 124 131 137 138 237 238 242 323 328 364 379 380 395 398 426 433 437 | 803 1000 1111 1230 1250 1281 1361 1407 1905 2067 2167 2477 2759 2759 2843 3088 3415 3502 3865 3896 4017 4019 4110 4466 4571 4689 4848 |

a -  sector includes small businesses

## Regional costs

The regional allocation of emissions for each scenario is shown in Table 9 to Table 12 below.

Table 9. Regional allocation of emissions of NOx (kt)

| Scenario | England | Scotland | Wales | N. Ireland |
| --- | --- | --- | --- | --- |
| UKREF | 992 | 102 | 59 | 32 |
| WGS31c//J1/H1 | 987 | 101 | 58 | 32 |

Table 10. Regional allocation of emissions of SO2 (kt)

| Scenario | England | Scotland | Wales | N. Ireland |
| --- | --- | --- | --- | --- |
| UKREF | 602 | 57 | 100 | 25 |
| WGS31c | 468 | 43 | 94 | 20 |
| J1 | 362 | 32 | 89 | 16 |
| H1 | 360 | 32 | 89 | 16 |

Table 11. Regional allocation of emissions of VOC (kt)

| Scenario | England | Scotland | Wales | N. Ireland | Offshore |
| --- | --- | --- | --- | --- | --- |
| UKREF | 883 | 207 | 55 | 26 | 129 |
| WGS31c | 852 | 184 | 47 | 26 | 92 |
| J1 | 773 | 174 | 43 | 23 | 87 |
| H1 | 689 | 155 | 39 | 21 | 61 |

Table 12. Regional allocation of emissions of NH3 (kt)

| Scenario | England | Scotland | Wales | N. Ireland |
| --- | --- | --- | --- | --- |
| Reference/WGS31c | 202 | 42 | 24 | 30 |
| J1/H1 | 182 | 36 | 20 | 26 |

The regional split of costs for each scenario is shown in Table 13.  The cost ranges shown in the Table represent uncertainty with respect to ammonia abatement measures, in particular whether or not urea substitution will be allowed.  This has a substantial influence on the costs in Northern Ireland and Wales under the J1 scenario.

Table 13.  Costs by region (£million/year) arising from movement from the UKREF scenario to the scenarios shown.

| Scenario | England | N. Ireland | Scotland | Wales | Offshore |
| --- | --- | --- | --- | --- | --- |
| WGS31c | 41 | 1 | 8 | 3 | 7 |
| J1 | 108 – 116 | 4 – 6 | 16 – 18 | 7 – 9 | 8 |
| H1 | 339 - 347 | 11 – 13 | 68 - 71 | 19 - 21 | 77 |

- PART 3:
Quantification of Benefits and the Comparison of Costs and Benefits

# Methods - Benefits

## Overview

The analysis presented in this Part of the report proceeds through:

- description of the effects of the four pollutants;

- quantification of the impacts and associated economic benefits of reducing emissions;

- comparison of these benefits with costs in a structured manner that retains account of the uncertainties that affect the assessment.

The framework for the analysis of benefits is shown in Figure 2.  The main models used were:

- ALPHA (Atmospheric Long-range Pollution Health/environment Assessment)(see AEA Technology, 1999a)

- ALPHA-UK (see Holland et al, 1999)

- RAINS (Amann et al, 1999)

ALPHA and ALPHA-UK were both developed by AEA Technology to quantify impacts of air pollution on health, building materials, agriculture, forestry and visibility.  ALPHA, operating at a resolution of 150 by 150 km provides analysis over the whole of Europe, and was used in the analysis carried out for UNECE and the European Commission.  As its name suggests, the GIS based ALPHA-UK, used in support of the recent review of the National Air Quality Strategy, provides a much more detailed assessment for the UK.  In this study it has been run at a resolution of 10 by 10 km.  Neither model includes analysis of ecosystem effects; for these RAINS was used for the European scale, and the databases held by ITE for a more detailed appraisal for the UK.

| Pollution load |  Concentration, deposition |
| --- | --- |
|  |  |
| Stock at risk |  Distribution of people, etc. |
|  |  |
| Sensitivity of stock at risk |  Baseline mortality, etc. |
|  |  |
| Exposure-response functions |  Rate of damage/unit pollution |
|  |  |
| Impact assessment |  Change in crop yield, mortality, etc |
|  |  |
| Valuation |  Willingness to pay/accept |

Figure 2.  Stages of the benefits analysis.

## Effects of the Pollutants

Reported effects of the four pollutants addressed by the Protocol and Directive are listed in Table 14, including effects of related secondary pollutants.  For some of the effects shown there is debate as to whether they are real or not.  However, there are undoubtedly some effects yet to be identified.

## Sources of data for the analysis

As Figure 2 clearly shows, the analysis requires a large amount of data.  The sources used are described in:

- Appendix 4 (assessment of pollution deposition and concentration in the UK)

- Appendix 5 (calculations made using the RAINS model)

- Appendix 6 (analysis of effects on health, building materials, agriculture, forestry and other ecosystems).

To the extent possible, exposure-response functions, valuations, etc. are based on earlier studies for DETR (for review of the NAQS) and the Department of Health (COMEAP and EAHEAP).  Where this is not possible alternative sources have been used, particularly the European Commission’s ExternE Project (European Commission 1995, 1999) which provides a framework for a holistic assessment and for a systematic and thorough consideration of uncertainties across all of the receptors of importance here.

## Characterising uncertainties

As noted above, one of the main difficulties in cost-benefit analysis concerns the fact that, to take a holistic view of benefits, it is necessary to consider effects that can be quantified with limited confidence alongside those that can be quantified well.  If all of the economic data on benefits are combined to give a single figure of benefit, it is not possible to gain an appreciation of the extent to which any comparison of cost and benefit may be considered robust.

In previous work a structured sensitivity analysis for air quality benefits appraisal was developed.  This sought to retain understanding of the confidence associated with the different types of benefit to be gained from pollution abatement in a transparent manner.  The same principle is applied here.

For this study the approach has been adapted in a manner that is more consistent with previous analyses conducted for DETR and the Department of Health, as regards the ranking and grouping of benefits.  It has also been extended to provide a more explicit assessment of possible ranges in the overall benefits.

Table 14.  Effects of abatement of emissions of SO2, NOx, NH3 and VOCs.  An assessment (albeit partially subjective) of the likely importance of each effect following a reduction in emissions is shown in the final column: ‘+++’ benefit from reduction, very important, through to ‘-‘ slight cost from abatement (e.g. through reduced fertilisation of crops with S or N).  ‘?’ denotes effects for which particular uncertainty exists, for example concerning the existence of chronic effects of air pollutants on morbidity, or the importance of visibility reductions.

| Effect | Importance of effects |
| --- | --- |
| Health |  |
| Ammonium sulphate and nitrate aerosols acute – mortality chronic – mortality acute – morbidity chronic – morbidity | +++ +++? ++ ++? |
| Ozone acute – mortality acute – morbidity chronic – morbidity | +++ ++ ? |
| SO2 acute – mortality acute – morbidity chronic – morbidity | +++ ++ ? |
| Direct effects of VOCs on mortality and morbidity | ++? |
| Direct effects of NO2 on mortality and morbidity | ++? |
| Altruistic effects of the above health impacts | +++? |
| Impacts on competitiveness of businesses linked to the above health effects (via changes in rates of absenteeism, demand for pharmaceutical products, consumer demand via changes in mortality rates) | ++? |
| Materials |  |
| SO2 / acid effects on utilitarian buildings | ++ |
| Effects on cultural assets | +++? |
| Effects on steel in re-inforced concrete | + |
| Effects of O3 on paint | no significant effect |
| Effects of ozone on rubber | + |
| Macroeconomic effects | ++? |
| Crops |  |
| Direct effects of SO2 and O3 on crop yield | ++? |
| Indirect SO2 and O3 effects on livestock | + |
| N deposition as fertiliser | -- |
| Interactions between pollutants, with pests and pathogens, climate... | --/++ |
| Acidification/liming | + |
| Macroeconomic effects | ++? |
| Forests |  |
| O3 effects on timber production | +? |
| Non-O3 effects | ++? |
| Non-timber benefits of forests | ++? |
| Exceedence of critical load for eutrophication | ++? |
| Exceedence of critical load for acidification | ++? |
| Other ecosystems |  |
| Exceedence of O3 critical level | ++ |
| Exceedence of critical load for eutrophication | +? |
| Exceedence of critical load for acidification | +++ |
| Visibility |  |
| Change in amenity | ++? |
| Secondary effects of pollution abatement measures on pollutants not considered under the Directive/Protocol (e.g. greenhouse gases) | --/+++ |

The benefits are quantified in a series of stages, starting with the aspects that can be quantified with the least uncertainty:

- pollutant exposures;

- assessment of the effects quantified by the Interdepartmental Group on Costs and Benefits (IGCB):

- critical loads exceedence;

- mortality and respiratory hospital admissions linked to short term pollutant exposures;

- damage to building materials;

- change in crop yields.

Analysis to this stage has previously been accepted by UK Government departments, for example in the review of the National Air Quality Strategy, and hence provides the results upon which the greatest stress is placed in this report.  However, it excludes assessment of a number of effects that have been reported in the literature.  This introduces a likely bias towards underestimation of the effects of air pollution.  It also excludes valuation of health effects on the grounds that available data are too uncertain.

The application of uncertain data clearly provides the threat that the results derived using them could be misleading.  However, the exclusion of effects, particularly those for which there is a strong logic for inclusion, also provides a threat that results will be misleading; it could be said to imply an unwarranted degree of assurance that present conditions are acceptable; that the benefits of pollution abatement are less than they really are.  With this in mind the analysis has been extended to a number of additional aspects, quantified in a sensitivity analysis that follows assessment of the issues identified above.  These aspects, in order of increasing uncertainty, are as follows:

- assessment of additional health effects for which COMEAP provided exposure-response functions, but did not use them in its quantification exercise;

- application of the ranges in valuation data identified by EAHEAP.  This monetisation will undoubtedly be controversial given the previous conclusion of Department of Health Ministers that available health valuation data were not robust enough to be applied in support of policy (DETR, 1999);

- application of additional exposure-response functions reported in the literature, but for which there are outstanding issues of interpretation and greater concern over the accuracy of predictions made.

In addition to this structured approach, individual elements of uncertainty are discussed as they arise in the next section and in the Appendices.

# Results - Benefits

## Pollutant exposures

Table 15 highlights change in exposure to photo-oxidants and PM10 (in the form of aerosols generated chemically in the atmosphere following emission of SO2, NOx and NH3).  The change in PM10 is expressed as a percentage against total PM10 (i.e. including primary particulates as well as the secondary aerosols) across the UK.

These results are particularly important to UK policy because imports of both classes of pollutant (and their precursors) create difficulty in meeting objectives laid down in the NAQS.  The third column of the table includes benefits from UK abatement as well as the other UNECE Member States.  Accordingly, the difference between columns 2 and 3 represents the contribution to UK benefits from abatement in countries outside of the UK.

AOT40 and AOT60 provide information on the extent to which peak ozone concentrations are reduced.  For example, AOT60 is calculated by subtracting 60ppb from hourly ozone concentrations over the year.  Results, where positive (i.e. for which measured or modelled ozone is in excess of 60 ppb) are then summed over the year to give total exposure in excess of 60 ppb.  The combination of time and concentration gives an indication of annual dose in excess of possible thresholds.

AOT40 is of interest because research on crops, particularly wheat, indicates a threshold around 40 ppb.  AOT60 is selected as an indicator for health impacts (WHO, 1997) though evidence for a threshold for ozone effects on health is far from conclusive, and is indeed contradicted by the results of a number of epidemiological studies.  Taken together, AOT40 and AOT60 provide some indication of progress to meeting the NAQS objective of 50 ppb as a running 8-hour mean.  Effects on peak concentrations do not provide good guidance on the consequences for mean exposures, which were not modelled in this study.

The results in Table 15 show a relatively small decline in mean particle exposure, but a significant (>10%) fall in metrics of peak ozone exposure for both scenarios J1 and H1.  The importance of the PM10 reductions associated with each scenario is possibly downplayed by expressing them in terms of % of mean exposure levels.  A substantial fraction (estimated at around a third to a half) of particle exposure arises from natural and other sources which are largely beyond human control.  On this basis, the reductions in particulate exposure shown in the Table should be roughly doubled, to show the effect on the component of concentration that can potentially be controlled.  Overall, the benefits to be gained under WGS31c from abatement within the UK are roughly half those of H1 and J1.

Table 15.  Fall in PM10 exposures as a consequence of moving to scenarios of increasing emissions abatement.  Figures for 1990 and UKREF show predicted total PM10 exposure metrics in 1990 and 2010 respectively, figures for other scenarios show % change compared to UKREF.

|  | Benefit to UK from UK abatement | Benefit to UK from UNECE abatement |
| --- | --- | --- |
| Population weighted mean particulate exposure, µg/m3 |  |  |
| 1990 | 25.51) | 25.51) |
| UKREF1) | 19.11) | 19.11) |
| WGS31c | -0.8% | -0.9% |
| J1 | -1.8% | -2.9% |
| H1 | -1.8% | -2.5% |
| - AOT40 ppm.hours.area2) |  |  |
| 1990 | 204 | 204 |
| UKREF | 148 | 148 |
| WGS31c | -7% | -13% |
| J1 | -13% | -25% |
| H1 | -22% | -35% |
| - AOT60 million person ppm.hours2) |  |  |
| 1990 | 125 | 125 |
| UKREF | 75 | 75 |
| WGS31c | -5% | -16% |
| J1 | -12% | -35% |
| H1 | -20% | -40% |

Note: nq = not quantified.

1) Baseline particles data based on Stedman et al (1997 and 1999; for 1995 and 2005 respectively) increased by a factor of 1.3 to account for under-estimation of certain species through the use of TEOMs in the monitoring network.

2) Ozone data are taken from estimates generated using the web version of the RAINS model.

## Critical loads exceedence

The concept of critical loads for deposition of acidifying and nutrifying pollutants has developed since the mid-1980s, and forms the basis of much of the analysis carried out for both UNECE and the European Commission in developing emission targets.  The critical load defines a threshold for pollutant deposition, beyond which ecological change is likely to occur.  In some cases the changes linked to exceedence of critical loads can be dramatic, as in the loss of trout and salmon from acidified freshwater systems.  In others the changes might be more subtle.  The subject is reviewed in depth in the reports of the Review Group on Acid Rain and INDITE (Impacts of Nitrogen Deposition in Terrestrial Ecosystems), produced for DETR.

Critical loads work for the UK up until 1994 (the date of the Second Sulphur Protocol) was based on a map of 1x1km acidity critical loads for soils.  The critical load values were assigned on the basis of the mineralogy of the dominant soil type in each 1km square.  At that point in time acid deposition was considered in terms of sulphur deposition only, ignoring any additional acidification from nitrogen deposition or any ameliorating effects of base cation (calcium & magnesium) deposition.  Exceedances of the critical load (ie the level of deposition above the critical load) were, therefore, calculated using sulphur deposition only. The exceedance maps generally showed high exceedance values in the north and west of Britain, where critical loads tend to be lower (due to thinner, base-poor soils) and sulphur deposition higher.  This sulphur deposition was generally highest in central Britain (the area around the major power station sources) and in the north and west where the amount of rainfall is also high.

The multi-pollutant multi-effect Protocol (signed in 1999) was developed for the control of sulphur and nitrogen pollutant emissions and addressed the problems of acidification and eutrophication.  For this work, acidification included the effects of both sulphur and nitrogen deposition, so new calculations had to be performed to give estimates of critical loads and their exceedances.  The methods developed and used in the UK are the same as those agreed internationally by the Task Force on Mapping, under the UNECE Convention on Long Range Transboundary Air Pollution (LRTAP).  Each country under the Convention calculates critical loads for ecosystems they consider to be sensitive to acidification and/or eutrophication.

In the UK critical loads are calculated for five soil-vegetation ecosystems ie acid grassland, calcareous grassland, heathland, coniferous woodland and deciduous woodland. These critical loads also take into account nitrogen and base cation uptake and other removal processes (ie, nitrogen immobilisation) within the ecosystems.  In addition, critical loads are calculated for 1445 lake or headwater streams throughout Great Britain; these are generally high altitude sites with small catchment areas.

To calculate exceedances of these acidity critical loads, non-marine sulphur, nitrogen (both oxidised and reduced) and non-marine base cation deposition are taken into account.  Deposition maps for 1992-94 show the highest non-marine sulphur in central England, the mountains of Wales and south-west Scotland.  Nitrogen deposition is generally lower across Scotland compared to England and Wales.  Non-marine base cation deposition is high across the western half of the UK but with the highest values in the north west of Scotland.  Inputs of sulphur and nitrogen deposition acidify, whereas non-marine base cation deposition “buffers” the incoming acidity and can ameliorate the effects of acidification.  So, the net incoming acid deposition is the sum of the non-marine sulphur and nitrogen deposition less the amount of non-marine base cation deposition.  Figure 3 clearly shows high net acid deposition across much of the UK, but with lower values in the far north of Scotland, the west coast of Scotland and the west of Northern Ireland. When this net deposition is compared with the critical load values, it gives rise to the small exceedance values, or even areas of non-exceedance in these parts of Scotland and Northern Ireland.  Similar patterns of exceedance are also obtained when estimates of acid deposition for 2010 are used.

In practice, non-marine base cation deposition is included in the calculation of critical loads, rather than as part of the net acid deposition, because:

- The EMEP model used to provide estimates of sulphur and nitrogen deposition at the European scale, does not provide values for non-marine base cation deposition.  The EMEP model deposition data are used in the calculation of critical load exceedances for Europe, and in particular for looking at future emission and deposition scenarios for 2010.  Individual countries under the LRTAP Convention use their national estimates of base cation deposition in their calculations of critical loads;

- The HARM model used in the UK to look at emission and deposition scenarios for the future also excludes base cation deposition.

However, the way in which base cation deposition is incorporated in the calculation of critical loads gives the same results as including it in the sum of net acid deposition.

The change in critical loads exceedence through the different scenarios in England, Northern Ireland, Scotland, Wales and the UK as a whole is shown in Table 16 and Table 17.  Table 18 and Table 19 show improvements compared to the UKREF scenario.  The data shown in these Tables relate to the area over which exceedence is predicted.  Further information is given in Appendix 7.  Table 16 and Table 17 have different total ecosystem areas for two reasons:

- critical loads for nutrient nitrogen are not calculated for UK freshwater ecosystems;

- the number of grid squares (and therefore ecosystem areas) for which nutrient nitrogen critical loads are available is slightly smaller than the number for which acidity data are available for the UK.

In general ecosystems at high elevation in the UK tend to be at the greatest risk.  Given the uneven distribution of ecosystems across the UK, it follows that exceedence of critical loads would not affect equally all species, types of ecosystem, etc. within each group shown in the Tables.  Even slight exceedence of critical loads (using % at risk, as reported here) could in theory have a significant and long-term impact, for example affecting the viability of a species.

Figure 3  Acid deposition in the UK, 1992-1994.

### England and Wales

The two countries are treated together as the trends shown are very similar.  Under the UKREF scenario the most significant exceedence of critical loads for both acidification and eutrophication would appear to affect acid grassland and heaths.  Even under the most restrictive (J1) scenario it is estimated that 25% of acidic grasslands (by area) would show exceedence of the critical load for acidification.  However, there is appreciable exceedence also for deciduous and coniferous forests and freshwaters.

### Northern Ireland

There is only very slight exceedence of the critical load for acidification on acidic grassland in Northern Ireland.  No other impacts are expected.

### Scotland

Problems of acidification in Scotland are less pronounced than in England and Wales, but greater than in Northern Ireland.  The ecosystems most affected are predicted to be acidic grasslands and coniferous forests.  Our results indicate that eutrification is unlikely to be a problem in Scotland under any scenario.

### United Kingdom

The general patterns seen for England and Wales are repeated for the UK as a whole.  They are, however, less pronounced, as a consequence of the limited exceedence of critical loads in Scotland.

In addition to the results presented above, maps of total areas exceeded were prepared for each scenario.  Examples are shown in Figure 4.  The maps demonstrate the uneven distribution of exceedence noted above, this reflecting variation in both the sensitivity of ecosystems and in deposition.  Of particular note are problems for acidic grasslands in Wales, the Pennines and Southern Scotland.

Table 16.  Exceedence of critical loads for acidification across the UK under different scenarios.  Ecosystem area in hectares, exceedences by % of area.

| England | Ecosystem area (ha) | UKREF | H1 | J1 |
| --- | --- | --- | --- | --- |
| Acid grassland | 1126445 | 37 | 25 | 23 |
| Calcareous grassland | 855030 | 0.0 | 0.0 | 0.0 |
| Heath | 139459 | 24 | 4.9 | 3.8 |
| Coniferous woodland | 181039 | 6.9 | 6.2 | 4.9 |
| Deciduous woodland | 647548 | 5.6 | 2.1 | 1.7 |
| Freshwaters | 142860 | 6.6 | 5.5 | 4.8 |
| All ecosystems | 3092381 | 17 | 10 | 9.5 |
| Northern Ireland | Ecosystem area (ha) | UKREF | H1 | J1 |
| Acid grassland | 132145 | 0.20 | 0.10 | 0.05 |
| Calcareous grassland | 146958 | 0.00 | 0.00 | 0.00 |
| Heath | 31176 | 0.00 | 0.00 | 0.00 |
| Coniferous woodland | 50629 | 0.00 | 0.00 | 0.00 |
| Deciduous woodland | 26839 | 0.00 | 0.00 | 0.00 |
| Freshwaters | 0 | 0.00 | 0.00 | 0.00 |
| All ecosystems | 387747 | 0.07 | 0.02 | 0.02 |
| Scotland | Ecosystem area (ha) | UKREF | H1 | J1 |
| Acid grassland | 3736432 | 6.2 | 2.8 | 2.2 |
| Calcareous grassland | 0 | 0.0 | 0.0 | 0.0 |
| Heath | 709889 | 0.3 | 0.1 | 0.1 |
| Coniferous woodland | 429652 | 4.5 | 3.5 | 3.1 |
| Deciduous woodland | 122452 | 1.4 | 0.4 | 0.2 |
| Freshwaters | 171155 | 0.3 | 0.2 | 0.2 |
| All ecosystems | 5169580 | 5 | 2.4 | 1.8 |
| Wales | Ecosystem area (ha) | UKREF | H1 | J1 |
| Acid grassland | 462379 | 54 | 29 | 25 |
| Calcareous grassland | 14400 | 0.0 | 0.0 | 0.0 |
| Heath | 111726 | 22 | 11 | 8.4 |
| Coniferous woodland | 76801 | 4.2 | 3.4 | 3.0 |
| Deciduous woodland | 236779 | 3.5 | 2.5 | 2.1 |
| Freshwaters | 26438 | 2.6 | 1.3 | 1.2 |
| All ecosystems | 928523 | 31 | 17 | 14 |
| United Kingdom | Ecosystem area (ha) | UKREF | H1 | J1 |
| Acid grassland | 5457401 | 16 | 9.5 | 8.4 |
| Calcareous grassland | 1016388 | 0.0 | 0.0 | 0.0 |
| Heath | 992250 | 6.1 | 2.0 | 1.6 |
| Coniferous woodland | 738121 | 4.5 | 3.9 | 3.3 |
| Deciduous woodland | 1033618 | 4.1 | 1.9 | 1.6 |
| Freshwaters | 340453 | 2.9 | 2.5 | 2.2 |
| All ecosystems | 9578231 | 11 | 6.2 | 5.4 |

Table 17.  Exceedence of critical loads for eutrophication across the UK under different scenarios.  Ecosystem area in hectares, exceedences by % of area.

| England | Ecosystem area (ha) | UKREF | H1 | J1 |
| --- | --- | --- | --- | --- |
| Acid grassland | 1126445 | 9.0 | 4.5 | 4.2 |
| Calcareous grassland | 855030 | 0.0 | 0.0 | 0.0 |
| Heath | 139459 | 11 | 5.1 | 4.9 |
| Coniferous woodland | 181039 | 0.6 | 0.1 | 0.1 |
| Deciduous woodland | 647548 | 1.3 | 0.4 | 0.4 |
| All ecosystems | 2949521 | 4.2 | 2.0 | 2.0 |
| Northern Ireland | Ecosystem area (ha) | UKREF | H1 | J1 |
| Acid grassland | 132145 | 0 | 0 | 0 |
| Calcareous grassland | 146958 | 0 | 0 | 0 |
| Heath | 31176 | 0 | 0 | 0 |
| Coniferous woodland | 50629 | 0 | 0 | 0 |
| Deciduous woodland | 26839 | 0 | 0 | 0 |
| All ecosystems | 387747 | 0 | 0 | 0 |
| Scotland | Ecosystem area (ha) | UKREF | H1 | J1 |
| Acid grassland | 3736432 | 0 | 0 | 0 |
| Calcareous grassland | 0 | 0 | 0 | 0 |
| Heath | 709889 | 0 | 0 | 0 |
| Coniferous woodland | 429652 | 0 | 0 | 0 |
| Deciduous woodland | 122452 | 0 | 0 | 0 |
| All ecosystems | 4998425 | 0 | 0 | 0 |
| Wales | Ecosystem area (ha) | UKREF | H1 | J1 |
| Acid grassland | 462379 | 7.7 | 5.5 | 4.7 |
| Calcareous grassland | 14400 | 0.0 | 0.0 | 0.0 |
| Heath | 111726 | 11 | 9.3 | 7.6 |
| Coniferous woodland | 76801 | 2.8 | 1.2 | 1.1 |
| Deciduous woodland | 236779 | 3.6 | 1.1 | 0.7 |
| All ecosystems | 902085 | 6.5 | 4.4 | 3.6 |
| United Kingdom | Ecosystem area (ha) | UKREF | H1 | J1 |
| Acid grassland | 5457401 | 2.5 | 1.4 | 1.3 |
| Calcareous grassland | 1016388 | 0.0 | 0.0 | 0.0 |
| Heath | 992250 | 2.8 | 1.8 | 1.5 |
| Coniferous woodland | 738121 | 0.4 | 0.1 | 0.1 |
| Deciduous woodland | 1033618 | 1.6 | 0.5 | 0.4 |
| All ecosystems | 9237778 | 2.0 | 1.1 | 1.0 |

Table 18.  Exceedence of critical loads for acidification across the UK under different scenarios: Improvement compared to the UKREF scenario.

|  |  | UKREF |  | J1 | H1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Critical loads exceedence by area |  | Reduction in exceedence against UKREF (ha) |  |  |
| England | Total area (ha) | ha | % of total | ha |  | ha |
| Acid grassland | 1126445 | 424663 | 38% | 163476 |  | 140025 |
| Calcareous grassland | 855031 | 0 | 0.0% | 0 |  | 0 |
| Heath | 139460 | 33949 | 24% | 28683 |  | 27132 |
| Coniferous woodland | 181039 | 12535 | 6.9% | 3729 |  | 1374 |
| Deciduous woodland | 647548 | 35947 | 5.6% | 25248 |  | 22044 |
| Freshwater | 142861 | 9495 | 6.6% | 2654 |  | 1599 |
| All Ecosystems | 3092383 | 516588 | 17% | 223791 |  | 192174 |
| Northern Ireland | Total area (ha) | ha | % of total | ha |  | ha |
| Acid grassland | 132145 | 260 | 0.2% | 199 |  | 183 |
| Calcareous grassland | 146958 | 0 | 0.0% | 0 |  | 0 |
| Heath | 31176 | 0 | 0.0% | 0 |  | 0 |
| Coniferous woodland | 50629 | 0 | 0.0% | 0 |  | 0 |
| Deciduous woodland | 26839 | 0 | 0.0% | 0 |  | 0 |
| Freshwater | 0 | 0 | 0.0% | 0 |  | 0 |
| All Ecosystems | 387747 | 260 | 0.1% | 199 |  | 183 |
| Scotland | Total area (ha) | ha | % of total | ha |  | ha |
| Acid grassland | 3736432 | 232950 | 6.2% | 152343 |  | 127628 |
| Calcareous grassland | 0 | 0 | 0.0% | 0 |  | 0 |
| Heath | 709889 | 2387 | 0.3% | 1836 |  | 1729 |
| Coniferous woodland | 429653 | 19192 | 4.5% | 5874 |  | 4199 |
| Deciduous woodland | 122453 | 1705 | 1.4% | 1513 |  | 1239 |
| Freshwater | 171155 | 453 | 0.3% | 123 |  | 102 |
| All Ecosystems | 5169582 | 256687 | 5.0% | 161688 |  | 134897 |
| Wales | Total area (ha) | ha | % of total | ha |  | ha |
| Acid grassland | 462380 | 251480 | 54% | 134942 |  | 115231 |
| Calcareous grassland | 14401 | 0 | 0.0% | 0 |  | 0 |
| Heath | 111727 | 24328 | 22% | 14939 |  | 12502 |
| Coniferous woodland | 76802 | 3228 | 4.2% | 914 |  | 634 |
| Deciduous woodland | 236780 | 8340 | 3.5% | 3413 |  | 2350 |
| Freshwater | 26439 | 700 | 2.6% | 391 |  | 368 |
| All Ecosystems | 928528 | 288077 | 31% | 154599 |  | 131086 |

Table 19.  Exceedence of critical loads for eutrophication across the UK under different scenarios: Improvement compared to the UKREF scenario.

|  |  | UKREF |  | J1 | H1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Critical loads exceedence by area |  | Reduction in exceedence against UKREF (ha) |  |  |
| England | Total area (ha) | ha | % of total | ha |  | ha |
| Acid grassland | 1126445 | 101715 | 9.0% | 53803 |  | 51440 |
| Calcareous grassland | 855031 | 0 | 0.0% | 0 |  | 0 |
| Heath | 139460 | 14981 | 11% | 8180 |  | 7867 |
| Coniferous woodland | 181039 | 1020 | 0.6% | 762 |  | 752 |
| Deciduous woodland | 647103 | 8591 | 1.3% | 5963 |  | 5948 |
| All Ecosystems | 2949078 | 126308 | 4.3% | 68708 |  | 66005 |
| Northern Ireland | Total area (ha) | ha | % of total | ha |  | ha |
| Acid grassland | 132064 | 0 | 0.0% | 0 |  | 0 |
| Calcareous grassland | 146958 | 0 | 0.0% | 0 |  | 0 |
| Heath | 31176 | 0 | 0.0% | 0 |  | 0 |
| Coniferous woodland | 50629 | 0 | 0.0% | 1 |  | 0 |
| Deciduous woodland | 26839 | 0 | 0.0% | 0 |  | 0 |
| All Ecosystems | 387666 | 0 | 0.0% | 0 |  | 0 |
| Scotland | Total area (ha) | ha | % of total | ha |  | ha |
| Acid grassland | 3736432 | 0 | 0.0% | 0 |  | 0 |
| Calcareous grassland | 0 | 0 | 0.0% | 0 |  | 0 |
| Heath | 709889 | 0 | 0.0% | 0 |  | 0 |
| Coniferous woodland | 429653 | 0 | 0.0% | 0 |  | 0 |
| Deciduous woodland | 122453 | 0 | 0.0% | 0 |  | 0 |
| All Ecosystems | 4998427 | 0 | 0.0% | 0 |  | 0 |
| Wales | Total area (ha) | ha | % of total | ha |  | ha |
| Acid grassland | 462380 | 35540 | 7.7% | 13749 |  | 10002 |
| Calcareous grassland | 14401 | 0 | 0.0% | 0 |  | 0 |
| Heath | 111727 | 12046 | 11% | 3535 |  | 1654 |
| Coniferous woodland | 76632 | 2181 | 2.8% | 1348 |  | 1283 |
| Deciduous woodland | 236728 | 8584 | 3.6% | 6860 |  | 5906 |
| All Ecosystems | 901867 | 58352 | 6.5% | 25492 |  | 18845 |

Figure 4.  Specimen maps showing the percentage change in area under critical loads exceedence for acidification between the UKREF and J1 scenarios.

## Results for impacts on crops, materials, mortality and respiratory hospital admissions

Results for the set of impacts quantified by the IGCB in its review of the NAQS are shown in Table 20.  Results disaggregated by country are shown in Table 21.  There are some apparent inconsistencies in the Table with respect to crops and materials damage, comparing, for example, damages in the UK caused by UK and full-UNECE emissions respectively.  These arise through the use of different models of pollutant chemistry and dispersion for analysis at different scales, and (in the case of the analysis of impacts on crops) as a result of the balance between positive and negative effects of reducing emissions of SO2 and nitrogenous pollutants.  They do not, however, greatly affect the outcome of the comparison of costs and benefits.  Results for the health impacts are based on best estimates of exposure-response functions.  Results are given in Appendix 8 for the range in functions provided by COMEAP/EAHEAP.

Table 20.  Reductions in pollution damages to crops and materials (in terms of monetary benefit), and from acute exposures on premature mortality and respiratory hospital admissions (RHAs) through movement from UKREF to each scenario.  Units shown in left hand column.

| Effect | Scenario | UK to UK | UK to | UNECE (including UK) |
| --- | --- | --- | --- | --- |
|  |  |  | UNECE | to UK |
| Crop damage | WGS31c | 6 | 10 | nq |
| (£ million) | J1 | 14 | 20 | 10 |
|  | H1 | 25 | 40 | 17 |
| Materials damage | WGS31c | 2 | 3 | 5 |
| (£ million) | J1 | 4 | 4 | 15 |
|  | H1 | 4 | 4 | 15 |
| Premature | WGS31c | 160 | 350 | 160 |
| mortality(cases) | J1 | 330 | 700 | 540 |
|  | H1 | 380 | 880 | 530 |
| RHAs, | WGS31c | 90 | 240 | 110 |
| additional/ | J1 | 190 | 530 | 350 |
| brought forward | H1 | 240 | 760 | 377 |

The results for crop damages shown in Table 20 imply a lower level of benefit to crops in the UK from abatement across the UNECE than from abatement in the UK alone.  This is a function of the complexity of the assessment of crop damage, which brings together effects that are both damaging to crops (i.e. direct effects of ozone, acidification) and effects that are potentially beneficial (fertilisation with sulphur and nitrogen).  For other receptors the issue of potential benefit from increased exposure does not arise.

Results for the health effects are broken down by pollutant (including the most important secondary species) for each region of the UK in Table 22 for the J1 scenario.  For effects on crops, materials, and, for health for SO2 and the secondary aerosols, the split by country follows the detailed mapped analysis.  The split for ozone effects on health has been made by extrapolation from the results for crops.  Benefits are concentrated in England, partly through the population distribution and partly through proximity to mainland Europe.  Benefits are spread fairly evenly across the pollutants listed, though ammonium aerosols play a lesser role than the other pollutants.

Table 21.  Regional breakdown of reductions in pollution damages to crops and materials (in terms of monetary benefit), and from acute exposures on premature mortality and respiratory hospital admissions (RHAs) through movement from UKREF to each scenario, for abatement across UNECE. Units shown in left hand column.  Results were not available for scenario WGS31c.

| Effect | Scenario | England | Northern Ireland | Scotland | Wales |
| --- | --- | --- | --- | --- | --- |
| Crop damage | J1 | 10 | 0 | -0.9 | 1.1 |
| (£ million) | H1 | 16 | 0 | -0.7 | 1.5 |
| Materials damage | J1 | 13 | 0.5 | 0.7 | 0.8 |
| (£ million) | H1 | 13 | 0.5 | 0.7 | 0.8 |
| Premature | J1 | 490 | 6 | 18 | 23 |
| mortality(cases) | H1 | 480 | 5 | 14 | 23 |
| RHAs, | J1 | 330 | 2 | 8 | 14 |
| additional/ | H1 | 350 | 2 | 7 | 19 |
| brought forward |  |  |  |  |  |

Some discussion of the health effects is needed to aid interpretation.  Firstly, the extent to which short term (acute) exposures affect mortality: clinical judgement suggests that those at risk are already ill, probably seriously, and as such are likely to have only a very limited life expectancy.  For respiratory hospital admissions it is uncertain to what extent the results show additional cases, and to what extent they are simply cases that would have occurred anyway within a limited period.  Again, clinical judgement suggests that those affected are not in the prime of health.  These factors clearly complicate valuation of health effects.

A further factor relates to uncertainty over the mechanism of pollutant action on health.  Knowledge of mechanisms would undoubtedly improve the quality of assessment for all of the health-damaging pollutants for which effects are quantified here.  Relative to mechanism, the following caveats should be drawn against the health assessment in this study:

- all fractions of PM10 are assumed to be equally potent;

- ozone effects are quantified only for the case where a threshold of 50 ppb is assumed.  The figure of 50 ppb follows the analysis carried out for Department of Health (1998), rather than the 60 ppb preferred by WHO.  A lack of data on average ozone levels (rather than peaks) prevented assessment of effects assuming that there is no threshold.

Potential effects of VOCs and NO2 have also been omitted from the analysis.  For VOCs the inventory would need to be speciated before any assessment of health effects could be undertaken.  Even then, exposure-response data tends to be lacking.  For NO2 the evidence for health effects from epidemiological studies is regarded here to be too inconsistent.  It could also generate a risk of double counting if exposure-response functions are not adequately characterised and the perceived effects of NO2 are, in reality, attributable to other pollutants.

On the other hand, there is evidence for a number of additional effects on health from exposure to air pollution.  These effects are explored below, and include hospital admissions for heart disease and effects on asthmatics.

Table 22.  % split between pollutants with respect to health effects.  The reference point for this assessment is the J1 scenario, mid estimate of acute effects on mortality.  Ozone effects calculated with threshold only.

| - Pollutant | - England | N. Ireland | Scotland | Wales | UK |
| --- | --- | --- | --- | --- | --- |
| NO3 | 17% | 0.2% | 0.7% | 0.7% | 18% |
| NH4 | 5% | 0.0% | 0.3% | 0.2% | 6% |
| SO4 | 17% | 0.4% | 1.3% | 0.9% | 1% |
| SO2 | 28% | 0.3% | 1.0% | 1.1% | 30% |
| Ozone | 23% | 0.0% | 0.1% | 1% | 26% |
| Overall | 90% | 1% | 3% | 4% | 81% |

## Application of additional exposure-response functions identified by COMEAP

The COMEAP and EAHEAP reports included a number of exposure-response functions for effects of fine particles, additional to those reported above.  However, quantification of these effects was considered to be less certain on grounds of:

- evidence being based on a relatively small number of studies;

- lack of data specific to the UK;

- consistency in outcomes of different studies; and

- availability of appropriate baseline data for incidence of effects.

However, given that the functions identified were based on the results of well-conducted studies, it is appropriate, in the context of this analysis, to accept them (whilst recognising the uncertainties that are present) in order to achieve a holistic overview of the effects of emissions abatement.

Results are given in Table 23 and show the number of cases either additional or brought forward, rather than the additional number of people affected.  Again, results are based on the use of best estimates of exposure-response functions.

Results in the Table for cardiovascular disease are not additional to those for ischaemic heart disease and congestive heart failure.  However, it is notable that both sets of functions indicate broadly similar totals.  Results for cerebrovascular hospital admissions (stroke) are, however, additional to both groups.

The following effects were quantified for asthmatics:

- bronchodilator usage

- cough

- wheeze

Of these effects the most objective are undoubtedly changes in the frequency of use of bronchodilators.  There is some possibility that adding together all three effects could lead to double counting (e.g. use of a bronchodilator to relieve other symptoms).  Accordingly, for a lower estimate of the number of symptoms, bronchodilator usage only is taken, whilst the upper estimate adds the number of all three types of effect together.  The difference is within a factor of 2 for all scenarios.  The results raise questions on which effects might be considered to be the most serious: effects on heart disease and stroke, because of severity at the level of the individual sufferer, or effects on asthmatics as a consequence of the much larger number of people affected.

Table 23.  Effects of secondary aerosols on heart disease, cardiovascular disease and asthmatics.

| Effect | Scenario | UK to UK | UK to | UNECE (including UK) |
| --- | --- | --- | --- | --- |
|  |  |  | UNECE | to UK |
| Cardiovascular | WGS31c |  |  | nq |
| disease in the | J1 | 85 | 150 | 140 |
| elderly (hospital | H1 | 86 | 150 | 120 |
| admissions) |  |  |  |  |
| Congestive heart | WGS31c |  |  | nq |
| failure (hospital | J1 | 50 | 88 | 80 |
| admissions) in | H1 | 50 | 89 | 67 |
| the elderly |  |  |  |  |
| Ischaemic heart | WGS31c |  |  | nq |
| disease (hospital | J1 | 47 | 83 | 75 |
| admissions) in | H1 | 47 | 84 | 63 |
| the elderly |  |  |  |  |
| Cerebrovascular | WGS31c |  |  |  |
| hospital | J1 | 50 | 88 | 80 |
| admissions | H1 | 50 | 89 | 67 |
| (all ages) |  |  |  |  |
| Bronchodilator | WGS31c |  |  | nq |
| usage in children | J1 | 155,000 | 280,000 | 250,000 |
| and adults | H1 | 160,000 | 280,000 | 210,000 |
| (person days) |  |  |  |  |
| Cough in | WGS31c |  |  | nq |
| asthmatic adults | J1 | 180,000 | 320,000 | 290,000 |
| and children | H1 | 180,000 | 320,000 | 240,000 |
| (person days) |  |  |  |  |
| Wheeze in | WGS31c |  |  | nq |
| asthmatic adults | J1 | 86,000 | 150,000 | 140,000 |
| and children | H1 | 86,000 | 150,000 | 120,000 |
| (person days) |  |  |  |  |

## Application of EAHEAP Valuations

Assessment from this point goes beyond that accepted by the Department of Health.  However, it can be justified on the grounds of seeing how total benefits may compare to costs.  Valuation of acute effects on mortality and respiratory hospital admissions is presented in Table 24.  The analysis combines the following data to provide ranges:

- low estimate of deaths brought forward through acute effects on mortality with the lower EAHEAP valuation (£2,600/case);

- mid estimate of deaths brought forward with the intermediate EAHEAP valuation (£110,000/case);

- high estimate of deaths brought forward with the upper EAHEAP valuation (£1.4 million/case).

All values used in the analysis were expressed in 1990£ rather than the 1998£ used by EAHEAP and cited here, to enable comparison with the costs of abatement.

For the lower bound the benefits of reduced mortality from acute exposure are minimal, less than 1% of costs.  Using the upper bound leads to benefits of reducing this one type of impact to exceed costs in all scenarios.  Using the intermediate position, benefits from reducing acute effects on mortality account for between 5 and 53% of costs, depending on scenario and the range of analysis.

Results for the intermediate valuation of mortality cases should not be regarded as a best estimate simply because they are intermediate.  Based on the EAHEAP report, there is no reason for preferring any one of the three positions on valuation there presented to the other two.

For valuation of respiratory hospital admissions ranges were derived as follows:

- low estimate of the number of hospital admissions combined with a valuation of £0/case, on the assumption that all cases are simply brought forward by a limited amount of time;

- high estimate of the number of hospital admissions combined with the upper EAHEAP valuation for RHAs of £3235/case (adding together the upper figures given by EAHEAP for willingness to pay [£735] and for NHS cost savings [£2,500]).  This upper estimate is based on the opposite assumption to the lower estimate: that all cases are additional, rather than simply ‘brought forward’.

Results show that the benefits from reducing RHAs are low compared to the costs of pollution abatement, making at most a 1 to 2% contribution for the scenarios studied here.

Sensitivity analysis has also been applied to test how large the value of statistical life (VOSL) would need to be for benefits to match costs, considering only benefits data for materials and crops from Table 20 and premature cases of mortality.  Given the exclusion of numerous other effects this sets a maximum for the required VOSL, which can then be compared against the EAHEAP data.  Analysis of so-called ‘switching values’ is indeed recommended in DETR guidance on conducting regulatory impact assessment.  The analysis has looked at both extremes.  First the low estimate of benefits was combined with the upper estimate of costs to give a maximum value.  Then the high estimate of benefits was combined with low costs to give a minimum value (subject to the constraint that only effects on crops, materials and acute effects on mortality are included: on this basis the value derived is clearly not a true minimum).  Results are given in Table 25.  In almost all cases the result lies between the intermediate and upper estimate from EAHEAP.

Table 24.  Valuation of acute effects on mortality and respiratory hospital admissions.

| Effect | Scenario |  | UK to UK | UK to | UNECE (including UK) |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | UNECE | to UK |
| Acute effects on |  | WGS31c |  |  | 0.21 |
| mortality) |  | J1 | 0.46 | 0.89 | 0.72 |
| (£ million) |  | H1 | 0.49 | 0.99 | 0.66 |
| Lower bound |  |  |  |  |  |
| Acute effects on |  | WGS31c |  |  | 12 |
| mortality |  | J1 | 26 | 54 | 42 |
| (£ million) |  | H1 | 29 | 69 | 41 |
| Intermediate |  |  |  |  |  |
| Acute effects on |  | WGS31c |  |  | 190 |
| mortality |  | J1 | 390 | 840 | 640 |
| (£ million) |  | H1 | 450 | 1,100 | 630 |
| Upper bound |  |  |  |  |  |
| RHAs (£million) |  | WGS31c | 0 | 0 | 0 |
| Lower bound |  | J1 | 0 | 0 | 0 |
|  |  | H1 | 0 | 0 | 0 |
| RHAs (£million) |  | WGS31c |  |  | 0.4 |
| Upper bound |  | J1 | 0.70 | 2.0 | 1.3 |
|  |  | H1 | 0.91 | 2.9 | 1.4 |

Table 25.  Determination of ‘switching values’ for the VOSL, including only acute effects on mortality and effects on agriculture and materials.  All values in 1990£.

|  |  | UK to UK | UK to UNECE | UNECE to UK |
| --- | --- | --- | --- | --- |
|  | PART 1: Upper estimates |  |  |  |
|  | Scenario: J1 |  |  |  |
| I | Acute deaths (cases, lower bound) | 253 | 484 | 394 |
| II | Crop + material damage (£M) | 16 | 25 | 25 |
| III | Costs of abatement for UK (£M) | 161 | 161 | 161 |
| IV | Residual cost (III – II) (£M) | 145 | 136 | 136 |
| V | VOSL – switching value/case (IV/I) (£) | 570,000 | 280,000 | 350,000 |
|  | Scenario: H1 |  |  |  |
| I | Acute deaths (cases, lower bound) | 266 | 541 | 357 |
| II | Crop + material damage (£M) | 28 | 45 | 33 |
| III | Costs of abatement for UK (£M) | 567 | 567 | 567 |
| IV | Residual cost (III – II) (£M) | 539 | 522 | 534 |
| V | VOSL – switching value/case (IV/I) (£) | 2,000,000 | 960,000 | 1,500,000 |
|  | PART 2: Lower estimates |  |  |  |
|  | Scenario: J1 |  |  |  |
| I | Acute deaths (cases, upper bound) | 393 | 848 | 644 |
| II | Crop + material damage (£M) | 16 | 25 | 25 |
| III | Costs of abatement for UK (£M) | 148 | 148 | 148 |
| IV | Residual cost (III – II) (£M) | 132 | 123 | 123 |
| V | VOSL – switching value/case (IV/I) (£) | 340,000 | 150,000 | 190,000 |
|  | Scenario: H1 |  |  |  |
| I | Acute deaths (cases, upper bound) | 449 | 1097 | 637 |
| II | Crop + material damage (£M) | 28 | 45 | 33 |
| III | Costs of abatement for UK (£M) | 554 | 554 | 554 |
| IV | Residual cost (III – II) (£M) | 526 | 509 | 521 |
| V | VOSL – switching value/case (IV/I) (£) | 1,200,000 | 460,000 | 820,000 |

## Valuation of additional effects for which COMEAP provided functions

The results from Section 6.4 were valued using data from the European Commission ExternE Project.  Results made only a minor contribution to total benefits (between £1 million and £3 million in total, depending on scenario and range considered).  Because the contribution compared to costs was small the results are not presented separately here, but are given in Appendix 8.

## Chronic effects on mortality

The COMEAP report (paragraphs 3.49 to 3.50) notes that quantification of chronic effects on health is prone to a high degree of uncertainty.  However, they also note that, if available data are reliable, then the overall impacts on health are likely to be substantially greater than estimates that ignore chronic effects on mortality.  Analysis in the Netherlands suggests a reduction in life expectancy amongst men of about one year on average, as a consequence of exposure to particle levels that are typical of the UK (Brunekreef, 1997).  Here quantification of chronic exposure effects on mortality is based on the results of earlier work in the UK by Hurley et al, under the EC GARP II Project (Markandya et al, 1999).  The assumptions underlying this part of the assessment are described in Appendix 6.

The outputs of the quantification of these chronic effects are estimates of the reduction in longevity (life years lost) spread across the population, as a result of long-term pollution exposure.  Results are, therefore, not in the same units as those for acute effects on mortality, which relate solely to change in the number of cases of death brought forward in each scenario.  It would, in theory, be possible to express results in terms of the number of deaths brought forward for a specified time period, though this would require additional analysis beyond the scope of this study.

EAHEAP did not specifically consider valuation of chronic effects on mortality.  However, some of their results did, in effect, quantify Willingness to Pay (WTP) against change in life expectancy related to air pollution exposure.  Their lower valuation (£2,600) was based on the loss of one month of life amongst the elderly, for those with a much reduced quality of life, whilst the intermediate valuation (£110,000) was based on the loss of a year of life for those in reasonable health.  If one accepts the EAHEAP approach as being broadly correct, it is likely that one would wish to introduce additional factors to generate a VOSL or range specific to chronic effects on mortality.  For the upper estimate £110,000 per life year lost was used.  For the low estimate the figure of £2,600 was multiplied by 12 to give £31,200 per year.  Given that this part of the analysis seeks to value an effect that will happen after a long period of exposure, it may be appropriate to discount effects over a number of years.  Accordingly, the £31,200 figure has been scaled back in accordance with earlier analysis under the ExternE Project (European Commission, 1999) to give a figure of £19,000 (1990£).  Doubtless, there would be debate about whether this is applicable or not, and whether additional factors should be introduced.  However, this analysis is not about generating specific data but, instead (given a lack of confidence in much of the data that are available) about testing alternative assumptions and investigating ranges.  Given the broad range applied here, and the outcome of the analysis, any final figure would not be so different as to significantly affect the results.  It could be argued that the upper estimate given here is too conservative – that the real figure could well be higher.  However, this is of limited (though still some) relevance to the present analysis, given the extent by which benefits exceed costs (see below) when the upper estimate is taken.

Results are shown in Table 26.  They are illustrative for a given set of assumptions and alternative, and equally plausible, assumptions are possible.  The ranges shown are therefore unlikely to represent the full range of possible answers, though broadening the range would have only a limited effect on the outcome of the comparison of costs and benefits.

Table 26.  Chronic effects on mortality.

|  | Scenario | UK to UK | UK to UNECE | UNECE (incl. UK) to UK |
| --- | --- | --- | --- | --- |
| Life years lost | WGS31c |  |  | 1,800-2,300 |
|  | J1 | 3,200-4,200 | 5,700-7,500 | 5,300-6,900 |
|  | H1 | 3,200-4,200 | 5,800-7,600 | 4,400-5,800 |
| £ million | WGS31c |  |  | 37-270 |
|  | J1 | 43-330 | 77-580 | 110-800 |
|  | H1 | 44-330 | 78-590 | 89-670 |

Clearly, these results would make a major difference to the analysis if accepted.  The upper end of the ranges shown for economic benefit would be sufficient on their own to more than match costs for all scenarios.

## Application of other Functions and valuations used in the studies for UNECE and the EC

A series of other effects are quantified and monetised in Appendix 8, including:

- ozone damage to forests;

- asthma attacks induced by ozone exposure;

- incidence of chronic bronchitis in adults;

- incidence of bronchitis in children;

- incidence of chronic cough;

- restricted activity days linked to exposure to secondary particles;

- minor restricted activity days linked to ozone exposure.

Quantification of these effects can be done with very limited confidence.  However, where possible, broad ranges have been applied to give an indication of the possible scale of benefits.  These ranges are used in the comparison of costs and benefits that follows.

## Comparison of Costs and Benefits

This chapter and Appendix 8 are structured to show readers what effects can be considered quantified with the highest confidence, and which with the least.  Appendix 8 includes running totals for monetised effects, enabling identification of the point at which benefits outweigh costs (if at all).  These results are summarised here (Table 27).  The lower bound contains conservative estimates of the benefits to be derived from reducing emissions.  This includes taking the lower bound for exposure-response functions and valuations, and elimination of any effects that might be double counted, though this transpires to be of very little importance.  The upper bound is naturally based on upper estimates for exposure-response functions and valuations, and also a slightly less cautious approach on the potential for double counting.  Again, this last point is of little importance.  For effects where double counting is probable precaution is applied to both the upper and lower bounds in the running totals.  Only a few relatively minor effects of those quantified are in the grey area where it is not clear whether or not their inclusion would lead to double counting.

Table 27.  Cost-benefit analysis: effects required under each scenario for benefits to outweigh costs.

|  | UK to UK | UK to UNECE | UNECE (incl. UK) to UK |
| --- | --- | --- | --- |
| - WGS31c |  |  |  |
| Lower bound | Not quantified | Not quantified | Not quantified |
| Upper bound | Not quantified | Not quantified | Not quantified |
| - J1 |  |  |  |
| Lower bound | Costs exceed sum of all quantified benefits | Costs exceed sum of all quantified benefits | Costs exceed sum of all quantified benefits |
| Upper bound | Crops Materials Acute mortality (NO3, SO4, NH4 only) | Crops Materials Acute mortality (NO3, SO4only) | Crops Materials Acute mortality (NO3, SO4only) |
| - H1 |  |  |  |
| Lower bound | Costs exceed sum of all quantified benefits | Costs exceed sum of all quantified benefits | Costs exceed sum of all quantified benefits |
| Upper bound | Crops Materials Acute mortality Acute RHAs Heart disease Asthma Chronic mortality (SO4 only) | Crops Materials Acute mortality (NO3, SO4, NH4, SO2 only) | Crops Materials Acute mortality (NO3, SO4, NH4, SO2 only) |

Note: Health effects are quantified against exposure-response functions for PM10.  There is no direct evidence of effects of nitrate aerosols from epidemiological studies, though some studies have looked specifically at associations with sulphates.

Table 27 shows that costs lie somewhere in the range calculated for the total benefits.  At first sight this does not appear to be particularly useful.  After all, the analysis started from the position of wanting to know for any scenario whether or not costs were likely to be bigger than benefits!  However, the ranges selected for each variable were very broad, seeking to quantify the full potential range in benefits.  The next stage of the assessment investigates benefit-cost ratios.

Table 28. Benefit-cost ratios, with all monetised effects taken into account (excludes damage to cultural heritage and natural ecosystems).  Positive numbers denote benefits in excess of costs by the factors quoted.  Negative numbers denote costs in excess of quantified benefits by the factors quoted.  Columns identify the source of emission and the receptor for benefits of abatement (in both cases this is either UK or full UNECE).  Costs applied are the costs to the UK in all cases.

|  | UK to UK | UK to UNECE | UNECE (incl. UK) to UK |
| --- | --- | --- | --- |
| - J1 (cost £161M) |  |  |  |
| Lower bound | -2.38 | -1.35 | -1.11 |
| Upper bound | 4.01 | 8.04 | 8.20 |
| - H1 (cost £567M) |  |  |  |
| Lower bound | -7.14 | -3.85 | -4.17 |
| Upper bound | 1.26 | 2.77 | 2.15 |
| - WGS31c (cost £61M) |  |  |  |
| Lower bound | Not quantified | Not quantified | Not quantified |
| Upper bound | Not quantified | Not quantified | Not quantified |

The results shown in Table 28 demonstrate where estimated costs lie between the upper and lower bounds for benefits.  Costs for J1 tend to be towards the lower end of the range, in other words, there appears a reasonable likelihood that the real benefit would exceed the estimated costs.  This is not the case for H1, however, where the bias is the other way round, with costs towards the upper end of the benefits range.

Part 4
Conclusions

# Conclusions

Earlier analyses of the costs of reducing UK emissions of SO2, NOx, VOCs and NH3 have been carried out by IIASA using the RAINS model.  The estimated costs of meeting the J1 and H1 scenario emissions were £447M and £899M, respectively.  This study has aimed to produce a more detailed analysis by using the latest available datasets and specifically UK-based models, with much higher spatial resolution.  The costs of the J1 and H1 scenarios have been estimated in this study to be £145-158M and £516-529M respectively, with the range being due to uncertainties in ammonia abatement costs (in particular whether or not urea substitution will be allowed).  In addition, the cost of meeting the emissions ceilings for the UK negotiated at the 31st Session of the UNECE Working Group on Strategies, on 2nd September 1999, has been estimated to be £61M.

The regional splits of costs for each scenario are shown in Table 29.  The uncertainty with respect to ammonia abatement measures has a substantial influence on the costs in Northern Ireland and Wales under the J1 scenario.

Table 29.  Costs by region (£M/year).

| - Scenario | England | N. Ireland | Scotland | Wales | Offshore |
| --- | --- | --- | --- | --- | --- |
| WGS31c | 41 | 1 | 8 | 3 | 7 |
| J1 | 108 – 116 | 4 – 6 | 16 – 18 | 7 – 9 | 8 |
| H1 | 339 - 347 | 11 – 13 | 68 - 71 | 19 - 21 | 77 |

The split in costs between the different pollutants is shown in Table 30.  In terms of overall cost, the role played by NOx and NH3 abatement is relatively small.  The costs of VOC abatement become more significant as a fraction of the total as one goes down the table, and provide more than 80% of the costs under the H1 scenario (compared to only 42% under J1, and about 27% for WGS311).

Table 30.  Costs by pollutant (£M/year).

| - Scenario | SO2 | NOx | VOC | NH3* | Total |
| --- | --- | --- | --- | --- | --- |
| WGS31c | 34 | 9 | 18 | 0 | 61 |
| J1 | 61 | 9 | 64 | 11 – 24 | 145 – 158 |
| H1 | 61 | 9 | 435 | 11 - 24 | 516 – 529 |

*Abatement costs for ammonia emissions are shown as a range because of uncertainty about the acceptability of urea substitution, the higher end of the range assuming that it is not adopted.

As noted in earlier work, for example the review of the NAQS, there are many benefits linked to abatement of the four pollutants considered here.  These benefits accrue to health, natural ecosystems, buildings, crops, forests and so on.  The key indicators of benefit are summarised below, these being the effects that can be described with most confidence.

Table 31.  Key benefits of each scenario, with results showing the total annual benefit to the UK of abatement across the UNECE region (except for H1, where only EU countries participate).  All monetised values in 1990£/year.

|  | 1990 | UKREF | J1 | H1 | WGS31c |
| --- | --- | --- | --- | --- | --- |
| Ecosystem area (%) subject to critical load exceedence for: |  |  |  |  |  |
| Acidification | 43% | 11% | 5.4% | 6.2% | 9.2% |
| Eutrophication | 11.2% | 2.0% | 1.0% | 1.1% | 1.3% |
| Change from UKREF in damage to: |  |  |  |  |  |
| Crops (£million) |  |  | £10M | £17M |  |
| Materials (£million) |  |  | £15M | £15M | £5M |
| Premature mortality (cases) |  |  | 540 | 530 | 160 |
| Respiratory hospital admissions |  |  | 350 | 380 | 110 |

This takes the analysis to the same point as the assessment carried out for the NAQS.  However, it is possible to go further, although subsequent analysis is subject to a higher level of uncertainty.  Based on these results, and uncertainty with respect to the valuation of acute effects of air pollution on mortality, it is logical to ask how large the value of statistical life (the unit value for a case of mortality) would need to be for benefits overall to match costs.  Considering only acute effects on mortality, damage to crops and buildings, the following results arise, all expressed in 1990£:

Scenario J1: £190,000 to £350,000

Scenario H1: £820,000 to £1,500,000

Results for H1 are similar to the upper point identified by EAHEAP, and for J1 in-between the EAHEAP upper and intermediate points

Ranges were ascribed to the key variables in the assessment, leading to a broad range in estimated benefits.  Costs tended to lie between the upper and lower bounds for benefits – a clearly inconclusive result.  However, assessment of benefit-cost ratios suggests that costs for J1 tend to be towards the lower end of the range, in other words, there appears a reasonable likelihood that the real benefit would exceed the estimated costs.  This is not the case for H1, however, where the bias is the other way round, with costs towards the upper end of the benefits range.  It should be remembered that there are a number of benefits that have not been quantified in monetary terms, and hence are outside this part of the analysis – including damage to ecosystems and cultural heritage (cathedrals, etc.).

The overall view on an appropriate position for the UK to take in negotiation is clearly a political question, particularly given the uncertainties identified in this report.  Our role is not to pre-empt any political decision by (for example) introducing our own interpretation of the precautionary principle.  It is hoped that the methods developed here for presenting information on uncertain impacts will prove useful, and provide real assistance to decision makers.

- PART 5
Appendices

Appendix 1: Abbreviations and terminology

Appendix 2: Scenario data from RAINS

Appendix 3: Monitoring requirements for the Ozone Directive

Appendix 4: Analysis using HARM and ELMO

Appendix 5: Analysis using RAINS and ASAM

Appendix 6: Methods for the analysis of benefits of abatement

Appendix 7: Detailed results for acidification and eutrophication

Appendix 8: Full results of the benefit analysis

Appendix 9: References

- Appendix 1
Abbreviations and Terminology

| AOT40 | Accumulated concentration of ozone over a threshold of 40 ppb |
| --- | --- |
| AOT60 | Accumulated concentration of ozone over a threshold of 60 ppb |
| CBA | Cost-benefit analysis |
| CLE | Current legislation scenario |
| CLRTAP | Convention on Long Range Transboundary Air Pollution |
| CRP | Current reduction plan scenario |
| CV | Contingent valuation |
| DETR | UK Department of the Environment, Transport and the Regions |
| DNMI | Norwegian Meteorological Institute |
| DTI | UK Department of Trade and Industry |
| EC | European Commission |
| ELMO | Edinburgh/Lancaster Model for Ozone |
| EU | European Union |
| HARM | Hull Acid Rain Model |
| IIASA | International Institute for Applied Systems Analysis |
| IOM | Institute of Occupational Medicine |
| ITE | Institute of Terrestrial Ecology |
| kt | Thousand tonnes |
| MFR | Maximum feasible reduction scenario |
| NH3 | Ammonia |
| NO | Nitrogen monoxide |
| NO2 | Nitrogen dioxide |
| NOx | Oxides of nitrogen (NO and NO2, but not N2O) |
| O3 | Ozone |
| PM10 | Fine particles less than 10 µm in diameter |
| PM2.5 | Fine particles less than 2.5 µm in diameter |
| IIASA Reference | IIASA reference scenario |
| SO2 | Sulphur dioxide |
| TFEAAS | Task Force on Economic Aspects of Abatement Strategies |
| UNECE | United Nations Economic Commission for Europe |
| VOCs | Volatile organic compounds |
| VOLY | Value of life year |
| VOSL | Value of statistical life |
| WTA | Willingness to accept |
| WTP | Willingness to pay |
| YOLL | Years of life lost |

- mathematical notation

The following prefixes and suffixes are used in this work;

Ex, E-x as a suffix to a number, denotes that the number in question should be multiplied by 10 to the power x or -x.  Hence 6.4E-3 is equal to 0.0064.

The following prefixes to units are also used;

n = nano = 10-9

µ or u = micro = 10-6

m = milli = 10-3

k = kilo = 103 = thousands

M = mega = 106 = millions

G = giga = 109 = billions

This system is standard notation in the sciences.  Note that m and M are not equivalent (by a factor of 109) and hence should not be interchanged.

- Appendix 2
Scenario data from RAINS

This appendix provides details of the emissions and costs of abatement in all European countries, according to the analysis carried out at IIASA using the RAINS model (Amann et al, 1999a, b).  Details of the exact scenarios assessed in the current study are provided in the main text of this report.

Scenarios 1990 and IIASA Reference were used for both the UNECE and EU analyses.  H1 and J1 were the final scenarios selected for consideration at negotiation on the EU Directive and UNECE Protocol respectively.  For the analysis presented here there is some inconsistency between the H1 and J1 scenarios.  H1 includes abatement in the EU15 alone, whilst J1 includes additional abatement in most UNECE countries.  The net result of this is that H1 is more expensive for the EU15, though provides a lower total abatement for the UNECE.  This, combined with the trans-boundary nature of the pollutants of interest, means that the two scenarios are not strictly comparable when considering the comparison of total benefits in each country with the national costs data.

Table A2.1 Annual emissions of SO2 (kt) in each scenario in 2010 (except 1990).

|  | 1990 | IIASA Reference | H1 | J1 | WGS31c |
| --- | --- | --- | --- | --- | --- |
| Austria | 93 | 40 | 40 | 35 | 39 |
| Belgium | 336 | 193 | 76 | 76 | 106 |
| Denmark | 182 | 90 | 77 | 60 | 55 |
| Finland | 226 | 116 | 116 | 116 | 116 |
| France | 1250 | 448 | 218 | 219 | 400 |
| Germany | 5280 | 581 | 463 | 463 | 550 |
| Greece | 504 | 546 | 546 | 546 | 546 |
| Ireland | 178 | 66 | 28 | 36 | 42 |
| Italy | 1679 | 567 | 566 | 290 | 500 |
| Luxembourg | 14 | 4 | 3 | 3 | 4 |
| Netherlands | 201 | 73 | 50 | 50 | 50 |
| Portugal | 284 | 141 | 141 | 141 | 170 |
| Spain | 2189 | 774 | 746 | 747 | 774 |
| Sweden | 119 | 67 | 67 | 67 | 67 |
| UK | 3805 | 980 | 497 | 499 | 625 |
| EU-15 | 16339 | 4687 | 3637 | 3349 | 4044 |
| Albania | 72 | 55 | 55 | 55 | 55 |
| Belarus | 843 | 494 | 494 | 494 | 480 |
| Bosnia-H | 487 | 415 | 415 | 162 | 415 |
| Bulgaria | 1842 | 846 | 846 | 378 | 856 |
| Croatia | 180 | 70 | 70 | 23 | 70 |
| Czech_R. | 1873 | 366 | 366 | 283 | 283 |
| Estonia | 275 | 175 | 175 | 175 | 175 |
| Hungary | 913 | 546 | 546 | 296 | 550 |
| Latvia | 121 | 104 | 104 | 104 | 107 |
| Lithuania | 213 | 107 | 107 | 107 | 145 |
| Norway | 52 | 32 | 32 | 18 | 22 |
| Poland | 3001 | 1397 | 1397 | 722 | 1397 |
| Moldova | 197 | 117 | 117 | 38 | 135 |
| Romania | 1331 | 594 | 594 | 148 | 918 |
| Russia | 5012 | 2344 | 2344 | 2186 | 2352 |
| Slovakia | 548 | 137 | 137 | 92 | 110 |
| Slovenia | 200 | 71 | 71 | 14 | 27 |
| Switzerland | 43 | 26 | 26 | 23 | 26 |
| Macedonia | 107 | 81 | 81 | 81 | 81 |
| Ukraine | 3706 | 1488 | 1488 | 1457 | 1457 |
| Yugoslavia | 585 | 269 | 269 | 217 | 269 |
| Non-EU | 21599 | 9732 | 9732 | 7071 | 9930 |
| SEA | 1152 | 1152 | 1152 | 1152 | 1152 |
| TOTAL | 39090 | 15571 | 14521 | 11572 | 15126 |

Table A2.2. Annual emissions of NOX (kt) in each scenario in 2010 (except 1990).

|  | 1990 | IIASA Reference | H1 | J1 | WGS31c |
| --- | --- | --- | --- | --- | --- |
| Austria | 192 | 103 | 91 | 91 | 107 |
| Belgium | 351 | 191 | 127 | 127 | 181 |
| Denmark | 274 | 128 | 127 | 113 | 127 |
| Finland | 276 | 152 | 152 | 152 | 170 |
| France | 1867 | 858 | 679 | 704 | 860 |
| Germany | 2662 | 1184 | 1051 | 1081 | 1081 |
| Greece | 345 | 344 | 264 | 344 | 344 |
| Ireland | 113 | 70 | 59 | 55 | 65 |
| Italy | 2037 | 1130 | 869 | 901 | 1000 |
| Luxembourg | 22 | 10 | 8 | 8 | 11 |
| Netherlands | 542 | 280 | 238 | 266 | 266 |
| Portugal | 208 | 177 | 144 | 144 | 260 |
| Spain | 1162 | 847 | 781 | 726 | 847 |
| Sweden | 338 | 190 | 152 | 159 | 148 |
| UK | 2839 | 1186 | 1181 | 1181 | 1181 |
| EU-15 | 13226 | 6849 | 5922 | 6054 | 6648 |
| Albania | 24 | 36 | 36 | 36 | 36 |
| Belarus | 402 | 316 | 316 | 290 | 255 |
| Bosnia-H | 80 | 60 | 60 | 53 | 60 |
| Bulgaria | 355 | 297 | 297 | 266 | 266 |
| Croatia | 82 | 91 | 91 | 87 | 87 |
| Czech_R. | 546 | 296 | 296 | 188 | 286 |
| Estonia | 84 | 73 | 73 | 73 | 73 |
| Hungary | 219 | 198 | 198 | 137 | 198 |
| Latvia | 117 | 118 | 118 | 118 | 84 |
| Lithuania | 153 | 138 | 138 | 134 | 110 |
| Norway | 220 | 178 | 178 | 142 | 156 |
| Poland | 1217 | 879 | 879 | 654 | 879 |
| Moldova | 87 | 66 | 66 | 64 | 90 |
| Romania | 518 | 458 | 458 | 328 | 437 |
| Russia | 3486 | 2653 | 2653 | 2653 | 2653 |
| Slovakia | 219 | 132 | 132 | 115 | 130 |
| Slovenia | 60 | 36 | 36 | 34 | 45 |
| Switzerland | 163 | 79 | 79 | 76 | 79 |
| Macedonia | 39 | 29 | 29 | 29 | 29 |
| Ukraine | 1888 | 1433 | 1433 | 1222 | 1222 |
| Yugoslavia | 211 | 152 | 152 | 132 | 152 |
| Non-EU | 10170 | 7718 | 7718 | 6830 | 7327 |
| SEA | 1629 | 1629 | 1629 | 1629 | 1630 |
| TOTAL | 25025 | 16196 | 15269 | 14513 | 15605 |

Table A2.3. Annual emissions of VOCs (kt) in each scenario in 2010 (except 1990).

|  | 1990 | IIASA Reference | H1 | J1 | WGS31c |
| --- | --- | --- | --- | --- | --- |
| Austria | 352 | 205 | 129 | 142 | 159 |
| Belgium | 374 | 193 | 102 | 103 | 144 |
| Denmark | 182 | 85 | 85 | 85 | 85 |
| Finland | 213 | 110 | 110 | 110 | 130 |
| France | 2382 | 1223 | 932 | 989 | 1100 |
| Germany | 3122 | 1137 | 924 | 995 | 995 |
| Greece | 336 | 267 | 173 | 261 | 261 |
| Ireland | 110 | 55 | 55 | 55 | 55 |
| Italy | 2055 | 1159 | 962 | 1030 | 1159 |
| Luxembourg | 19 | 7 | 6 | 7 | 9 |
| Netherlands | 490 | 233 | 156 | 157 | 191 |
| Portugal | 212 | 144 | 102 | 102 | 202 |
| Spain | 1008 | 669 | 662 | 648 | 669 |
| Sweden | 511 | 290 | 219 | 241 | 241 |
| UK | 2667 | 1351 | 964 | 1101 | 1200 |
| EU-15 | 14031 | 7128 | 5581 | 6024 | 6600 |
| Albania | 31 | 41 | 41 | 41 | 41 |
| Belarus | 371 | 309 | 309 | 298 | 309 |
| Bosnia-H | 51 | 48 | 48 | 48 | 48 |
| Bulgaria | 195 | 190 | 190 | 185 | 185 |
| Croatia | 103 | 111 | 111 | 86 | 90 |
| Czech_R. | 442 | 305 | 304 | 156 | 220 |
| Estonia | 45 | 49 | 49 | 49 | 49 |
| Hungary | 204 | 160 | 160 | 137 | 137 |
| Latvia | 63 | 56 | 56 | 56 | 136 |
| Lithuania | 111 | 105 | 105 | 105 | 92 |
| Norway | 297 | 195 | 195 | 195 | 195 |
| Poland | 797 | 807 | 807 | 475 | 800 |
| Moldova | 50 | 42 | 42 | 42 | 100 |
| Romania | 503 | 504 | 504 | 500 | 523 |
| Russia | 3542 | 2786 | 2786 | 2723 | 2786 |
| Slovakia | 151 | 140 | 140 | 140 | 140 |
| Slovenia | 55 | 40 | 40 | 40 | 40 |
| Switzerland | 278 | 144 | 144 | 144 | 144 |
| Macedonia | 19 | 19 | 19 | 19 | 19 |
| Ukraine | 1161 | 851 | 851 | 770 | 797 |
| Yugoslavia | 142 | 139 | 139 | 138 | 139 |
| Non-EU | 8609 | 7041 | 7041 | 6345 | 6990 |
| SEA | 0 | 0 | 0 | 0 | 0 |
| TOTAL | 22640 | 14168 | 12621 | 12369 | 13590 |

Table A2.4. Annual emissions of NH3 (kt) in each scenario in 2010 (except 1990).

|  | 1990 | IIASA Reference | H1 | J1 | WGS31c |
| --- | --- | --- | --- | --- | --- |
| Austria | 77 | 67 | 67 | 66 | 66 |
| Belgium | 97 | 96 | 57 | 60 | 74 |
| Denmark | 77 | 72 | 71 | 69 | 69 |
| Finland | 40 | 31 | 31 | 31 | 31 |
| France | 807 | 777 | 718 | 642 | 780 |
| Germany | 757 | 571 | 413 | 413 | 550 |
| Greece | 80 | 74 | 74 | 73 | 73 |
| Ireland | 127 | 126 | 123 | 116 | 116 |
| Italy | 462 | 432 | 430 | 356 | 419 |
| Luxembourg | 7 | 7 | 7 | 7 | 7 |
| Netherlands | 233 | 136 | 104 | 105 | 128 |
| Portugal | 71 | 67 | 67 | 65 | 108 |
| Spain | 352 | 353 | 353 | 353 | 353 |
| Sweden | 61 | 48 | 48 | 48 | 57 |
| UK | 329 | 297 | 264 | 264 | 297 |
| EU-15 | 3578 | 3154 | 2826 | 2668 | 3128 |
| Albania | 32 | 35 | 35 | 32 | 35 |
| Belarus | 219 | 163 | 163 | 140 | 158 |
| Bosnia-H | 31 | 23 | 23 | 22 | 23 |
| Bulgaria | 141 | 126 | 126 | 105 | 108 |
| Croatia | 40 | 37 | 37 | 29 | 30 |
| Czech_R. | 107 | 108 | 108 | 101 | 101 |
| Estonia | 29 | 29 | 29 | 29 | 29 |
| Hungary | 120 | 137 | 137 | 77 | 90 |
| Latvia | 43 | 35 | 35 | 35 | 44 |
| Lithuania | 80 | 81 | 81 | 72 | 84 |
| Norway | 23 | 21 | 21 | 21 | 23 |
| Poland | 505 | 541 | 541 | 468 | 468 |
| Moldova | 47 | 48 | 48 | 41 | 42 |
| Romania | 292 | 304 | 304 | 227 | 210 |
| Russia | 1282 | 894 | 894 | 894 | 894 |
| Slovakia | 60 | 47 | 47 | 39 | 39 |
| Slovenia | 23 | 21 | 21 | 16 | 20 |
| Switzerland | 72 | 66 | 66 | 63 | 63 |
| Macedonia | 17 | 16 | 16 | 15 | 16 |
| Ukraine | 729 | 649 | 649 | 588 | 592 |
| Yugoslavia | 90 | 82 | 82 | 64 | 82 |
| Non-EU | 3980 | 3462 | 3462 | 3077 | 3151 |
| SEA | 0 | 0 | 0 | 0 | 0 |
| TOTAL | 7558 | 6616 | 6288 | 5745 | 6279 |

Table A2.5. Total annual costs of emissions abatement for SO2, NOX, VOC and NH3 (Meuro/year) in each scenario in 2010.  Costs for IIASA Reference are compared to 1990.  For other scenarios costs are relative to IIASA Reference.

| Costs | IIASA Reference | H1 | J1 | WGS31c |
| --- | --- | --- | --- | --- |
| Austria | 1093 | 119 | 76 | 20 |
| Belgium | 1704 | 1053 | 886 | 191 |
| Denmark | 623 | 6 | 22 | 19 |
| Finland | 889 | 0 | 0 | -5 |
| France | 8659 | 916 | 96 | 86 |
| Germany | 13813 | 2147 | 1567 | 516 |
| Greece | 1482 | 338 | 2 | 2 |
| Ireland | 618 | 44 | 168 | 155 |
| Italy | 9644 | 403 | 417 | 74 |
| Luxembourg | 98 | 4 | 2 | 0 |
| Netherlands | 2588 | 971 | 803 | 164 |
| Portugal | 1530 | 57 | 59 | -14 |
| Spain | 6495 | 22 | 51 | 0 |
| Sweden | 1554 | 87 | 45 | -29 |
| UK | 7964 | 1348 | 671 | 313 |
| EU-15 | 58754 | 7514 | 5704 | 1492 |
| Albania | 0 | 0 | 1 | 0 |
| Belarus | 0 | 0 | 12 | 26 |
| Bosnia-H | 1 | 0 | 58 | 0 |
| Bulgaria | 157 | 0 | 81 | 17 |
| Croatia | 52 | 0 | 26 | 6 |
| Czech_R. | 979 | 0 | 280 | 89 |
| Estonia | 0 | 0 | 0 | 0 |
| Hungary | 586 | 0 | 545 | 113 |
| Latvia | 0 | 0 | 0 | 49 |
| Lithuania | 0 | 0 | 4 | 31 |
| Norway | 623 | 0 | 25 | 10 |
| Poland | 3342 | 0 | 838 | 182 |
| Moldova | 0 | 0 | 33 | 2 |
| Romania | 157 | 0 | 541 | 617 |
| Russia | 715 | 0 | 54 | 0 |
| Slovakia | 423 | 0 | 43 | 19 |
| Slovenia | 128 | 0 | 25 | 18 |
| Switzerland | 949 | 0 | 9 | 5 |
| Macedonia | 1 | 0 | 1 | 0 |
| Ukraine | 328 | 0 | 82 | 75 |
| Yugoslavia | 92 | 0 | 128 | 0 |
| Non-EU | 8534 | 0 | 2787 | 1261 |
| Atlantic | 0 | 0 | 0 | 0 |
| Baltic | 0 | 0 | 0 | 0 |
| North_Sea | 0 | 0 | 0 | 0 |
| SEA | 0 | 0 | 0 | 0 |
| TOTAL | 67288 | 7514 | 8490 | 2753 |

- Appendix 3 
Article 5 Assessment of Ozone

As part of the proposed European Directive relating to ozone in ambient air, a requirement will be placed upon member states to establish adequate information on ambient levels of ozone.  In accordance with Article 5 of the Framework Directive, a further requirement is placed on member states to undertake preliminary investigation of the ambient levels of pollutants prior to the implementation of related Daughter Directives.

The objectives of these assessments are to establish estimates for the overall distribution and levels of pollutants, and to identify additional monitoring requirements, which may be necessary in order to fulfil obligations under the Framework and Daughter Directives.  In this report, observations and recommendations are raised regarding the number of additional monitoring locations, which may be required for ozone.  Estimated cost data for these additional monitoring requirements are also presented.

- Methodology

For the purpose of this assessment, the UK has been divided into 16 zones, based on official government office boundaries within England and boundaries provided or authorised by the relevant offices within Scotland, Wales and Northern Ireland as part of the Article 5 Assessment performed for the first Daughter Directive on NO2, SO2, PM10 and lead.  A further 28 agglomeration zones (areas of urban population > 250000) have also been agreed based on DETR GIS information for England and Wales, localities information in Scotland and CORINE land cover information within Northern Ireland.  Monitoring requirements within each zone and agglomeration have been assessed separately.

Estimates of the ozone concentration within each zone and agglomeration have been derived from the DETR’s automatic urban network (AUN) and rural networks.  In particular, exceedence of the Long-term ozone objective for protection of human health (see Annex I, Table III of the Daughter Directive) was investigated in each zone and agglomeration according to the guidance presented in Article 9 of the Daughter Directive for ozone.  Under the Directive, monitoring is mandatory in all zones and agglomerations where the measured 8-hour running mean ozone concentration exceeds 120 g/m3.  It was identified that this objective was exceeded in all agglomerations and zones within which monitoring has taken place between 1993 and 1999, with the exception of Glasgow and Coventry/Bedworth, which only commenced monitoring in 1997 and 1996 respectively.  No monitoring data was available in seven agglomerations and four zones.

As a conservative estimate therefore, and taking into account the lack of data for Glasgow and zones and agglomerations where monitoring was not performed, it was assumed that ozone concentrations exceeded the long-term objective in all zones and agglomerations in the UK, as a result further investigation of the exceedence long-term objective for protection of vegetation was not performed.  Monitoring is therefore required in all agglomerations and zones in the UK.

The number of additional monitoring sites required in each zone and agglomeration has been calculated using the guidance presented in Annex V Table I of the Daughter Directive.  Calculations are based on the population of the individual zones and agglomerations, exceedence data for the Long-term objective for the protection of human health and the existing (June 1999) DETR rural network and AUN ozone monitoring locations.

It should be noted that the availability of other methods of assessment (empirical and chemical models) have been taken into account and this enables the number of sites required in each zone and agglomeration to be reduced, in accordance with Article 9 paragraph 5 of the Daughter Directive.

- observations

Tables 1 and 2 present the number of additional sites required within agglomerations and zones throughout the UK.  The minimum number of sites required assuming that fixed monitoring is the sole source of information (Minimum Site No.  A), and the minimum number of sites required assuming that other means of assessment are available (Minimum Site No. B) are presented.  The latter scenario is appropriate for the UK and additional site numbers required for compliance with the Daughter Directive have been calculated using these statistics.  The existing number of ozone monitoring sites in the current DETR rural network and AUN are also presented.  The additional siting requirements presented are based on the difference between the existing number of sites and the Minimum Site No. B statistic.  These numbers assume the purchase of additional monitors, however, it should be noted that in some cases the short fall in existing monitors may be made up by the relocation of existing monitors which are currently located in areas that do not require monitoring under the Daughter Directive.  Such cases are marked with an asterisk (*).

Assumptions have been made at this stage that the definition of station location (Urban, Suburban etc.) used within the Daughter Directive are directly comparable with those used in the UK.  It should be noted however, that this may not be the case as such descriptions are not likely to be entirely categorical.  Hence, it may be possible for monitoring locations described as urban in the UK to accommodate for a requirement to sample in a suburban area as defined by the Daughter Directive and visa versa.  This observation may enable the number of additional monitoring sites required to be reduced, although no further investigation will be made within this assessment.

| Table 1 Ozone monitoring requirements in UK agglomerations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. England |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Minimum Site No. A |  |  | Minimum Site No. B |  |  | Existing Sites |  |  | Additional Sites |  |  |  |  |  |
| Agglomeration | Population | Urban | Suburban | Urban | Suburban | Urban | Suburban | Urban | Suburban |  |  |  |
| Greater London Urban Area | 7650944 | 2 | 5 | 1 | 3 | 10 | 4 | 0 | 0 |  |  |  |
| West Midlands Urban Area | 2296180 | 2 | 3 | 1 | 2 | 4 | 0 | 0 | 2 | * |  |  |
| Greater Manchester Urban Area | 2277330 | 2 | 3 | 1 | 2 | 2 | 1 | 0 | 1 | * |  |  |
| West Yorkshire Urban Area | 1445981 | 1 | 2 | 1 | 1 | 2 | 0 | 0 | 1 | * |  |  |
| Tyneside | 885981 | 0 | 2 | 0 | 1 | 1 | 0 | 0 | 1 | * |  |  |
| Liverpool Urban Area | 837998 | 0 | 2 | 0 | 1 | 1 | 0 | 0 | 1 | * |  |  |
| Sheffield Urban Area | 633362 | 0 | 2 | 0 | 1 | 2 | 0 | 0 | 1 | * |  |  |
| Nottingham Urban Area | 613726 | 0 | 2 | 0 | 1 | 1 | 0 | 0 | 1 | * |  |  |
| Bristol Urban Area | 522784 | 0 | 2 | 0 | 1 | 1 | 0 | 0 | 1 | * |  |  |
| Brighton/Worthing/Littlehampton | 437592 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 1 |  |  |  |
| Leicester Urban Area | 416601 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | * |  |  |
| Portsmouth Urban Area | 409341 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 1 |  |  |  |
| Teesside Urban Area | 369609 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 |  |  |  |
| The Potteries | 367976 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | * |  |  |
| Bournemouth Urban Area | 358321 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 1 |  |  |  |
| Reading/Wokingham Urban Area | 335757 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | * |  |  |
| Coventry/Bedworth | 331248 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | * |  |  |
| Kingston upon Hull | 310636 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | * |  |  |
| Southampton Urban Area | 276752 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | * |  |  |
| Birkenhead Urban Area | 270207 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 1 |  |  |  |
| Southend Urban Area | 266749 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 1 |  |  |  |
| Blackpool Urban Area | 261355 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 1 |  |  |  |
| Preston Urban Area | 256411 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 1 |  |  |  |
| Minimum site no. A assumes fixed monitoring as sole source of information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Minimum site no. B assumes that other sources of information are available and represents |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| a reduction in the Minimum site no. A value of one third (see Article 9, para 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| * indicates that compliance with the Minimum site no. B requirement may be achieved by relocation of existing monitoring locations |  |  |  |  |  |  |  |  |  |  |  |  |
| (Table 1 continued) |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. Wales |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Minimum Site No. A |  |  | Minimum Site No. B |  |  | Existing Sites |  |  | Additional Sites |  |  |  |  |  |
| Agglomeration | Population | Urban | Suburban | Urban | Suburban | Urban | Suburban | Urban | Suburban |  |  |  |
| Swansea | 272456 | 0 | 1 | 0 | 1 | 2 | 0 | 0 | 1 | * |  |  |
| Cardiff | 306904 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | * |  |  |
| 3. Scotland |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Minimum Site No. A |  |  | Minimum Site No. B |  |  | Existing Sites |  |  | Additional Sites |  |  |  |  |  |
| Agglomeration | Population | Urban | Suburban | Urban | Suburban | Urban | Suburban | Urban | Suburban |  |  |  |
| Edinburgh Urban Area | 416232 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | * |  |  |
| Glasgow Urban Area | 1315544 | 0 | 2 | 0 | 1 | 1 | 0 | 0 | 1 | * |  |  |
| 4. Northern Ireland |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Minimum Site No. A |  |  | Minimum Site No. B |  |  | Existing Sites |  |  | Additional Sites |  |  |  |  |  |
| Agglomeration | Population | Urban | Suburban | Urban | Suburban | Urban | Suburban | Urban | Suburban |  |  |  |
| Belfast | 475987 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | * |  |  |
| Minimum site no. A assumes fixed monitoring as sole source of information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Minimum site no. B assumes that other sources of information are available and represents |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| a reduction in the Minimum site no. A value of one third (see Article 9, para 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| * indicates that compliance with the Minimum site no. B requirement may be achieved by relocation of existing monitoring locations |  |  |  |  |  |  |  |  |  |  |  |  |

| Table 2 Ozone monitoring requirements in UK zones |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. England |  |  |  |  |  |  |  |  |  |  |
|  |  | Minimum Site No. A |  |  | Minimum Site No. B |  |  | Existing Sites |  |  | Additional Sites |  |  |  |
| Zone | Population | Suburban | Rural | Suburban | Rural | Suburban | Rural | Suburban | Rural |  |
| Greater London | 7650944 | na | na | na | na | na | na | na | na |  |
| North East | 1287979 | 1 | 3 | 1 | 2 | 0 | 0 | 1 | 2 |  |
| North West & Merseyside | 2823559 | 1 | 7 | 1 | 5 | 1 | 0 | 0 | 5 |  |
| Yorkshire & Humberside | 2446545 | 1 | 5 | 1 | 3 | 0 (1) | 1 | 1 | 2 | * |
| East Midlands | 2923045 | 1 | 7 | 1 | 5 | 1 | 0 | 0 | 5 |  |
| West Midlands | 2154783 | 1 | 5 | 1 | 3 | 0 (1) | 1 | 1 | 2 | * |
| Eastern | 4788766 | 1 | 9 | 1 | 6 | 0 (2) | 2 | 1 | 4 | * |
| South East | 3702634 | 1 | 7 | 1 | 5 | 0 | 3 | 1 | 2 |  |
| South West | 3728319 | 1 | 7 | 1 | 5 | 0 (1) | 2 | 1 | 3 | * |
| 2. Wales |  |  |  |  |  |  |  |  |  |  |
|  |  | Minimum Site No. A |  |  | Minimum Site No. B |  |  | Existing Sites |  |  | Additional Sites |  |  |  |
| Zone | Population | Suburban | Rural | Suburban | Rural | Suburban | Rural | Suburban | Rural |  |
| South Wales | 1544120 | 1 | 4 | 1 | 3 | 0 | 1 | 1 | 2 |  |
| North Wales | 582488 | 1 | 2 | 1 | 1 | 0 | 0 | 1 | 1 |  |
| Minimum site no. A assumes fixed monitoring as sole source of information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Minimum site no. B assumes that other sources of information are available and represents |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| a reduction in the Minimum site no. A value of one third (see Article 9, para 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Numbers in parentheses indicate existing urban monitoring locations in zones |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| * indicates that compliance with the Minimum site no. B requirement for suburban locations may be achieved by relocation of existing urban monitoring sites |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (Table 2 continued) |  |  |  |  |  |  |  |  |  |  |
| 3. Scotland |  |  |  |  |  |  |  |  |  |  |
|  |  | Minimum Site No. A |  |  | Minimum Site No. B |  |  | Existing Sites |  |  | Additional Sites |  |  |  |
| Zone | Population | Suburban | Rural | Suburban | Rural | Suburban | Rural | Suburban | Rural |  |
| Scottish Borders | 246659 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 |  |
| Central Scotland | 1628460 | 1 | 4 | 1 | 3 | 0 | 1 | 1 | 2 |  |
| North East Scotland | 933485 | 1 | 2 | 1 | 1 | 0 | 0 | 1 | 1 |  |
| Highland | 364639 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 0 |  |
| 4. Northern Ireland |  |  |  |  |  |  |  |  |  |  |
|  |  | Minimum Site No. A |  |  | Minimum Site No. B |  |  | Existing Sites |  |  | Additional Sites |  |  |  |
| Zone | Population | Suburban | Rural | Suburban | Rural | Suburban | Rural | Suburban | Rural |  |
| Northern Ireland | 1101868 | 1 | 3 | 1 | 2 | 0 (2) | 1 | 1 | 1 | * |
| Minimum site no. A assumes fixed monitoring as sole source of information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Minimum site no. B assumes that other sources of information are available and represents |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| a reduction in the Minimum site no. A value of one third (see Article 9, para 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Numbers in parentheses indicate existing urban monitoring locations in zones |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| * indicates that compliance with the Minimum site no. B requirement for suburban locations may be achieved by relocation of existing urban monitoring sites |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- Summary of additional monitoring requirements

On the basis of the assessments for zones and agglomerations presented in Tables 1 and 2 monitoring requirements have been estimated for England, Wales, Scotland and Northern Ireland.  Total additional siting requirements for these regions of the UK are presented in Table 3 below.  Where a range is given below, options are available for the relocation of existing monitoring sites to areas that require monitoring under the Daughter Directive.

Table 3 Summary of the total number of additional ozone monitoring sites required within the UK for compliance with the proposed Daughter Directive for Ozone

|  | England | Wales | Scotland | Northern Ireland |
| --- | --- | --- | --- | --- |
| Agglomerations | 7-22 | 0-2 | 0-2 | 0-1 |
| Other Zones | 27-31 | 5 | 6 | 0-2 |
| Total | 34-53 | 5-7 | 6-8 | 0-3 |

Hence from Table 3, the minimum number of additional monitoring sites required within the UK is 45, which assumes the relocation of 26 existing monitoring sites.  If no relocation of existing monitoring is carried out the minimum number of additional monitoring sites required within the UK is 71.

- Observations on monitoring requirements

On a purely scientific basis the number of additional monitoring sites required by the Daughter Directive would appear too excessive.  Clearly, there is an additional monitoring requirement within agglomerations not well serviced by the existing networks to provide information to the public on possible health exposures.  However, it is felt that the number of additional sites required in rural areas of zones is excessive when the spatial variation of rural ozone at sub-zonal level is expected to be small.

The following features of the Daughter Directive may be identified as leading to the requirement for large number of additional monitoring sites:

- The use of the Long-term objective for protection of human health as trigger for mandatory monitoring in zones and agglomerations, as this objective is particularly stringent when applied to current concentrations.

- Minimal relief, in terms of the reduction in required monitor provided by the availability of ‘other means of assessment’.

- Monitoring requirements in rural areas are based on the total population the zone (urban + rural population) rather than just the rural population

- estimated costs of additional ozone monitoring

A proportion of the additional ozone monitors required may be found from existing local authority sites and affiliated to the national network.  As a worst case, however, we have assumed that none can be affiliated.

An automatic ozone analyser costs approximately £7,000.  The Local Site Operator and maintenance contract would typically be around £10,000 per year.  Running costs including management, QA/QC, data handling and electricity would be in the region of £2,000 per year.  The estimated cost for each ozone-monitoring site would be in the region of £19,000 for the first year and £12,000 for subsequent years.

Based on the estimated numbers of additional sites summarised in Table 3, approximate costs have been calculated and are presented in Table 4 below.  These figures assume that the monitors can be housed in an existing building and do not include the cost of sourcing electricity supply, (of particular implication at rural locations).  A range of costs is provided in cases where monitoring requirements may be achieved by relocation of existing analysers.  The lower figure reflects the cost of monitoring if all existing monitors currently located in areas that do not require monitoring under the Daughter Directive are relocated to areas that do require monitoring.  The higher cost reflects the cost of additional monitoring if no relocation of existing monitors were to occur.

Table 4 Summary of the minimum additional monitoring requirements for ozone in the UK with cost data

|  | No. of new O3 sites | Cost in year 1 | Cost in subsequent years |
| --- | --- | --- | --- |
| England | 34-53 | £646,000-1,007,000 | £408,000-636,000 |
| Wales | 5-7 | £95,000-133,000 | £60,000-84,000 |
| Scotland | 6-8 | £114,000-152,000 | £72,000-96,000 |
| Northern Ireland | 0-3 | £0-57,000 | £0-36,000 |
| Total | 45-71 | £855,000-1,349,000 | £540,000-852,000 |

- Appendix 4 
Analysis using HARM and ELMO

- The models

HARM and ELMO share a similar basic structure; both are Lagrangian statistical models, which show the changing chemistry of a parcel of air as it moves through time towards a designated set of receptor sites.  The air parcels follow a series of 72 straight-line trajectories towards each receptor and the final concentrations (in air and rain) at the sites are calculated by applying a weighting from a wind rose.  Both models employ a highly simplified meteorology assuming constant wind speed and boundary layer height, a single wind rose and washout parameterised for constant drizzle.  The meteorological values in HARM are set to represent annual average conditions, whilst those in ELMO represent a late summer afternoon when conditions for ozone production are most favourable.  The spatial resolution of the emissions data used in both HARM and ELMO is 50 x 50 km over the EMEP area and 10 km x 10 km over the UK.  Species dependent deposition velocities are used to generate dry deposition, using single values across the EMEP area and land use dependent values for the UK.  Some of the features of the two models are set out in Table A4.1 below.

Table A4.1 Selected features of HARM and ELMO

|  | HARM | ELMO |
| --- | --- | --- |
| Boundary layer height | 800 m | 1400 m |
| Wind speed | 10.4 m s-1 | 3.6 m s-1 |
| Wind rose | National annual average (Jones, 1981) | Derived from Heathrow Airport for hours where O3 > 50 ppb |
| Emissions | SO2, NOx, NH3 HCl | SO2, NOx, NH3, HCl, CO, NMVOCs and DMS |
| Values at edge of model domain | All set to zero except O3 set at 30 ppb | NO, NO2, CO, CH4 and O3 set to estimated global background for 1990, all other species set to zero |

HARM has been used extensively to estimate the concentration and deposition of potentially acidifying compounds of sulphur (S) and nitrogen (N) and hydrogen chloride.  Full descriptions of the model, its parameters and its validation against data from the UK’s monitoring networks are available in the peer-reviewed scientific literature (Metcalfe et al, 1995; Metcalfe et al, 1998).  HARM was also one of the models included in an intermodel comparison carried out by the UK Review Group on Acid Rain (RGAR) where it was shown to be at least as good as any other model in use in the UK (RGAR, 1997).  In this exercise HARM11.5 has been used to model estimates of

- depositions of S, oxidised N and reduced N (as kg S or N) for the UK at 10 km resolution;

- concentrations of secondary PM10 (in g/m3).

One significant difference between HARM11.5 and previous versions of the model is its representation of the dry deposition of NH3 which is now based on a fraction of emission being deposited directly in the emitting square (an alpha factor).  There are individual values for each 10 km grid and these were supplied by Mark Sutton of ITE Bush.

ELMO is a new model designed to estimate peak ozone concentrations.  The chemical scheme in ELMO comes directly from the STOCHEM model (Collins et al., 1997; Stevenson et al., 1998) and includes 70 chemical species and >160 chemical reactions.  The NMVOC emissions inventory in ELMO is not speciated, but within the model the overall totals are allocated to 11 major species (formaldehyde, ethane, acetaldehyde, ethylene, propene, propane, methanol, acetone, toluene, N butane and 0-xylene) which represent different types of ozone sources.  Isoprene, the major natural biogenic hydrocarbon, is also represented.  The latitude (50oN) and month (July) are set to determine day length, temperature and humidity and hence the reaction rate coefficients.  Photolysis rates vary diurnally and assume clear sky conditions.  ELMO output for peak ozone concentrations has been compared with O3 maxima from the UK’s rural monitoring network for 1995 and appears to provide a reasonable representation of the distribution, although the modelled concentrations are lower than those measured.  This underestimation may be explained, in part, by the model’s weighting and averaging trajectories from each of the 72 directions.  ELMO O3 estimates have been converted in to AOT40, AOT60, WHO and EPAQS exceedences using empirical relationships between modelled and observed data at the rural monitoring sites.  The model has also been used to estimate summertime concentrations of secondary PM10.

- Input data

The emissions used in the scenarios run using HARM and ELMO are given in Table A4.2.  These differ slightly from the emissions shown in Table 1 of the main report, as offshore emissions are not included in HARM and ELMO.

Data for the EMEP area were taken from the 6th and 7th IIASA Interim Reports and from the UNECE TFIAM 23rd Meeting March 1999.  David Simpson (IVL, Sweden) supplied isoprene emissions over the EMEP area.  The emission totals in the model do not correspond exactly with those listed in the reports as the model includes emissions from additional sources such as shipping and sea areas and other countries represented in the EMEP emission inventories but not listed in the IIASA tables.  The only future estimate of CO emissions over the EMEP area is based on current reduction plans (EMEP/MSC-W Report 1/98).  For the UK, spatially disaggregated emissions inventories were prepared by AEA Technology (Justin Goodwin) for SO2, NOx, HCl, NMVOCs and CO.  Future emissions of NH3 were derived by scaling 1996 emissions supplied by ITE Bush (Mark Sutton).  Separate scaling factors were applied for England, Scotland, Wales and Northern Ireland.  Previous NMVOC inventories had not included isoprene and this was added separately; this latest inventory did and so all ELMO runs include an extra 80 k tonnes of isoprene emission.  The UK emissions used in the UKREF scenario reflect estimates of the possible amounts and distributions of emissions based on current understanding of future trends.  They have no official status.  For the UKREF run, emissions over the EMEP area were set at IIASA Reference (see above)

Table A4.2.  Emissions totals used in HARM and ELMO scenarios

| HARM scenarios (in k tonnes) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario | UK |  |  |  | EMEP |  |  |
|  | SO2 | NOx | NH3 | HCl | SO2 | NOx | NH3 |
| 1990 | 3754 | 2800 | 329 | 375 | 37946 | 22556 | 7608 |
| UKREF | 765 | 1107 | 319 | 38 | 18872 | 16239 | 6870 |
| H1 | 477 | 1063 | 290 | 15 | 18238 | 15568 | 6615 |
| J1 | 479 | 1063 | 290 | 15 | 15290 | 14810 | 6071 |
| ELMO scenarios (in k tonnes) as above with the addition of: |  |  |  |  |  |  |  |
| Scenario | UK |  |  |  | EMEP |  |  |
|  | VOC | CO | Isoprene |  | VOC | CO | Isoprene |
| 1995 | 2050 | 5424 | 80 |  | 16966 | 67607 | 3786 |
| UKREF | 1385 | 2872 | 80 |  | 13603 | 80480 | 3786 |
| H1 | 1080 | 2872 | 80 |  | 12859 | 80480 | 3786 |
| J1 | 1179 | 2872 | 80 |  | 11676 | 80480 | 3786 |

- Outputs

HARM modelled depositions of S, oxidised and reduced N were supplied to ITE Monks-Wood in grid format to generate maps of critical loads exceedence and to AEA Technology.  The spatial changes in depositions of total (wet + dry) S, oxidised N and reduced N are illustrated for some of the scenarios in Figures A4.1 to A4.3 respectively.  Data for 1990 are included for comparative purposes.  The changing depositions under all the scenarios are summarised in budget terms in Table A4.3.

Table A4.3.  HARM modelled deposition budgets in k tonnes.

| Scenario | wet S | dry S | tot S | wet N | dry N | tot N | wet NH-N | dry NH-N | tot NH-N |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1990 | 215.6 | 178.3 | 393.9 | 119.7 | 69.3 | 189.0 | 85.4 | 53.1 | 138.5 |
| UKREF | 68.4 | 47.4 | 115.8 | 75.1 | 28.1 | 103.2 | 48.4 | 52.8 | 101.2 |
| H1 | 55.5 | 35.5 | 91.0 | 68.9 | 25.5 | 94.4 | 42.3 | 49.1 | 91.4 |
| J1 | 51.2 | 33.8 | 85.0 | 68.2 | 25.2 | 93.4 | 40.7 | 48.8 | 89.5 |

The contribution of UK deposition from UK sources is of particular policy relevance and in Table A4.4 this contribution is summarised in percentage terms.  There is a sharp contrast between reduced N and the other pollutants.  The % of UK to UK deposition for NH-N remains high and static, reflecting its much shorter average transport distance (except in the aerosol phase).

Table A4.4.  Modelled UK to UK contribution of deposition as %

| Scenario | tot S | tot N | totNH-N |
| --- | --- | --- | --- |
| 1990 | 64 | 58 | 70 |
| UKREF | 42 | 43 | 78 |
| H1 | 35 | 45 | 78 |
| J1 | 37 | 45 | 79 |

One of the major changes as a result of emissions reduction policies is in the source of acidity.  This is illustrated in Figure A4.4.  In the past, the majority of deposited H+ was provided by S deposition, but in the future the main source will be N.  Apart from some areas of north west Scotland, N>S as a source of H+ in all areas except those near major coal fired power stations (e.g. the lower Ouse and Trent valleys).

HARM modelled concentrations of secondary aerosols (PM10) are shown in Figure A4.5.  There is a clear south east – north west gradient in all cases.  Reducing emissions beyond UKREF brings clear benefits to the south east of England.

ELMO output was supplied to AEA Technology and to Imperial College; some of this output is described below.  The basic model output is peak ozone concentration in ppb (Figure A4.6).  Modelled 1995 values are shown for reference.  A comparison with mapped data from the monitoring networks is not straightforward due to the common use of means and a variety of correction factors (PORG, 1997).  A comparison with site measurements is easier (see above).  As well as being a key oxidant in the troposphere, it is recognised that high concentrations of O3 can be damaging to vegetation (especially crops), materials and to human health.  There are a variety of damage thresholds set for these different receptors (PORG, 1997; DoE, 1997).  These include an AOT40 index for critical levels for vegetation and WHO and EPAQS standards for health.  AOT40 represents accumulated exposure to concentrations > 40 ppb.  The EPAQS standard is set at 50 ppb as a running 8 hour mean and the WHO standard is based on AOT60.  ELMO can be used to estimate O3 concentrations relative to these standards.  Modelled EPAQS exceedences are shown in Figure A4.7 and modelled AOT40 (hours above the threshold) in Figure A4.8.  The benefits of reductions in emissions beyond the UKREF scenario are clear in both these cases.

Figure A4.1 HARM modelled S deposition.

Figure A4.2 HARM modelled oxidised N deposition.

Figure A4.3.  HARM modelled reduced N deposition.

Figure A4.4  HARM modelled sulphur vs. total nitrogen deposition.

Figure A4.5 HARM modelled secondary PM10 (in g/m3).

Figure A4.6 ELMO modelled peak ozone.

Figure A4.7  ELMO modelled EPAQS exceedences for ozone.

Figure A4.8 ELMO modelled AOT40.

- Appendix 5
Analysis using RAINS and ASAM

- Introduction

In this appendix further implications of the various scenarios are presented.  These effects have been modelled using RAINS and ASAM, two integrated assessment models, which cover the whole of the UNECE region at a resolution of 150x150 km.  Both models are based on the EMEP Lagrangian meteorological modelling of the relationship between emission sources and receptor areas in Europe.  All of the results presented here are based on the RAINS model, the official model used in the UNECE Task Force on Integrated Assessment Modelling, except for those concerning human exposure to secondary particulates which is not covered by RAINS.  For these calculations the ASAM model is used.  The model has been shown to agree well with the RAINS model and is used for sensitivity analysis and supporting work at the UNECE Task Force on Integrated Assessment Modelling.

Implications of scenarios are presented for (a) the UK and (b) the whole of the UNECE region.  Effects analysed are (a) human exposure to secondary particulates (b) acidification of ecosystems (c) eutrophication of ecosystems (d) AOT40 and AOT60 and the associated levels of ozone exposure to crops and human health.

- Explanation of Scenarios

Table A5.1 shows the emission levels for the UK in the scenarios covered.  The IIASA Reference scenario is the one calculated by IIASA for use at the Task Force on Integrated Assessment Modelling.  The full set of European emissions under this scenario and H1 and J1 were given in Appendix II.

The underlying scenario is a variant of the IIASA Reference scenario in which European emissions remain at IIASA Reference whilst UK emissions are altered as shown in Table A5.1.

The ‘additional’ scenarios such as UK1 and UK2 are variants of J1, that is, European emissions remain at J1 whilst the UK alters its emissions to those shown in Table A5.1.

Table A5.1 Annual UK emission ceilings for scenarios examined in 2010 (kt)

| SCENARIO | SO2 | NOx | NH3 | VOC |
| --- | --- | --- | --- | --- |
| Basic scenarios: |  |  |  |  |
| IIASA Reference | 980 | 1186 | 297 | 1351 |
| UKREF | 784 | 1187 | 329 | 1300 |
| H1 | 497 | 1181 | 264 | 964 |
| J1 | 499 | 1181 | 264 | 1101 |
| Additional scenarios: |  |  |  |  |
| UK1 | 600 | 1181 | 297 | 1200 |
| UK1a | 550 | 1150 | 274 | 1200 |
| UK2 | 550 | 1100 | 274 | 1200 |
| UK3 | 550 | 1000 | 274 | 1200 |
| UK4 | 550 | 1150 | 274 | 1101 |
| UK5 | 550 | 1100 | 274 | 1101 |
| UK6 | 550 | 1000 | 274 | 1101 |
| UK7 | 550 | 1181 | 274 | 1150 |
| UK8 | 550 | 1100 | 274 | 1150 |
| UK9 | 550 | 1100 | 274 | 1351 |
| UK10 | 550 | 1186 | 274 | 1351 |
| UKS11 | 525 | 1181 | 264 | 1101 |
| UKS13 | 700 | 1100 | 264 | 1101 |

- AOT40 and AOT60: Exposure of humans, vegetation and crops to ozone in excess of critical levels

- METHODOLOGY

AOT60: Method a

The RAINS web model was used to extract tables detailing AOT60 totals for the U.K. and for the UNECE region as a whole.  RAINS provides average AOT60 values for each country in ppm hours.  It also provides population exposures in units of million persons.ppm hours.  The results are presented in Table A5.2a. Note that RAINS calculates population exposures using a threshold of 0.4 ppm hours.

AOT60: Method b

Since the values of AOT60 were rather small, and RAINS only automatically produces results to one decimal place, more accurate information was extracted from the raw data which could be extracted from the model, totals of AOT60 were recalculated (referred to as method b).  These values are shown in Table A5.2b, together with the matching population exposures.  A threshold of 0.4 ppm hours has been used in this case.

AOT40: Method c

RAINS was also used to extract values of AOT40 (method c).  These are the average values in excess of 3 ppm hour critical level for vegetation.  RAINS also provides tables of vegetation exposure using this threshold (Table A5.2c).

AOT40: Method d

RAINS was also used to provide raw data for AOT40 so that average AOT40 values (without a threshold) could be calculated for the UK and the UNECE (method d, Table A5.2d).  Vegetation exposure was not calculated using this method.

Crop exposure to AOT40

Crop areas exposed to an excess of 3 ppm hours AOT40 were calculated using crop areas obtained from IIASA (Tables A5.2e, f).

- Table A5.2a.  AOT60 mean values and population exposures in UK and UNECE (method a).  J1 shows the difference between the scenario selected and J1

| UK | AOT60 (mean) | J1 | %loss in benefit | AOT60pop | J1 | %loss in benefit |
| --- | --- | --- | --- | --- | --- | --- |
|  | Ppm hrs | Ppm hrs |  | Million person ppm hrs |  |  |
| IIASA Reference | 1.30 | 0.40 |  | 77 | 28 |  |
| UKREF | 1.3 | 0.40 |  | 75 |  |  |
| J1 | 0.9 | 0.00 |  | 49 | 0 |  |
| H1 | 0.8 | -0.1 | -25 | 45 | -4 | -14.3 |
| UK7 | 0.9 | 0 | 0 | 52 | 3 | 10.7 |
| UK8 | 0.9 | 0 | 0 | 53 | 4 | 14.3 |
| UK9 | 1.1 | 0.2 | 50 | 62 | 13 | 46.4 |
| UK10 | 1.1 | 0.2 | 50 | 61 | 12 | 42.9 |
| UNECE | AOT60 | J1 | %loss in benefit | AOT60pop | J1 | %loss in benefit |
| IIASA Reference | 0.8 | 0.3 |  | 570 | 224 |  |
| UKREF | 0.8 | 0.3 |  | 566 |  |  |
| J1 | 0.5 | 0 |  | 346 | 0 |  |
| H1 | 0.6 | 0.1 | 33 | 385 | 39 | 17.4 |
| UK7 | 0.5 | 0 | 0 | 351 | 5 | 2.2 |
| UK8 | 0.5 | 0 | 0 | 352 | 6 | 2.7 |
| UK9 | 0.5 | 0 | 0 | 369 | 23 | 10.3 |
| UK10 | 0.5 | 0 | 0 | 367 | 21 | 9.4 |

- Table A5.2b.  AOT60 mean values and population.  Exposures in UK and UNECE (method b).  J1 shows the difference between the scenario selected and J1

| UK | AOT60 (mean excess) | J1 | %loss in benefit | AOT60pop | J1 | %loss in benefit |
| --- | --- | --- | --- | --- | --- | --- |
|  | Ppm hrs | Ppm hrs |  | Million person ppm hrs |  |  |
| IIASA Reference | 0.95 | 0.31 |  | 79 | 26 |  |
| UKREF | 0.93 |  |  | 77 |  |  |
| J1 | 0.64 | 0.00 |  | 53 | 0 |  |
| H1 | 0.60 | -0.04 | -14.4 | 49 | -4 | -8.3 |
| UK7 | 0.66 | 0.02 | 6.5 | 55 | 2 | 7.7 |
| UK8 | 0.67 | 0.03 | 9.7 | 56 | 3 | 11.5 |
| UK9 | 0.76 | 0.12 | 38.7 | 64 | 11 | 42.3 |
| UK10 | 0.75 | 0.11 | 35.5 | 63 | 10 | 38.5 |
| UNECE | AOT60 | J1 | %loss in benefit | AOT60pop | J1 | %loss in benefit |
| IIASA Reference | 0.47 | 0.17 |  | 604 | 208 |  |
| UKREF | 0.47 |  |  | 600 |  |  |
| J1 | 0.30 | 0.00 |  | 396 | 0 |  |
| H1 | 0.34 | 0.04 | 23.5 | 429 | 33 | 15.9 |
| UK7 | 0.31 | 0.01 | 5.9 | 400 | 4 | 1.9 |
| UK8 | 0.31 | 0.01 | 5.9 | 401 | 5 | 2.4 |
| UK9 | 0.32 | 0.02 | 11.8 | 417 | 21 | 10.0 |
| UK10 | 0.32 | 0.02 | 11.8 | 416 | 20 | 9.6 |

- Table A5.2c.  AOT40 mean values in excess of 3 ppm hours, and vegetation exposures in UK and UNECE (method a).

| UK | AOT40 | J1 | %loss in benefit | AOT40veg | J1 | %loss in benefit |
| --- | --- | --- | --- | --- | --- | --- |
|  | Ppm hrs |  |  | Ppm hrs area |  |  |
| IIASA Reference | 1.9 | 0.5 |  | 153 | 42 |  |
| UKREF | 1.8 | 0.1 |  | 148 |  |  |
| J1 | 1.4 | 0 |  | 111 | 0 |  |
| H1 | 1.2 | -0.2 | -40 | 96 | -15 | -35.7 |
| UK7 | 1.4 | 0 | 0 | 116 | 5 | 11.9 |
| UK8 | 1.5 | 0.1 | 20 | 120 | 9 | 21.4 |
| UK9 | 1.7 | 0.3 | 60 | 140 | 29 | 69.0 |
| UK10 | 1.6 | 0.2 | 40 | 136 | 25 | 59.5 |
| UNECE | AOT40 | J1 | %loss in benefit | AOT40veg | J1 | %loss in benefit |
| IIASA Reference | 2.5 | 0.6 |  | 13100 | 2858 |  |
| UKREF | 2.4 |  |  | 13068 |  |  |
| J1 | 1.9 | 0 |  | 10242 | 0 |  |
| H1 | 1.9 | 0 | 0 | 10245 | 3 | 0.1 |
| UK7 | 1.9 | 0 | 0 | 10271 | 29 | 1.0 |
| UK8 | 1.9 | 0 | 0 | 10274 | 32 | 1.1 |
| UK9 | 1.9 | 0 | 0 | 10394 | 152 | 5.3 |
| UK10 | 1.9 | 0 | 0 | 10391 | 149 | 5.2 |

- Table A5.2d.  AOT40 mean values in UK and UNECE (method b).

| UK | AOT40 | J1 | %loss in benefit |
| --- | --- | --- | --- |
|  | Ppm hrs |  |  |
| IIASA Reference | 4.90 | 0.6 |  |
| UKREF | 4.82 |  |  |
| J1 | 4.30 | 0 |  |
| H1 | 4.11 | -0.29 | -48.3 |
| UK7 | 4.36 | 0.06 | 10.1 |
| UK8 | 4.39 | 0.09 | 15.3 |
| UK9 | 4.64 | 0.28 | 47.5 |
| UK10 | 4.60 | 0.24 | 40.7 |
| UNECE | AOT40 | J1 | %loss in benefit |
| IIASA Reference | 5.39 | 0.63 |  |
| UKREF | 5.38 |  |  |
| J1 | 4.76 | 0.00 |  |
| H1 | 4.94 | 0.18 | 28.6 |
| UK7 | 4.77 | 0.01 | 1.6 |
| UK8 | 4.77 | 0.01 | 1.6 |
| UK9 | 4.81 | 0.04 | 6.3 |
| UK10 | 4.81 | 0.04 | 6.3 |

- Table A5.2e.  Arable crop areas exposed to AOT40 in excess of threshold, 1000 km2.

|  | IIASA Reference | J1 |
| --- | --- | --- |
| UK | 63 | 58 |
| UNECE | 1995 | 1958 |

Table A5.2f.  Permanent crop areas exposed to AOT40 in excess of threshold, 1000 km2.

|  | IIASA Reference | J1 |
| --- | --- | --- |
| UK | 0.52 | 0.50 |
| UNECE | 135.1 | 135.1 |

- Scenarios explained in terms of VOC and NOx emissions

The basic scenarios examined are IIASA Reference, UKREF, H1 and J1.

UK10 is J1 with the emissions of the UK at the IIASA Reference values for NOx and VOC, i.e. designed to investigate the non-participation of the UK in further ozone reduction strategies.

UK7 is UK10 with VOC emission ceiling of 1150 kt/yr applied (approaching J1 at 1100); i.e. designed to investigate the non-participation of the UK in reducing NOx emissions.

UK8 is UK7 with NOx reduced from IIASA Reference value of 1181 to 1100 kt/yr

UK9 is UK10 with NOx reduced from IIASA Reference value of 1181 to 1100 kt/yr

These last two scenarios investigate the possibility of exchanging UK NOx reductions in place of UK VOC reductions.

- RESULTS

AOT60

a.  UK values of AOT60

IIASA Reference to J1 produces a benefit in terms of a reduction in UK AOT60 of 32%.  This applies whether mean AOT60, or excess AOT60 over the threshold of 0.4 ppm hours, is considered.

Moving from J1 to UK10, by relaxing the commitments of the UK to IIASA Reference, causes 36% of this benefit to be lost; for UK7, in which VOC emissions move closer to J1, only 6% is lost.  These losses are a result of the non-abatement of VOC in the UK.

Taking UK10, if NOx is now reduced below the J1 value to 1100 kt/yr (scenario UK9), whilst VOC remain at IIASA Reference, the loss increases to39%.  Taking UK7, if NOx is again reduced to 1100 kt/yr (scenario UK8) loss increases from 6 to 10%.  These increased losses demonstrate the increased ozone resulting from abating NOx beyond IIASA Reference whilst VOC remain at the IIASA Reference scenario (UK9) or between the IIASA Reference scenario and J1 (UK8).

Thus, if NOx is reduced below J1, much of the benefit of J1 in reducing ozone in the UK will be lost unless VOC are also reduced.  It is not beneficial, in terms of ozone reductions to trade off reductions in VOC, which may be difficult or expensive, with the less expensive reductions in NOx.

b.  UK Population Exposure to AOT60

Population exposures are similarly affected.  IIASA Reference to J1 produces a benefit through reducing population exposure in the UK by 36% (method a) or 33% (method b).

If UK emissions remain at IIASA Reference (scenario UK10) 21% of this benefit is lost.  If NOx emissions remain at IIASA Reference whilst VOC emissions approach J1 (scenario UK7), only 2% of this improvement is lost.

Taking UK10, if NOx is reduced below J1, loss increases to 23%.

Taking UK7, if NOx is reduced below J1, loss increases to 6%.

Thus the benefits of J1 to the UK in terms of population exposure to ozone are reduced by 21% if UK remains at the IIASA Reference scenario and by 23% if additional reductions in NOx occur.

c.  UNECE values

IIASA Reference to J1 produces a benefit through reducing UNECE AOT60 of 37%, whether mean values or mean excess values over the threshold are considered.

Only the more detailed analysis reveals the losses of up to 12% (of the improvement) observed for the scenarios in which UK VOC is not reduced from the IIASA Reference value of 1351 kt.  When VOC is reduced to 1150 kt/yr, losses are still 6%.

Thus, significant losses in benefit also accrue in Europe if UK VOC remains at IIASA Reference.

Correspondingly, population exposures in the UNECE region fall from IIASA Reference to J1 by 39% (method a) or 34% (method b).  The more detailed analysis (method b) indicates that leaving UK emissions at IIASA Reference (UK10) results in a 20% loss if this improvement.  Reducing VOC closer to J1 (UK7) reduces this loss to 4%.  Simultaneous reduction of NOx below J1 to 1100 kt/yr increases each of these losses by 1%.

Therefore, these investigations reinforce the message that at the J1 scenario it is important to reduce the VOC emissions to 1101 kt/yr if ozone is to be maintained at J1 levels.  Decreasing NOx and increasing VOC emissions causes AOT60 and population exposure to AOT60 to increase both in the UK and in Europe as a whole.

AOT40

a.  UK values of AOT40

IIASA Reference to J1 reduces UK mean AOT40 in excess of 3 ppm hours by 26% (method a) or 24% of mean AOT40 (method b).  Use of UK7 results in a loss of 10% of this benefit (method b) and UK10 a loss of 41% of the improvement, showing substantial increases in ozone if VOC are not reduced beyond IIASA Reference.  If NOx is reduced below IIASA Reference, the losses in benefit are larger, as high as 48% for the case where VOC remains at IIASA Reference and NOx is reduced to 1100 kt/yr.

b.   UNECE values of AOT40

IIASA Reference to J1 produces a reduction in UNECE mean AOT40 in excess of 3 ppm hours of 12 %.

Use of UK7 to 8 results in a loss of 1 to 2% of this European improvement, whilst scenarios UK9 to 10 affect this by 7%.  Thus, significant losses in benefit also accrue in Europe if UK VOC emissions remain at IIASA Reference.

c.  Vegetation exposure to AOT40

Vegetation exposures could only be calculated using method a.  IIASA Reference to J1 produces a reduction of 27% in the UK and 21% in the UNECE region.  UK7 produces a loss in these improvements of 12% and UK10, of 60%.  If NOx emissions are reduced to 1150 kt/yr whilst VOC remain at IIASA Reference, 69% of the benefit of J1 for vegetation exposure to ozone in the UK is lost.  Losses in the UNECE of about 5% accompany this.

d.  Crop exposure to AOT40

Of the 67,545 km2 of arable crop land in the UK, 66,041 km2 fall in EMEP grid cells for which RAINS indicated AOT40 levels in excess of the critical level for crops.  Therefore, the maximum area that could be exposed, according to RAINS, is 66,041 km2.

There is additional data for a total of 522 km2 permanent crops grown in the UK.  According to MAFF, permanent crops comprise orchards, hops, and other fruit.

There is a 7.5% reduction in the area of UK arable crops exposed to excess levels of AOT40 upon moving from IIASA Reference to J1.  The area concerned decreases from 63,000 to 58,000 km2.

The attached map A5.1 shows the location of arable crops in the UK, and the location of those exposed, for the two scenarios, is shown on maps A5.2 and A5.3.  The decrease in area exposed lies in Northern Ireland and S Scotland.

Since the area of permanent crops in the UK is rather small, the observed increases in protection levels are also small.

There are also considerable improvements in AOT40 levels in the areas of arable crops that remain exposed.  Crop exposure has specifically been calculated to fall from 178 ppm hours 1000 km2 (IIASA Reference) to 132 ppm hours 1000 km2 (J1) in the UK.

Maps A5.2 and A5.3 show the crop exposure in each EMEP grid cell in the UK for the two scenarios.  The biggest reductions in crop exposure are in the areas where most of the crops are to be found.  In (17,14) exposure falls from 356 to 266 ppm hours 100 km2 in excess of the threshold; whilst in (18,14) it falls from 307 to 247; and in (17,13) from 244 to 184.  These three grid cells hold the largest areas of arable crops in the UK.

- Conclusions

These investigations produce the message that if the full benefit of a J1 scenario is to be realised in the UK as far as exposures of humans to AOT60 and vegetation to AOT40 is concerned, it is important to reduce the VOC emissions to 1101 kt/yr.  This maintains ozone at J1 levels.  Decreasing NOx below J1 and allowing VOC emissions to increase above J1, in at attempt to ‘trade’ VOC reductions for NOx reductions, causes AOT60 and population exposure to AOT60 to increase both in the UK and in Europe as a whole.

For AOT40 and AOT60 the percentage of the benefit of J1 which is lost as a result of switching to the ‘UK’ suite of scenarios is rather large, both for human exposure and vegetation exposure.  Only the UK7 scenario, in which VOC is reduced to 1150 kt/yr whilst NOx remains at IIASA Reference, shows a relatively small increase in ozone exposures compared to J1.

The J1 scenario has considerable benefits in reducing crop exposure to AOT40 in the UK.  Application of alternative UK scenarios would show similar trends as seen for AOT40 vegetation exposure.

- ACIDIFICATION

- Relevant scenarios

Apart from the basic scenarios, two additional scenarios are examined in which UK sulphur emissions are increased from J1 (499 kt SO2/yr) to 525 (UKS11) and 700 (UKS13).  It is the effect of the increasing S emissions that is being examined here.

- Methods and Results

RAINS was used to extract areas protected from acidification in different countries under the various scenarios.  Maps of areas protected were also produced in order that effects in individual grid cells could be examined.

- Changes in area protection in individual grid cells

- Basic scenarios

This has not been calculated.

- Additional scenarios

The maximum change for J1 to UKS11 is in the UK where area protection from acidification decreases by 1.3% in a Welsh/English grid cell, EMEP (16,14) and by 0.7% in the N. Pennines (16,15).  Changes in the rest of the UK, and outside, are very small, less than 0.5%

The maximum change for J1 to UKS13 is in the UK where protection decreases by 9 and 5% in grid cells (16,14) and (16,15) respectively.  This means that 35% of each of these grid cells remains unprotected under UKS13 (compared to 30% and 26% at J1).  The main areas affected by the increase in S emissions are all of Wales, N England and S Scotland.

On moving from J1 to UKS13, losses of between 2 and 5% in area protected occur in coastal S Norway, S Wales, parts of Scotland and parts of E. Anglia.  Changes of 1 or 2 % occur in the rest of the UK (except N Ireland), coastal S Norway and Sweden, and the sensitive area along the German/Netherlands border.

- b.  Changes in area protection in countries

Table A5.3 gives the country changes in areas unprotected from acidification resulting from the basic scenarios.

(i) 	Basic scenarios

IIASA Reference to J1 reduces UK areas unprotected from acidification by 46%.  H1  is slightly less beneficial in the UK at 45%.

IIASA Reference to J1 reduces UNECE areas unprotected from acidification by 55%.  Use of H1 reduces this benefit by 69%.  Countries bearing this loss are mostly the non-EU countries, although there is some deterioration in Germany.

- Additional scenarios

UKS11: Increasing SO2 to 525 kt/yr instead of 499 kt/yr

This decreases this improvement in the UK by less than 4%, so that the area which is no longer protected is 0.2% of UK ecosystems sensitive to acidification.  In the ECE region, the percentage improvement does not change significantly.  Very small losses occur in Germany, Netherlands and Norway.

UKS13: Increasing SO2 to 700 kt/yr instead of 499 kt/yr

This decreases the improvement in the UK by 26% so that the total area unprotected increases to 5.1% (compared to 6.6% at J1).  This translates to a loss of protection for 140,000 hectares.  In the UNECE the improvement is decreased by less than 0.1%, but in Norway a further 0.3% of ecosystems lose protection.  Although 0.1% appears to be a small change; 276,000 hectares lose protection as compared with J1.

- Conclusions

In the UK the benefits associated with scenario J1 are similar to those associated with H1. However, this change in scenario has more significant impacts, in terms of a greater reduction in benefits, when analysed for the UNECE as a whole. The additional scenarios UKS11 and UKS13 indicate the likely impacts of relaxing the UK SO2 emission ceiling. A relaxation in the UK SO2 emission ceiling of 201kt/yr being associated with 140,000 hectares of unprotected UK land.

- EUTROPHICATION

- Methods and Results

RAINS was used to identify areas protected from eutrophication in different countries under the various scenarios.

Changes in area protection in countries for the basic scenarios

a.  UK

These are shown in Table 4.  IIASA Reference to J1 or H1 decreases UK areas unprotected from eutrophication by 50%.

By using the additional scenario UK1a, increases in NH3 emissions relative to J1 are compensated for by reductions in NOx.  However, in UK1, NH3 emissions are substantially larger than in J1 or UK1a whilst NOx emissions remain at J1.  Therefore these losses can be attributed to the increase in NH3 emissions from 264 kt/yr (J1) to 297 kt/yr (FOURTH).

b.  UNECE

IIASA Reference to J1 reduces UNECE areas unprotected from eutrophication by 19%, IIASA Reference to H1 by 2%.

Neither UK1a nor UK1 affect these values, nor any of the protection levels in non-EU countries (with the exception of UK1 which slightly increases exceedence in Switzerland).

In the EU, use of UK1a or UK1 increases exceedence in Belgium, Denmark, Germany, Luxembourg and the Netherlands, with UK1 having the most significant effect.

- Conclusions

A decrease in NOx emissions can compensate for a small increase in NH3 emissions in terms of eutrophication.  However, if NH3 emissions remain at IIASA Reference levels, and NOx is not reduced, there are very dramatic losses in eutrophication protection levels in the UK compared to J1, and additional losses are observed in nearby EU countries.

Table A5.3.  Country areas unprotected from acidification as a function of scenario (in percentage terms and as hectares) ( indicates difference between scenario and J1, a positive value indicating a greater area unprotected than for J1).

| Country | IIASA Reference | Base Line | J1 |  UK1a |  UK1 | H1 | IIASA Reference | Base Line | J1 |  UK1a |  UK1 | H1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (%) |  |  |  |  |  | (ha) |  |  |  |  |  |
| Austria | 3.2 | 3.2 | 1.4 | 0.0 | 0.0 | 2.0 | 162 | 161 | 68 | 0 | 0 | 99 |
| Belgium | 22.1 | 22.0 | 7.3 | 0.0 | 0.1 | 7.5 | 155 | 155 | 51 | 0 | 1 | 52 |
| Denmark | 2.3 | 2.2 | 1.2 | 0.0 | 0.0 | 1.5 | 9 | 9 | 5 | 0 | 0 | 6 |
| Finland | 4.3 | 4.3 | 2.8 | 0.0 | 0.0 | 4.2 | 1184 | 1180 | 756 | 0 | 2 | 1150 |
| France | 0.7 | 0.7 | 0.3 | 0.0 | 0.0 | 0.3 | 218 | 217 | 84 | 0 | 0 | 88 |
| Germany | 15.8 | 15.5 | 5.5 | 0.0 | 0.1 | 7.1 | 1616 | 1594 | 567 | 2 | 11 | 727 |
| Greece | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Ireland | 1.3 | 1.2 | 0.9 | 0.0 | 0.0 | 1.0 | 12 | 11 | 8 | 0 | 0 | 9 |
| Italy | 0.7 | 0.7 | 0.5 | 0.0 | 0.0 | 0.5 | 74 | 73 | 51 | 0 | 0 | 58 |
| Luxembg | 6.0 | 5.9 | 0.8 | 0.0 | 0.0 | 0.9 | 5 | 5 | 1 | 0 | 0 | 1 |
| Netherlands | 60.4 | 60.0 | 23.6 | 0.1 | 0.5 | 23.8 | 193 | 192 | 75 | 0 | 2 | 76 |
| Portugal | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 1 | 1 | 1 | 0 | 0 | 1 |
| Spain | 0.2 | 0.2 | 0.2 | 0.0 | 0.0 | 0.2 | 18 | 18 | 17 | 0 | 0 | 17 |
| Sweden | 4.1 | 4.1 | 3.0 | 0.0 | 0.0 | 3.7 | 1607 | 1580 | 1166 | 2 | 17 | 1420 |
| UK | 12.3 | 11.5 | 6.6 | 0.1 | 1.9 | 6.8 | 1180 | 1107 | 636 | 7 | 182 | 647 |
| EU15 | 4.3 | 4.2 | 2.3 | 0.0 | 0.1 | 2.9 | 6434 | 6302 | 3486 | 12 | 216 | 4352 |
| Albania | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Belarus | 20.9 | 20.8 | 13.6 | 0.0 | 0.0 | 20.6 | 1049 | 1048 | 686 | 0 | 1 | 1034 |
| Bosnia-H | 9.0 | 9.0 | 0.0 | 0.0 | 0.0 | 9.0 | 131 | 131 | 0 | 0 | 0 | 131 |
| Bulgaria | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Croatia | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Czech Rep | 17.8 | 17.7 | 3.0 | 0.0 | 0.1 | 10.7 | 474 | 470 | 81 | 0 | 2 | 285 |
| Estonia | 0.6 | 0.6 | 0.4 | 0.0 | 0.0 | 0.5 | 11 | 11 | 8 | 0 | 0 | 10 |
| Hungary | 22.9 | 22.9 | 13.0 | 0.0 | 0.0 | 18.9 | 65 | 65 | 37 | 0 | 0 | 54 |
| Latvia | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Lithuania | 4.1 | 4.1 | 0.3 | 0.0 | 0.0 | 4.1 | 78 | 78 | 5 | 0 | 0 | 77 |
| Norway | 11.6 | 11.4 | 8.7 | 0.0 | 0.2 | 10.1 | 2571 | 2521 | 1928 | 6 | 45 | 2236 |
| Poland | 7.8 | 7.7 | 1.0 | 0.0 | 0.0 | 6.4 | 1356 | 1344 | 173 | 0 | 1 | 1117 |
| Moldova | 2.4 | 2.4 | 0.9 | 0.0 | 0.0 | 2.4 | 29 | 29 | 10 | 0 | 0 | 29 |
| Romania | 0.8 | 0.8 | 0.3 | 0.0 | 0.0 | 0.8 | 51 | 51 | 17 | 0 | 0 | 51 |
| Russia | 1.2 | 1.2 | 0.3 | 0.0 | 0.0 | 1.2 | 4074 | 4072 | 1027 | 0 | 1 | 4061 |
| Slovakia | 14.7 | 14.7 | 7.4 | 0.0 | 0.0 | 13.0 | 295 | 295 | 149 | 0 | 0 | 261 |
| Slovenia | 2.1 | 2.1 | 0.4 | 0.0 | 0.0 | 2.0 | 19 | 19 | 4 | 0 | 0 | 19 |
| Switzerland | 4.6 | 4.6 | 2.8 | 0.0 | 0.0 | 3.2 | 57 | 56 | 35 | 0 | 0 | 40 |
| Macedonia | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Ukraine | 7.8 | 7.8 | 2.9 | 0.0 | 0.0 | 7.7 | 643 | 643 | 237 | 0 | 1 | 636 |
| Yugoslavia | 0.1 | 0.1 | 0.0 | 0.0 | 0.0 | 0.1 | 2 | 2 | 0 | 0 | 0 | 2 |
| Non-EU | 2.5 | 2.5 | 1.0 | 0.0 | 0.0 | 2.3 | 10907 | 10835 | 4398 | 7 | 51 | 10042 |
| TOTAL | 3.0 | 3.0 | 1.4 | 0.0 | 0.0 | 2.5 | 17340 | 17137 | 7884 | 19 | 267 | 14394 |

Table A5.4.  Country areas unprotected from eutrophication as a function of scenario (in percentage terms and as hectares) (Delta indicates difference between scenario and J1, a positive value indicating a greater area unprotected than for J1).

| Country | IIASA Reference | Base Line | J1 |  UK1a |  UK1 | H1 | IIASA Reference | Base Line | J1 |  UK1a |  UK1 | H1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (%) |  |  |  |  |  | (ha) |  |  |  |  |  |
| Austria | 57.7 | 57.8 | 41.5 | 0.0 | 0.0 | 46.8 | 3445 | 3447 | 2476 | 2 | 2 | 2796 |
| Belgium | 96.5 | 96.6 | 81.3 | 0.2 | 0.4 | 83.6 | 677 | 678 | 571 | 2 | 3 | 587 |
| Denmark | 37.8 | 38.2 | 26.8 | 0.1 | 0.2 | 29.0 | 119 | 120 | 84 | 0 | 1 | 91 |
| Finland | 15.3 | 15.4 | 10.5 | 0.0 | 0.0 | 13.1 | 2530 | 2535 | 1731 | 8 | 6 | 2158 |
| France | 79.2 | 79.2 | 68.1 | 0.0 | 0.0 | 70.9 | 25159 | 25161 | 21630 | 3 | 4 | 22525 |
| Germany | 89.5 | 89.6 | 71.3 | 0.1 | 0.1 | 72.9 | 9184 | 9189 | 7311 | 11 | 15 | 7479 |
| Greece | 9.6 | 9.6 | 3.5 | 0.0 | 0.0 | 8.6 | 236 | 236 | 85 | 0 | 0 | 212 |
| Ireland | 6.4 | 6.4 | 3.3 | 0.0 | 0.0 | 5.9 | 58 | 58 | 30 | 0 | 0 | 53 |
| Italy | 31.7 | 31.7 | 21.0 | 0.0 | 0.0 | 28.9 | 3795 | 3795 | 2512 | 1 | 1 | 3460 |
| Luxembg | 91.5 | 91.6 | 72.6 | 0.1 | 0.1 | 75.6 | 81 | 81 | 64 | 0 | 0 | 67 |
| Netherlands | 91.0 | 91.1 | 87.0 | 0.1 | 0.1 | 87.0 | 291 | 292 | 278 | 0 | 0 | 278 |
| Portugal | 27.5 | 27.5 | 22.4 | 0.0 | 0.0 | 26.4 | 776 | 776 | 632 | 0 | 0 | 747 |
| Spain | 13.9 | 13.9 | 10.2 | 0.0 | 0.0 | 11.6 | 1185 | 1186 | 868 | 1 | 2 | 987 |
| Sweden | 4.7 | 4.8 | 3.3 | 0.0 | 0.0 | 3.9 | 891 | 894 | 619 | 3 | 3 | 737 |
| UK | 1.4 | 2.6 | 0.7 | 0.0 | 0.4 | 0.7 | 126 | 238 | 62 | 1 | 34 | 63 |
| EU15 | 40.1 | 40.3 | 32.3 | 0.0 | 0.1 | 35.0 | 44554 | 48686 | 38954 | 31 | 72 | 42240 |
| Albania | 18.9 | 18.9 | 15.2 | 0.0 | 0.0 | 17.5 | 200 | 200 | 162 | 0 | 0 | 185 |
| Belarus | 25.7 | 25.7 | 18.4 | 0.0 | 0.0 | 25.1 | 1293 | 1294 | 924 | 1 | 1 | 1261 |
| Bosnia-H | 50.0 | 50.0 | 31.7 | 0.0 | 0.0 | 46.3 | 724 | 724 | 460 | 0 | 0 | 671 |
| Bulgaria | 68.7 | 68.7 | 25.5 | 0.0 | 0.0 | 65.8 | 3398 | 3398 | 1261 | 0 | 0 | 3258 |
| Croatia | 6.8 | 6.8 | 3.6 | 0.0 | 0.0 | 6.5 | 18 | 18 | 10 | 0 | 0 | 17 |
| Czech Rep | 87.0 | 87.0 | 74.5 | 0.0 | 0.1 | 82.8 | 2312 | 2313 | 1980 | 2 | 2 | 2200 |
| Estonia | 39.1 | 39.1 | 31.7 | 0.1 | 0.0 | 36.0 | 739 | 739 | 599 | 0 | 0 | 681 |
| Hungary | 52.8 | 52.8 | 44.1 | 0.0 | 0.0 | 51.8 | 150 | 151 | 126 | 0 | 0 | 148 |
| Latvia | 57.2 | 57.2 | 52.2 | 0.0 | 0.0 | 56.8 | 1553 | 1553 | 1418 | 1 | 1 | 1543 |
| Lithuania | 71.6 | 71.6 | 47.2 | 0.0 | 0.0 | 71.4 | 1357 | 1357 | 895 | 0 | 0 | 1353 |
| Norway | 2.0 | 2.0 | 0.3 | 0.0 | 0.0 | 0.4 | 280 | 285 | 35 | 0 | 1 | 58 |
| Poland | 93.5 | 93.5 | 85.9 | 0.0 | 0.0 | 92.6 | 16217 | 16219 | 14895 | 5 | 5 | 16062 |
| Moldova | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Romania | 40.1 | 40.1 | 28.4 | 0.0 | 0.0 | 39.2 | 2497 | 2497 | 1770 | 0 | 0 | 2444 |
| Russia | 7.6 | 7.6 | 6.7 | 0.0 | 0.0 | 7.5 | 26263 | 26286 | 23131 | 7 | 5 | 25877 |
| Slovakia | 75.2 | 75.3 | 46.8 | 0.0 | 0.0 | 71.1 | 1509 | 1510 | 938 | 1 | 1 | 1427 |
| Slovenia | 17.3 | 17.3 | 9.6 | 0.0 | 0.0 | 15.4 | 157 | 157 | 87 | 0 | 0 | 140 |
| Switzerland | 82.8 | 82.8 | 64.6 | 0.0 | 0.1 | 73.5 | 1886 | 1887 | 1472 | 1 | 1 | 1674 |
| Macedonia | 14.9 | 14.9 | 10.2 | 0.0 | 0.0 | 13.0 | 159 | 159 | 108 | 0 | 0 | 138 |
| Ukraine | 64.7 | 64.7 | 46.8 | 0.0 | 0.0 | 64.4 | 5332 | 5332 | 3860 | 0 | 0 | 5303 |
| Yugoslavia | 58.4 | 58.4 | 37.5 | 0.0 | 0.0 | 55.9 | 1992 | 1992 | 1278 | 0 | 0 | 1909 |
| Non-EU | 16.0 | 16.1 | 13.1 | 0.0 | 0.0 | 15.6 | 68037 | 68072 | 55407 | 19 | 18 | 66350 |
| TOTAL | 21.4 | 21.4 | 17.3 | 0.0 | 0.0 | 19.9 | 116591 | 116757 | 94360 | 51 | 90 | 108590 |

- HUMAN EXPOSURE TO SECONDARY PARTICULATE MATERIAL

- Method and Results

ASAM was used to calculate the concentrations of SO4, NO3 and NH4 aerosols under the four standard scenarios IIASA Reference, UKREF, H1 and J1.  Variants UKS11 and UKS13 were examined which are based on J1 but have higher emissions of SO2 in the UK.  Exposures were calculated by assuming that exposure to unit mass of any of the three components contributed equally to the overall exposure.  No threshold is used in calculating the population exposures.  The results are shown in Table A5.5.

(i)  Basic Scenarios

In the UK, IIASA Reference to J1 reduces human exposure to secondary particulates by 14%, incorporating a 31% reduction in the sulphate aerosol exposure.

The abatement of emissions from IIASA Reference to J1 causes a 23% reduction in UNECE human exposure to secondary particulates, which incorporates a 25% reduction in UNECE human exposure to sulphate aerosol.  H1 is rather less beneficial owing to the lack of reductions in non-EU countries.  Moving to UK1 has little effect on UNECE exposure levels

- Additional Scenarios

Moving from J1 to UKS11 has a negligible influence on these changes.

However, moving from J1 to UKS13 reduces the UNECE benefit to overall human exposure by 2%, whilst the UK benefit is reduced by 13%, a substantial change.

- Table A5.5.  ASAM calculations of human exposure to particulates for various scenarios

- Units: person g.

| SCENARIO | SO4 | NO3 | NH4 | TOTAL |
| --- | --- | --- | --- | --- |
| UNECE |  |  |  |  |
| IIASA Reference | 1922 | 4737 | 1307 | 7966 |
| UKREF | 1903 | 4738 | 1311 | 7952 |
| J1 | 1439 | 3714 | 1003 | 6155 |
| H1 | 1667 | 3895 | 1091 | 6652 |
| UKS11 | 1441 | 3714 | 1003 | 6158 |
| UKS13 | 1458 | 3714 | 1003 | 6174 |
| UK |  |  |  |  |
| IIASA Reference | 118 | 342 | 88 | 547 |
| UKREF | 108 | 342 | 90 | 541 |
| J1 | 82 | 315 | 73 | 470 |
| H1 | 84 | 316 | 75 | 474 |
| UKS11 | 83 | 315 | 73 | 472 |
| UKS13 | 92 | 315 | 73 | 480 |

- Conclusion

Increasing the UK SO2 emission ceiling from 499 kt/yr (J1) to 525 (UKS11) has little effect on human exposure to particulates.  However increasing the ceiling to 700 (UKS13) has a very significant effect in increasing human exposure to particulate matter in the UK, and effects are also seen in other parts of the UNECE region.

- OVERALL CONCLUSIONS

Almost any move away from J1 causes significant losses to the benefits, which would accrue in the UK as a result of moving to J1 from IIASA Reference.  In particular, VOC emissions need to be reduced in order to combat tropospheric ozone.  Increased VOC emissions cannot be compensated for by reduced NOx emissions below J1.

As far as acidification is concerned, NOx and NH3 emission can be ‘traded’.  However, if NH3 emissions remain at IIASA Reference and NOx is not reduced below IIASA Reference, there are very significant increases in eutrophication in the UK.  If NOx is reduced below IIASA Reference, then VOC emissions need to be reduced below J1 in order that tropospheric ozone does not increase.

Significant increases in SO2 emissions above J1 cause significant increases in acidification in the UK.  Human exposure to secondary particulates in this scenario also increases.

The scenario UK7 was the only one of the ‘additional scenarios’ which did not compare rather unfavourably with J1 for tropospheric ozone formation.

- Appendix 6
Methods for the Analysis of the Benefits of Abatement

- Overview

This Appendix identifies the data sources used for valuation, exposure-response and other data.

The overall structure of the analysis proceeds through the series of stages shown in Figure 2 of Chapter 5.  Emissions are modelled to provide data on the change in concentration and deposition across the UK (using the HARM and ELMO models) and Europe as a whole (using the EMEP models).  The dispersion/concentration maps are then overlaid onto data showing the distribution of the ‘stock at risk’ (people, buildings, crops, etc.), and information on the sensitivity of the stock at risk (death rates, age structure of the population, type of ecosystems, etc.).  This provides information on exposure, to which can be applied exposure-response functions to derive estimates of impact in biological and physical terms.  The final stage is valuation of impacts, where possible.

As stated elsewhere in this report, there are significant uncertainties in the assessment of the benefits of air pollution abatement.  However, we believe that it is possible to take an objective view in the analysis through comprehensive treatment of these uncertainties.

The analysis generated several types of result:

- UK benefits from UK abatement (‘Partial-UK’)

- UNECE benefits from UK abatement (‘UNECE’)

- UK benefits from pan-UNECE abatement (‘Full-UK’)

The codes in brackets are used below where appropriate.  The main models used were ALPHA for Partial-UK and UNECE, and ALPHA-UK for the Full-UK assessment.

The methods are ordered in accordance with the sensitivity analysis carried out in the full benefits assessment (see Appendix 8, which provides numeric inputs rather than just the sources that are given here).  Also described are the additional sensitivity analyses conducted within the main framework.

- Group i effects

- Materials damage

UK assessment: IGCB report (DETR, 1998)

UNECE assessment: Holland et al (1999)

- Crops damage

UK assessment: IGCB report (DETR, 1998)

UNECE assessment: Holland et al (1999)

- Ozone exposures

RAINS model, web version (see Appendix 5)

- Acidification and eutrophication

ITE databases (Full-UK, see also Appendix 7) and RAINS model, web version (Partial-UK and UNECE, see also Appendix 5).

- Particulate exposures

HARM, ASAM for the full-UK analysis.

EMEP for the Partial-UK and UNECE analysis.

- Acute effects on mortality and respiratory hospital admissions (RHAs)

Exposure response functions from COMEAP, including 95% confidence limits for error analysis.

- Group II Effects

- Numerous acute effects on morbidity

Functions and ranges from COMEAP and EAHEAP for the following effects:

- Cardiovascular disease

- Congestive heart failure

- Ischaemic heart disease

- Cerebrovascular hospital admissions

- Bronchodilator usage for asthmatic adults and children

- Cough for asthmatic adults and children

- Wheeze for asthmatic adults and children.

Note that to sum all of these effects would involve double counting.  Accordingly, when it comes to monetisation, estimates for cardiovascular disease (used for the lower bound accumulated benefits) are not added to congestive heart failure and ischaemic heart disease (which are used for the upper bound).  Also, there is debate on the meaning of the three effects on asthmatics, and whether there is potential there for double counting.  For the lower end of the monetised range only bronchodilator usage was included, whilst for the upper end all effects were included.

- Group III effects

- Monetisation of acute effects on mortality and RHAs

Acute mortality was valued using the EAHEAP range.  Lower bound estimates of cases (as calculated above) were combined with the lower bound from EAHEAP (£2,600), and vice versa for the upper bound (valued at £1.4 million/case).  An intermediate case taking the COMEAP mid-point and EAHEAP intermediate valuation of £110,000/case was also calculated, but not used in the accumulated ranges.

- Group IV effects

- Monetisation of various acute effects on morbidity (from GROUP II)

All effects valued using ExternE data (European Commission, 1995, 1999).  See also the note on Group II effects, describing the way in which data were combined to give the accumulated high and low estimates of benefit, to avoid possible double counting.

- Group V Effects

- Chronic mortality

This part of the analysis was based on work carried out by Fintan Hurley and Brian Miller at the Institute of Occupational Medicine, using the Pope et al function.  The underlying analysis is complex, being based on manipulation of life-tables.  The work is summarised in Markandya et al (1999) and a forthcoming report by Hurley et al for the Department of Health.  Numerous assumptions are available for the assessment of chronic mortality impacts.  For this analysis it was assumed that the change in risk was the same at all ages, with effects calculated for (essentially) a one-year reduction in particle concentrations.  Other assumptions are plausible and will be explored in the Hurley et al report.

Unlike the estimates of acute effects on mortality expressed in terms of number of cases (with no data on the reduction in life expectancy), the estimated chronic effects on mortality are quantified in terms of life years lost (with no data on the number of cases).  As the EAHEAP low (£2,600/case with an average of 1 month of life lost) and intermediate (£110,000/case with an average of 1 year of life lost) points provide an estimate of willingness to pay for increased longevity, we have applied these data.  There will be debate on the applicability of these valuations for chronic mortality.  However, it would appear to us to be curious if the final valuations agreed were so substantially different that they changed the results gained from applying this broad range.

- Group VI Effects

- Forests, numerous chronic and acute effects on morbidity

There is significant debate on the meaning of the effects listed in this Group.  In the UK, for example, chronic bronchitis is not an illness associated with children, though the US literature assesses it as an endpoint.  For chronic bronchitis in adults it would appear from the forthcoming report by Hurley et al (1999) that there are problems in distinguishing between those contracting bronchitis for the first time ever, and those suffering repeat episodes.  Similarly there are difficulties in interpretation of ‘restricted activity days’ (RADS).

In view of these problems with interpretation we have applied broad ranges for the valuation of the most prominent of this group of effects.  The main data sources are Holland et al (1999), Hurley et al (1999, forthcoming) and European Commission (1995, 1999).

The tables in Appendix 8 provide running totals for benefits, starting with those effects that can be quantified with the best confidence and proceeding through benefits of diminishing certainty.  This is done in such a way as to provide both lower and upper bounds.  High and low values are selected for inputs where required in order to derive these ranges.

- Data Used in the Analysis

Modelling Pollution Concentrations and Deposition

Most of the pollution dispersion and chemistry analysis carried out for this study was based on the use of the HARM and ELMO models and was described in Appendix IV.  This analysis was carried out to give a resolution of 10 x 10 km.  Analysis for the UNECE area was based on the EMEP 150 x 150 km grid.  For the UNECE scale, model runs were carried out at the Norwegian Meteorological Institute, relating emissions in a number of years to air concentrations and deposition of all of the pollutants relevant to this study.  Average transfer coefficients for each country to grid cell combination were calculated for a six year period (covering 1989 to 1994), in order to account for meteorological variation.  Data on ozone were supplied specifically for several of the scenarios assessed under separate analyses for the UNECE Task Force on Economic Aspects of Abatement Strategies, based on average meteorology over a five-year period.

Health Effects Assessment

Stock at risk data and atmospheric modelling

For the UK, population data were taken from the 1991 census, disaggregated by age, and expressed on a 10 x 10 km grid to match the pollution data.

The main source of population data used for the UNECE area analysis was the RIVM land use database (Veldkamp and van der Velde, 1995).  These data were transferred to the EMEP 150 x 150 grid, and disaggregated to urban and rural populations.  The Bosnian population, absent from the original inventory, was taken from UN sources.

Additional data were required to define the fraction of the population in various groups considered to be at special risk - the elderly, children, and asthmatics.  In addition, death rate data were required.  Data on age structure and death rates were obtained from Rayner et al (1994), drawing on relevant UN reports (Demographic Yearbook, Population and Vital Statistics Report and World Population Prospects).  Over Europe these provide average factors as follows;

Fraction of children in European population:	0.2

Fraction of adults in European population:	0.8

Fraction of people > 65 years in European population:	0.14

Annual death rate per thousand people:	10.2

The following estimates are made for asthmatics (R. Anderson, personal communication, October 1997);

Child asthmatics as a fraction of the UK population:	0.02

Adult asthmatics as a fraction of the UK population:	0.04

Exposure-response functions

Available literature on health effects of air pollution has been reviewed by the Department of Health (1998) and by Hurley, Donnan and their colleagues at IOM, providing the exposure-response functions listed in Table AII.1.  Preference is given to the DH preferred function where both it and supporting data on incidence of effects in the UK are available.

Table A6.1.  Quantification of human health impacts.  The exposure response slope, fer, has dimensions of [units/(yr-person-ug/m3)] for morbidity excluding RHAs, [% change in annual mortality,RHA rate/(ug/m3)] for acute effects on mortality ad RHAs, and years of life lost for chronic effects on mortality.

| Receptor | Impact Category |  |  | Reference |  |  |  |  |  |  |  | Pollutant |  |  |  | fer 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ASTHMATICS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| adults | Bronchodilator usage |  |  | Dusseldorp et al, 1995 |  |  |  |  |  |  |  | PM10 |  |  |  | 0.163 |  |  |  |  |
|  | Cough |  |  | Dusseldorp et al, 1995 |  |  |  |  |  |  |  | PM10 |  |  |  | 0.168 |  |  |  |  |
|  | Lower respiratory symptoms (wheeze) |  |  | Dusseldorp et al, 1995 |  |  |  |  |  |  |  | PM10 |  |  |  | 0.061 |  |  |  |  |
| children | Bronchodilator usage |  |  | Roemer et al, 1993 |  |  |  |  |  |  |  | PM10 |  |  |  | 0.078 |  |  |  |  |
|  | Cough |  |  | Pope, Dockery, 1992 |  |  |  |  |  |  |  | PM10 |  |  |  | 0.133 |  |  |  |  |
|  | Lower respiratory symptoms (wheeze) |  |  | Roemer et al, 1993 |  |  |  |  |  |  |  | PM10 |  |  |  | 0.103 |  |  |  |  |
| all | Asthma attacks (AA) |  |  | Whittemore, Korn, 1980 |  |  |  |  |  |  |  | O3 |  |  |  | 4.29E-3 |  |  |  |  |
| ELDERLY 65 years + |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Congestive heart failure (CHF) |  |  | Schwartz, Morris, 1995 |  |  |  |  |  |  |  | PM10 |  |  |  | 1.85E-5 |  |  |  |  |
| CHILDREN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Chronic bronchitis |  |  | Dockery et al, 1989 |  |  |  |  |  |  |  | PM10 |  |  |  | 1.61E-3 |  |  |  |  |
|  | Chronic cough |  |  | Dockery et al, 1989 |  |  |  |  |  |  |  | PM10 |  |  |  | 2.07E-3 |  |  |  |  |
| ADULTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Restricted activity days (RAD)3 | Ostro, 1987 |  |  |  |  |  |  |  | PM10 |  |  |  |  |  |  |  | 0.025 |  |  |
|  | Minor restricted activity day(MRAD)4 | Ostro, Rothschild, 1989 |  |  |  |  |  |  |  | O3 |  |  |  |  |  |  |  | 9.76E-3 |  |  |
|  | Chronic bronchitis | Abbey et al, 1995 |  |  |  |  |  |  |  | PM10 |  |  |  |  |  |  |  | 4.9E-5 |  |  |
| ENTIRE POPULATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Respiratory hospital admissions (RHA) | EAHEAP (1999) |  |  |  |  |  |  |  | PM10 SO2 O3 |  |  |  |  |  |  |  | 0.08% 0.05% 0.07% |  |  |
|  | Cerebrovascular hospital admissions (CVA) | Wordley et al, 1997 |  |  |  |  |  |  |  | PM10 |  |  |  |  |  |  |  | 5.04E-6 |  |  |
|  | Symptom days | Krupnick et al, 1990 |  |  |  |  |  |  |  | O3 |  |  |  |  |  |  |  | 0.033 |  |  |
| DEATH RATES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Acute Mortality | EAHEAP (1999) |  |  |  |  |  |  |  | PM10 SO2 O3 |  |  |  |  |  |  |  | 0.075%0.06% 0.06% |  |  |
|  | Chronic Mortality | Pope et al, 1995 |  |  |  |  |  |  |  | PM10 |  |  |  |  |  |  |  | 0.00036 |  |  |

1 Sources: [ExternE, European Commission, 1995; 1999] and [Hurley and Donnan, 1997].

3 Assume that all days in hospital for RHA, CHF and CVA are also restricted activity days (RAD).  Also assume that the average stay for each is 10, 7 and 45 days respectively.

Thus, net RAD = RAD - (RHA*10) - (CHF*7) - (CVA*45).

4  Assume asthma attacks are also MRAD, and hence should deducted from the MRAD total.

Use of the non-COMEAP functions listed here is based on the assumption that the incidence rates of different types of illness are the same in the UK and elsewhere in Europe as in the location where studies were originally undertaken.  The calculations required were carried out by Hurley and colleagues at IOM in the EC DGXII ExternE Project.  The following illustrative case was kindly supplied for this project by Fintan Hurley.  The Table numbers referred to are to be found in the paper by Roemer et al (1993).

Calculations on bronchodilator usage, from Roemer et al. (1993)

- Prevalence from Table 4.  Take a weighted average of 9.0 (27 days); 10.3 (32d); 11.5 (11d) and 12.1 (9d) to get 10.2.

- Regression coefficient from Table 6.  Treated as if it is linear regression; though it may be logistic.  The answers should be similar.  Taking it as linear, we have:

- Change in prevalence per g.m-3 PM10 is 0.023% or 0.23% per 10 g.m-3 PM10.

- With a s.e. of 0.008, the 95% CI is given by  2s.e.; i.e. 0.16%; so for 10 g.m-3 PM10 we have a 95% CI of 0.07%, 0.39%.

- Sticking with the main calculation, the average prevalence is 10.3%; and so, for an extra 10 g.m-3 PM10, we get a prevalence of 10.53%.

- The % increase in prevalence is 0.23/10.3 = 0.023, i.e. a RR of 1.023 per 10 g.m-3 PM10.

- Still taking it as linear, we have:

Per 1 g.m-3 PM10, an increase of 0.023% prevalence on a given day; i.e.

0.00023 x 365 = 0.084 occurrences per child asthmatic per year; i.e.

84 extra occurrences per 1000 child asthmatics per year, where ‘occurrence’ is the use of a bronchodilator on one day where otherwise there would have been no usage.

We used 77.9 rather than 84, because we applied the increase as a RR to the odds, as in logistic regression, not as a linear additive effect, as above.

The assessment of the chronic effects on mortality was based on a reasonably central series of model runs carried out at IOM using life tables and described in a separate study (Markandya et al, 1999).  The analysis assessed the effect of a one year pulse of pollution on the life expectancy of the UK.  It was assumed that the change in mortality risk was the same for all people exposed.  This analysis, and the effect of alternative assumptions will be described more fully in a forthcoming report to Department of Health (Hurley et al, 1999).

The ozone assessment was carried out assuming the presence of a threshold of 50 ppb only, following from the EAHEAP analysis.  This should not be taken as presumption against the no-threshold position, it was done simply because of a lack of modelled data for the UK to show trends in mean exposure to ozone for the scenarios considered.  The threshold calculations were carried out in a rather indirect manner, assuming that exposure to levels in excess of 50 ppb will be reflected in the change in AOT40 and AOT60.  Appropriate ozone data were limited to 1990 and 2010.  The assessment proceeded through the following stages:

- It was assumed that the results presented by EAHEAP were for a year in which average ozone was similar to 1990 concentrations.

- The EAHEAP ozone results assuming a 50 ppb threshold were divided by population-weighted AOT60 for 1990 to give deaths/RHAs per million person ppm.hrs.

- The result of [2] was multiplied by the change in AOT60 million person ppm.hrs to give the change in deaths and RHAs per scenario.

- Stages [2] and [3] were repeated using the AOT40.

- An average was taken of the results of [3] and [4].

Valuation data for mortality are given in the main text of the report, and were derived from EAHEAP.  For acute effects all three values adopted by EAHEAP were used to derive the ranges shown in the Tables of Appendix 8; low (£2600), intermediate (£110,000) and high (£1,400,000).  For chronic effects a range of £31,200 per year lost to £110,000 per year lost were used.  Derivation of these figures is given in the main text.

Following EAHEAP, a range of £0 to £3235 per respiratory hospital admission was used.  The lower figure assumes that all RHAs were simply events that were brought forward by a short time.  The upper figure combines WTP and costs of illness incurred per case by the NHS.

Other values were taken from the ExternE Project, and are shown in Table A6.2.

Table A6.2 Values used for assessment of morbidity impacts (1990 euro; Markandya, in European Commission, 1999).

| Endpoint | Value | Estimation Method and Comments |
| --- | --- | --- |
| Acute Morbidity |  |  |
| Restricted Activity Day (RAD) | 63 | CVM in US estimating WTP. |
| Symptom Day (SD) and Minor Restricted Activity Day | 6.3 | CVM in US estimating WTP. Account has been taken of Navrud’s study. |
| Chest Discomfort Day or Acute Effect in Asthmatics (Wheeze) | 6.3 | CVM in US estimating WTP. Same value applies to children and adults. |
| Emergency Room Visits (ERV) | 186 | CVM in US estimating WTP. |
| Cardiovascular Hospital Admissions | 6,560 | As above. |
| Acute Asthma Attack | 31 | COI (adjusted to allow for difference between COI and WTP). Applies to both children and adults. |
| Chronic Morbidity |  |  |
| Chronic Illness (VSC) | 1,000,000 | CVM in US estimating WTP. |
| Chronic Bronchitis in Adults | 88,000 | Rowe et al (1995). |
| Non fatal Cancer | 375,000 | US study. |
| Malignant Neoplasms | 375,000 | Valued as non-fatal cancer. |
| Chronic Case of Asthma | 88,000 | Based on treating chronic asthma as new cases of chronic bronchitis. |
| Cases of change in prevalence of bronchitis in children | 225 | Treated as cases of acute bronchitis. |
| Cases of change in prevalence of cough in children | 188 | As above. |

Damage to Materials

Stock at risk data

The stock at risk is derived from data on building numbers and construction materials taken from building survey information.  Sources of data are as follows;

Eastern Europe (including the former East Germany):

Kucera et al (1993b), Tolstoy et al (1990) - data for Prague

Scandinavia:

Kucera et al (1993b), Tolstoy et al (1990) - data for Stockholm and Sarpsborg

UK, Ireland:

Ecotec (1996), data for UK extrapolated to Ireland

Former West Germany:

Hoos et al (1987) - data for Dortmund and Köln

Other western Europe:

Average of material use per person from Hoos et al, Kucera et al and Tolstoy et al (excluding Prague), and Ecotec.

For galvanised steel in structural (non-building) applications an average was derived from European Commission (1995) and Kucera et al (1993b).

Meteorological, atmospheric and background pollution data

The exposure-response functions require data on meteorological conditions.  Of these, the most important are precipitation and humidity.  Data have been taken from Kucera (1994).

Dose-response functions

The main source of data for exposure response functions used here is the work conducted under the UNECE Programme (Kucera, 1993a, 1993b, 1994).  This section lists the dose-response functions used, which should be assumed to originate from the work of Kucera unless otherwise referenced.  The following key applies to all equations given:

ER	=	erosion rate (um/year)

P	=	precipitation rate (m/year)

SO2	=	sulphur dioxide concentration (ug/m3)

O3	=	ozone concentration (ug/m3)

H+	=	acidity (meq/m2/year)

RH	=	average relative humidity, %

f1	=	1-exp[-0.121.RH/(100-RH)]

TOW	=	fraction of time relative humidity exceeds 80% and temperature >0°C

ML	=	mass loss (g/m2) after 4 years

In all the ICP functions, the original H+ concentration term (in mg/l) has been replaced by an acidity term using the conversion:

P.H+ (mg/l) = 0.001.H+ (acidity in meq/m2/year)

To convert mass loss for stone and zinc into an erosion rate in terms of material thickness, respective densities of 2.0 and 7.14 tonnes/m3 are assumed.  The functions used are as follows;

Unsheltered limestone (4 years):  ML = 8.6 + 1.49.TOW.SO2 + 0.097.H+

Unsheltered sandstone (4 years) (also mortar): ML = 7.3 + 1.56.TOW.SO2 + 0.12.H+

Brickwork: no effect

Concrete; assumed no effect, though air pollution may affect steel reinforcement

Carbonate paint: ER/tc = 0.01[P]8.7(10-pH - 10-5.2)+0.006.SO2.f1 (Haynie, 1986)

Silicate paint: ER/tc = 0.01[P]1.35(10-pH - 10-5.2)+0.00097.SO2.f1 (Haynie, 1986)

Steel: assumed either painted or galvanised, not assessed independently

Unsheltered zinc (4 years): ML  =   14.5 + 0.043.TOW.SO2.O3 + 0.08.H+

Sheltered zinc (4 years): ML =    5.5 + 0.013.TOW.SO2.O3

Aluminium: assumed too corrosion resistant to be affected significantly.

Calculation of repair frequency

It is assumed that maintenance is ideally carried out after a given thickness of material has been lost.  This parameter is set to a level beyond which basic or routine repair schemes may be insufficient, and more expensive remedial action would be needed.  A summary of the critical thickness loss for maintenance and repair are shown in Table AII.6.

Table A6.3.  Assumed critical thickness for maintenance or repair measures for building materials.

| Material | Critical thickness loss |
| --- | --- |
| Natural stone | 5 mm |
| Rendering | 5 mm |
| Mortar | 5 mm |
| Zinc: Construction - sheet and strip Other construction, agriculture and street furniture Pylons, other transport | 25 um 50 um 100 um |
| Galvanised steel | 50 um |
| Paint | 20 um |

Repair costs

The following repair costs were applied for the interval in material lifetime between scenarios defined by the application of data on concentration, dose-response and critical thickness.

Table A6.4.  Repair and maintenance costs [euro/m2] applied in this analysis.

| Material | euro/m2 |
| --- | --- |
| Zinc | 21 |
| Galvanised steel | 25 |
| Natural stone | 235 |
| Rendering, mortar | 25 |
| Paint | 11 |

Effects of Air Pollution on Agricultural Systems

Geographically distributed data on crop production were obtained from ITE for the UK.  Across the rest of Europe the data adopted were national totals for production of each crop type.

Acidification of agricultural soils

Lime is routinely applied to farmland to counteract acidification linked to farming practises, including harvest.  Atmospheric deposition increases the amount of lime required to maintain acidity levels.  The basis of the method applied here to calculate the costs associated with this change in demand for lime is as follows:

- The total amount of acidifying pollutant deposited to the land surface in a given area;

- The amount which falls on soils which require lime (excluding, for example, urban areas, water and soils on calcareous drifts);

- The cost of neutralising this amount of acidic deposition with lime;

- The increased acidic deposition in this area resulting from the change from one scenario to another;

- The additional cost of neutralising the difference in inputs to soils, which require lime (priced at 16.8 euro per tonne of lime).

Fertilisational effects of nitrogen deposition

Nitrogen is of course an essential plant nutrient, applied by farmers in large quantity to their crops.  The deposition of additional nitrogen to agricultural soils is thus beneficial (assuming that the dosage of any fertiliser applied by a farmer is not excessive).  The analysis quantifies total deposition of nitrogen to arable land and permanent pastures.  The benefit is calculated directly from the cost of nitrate fertiliser, 430 euro /tonne of nitrogen (Nix, 1990).  Given that additional inputs will still be needed under current conditions to meet crop N requirements for intensive agricultural systems there is a negligible saving in the time required for fertiliser application (if any), so it seems reasonable to cost benefits purely in terms of the (perhaps theoretical) reduction in N required as fertiliser.  This analysis probably tends to overestimates the benefit of N deposition.  N is deposited from the atmosphere throughout the year, including times when crops are not actively growing.  The potential for deposited N to drain off and cause eutrophication is not monetised.

Similar analysis has not been performed for afforested areas.  There is concern that prolonged deposition of N to these areas can lead to nutrient imbalance (Schulze et al, 1989), and hence that observed benefits in terms of enhanced productivity are not sustainable.

Ozone effects

Ozone damage to crops has been calculated using EMEP’s accumulated ozone above a threshold of 40 ppb (AOT40) metric, where AOT40 is defined by:

AOT40=max(O3−40,0).dt

The time integral is over the growing season, which, for crops, is taken to be daylight hours in the months May-July.  The metric has the units ppb.hours, or ppm.hours.

Functions are listed in tables AII.8 and AII.9.

Table A6.5. Estimated sensitivity of different crops to ozone.  Species written in normal type are discussed in the review by Jones et al (1997).  Species written in italics are not specifically discussed by Jones et al, but do feature in European crop production statistics.  Sensitivity in these cases is estimated by analogy with similar crops..

| Tolerant crops | maize raspberries cabbages | barley olives | leaf crops olive oil | sugar beet strawberries |
| --- | --- | --- | --- | --- |
| Slightly sensitive crops | pasture grass rice | sorghum millet | oats | rye |
| Sensitive crops | wheat potato apples lemons limes flax hemp | clover tomato oranges peaches pears hops linseed | soybeans sunflower plums grapefruit tangerine onion rapeseed | beans grapes watermelons carrots cucumbers dates sesame seed |
| Very sensitive crops | tobacco |  |  |  |

Table A6.6.  Ozone exposure-response functions.

| Crop type | Exposure Response Function |
| --- | --- |
|  | % loss per ppm.hour AOT40 |
| Tolerant crops | 0 |
| Slightly sensitive crops | 1.0 |
| Sensitive crops | 1.75 |
| Very sensitive crops | 3.57 |
| Meat and milk products | 0.5 |

SO2 effects

The following functions were used to quantify % yield change (y) from SO2 effects on agriculture, derived from the work of Baker et al (1986), accounting for the fertilisational effect of sulphur at low concentration (European Commission, 1995);:

y = 0.74(SO2) - 0.055(SO2)2 	(from 0 to 13.6 ppb SO2)

y = -0.69(SO2) + 9.35 		(above 13.6 ppb SO2)

These functions have been applied to the following crops:

| maize | barley | wheat | sorghum |
| --- | --- | --- | --- |
| oats | rye | millet | rice |
| leaf crops | sugar beet | raspberries | strawberries |
| soybeans | beans | potato | tomato |
| sunflower | carrots | cucumber | flax |
| hops | hemp | linseed | sesame seed |
| tobacco |  |  |  |

For pasture the following function has been used, based on a review by Roberts (1984).  All data used to derive the functions was taken from studies on Lolium perenne, the most common pasture grass in Europe.  Again, the functions have been adapted to account for fertilisation of crops below the lowest exposure adopted experimentally.

y = 0.20(SO2) - 0.013(SO2)2 	(from 0 to 15.3 ppb)

y = -0.18(SO2) + 2.75 		(above 15.3 ppb)

Meat and milk production are assumed to be 50% as sensitive as pasture grass, on which livestock are primarily dependent for food.

Valuation of crop losses

Valuation of crop losses has been undertaken using prices from United Nations Food and Agriculture Organisation (FAO, 1994).

Ecosystem Damage

See Appendices V and VII.

Forest Damage

Two functions have been applied for assessment of ozone effects on forest productivity (Karenlampi and Skarby, 1996):

Species: beech;  % productivity change = -0.27x

Species: Norway spruce;  % productivity change = -0.18x

where x = ozone expressed as AOT40, (the ozone concentration accumulated over a threshold of 40 ppb in daylight hours over the growing season), expressed in ppm.hours.

An average is taken of the results of applying the two functions to all European forested areas.  Given the uncertainties in derivation of the two functions it is not appropriate to provide separate quantification of impacts for hard and soft woods.  The analysis probably overestimates the damages linked to forest damage from ozone, on the grounds that forest managers would probably respond to mitigate damage through alterations to management procedures over the lifetime of forests.  However, there is also scope for underestimation of overall effects on forest productivity given that no economic assessment is made of the effects on forestry of acidification, N deposition and exposure to SO2 due to a lack of data.  Impacts are quantified only to the extent of identifying areas in which ecosystems (including forests) experience exceedence of critical loads and critical levels.

Appendix 7
Detailed results for acidification and eutrophication

- Introduction

The following results were provided by ITE, following analysis based on the outputs of the HARM model.  The results are disaggregated by region and ecosystem type for acidification and eutrophication.  Results are shown one scenario on each page. Ecosystem area and Exceedence area are both measured in ha.

- Acidification

| UKREF scenario |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| England | Ecosystem Area |  | Exceedence Area | %Exceedence Area | Accum Exceedence |
| Acid grassland | 1126445 |  | 424663 | 37.7 | 83823 |
| Calcareous grassland | 855031 |  | 0 | 0.0 | 0 |
| Heath | 139460 |  | 33949 | 24.3 | 2838 |
| Coniferous woodland | 181039 |  | 12535 | 6.9 | 1617 |
| Deciduous woodland | 647548 |  | 35947 | 5.6 | 4586 |
| Freshwater | 142861 |  | 9495 | 6.6 | 2850 |
| All Ecosystems | 3092383 |  | 516588 | 16.7 | 95714 |
| Northern Ireland | Ecosystem Area |  | Exceedence Area | %Exceedence Area | Accum Exceedence |
| Acid grassland | 132145 |  | 260 | 0.2 | 22 |
| Calcareous grassland | 146958 |  | 0 | 0.0 | 0 |
| Heath | 31176 |  | 0 | 0.0 | 0 |
| Coniferous woodland | 50629 |  | 0 | 0.0 | 0 |
| Deciduous woodland | 26839 |  | 0 | 0.0 | 0 |
| Freshwater | 0 |  | 0 | 0.0 | 0 |
| All Ecosystems | 387747 |  | 260 | 0.1 | 22 |
| Scotland | Ecosystem Area |  | Exceedence Area | %Exceedence Area | Accum Exceedence |
| Acid grassland | 3736432 |  | 232950 | 6.2 | 23070 |
| Calcareous grassland | 0 |  | 0 | 0.0 | 0 |
| Heath | 709889 |  | 2387 | 0.3 | 171 |
| Coniferous woodland | 429653 |  | 19192 | 4.5 | 2317 |
| Deciduous woodland | 122453 |  | 1705 | 1.4 | 93 |
| Freshwater | 171155 |  | 453 | 0.3 | 97 |
| All Ecosystems | 5169582 |  | 256687 | 5.0 | 25747 |
| Wales | Ecosystem Area |  | Exceedence Area | %Exceedence Area | Accum Exceedence |
| Acid grassland | 462380 |  | 251480 | 54.4 | 59492 |
| Calcareous grassland | 14401 |  | 0 | 0.0 | 0 |
| Heath | 111727 |  | 24328 | 21.8 | 4750 |
| Coniferous woodland | 76802 |  | 3228 | 4.2 | 830 |
| Deciduous woodland | 236780 |  | 8340 | 3.5 | 1415 |
| Freshwater | 26439 |  | 700 | 2.6 | 128 |
| All Ecosystems | 928528 |  | 288077 | 31.0 | 66616 |

| J1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| England | Ecosystem Area | Exceedence Area | %Exceedence Area | Accum Exceedence |
| Acid grassland | 1126445 | 261187 | 23.2 | 29698 |
| Calcareous grassland | 855031 | 0 | 0.0 | 0 |
| Heath | 139460 | 5265 | 3.8 | 274 |
| Coniferous woodland | 181039 | 8805 | 4.9 | 403 |
| Deciduous woodland | 647548 | 10698 | 1.7 | 1170 |
| Freshwater | 142861 | 6841 | 4.8 | 1584 |
| All Ecosystems | 3092383 | 292797 | 9.5 | 33128 |
| Northern Ireland | Ecosystem Area | Exceedence Area | %Exceedence Area | Accum Exceedence |
| Acid grassland | 132145 | 61 | 0.0 | 4 |
| Calcareous grassland | 146958 | 0 | 0.0 | 0 |
| Heath | 31176 | 0 | 0.0 | 0 |
| Coniferous woodland | 50629 | 0 | 0.0 | 0 |
| Deciduous woodland | 26839 | 0 | 0.0 | 0 |
| Freshwater | 0 | 0 | 0.0 | 0 |
| All Ecosystems | 387747 | 61 | 0.0 | 4 |
| Scotland | Ecosystem Area | Exceedence Area | %Exceedence Area | Accum Exceedence |
| Acid grassland | 3736432 | 80608 | 2.2 | 5356 |
| Calcareous grassland | 0 | 0 | 0.0 | 0 |
| Heath | 709889 | 551 | 0.1 | 32 |
| Coniferous woodland | 429653 | 13318 | 3.1 | 676 |
| Deciduous woodland | 122453 | 192 | 0.2 | 3 |
| Freshwater | 171155 | 331 | 0.2 | 62 |
| All Ecosystems | 5169582 | 94999 | 1.8 | 6129 |
| Wales | Ecosystem Area | Exceedence Area | %Exceedence Area | Accum Exceedence |
| Acid grassland | 462380 | 116538 | 25.2 | 21567 |
| Calcareous grassland | 14401 | 0 | 0.0 | 0 |
| Heath | 111727 | 9389 | 8.4 | 1129 |
| Coniferous woodland | 76802 | 2314 | 3.0 | 258 |
| Deciduous woodland | 236780 | 4926 | 2.1 | 127 |
| Freshwater | 26439 | 310 | 1.2 | 62 |
| All Ecosystems | 928528 | 133477 | 14.4 | 23142 |

| H1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| England | Ecosystem Area | Exceedence Area | %Exceedence Area | Accum Exceedence |
| Acid grassland | 1126445 | 284638 | 25.3 | 36393 |
| Calcareous grassland | 855031 | 0 | 0.0 | 0 |
| Heath | 139460 | 6816 | 4.9 | 395 |
| Coniferous woodland | 181039 | 11160 | 6.2 | 593 |
| Deciduous woodland | 647548 | 13903 | 2.1 | 1404 |
| Freshwater | 142861 | 7896 | 5.5 | 1755 |
| All Ecosystems | 3092383 | 324414 | 10.5 | 40540 |
| Northern Ireland | Ecosystem Area | Exceedence Area | %Exceedence Area | Accum Exceedence |
| Acid grassland | 132145 | 77 | 0.1 | 5 |
| Calcareous grassland | 146958 | 0 | 0.0 | 0 |
| Heath | 31176 | 0 | 0.0 | 0 |
| Coniferous woodland | 50629 | 0 | 0.0 | 0 |
| Deciduous woodland | 26839 | 0 | 0.0 | 0 |
| Freshwater | 0 | 0 | 0.0 | 0 |
| All Ecosystems | 387747 | 77 | 0.0 | 5 |
| Scotland | Ecosystem Area | Exceedence Area | %Exceedence Area | Accum Exceedence |
| Acid grassland | 3736432 | 105322 | 2.8 | 7727 |
| Calcareous grassland | 0 | 0 | 0.0 | 0 |
| Heath | 709889 | 658 | 0.1 | 48 |
| Coniferous woodland | 429653 | 14994 | 3.5 | 945 |
| Deciduous woodland | 122453 | 465 | 0.4 | 11 |
| Freshwater | 171155 | 351 | 0.2 | 70 |
| All Ecosystems | 5169582 | 121790 | 2.4 | 8801 |
| Wales | Ecosystem Area | Exceedence Area | %Exceedence Area | Accum Exceedence |
| Acid grassland | 462380 | 136249 | 29.5 | 25592 |
| Calcareous grassland | 14401 | 0 | 0.0 | 0 |
| Heath | 111727 | 11826 | 10.6 | 1495 |
| Coniferous woodland | 76802 | 2594 | 3.4 | 333 |
| Deciduous woodland | 236780 | 5990 | 2.5 | 275 |
| Freshwater | 26439 | 332 | 1.3 | 69 |
| All Ecosystems | 928528 | 156991 | 16.9 | 27764 |

- Eutrophication

| UKREF scenario |  |  |  |  |
| --- | --- | --- | --- | --- |
| England | Ecosystem Area | Exceedence Area | %Exceedence Area | Accum Exceedence |
| Acid grassland | 1126445 | 101715 | 9.0 | 8933 |
| Calcareous grassland | 855031 | 0 | 0.0 | 0 |
| Heath | 139460 | 14981 | 10.7 | 1289 |
| Coniferous woodland | 181039 | 1020 | 0.6 | 77 |
| Deciduous woodland | 647103 | 8591 | 1.3 | 845 |
| All Ecosystems | 2949078 | 126308 | 4.3 | 11145 |
| Northern Ireland | Ecosystem Area | Exceedence Area | %Exceedence Area | Accum Exceedence |
| Acid grassland | 132064 | 0 | 0.0 | 0 |
| Calcareous grassland | 146958 | 0 | 0.0 | 0 |
| Heath | 31176 | 0 | 0.0 | 0 |
| Coniferous woodland | 50629 | 0 | 0.0 | 0 |
| Deciduous woodland | 26839 | 0 | 0.0 | 0 |
| All Ecosystems | 387666 | 0 | 0.0 | 0 |
| Scotland | Ecosystem Area | Exceedence Area | %Exceedence Area | Accum Exceedence |
| Acid grassland | 3736432 | 0 | 0.0 | 0 |
| Calcareous grassland | 0 | 0 | 0.0 | 0 |
| Heath | 709889 | 0 | 0.0 | 0 |
| Coniferous woodland | 429653 | 0 | 0.0 | 0 |
| Deciduous woodland | 122453 | 0 | 0.0 | 0 |
| All Ecosystems | 4998427 | 0 | 0.0 | 0 |
| Wales | Ecosystem Area | Exceedence Area | %Exceedence Area | Accum Exceedence |
| Acid grassland | 462380 | 35540 | 7.7 | 4956 |
| Calcareous grassland | 14401 | 0 | 0.0 | 0 |
| Heath | 111727 | 12046 | 10.8 | 2022 |
| Coniferous woodland | 76632 | 2181 | 2.8 | 172 |
| Deciduous woodland | 236728 | 8584 | 3.6 | 694 |
| All Ecosystems | 901867 | 58352 | 6.5 | 7844 |

| J1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| England | Ecosystem Area | Exceedence Area | %Exceedence Area | Accum Exceedence |
| Acid grassland | 1126445 | 47912 | 4.3 | 2669 |
| Calcareous grassland | 855031 | 0 | 0.0 | 0 |
| Heath | 139460 | 6801 | 4.9 | 451 |
| Coniferous woodland | 181039 | 259 | 0.1 | 35 |
| Deciduous woodland | 647103 | 2628 | 0.4 | 401 |
| All Ecosystems | 2949078 | 57600 | 2.0 | 3556 |
| Northern Ireland | Ecosystem Area | Exceedence Area | %Exceedence Area | Accum Exceedence |
| Acid grassland | 132064 | 0 | 0.0 | 0 |
| Calcareous grassland | 146958 | 0 | 0.0 | 0 |
| Heath | 31176 | 0 | 0.0 | 0 |
| Coniferous woodland | 50629 | 0 | 0.0 | 0 |
| Deciduous woodland | 26839 | 0 | 0.0 | 0 |
| All Ecosystems | 387666 | 0 | 0.0 | 0 |
| Scotland | Ecosystem Area | Exceedence Area | %Exceedence Area | Accum Exceedence |
| Acid grassland | 3736432 | 0 | 0.0 | 0 |
| Calcareous grassland | 0 | 0 | 0.0 | 0 |
| Heath | 709889 | 0 | 0.0 | 0 |
| Coniferous woodland | 429653 | 0 | 0.0 | 0 |
| Deciduous woodland | 122453 | 0 | 0.0 | 0 |
| All Ecosystems | 4998427 | 0 | 0.0 | 0 |
| Wales | Ecosystem Area | Exceedence Area | %Exceedence Area | Accum Exceedence |
| Acid grassland | 462380 | 21790 | 4.7 | 1809 |
| Calcareous grassland | 14401 | 0 | 0.0 | 0 |
| Heath | 111727 | 8511 | 7.6 | 856 |
| Coniferous woodland | 76632 | 834 | 1.1 | 15 |
| Deciduous woodland | 236728 | 1724 | 0.7 | 103 |
| All Ecosystems | 901867 | 32859 | 3.6 | 2782 |

| H1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| England | Ecosystem Area | Exceedence Area | %Exceedence Area | Accum Exceedence |
| Acid grassland | 1126445 | 50276 | 4.5 | 3276 |
| Calcareous grassland | 855031 | 0 | 0.0 | 0 |
| Heath | 139460 | 7115 | 5.1 | 525 |
| Coniferous woodland | 181039 | 269 | 0.1 | 37 |
| Deciduous woodland | 647103 | 2643 | 0.4 | 423 |
| All Ecosystems | 2949078 | 60302 | 2.0 | 4261 |
| Northern Ireland | Ecosystem Area | Exceedence Area | %Exceedence Area | Accum Exceedence |
| Acid grassland | 132064 | 0 | 0.0 | 0 |
| Calcareous grassland | 146958 | 0 | 0.0 | 0 |
| Heath | 31176 | 0 | 0.0 | 0 |
| Coniferous woodland | 50629 | 0 | 0.0 | 0 |
| Deciduous woodland | 26839 | 0 | 0.0 | 0 |
| All Ecosystems | 387666 | 0 | 0.0 | 0 |
| Scotland | Ecosystem Area | Exceedence Area | %Exceedence Area | Accum Exceedence |
| Acid grassland | 3736432 | 0 | 0.0 | 0 |
| Calcareous grassland | 0 | 0 | 0.0 | 0 |
| Heath | 709889 | 0 | 0.0 | 0 |
| Coniferous woodland | 429653 | 0 | 0.0 | 0 |
| Deciduous woodland | 122453 | 0 | 0.0 | 0 |
| All Ecosystems | 4998427 | 126 | 0.0 | 0 |
| Wales | Ecosystem Area | Exceedence Area | %Exceedence Area | Accum Exceedence |
| Acid grassland | 462380 | 25538 | 5.5 | 2135 |
| Calcareous grassland | 14401 | 0 | 0.0 | 0 |
| Heath | 111727 | 10393 | 9.3 | 990 |
| Coniferous woodland | 76632 | 898 | 1.2 | 29 |
| Deciduous woodland | 236728 | 2678 | 1.1 | 143 |
| All Ecosystems | 901867 | 39507 | 4.4 | 3297 |

- Appendix 8
Full Results of the Benefits Analysis

Results are presented here for the two scenarios, J1 and H1 in that order, with results in all cases incremental to the UKREF scenario for the UK.  Results for each scenario are presented in three sets:

- Benefits in the UK linked to UK abatement

- Benefits in UNECE from UK abatement

- Benefits in the UK (by region) from UNECE-wide abatement.

Results are expressed as impacts or monetary benefits for one year only (2010).  There will be additional benefits before and after 2010 of course, just as there will be additional costs.  For chronic effects (on health, forests, etc.) benefits are quantified such that they include benefits in years beyond 2010 arising from emissions abatement in that year compared to UKREF

The sources of the input data were shown in Appendix 6: exposure-response functions and valuation data are given in column 3 of each table.  Reference to Appendix 6 is needed to identify the units of the functions given.

The Tables provide the basis for the sensitivity analysis in this report.  We start with the effects that can be quantified with the greatest confidence and proceed sequentially through to the most debatable effects.  To this is added additional sensitivity analysis, providing high and low estimates for the most significant effects, and for their valuation.  This allows the development of ranges down through the Tables for the accumulated high and low totals.  These totals are coloured red to the point where benefits outweigh costs, the switching point being coloured green.

Yellow blocks contain data on the physical/biological effect of pollution abatement, and blue blocks the monetised equivalents.  Impacts may be separated from their monetised equivalent where the monetisation process is considered to introduce significant uncertainty.

All cost data are given in 1990£ million, for direct comparison with the cost estimates.  Unit valuation data given in column 3 of each data are, however, expressed in the original units (1990 euro).

For each type of effect we have sought to calculate upper and lower bounds.  These are then fed through to the upper and lower running totals respectively.  Final ranges are extremely broad, though we believe still useful (see section 6.9).

- Appendix 9
References

Abbey D.E., Lebowitz M.D., Mills P.K., Petersen F.F., Lawrence Beeson W. and Burchette R.J. (1995) Long-term ambient concentrations of particulates and oxidants and development of chronic disease in a cohort of nonsmoking California residents. Inhalation Toxicology 7, 19-34.

AEA Technology (1998a) Cost Benefit Analysis of Proposals Under the UNECE Multi-Pollutant, Multi-Effect Protocol.  Contract report for the UK Department of the Environment, Transport and the Regions.

AEA Technology (1998b) Economic Evaluation of Proposals for Emission Ceilings for Atmospheric Pollutants.  Report for European Commission, DGXI.

AEA Technology (1999a) Economic Evaluation of Proposals Under the UNECE Multi-Effects and Multi-Pollutant Protocol. Report for European Commission, DGXI.

AEA Technology (1999b) Economic Evaluation of Proposals for Emission Ceilings for Atmospheric Pollutants.  Report for European Commission, DGXI.

AEA Technology (1999c) Cost-benefit analysis for the Protocol to abate acidification, eutrophication and ground level ozone in Europe.  Contract report for the Dutch Ministry of Environment, Spatial Planning and Housing (MVROM), The Hague, Netherlands.

Amann, M., Bertok, I., Cofala, J., Gyarfas, F., Heyes, C., Klimont, Z., Makowski, M. Schopp, W. and Syri, S. (1999a)  Cost effective control of acidification and ground level ozone.  Seventh Interim report to the European Commission, DGXI.

Amann, M., Bertok, I., Cofala, J., Gyarfas, F., Heyes, C., Klimont, Z., Makowski, M. Schopp, W. and Syri, S. (1999) Integrated Assessment Modelling for the Protocol to Abate Acidification, Eutrophication and Ground-level Ozone in Europe.  Air & Energy 132, Ministry of Housing, Spatial Planning and the Environment, Directorate Air and Energy, The Hague, The Netherlands

Anderson H.R., Ponce de Leon A., Bland J.M., Bower J.S. and Strachan D.P. (1996). Air pollution and daily mortality in London: 1987-92. BMJ 312: 665-9.

Baker, C.K. and Fullwood, A.E. (1986) Leaf damage following crop spraying in winter barley exposed to sulphur dioxide.  Crop Protection 5, 365-367.

Bates D.V., Baker-Anderson M. and Sizto R. (1990).  Asthma attack periodicity:  A study of hospital emergency visits in Vancouver.  Environ Res 51, 51-70.

Brunekreef, B. (1997) Air pollution and life expectancy: is there a relation?  Occup. Environ. Med., 54, 731-734.

Cocks, A. T. and Rodgers,I. R. (1997) Integrated Assessment Modelling and the Acidification Strategy.  National Power, Report no. TECH/ECP/077/97.  National Power, Swindon, UK.

Cody R.P., Weisel C.L., Birnbaum G. and Lioy P.J. (1992).  The effect of ozone associated with summertime photo-chemical smog on the frequency of asthma visits to hospital emergency departments.  Environ Res 58, 184-194.

Collins, W.J., Stevenson, D.S., Johnson, C.E. and Derwent, R.G. (1997) Tropospheric ozone in a global-scale three-dimensional Lagrangian model and its response to NOx emissions controls.  Journal of Atmospheric Chemistry 26, 223-274.

COMEAP (1999)  Committee on the Medical Effects of Air Pollutants.  Quantification of the Effects of Air Pollution on Health in the United Kingdom.  The Stationery Office, London.

Dab, W., Quenel, S.M.P., Le Moullec, Y., Le Tertre, A., Thelot, B., Monteil, C., Lameloise, P., Pirard, P., Momas, I., Ferry, R. and Festy, B. (1996). Short term respiratory health effects of ambient air pollution: results of the APHEA project in Paris. J Epidem Comm Health 50 (suppl 1): S42-46.

DETR (1998) An Economic Analysis of the National Air Quality Strategy Objectives.  An Interim Report of the Interdepartmental Group on Costs and Benefits.  DETR, London.

DETR (1999) Review of the United Kingdom National Air Quality Strategy.  A Consultation Document.  DETR, London.

Department of Health (1998)  Committee on the Medical Effects of Air Pollutants (COMEAP).  Quantification of the Effects of Air Pollution on Health in the United Kingdom.  The Stationery Office, London.

Department of Health (1999)  Economic Appraisal of the Health Effects of Air Pollution (EAHEAP).  The Stationery Office.

Dockery, D.W., Speizer, F.E., Stram, D.O., Ware, J.H., Spengler, J.D. and Ferries, B.G. (1989).  Effects of inhalable particles on respiratory health of children.  Am Rev Respir Dis 139, 587-594.

DoE (1997) The United Kingdom Air Quality Strategy.

Dusseldorp, A., Kruize, H., Brunekreef, B., Hofschreuder, P., de Meer, G. and van Oudvorst, A.B. (1995). Associations of PM10 and airborne iron with respiratory health of adults near a steel factory. Am J Respir Crit Care Med 152, 1932-9.

EAHEAP (1999) Economic Appraisal of the Health Effects of Air Pollution.  Deapartment of Health.  The Stationery Office.

Ecotec (1996).  An evaluation of the benefits of reduced sulphur dioxide emissions from reduced building damage.  Ecotec Research and Consulting Ltd., Birmingham, UK.

European Commission (1995) DGXII, Science, Research and Development, JOULE Programme.  Externalities of fuel cycles, ExternE Project.  Report number 2, Methodology.

European Commission DGXII, Science, Research and Development JOULE (1999).  Externalities of Energy, ‘ExternE’ Project.  Methodology Report, 2nd Edition (to be published).

FAO (1994) Food and Agriculture Organisation.  Prices of Agricultural Products and Selected Inputs in Europe and North America, 1992/93.

Haynie, F.H. (1986).  Atmospheric Acid Deposition Damage due to Paints.  US Environmental Protection Agency Report EPA/600/M-85/019.

Holland, M. R., Forster, D., Wenborn, M. and King, K. (1999) Economic Evaluation of Proposals for the Multi-Effect, Multi-Pollutant Protocol.  Report to the UNECE Task Force on Economic Aspects of Abatement Strategies, made under a contract to European Commission, DGXI.

Hoos, D., Jansen, R., Kehl, J., Noeke, J., and Popal, K. (1987).  Gebäudeschäden durch Luftverunreinigungen - Entwurf eines Erhebungsmodells und Zusammenfassung von Projektergebnissen.  Institut für Umweltschutz, Universität Dortmund, 1987.

Hurley, J. F., and Donnan, P.T. (1997) Effects of Air Pollution on Health.  Report for the ExternE Project, EC DGXII (JOULE Programme).  To be published in European Commission, 1998.

Hurley, J.F. et al (1999) Quantification and valuation of the health effects of fine particles.  Forthcoming report for the Department of Health.

Jones, A. (1981) The estimation of long range dispersion and deposition of continous releases of radionuclides to atmosphere.  National Radiological Protection Board Report NRPB-123.

Jones, H.E., Howson, G., Rosengren-Brinck, U., Hornung, M. and Linares, P. (1997)  Review of the Effects of Air Pollutants on Agricultural Crops, Forestry and Natural Vegetation.  Institute of Terrestrial Ecology Report T07074f5.

Karenlampi, L., and Skarby, L. (1996) Critical Levels for Ozone in Europe: Testing and Finalising the Concepts (A UNECE Workshop Report).  Department of Ecology and Environmental Science; University of Kuopio.

Krupnick A.J., Harrington W., Ostro B. (1990).  Ambient ozone and acute health effects:  Evidence from daily data.  J. Environ Econ Manage 18, 1-18.

Kucera, V., Henriksen, J., Leygraf, C., Coote, A.T., Knotkova, D. and Stöckle, B. (1993a).  Materials Damage Caused by Acidifying Air Pollutants - 4 Year Results from an International Exposure Programme within UNECE.  International Corrosion Congress, Houston, September 1993.

Kucera, V., Henriksen, J., Knotkova, D., Sjöström, Ch. (1993b).  Model for Calculations of Corrosion Cost Caused by Air Pollution and Its Application in Three Cities.  Report No. 084, Swedish Corrosion Institute, Roslagsvägen, 1993.

Kucera, V. (1994).  The UNECE International Cooperative Programme on Effects on Materials, Including Historic and Cultural Monuments.  Report to the working group on effects within the UNECE in Geneva, Swedish Corrosion Institute, Stockholm, 1994.

Markandya (1999) Green Accounting Research Project Phase II.  A. Markandya (Ed.) University of Bath.

Metcalfe, S.E., Derwent, R.G., Whyatt, J.D. and Dyke, H. (1998) Nitrogen deposition and strategies for the control of acidification and eutrophication across Great Britain.  Water, Air and Soil Pollution 107, 121-145.

Metcalfe, S.E., Whyatt, J.D. and Derwent, R.G. (1995) A comparison of model and observed network estimates of sulphur deposition across Great Britain for 1990 and its likely source attribution.  Quarterly Journal of the Royal Meteorological Society 121, 1387-1411.

NAEI (1997) National Atmospheric Emissions Inventory.  DETR, London.

NERA (1997) Valuation of Deaths from Air Pollution.  National Economic Research Associates.  Draft Final Report for the Department of the Environment, Transport and the Regions.

Nix, J. (1990) Farm Price Pocketbook, 20th Edition.  Wye College, London.

Ostro B.D. (1987).  Air pollution and morbidity revisited:  A specification test.  J Environ Econ Manage 14, 87-98.

Ostro B.D. and Rothschild S. (1989).  Air pollution and acute respiratory morbidity:  An observational study of multiple pollutants.  Environ Res 50, 238-247.

Ponce de Leon A., Anderson H.R., Bland J.M., Strachan D.P. and Bower J. (1996). Effects of air pollution on daily hospital admissions for respiratory disease in London between 1987-88 and 1991-92. J Epidem Comm Health 50 (suppl 1): S63-70.

Pope C.A. and Dockery D.W. (1992). Acute health effects of PM10 pollution on symptomatic and asymptomatic children. Am Rev Respir Dis 145, 1123-1126.

Pope C.A. III, Thun M.J., Namboodiri M.M., Dockery D.W., Evans J.S., Speizer F.E. and Heath C.W. Jr. (1995). Particulate air pollution as predictor of mortality in a prospective study of US adults. Am J Resp Crit Care Med 151: 669-674.

PORG (1997) Ozone in the United Kingdom.  4th Report of the Photochemical Oxidants Review Group.

Rayner, C. (1994)  Philip’s Geographical Digest 1994-95.  Heinemann-Philip Atlases, Heinemann Educational Books Ltd., Oxford, UK.

RGAR (1997) Review Group on Acid Rain.  DETR, London.

Roberts, T.M. (1984) Long term effects of SO2 on crops; an analysis of dose - response relations.  Philosophical Transactions of the Royal Society, London, Series B 305, 299-316.

Roemer W., Hoek G. and Brunekreef B. (1993). Effect of ambient winter air pollution on respiratory health of children with chronic respiratory symptoms. Am Rev Respir Dis 147, 118-124.

Rowe, R.D. et al (1995) New York State Environmental Externalities Cost Study.  Oceana Publications Ltd. New York.

Schulze, E.-D., Oren, R., and Lange, O.L. (1989) Processes leading to forest decline: a synthesis.  From ‘Forest Decline and Air Pollution: A Study of Spruce (Picea abies) on Acid Soils’, Schulze, E.-D., Lange, O.L. and Oren, R. (Eds.) Springer-Verlag, Berlin, pp 459-468.

Schwartz J. (1993). Particulate air pollution and chronic respiratory disease. Environ Res 62, 7-13.

Schwartz J. and Morris R. (1995) Air pollution and hospital admissions for cardiovascular disease in Detroit, Michigan. Am J Epidem 142, 23-35.  Am J Epidem 137, 701-705.

SEI (1999).  Costs and Strategies presented by Industry during the Negotiation of Environmental Regulations.  Prepared for the Swedish Ministry. Stockholm Environment Institute, University of York, UK.

Simpson, D. 1992. "Long period modelling of photochemical oxidants in Europe. Model calculations for July 1985". Atmospheric Environment, vol. 26A, p.1609-1634.

Simpson, D. 1993. "Photochemical model calculations over Europe for two extended summer periods: 1985 and 1989. Model results and comparison with observations". Atmospheric Environment, vol. 27A, p.921-943.

Simpson, D. and Malik, S. (1996)  Photochemical Oxidant Modelling and Source Receptor Relationships for Ozone in ‘Transboundary Air Pollution in Europe’ (Barrett, K. and Berge, E., Eds.)  EMEP MSC-W Report 1/96.  Det Norske Meteorologiske Institutt.

Sisler, J. F. et al (1994) The relative importance of soluble aerosols to spatial and seasonal trends of impaired visibility in the United States.  Atmospheric Environment, 28, no. 5.

Tolstoy, N., Andersson, G., Sjöström, Ch., and Kucera, V. (1990).  External Building Materials - Quantities and Degradation. Research Report TN:19.  The National Swedish Institute for Building Research, Gävle, Sweden, 1990.

Veldkamp, H. and R. van de Velde, (1995), Mapping Land Use and Land Cover for Environmnetal Monitoring on a European Scale, in: M. Posch, P.A.M. de Smet, J.-P.Hettelingh and R.J.Downing, Calculation and Mapping Critical thresholds in Europe: Status report 1995, RIVM-Coordination Center for Effects, Bilthoven, The Netherlands, 198 pp.

Whittemore, A.S. and Korn, E.L. (1980). Asthma and air pollution in the Los Angeles area. Am J Public Health 70, 687-696.

WHO (1997) World Health Organisation Air Quality Guidelines for Europe 1996: Particulate Matter.  Edited Final draft, September 1997.  WHO Regional Office for Europe, Bilthoven, the Netherlands.

Wordley J., Walters S. and Ayres J.G. (1997)  Short term variations in hospital admissions and mortality and particulate air pollution. (In press). Carcinogenic Effects of Radionuclide Emissions.

The individual elements of uncertainty, arising at each stage of the analysis of each type of impact are certainly important in that they affect the overall confidence in the final results, but they are of secondary importance to the aggregated uncertainty.

For example, if one accepts, as COMEAP have done, that air pollution is causally linked to mortality and respiratory hospital admissions, it may well seem likely that it is also linked to less severe effects, such as increased prevalence of the symptoms of asthma.

The population weighted mean particulate exposure is calculated by summing the product of (population*concentration) over all grid cells for the country, and then divided by total population.  AOT40 is calculated by summing concentrations of ozone in excess of 40 ppb over the growing season for plants.  Hence exposure to 50 ppb for 8 hours would lead to an AOT40 of (8*(50-40)) = 80 ppb.hours.  The accumulation of ppb.hours over the growing season leads to the use of the unit ppm.hours (parts per million.hours).  This is multiplied by the area affected to give the units shown in the Table.  AOT60 is calculated similarly over the summer months, and multiplied by the number of people exposed to give the units of person.ppm.hours.

Further details on the derivation and calculation of critical loads in the UK can be found in the following report, also on the ITE Monks Wood web site (http://www.nmw.ac.uk/ite/monk/critical_loads/nclmp.html):

Hall, J., Bull, K., Bradley, I., Curtis, C, Freer-Smith, P., Hornung, M., Howard, D., Langan, S., Loveland, P., Reynolds, B., Ullyett, J., Warr. T.   1998.  Status of UK Critical Loads and Exceedances January 1998.  Part 1 – Critical Loads and Critical Load Maps.  Report to Department of Environment, Transport & the Regions.  NERC/DETR Contract EPG1/3/116.

More specifically, RAINS converts AOT60 values below this threshold to zero, but leaves values above the threshold unchanged.

In this case, all AOT60 values are the excess over the 0.4 ppm hour threshold.